| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0888-v3 |
| --- | --- |

| Title: | CE1-related: Picture boundary CU split satisfying the VPDU constraint |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chia-Ming Tsai, Shih-Ta Hsiang, Chih-Wei Hsu, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chia-ming.tsai, shih-ta.hsiang, cw.hsu, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes two solutions to satisfy the virtual pipeline data unit (VPDU) constraint at picture boundaries, where the VPDU size is targeted to be 64x64-luma/32x32-chroma for 4:2:0 chroma sampling format. In VTM3.0, the VPDU partitioning constraint is enforced for coding units (CUs) that are entirely inside the picture but may be violated for CUs at picture boundaries. In solution 1, if one coding tree unit (CTU) of 128x128-luma/64x64-chroma is not completely inside the picture, quad-tree (QT) split is applied to partition the CTU in the first depth instead of the explicit selection between QT split and binary-tree (BT) split. In solution 2, if “one CU at the bottom picture boundary is not completely inside the picture, the CU width is larger than the VPDU width (i.e., 64 luma samples), and the CU height is smaller than or equal to the VPDU height (i.e., 64 luma samples),” then the implicitly horizontal BT split is replaced with an implicitly vertical BT split. Similarly, if “one CU at the right picture boundary is not completely inside the picture, the CU height is larger than the VPDU height, and the CU width is smaller than or equal to the VPDU width,” then the implicitly vertical BT split is replaced with an implicitly horizontal BT split. In both solutions, the VPDU constraint is satisfied for the entire picture including all picture boundaries. BD-rate results of these two solutions are reported as follows.

Solution 1:
RA BD-rates = 0.06% (Y), 0.08% (U), 0.06% (V)
LB BD-rates = 0.13% (Y), -0.02% (U), 0.07% (V)

Solution 2:
RA BD-rates = 0.05% (Y), 0.04% (U), 0.06% (V)
LB BD-rates = 0.07% (Y), -0.06% (U), 0.03% (V)

It is asserted that the runtime impact is small if accurately measured.

# Introduction

VPDUs are defined as non-overlapping MxM-luma(L)/NxN-chroma(C) units in a picture [1]. The VPDU constraint is described as follows.

VPDU constraint: For each CTU, the following two conditions shall not be violated, and the processing order of CUs shall not leave a VPDU and re-visit it later.
Condition 1: For each VPDU containing one or multiple CUs, the CUs are completely contained in the VPDU.
Condition 2: For each CU containing one or more VPDUs, the VPDUs are completely contained in the CU.

Examples in Figure 1 are allowed BT and TT splits by the constraint. In VVC, we target M and N equal to 128 and 64, respectively.

Figure 1.  Examples of allowed BT and TT splits for 64x64-L/32x32-C pipelining (VPDUs indicated by dashed lines)

However, in VTM3.0 [2], CU splits at picture boundaries could violate the VPDU constraint. For example, in Figure 2, the processing order of CUs leaves the top left VPDU and then re-visits the top left VPDU later. Therefore, hardware decoders have to handle exceptions at picture boundaries, which is not desirable.

Figure 2.  An example for the processing order of CUs leaving a VPDU and re-visiting it later. (VPDUs are indicated by dashed lines, picture boundary is the blue line, the region outside the picture is in gray, vertical BT split is the green line, and the numbers show the processing orders of the four CUs.)

# Proposed methods

## Solution 1

If one CTU of 128x128-luma/64x64-chroma is not completely inside the picture, QT split is applied to partition the CTU in the first depth instead of BT split.

An example is shown in Figure 3.

Figure 3.  The two examples on the left side (i.e., (a) and (c)) for applying QT split in the first depth (Depth = 0) to a CTU that has 128xN/Nx128 luma samples inside the picture, and the two examples on the right side (i.e., (b) and (d)) show the processing order for applying horizontal/vertical BT split in the second depth (Depth = 1). (VPDUs are indicated by dashed lines, picture boundary is the blue line, and the region outside the picture is in gray.)

## Solution 2

If “one CU at the bottom picture boundary is not completely inside the picture, the CU width is larger than the VPDU width, and the CU height is smaller than or equal to the VPDU height,” then the implicitly horizontal BT split is replaced with an implicitly vertical BT split.

If “one CU at the right picture boundary is not completely inside the picture, the CU height is larger than the VPDU height, and the CU width is smaller than or equal to the VPDU width,” then the implicitly vertical BT split is replaced with an implicitly horizontal BT split.

An example of solution 2 is shown in Figure 4. The CTU is not completely inside the picture, the picture boundary process is applied. In stage 1, only QT and horizontal BT are allowed at depth 0 in the current VVC and are illustrated in Figure 4 (a) and (b), respectively. In stage 2 of Figure 4 (a), according to the current VVC, only QT split or horizontal BT split are allowed for the bottom two CUs of depth 1, and it is shown that selecting horizontal BT split for the bottom two CUs of depth 1 will not violate the VPDU constraint. In stage 2 of Figure 4 (b), according to the current VVC, only horizontal BT split is allowed for the bottom CU of depth 1; however, the horizontal BT split forced for the bottom CU of depth 1 will violate the VPDU constraint. Therefore, in this case the proposed solution 2 replaces the implicitly horizontal BT split with an implicitly vertical BT split in stage 2.

Figure 4.  Two examples at the bottom picture boundary for applying picture boundary process to one CTU with 128x112 luma samples inside the picture. (a) QT split is signalled at depth 0 and horizontal BT split is signalled at the bottom two CUs of depth 1, which does not violate the VPDU constraint. (b) Horizontal BT split is signalled at depth 0 and the proposed implicitly vertical BT split is applied to the bottom CU of depth 1 to satisfy the VPDU constraint. (VPDUs are indicated by dashed lines, picture boundary is the blue line, and the region outside the picture is in gray.)

# Experimental results

The proposed methods have been implemented on VTM3.0 [2]. The simulations were performed following the JVET common test conditions (CTC) [3]. Table 1 and Table 2 are the results for solution 1 and solution 2, respectively. Runtime percentages are not accurate. It is asserted that the runtime impact is small if accurately measured.

Table 1.  Results of solution 1.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.10% | 0.05% | 100% | 104% |
| Class A2 | 0.04% | 0.06% | 0.09% | 100% | 101% |
| Class B | 0.07% | 0.09% | 0.03% | 100% | 103% |
| Class C | 0.03% | 0.05% | 0.08% | 98% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | 0.08% | 0.06% | 99% | 102% |
| Class D | 0.07% | 0.13% | 0.09% | 98% | 100% |
| Class F | 0.04% | 0.05% | 0.06% | 99% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.07% | -0.19% | 0.15% | 100% | 99% |
| Class C | 0.03% | 0.15% | 0.08% | 100% | 100% |
| Class E | 0.35% | 0.06% | -0.07% | 99% | 103% |
| Overall | 0.13% | -0.02% | 0.07% | 100% | 100% |
| Class D | 0.06% | 0.40% | 0.24% | 101% | 101% |
| Class F | 0.11% | 0.47% | 0.08% | 100% | 100% |

Table 2.  Results of solution 2.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.03% | 0.05% | 100% | 113% |
| Class A2 | 0.02% | 0.03% | 0.04% | 100% | 105% |
| Class B | 0.08% | 0.03% | 0.08% | 100% | 104% |
| Class C | 0.02% | 0.07% | 0.05% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.04% | 0.06% | 100% | 105% |
| Class D | 0.06% | 0.07% | 0.03% | 100% | 98% |
| Class F | 0.03% | 0.02% | 0.02% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.08% | -0.17% | 0.01% | 100% | 99% |
| Class C | -0.02% | 0.14% | 0.18% | 100% | 99% |
| Class E | 0.18% | -0.15% | -0.14% | 100% | 105% |
| Overall | 0.07% | -0.06% | 0.03% | 100% | 101% |
| Class D | 0.03% | -0.09% | 0.20% | 100% | 98% |
| Class F | 0.05% | 0.07% | 0.26% | 100% | 99% |

# Conclusions

This contribution proposes two solutions to ensure that the VPDU constraint is satisfied at picture boundaries.

# References

- C.-M. Tsai, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei, “CE1.2.1: Constraint for binary and ternary partitions,” Document of Joint Video Experts Team, JVET-L0081.

- VTM3.0: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-L1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).