| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M1021_v2 |
| --- | --- |

| Title: | Description of Core Experiment 1 (CE1): Post-prediction and post-reconstruction filtering |  |  |
| --- | --- | --- | --- |
| Status: | Output document from JVET |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | Jacob Ström Sergey Ikonin Vadim Seregin | Tel: Email: | +46 702672192 jacob.strom@ericsson.com, sergey.ikonin@huawei.com, vseregin@qti.qualcomm.com. |
| Source: | CE1 coordinators |  |  |

_____________________________

# Abstract

The goal of this Core Experiment (CE) is to study technologies of post-prediction/reconstruction filtering methods. The techniques are evaluated according to BD-rate gain, complexity (in both encoder and decoder). Participants in this activity are Ericsson, Huawei, HHI, Qualcomm, Panasonic, Interdigital, Mediatek, Technicolor.

The software basis for the mandatory test in this CE is VTM-4.0, Test sequences, configurations and test conditions are according to JVET-M1021 for SDR, RA and LDB configurations are tested.

# Test conditions and evaluation criteria

VTM-4.0 will be used as anchor in this CE. Proposals will primarily be compared with respect to objective quality, but complexity will also be measured according to the subtest requirements.

# Tool descriptions

## 2.1 Test specification

| Test# |  | Description |  | Document# |  |
| --- | --- | --- | --- | --- | --- |
| 1.1 |  | Reduced complexity bilateral filter |  | JVET-M0885, JVET-M0884 |  |
| 1.2 |  | Hadamard transform domain filter |  | JVET-M0468 |  |
| 1.4 |  | Uniform Luma Inter Prediction Filter |  | JVET-M0042 |  |
| 1.5 |  | Local illumination compensation |  | JVET-M0088, M0115, M0182, M0224, M0450, M0500 |  |
| 1.6 |  | Intra Reference Sample Deblocking |  | JVET-M0138 |  |
| 1.7 |  | Combined tests |  | Combinations of above documents |  |

## 2.2  Detailed description of tests

In this description, will use the term processing to refer to either filtering (1.1-1.4) or illumination compensation (1.5). The following constraints apply to all tests except for the loop filter tests (CE1-1.5 and CE1-2.3 and CE1-1.6):

For all tests, the size of the smallest processed block will be 64 luma samples.

Processing should not be applied on intra blocks, nor should it be applied to combined intra/inter blocks (CIIP blocks) or intra block copy (IBC) / current picture reference (CPR) blocks.

For all tests, the following three constraint conditions may be tested:

- No restriction on using samples to the left and above the block (mandatory)

- Samples to the left and above can be used, but only if they come from an inter block (i.e., not from an intra block or a CIIP block or an IBC/CPR block). (mandatory)

- No samples outside the block can be used. (non-mandatory)

For loop-filter tests, these constraints are relaxed:

- There is no restriction on the smallest processed block.

- There is no restriction to apply filtering on intra blocks, i.e., both intra and inter blocks can be filtered.

- There is no restriction on using samples to the left or above the block, as long as the used samples reside in the same CTU as the filtered pixels.

For CE1-1.6, these constraints are relaxed:

- No restriction to apply for intra blocks or intra/inter blocks and thus also no restriction to use samples to the left or above the block (same as intra prediction in VTM).

CE 1.1 Reduced complexity parametrizable bilateral filter (JVET-M0885, JVET-M0894)

This sub-CE conducts a joined test on parametrized bilateral filtering method based on the following contributions:

JVET-M0885 J. Strom (Ericsson), Non-CE: Reduced complexity bilateral filter

JVET-M0894 M. Karczewicz (Qualcomm), Non-CE: Test on parametrizable bilateral filter from JVET-L0406 in VTM3.0

Bilateral filters of JVET-M0885 and JVET-M0894 both share the following design elements:

- 3x3 plus-shape filter support, as proposed in JVET-D0069

- Non-local filtering/weights derivation for inter, as proposed in JVET-J0021

- Weights derivation method:

  - Originally proposed in JVET-D0069, modified in JVET-J0021

- Block-based adaptation, as proposed in JVET-D0069

- Identical constrains on intra/inter applicability, QP and block-size constrains.

- PWL parametrization of BiF weights:

  - Linear model (1 piece) in JVET-M0885

  - 2-piece PWL in JVET-L0406

And finally, JVET-M0885 proposed a complexity reductions obtained by using shifted factors in the multiplications.

A lower-complexity BiF with 1 piece linear model parametrization (as in JVET-M0885) is selected as starting point in tests CE1-1.1a-c.

In JVET-M0885, the first multiplication was reduced to a two-bit multiplier. However, it is not clear that this is the best trade-off between BD-rate gain and multiplier complexity. Therefore, tests CE1-1.2a-c will perform tests where the

multipliers have been shifted to provide a better trade-off between complexity and BD-rate.

Test CE1-1.3a-c will investigate the loss associated with removing the latency introduced by the non-local filtering step, since this was identified at the Macau meeting as needing an extra cycle of latency.

It was requested at the Marrakech meeting to investigate making the bilateral filter switchable on a per-block level. This is tested in CE1-1.4a-c.

It was also requested that the performance of the should be investigated of using the bilateral filter as a loop-filter. This is tried out in CE-1.5.

CE1-1.6a-c will test the bilateral filter as a post prediction filter.

Different order PWL model for BiF parametrization were used in JVET-M0885 and JVET-M0894. However, it is not clear if parametrical model of base design provides the best trade-off between BD-rate gain complexity. The CE1-1.7 will conduct test on parametrization of the BiF. Note that the parameterizable technology from JVET-M0894 and JVET-L0406 (more than one linear piece) only applies to CE1-1.7.

The table below summarizes the planned test and lists tester and cross-checkers.

| # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| CE1-1.1a | Base BiF design as in JVET-M0885 | Ericsson | J. Rasch, A. Karabutov |
| CE1-1.1b | Base BiF design, restriction b | Ericsson | J. Rasch, A. Karabutov |
| CE1-1.1c | Base BiF design, restriction c | Ericsson | J. Rasch, A. Karabutov |
| CE1-1.2a | BiF with reduced multiplication complexity | Ericsson | J. Rasch, S. Ikonin |
| CE1-1.2b | BiF with reduced multiplication complexity, restriction b | Ericsson | J. Rasch, S. Ikonin |
| CE1-1.2c | BiF with reduced multiplication complexity, restriction c | Ericsson | J. Rasch, S. Ikonin |
| CE1-1.3a | BiF w/o non-local samples | Ericsson | J. Rasch, S. Ikonin |
| CE1-1.3b | BiF w/o non-local samples, restriction b | Ericsson | J. Rasch, S. Ikonin |
| CE1-1.3c | BiF w/o non-local samples, restriction c | Ericsson | J. Rasch, S. Ikonin |
| CE1-1.4a | BiF w/ per-block signaling | Qualcomm | A. Karabutov |
| CE1-1.4b | BiF w/ per-block signaling, restrictions b | Qualcomm | A. Karabutov |
| CE1-1.4c | BiF w/ per-block signaling, restrictions c | Qualcomm | A. Karabutov |
| CE1-1.5 | BiF as loop filter | Ericsson | J. Rasch, S. Ikonin |
| CE1-1.6 | BiF as post-prediction | Qualcomm |  |
| CE1-1.7 | Parametrization aspects of BiF | Qualcomm |  |

CE 1.2 Hadamard transform domain filter (JVET-M0468)

This sub-CE will test performance of the Hadamard transform domain filter corresponding to Test3 of JVET-M0468. In accordance to the meeting discussion the filter will also be tested with block-level flag and as conventional in-loop filter.

The following tests will be conducted:

| Test | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE1-2.1a | Filter without restriction on using reconstructed samples. | S. Ikonin | J. Ström |
| CE1-2.1b | Filter using reconstructed samples from inter blocks only | S. Ikonin | J. Ström |
| CE1-2.1c | Filter without using of reconstructed samples | S. Ikonin | J. Ström |
| CE1-2.2a | Filter is applied based on block-level flag and without restriction on using reconstructed samples. | S. Ikonin | J. Ström |
| CE1-2.2b | Filter is applied based on block-level flag and using reconstructed samples from inter blocks only. | S. Ikonin | J. Ström, Qualcomm |
| CE1-2.2c | Filter is applied based on block-level flag and without using of reconstructed samples | S. Ikonin | J. Ström, Qualcomm |
| CE1-2.3 | Filter is applied as conventional in-loop filter (no restriction on block size or block type, samples outside the block can be used). | S. Ikonin | J. Ström, Qualcomm |
| CE1-2.4a | Filter is applied as prediction post-filter based on block-level flag and without restriction on using reconstructed samples. | S. Ikonin | Qualcomm |
| CE1-2.4b | Filter is applied as prediction post-filter based on block-level flag and using reconstructed samples from inter blocks only. | S. Ikonin | Qualcomm |
| CE1-2.4c | Filter is applied as prediction post-filter based on block-level flag and without using of reconstructed samples | S. Ikonin | Qualcomm |

CE 1.4 Uniform Luma Inter Prediction Filter (JVET-M0042)

The luma inter prediction filter uses different uniform filter modes to improve the inter prediction signal. The following restrictions shall be tested on all tests:

- use reconstructed samples (constraint a above)

- use only inter reconstructed samples (constraint b above)

- use no reconstructed samples (constraint c above)

| Test | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE1-4.1a | Adapt filter to block size, restriction a | HHI | S. Ikonin |
| CE1-4.1b | Adapt filter to block size, restriction b | HHI | S. Ikonin |
| CE1-4.1c | Adapt filter to block size, restriction c | HHI | S. Ikonin |
| CE1-4.2a | Adapt encoder search, restriction a | HHI | S. Ikonin |
| CE1-4.2b | Adapt encoder search, restriction b | HHI | S. Ikonin |
| CE1-4.2c | Adapt encoder search, restriction c | HHI | S. Ikonin |
| CE1-4.3a | Adapt filter application to certain inter modes, restriction a | HHI | J. Ström, A. Karabutov |
| CE1-4.3b | Adapt filter application to certain inter modes, restriction b | HHI | J. Ström, A. Karabutov |
| CE1-4.3c | Adapt filter application to certain inter modes, restriction c | HHI | J. Ström, A. Karabutov |
| CE1-4.4a | Adapt boundary condition, restriction a | HHI | J. Ström, A. Karabutov |
| CE1-4.4b | Adapt boundary condition, restriction b | HHI | J. Ström, A. Karabutov |
| CE1-4.5a | Adapt filter coefficients, restriction a | HHI | J. Ström, A. Karabutov |
| CE1-4.5b | Adapt filter coefficients, restriction b | HHI | J. Ström, A. Karabutov |
| CE1-4.5c | Adapt filter coefficients, restriction c | HHI | J. Ström, A. Karabutov |

CE 1.5 Local illumination compensation (JVET-M0088, M0115, M0182, M0224, M0450, M0500)

Base LIC version as described in the meeting notes:

- Usage of reconstructed intra coded, CIIP and IBC blocks, as this would have impact on complexity and parallel implementation pipelining (test with and without, and further study the impact)

- Disabling LIC over CTU boundaries is not necessary

- Disable for CIIP and IBC blocks

- Disable LIC for the blocks with less than 64 luma samples.

- No temporal inheritance of LIC flag

- No pruning based on LIC flag in merging candidate list generation

- Disable LIC for all subblock modes, including triangle mode and BDOF

- It is not applied to 128xN and Nx128 blocks

- It is not applied to bi-prediction

When LIC is not applied, LIC flag is not signaled.

There are 2 variants of the base version:

- all reconstructed samples are used

- neighbor samples of intra, CIIP and IBC coded blocks are excluded, i.e. only inter reconstructed samples are used

Below tests will be performed for each base version.

A combination of the several CE tests will be performed for better coding efficiency.

| Test | Description | Tester |  | Cross-checker |
| --- | --- | --- | --- | --- |
| CE1-5.1 | Base LIC version | Vadim Seregin vseregin@qti.qualcomm.com |  | Alexander Karabutov karabutov.alexander@huawei.com |
| CE1-5.2 | Test 5.1, where LIC parameters are derived for the first VPDU and are used for all other VPDUs for 128xN and Nx128 blocks | Kiyofumi Abe abe.kiyo@jp.panasonic.com |  | Saurav Bandyopadhyay Saurav.Bandyopadhyay @InterDigital.com Alexander Karabutov karabutov.alexander@huawei.com |
| CE1-5.3 | Test 5.1, where LIC is applied after bi-prediction is derived. | Saurav Bandyopadhyay Saurav.Bandyopadhyay @InterDigital.com |  | Kiyofumi Abe abe.kiyo@jp.panasonic.com |
| CE1-5.4 | Test 5.4, but LIC is not applied with BPWA | Philippe Bordes philippe.bordes@technicolor.com |  | Chia-Ming Tsai chia-ming.tsai@mediatek.com |
| CE1-5.5 | Test 5.4, but LIC is not applied with BPWA and encoder speed-up |  | Chia-Ming Tsai chia-ming.tsai@mediatek.com | Philippe Bordes philippe.bordes@technicolor.com |
| CE1-5.6 | Test 5.1 with modified bS derivation of DBF using lic_flag | Kiyofumi Abe abe.kiyo@jp.panasonic.com |  | Saurav Bandyopadhyay Saurav.Bandyopadhyay @InterDigital.com |
| CE1-5.7 | Test 5.1 with 16x16 unit processing |  | Kiyofumi Abe abe.kiyo@jp.panasonic.com | Vadim Seregin vseregin@qti.qualcomm.com |

CE1-6 Intra Reference Sample Deblocking (JVET-M0138)

Test CE1-6.1 IRSDB

JVET-M0138 presents an Intra Reference Sample Deblocking (IRSDB) method for angular intra prediction of blocks with both width and height equal to or larger than 32.

IRSDB is designed to symmetrically deblock 7 unfiltered reference samples on respective side of a block boundary to eliminate the discontinuity between reference samples which are populated from two neighboring large blocks. IRSDB first define a set of checkpoints that includes the following positions:

- at the sample to the top-left of the current block

- at each position (X, Y) above the current block that X is divisible by 32

- at each position (X, Y) to the left of the current block that is Y divisible by 32

The proposed IRSDB method in JVET-M0138 will be tested using following simplifications/modification:

- Remove the boundary check to determine if checkpoint position is at a PU or TU boundary and if block P and block Q are large blocks.

- Remove the clip from the filtering.

Test CE1-6.2 HEVC strong reference sample smoothing

HEVC strong reference sample smoothing filter is a method in HEVC which not is present in VVC but exists in VTM (default turned off). The HEVC strong reference sample smoothing filter is applied to the luma reference samples of large blocks that have both a block width and a block height larger than or equal to 32 luma samples. The HEVC strong reference sample smoothing filter firstly test the signal activity. If the signal activity for both reference samples at the left and above the current block are below a threshold, the reference samples are modified by a linear interpolation between the top-right reference sample and the top-left reference sample, and by a linear interpolation between the bottom-left reference sample and the top-left reference sample.

In the CE test, the HEVC strong reference sample smoothing implementation is aligned with HEVC to only modify twice as many samples as the block width minus 1 for above reference samples and twice as many samples as the block height minus 1 for left reference samples both modifications based on linear interpolation between the top-left reference sample and a sample at a distance of twice the block width respectively twice the block height from the top-reference sample.

## Evaluation specific for CE1-6.1 and CE1-6.2

The following aspects will be investigated:

- Subjective performance for video sequences.

The following sequences will be used for subjective performance comparisons

| Num | Name | Resolution | fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | Tango2 | 3840x2160p | 60 | 600 | RA | 34, 39 |
| 02 | Cactus | 1920x1080p | 50 | 500 | RA | 34, 39 |
| 03 | KristenAndSara | 1280x720p | 60fps | 600 | LDP | 34, 39 |

- Report complexity measures according to section 2.2 for both CE1.6.1 and CE1.6.2.

| Test | Description |  |  | Tester | Cross-checker |  |
| --- | --- | --- | --- | --- | --- | --- |
| CE1-6.1 | Intra reference sample deblocking |  |  | Ericsson |  |  |
| CE1-6.2 | HEVC strong reference sample smoothing |  |  | Ericsson |  |  |

CE 1.7 Combined tests

The following combined tests will be conducted:

| Test |  | Description |  |  |  | Tester | Cross-checker |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CE1-7.1 | Uniform Luma Inter Prediction Filter and BIF as a loop-filter on top, restriction b |  |  | HHI Ericsson |  |  |  | A. Karabutov |  |
| CE1-7.2 | Combine Uniform Luma Inter Prediction Filter and BIF mutually exclusive, restriction b |  |  | HHI Ericsson |  |  |  | A. Karabutov |  |
| CE1-7.3 | Uniform Luma Inter Prediction Filter and Hadamard filter on top, restriction b |  |  | HHI Huawei |  |  |  | J. Ström |  |
| CE1-7.4 | Combine Uniform Luma Inter Prediction Filter and Hadamard filter mutually exclusive, restriction b |  |  | HHI Huawei |  |  |  | J. Ström |  |
| CE1-7.5 | Combination of CE1-4.1a with CE1-5.1, when methods can be applied one on top of another one (other versions of the CE1-4 and CE1-5 methods studied in CE can be used in the combination test) |  |  | Jennifer Rasch Jennifer.Rasch@hhi.fraunhofer.de Vadim Seregin vseregin@qti.qualcomm.com |  |  |  |  |  |
| CE1-7.6 | Combination of CE1-4.1a with CE1-5.1, when methods are applied in mutual exclusive manner (other versions of the CE1-4 and CE1-5 methods studied in CE can be used in the combination test) |  |  | Jennifer Rasch Jennifer.Rasch@hhi.fraunhofer.de Vadim Seregin vseregin@qti.qualcomm.com |  |  |  |  |  |
| CE1-7.7 | Combination of CE1-2.1 with CE1-5.1 (other versions of the CE1-2 and CE1-5 methods studied in CE can be used in the combination tests) | Huawei |  |
| CE1-7.8 | Combination of CE1-2.1 with CE1-2.4 (other versions of the CE1-2 studied in CE can be used in the combination tests) | Huawei |  |

## Complexity analysis

For all sub-CEs, the following complexity figures should be reported.

| Test# | filter shape | Computational complexity (# of multiplications/ additions/shifts/checks*) per sample | Sequential operation to get one sample processed, e,g, S=A+B+C+D is 2 sequential additions | Operations before block processing | Precision of multiply | Parallel friendly (each sample can be filtered independently from other samples) | Number of clock cycles | memory requirement | How to derive filter coeffs | Min. and max. CU Size for applying filters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 |  |  |  |  |  |  |  |  |  |  |
| 1.2 |  |  |  |  |  |  |  |  |  |  |
| 1.4 |  |  |  |  |  |  |  |  |  |  |
| 1.5.1 |  |  |  |  |  |  |  |  |  |  |
| 1.5.2 |  |  |  |  |  |  |  |  |  |  |
| 1.5.3 |  |  |  |  |  |  |  |  |  |  |
| 1.5.4 |  |  |  |  |  |  |  |  |  |  |
| 1.5.5 |  |  |  |  |  |  |  |  |  |  |
| 1.6.1 |  |  |  |  |  |  |  |  |  |  |
| 1.6.2 |  |  |  |  |  |  |  |  |  |  |

* max/min/abs are countered as checks

# Interdependency with other CEs

No known interdependencies are known.

# Test conditions and data to be provided

The common test conditions are used to evaluate the objective coding gains of different methods.

# Time-line and Responsibilities

T1: 2019-Feb-8: 	Final CE description uploaded

T2: Test model SW release (planned 2019-02-11) + 2 weeks or 4 March, whichever is earlier:		 Integration of all tools into a separate CE branch of the VTM is completed and announced to JVET reflector.

- Initial study by cross-checkers can begin.

- Proponents may continue to modify the software in this branch until T3

- 3rd parties are encouraged to study and make contributions to the next meeting with proposed changes

T3: 3 weeks before the next JVET meeting or T2 + 1 week, whichever is later: Any changes to the CE test branches of the software must be frozen, so the cross-checkers can know exactly what they are cross-checking.

Full test results must be provided at this time (at least for proposals targeting to be promoted to VTM at the next meeting)

# Appendix of Participants

| Nr. | Name | Company | Email | Proponents | Cross-checker |
| --- | --- | --- | --- | --- | --- |
| 1 |  | Jacob Ström |  | Ericsson |  | jacob.strom@ericsson.com |  | X |  | X |  |
| 2 | Zhi Zhang | Ericsson | zhi.zhang@ericsson.com | X |  |
| 3 |  | Sergey Ikonin |  | Huawei |  | sergey.ikonin@huawei.com |  | X |  | X |  |
| 4 |  | Karabutov Alexander |  | Huawei |  | karabutov.alexander@huawei.com |  | X |  | X |  |
| 5 |  | Sychev Maxim |  | Huawei |  | Sychev.Maxim@huawei.com |  | X |  |  |  |
| 6 |  | Filippov Alexey |  | Huawei |  | Alexey.Filippov@huawei.com |  | X |  |  |  |
| 7 |  | Rufitskiy Vasily |  | Huawei |  | Vasily.Rufitskiy@huawei.com |  | X |  |  |  |
| 8 |  | Yanghaitao (Haitao) |  | Huawei |  | haitao.yang@huawei.com |  | X |  |  |  |
| 9 |  | Vadim Seregin |  | Qualcomm |  | vseregin@qti.qualcomm.com |  | X |  |  |  |
| 10 |  | Dmytro Rusanovskyy |  | Qualcomm |  | dmytror@qti.qualcomm.com |  | X |  | X |  |
| 11 |  | Nan Hu |  | Qualcomm |  | nanh@qti.qualcomm.com |  | X |  |  |  |
| 12 |  | Jennifer Rasch |  | HHI |  | Jennifer.Rasch@hhi.fraunhofer.de |  | X |  | X |  |
| 13 |  | Jonathan Pfaff |  | HHI |  | jonathan.pfaff@hhi.fraunhofer.de |  | X |  |  |  |
| 14 |  | Anastasia Henkel |  | HHI |  | anastasia.henkel@hhi-extern.fraunhofer.de |  | X |  |  |  |
| 15 |  | Chih-Wei Hsu |  | MediaTek |  | cw.hsu@mediatek.com |  | X |  |  |  |
| 16 |  | Ching-Yeh Chen |  | MediaTek |  | chingyeh.chen@mediatek.com |  | X |  |  |  |
| 17 |  | Tzu-Der Chuang |  | MediaTek |  | peter.chuang@mediatek.com |  | X |  |  |  |
| 18 |  | Chia-Ming Tsai |  | MediaTek |  | chia-ming.tsai@mediatek.com |  | X |  |  |  |
| 19 |  | Zhi-Yi Lin |  | MediaTek |  | zhi-yi.lin@mediatek.com |  | X |  |  |  |
| 20 |  | Man-Shu Chiang |  | MediaTek |  | man-shu.chiang@mediatek.com |  | X |  |  |  |
| 21 |  | Chun-Chia Chen |  | MediaTek |  | chunchia.chen@mediatek.com |  | X |  |  |  |
| 22 |  | Yu-Wen Huang |  | MediaTek |  | yuwen.huang@mediatek.com |  | X |  |  |  |
| 23 |  | Olena Chubach |  | MediaTek |  | Olena.Chubach@mediatek.com |  | X |  |  |  |
| 24 |  | Philippe Bordes |  | Technicolor |  | philippe.bordes@technicolor.com |  | X |  |  |  |
| 25 |  | Franck Galpin |  | Technicolor |  | Franck.Galpin@technicolor.com |  | X |  |  |  |
| 26 |  | Tangi Poirier |  | Technicolor |  | Tangi.Poirier@technicolor.com |  | X |  |  |  |
| 27 |  | Fabrice LeLeannec |  | Technicolor |  | Fabrice.Leleannec@technicolor.com |  | X |  |  |  |
| 28 |  | Kiyofumi Abe |  | Panasonic |  | abe.kiyo@jp.panasonic.com |  | X |  | X |  |
| 29 |  | Tadamasa Toma |  | Panasonic |  | toma.tadamasa@jp.panasonic.com |  | X |  |  |  |
| 30 |  | Jingya Li |  | Panasonic |  | jingya.li@sg.panasonic.com |  | X |  |  |  |
| 31 |  | Che-Wei Kuo |  | Panasonic |  | chewei.kuo@sg.panasonic.com |  | X |  |  |  |
| 32 |  | Chong Soon Lim |  | Panasonic |  | chongsoon.lim@sg.panasonic.com |  | X |  |  |  |
| 33 |  | Saurav Bandyopadhyay |  | Interdigital |  | saurav.bandyopadhyay@interdigital.com |  |  |  | X |  |
| 34 |  | Yuwen He |  | Interdigital |  | yuwen.he@interdigital.com |  |  |  |  |  |
| 35 |  | Daniel Luo |  | Interdigital |  | Daniel.Luo@InterDigital.com |  |  |  |  |  |
| 36 | Kevin Reuze | Qualcomm | kreuz@qti.qualcomm.com |  | x |

# References

[1] F.Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Macao, CN, 3–12 Oct. 2018.

[2] J. Ström, ”Non-CE: Reduced complexity bilateral filtering”, JVET-M0885, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019

[3] S. Ikonin, V. Stepin, J. Chen, “Non-CE: Hadamard transform domain filter”, JVET-M0468, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019

[4] M. Karczewicz, “Non-CE: Test on parametrizable bilateral filter from JVET-L0406 in VTM3.0”, JVET-M0894, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019

[5] J. Rasch, A. Henkel, J. Pfaff, H. Schwarz, D. Marpe, T. Wiegand. ”CE10: Uniform Directional Diffusion Filters For Video Coding”, JVET-M0042, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019

[6] V. Seregin, W.-J. Chien, T. Hsieh, N. Hu, M. Karczewicz, “CE10-related: Unidirectional illumination compensation”, JVET-M0500, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019

[7] K.Abe, T. Toma, J. Li, C.-W. Kuo, V. Drugeon, “CE10-related: LIC restriction for pipeline structure”, JVET-M0088, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019

[8] C.-M. Tsai, C.-C. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, “CE10-related: Simplification of local illumination compensation”, JVET-M0182, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019