| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M1023-v3 |
| --- | --- |

| Title: | Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Core experiment description |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera Ling Li Alexey Filippov | Email: | geertv@qti.qualcomm.com l.li@lge.com Alexey.Filippov@huawei.com |
| Source: | CE3 coordinators |  |  |

_____________________________

# Abstract

This document describes Core Experiment 3 (CE3) on intra prediction and mode coding. The experiments are conducted after the 13th JVET meeting and the results, including cross-checks, will be reported to the 14th JVET meeting.

# Introduction

In CE3, a total of 37 tests are defined, which are organized in the following sub-CE categories:

- CE3-1: Cross-component prediction (18 tests)

- CE3-2: Small block size restrictions (5 tests)

- CE3-3: MPM list harmonization (12 tests)

- CE3-4: Affine linear weighted intra prediction (2 tests)

# Test conditions and guidelines

CTC conditions as agreed at the 13th JVET meeting (Marrakech, MA) for SDR will be used for the evaluation of objective performance (JVET-M1010). The anchor is the ‘VVC Test Model’ or VTM. Both the ‘All Intra’ and ‘Random Access’ test configurations shall be used and providing the results for each of these configurations is mandatory. Additionally, for CE3-2 subtests, ‘Low Delay B’ test configurations shall also be used. The VTM version 4.0 software shall be used with ‘vtm’ configuration files to produce the VTM anchor. Tools shall be implemented into the VTM version 4.0 software.

Proposals will primarily be compared with respect to objective compression efficiency while complexity will also be assessed.

BD-rates, encoding and decoding time comparisons, shall be reported by all proponents for at least the following case: #2 vs. #1. Additional test results may be provided, for example: #2 vs. #3.

Numbering is defined as follows for the case where ‘ToolA’ is replacing an existing ‘ToolB’ in VTM:

1.           VTM,						VTM anchor

2.           VTM + ToolA,			VTM with ToolA enabled (as a “tool on” test)

3.           VTM − ToolB            	VTM with ToolB disabled

Note: If ‘ToolB’ does not exist, then #3 = #2.

Source codes corresponding with the test descriptions contained in this document shall be uploaded to the following CE3 GitLab project according to the agreed timeline:

https://vcgit.hhi.fraunhofer.de/JVET-M-CE3/VVCSoftware_VTM.git

# Timeline

| T1 | 8 February 2019 | CE test description finalized |
| --- | --- | --- |
| T2 | 25 February 2019 | Initial software of CE tests released; initial study by cross-checkers begins |
| T3 | 4 March 2019 | Software of CE tests is finalized, and full test results are provided; final cross-check begins |
| T4 | 12 March 2019 | Document upload: test results, cross-check reports, CE3 summary report |

# CE3-1: Cross-component prediction

## Contribution documents

### Reducing the number of reference samples for CCLM model derivation

JVET-M0108 CE3-related: Reducing the number of reference samples and table size in LM Chroma process [E. François, T. Poirier, F. Le Léannec (Technicolor)]

JVET-M0211 CE3-related: Fixed Reference Samples Design for CCLM [J.-Y. Huo, X.-W. Li, J.-L. Wang, Y.-Z. Ma, F.-Z. Yang (Xidian Univ.), S. Wan (NPU), Y.-F. Yu, Y. Liu (OPPO)]

JVET-M0212 CE3-related: Improved reference samples range for MDLM [S. Wan (NPU), Q.-H.Ran, X.-W. Li, Y.-Z. Ma, J.-Y. Huo, F.-Z. Yang (Xidian Univ.), Y.-F. Yu, Y. Liu (OPPO)]

JVET-M0219 CE3-related: Reduced number of reference samples for CCLM parameter calculation [J. Choi, J. Heo, S. Yoo, L. Li, J. Choi, J. Lim, S. Kim (LGE)]

JVET-M0229 CE3-related: Simplification of LM Mode [Y.-W. Chen, X. Wang (Kwai Inc.)]

JVET-M0274 CE3-related: Modified linear model derivation for CCLM modes [M. Wang, K. Zhang, L. Zhang, H. Liu, J. Xu, S. Wang (Bytedance), J. Li, S. Wang, W. Gao (Peking University)]

### Multi-model LM

JVET-M0384 Non-CE3: LM in the middle [C. Gisquet, G. Laroche, P. Onno, J. Taquet (Canon)]

JVET-M0098 CE3: Joint test on MMLM (Test 2.2.1) [A. K. Ramasubramonian, G. Van der Auwera, T. Hsieh, V. Seregin, M. Karczewicz (Qualcomm), H.-J. Jhu, Y.-J. Chang (Foxconn)]

## Defined tests

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-1.1.1 | Selection of the up to 6 samples is applied to the down-sampled luma samples | Edouard Francois (Technicolor) | Yi-Wen Chen (Kwai) |
| 3-1.2.1 | Reduction of reference samples to N points for CCLM (CCLM_LT, CCLM_T, and CCLM_L), using an offset, with N being set according to the size of the block | Junyan Huo (Xidian) | Xiang Ma (Huawei) |
| 3-1.2.2 | Reduction of reference samples to N points for CCLM (CCLM_LT, CCLM_T, and CCLM_L), using an offset, with N being set fixed (i.e., uniform for all block sizes) | Junyan Huo (Xidian) | Xiang Ma (Huawei) |
| 3-1.3.1 | Skipping initial P reference samples for CCLM_L and CCLM_T, with P being set according to the size of the block | Shuai Wan (NPU) | Xiang Ma (Huawei) |
| 3-1.3.2 | Test 3-1.3.1 + Test 3-1.2.1 | Shuai Wan (NPU) | Xiang Ma (Huawei) |
| 3-1.3.3 | Test 3-1.3.1 + Test 3-1.2.2 | Shuai Wan (NPU) | Xiang Ma (Huawei) |
| 3-1.4.1 | CCLM parameter calculation with half number of reference samples | Jangwon Choi (LGE) | Yi-Wen Chen (Kwai) |
| 3-1.4.2 | CCLM parameter calculation with half and upper-limited reference samples | Jangwon Choi (LGE) | Yi-Wen Chen (Kwai) |
| 3-1.5 | CCLM model derived from four neighbouring luma samples | K. Zhang (Bytedance) | Yi-Wen Chen (Kwai) |
| 3-1.6.1 | Identify the min/max luma samples without down-sampling process; apply the 6-taps filter on the identified min/max samples to generate the filtered (down-sampled) min/max luma samples for the derivation of α and β. | Yi-Wen Chen (Kwai) | Jangwon Choi (LGE) |
| 3-1.6.2 | Combination of test 3-1.6.1 with JVET-M0108 | Yi-Wen Chen (Kwai) | Kai Zhang (Bytedance) |
| 3-1.6.3 | Combination of test 3-1.6.1 with JVET-M0211 | Yi-Wen Chen (Kwai) | Luong Pham Van (Qualcomm) |
| 3-1.6.4 | Combination of test 3-1.6.1 with JVET-M0211+JVET-M0212 | Yi-Wen Chen (Kwai) | Edouard Francois (Technicolor) |
| 3-1.6.5 | Combination of test 3-1.6.1 with JVET-M0219 | Yi-Wen Chen (Kwai) | Luong Pham Van (Qualcomm) |
| 3-1.6.6 | Combination of test 3-1.6.1 with JVET-M0274 | Yi-Wen Chen (Kwai) | Jangwon Choi (LGE) |
| 3-1.7 | Multiple-model LM with small block size restriction | Po-Han Lin (Foxconn) | Christophe Gisquet (Canon) |
| 3-1.8.1 | Multiple-model LM using piecewise linear model and block size restriction | Christophe Gisquet (Canon) | Adarsh K. Ramasubramonian (Qualcomm) |
| 3-1.8.2 | More stringent block size restriction on CCLMs with JVET-M0384 included. | Christophe Gisquet (Canon) | Adarsh K. Ramasubramonian (Qualcomm) |

## Test descriptions

### Reducing the number of reference samples in LM Chroma process (JVET-M0108 Technicolor)

It is proposed to identify the minimum and maximum neighbouring luma values from a reduced set of up to 6 samples, selected at following positions:

- Upper line: 3 locations (i – 1, 0), (i – 1, nS – 1), (i – 1, nS/2)

- Left column: 3 locations (0, j – 1), (nS – 1, j – 1), (nS/2, j – 1)

where nS corresponds to the number of samples in the upper line / left column of the chroma and downsampled luma blocks used for deriving the linear model parameters, and (i, j) is a sample position related to the block’s origin in the picture.

In case of CCLM, up to 6 samples are used. In case of CCLM_L and CCLM_T only 3 samples, from the upper line / left column, are used.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-1.1.1 | Selection of the up to 6 samples is applied to the down-sampled luma samples | Edouard Francois (Technicolor) | Yi-Wen Chen (Kwai) |

### Offset-based reference sample number reduction for CCLM (JVET-M0211 Xidian/NPU/OPPO)

Proposed is an offset-based method of fetching partial reference samples to reduce the number of reference samples used in the MaxMin method for CCLM (CCLM_LT, CCLM_T and CCLM_L). It can be either applied after the down-sampling procedure, which is generally used to get the reference Luma samples, or before the down-sampling procedure.

Let L be the number of reference samples of a block. Get totally N samples used in the MaxMin method starting from the (offset)th sample of available neighbouring samples of the block in every delta samples, wherein delta is set equal to L/N, and the offset is set equal to delta/2.

Figure 1 shows an example to get 4 reference samples from a 16-pixel line.

Figure 1: Reference samples in proposed method (delta=4, offset=2)

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-1.2.1 | Reduction of reference samples to N points for CCLM (CCLM_LT, CCLM_T, and CCLM_L), using an offset, with N being set according to the size of the block | Junyan Huo (Xidian) | Xiang Ma (Huawei) |
| 3-1.2.2 | Reduction of reference samples to N points for CCLM (CCLM_LT, CCLM_T, and CCLM_L), using an offset, with N being set fixed (i.e., uniform for all block sizes) | Junyan Huo (Xidian) | Xiang Ma (Huawei) |

### Skipping initial P reference samples for CCLM_L and CCLM_T (JVET-M0212 NPU/Xidian/OPPO)

When a block is coded in the CCLM_L or CCLM_T mode, the first P samples of the available neighbouring samples of the mode are not used in the MaxMin method. Figure 2 shows an example to skip the first quarter of the reference samples. It can be either applied after the down-sampling procedure, which is generally used to get the reference Luma samples, or before the down-sampling procedure.

Figure 2: Skipping initial samples in proposed method.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-1.3.1 | Skipping initial P reference samples for CCLM_L and CCLM_T, with P being set according to the size of the block. | Shuai Wan (NPU) | Xiang Ma (Huawei) |
| 3-1.3.2 | Test 3-1.3.1 + test 3-1.2.1 | Shuai Wan (NPU) | Xiang Ma (Huawei) |
| 3-1.3.3 | Test 3-1.3.1 + test 3-1.2.2 | Shuai Wan (NPU) | Xiang Ma (Huawei) |

### Reduced number of reference samples for CCLM parameter calculation (JVET-M0219 LGE)

In this test, the number of reference sample pairs, which are used in the CCLM (CCLM_LT, CCLM_T and CCLM_L) parameter determination is reduced to half based on pre-defined upper limit number so that the worst pixel comparison complexity is reduced by half (i.e., from 4 samples to 2 samples).

Moreover, the reduced reference samples are considered in the luma downsampling stage of CCLM prediction, so that the operation complexity of luma downsampling and the buffer size to store the downsampled luma are reduced significantly.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-1.4.1 | CCLM parameter calculation with half number of reference samples | Jangwon Choi (LGE) | Yi-Wen Chen (Kwai) |
| 3-1.4.2 | CCLM parameter calculation with half and upper-limited reference samples | Jangwon Choi (LGE) | Yi-Wen Chen (Kwai) |

### CCLM model derived from four neighbouring luma samples (JVET-M0274 Bytedance, Peking University)

It is proposed that only 4 neighbouring luma samples are used to derive the CCLM model. The smaller two values of the four are denoted as x0A and x1A, and larger two values of the four are denoted as x0B and x1B. Their corresponding chroma sample values are denoted as y0A, y1A, y0B and y1B. Then xA, xB, yA and yB are derived as:

xA=(x0A + x1A +1)>>1; xB=(x0B + x1B +1)>>1; yA=(y0A + y1A +1)>>1; yB=(y0B + y1B +1)>>1

Then α and β are obtained according to the same equation as in VTM-3.0.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-1.5 | CCLM model derived from four neighbouring luma samples | K. Zhang (Bytedance) | Yi-Wen Chen (Kwai) |

### Simplification of LM Mode (JVET-M0229 Kwai)

In current CCLM, a six-tap filter is applied to derive down-sampled luma samples. To reduce the computational complexity, it is proposed to use the corresponding reconstructed luma samples directly (without down-sample filtering) to find the min/max luma samples as shown in the right part of Figure 3, which could also be described as taking the luma samples using the Equation 1. After the min/max luma samples are identified, their corresponding down-sampled luma samples are then generated using the conventional six-tap filter to derive the α and β.

Rec'Lx,y=RecL2x,2y                         	(1)

Figure 3: Reconstructed luma samples (RecL) and the reconstructed chroma samples (RecC) that are used directly to derive α and β in the proposed scheme.

In the CE test, the tests of combining this method with other downsampling methods (e.g., JVET-M0108, JVET-M0211, JVET-M0212, JVET-M0219 and JVET-M0274, if they are able to be combined) that reduce the number of comparisons will be conducted.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-1.6.1 | Identify the min/max luma samples without down-sampling process; apply the 6-taps filter on the identified min/max samples to generate the filtered (down-sampled) min/max luma samples for the derivation of α and β. | Yi-Wen Chen (Kwai) | Jangwon Choi (LGE) |
| 3-1.6.2 | Combination of test 3-1.6.1 with JVET-M0108 | Yi-Wen Chen (Kwai) | Kai Zhang (Bytedance) |
| 3-1.6.3 | Combination of test 3-1.6.1 with JVET-M0211 | Yi-Wen Chen (Kwai) | Luong Pham Van (Qualcomm) |
| 3-1.6.4 | Combination of test 3-1.6.1 with JVET-M0211+JVET-M0212 | Yi-Wen Chen (Kwai) | Edouard Francois (Technicolor) |
| 3-1.6.5 | Combination of test 3-1.6.1 with JVET-M0219 | Yi-Wen Chen (Kwai) | Luong Pham Van (Qualcomm) |
| 3-1.6.6 | Combination of test 3-1.6.1 with JVET-M0274 | Yi-Wen Chen (Kwai) | Jangwon Choi (LGE) |

### Multiple-model LM (JVET-M0098 Foxconn, Qualcomm)

Cross-component linear model (CCLM) enables prediction of chroma samples from the luma reconstructed samples using a linear model; MMLM uses two models for prediction. Reconstructed luma and chroma reference samples above and left of the current coding unit are used to derive the model parameters. The min-max method for model parameter derivation and the downsampling filters used are aligned with CCLM. Three modes corresponding to CCLM-TL, CCLM-T, and CCLM-L are defined: MMLM-TL, MMLM-T and MMLM-L. The neighbouring luma reference samples are classified into two groups based on the value Threshold (as shown in Figure 4). Threshold is calculated as the average value of the neighbouring reconstructed luma samples. A neighbouring sample with Rec′L[x,y] <= Threshold is classified into group 1; while a neighbouring sample with Rec′L[x,y] > Threshold is classified into group 2. The samples of the luma corresponding to the current block are also classified based on the same rule for the classification of neighbouring luma samples. MMLM would not be applied to smaller blocks.

Figure 4: An example of classifying the neighbouring samples into two groups

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-1.7 | Multiple-model LM with small block size restriction | Po-Han Lin (Foxconn) | Christophe Gisquet (Canon) |

### Multiple-model LM using piecewise linear model (JVET-M0384 Canon)

This proposal presents a new method to perform the multiple-model linear mode using straight lines. This goes through the definition of a knee-point between the two lines, derived from the luma and chroma sample averages and extremum points. The method only operates on larger blocks to not affect the linear model worst case.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-1.8.1 | Multiple-model LM using piecewise linear model and block size restriction | Christophe Gisquet (Canon) | Adarsh K. Ramasubramonian (Qualcomm) |
| 3-1.8.2 | More stringent block size restriction on CCLMs with JVET-M0384 included. | Christophe Gisquet (Canon) | Adarsh K. Ramasubramonian (Qualcomm) |

# CE3-2: Small block size restrictions

## Contribution documents

JVET-M0065 Non-CE3: Intra chroma partitioning and prediction restriction [T. Zhou, T. Ikai (Sharp)]

JVET-M0169 CE3-related: Shared reference samples for multiple chroma intra CBs [Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-M0245 AHG16-related: Chroma block coding and size restriction [C. Rosewarne, A. Dorrell (Canon)]

## Defined tests

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-2.1.1 | Disable 2x2/4x2/2x4 in dual tree | T. Zhou (Sharp) | Zhi-Yi Lin (MediaTek) |
| 3-2.1.2 | Disable 2x2/4x2/2x4 in dual tree and use DC only for 2x2/4x2/2x4 intra chroma prediction | T. Zhou (Sharp) | Zhi-Yi Lin (MediaTek) |
| 3-2.1.3 | 3-2.1.1 + 3-2.2 | T. Zhou (Sharp) | Chris Rosewarne (Canon) |
| 3-2.2 | Share reference samples for at least 16 chroma samples in both dual and shared tree | Zhi-Yi Lin (MediaTek) | Chris Rosewarne (Canon) |
| 3-2.3 | Disable intra chroma 2x2, 2x4 and 4x2 in dual tree. And use a 2x8 or 8x2 coefficient group size instead of a 2x2 coefficient group size on remaining intra chroma blocks which has width or height size as 2. | Chris Rosewarne (Canon) | T. Zhou (Sharp) |

## Test descriptions

### Intra chroma partitioning and prediction restriction (JVET-M0065 Sharp)

This contribution proposes to restrict intra chroma block partitioning in dual tree and intra chroma prediction modes in small blocks such as 2x2, 2x4, 4x2. Specifically, in this proposal, QT and MTT partitioning for small chroma blocks are disallowed in dual tree as a partition restriction. Also intra chroma prediction mode in small blocks is restricted where DC mode is always used without mode (intra_chroma_pred_mode) signalling.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-2.1.1 | Disable chroma 2x2/4x2/2x4 in dual tree | T. Zhou (Sharp) | Zhi-Yi Lin (MediaTek) |
| 3-2.1.2 | Disable chroma 2x2/4x2/2x4 in dual tree and use DC only for 2x2/4x2/2x4 intra chroma prediction | T. Zhou (Sharp) | Zhi-Yi Lin (MediaTek) |
| 3-2.1.3 | 3-2.1.1 + 3-2.2 | T. Zhou (Sharp) | Chris Rosewarne (Canon) |

### Shared reference samples for multiple chroma intra CBs (JVET-M0169 MediaTek)

In typical hardware video encoders and decoders, processing throughput drops when a picture has more small blocks. Such processing throughput drop mainly comes from small intra blocks because small inter blocks can be processed in parallel while intra blocks has data dependency between neighbouring blocks (e.g., the predictor generation of an intra block requires top and left boundary reconstructed samples from neighbouring blocks) and must be processed sequentially.

In HEVC, the worst-case processing throughput occurs when 4x4 chroma intra blocks are processed. In VTM3.0, the size of the smallest chroma intra block is 2x2, and the reconstruction process of a chroma intra block becomes very complex after adopting tools including cross-component linear model (CCLM), 4-tap interpolation filters, position-dependent intra prediction combination (PDPC), and combined inter intra prediction (CIIP). The worst-case processing throughput in VVC is only 1/4 of that in HEVC.

It is proposed to enable parallel generation of multiple chroma intra coding blocks (CBs) within a parallel region of size larger than or equal to 16 chroma samples. Parallel regions are first determined, and all the chroma blocks within the same parallel region use the reference samples that belong to neighbouring blocks of the parallel region and are at the top and left boundaries of the parallel region for chroma intra prediction. By applying the proposed method, the worst-case processing throughput occurs when 4x4, 2x8, or 8x2 chroma blocks, instead of a 2x2 chroma blocks, are processed. The worst-case processing throughput is the same as that in HEVC.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-2.2 | Share reference samples for at least 16 chroma samples in both dual and shared tree | Zhi-Yi Lin (MediaTek) | Chris Rosewarne (Canon) |

### Chroma block coding and size restriction (JVET-M0245 Canon)

The proposal has two aspects:

- In chroma, restrictions in the chroma coding tree of intra slices prohibits splits that would result in any 2x2, 2x4, or 4x2 chroma blocks.

- In chroma, blocks of width or height of 2 use a 2x8 or 8x2 coefficient group size, respectively. As a consequence, coefficient groups always contain 16 coefficients (excepting the coefficient group containing the last significant coefficient).

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-2.3 | Disable intra chroma 2x2, 2x4 and 4x2 in separate tree. And use a 2x8 or 8x2 coefficient group size instead of a 2x2 coefficient group size on remaining intra chroma blocks which has width or height size as 2. | Chris Rosewarne (Canon) | T. Zhou (Sharp) |

# CE3-3: MPM list harmonization

## Contribution documents

JVET-M0210 Non-CE3: Intra prediction information coding [J. Yao, J. Zhu, W. Cai, K. Kazui (Fujitsu)]

JVET-M0239 Non-CE3: Modification of MPM derivation [J. Lee, H. Lee, S.-C. Lim, J. Kang, H. Y. Kim (ETRI)]

JVET-M0295 CE3-related: Harmonization of MPM list construction [B. Wang, S. Esenlik, A. M. Kotra, H. Gao, J. Chen (Huawei)]

- Remark: method 1 + JVET-M0783 (= JVET-M0815)

JVET-M0494 CE3-related: Modifications on MPM list generation [L. Zhao, X. Zhao, X. Li, S. Liu (Tencent)]

- Remark: methods 2, 4, 5

JVET-M0528 Non-CE3: A unified luma intra mode list construction process [F. Bossen, K. Misra (Sharp Labs of America)]

JVET-M0783 CE3-related: Modification of MPM list order [J. Heo, J. Choi, J. Choi, S. Yoo, L. Li, J. Lim, S. Kim (LGE)]

JVET-M0815 CE3-related: Harmonization on MPM list [L. Li, J. Heo, J. Choi, S. Yoo, J. Choi, J. Lim, S. Kim (LGE)]

## Defined tests

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-3.1.1 | Unified MPM list generation with changed order to include tools such as MRL, intra sub-partition coding, etc. | B. Wang (Huawei) / L. Li (LGE) | Zhang Zhi (Ericsson) |
| 3-3.1.2 | Harmonization of luma intra mode coding (including MRL, intra sub-partitioning, etc.) | L. Li (LGE) / B. Wang (Huawei) | Zhang Zhi (Ericsson) |
| 3-3.2.1 | Simplification of MPM derivation | Jinho Lee (ETRI) | L. Zhao (Tencent) |
| 3-3.2.2 | Simplification of MPM derivation with changed order described in JVET-M0295 | Jinho Lee (ETRI) | L. Zhao (Tencent) |
| 3-3.2.3 | Withdrawn | Jinho Lee (ETRI) | L. Zhao (Tencent) |
| 3-3.3.1 | Unification of MPM list generation with prioritization of Planar mode in MPM list | L. Zhao (Tencent) | S. de Luxán Hernández (HHI) |
| 3-3.3.2 | Unification of MPM list generation with Planar and DC modes being first two MPMs of zero reference line, each of the first two bins of MPM index is coded using one context | L. Zhao (Tencent) | S. de Luxán Hernández (HHI) |
| 3-3.3.3 | Unification of MPM list generation with Planar and DC modes being first two MPMs of zero reference line, the first bin of MPM index is coded using more than one context | L. Zhao (Tencent) | S. de Luxán Hernández (HHI) |
| 3-3.4.1 | Planar/DC coded as separate modes | F. Bossen (Sharp Labs) | A. Filippov (Huawei) |
| 3-3.4.2 | Same as previous test, but with modified encoder search described in JVET-M0528 | F. Bossen (Sharp Labs) | A. Filippov (Huawei) |
| 3-3.4.3 | Planar/DC coded as separate modes, use 4MPMs described in JVET-M0494 | F. Bossen (Sharp Labs) | A. Filippov (Huawei) |
| 3-3.5.1 | Explicitly signal Planar&DC MPM list and intra_luma_ref_idx signalling only for angular mode | Jie Yao (Fujitsu) | S. Iwamura (NHK) |
| 3-3.5.2 | Test 3.3.5.1, use 4MPMs described in JVET-M0494 | Jie Yao (Fujitsu) | S. Iwamura (NHK) |

## Specific test conditions for CE3-3

In addition to VTM-4.0, a codebase is also used for evaluating the tests in CE3-3. In the codebase, a macro (e.g., ALL_MPM_RD) is added on top of VTM-4.0 to always include all MPMs (6 MPMs in VTM-4.0) for full encoder R-D checking in intra mode decision, SW changes (in IntraSearch.cpp) are described as below:

#if ALL_MPM_RD

for (int j = 0; j < NUM_MOST_PROBABLE_MODES; j++)

#else

for (int j = 0; j < numCand; j++)

#endif

For evaluating the coding performance of the following tests, two sets of results shall be reported for each CE3-3 test, i.e., Test A and Test B:

Test A:

- Anchor: VTM-4.0

- Tested: VTM-4.0 with test method

Test B:

- Anchor: Codebase

- Tested: Codebase with test method

Separate test results (A and B) shall be provided for modifications to the encoder search algorithm other than the one described above.

## Test descriptions

### Harmonization of MPM lists (JVET-M0295 Huawei / JVET-M0815 LGE)

When using non-zero reference line, the MPM list uses only 4 modes. These 4 modes are derived by excluding Planar and DC modes from the zero-reference line MPM list, and all other remaining angular MPM of zero-reference line list are re-used as the MPM list of non-zero reference lines.

If one neighbouring mode is Planar or DC mode, and the other neighbouring mode is angular mode, it is proposed that, Planar mode is firstly inserted into the MPM list, and MPM list is derived as follows:

candModeList[ 0 ] = PLANAR_IDX

candModeList[ 1 ] = maxAB

candModeList[ 2 ] = DC_IDX

candModeList[ 3 ] = 2 + ( ( maxAB + 61 ) % 64 )

candModeList[ 4 ] = 2 + ( ( maxAB − 1 ) % 64 )

candModeList[ 5 ] = 2 + ( ( maxAB + 60 ) % 64 )

If two neighbouring modes are different and two neighbouring modes are greater than DC mode, the first four entries in the MPM list is changed to:

candModeList[ 0 ] = leftIntraDir

candModeList[ 1 ] = PLANAR_IDX

candModeList[ 2 ] = aboveIntraDir

candModeList[ 3 ] = DC_IDX

In addition, harmonization and simplification of luma intra mode coding is proposed. One general MPM list is generated as common ground, and subset of the general MPM list could be used with small modifications for other intra coding tools such as MRL, intra sub-partitioning (ISP) and so on.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-3.1.1 | Unified MPM list generation with changed order to include tools such as MRL, intra sub-partition coding, etc. | B. Wang (Huawei) L. Li (LGE) | Zhang Zhi (Ericsson) |
| 3-3.1.2 | Harmonization of luma intra mode coding (including MRL, intra sub-partitioning, etc.) | L. Li (LGE) B. Wang (Huawei) | Zhang Zhi (Ericsson) |

### Modification of MPM derivation (JVET-M0239 ETRI)

When luma reference line index is not equal to zero, candidate mode C is derived by comparing A and B. If the mode C is an angular mode, adjacent angular modes are included in MPM list, and if the mode C is a non-angular mode, default modes [50, 18, 34, 66, 2, 26] are included. On the other hand, if luma reference line index is equal to zero or intra sub-partitions coding mode is enabled, normal MPM list is used.

In addition, the simplified MPM derivation method is combined with the unified MPM list described in JVET-M0528.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-3.2.1 | Simplification of MPM derivation | Jinho Lee (ETRI) | L. Zhao (Tencent) |
| 3-3.2.2 | Simplification of MPM derivation with changed order described in JVET-M0295 | Jinho Lee (ETRI) | L. Zhao (Tencent) |
| 3-3.2.3 | Withdrawn | Jinho Lee (ETRI) | L. Zhao (Tencent) |

### Modifications of MPM list generation (JVET-M0494 Tencent)

#### Prioritization of Planar and DC modes in MPM list

If one neighbouring mode is Planar or DC mode, and the other neighbouring mode is angular mode, it is proposed that, Planar mode is firstly inserted into the MPM list, and MPM list is derived as follows:

candModeList[ 0 ] = PLANAR_IDX

candModeList[ 1 ] = maxAB

candModeList[ 2 ] = DC_IDX

candModeList[ 3 ] = 2 + ( ( maxAB + 61 ) % 64 )

candModeList[ 4 ] = 2 + ( ( maxAB − 1 ) % 64 )

candModeList[ 5 ] = 2 + ( ( maxAB + 60 ) % 64 )

#### Unification of MPM list generation

The size of MPM list of non-zero reference lines is reduced from 6 to 4, Planar and DC modes in zero reference line MPM list are excluded for the non-zero reference line MPM list, and all other remaining angular MPM of zero reference line list are used as the MPM list of non-zero reference lines. In addition, the MPM list of intra sub-partition (ISP) mode is also unified by using one MPM list generation applied for non-ISP modes.

Alternatively, the size of MPM list of non-zero reference lines is reduced from 6 to 4, and Planar and DC modes are always placed as the first two MPM candidates, and the 4 angular MPM candidates are derived in the same way used for zero reference line. The first two bins of the MPM index are context coded. For non-zero reference lines, only the 4 angular MPMs can be applied. In addition, the MPM list of intra sub-partition (ISP) mode is also unified by using one MPM list generation applied for non-ISP modes.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-3.3.1 | Unification of MPM list generation with prioritization of Planar mode in MPM list | L. Zhao (Tencent) | S. de Luxán Hernández (HHI) |
| 3-3.3.2 | Unification of MPM list generation with Planar and DC modes being first two MPMs of zero reference line, each of the first two bins of MPM index is coded using one context | L. Zhao (Tencent) | S. de Luxán Hernández (HHI) |
| 3-3.3.3 | Unification of MPM list generation with Planar and DC modes being first two MPMs of zero reference line, the first bin of MPM index is coded using more than one context | L. Zhao (Tencent) | S. de Luxán Hernández (HHI) |

### A unified luma intra mode list construction process (JVET-M0528 Sharp Labs of America)

A single list construction process is used, comprising solely of angular modes. This process is the same as the one currently being used for non-zero reference line index, but other processes are equally applicable (see JVET-M0494). Since the list no longer contains non-angular modes (PLANAR, DC modes) the luma intra mode signalling is modified to first signal a flag indicating if the selected mode is angular. For non-angular modes a further flag is received indicating if the mode is PLANAR or DC. The modified pseudo-syntax is shown below:

if (!intra_luma_ref_idx)
	intra_luma_angular_mode_flag
	if (!intra_luma_angular_mode_flag)
		intra_luma_planar_flag // Planar if true, DC otherwise
if (intra_luma_ref_idx || intra_luma_angular_mode_flag)
	if (intra_luma_ref_idx)
		intra_luma_mpm_flag=1
	else
		intra_luma_mpm_flag
	if (intra_luma_mpm_flag)
		intra_luma_mpm_idx
	else
		intra_luma_mpm_remainder

intra_luma_angular_mode_flag and intra_luma_planar_flag are context-coded.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-3.4.1 | Planar/DC coded as separate modes | F. Bossen (Sharp Labs) | A. Filippov (Huawei) |
| 3-3.4.2 | Same as previous test, but with modified encoder search described in M0528 | F. Bossen (Sharp Labs) | A. Filippov (Huawei) |
| 3-3.4.3 | Planar/DC coded as separate modes, use 4MPMs described in M0494 | F. Bossen (Sharp Labs) | A. Filippov (Huawei) |

### Intra prediction information coding (JVET-M0210, Fujitsu)

In VTM-3.0, for the intra prediction coding process, intra_luma_ref_idx is always signaled, even for Planar and DC which are excluded from multiple reference line intra prediction. Also, two different MPM list derivation schemes (with or without Planar&DC) are applied. This contribution proposes a new Intra prediction information coding method (Figure 5) in which a syntax element, intra_luma_non_ang_flag, is signaled at first for intra block to determine whether IntraMode is angular or non-angular. For non-angular mode, intra_luma_planar_flag is signaled to indicate whether IntraMode is planar or DC. For angular mode, syntax elements intra_luma_ref_idx, intra_luma_mpm_flag, intra_luma_mpm_idx and intra_luma_mpm_remainder are signaled as in VTM-3.0. The mpm list is only applied for angular mode thus simplifying MPM list derivation process.

parse intra_luma_non_ang_flag

if(intra_luma_non_ang_flag == 1)

parse intra_luma_planar_flag

IntraMode = (intra_luma_planar_flag == 1) ? Planar : DC

else

parse intra_luma_mpm_flag

if (intra_luma_mpm_flag)

parse intra_luma_mpm_idx, intra_luma_ref_idx

IntraMode = mpm1[intra_luma_mpm_idx] // MPM without Planar&DC

else

Parse intra_luma_mpm_remainder

IntraMode = intra_luma_mpm_remainder

Figure 5: Intra prediction information coding process for proposed method

In the proposed method, since the MPM list derivation process comprises only angular modes, we propose to simply add non-angular modes (Planar&DC) into RD check list which can achieve gains with reasonable increase on encoding time.

Two tests are reported:

- Test 1: Proposed intra prediction information coding method with no RD check list changes.

- Test 2: Proposed intra prediction information coding method while using 4MPMs described in JVET-M0494.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-3.5.1 | Explicitly signal Planar&DC MPM list and intra_luma_ref_idx signalling only for angular mode | Jie Yao (Fujitsu) | S. Iwamura (NHK) |
| 3-3.5.2 | Test 3-3.5.1, use 4MPMs described in JVET-M0494 | Jie Yao (Fujitsu) | S. Iwamura (NHK) |

# CE3-4: Affine linear weighted intra prediction

## Contribution document

JVET-M0043 CE3: Affine linear weighted intra prediction (test 1.2.1, test 1.2.2) [J. Pfaff, B. Stallenberger, M. Schäfer, P. Merkle, P. Helle, R. Rischke, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

## Defined tests

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-4.1 | Affine linear weighted intra prediction, i.e., to store all matrices and offsets needed, is restricted to be not more than 8 Kilobyte | Jonathan Pfaff (HHI) | Vasily Rufitskiy (Huawei) |
| 3-4.2 | Architectural variations of the affine linear weighted intra prediction will be studied which do not exceed the total memory requirement of 8 Kilobyte | Jonathan Pfaff (HHI) | Vasily Rufitskiy (Huawei) |

## Test descriptions

### Affine linear weighted intra prediction with reduced table sizes (JVET-M0043 HHI)

In this CE test, the affine linear weighted intra predictors from document JVET-M0043 are studied but the total number of coefficients needed for the method is reduced. The affine linear weighted intra predictors from JVET-M0043 generate an intra prediction signal in the following three steps:

- Out of the reconstructed boundary samples, input-samples are extracted by averaging.

- A matrix vector multiplication, followed by addition of an offset, is carried out with the averaged samples as an input. Depending on the block-shape, the result might be a reduced prediction signal on a subsampled set of samples in the original block.

- Depending on the block-shape, the prediction signal at the remaining positions is generated from the prediction signal on the subsampled set by linear interpolation.

In the first CE test, the memory requirement to store all parameters needed for the affine linear weighted intra prediction, i.e., to store all matrices and offsets needed, is restricted to be not more than 8 Kilobyte. In the second CE test, further architectural variations of the affine linear weighted intra prediction will be studied which also do not exceed the total memory requirement of 8 Kilobyte.

# Participants

The following table enumerates participants with tools in CE3 as well as participating cross-checkers and interested persons.

| Name | Company | Email | CE3-x Proponent | Cross-checker |
| --- | --- | --- | --- | --- |
| Geert Van der Auwera Vadim Seregin Nan Hu Ted Hsieh Adarsh Krishnan Ramasubramonian Luong Pham Van Chun-Chi Chen Yan Zhang Chao-Hsiung Hung Hilmi Enes Egilmez | Qualcomm | geertv@qti.qualcomm.com vseregin@qti.qualcomm.com nanh@qti.qualcomm.com thsieh@qti.qualcomm.com aramasub@qti.qualcomm.com lphamvan@qti.qualcomm.com chunchic@qti.qualcomm.com yzh@qti.qualcomm.com chaohsiu@qti.qualcomm.com hegilmez@qti.qualcomm.com | 1 | x |
| Xin Zhao Liang Zhao Jing Ye Xiang Li Shan Liu | Tencent | xinzzhao@tencent.com leolzhao@tencent.com jingye@tencent.com xlxiangli@tencent.com shanl@tencent.com | 3 |  |
| Fabien Racapé Gagan Rath Fabrice Urban Franck Galpin Francois Edouard Christophe Chevance | Technicolor | Fabien.Racape@technicolor.com Gagan.rath@technicolor.com Fabrice.urban@technicolor.com Franck.galpin@technicolor.com Edouard.Francois@technicolor.com christophe.chevance@technicolor.com | 1 | x |
| Jani Lainema | Nokia | jani.lainema@nokia.com |  |  |
| Virginie Drugeon | Panasonic | Virginie.Drugeon@eu.panasonic.com |  |  |
| Jaehyun Lim SeungHawn Kim Jin Heo Jangwon Choi Sunmi Yoo Ling Li Hyeongmun Jang Naeri Park Jungah Choi | LGE | jaehyun.lim@lge.com seunghwan3.kim@lge.com jin78.heo@lge.com jangwon84.choi@lge.com sunmi.yoo@lge.com l.li@lge.com hm.jang@lge.com naeri.park@lge.com jungah.choi@lge.com | 1, 3 | x |
| Jungwon Kang Jinho Lee Sung-Chang Hahyun Lee | ETRI | jungwon@etri.re.kr jinosoul@etri.re.kr sclim@etri.re.kr hanilee@etri.re.kr | 3 |  |
| Jonathan Pfaff Heiko Schwarz Benjamin Bross Philipp Merkle Santiago de Luxán Hernández Philipp Helle Paul Keydel Bjoern Stallenberger | HHI | jonathan.pfaff@hhi.fraunhofer.de heiko.schwarz@hhi.fraunhofer.de benjamin.bross@hhi.fraunhofer.de philipp.merkle@hhi.fraunhofer.de santiago.de.luxan@hhi.fraunhofer.de philipp.helle@hhi.fraunhofer.de paul.keydel@hhi.fraunhofer.de bjoern.stallenberger@hhi.fraunhofer.de | 4 | x |
| Po-Han Lin Yu-Chiao Yang Tsung-Hua Li Hong-Jheng Jhu | Foxconn | PohanLin@fginnov.com yuciaoyang@fginnov.com thli@fginnov.com HongJheng@fginnov.com | 1 |  |
| Alexey Filippov Sergey Ikonin Jianle Chen Anand Meher Kotra Xiang Ma Vasily Rufitskiy Sychev Maxim Cheung Auyeung Biao Wang Seungwook Hong Haitao Yang Semih Esenlik | Huawei | Alexey.Filippov@huawei.com Sergey.Ikonin@huawei.com jianle.chen@huawei.com anand.meher.kotra@huawei.com maxiang6@huawei.com Vasily.Rufitskiy@huawei.com Sychev.Maxim@huawei.com cheung.auyeung@huawei.com biao.wang@huawei.com seuhong@outlook.com haitao.yang@huawei.com semih.esenlik@huawei.com | 3 | x |
| Kei Kawamura Yoshitaka Kidani Sei Naito Kyohei Unno | KDDI | ki-kawamura@kddi.com yo-kidani@kddi.com se-naitou@kddi.com ky-unno@kddi-research.jp |  |  |
| Peter Chuang, ChingYeh Chen, Yu-Wen Huang, Chih-Wei Hsu, Chia-Ming Tsai, Zhi-Yi Lin, Mohammed Sarwer Olena Chubach | MediaTek | peter.chuang@MediaTek.com chingyeh.chen@MediaTek.com yuwen.huang@MediaTek.com cw.hsu@MediaTek.com Chia-ming.Tsai@MediaTek.com Zhi-Yi.Lin@MediaTek.com Mohammed.Sarwer@MediaTek.com Olena.Chubach@mediatek.com | 2 | x |
| Yinji Piao Narae Choi Kiho Choi | Samsung | yj1003.piao@samsung.com narae1.choi@samsung.com kiho14.choi@samsung.com m.w.park@samsung.com |  |  |
| Ching-Chieh Lin PoHan Lin | ITRI | jacklin@itri.com pohan@itri.com |  |  |
| Steve Keating Karl Sharman Masaru Ikeda | Sony | Steve.Keating@sony.com karl.sharman@sony.com masaru.ikeda@sony.com |  |  |
| Shimpei Nemoto Shunsuke Iwamura | NHK | nemoto.s-fy@nhk.or.jp iwamura.s-gc@nhk.or.jp |  | x |
| Andre Seixas Dias Saverio Blasi Gosala Kulupana | BBC | Andre.SeixasDias@bbc.co.uk saverio.blasi@bbc.co.uk gosala.kulupana@bbc.co.uk |  |  |
| Andrew Segall Kiran Misra Yukinobu Yasugi Takeshi Chujoh Tomohiro Ikai Eiishi Sasaki Tomonori Hashimoto Frank Bossen Tianyang Zhou | Sharp | asegall@sharplabs.com misrak@sharplabs.com yasugi.yukinobu@sharp.co.jp chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp eiichi.sasaki@sharp.co.jp tomonori.hashimoto@sharp.co.jp frank@bossentech.com zhou.tianyang@sharp.co.jp | 2, 3 | x |
| Zhi Zhang Rickard Sjoberg | Ericsson | zhi.a.zhang@ericsson.com rickard.sjoberg@ericsson.com |  |  |
| Yongbing Lin Quanhe Yu Jianhua Zheng | HiSilicon | linyongbing@hisilicon.com yuquanhe@hisilicon.com zhengjianhua@hisilicon.com |  |  |
| Dongcheol Kim Jason Jung | WILUS | dongcheol.kim@wilusgroup.com jason.jung@wilusgroup.com |  |  |
| Xingchun Qi Shurui Ye Liying Xu | Hikvision | qixingchun@hikvision.com yeshurui@hikvision.com xuliying5@hikvision.com |  |  |
| D. Y. Kim | Chips& Media | brad.kim@chipsnmedia.com |  |  |
| Elie Mora Anthony Nasrallah | ATEME | e.mora@ateme.com a.nasrallah@ateme.com |  |  |
| Lin Tao | Tongji Univ. | lintao@tongji.edu.cn |  |  |
| Xianglin Wang Yi-Wen Chen Xiaoyu Xiu Tsung-Chuan Ma | Kwai | xianglinwang@kwai.com yiwenchen@kwai.com xiaoyuxiu@kwai.com tsung-chuanma@kwai.com | 1 | x |
| Sriram Sethuraman | Ittiam | sriram.sethuraman@ittiam.com |  |  |
| Chris Rosewarne Patrice Onno Guillaume Laroche Jonathan Taquet Christophe Gisquet | Canon | Chris.Rosewarne@cisra.canon.com.au Patrice.Onno@crf.canon.fr guillaume.laroche@crf.canon.fr jonathan.taquet@crf.anon.fr Christophe.Gisquet@crf.canon.fr | 1, 2 | x |
| Byeungwoo Jeon Jeeyoon Park | SKKU | bjeon@skku.edu jiyoonpark@skku.edu |  |  |
| Liu Hongbin Kai Zhang Li Zhang Weijia Zhu | Bytedance | liuhongbin.01@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com weijia.zhu@bytedance.com | 1 | x |
| Hari Kalva | Florida Atlantic Univ. | hari.kalva@fau.edu |  |  |
| Minhua Zhou Peisong Chen Wade Wan | Broadcom | minhua.zhou@broadcom.com peisong.chen@broadcom.com wade.wan@broadcom.com |  |  |
| Rahul Vanam Yuwen He Saurav Bandyopadhyay Philippe Hanhart | Interdigital | rahul.vanam@interdigital.com yuwen.he@interdigital.com Saurav.Bandyopadhyay@InterDigital.com Philippe.Hanhart@InterDigital.com |  |  |
| Pierrick Philippe | Orange | pierrick.philippe@orange.com |  |  |
| Weimin Zeng Eric Chai | Ubilinx | weimin.zeng@ublnx.com eric.chai@ublnx.com |  |  |
| Dan Grois | Comcast | groisz@yahoo.com |  |  |
| Jongseok Lee | Kwangwoon University | suk2080@kw.ac.kr |  |  |
| Fannie Yu | OPPO | yuyuanfang@oppo.com |  |  |
| Peng Yin | Dolby | pyin@dolby.com |  |  |
| Haechul Choi Daehyeok Gwon Heeji Han Jaeryun Choe | Hanbat University | choihc@hanbat.ac.kr skyeye0530@naver.com sarhakor@naver.com tjswo9395@naver.com |  |  |
| Grace Yu | Intel | grace.yu@intel.com |  |  |
| Yiming Li Zizheng Liu | Wuhan University | userlym@whu.edu.cn lzz8246@whu.edu.cn |  |  |
| Jianqing Zhu Kimihiko Kazui Jie Yao | Fujitsu | zhujianqing@cn.fujitsu.com kazui.kimihiko@jp.fujitsu.com yaojie@cn.fujitsu.com | 3 |  |
| Junyan Huo Yanzhuo Ma | Xidian University | jyhuo@mail.xidian.edu.cn yzma@mail.xidian.edu.cn | 1 |  |
| Dong-jae Won | Sejong University | bigdj2002@sju.ac.kr |  |  |
| Shuai Wan | Northwestern Polytechnical University | swan@nwpu.edu.cn | 1 |  |
|  | Digital Insights | yjahn@digitalinsights.co.kr |  |  |
|  | Alibaba | yuchen.s@alibaba-inc.com |  |  |
| Hong-Jheng Jhu | National Chiao Tung University | ted0958.am96@g2.nctu.edu.tw |  |  |
| Xiaozhen Zheng | DJI | xiaozhen.zheng@dji.com |  |  |
| Yong-uk Yoon | Korea Aerospace University | yuyoon@kau.kr |  |  |