| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-M1025 |
| --- | --- |

| Title: | Description of Core Experiment 5 (CE5): Adaptive Loop Filter |  |  |
| --- | --- | --- | --- |
| Status: | Output document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Ching-Yeh Chen, Vadim Seregin | Tel Email: | chingyeh.chen@mediatek.com vseregin@qti.qualcomm.com |  |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document describes Core Experiment 5 (CE5) on Adaptive Loop Filter. The experiments are conducted after the 13th JVET meeting and the results, including cross-checks, are reported as input contributions to the 14th JVET meeting.

# Introduction

The following tests will be performed:

| Test# | Description | Document# |
| --- | --- | --- |
| CE5.1 | Virtual boundary processing with padding and weighted averaging | JVET-M0164 |
| CE5.2.1 | Virtual boundary processing with padding and truncated filtering without adaptive on/off control | JVET-M0301 |
| CE5.2.2 | Virtual boundary processing with padding and truncated filtering with adaptive on/off control | JVET-M0301 |
| CE5.3.1 | Non- linear ALF with best coding efficiency performance | JVET-M0385 |
| CE5.3.2 | Non-linear ALF with the reduced number of clipping samples | JVET-M0385 |
| CE5.3.3 | Non-linear ALF with a signalling of clipping values and filter coefficients | JVET-M0385 |
| CE5.4.1 | Luma CTBs with ALF on can switch among 1 slice-level signaled filter set, 16 fixed filter sets and those available sets in APS. All chroma CTBs with ALF on in a picture can only use the slice-level signaled filter or one temporal filter | JVET-M0429 |
| CE5.4.2 | Test CE5.4.1 with QP = 12, 17, 22 and 27 | JVET-M0429 |
| CE5.4.3 | Test CE5.4.1 without fixed filters | JVET-M0429 |
| CE5.4.4 | Test CE5.4.1 without temporal filters | JVET-M0429 |
| CE5.4.5 | Test CE5.4.4 with QP = 12, 17, 22 and 27 | JVET-M0429 |

Tests CE5.1 and CE5.2.1 include subjective evaluation. Informal subjective viewing is planned to be conducted during the Geneva meeting for both the proposals. The informal subjective test mainly checks for any visual artifcats which are created or observed near the CTU boundary when compared to the anchor VTM-4.0. ABAB test comparison is planned to be used where A or B could randomly be the anchor or the test.

The following sequences and QP points are planned to be used (tentative)

| Num | Name | Resolution | fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| 02 | Johnny | 1280x720p | 60fps | 600 | LDP | 37, 32 |
| 04 | KristenAndSara | 1280x720p | 60fps | 600 | LDP | 37, 32 |

# Detailed description of tests

#### CE5.1 Adaptive loop filter with virtual boundary processing (JVET-M0164)

In JVET-M0164, ALF with virtual boundary (VB) processing is proposed to remove the line buffer usage. When one sample located at one side of a VB is filtered, accessing samples located at the other side of the VB is forbidden. The originally required samples at the other side of the VB are replaced with padded samples. To accommodate deblocking filter (DF) and sample adaptive offset (SAO) in VTM3.0, the VBs are set as four luma lines and two chroma lines above CTU row boundaries.

| Test | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE5.1 | Virtual boundary processing with padding and weighted averaging | MediaTek |  |

#### CE5.2 Non-CE: Loop filter line buffer reduction (JVET-M0301)

In JVET-M0301, ALF with virtual boundary processing is also used. Modified ALF classification only uses the samples which are above the VB to classify the given 4 x 4 block which is above VB. Similarly, for the classification of the 4 x 4 block below VB, samples belonging to the lines below the VB are used. Modified ALF filtering uses a combination of conditional disabling and truncated versions of the original ALF filter.

Modified ALF block classification is depicted in the figure below:

Modified ALF filtering is shown below:

Conditional disabling of the ALF filtering is as follows:

In Figure 4, if absolute sum of the padded coefficients is greater than a threshold (T) times the absolute sum of the non-padded coefficients, then the filter is disabled for those respective lines.

For e.g. with lines M and H, if (2*abs  c0>(T* 2*abs c1+abs c2+absc3+absc4+absc5+absc6+absc7+absc8+absc9+absc10+absc11+absc12), then the ALF filtering is disabled for lines M and H.

Similarly for lines L and I, if (2*(abs c0+ abs c1 + abs c2+ abs c3)>(T* 2*absc4+absc5+absc6+absc7+absc8+absc9+absc10+absc11+absc12), then the ALF filtering is disabled for lines L and I.

Similarly for lines J and K, if (2*(abs c0+ abs c1 + abs c2+ abs c3+absc4+absc5+absc6+absc7+absc8)>(T* 2*absc9+absc10+absc11+absc12), then the ALF filtering is disabled for lines J and K.

Where T is generally a threshold which can take a fractional values ranging between 0 and 1. For example, T value can be 0.5, 0.6 or 0.7

Similarly for Chroma, as depicted in Fig. 5, for lines N and K, if (2*abs  c0>(T* 2*abs c1+abs c2+absc3+absc4+absc5)+absc6), then the ALF filtering is disabled for lines N and K.

for lines M and L, if ((2*(abs c0+ abs c1 + abs c2+ abs c3)>(T* 2*absc4+absc5)+absc6), then the ALF filtering is disabled for lines M and L.

the function abs(x) is defined as follows;Abs( x ) =  x;x  >=  0-x;x < 0

| Test | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE5.2.1 | Virtual boundary processing with padding and truncated filtering without adaptive on/off control | Huawei |  |
| CE5.2.2 | Virtual boundary processing with padding and truncated filtering with adaptive on/off control | Huawei |  |

#### CE5.3 Non-linear Adaptive Loop Filter (JVET-M0385)

.In JVET-M0385, an adaptive clipping operation is performed on the input samples values of the Adaptive Loop Filter in VTM3.0. The goal of this adaptive clipping is to introduce some non-linearities to cap the difference between the central input sample value and the other neighbor input sample values used during the ALF filtering process.

The number of clipping samples can be reduced [TBD]

The proposed tests on non-linear ALF do not have SIMD implementation for ALF filtering. For fair comparison, the SIMD implementation of the ALF filtering will be disabled in the anchors.

| Test | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE5.3.1 | Non- linear ALF with best coding efficiency performance | Canon |  |
| CE5.3.2 | Non-linear ALF with the reduced number of clipping samples | Canon |  |
| CE5.3.3 | Non-linear ALF with a signalling of clipping values and filter coefficients | Canon |  |

#### CE5.4 Coding tree block based adaptive loop filter (JVET-M0429)

In JVET-M0429, each luma CTB with ALF on can select one out of 22 filter sets (16 fixed, 5 temporal and 1 picture optimized). All chroma CTBs with ALF on in a picture can only use 1 picture optimized filter or 1 temporal chroma filter. The signaling mechanism will mimic adopted APS method for signaling ALF parameters in tile group header.

| Test | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE5.4.1 | Luma CTBs with ALF on can switch among 1 slice-level signaled filter set, 16 fixed filter sets and those available sets in APS. All chroma CTBs with ALF on in a picture can only use the slice-level signaled filter or one temporal filter | Qualcomm |  |
| CE5.4.2 | Test CE5.4.1 with QP = 12, 17, 22 and 27 | Qualcomm |  |
| CE5.4.3 | Test CE5.4.1 without fixed filters | Qualcomm |  |
| CE5.4.4 | Test CE5.4.1 without temporal filters | Qualcomm |  |
| CE5.4.5 | Test CE5.4.4 with QP = 12, 17, 22 and 27 | Qualcomm |  |

# Test conditions

CTC conditions as agreed at the 13th JVET meeting (Marrakesh, Morocco) for SDR will be used for the evaluation of objective performance (JVET-M1010). The software basis and anchor is VTM-4. Proposals will be compared with respect to objective compression efficiency. And subjective quality check is required for those tests related to line buffer reduction.

# Timeline

| T1 | End of 13th JVET meeting + 3 weeks: Feb. XX, 2019 | CE test description finalized |
| --- | --- | --- |
| T2 | VTM4.0 release + 2 weeks: Feb. XX, 2019 | Initial software of CE tests released; initial study by cross-checkers begins |
| T3 | 3 weeks before 12th JVET meeting: Mar. XX, 2019 | Software of CE tests is finalized, and full test results are provided; final cross-check begins |
| T4 |  | Document upload: test results, cross-check reports, CE summary report |

# Appendix of Participants

| # | Name | Company | E-mail | Proponents | Cross-checker |
| --- | --- | --- | --- | --- | --- |
| 1 | Jonathan TAQUET | Canon | Jonathan.TAQUET@crf.canon.fr | X |  |
| 2 | Patrice Onno | Canon | patrice.onno@crf.canon.fr | X |  |
| 3 | Alexey Filippov | Huawei | alexey.filippov@huawei.com | X |  |
| 4 | Vasily Rufitskiy | Huawei | vasily.rufitskiy@huawei.com | X |  |
| 5 | Sergey Ikonin | Huawei | sergey.ikonin@huawei.com | X |  |
| 6 | Victor Stepin | Huawei | stepin.victor@huawei.com | X |  |
| 7 | Roman Chernyak | Huawei | chernyak.roman@huawei.com | X |  |
| 8 | Semih Esenlik | Huawei | semih.esenlik@huawei.com | X |  |
| 9 | Yu-Wen Huang | MediaTek | yuwen.huang@mediatek.com | X |  |
| 10 | Ching-Yeh Chen | MediaTek | chingyeh.chen@mediatek.com | X |  |
| 11 | Tzu-Der Chuang | MediaTek | peter.chuang@mediatek.com | X |  |
| 12 | Chen-Yen Lai | MediaTek | jenny.lai@mediatek.com | X |  |
| 13 | Vadim Seregin | Qualcomm | vseregin@qti.qualcomm.com | X |  |
| 14 | Nan Hu | Qualcomm | nanh@qti.qualcomm.com | X |  |
| 15 | Wei-Jung Chien | Qualcomm | wchien@qti.qualcomm.com | X |  |
| 16 | Marta Karczewicz | Qualcomm | martak@qti.qualcomm.com | X |  |
| 17 |  |  |  |  |  |
| 18 |  |  |  |  |  |
| 19 |  |  |  |  |  |
| 20 |  |  |  |  |  |
| 21 |  |  |  |  |  |
| 22 |  |  |  |  |  |
| 23 |  |  |  |  |  |
| 24 |  |  |  |  |  |
| 25 |  |  |  |  |  |
| 26 |  |  |  |  |  |
| 27 |  |  |  |  |  |
| 28 |  |  |  |  |  |
| 29 |  |  |  |  |  |
| 30 |  |  |  |  |  |
| 31 |  |  |  |  |  |
| 32 |  |  |  |  |  |
| 33 |  |  |  |  |  |
| 34 |  |  |  |  |  |
| 35 |  |  |  |  |  |
| 36 |  |  |  |  |  |
| 37 |  |  |  |  |  |
| 38 |  |  |  |  |  |
| 39 |  |  |  |  |  |
| 40 |  |  |  |  |  |
| 41 |  |  |  |  |  |
| 42 |  |  |  |  |  |
| 43 |  |  |  |  |  |
| 44 |  |  |  |  |  |
| 45 |  |  |  |  |  |
| 46 |  |  |  |  |  |
| 47 |  |  |  |  |  |
| 51 |  |  |  |  |  |
| 52 |  |  |  |  |  |
| 53 |  |  |  |  |  |
| 54 |  |  |  |  |  |
| 55 |  |  |  |  |  |