| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M1026-v4 |
| --- | --- |

| Title: | Description of Core Experiment 6 (CE6): Transforms and transform signalling |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Core experiment description |  |  |
| Author(s) or Contact(s): | Xin Zhao Hilmi E. Egilmez | Tel: Email: | xinzzhao@tencent.com hegilmez@qti.qualcomm.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

The purpose of this Core Experiment (CE) is to investigate the coding tools related to transforms and transform signalling on top of the Test Model. In this CE, the tradeoff between compression efficiency and complexity of different transformation tools will be investigated. In addition to encoder and decoder run-times, other complexity metrics, e.g., operation counts and storage cost, will be also evaluated. The list of tools and experiments to be performed are detailed in this document.

# Introduction

In CE6, a total of 22 tests are defined, which are organized in the following sub-test categories:

- CE6-1: Unification between MTS and Transform Skip mode (4 test)

- CE6-2: Reduced transform candidates of MTS (13 tests)

- CE6-3: Secondary transform (5 tests)

# Participant list

| Affiliation | Name | E-mail | P/C |
| --- | --- | --- | --- |
| BBC | Gosala Kulupana | gosala.kulupana@bbc.co.uk |  |
| Broadcom | Minhua Zhou | minhua.zhou@broadcom.com |  |
|  | Peisong Chen | peisong.chen@broadcom.com |  |
|  | Wade Wan | wade.wan@broadcom.com |  |
| Bytedance | Li Zhang | lizhang.idm@bytedance.com |  |
| Cannon | Chris Rosewarne | Chris.Rosewarne@cisra.canon.com.au |  |
| Chosun University | Bumshik Lee | bslee@chosun.ac.kr |  |
|  | Sandeep Shrestha | sandeepshrestha407@chosun.kr |  |
| Ericsson | Christopher Hollmann | christopher.hollmann@ericsson.com | P&C |
| ETRI | Jung Won Kang | jungwon@etri.re.kr |  |
| Fujitsu | Jianqing Zhu | zhujianqing@cn.fujitsu.com |  |
|  | Jie Yao | yaojie@cn.fujitsu.com |  |
| HikVision | Xiaoqiang Cao | caoxiaoqiang@hikvision.com |  |
| HiSilicon | Yongbing Lin | linyongbing@hisilicon.com |  |
|  | Jianhua Zheng | zhengjianhua@hisilicon.com |  |
| Huawei | Jianle Chen | jianle.chen@huawei.com |  |
|  | Haitao Yang | haitao.yang@huawei.com |  |
|  | Yin Zhao | yin.zhao@huawei.com |  |
|  | Alexander Karabutov | karabutov.alexander@huawei.com |  |
|  | Semih Esenlik | semih.esenlik@huawei.com |  |
|  | Han Gao | han.gao@huawei.com |  |
|  | Seungwook Hong | seuhong@outlook.com |  |
| Humax | Yonggil Lee | yglee@humaxdigital.com |  |
| Intel | Samuel Wong | samuel.wong@intel.com |  |
|  | Grace Yu | grace.yu@intel.com |  |
| InterDigital | Saurav Bandyopadhyay | Saurav.Bandyopadhyay@InterDigital.com |  |
| Kwai | Xianglin Wang | xianglinwang@kwai.com |  |
|  | Yi-Wen Chen | yiwenchen@kwai.com |  |
| LGE | Seung-Hwan Kim | seunghwan3.kim@lge.com | P |
|  | Mehdi Salehifar | mehdi.salehifar@lge.com |  |
|  | Moonmo Koo | moonmo.koo@lge.com |  |
|  | Jaehyun Lim | jaehyun.lim@lge.com |  |
|  | Hyeongmun Jang | hm.jang@lge.com |  |
| MediaTek | Olena Chubach | Olena.Chubach@mediatek.com |  |
|  | Shih-Ta Hsiang | Shih-ta.Hsiang@mediatek.com |  |
| Nokia | Jani Lainema | jani.lainema@nokia.com | P |
| Oppo | Fannie.Yu | yuyuanfang@oppo.com |  |
| Orange | Pierrick Philippe | pierrick.philippe@orange.com | C |
| Panasonic | Kiyofumi Abe | abe.kiyo@jp.panasonic.com |  |
|  | Toma Tadamasa | toma.tadamasa@jp.panasonic.com |  |
| Qualcomm | Amir Said | asaid@qti.qualcomm.com |  |
|  | Hilmi E. Egilmez | hegilmez@qti.qualcomm.com |  |
|  | Yung-Hsuan Chao | yunghsua@qti.qualcomm.com |  |
|  | Vadim Seregin | vseregin@qti.qualcomm.com |  |
|  | Ted Hsieh | thsieh@qti.qualcomm.com |  |
|  | Nan Hu | nanh@qti.qualcomm.com |  |
|  | Chao-Hsiung Hung | chaohsiu@qti.qualcomm.com |  |
| Samsung | Kiho Choi | kiho14.choi@samsung.com | P |
| Sejong Univ. | Dong-jae Won | bigdj2002@sju.ac.kr |  |
| Sony | Masaru Ikeda | Masaru.Ikeda@sony.com | C |
|  | Takeshi Tsukuba | takeshi.tsukuba@sony.com |  |
|  | Teruhiko Suzuki | teruhiko.s@sony.com |  |
| Technicolor | Fabrice Le Léannec | Fabrice.Leleannec@technicolor.com |  |
|  | Fabien Racape | Fabien.Racape@technicolor.com |  |
|  | Eduard Francois | Edouard.Francois@technicolor.com |  |
|  | Karam Naser | Karam.NASER@technicolor.com |  |
| Tencent | Xin Zhao | xinzzhao@tencent.com | P&C |
|  | Xiang Li | xlxiangli@tencent.com |  |
|  | Liang Zhao | leolzhao@tencent.com |  |
|  | Sehoon Yea | sehoonyea@tencent.com |  |
| Ubilinx | Eric Chai | eric.chai@ublnx.com |  |
|  | Weimin Zeng | weimin.zeng@ublnx.com |  |
| WILUS Inc. | Jason Jung | jason.jung@wilusgroup.com |  |
|  | John Son | john.son@wilusgroup.com |  |
| ZTE | Hong Wang | philcage@163.com |  |

# Description of tools

## CE6-1: Unification between MTS and Transform Skip mode (4 tests)

### Test descriptions

JVET-M0501 CE6 related: Unification of Transform Skip mode and MTS [X. Zhao, X. Li, S. Liu (Tencent)]

Transform Skip (TS) mode and MTS are unified such that TS mode becomes part of MTS by replacing DCT-8 with Identity Transform (IDT). Such that transform skipping can be done in either the horizontal direction, vertical direction or both directions. The replacement of DCT-8 by IDT can be done up to a certain block size, which can be configured (e.g., 4x4, 8x8, 16x16 or 32x32).

Since the adoption of JVET-M0464 changes the encoder search and MTS index binarization, further binarization changes which include IDT as part of MTS are also tested. In the following tests, (DCT8, DCT8) pair in MTS is replaced by (IDT, IDT) for all block sizes (up to 32-length).

### Summary of Tests

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE6-1.1a | One dimensional TS (IDT used as horizontal or vertical transform) is applied up to 32-point. | X. Zhao (Tencent) | T. Tsukuba (Sony) |
| Withdrawn | One dimensional TS (IDT used as horizontal or vertical transform) is applied up to 4-point. | X. Zhao (Tencent) |  |
| Withdrawn | One dimensional TS (IDT used as horizontal or vertical transform) is applied up to 8-point. | X. Zhao (Tencent) |  |
| Withdrawn | One dimensional TS (IDT used as horizontal or vertical transform) is applied up to 16-point. | X. Zhao (Tencent) |  |

## CE6-2: Reduced transform candidates of MTS (9 tests)

### Test descriptions

JVET-M0304 CE6-related: 2-mode MTS with shape adaptive transform selection [J. Lainema (Nokia)]

This experiment measures performance of the 2-mode MTS and implicit transform selection approach proposed in JVET-M0304. Proposed tool removes the signaling for mts_idx and thus uses only 2 MTS modes. It also uses implicit shape-based transform selection when tu_mts_flag is zero.

Impact of different TT and BT splits for 64x64 intra picture CUs is measured as part of the test. Also results for the 2-mode MTS mode alone without the implicit transform selection for tu_mts_flag zero is measured.

The implicit transform selection tool decides horizontal and vertical transforms for Intra coded CUs based on the dimensions of the CU. If a CU is non-square, DST-7 is selected for the shorter side of the CU and DCT-2 is used for the longer one. In the case of explicit transform selection, the maximum size of DST-7 transform is limited to 16-points.

JVET-M0366 CE6 related: Transform Simplification [C. Hollman, D. Saffar, P. Wennersten, J. Ström (Ericsson)]

In JVET-M0366 a simplification of the Multiple Transform Selection tool was proposed. The proposed changes include:

- Remove the combination (DCT-8, DCT-8) from the MTS tool,

- Replace the DCT-8 for size 32 with the existing DCT-2,

- Replace the mts_tu_idx with two flags (mts_dst_flag, mts_tu_flag) indicating which transform to use, and

- Change the CABAC context selection of the mts_cu_flag to be dependent on block size (inter and intra coded blocks) and intra mode (intra coded blocks).

During the discussion in the BoG on CE6-related documents is was commented that the proposed change to the context selection of the mts_cu_flag introduces a parsing dependency, as the intra mode needs to be reconstructed before the flag can be parsed. In the CE test, this dependency is removed.

The following aspects of JVET-M0366 are to be studied in a CE:

- Remove the combination (DCT-8, DCT-8) from consideration in the MTS tool,

- Replace the DCT-8 for size 32 with the existing DCT-2,

- Change the coding of the mts_tu_flag according to Table 1 (as in M0366), and

- The mts_cu_flag is coded in the same way it will be coded in VTM-4.0.

As the adoption of M0464 at the Marrakech changes the encoder search, the proposed changes are tested with three different encoder settings to investigate whether the improvement is related to the configuration of the encoder. The changes are made to the configuration parameters MTSIntraMaxCand and MTSInterMaxCand for both anchor and test software. Three different settings will be tested:

- CE6-2.3a: MTSIntraMaxCand = 3, MTSInterMaxCand = 3

- CE6-2.3b: MTSIntraMaxCand = 4, MTSInterMaxCand = 4

- CE6-2.3c: MTSIntraMaxCand = 5, MTSInterMaxCand = 5

Table 1: Bin coding for mts_tu_flag

| Horizontal | Vertical | Intra coded block |  | Inter coded block |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Intra mode | Coded bin | Height > Width | Coded bin |
| DST-7 | DCT-X | 0-34 | 0 | yes | 0 |
| DST-7 | DCT-X | 35-66 | 1 | no | 1 |
| DCT-X | DST-7 | 0-34 | 1 | yes | 1 |
| DCT-X | DST-7 | 35-66 | 0 | no | 0 |

JVET-M0202 CE6-related: Simplification related to MTS with reduced modes [K. Choi, M. Park, M. W. Park, W. Choi (Samsung)]

In order to reduce encoding complexity of using MTS, in this CE test, 4 testing including non-normative and normative changes are tested as follows:

- Test1: Non-normative change with 3modes

  - {H:DST7, V:DST7}, {H:DST7, V:DCT8}, {H:DCT8, V:DST7}, {H: DCT8, V:DCT8}

- Test2: Non-normative change with 2modes

  - {H:DST7, V:DST7}, {H:DST7, V:DCT8}, {H:DCT8, V:DST7}, {H: DCT8, V:DCT8}

- Test3: Normative change with 3modes

  - {H:DST7, V:DST7}, {H:DST7, V:DCT8}, {H:DCT8, V:DST7}

- Test4: Normative change with 3modes

  - {H:DST7, V:DST7}, {H:DST7, V:DCT8}

For the Test 3 and 4, the truncated unary was used for those tests as shown in the Table 1 and 2.

Table 2: Syntax design for Test3

|  | Horizontal Kernel | Vertical Kernel | Code |
| --- | --- | --- | --- |
| Mode 0 | DST7 | DST7 | 0 |
| Mode 1 | DST7 | DCT8 | 10 |
| Mode 3 | DCT8 | DST7 | 11 |

Table 2. Syntax design for Test4

|  | Horizontal Kernel | Vertical Kernel | Code |
| --- | --- | --- | --- |
| Mode 0 | DST7 | DST7 | 0 |
| Mode 1 | DST7 | DCT8 | 1 |

### Summary of Tests

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE6-2.1a | 2-mode MTS without implicit transform selection, Allowed intra picture 64x64 CU splits: QT, TT, BT | J. Lainema (Nokia) | C. Hollmann (Ericsson) |
| CE6-2.1b | 2-mode MTS without implicit transform selection, Allowed intra picture 64x64 CU splits: QT, TT | J. Lainema (Nokia) | C. Hollmann (Ericsson) |
| CE6-2.1c | 2-mode MTS without implicit transform selection, Allowed intra picture 64x64 CU splits: QT | J. Lainema (Nokia) | C. Hollmann (Ericsson) |
| Withdrawn | 2-mode MTS with implicit transform selection, Allowed intra picture 64x64 CU splits: QT, TT, BT | J. Lainema (Nokia) |  |
| Withdrawn | 2-mode MTS with implicit transform selection, Allowed intra picture 64x64 CU splits: QT, TT | J. Lainema (Nokia) |  |
| Withdrawn | 2-mode MTS with implicit transform selection, Allowed intra picture 64x64 CU splits: QT | J. Lainema (Nokia) |  |
| CE6-2.2a | Non-normative change with 3 modes | K. Choi (Samsung) | X. Zhao (Tencent) |
| CE6-2.2b | Non-normative change with 2 modes | K. Choi (Samsung) | X. Zhao (Tencent) |
| CE6-2.2c | Normative change with 3 modes | K. Choi (Samsung) | X. Zhao (Tencent) |
| CE6-2.2d | Normative change with 2 modes | K. Choi (Samsung) | X. Zhao (Tencent) |
| CE6-2.3a | Simplified transform selection with reduced modes and no size-32 DCT-8, Number of evaluated candidates: 3 | C. Hollmann (Ericsson) | J. Lainema (Nokia) |
| CE6-2.3b | Simplified transform selection with reduced modes and no size-32 DCT-8, Number of evaluated candidates: 4 | C. Hollmann (Ericsson) | J. Lainema (Nokia) |
| CE6-2.3c | Simplified transform selection with reduced modes and no size-32 DCT-8, Number of evaluated candidates: 5 | C. Hollmann (Ericsson) | J. Lainema (Nokia) |

## CE6-3: Secondary transform (5 tests)

### Test descriptions

JVET-M0292 CE6-5.1: Reduced Secondary Transform (RST) [M. Koo, M. Salehifar, J. Lim, S. Kim (LGE)]

Secondary transform is applied between forward primary transform and quantization (at encoder) and between de-quantization and inverse primary transform (at decoder side). 4x4 non-separable secondary transform (16x16 or 16x8 matrix) called 4x4 RST is applied for small blocks (i.e., min (width, height) < 8) and 8x8 reduced non-separable secondary transform (64x16 or 64x8 matrix) called 8x8 RST is applied for larger blocks (i.e., min (width, height) > 4). Based on intra prediction mode, corresponding transform set is determined according to pre-defined mapping rule. In this test totally 4 transform sets are used. Each transform set consists of 2 transform kernels, and an index is signalled to indicate which transform kernel to be used.

For test 1 (6-3.1a), as described above, 4x4 RST (where 4x4 RST kernel size is equal to 16x16 matrix) is applied for small blocks (min (width, height) < 8), and 8x8 RST (where 4x4 RST kernel size is 16x64 matrix) is applied for larger block (min (width, height) > 4).

For test 2 (6-3.1b), in order to limit worst case number of multiplications of per sample to 8, more reduced 4x4 RST and 8x8 RST is applied to 4x4 block and 8x8 block, respectively. Specifically, 4x4 RST (where 4x4 RST kernel size is equal to 16x8 matrix) is applied for 4x4 block, and 8x8 RST (where 4x4 RST kernel size is 8x64 matrix) is applied for 8x8 block.

For test 3 (6-3.1c), on top of test 2, secondary transform for 4x4 TU is skipped.

For test 4 (6-3.1d), instead of 64x16, 48x16 matrices are used for 8x8 RST to reduce memory space for storing transform kernels. Hence, memory usage for storing all RST matrices can be reduced from 10KB to 8KB.

For test 5 (6-3.1e), all conditions are the same as test 1, but only one transform kernel per set is applied. Hence, 5KB (instead of 10 KB) is required to store transform kernels.

### Summary of Tests

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE6-3.1a | ( 4 transform sets. 2 transforms per set ) | M. Koo (LGE) | P. Philippe (Orange) |
| CE6-3.1b | ( 4 transform sets. 2 transforms per set ) + (Secondary transform uses at most maximum 8 multiplications/sample ) | M. Koo (LGE) | P. Philippe (Orange) |
| CE6-3.1c | ( 4 transform sets. 2 transforms per set ) + ( Secondary transform uses at most maximum 8 multiplications/sample ) + ( Secondary transform is disabled for 4x4 TU ) | M. Koo (LGE) | P. Philippe (Orange) |
| CE6-3.1d | ( 4 transform sets. 2 transforms per set ) + ( Secondary transform uses at most maximum 8 multiplications/sample ) + ( 16x48 matrices are employed instead of 16x64 ones ) | M. Koo (LGE) | P. Philippe (Orange) |
| CE6-3.1e | ( 4 transform sets. 1 transforms per set ) | M. Koo (LGE) | P. Philippe (Orange) |

# Test conditions

The following test conditions shall be used for this CE:

## Software

The Test Model VTM-4.0 [1] shall be used for these experiments, as specified in JVET-M1010 [2].

## Test configurations

This CE shall use the following two test conditions:

- Common Test Conditions (CTC).

- CTC with inter MTS enabled

The anchor for the above first test condition is VTM-4.0 using CTC, and the anchor for the above second test condition is VTM-4.0 using CTC w/ inter MTS enabled.

The following requirements apply to all tests defined in this CE:

- Any encoder or decoder optimizations (e.g., SIMD or encoder optimizations) that are not an essential part of the proposed method shall be described, and individual experiment results on the related optimizations shall be reported.

- For CE6-3 tests which have impact on the transform precision, i.e., modifications on transform cores or addition of stages/transforms (such as adjustment and secondary), results using low-QP configurations, i.e., QP 12, 17, 22 and 27 shall be provided. For low-QP configuration, simulations results using All Intra configuration are mandatory and simulation results using Random Access and Low Delay B (or P) configurations are optional, and the total number of frames to be coded is modified using the following encoder configuration:

--FramesToBeEncoded=100

- For CE6-1 tests which are related to transform skip mode, additional test results using TGM sequences need to be also reported.

# Complexity metrics

For the understanding of complexity, the following information shall be provided for each test planned for this CE:

- Operation counts (number of multiplications, additions, and bit-shift operations) for individual transforms. If encoder and decoder use different number of transform operations as compared to the VTM, operation counts shall be reported for both encoder and decoder separately.

- Memory requirements for storing transform cores (in bytes).

- List of all combinations of transform types (e.g., DCT-2 and DST-7) and transforms block sizes (e.g., 4x4, 4x8, 8x4 and 8x8) used for primary transformation.

- List of all combinations of transforms block size and transform type used for secondary transformation if any.

# Timeline

| T1 | Feb. 08, 2019 | - CE description is finalized and uploaded |
| --- | --- | --- |
| T2 | Feb. 25, 2019 (Two weeks after VTM-4.0 SW release) | - Proponents provide initial SW of CE tools - Cross-checkers may start initial study of tools / software |
| T3 | Mar. 04, 2019 | - SW of each CE test is finalized - Cross-checkers start final cross checks |
| T4 | Mar. 12, 2019 | - Complete results of each CE test shall be provided - Cross-checker report crosschecking results to proponents and CE coordinators - CE6 summary report is provided |

# References

- VVC Test Model VTM-4.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-M1010, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019.