| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M1031-v2 |
| --- | --- |

| Title: | Description of Core Experiment 11 (CE11): Deblocking |  |  |
| --- | --- | --- | --- |
| Status: | Output Document to JVET |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Andrey Norkin Anand Meher Kotra | Tel: Email: | anorkin@netflix.com anand.meher.kotra@huawei.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

The goal of this Core Experiment (CE) is to improve and/or enhance deblocking filtering techniques in the test model (VTM). Techniques covered by this CE are related to improvements to already adopted longer tap deblocking filter, filtering on the 4x4 boundaries.

The deblocking filter techniques are evaluated according to subjective picture quality as well as BD-rate gains. These are organized in CE11.1 and CE11.2 which address aspects of the long deblocking filter improvements, and filtering at 4x4 block boundaries and sub-pu deblocking. Participants in this activity are Ericsson, HHI, Huawei, MediaTek.

The software basis for this CE is VTM-4.0 [2]. For the test sequences, configurations and test conditions, in JVET-M1010 [1] for SDR sequences are used. For subjective viewing, additional results with sequences encoded with higher QPs may be required.

# Participants

| Nr. | Name | Company | Email |
| --- | --- | --- | --- |
| 1 | Alexis Tourapis | Apple | atourapis@apple.com |
| 2 | Li Zhang | Bytedance | lizhang.idm@bytedance.com |
| 3 | Patrice Onno | Canon | patrice.onno@crf.canon.fr |
| 4 | Jonathan Taquet | Canon | jonathan.taquet@crf.canon.fr |
| 5 | Christophe Gisquet | Canon | christophe.gisquet@crf.canon.fr |
| 6 | Dan Grois | Comcast | grois@ieee.org |
| 7 | Yongjo Ahn | Digital Insights | yjahn@digitalinsights.co.kr |
| 8 | Xiaozhen Zheng | DJI | xiaozhen.zheng@dji.com |
| 9 | Peng Yin | Dolby | pyin@dolby.com |
| 10 | Taoran Lu | Dolby | tlu@dolby.com |
| 11 | Kenneth Andersson | Ericsson | kenneth.r.andersson@ericsson.com |
| 12 | Jungwon Kang | ETRI | jungwon@etri.re.kr |
| 13 | Hahyun Lee | ETRI | hanilee@etri.re.kr |
| 14 | Yao-Jen Chang | Foxconn | yaojen.chang.foxconn@gmail.com |
| 15 | Tim Li | Foxconn | tim410177@gmail.com |
| 16 | Hong-Jheng Jhu | Foxconn | ted0958.am96@g2.nctu.edu.tw |
| 17 | Benjamin Bross | HHI | benjamin.bross@hhi.fraunhofer.de |
| 18 | Christian Helmrich | HHI | christian.helmrich@hhi.fraunhofer.de |
| 19 | Adam Wieckowski | HHI | adam.wieckowski@hhi.fraunhofer.de |
| 20 | Jianhua Zheng | HiSilicon | zhengjianhua@hisilicon.com |
| 21 | Xu Chen | HiSilicon | anderson.chen@hisilicon.com |
| 22 | Na Zhang | HiSilicon | zhangna25@hisilicon.com |
| 23 | Haechul Choi | HNU | choihc@hanbat.ac.kr |
| 24 | Heeji Han | HNU | sarhakor@naver.com |
| 25 | Daehyeok Gwon | HNU | skyeye0530@naver.com |
| 26 | Jaeryun Choe | HNU | tjswo9395@naver.com |
| 27 | Sergey Ikonin | Huawei | sergey.ikonin@huawei.com |
| 28 | Filippov Alexey | Huawei | alexey.filippov@huawei.com |
| 29 | Vasily Rufitskiy | Huawei | vasily.rufitskiy@huawei.com |
| 30 | Biao Wang | Huawei | biao.wang@huawei.com |
| 31 | Jianle Chen | Huawei | Jianle.chen@huawei.com |
| 32 | Grace Yu | Intel | grace.yu@intel.com |
| 33 | Samuel Wong | Intel | swong10h266@gmail.com |
| 34 | Rahul Vanam | InterDigital | Rahul.vanam@interdigital.com |
| 35 | Kyohei Unno | KDDI | ky-unno@kddi-research.jp |
| 36 | Kei Kawamura | KDDI | ki-kawamura@kddi.com |
| 37 | Yi-Wen Chen | Kwai | yiwenchen@kwai.com |
| 38 | Xianglin Wang | Kwai | xianglinwang@kwai.com |
| 39 | Seung Hwan Kim | LG | seunghwan3.kim@lge.com |
| 40 | Junghak Nam | LG | junghak.nam@lge.com |
| 41 | Hyeongmun Jang | LG | hm.jang@lge.com |
| 42 | Jie Zhao | LG | jie.zhao@lge.com |
| 43 | Jaehyun Lim | LG | jaehyun.lim@lge.com |
| 44 | Chih-Wei Hsu | MediaTek | cw.hsu@mediatek.com |
| 45 | Chia-ming Tsai | MediaTek | Chia-ming.Tsai@mediatek.com |
| 46 | Ching-Yeh Chen | MediaTek | chingyeh.chen@mediatek.com |
| 47 | Tzu-Der Chuang | MediaTek | Peter.Chuang@mediatek.com |
| 48 | Yu-Wen Huang | MediaTek | yuwen.huang@mediatek.com |
| 49 | Atsuro Ichigaya | NHK | ichigaya.a-go@nhk.or.jp |
| 50 | Shunsuke Iwamura | NHK | iwamura.s-gc@nhk.or.jp |
| 51 | Shimpei Nemoto | NHK | nemoto.s-fy@nhk.or.jp |
| 52 | Fannie Yu | Oppo | yuyuanfang@oppo.com |
| 53 | Tadamasa Toma | Panasonic | toma.tadamasa@jp.panasonic.com |
| 54 | Kiyofumi Abe | Panasonic | abe.kiyo@jp.panasonic.com |
| 55 | Dmytro Rusanovskyy | Qualcomm | dmytror@qti.qualcomm.com |
| 56 | Wei-Jung Chien | Qualcomm | wchien@qti.qualcomm.com |
| 57 | Chun-Chi Chen | Qualcomm | chunchic@qti.qualcomm.com |
| 58 | Yan Zhang | Qualcomm | yzh@qti.qualcomm.com |
| 59 | Chao-Hsiung Hung | Qualcomm | chaohsiu@qti.qualcomm.com |
| 60 | Kiho Choi | Samsung | kiho14.choi@samsung.com |
| 61 | Woongil Choi | Samsung | woongil.choi@samsung.com |
| 62 | Weijia Zhu | Sharp | zhuw@sharplabs.com |
| 63 | Andrew Segall | Sharp | asegall@sharplabs.com |
| 64 | Kiran Misra | Sharp | misrak@sharplabs.com |
| 65 | Takeshi Chujoh | Sharp | chujoh.takeshi@sharp.co.jp |
| 66 | Tomohiro Ikai | Sharp | ikai.tomohiro@sharp.co.jp |
| 67 | Dongjae Won | SJU | bigdj2002@sju.ac.kr |
| 68 | Jaemin Ha | SJU | hajama0123@sju.ac.kr |
| 69 | Masaru Ikeda | Sony | masaru.ikeda@sony.com |
| 70 | Teruhiko Suzuki | Sony | teruhiko.s@sony.com |
| 71 | Eric Chai | Ubilinx | eric.chai@ublnx.com |
| 72 | Weimin Zeng | Ubilinx | weimin.zeng@ublnx.com |
| 73 | Jason Jung | WILUS Inc | jason.jung@wilusgroup.com |

# Test conditions and evaluation criteria

The proposals should be encoded according to two test conditions, which correspond to two anchors.

Anchor-1 is VTM-4.0 according to the CTC [1].

Anchor-2 is VTM-4.0 with ALF switched off (other conditions are the same as in Anchor 2).

The proposals in the CE shall be implemented based on VTM-4.0. The results for two test conditions shall be provided: Test Conditions 1 use configuration of Anchor 1, Test Conditions 2 use configuration of Anchor 2. The results for LDP configuration are mandatory for this CE.

Proposals will be compared with respect to subjective, objective quality, and complexity.

# Tool description

## Introduction

The main goal with this Core Experiment is to enhance the subjective quality of the Test Model and to ensure that CUs can be filtered in parallel.

Following aspects will be studied:

- Long deblocking filters improvements

- Deblocking for 4x4 block boundaries

Different technologies will be compared at a subjective viewing at the next meeting.

# CE 11-1: long-tap deblocking filter improvements

Proposal in this sub-CE investigate improvements to the already adopted longer deblocking filter of VVC.

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE11-1.1 | Christian Helmrich christian.helmrich@hhi.fraunhofer.de JVET-M0092 | Kenneth Andersson ?? kenneth.r.andersson@ericsson.com |

## Description of technologies in CE11.1

#### CE11-1.1 Very strong deblocking filtering with conditional activation signaling JVET-M0092

In JVET-L0523, it was proposed to introduce a signaled activation flag, controlled by the encoder, to selectively disable strong deblocking on a CTU-by-CTU basis.  To minimize the signaling overhead required for this flag, a conditional transmission of the activation flag is employed on a CTU-wise level based on the partitioning and residual coding information already present in the bit-stream.  In other words, the activation flag is not signaled in each CTU but only in selected ones.  As a result, Bjøntegaard delta (BD) rate increase caused by said flag (a 1-bit element) is less than 0.04 % on average.  In CTUs for which no flag is transmitted, strong deblocking is always disabled, and the deblocking resorts to the conventional weaker filtering as known from HEVC.
In JVET-L0523, the authors proposed the selective signaling of an in-loop filter control parameter (FCP) only in CTUs which are partitioned into fewer than A = 9 coding units (CUs) and in which residual coefficient coding, as indicated by a non-zero coded block flag (CBF), is employed in at least B = 1 of its transform units (TUs).
In JVET-M0092, the abovementioned A+B condition for signaling of the FCPs was extended to cover slightly more CTUs while keeping the signaling rate at roughly the same level of less than about 0.05% of the mean bit-rate.  Specifically, the FCP is now written to, and read from, the bit-stream (1) for all CTUs partitioned into fewer than A = 10 CUs and at least B = 1 coded TUs or, if no TU is coded with a non-zero CBF, (2) for CTUs partitioned into c CUs with 3 < c < A.

The following is proposed to be tested:

- selective signaling of an in-loop filter control parameter (FCP) per CTU to allow the application of very strong deblocking filtering; this FCP shall only be signaled in a CTU if that CTU is partitioned into fewer than A sub-units (TUs) and if residual coefficient coding is used in at least B of the TUs.

Details including the encoder decision are provided in JVET-L0523

# CE 11-2: Deblocking at 4x4 block boundaries

The proposals in this CE study application of deblocking filtering to the 4x4 boundaries. Common feature of proposals studied in this CE is filtering on the block boundaries coinciding with the 4x4 sample grid and making sure it does not introduce dependency of deblocking for an edge on a parallel edge preceding in a filtering order. Test CE11.2.2 investigates whether filtering any edges adjacent to the blocks of size 4 can be avoided.

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE11-2.1 | Kenneth Andersson kenneth.r.andersson@ericsson.com Anand Meher Kotra Anand.meher.kotra@huawei.com Chia-Ming Tsai chia-ming.tsai@mediatek.com JVET-M0299 | Hyeongmun Jang ?? hm.jang@lge.com |
| CE11-2.2 | Hyeongmun Jang hm.jang@lge.com JVET-M0337 | Kenneth Andersson ?? kenneth.r.andersson@ericsson.com |
| CE11-2.3 | Anand Meher Kotra Anand.meher.kotra@huawei.com | ?? |

## Description of technologies in CE11.2

#### CE11-2.1 Deblocking for 4xN, Nx4, 8xN and Nx8 blocks JVET-M0299

In current VVC software large rectangular blocks of size Nx8 and Nx4 are not deblocked where N can be up to 64. This test applies deblocking on 4x4 grid to allow for deblocking of Nx4 blocks aligned with the current 8x8 grid but also for Nx8 blocks that not are aligned with the current grid. The number of samples to read and modify is limited to allow for parallel friendly processing. If at least one side of the block boundary has a length orthogonal to the block boundary equal to 4 only use the weak filter. Furthermore, only one sample is filtered on each side of the block boundary in this case.

#### CE11-2.2 Parallel deblocking filter JVET-M0337

Deblocking filter can be processed in parallel because it is not applied to 4x4 TU and PU boundaries. Within this CE, the performance of the parallel deblocking method described in JVET-D0044 is to be studied in terms of subjective quality and computational complexity.

For the luma component, deblocking filtering process is not applied on block boundary of 4xN block for vertical edge and Nx4 block for horizontal edge

For the chroma component, deblocking filtering process is not applied on block boundary of 2xN block for vertical edge and Nx2 block for horizontal edge.

#### CE11-2.3 Disabling of Sub-Pu deblocking

This test would try to evaluate if sub-pu deblocking brings visual quality improvement. Currently in VTM-3.0, the sub-pu edges are only deblocked if the coding unit edge which uses sub-pu tool like Affine, ATMVP aligns with the 8 x 8 sample grid. Therefore the current test would evaluate if disabling the sub-pu deblocking completely would result in any visual quality degradation.

# Expected Gains

- Subjective picture quality improvement while preferably keeping rate-distortion performance.

- Coding efficiency improvement.

# Evaluation Criteria

1. Measure impact on Bitrate/PSNR. The BD-rate is calculated for CTC and for CTC with ALF switched off relative to Anchor-1 and Anchor-2 respectively. The results for LDP configurations are mandatory for both CTC and CTC with ALF switched off.

2. Visual impact on output pictures of representative sequences. The subjective test is expected to be held to determine the visual impact. The subjective test will use 10 second versions of the sequences. The proponents shall provide the bitstreams and decoders for the sequences that are chosen for evaluation in the subjective test.

3. Complexity of the proposals evaluated in the CE shall be compared. In particular, the following aspects of the proposals shall be summarized:

- decoding/encoding time vs the anchor

- number of samples from the block boundary that can be modified by deblocking (may be per block size)

- number of samples from the block boundary that are needed for deblocking decisions (may be per block size)

- number of filtering operations (such as additions, multiplications, comparisons)

- number of filtering operations including decisions and filtering (per sample)

- number of line buffers

- parallelization capabilities, such as size of a unit of a picture that can be processed in parallel to other units.

- whether proposals increase the worst case complexity and if so, by how much.

- data should be provided for both luma and/or chroma if they are modified.

# Subjective test(s)

There are two possibilities for performing a subjective test.

a) A subjective test will be conducted by the test chair before the Geneva meeting.

The following test sequences and conditions will be used in the subjective tests. The sequence length is 10 seconds. This means that the versions of some sequences encoded for the subjective test are longer than those used in the CTC [1].

The subjective test sequences shall be provided by Feb 15, 2019 (uploaded to the JVET ftp folder /ce/JVET-M1031_Deblocking), coding configuration with ALF ON shall be used.

The anchors for the subjective viewing will use the ALF ON configuration and will be provided by A.M. Kotra.

The following sequences and QP(tentative) will be used for subjective evaluation in CE11.1 and CE 11.2 categories:

| Num | Name | Resolution | fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | FoodMarket | 3840x2160p | 60fps | 600 | RA | 39, 34 |
| 02 | Campfire | 3840x2160p | 30fps | 300 | RA | 39, 34 |
| 03 | Kimono | 1920x1080p | 24fps | 240 | LDB | 34, 39 |
| 04 | KristenAndSara | 1280x720p | 60fps | 600 | LDP | 34, 39 |
| 05 | Red Kayak* | 1920x1080p | 30fps | 300 | RA | 34, 39 |

* First 300 frames of the RedKayak sequence shall be used.

b) Expert viewing at the Geneva meeting may be performed.

The sequences listed in this section shall be provided at the Geneva meeting by the proponents for their corresponding sub-CEs (CE11.1, CE11.2). The sequences coded with ALF switched on and ALF switched off.

## Subjective test description:

The test material at resolution lower than 4K will be expanded to full screen by the player and evaluated together with the 4K test material.

# Test conditions and data to be provided

The test conditions are according to CTC on VTM with ALF turned on and VTM with ALF turned off. In addition, the proponents need to provide results for additional QP and additional test sequences chosen by the CE (see above). The sequences length for the subjective viewing is 10 seconds. The complexity of the proposals according to Section 8 should be reported by the proponents.

# Time-line and Responsibilities

T1: 2019-February-08:		Final CE description uploaded

T2:  T1 + 2 weeks:		   Cross-checking begins, proponents provide software

T3: 2019-February-26: 		Final version of CE software, full test results must be provided at this time.

Sequences for the subjective test are uploaded to the ftp.

# References

- “JVET common test conditions and software reference configurations”, JVET-M1010.

- “Algorithm description for Versatile Video Coding and Test Model 2 (VTM3)”, JVET-M1002.