| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0007 |
| --- | --- |

| Title: | JVET AHG report: Coding of HDR/WCG Material (AHG7) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | A. Segall W. Husak E. Francois D. Rusanovskyy | Email: | asegall@sharplbas.com WJH@dolby.com Edouard.Francois@technicolor.com dmytror@qti.qualcomm.com |
| Source: | AHG7 |  |  |

_____________________________

# Abstract

This document summarizes the activity of AHG7: Coding of HDR/WCG Material between the 13th meeting in Marrakech, MA (9 – 18 January 2019) and the 14th meeting in Geneva, CH (19 – 27 March 2019).

# Mandates

The AHG was established with the following mandates:

- Study and evaluate available HDR/WCG test content.

- Study objective metrics for quality assessment of HDR/WCG material, including investigation of the correlation between subjective and objective results of the CfP responses.

- Compare the performance of the VTM and HM for HDR/WCG content.

- Prepare for expert viewing of HDR content at the 14th JVET meeting if feasible.

- Coordinate implementation of HDR anchor aspects in the test model software with AHG3.

- Study additional aspects of coding HDR/WCG content.

# Activities

The AHG used the main JVET reflector, jvet@lists.rwth-aachen.de, with an [AHG7] indication on message headers.  The primary activity of the AhG was related to the mandates of comparing the performance of the VTM for HDR/WCG content. This work is described in the following subsection.

## Anchor Discussion

During the anchor generation process, it was observed that the definition of the HDR anchors currently does not use the “luma mapping with chroma scaling” tool (LMCS), otherwise known as reshaping. There is no knowledge of this being an intentional decision at the Marrakech meeting, and it appears more due to the following scenario happening:

- January 30th:	LMCS tool committed to the VTM.

VTM configuration files enable luma dQP for coding HDR-PQ sequences

HDR CTC specifies that the configuration files should be used.

- February 4th: 	VTM-4.0rc1 tagged

- February 7th: 	Bug found with the combination of LMCS and luma dQP tool enabled.

Patch submitted by LMCS proponent to disable LMCS when luma dQP is

enabled.

- February 11th: 	VTM-4.0 tagged

VTM configuration files still enable luma dQP

This created a poorly defined situation, as the HDR CTC describes that luma dQP should be enabled as defined in the configuration files, and there was no update for the HDR CTC scheduled as an output of the Marrakech meeting.

Further evaluation showed that the performance of the anchor was not dramatically affected.  And, given that the time between tagging VTM-4.0 and the document release date for the Geneva meeting was less than one month, it was deemed best to defer clarifying and/or correcting this issue at the Geneva meeting.

## Performance Comparison

The AhG performed experiments comparing the performance of VTM 4.0 and VTM 3.0 codecs on HDR content.  A summary of the performance is provided below.

Please note, and to be abundantly clear, results marked as “VTM 4.0 (Luma dQP Disabled)” define the performance of the VTM 4.0 with LMCS enabled and Luma dQP disabled.  “VTM 3.0 (Luma dQP Enabled)” define the existing VTM 3.0 HDR anchor.

### VTM 4.0 versus VTM 3.0

### VTM 4.0 versus HM 16.18

### VTM 4.0 LMCS performance

### VTM 4.0 performance – HLG Content

# Contributions

There is one contributions related to HDR video coding:

| JVET-N0221 | m46879 | Chroma Quantization Parameter Qpc Table for HDR Signal | T. Lu, F. Pu, P. Yin, S. McCarthy, W. Husak, T. Chen(Dolby) |
| --- | --- | --- | --- |

# Additional Comments

While not part of any official AhG activity, informal comments related to the AhG activity were made to one or more of the AhG chairs and captured here:

Question: Should the non-PSNR metrics be removed for HLG content?

Question: Should the fixed bit-rate category be removed from the CTC?

# Recommendations

The AHG recommends the following:

- Review all input contributions

- Discuss the configuration of the HDR anchor and align VTM 5 and/or reissue the HDR CTC document as needed