| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0013 |
| --- | --- |

| Title: | JVET AHG report: Tool reporting procedure (AHG13) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Wei-Jung Chien, Jill Boyce (co-chairs) Yi-Wen Chen, Roman Chernyak, Kiho Choi, Ryoji Hashimoto, Yu-Wen Huang, Hyeongmun Jang, Shan Liu, Daniel Luo (vice-chairs) | Email: | wchien@qti.qualcomm.com jill.boyce@intel.com yiwenchen@kwai.com chernyak.roman@huawei.com kiho14.choi@samsung.com ryoji.hashimoto.te@renesas.com yuwen.huang@mediatek.com hm.jang@lge.com shanl@tencent.com daniel.luo@interdigital.com |
| Source: | AHG13 |  |  |

_____________________________

# Abstract

This document summarizes the activity of AHG13: “Tool reporting procedure” between the 13th Meeting in Marrakech, MA (9–18 Jan. 2019) and the 14th meeting in Geneva, CH (19–27 March 2019). Tool on/off experimental results vs. VTM anchor are provided for the tools specified in JVET-M1005.

# Mandates

The AHG was established with the following mandates:

- Prepare output document JVET-M1005, which describes the methodology of tool-off testing and a list of tools to be tested by identified testers.

- Provide configurations files, bitstreams, and results of the tool-on/tool-off testing.

- Use the tool usage counts and memory bandwidth usage to study the decoder complexity of features in on/off testing.

- Prepare a report with results of the tests.

# Activities

The initial version of JVET-M1005 “Methodology and reporting template for tool testing” was provided on Jan. 30th.  The document contained a reporting template.

All tests described in JVET-M1005 were conducted. VTM tool tests were conducted on VTM-4.0 software with VTM configuration by switching off or on specific tool either in configuration files or macros.

The tested tools, testers, and cross-checkers are listed in the tables below.

## Tools included in VTM (Tool off test vs VTM Anchor)

Table 1 List of adoptions included in VTM (Tool off test (unless specified) vs VTM anchor)

| Tool Name | Acronym | Document reference(s) | AI | RA | LD | Tester | Crosscheck |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Chroma separate tree | CST | JVET-K0230, JVET-K0556 | X | X | X | Tzu-Der Chuang (peter.chuang@mediatek.com) | Wei-Jung Chien (wchien@qti.qualcomm.com) |
| Dependent quantization* | DQ | JVET-L0274, JVET-M0173, JVET-M0251, JVET-M0470, | X | X | X | Tzu-Der Chuang (peter.chuang@mediatek.com) | Daniel Luo (daniel.luo@interdigital.com) |
| Cross-component linear model | CCLM | JVET-L0085, JVET-L0136, JVET-L0191, JVET-L0338, JVET-L0340, | X | X | X | Roman Chernyak (chernyak.roman@huawei.com) | Shan Liu (shanl; leolzhao@ tencent.com) |
| multiple transform set | MTS | JVET-L0059, JVET-L0111, JVET-L0118, JVET-L0285, JVET-M0297, JVET-M0464, JVET-M0497 | X | X | X | Kiho Choi (kiho14.choi@samsung.com) | Shan Liu (shanl; xinzzhao@ tencent.com) |
| Adaptive loop filter | ALF | JVET-L0082, JVET-L0083, JVET-L0147, JVET-L0392, JVET-L0664, JVET-M0132 | X | X | X | Wei-Jung Chien (wchien@qti.qualcomm.com) | Daniel Luo (daniel.luo@interdigital.com) |
| Affine motion model | AFF | JVET-L0045, JVET-L0047, JVET-L0142, JVET-L0265, JVET-L0260, JVET-L0271, JVET-L0632, JVET-L0694, JVET-M0145, JVET-M0192 |  | X | X | Roman Chernyak (chernyak.roman@huawei.com) | Shan Liu (shanl; guichunli@ tencent.com) |
| subblock-based temporal merging candidates | SbTMVP | JVET-L0055, JVET-L0104, JVET-L0195, JVET-L0257, JVET-L0369, JVET-L0468, JVET-M0273 |  | X | X | Shan Liu (shanl; guichunli@ tencent.com) | Wei-Jung Chien (wchien@qti.qualcomm.com) |
| Adaptive motion vector resolution | AMVR | JVET-L0377, JVET-M0246 |  | X | X | Shan Liu (shanl; guichunli@ tencent.com) | Wei-Jung Chien (wchien@qti.qualcomm.com) |
| Pairwise merge candidate | PMC | JVET-L0090, JVET-M0193 |  | X | X | Tzu-Der Chuang (peter.chuang@mediatek.com) | Roman Chernyak (chernyak.roman@huawei.com) |
| Triangular partition mode | TPM | JVET-L0124, JVET-L0208, JVET-M0328, JVET-M0883 | X | X | X | Kiho Choi (kiho14.choi@samsung.com) | Shan Liu (shanl; leolzhao@ tencent.com) |
| Bi-directional optical flow | BDOF | JVET-L0256, JVET-M0487 |  | X | X | Kiho Choi (kiho14.choi@samsung.com) | Tzu-Der Chuang (peter.chuang@mediatek.com) |
| Combined intra/inter prediction | CIIP | JVET-L0100 |  | X | X | Kiho Choi (kiho14.choi@samsung.com) | Tzu-Der Chuang (peter.chuang@mediatek.com) |
| Merge with MVD | MMVD | JVET-L0054, JVET-M0068, JVET-M0171, JVET-M0255 |  | X | X | Kiho Choi (kiho14.choi@samsung.com) | Hyeongmun Jang (hm.jang@lge.com) |
| Bi-predictive with CU weights | BiCW | JVET-L0646, JVET-M0111, JVET-M0264 |  | X | X | Daniel Luo (daniel.luo@interdigital.com) | Tzu-Der Chuang (peter.chuang@mediatek.com) |
| Multi-reference line prediction | MRLP | JVET-L0283 | X | X | X | Shan Liu (shanl; leolzhao@ tencent.com) | Hyeongmun Jang (hm.jang@lge.com) |
| Intra block copy mode** | IBC | JVET-M0407, JVET-M0483 | X | X | X | Shan Liu (shanl; xiaozhongxu@ tencent.com) | Wei-Jung Chien (wchien@qti.qualcomm.com) |
| Intra sub-partitioning | ISP | JVET-M0102 | X | X | X | Roman Chernyak (chernyak.roman@huawei.com) | Hyeongmun Jang (hm.jang@lge.com) |
| Decoder motion vector refinement | DMVR | JVET-M0147 |  | X |  | Daniel Luo (daniel.luo@interdigital.com) | Roman Chernyak (chernyak.roman@huawei.com) |
| Sub-block transform | SBT | JVET-M0140 |  | X | X | Roman Chernyak (chernyak.roman@huawei.com) | Shan Liu (shanl; xinzzhao@ tencent.com) |
| Luma mapping with chroma scaling | LMCS | JVET-M0427 | X | X | X | Taoran Lu (tlu@dolby.com) | Hyeongmun Jang (hm.jang@lge.com) |
| CABAC | CABAC | JVET-M0453, JVET-M0725 | X | X | X | Yi-Wen Chen (yiwenchen@kwai.com) | Hyeongmun Jang (hm.jang@lge.com) |
| Symmetric motion vector difference | SMVD | JVET-M0444 |  | X | X | Yi-Wen Chen(yiwenchen@kwai.com) | Hyeongmun Jang (hm.jang@lge.com) |
| Transform skip mode | TSM | JVET-M0464 | X | X | X | Shan Liu (shanl; xinzzhao@ tencent.com) | Wei-Jung Chien (wchien@qti.qualcomm.com) |
| Shared merging candidate cist | SML | JVET-M0170 |  | X | X | Shan Liu (shanl; xlxiangli@ tencent.com) | Hyeongmun Jang (hm.jang@lge.com) |
| Shape-adaptive transform selection*** | SATS | JVET-M0303 | X | X | X | Shan Liu (shanl; xinzzhao@ tencent.com) | Kiho Choi (kiho14.choi@samsung.com) |

* Test was conducted by disabling DQ and enabling Sign Data Hiding

** Test was conducted by enabling IBC

*** Test was conducted by disabling MTS and enabling SATS

# Results

The results of the tests are summarized in Table 2 below. The attached spreadsheet provides additional data. Scatter plots are also provided for the tested tools in random access configuration, comparing PSNR-Y based bd-rate on the Y axis vs. each of Enc runtime ratio, Dec runtime ratio, and a weighted average of Enc and Dec runtime ratio, (Enc + a*Dec)/(a+1), with a configurable weight, a. The exemplary weighting is set to 6 and can be adjusted in the spreadsheet attached to this report.

Full experimental results and configuration files can be found at the link below:

https://hevc.hhi.fraunhofer.de/svn/svn_VVCTestConfig/branches/VTM-4.0/

There was no bitrate or PSNR differences between testers and cross-checkers.

Encoder and Decoder runtime ratios provided by both the testers and cross-checkers are included in the reporting template, to identify if there were significant runtime differences.

Table 3 Simulation results in all intra configuration (AI) of VTM tool tests. (VTM anchor)

|  |  |  |  | AI |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Acronym | BDR-Y | BDR-U | BDR-V | Tester EncTime | Tester DecTime | XChecker EncTime | XChecker DecTime |
| CST | 0.29% | 11.28% | 12.41% | 121% | 101% | 121% | 99% |
| DQ | 1.84% | 1.04% | 0.73% | 89% | 104% | 86% | 106% |
| CCLM | 2.08% | 19.05% | 19.23% | 99% | 100% | 99% | 99% |
| MTS | 1.95% | 1.39% | 1.39% | 63% | 96% | 63% | 95% |
| ALF | 2.25% | 2.96% | 3.17% | 100% | 88% | 100% | 89% |
| MRLP | 0.45% | 0.24% | 0.25% | 96% | 99% | 97% | 100% |
| CPR | -0.46% | -0.38% | -0.42% | 148% | 100% | 147% | 100% |
| SAO | 0.24% | 0.33% | 0.62% | 100% | 97% | 100% | 96% |
| ISP | 0.61% | 0.45% | 0.46% | 83% | 95% | 84% | 97% |
| ILRS | 0.87% | -1.17% | -1.00% | 101% | 100% | 100% | 100% |
| CABAC | 1.01% | 1.10% | 1.21% | 95% | 103% | 93% | 97% |
| TSM | 0.26% | 0.35% | 0.34% | 100% | 97% | 100% | 99% |
| SATS | -1.04% | -1.36% | -1.36% | 99% | 103% | 99% | 102% |

Table 4 Simulation results in random access configuration (RA) of VTM tool tests. (VTM anchor)

|  |  |  |  | RA |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Acronym | BDR-Y | BDR-U | BDR-V | Tester EncTime | Tester DecTime | XChecker EncTime | XChecker DecTime |
| CST | 0.16% | 4.90% | 6.35% | 102% | 99% | 101% | 99% |
| DQ | 1.57% | 0.78% | 0.25% | 99% | 103% | 96% | 102% |
| CCLM | 0.91% | 17.37% | 17.31% | 99% | 100% | 101% | 102% |
| MTS | 0.87% | 0.47% | 0.59% | 97% | 100% | 95% | 97% |
| ALF | 3.84% | 3.53% | 3.11% | 101% | 88% | 101% | 88% |
| AFF | 2.62% | 1.84% | 1.75% | 88% | 99% | 89% | 99% |
| SbTMVP | 0.46% | 0.27% | 0.30% | 100% | 100% | 101% | 100% |
| AMVR | 1.04% | 1.50% | 1.46% | 85% | 101% | 86% | 100% |
| PMC | 0.19% | 0.04% | 0.13% | 102% | 100% | 99% | 99% |
| TPM | 0.39% | 0.61% | 0.61% | 91% | 100% | 92% | 102% |
| BDOF | 0.75% | 0.15% | 0.10% | 96% | 94% | 98% | 93% |
| CIIP | 0.51% | 0.42% | 0.55% | 96% | 99% | 98% | 99% |
| MMVD | 0.53% | 0.48% | 0.52% | 89% | 101% | 88% | 100% |
| BPWA | 0.42% | 0.52% | 0.57% | 93% | 103% | 96% | 101% |
| MRLP | 0.23% | -0.06% | 0.00% | 99% | 100% | 99% | 100% |
| IBC | 0.01% | -0.07% | -0.05% | 99% | 99% | 99% | 100% |
| SAO | 0.57% | 0.87% | 0.92% | 101% | 99% | 100% | 99% |
| ISP | 0.26% | 0.29% | 0.05% | 97% | 98% | 99% | 100% |
| DMVR | 0.73% | 1.05% | 1.08% | 100% | 94% | 99% | 94% |
| SBT | 0.38% | 0.32% | -0.04% | 94% | 99% | 93% | 97% |
| ILRS | 1.38% | -2.46% | -2.37% | 93% | 98% | 94% | 99% |
| CABAC | 0.94% | 1.00% | 0.79% | 100% | 102% | 98% | 98% |
| SMVD | 0.26% | 0.24% | 0.24% | 97% | 104% | 96% | 100% |
| TSM | 0.18% | 0.03% | -0.06% | 98% | 99% | 98% | 99% |
| SML | -0.01% | -0.05% | -0.06% | 101% | 102% | 101% | 101% |
| SATS | -0.53% | -0.50% | -0.65% | 101% | 102% | 101% | 102% |

Table 5 Simulation results in low delay B configuration (LDB) of VTM tool tests. (VTM anchor)

|  |  |  |  | LDB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Acronym | BDR-Y | BDR-U | BDR-V | Tester EncTime | Tester DecTime | XChecker EncTime | XChecker DecTime |
| CST | 0.08% | 1.71% | 3.03% | 99% | 101% | 100% | 101% |
| DQ | 1.33% | 1.47% | 0.57% | 96% | 102% | 98% | 102% |
| CCLM | 0.05% | 4.63% | 5.56% | 100% | 99% | 100% | 104% |
| MTS | 0.24% | 0.58% | 0.21% | 100% | 100% | 98% | 97% |
| ALF | 2.56% | 3.33% | 3.03% | 103% | 93% | 100% | 90% |
| AFF | 2.07% | 1.31% | 0.98% | 84% | 96% | 85% | 96% |
| SbTMVP | 0.63% | 0.78% | 0.39% | 101% | 97% | 101% | 101% |
| AMVR | 0.38% | 0.66% | 0.95% | 91% | 102% | 92% | 102% |
| PMC | -0.02% | 0.08% | -0.22% | 99% | 100% | 100% | 100% |
| TPM | 0.95% | 1.34% | 1.27% | 88% | 101% | 88% | 104% |
| CIIP | 0.47% | 0.99% | 1.13% | 95% | 98% | 93% | 100% |
| MMVD | 0.45% | 0.42% | 0.27% | 92% | 100% | 90% | 101% |
| BPWA | 0.29% | 0.31% | 0.23% | 97% | 100% | 97% | 100% |
| MRLP | 0.09% | 0.36% | 0.09% | 98% | 100% | 100% | 101% |
| IBC | 0.05% | 0.24% | -0.05% | 105% | 99% | 103% | 98% |
| SAO | 1.24% | 2.48% | 3.92% | 102% | 97% | 101% | 99% |
| ISP | 0.13% | 0.35% | -0.20% | 99% | 99% | 100% | 101% |
| SBT | 0.66% | 1.22% | -0.20% | 92% | 99% | 90% | 95% |
| ILRS | 0.76% | -1.71% | -2.10% | 93% | 96% | 92% | 98% |
| CABAC | 0.88% | 0.55% | 0.16% | 99% | 100% | 97% | 101% |
| TSM | 0.11% | 0.07% | 0.17% | 97% | 97% | 98% | 102% |
| SML | -0.01% | 0.05% | -0.04% | 103% | 105% | 100% | 100% |
| SATS | -0.13% | -0.01% | 0.00% | 100% | 103% | 102% | 102% |

Table 9 Pixel usage and memory bandwidth results of VTM tool “off” test. (VTM anchor)

|  | AI |  | RA |  |  | LDB |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Acronym | Pixel usage | Pixel usage | Ave mem BW | Max mem BW | Pixel usage | Ave mem BW | Max mem BW |
| CCLM | 50.87% | 3.34% |  |  | 0.81% |  |  |
| ALF | 99.00% | 55.66% |  |  | 44.87% |  |  |
| AFF |  | 20.40% |  |  | 26.99% |  |  |
| SBTMC |  | 12.34% |  |  | 13.81% |  |  |
| AMVR |  | 4.91% |  |  | 2.12% |  |  |
| TAP |  | 2.52% |  |  | 6.02% |  |  |
| BDOF |  | 43.80% |  |  |  |  |  |
| CIIP |  | 1.30% |  |  | 1.65% |  |  |
| MMVD |  | 8.70% |  |  | 9.12% |  |  |
| BPWA |  | 10.17% |  |  | 8.54% |  |  |
| MRLP | 6.41% | 0.65% |  |  | 0.31% |  |  |
| SAO | 54.71% | 11.13% |  |  | 18.41% |  |  |
| DMVR |  | 40.21% |  |  |  |  |  |
| SBT |  | 2.53% |  |  | 4.15% |  |  |
| SMVD |  | 3.00% |  |  |  |  |  |
| TSM | 1.61% | 0.58% |  |  | 1.01% |  |  |

Figure 1. PSNR-Y vs encoding runtime ratio of VTM with VTM tool tests (VTM anchor)

Figure 2. PSNR-Y vs decoding runtime ratio of VTM with VTM tool tests (VTM anchor)

Figure 3. PSNR-Y vs weighted runtime ratio (a = 6) of VTM with VTM tool tests (VTM anchor)

# Related contributions

| JVET-N0123 | AHG13/Non-CE6/CE8: Chroma Transform Skip | T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki (Sony) |
| --- | --- | --- |

Additional tests on transform skip mode (TSM) were conducted in the contribution.

- TSM: Disable transform skip on top of VTM-4.0 (AhG13 Test, JVET-M0464 enabled)

- TSM1: Restrict transform skip size up to 4x4 on top of VTM-4.0 (JVET-M0464 enabled)

  - Encoder option: --MTS=1 --TransformSkipLog2MaxSize=2

  - Chroma Transform skip: N/A

  - Configuration for TSM_1 are found in config_TSM_1.

- TSM2: Restrict transform skip size up to 4x4 on top of VTM-4.0 with JVET-M0464 disabled (including ticket#185)

  - Encoder option: --EMT=1 --EMTFast=1 --TransformSkipLog2MaxSize=2 with JVET-M0464 macro disabled

  - Chroma Transform skip: 2x2/4x2/2x4/4x4 can be used

  - Configuration and source changes for TSM_2 are found in config_TSM_2.

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over TM- 4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| TSM | 0.18% | 0.03% | -0.06% | 98% | 99% |
| TSM1 | 0.17% | -0.07% | -0.07% | 100% | 100% |
| TSM2 | 0.00% | -0.10% | -0.42% | 104% | 101% |

| JVET-N0238 | AHG13: ISP Tool-off Tests in VTM-4.0.1 | S. De-Luxán-Hernández, B. Bross, H. Schwarz, D. Marpe, T. Wiegand (HHI) |
| --- | --- | --- |

Additional tests on intra sub-partitioning (ISP) were conducted in the contribution.

- ISP: Disable ISP on top of VTM-4.0 (AhG13 Test)

  - Anchor: VTM-4.0 with ISPFast set to 0.

- ISP1: Disable ISP on top of VTM-4.0.1

  - Anchor: VTM-4.0.1 with ISPFast set to 0.

- ISP2: Disable ISP on top of VTM-4.0.1

  - Anchor: VTM-4.0.1 with ISPFast set to 1.

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over TM- 4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| ISP | 0.61% | 0.45% | 0.46% | 83% | 95% |
| ISP1 | 0.61% | 0.45% | 0.46% | 83% | 96% |
| ISP2 | 0.58% | 0.44% | 0.43% | 87% | 96% |

# Recommendations

The AHG recommends the following:

- Consider the reported tool test results during tool adoption decision making

- Review related contributions

- Refine list of tested tools and test methodology for the next meeting cycle

  - Consider the reported tool test results as a benchmark for CE tests

  - Consider including reporting of compute system information for testers and cross-checkers

  - Consider additional performance or complexity metrics