| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0014-v2 |
| --- | --- |

| Title: | JVET AHG report: Progressive intra refresh (AHG14) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jean-Marc Thiesse Alberto Duenas Kimihiko Kazui Rickard Sjöberg Alexandros Tourapis | Tel: Email: | jean-marc.thiesse@vitec.com Alberto.Duenas@arm.com kazui.kimihiko@jp.fujitsu.com rickard.sjoberg@ericsson.com atourapis@apple.com |
| Source: | AhG14 on Progressive intra refresh |  |  |

_____________________________

# Abstract

This document summarizes activities of AhG on Progressive intra refresh between the 13th and the 14th JVET meetings.

# Mandates

# The mandates of AHG14 are as follows:

- Define relevant test conditions for the study of progressive intra refresh for random access without intra frames

- Update the implementation of encoder-only intra refresh in the VTM model in the AHG14 fork of the software repository.

- Evaluate different ways to produce intra refresh within VVC and characterize their coding efficiency impact, subjective quality, and delay characteristics, including encoder-only approaches and normative approaches

- Consider the use of constrained intra prediction and tile-based approaches

- Study recovery point handling, including practical implementation issues and perfect-versus-approximate decoded picture recovery.

- Consider the potential need for starting a coded video sequence without an intra picture.

# Activities

There were several mail exchanges on the main JVET email reflector (jvet@lists.rwth-aachen.de) with an [AHG14] indication on message headers. Three topics have been discussed on these mails:

- First, the relevance of intra refresh for low delay has been discussed. Thanks to the early sharing of a latency study by Huawei, the appropriateness of intra refresh was demonstrated without or together with DU.

- Second, there were emails regarding the opportunity of having normative changes to support intra refresh. One expert indicates that the potential additional checks for the decoder may be complex to support and this should be justified by a very high performance benefit threshold. Another expert indicated a use-case where having a smaller latency could be relevant as well as having a granularity to balance between latency and coding efficiency.

Another expert explains that practical implementation using intra refresh are complex when handling of entertainment/broadcasting contents occurs. In particular, scene changes and Commercial insertion have been mentioned. Also, this expert and other expert insist on the complexity due to camera motions which are not in the same direction than the intra refresh scanning.

- Third, there was a discussion on modifying the test condition which leads to the following conditions being agreed and proposed:

•	Basis = Low-Delay B Main10 configuration.

•	Do not use a pyramidal B structure (=> GOPSize = 1) in order to have all pictures with a somewhat similar bits sizes.

•	Use a QP offset of +1 between the first I and the P picture.

•	Set the intra refresh period equal to 1 second, meaning that the stream shall be refreshed after a cycle of 1 second (can be less).

•	PSNR and bitrates should be computed without the first I picture + the first cycle minus 1 (meaning that the first 30 pictures are not considered in 30Hz, 60 in 60Hz…)

An updated version of the encoder only implementation has been shared on the AHG 14 Gitlab (https://vcgit.hhi.fraunhofer.de/jvet-l-ahg-14/VVCSoftware_VTM/tree/AHG14-IntraRefresh_EncOnly).

# Related contributions

The following contributions are identified for the AHG.

- Contribution on general aspects

- JVET-N0114, “AHG14: A delay analysis for IRAP and GDR”, Hendry, Y.-K. Wang, M. Sychev (Huawei)

- Contributions on normative intra refresh handling including software

- JVET-N0080, “AHG14: Revised software for ultra low-latency encoding”, K. Kazui(Fujitsu)

- JVET-N0391, “AHG14: Updates on intra refresh proposal”, J.-M. Thiesse, D. Gommelet, D. Nicholson (VITEC)

- Contributions on normative intra refresh handling

- JVET-N0115, “AHG14/AHG17: GDR - gradual decoding refresh”, Hendry, Y.-K. Wang, J. Chen, S. Hong (Huawei)

- JVET-N0116, “AHG14/AHG17: DDR - distributed decoding refresh”, Y.-K. Wang, M. Sychev, Hendry (Huawei)

- JVET-N0275, “AHG14/AHG8/AHG17: Sections for intra refresh and inloop filter disabling”, J. Boyce, R. Lei, L. Xu

- JVET-N0310, “AHG14: On gradual decoding refresh”, M. M. Hannuksela, K. Kammachi-Sreedhar, A. Aminlou (Nokia)

- Contributions on clean random access in case of intra refresh

- JVET-N0101, “AHG14/ AHG17: On Gradual and Clean Random Access”, S. Deshpande (Sharp)

- JVET-N0495, “AHG14: Recovery point indication NAL unit”, R. Sjöberg, M. Pettersson, M. Damghanian (Ericsson)

# Recommendations

The AhG recommends:

- To review all related contributions.

- To integrate the encoder-only modifications into the next VTM software, update the anchor appropriately, and to continue investigating the benefits of normative progressive intra refresh solutions.

- To consider updating the common test conditions for integrating the intra refresh scenario.