| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0016-v1 |
| --- | --- |

| Title: | JVET AHG report: Implementation studies (AHG16) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | M. Zhou, J. An, E. Chai, K. Choi, S. Sethuraman, T. Hsieh, X. Xiu | Email: | minhua.zhou@broadcom.com jicheng.ajc@alibaba-inc.com eric.chai@ublnx.com kiho14.choi@samsung.com sriram.sethuraman@ittiam.com thsieh@qti.qualcomm.com xiaoyuxiu@kwai.com |
| Source: | AHG16 |  |  |

_____________________________

# Abstract

This document summarizes the activity of AHG16: implementation studies, between the 13th JVET meeting in Marrakesh, MA (9–18 January 2019) and the 14th JVET meeting in Geneva, CH (19–27 March 2019).

The AHG was established with the following mandates:

- Study draft and proposed coding tools to identify implementation issues relating to decoder pipelines, decoder throughput, and other aspects of implementation difficulty.

- Solicit hardware analysis of complex tools.

- Particularly consider intra reconstruction throughput for small blocks.

- Provide feedback on potential solutions to address identified issues.

# Activities

There were not many email exchanges on the main JVET email reflector (jvet@lists.rwth-aachen.de) with an [AHG16] indication on message headers. A summary of the AHG activities is provided as follows:

## Comments posted on the email reflector before the finalization of CE descriptions

- Small block size (e.g. 2x2) chroma intra-prediction is a known issue when the QTBTT was adopted. Processing four times of intra chroma blocks (compared to 4x4 minimum chroma block size) alone is not something affordable for high performance VVC decoder designs. The CCLM has made the situation worse because the luma and chroma reconstruction can no longer be made parallel.  It is desirable to have a consistent solution to resolve this issue. Potential solutions for the separate tree case were proposed at the last meeting; more difficult might be to find a solution for the combined tree case.

- Memory bandwidth remains an issue to be resolved for motion compensation of both the regular inter prediction mode and the affine mode. For the regular inter prediction mode, the HEVC design (i.e. bi-pred 8x8, uni-pred 8x4/4x8) might be a straightforward solution if no better options can be found. For the affine mode where 4x4 sub-block size is used for motion compensation, designs should consider not only the constraint of the worst case memory bandwidth consumption and the worst number of reference block requests out of the cache memory, but also the computational complexity reduction of the 4x4 block based motion compensation process.

- Separate luma/chroma partitioning tree introduces a long latency for chroma intra prediction when the CCLM is on. In the worst case, the chroma intra prediction cannot be kicked off until the whole 64x64 luma block is reconstructed (e.g. when a 64x64 luma block is split horizontally while the chroma blocks are split vertically, and vice versa). It might be interesting to see if the CCLM can be disabled for 4x4 blocks without harming coding efficiency.

- Having hard coded limits on the maximum number of context coded bins per 4x4 CG does not appear to be useful for implementation, replacing them with implicit limits is preferable as it simplifies design and avoids potential quality issues.

- The quantization noise can increase the dynamic range of inverse transform output. It is recommended to clip the output of the non-separable secondary inverse transforms (NSST) 16-bit range before fed into the primary inverse transforms, to avoid potential impact on the primary inverse transforms in terms of the intermediate precisions and intermediate transform buffer size.

- Sub-partitions of 1xN and 2xN in the newly adopted sub-partition intra prediction create implementation difficulties; it is desirable to remove those elements from the current design.

- Intra Block Copy (IBC) is now treated a separate mode in VTM4.0. The separate IBC merge/AMVP list derivation is down to 4x4 block level, the IBC merge list derivation should be simplified significantly to meet the throughput requirement. Moreover, the current IBC design is optimized for 128x128 CTU size only in terms of the local buffer usage (~23 Kbytes). If the CTU size is reduced to 64x64 for instance, only ¼ of the 23 Kbytes local memory gets used and the rest of local buffer gets idle. This should be fixed.

- It is desirable to split the large block size local illumination compensation (LIC) into multiple of 16x16 blocks (similar to what has been done for the DMVR), to minimize the buffer usage and processing overhead.  In the current design of inter prediction loop, the motion compensation, DMVR and BDOF can all be processed on a 16x16 block basis. This additional LIC variant is tested in CE1.

- It might be more straightforward to design a de-blocking filter which operates on 4x4 grids and uses a line buffer of e.g. 6 luma lines (increase the line buffer size for chroma accordingly if it helps), to minimize the number of conditional checks and avoid inconsistent operations along different edges (e.g. inside the CTU and along the top CTU boundary).

## Other feedbacks provided to the proponents

- For palette mode suggested the following changes to reduce the buffer size and improve the throughput:

  - Limit the number of palette_index_idc to a maximum (e.g. 256). For every 256 index runs (or some other number), interleave palette_index_idc and copy_above_palette_index_flag/palette_run. In which case the palette_index_idc only need to be buffered 256 entries.

  - And signal the quantized sample values immediately after the “escape code” to avoid multiple-pass CU block reconstruction.

  - Restrict the palette size especially for small size PUs to constrain the worst case bin rate.

  - Fix the maximum palette size for the standard (currently it is signalled in SPS).

- For newly adopted luma in-loop reshaping (aka DRA), suggested the following changes to reduce the implementation cost and latency:

  - Avoid computing the luma average value at TU level to reduce the latency. In the current design, the chroma residual scaling cannot take place until the whole TU size of the luma prediction block is generated and the average is computed, resulting in high latency in the chroma reconstruction even in the shared tree case for large TUs (e.g. 64x64).

  - Avoid storing the chroma residual in high-precision (currently 16-bit) for the chroma residual scaling purpose. (roughly 2.5 Kbytes local buffer saving if it is reduced to 11-bit for 10-bit video).

  - Signal the temporal domain reshaping parameters using picture-level parameter sets to avoid potential negative impact on “trick mode”.

- For newly adopted intra sub-partition prediction (ISP), identified that the current design breaks the 64x64 decoder pipeline, because for e.g. 128x64 or 64x128 CUs, the luma part is split but the chroma part is not, which creates 64x32 or 64x32 size chroma TUs. For the 64x64 block based decoder pipeline, the chroma TU size can only be up to 32x32. This was communicated with the proponents.

- For Intra Block Copy (IBC), suggested to disallow the IBC local copy from beyond the most recently decoded three (preceding) VDPUs and the already decoded area of the current VDPU.  It is asserted that reading and writing data from the same memory is not desirable, as it may create a race condition in terms of memory access.  It is also desirable to add assertions to the decoder side to make sure that the IBC copies the data from the valid reference area.

Fig. 1: The first CU of the current VDPU (i.e. VDPU7) can copy from VDPU3 in the current IBC design

## Memory bandwidth study for VTM4.0

Broadcom conducted a memory bandwidth study by running both the VTM4.0 and VTM3.0 for CTC with a commercial motion compensation cache model integrated. The summary results of the random access configuration are provided in the Table 1 (for informational purposes).

The DMVR is the tool which may cause the difference in memory bandwidth consumption between the VTM4.0 and VTM3.0.  For RA configuration, this tool appears to increase the average memory bandwidth consumption by roughly 3% (see ABW_diff column) relative to VTM3.0, but the increase in the peak memory bandwidth (see MBW_diff column) is very limited (roughly 1% in class A1).

|  | Random Access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VTM4.0 Over VTM-3.0 |  |  |  |  |  |  |  |
|  | Y | U | V | TCM_diff | ABW_diff | MBW_diff | EncT | DecT |
| Class A1 | -5.44% | 6.42% | 3.71% | 2.77% | 2.82% | 0.97% | 155% | 100% |
| Class A2 | -6.85% | 5.56% | 6.23% | 2.43% | 2.42% | 0.00% | 146% | 90% |
| Class B | -5.06% | 6.08% | 5.41% | 1.48% | 1.47% | -1.10% | 151% | 100% |
| Class C | -5.14% | 6.98% | 7.68% | 2.66% | 2.81% | 0.23% | 155% | 112% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -5.52% | 6.28% | 5.84% | 3.12% | 3.34% | 3.19% | 152% | 101% |
| Class D | -4.91% | 6.22% | 6.88% | 3.68% | 3.81% | 3.19% | 165% | 114% |
| Class F (optional) | -17.34% | -9.43% | -9.17% | 5.71% | 6.70% | 0.63% | 149% | 113% |

Table 1. Memory bandwidth comparison (VTM4.0 vs.VTM3.0, RA)

Where

- TCM_diff : Total cache misses (over all the frames coded), percentage increase relative to VTM3.0.

- ABW_diff: Average memory bandwidth (over all the frames coded), percentage increase relative to VTM3.0.

- MBW_diff : Worst case (Max) memory bandwidth (among all the frames coded), percentage increase relative to VTM3.0.

MBW_diff is the most important measure, which shows the percentage of the worst case memory bandwidth increase of VTM4.0 relative to VTM3.0.

## Bin to bit ratio in VTM4.0 and HM16.19

It has been observed that the bin to bit ratio in VTM4.0 is double digits percentage higher than that of HM16.19 for CTC. The summary results of the RA configuration are provided in Table 2 for informational purposes (data from JVET-N0049).

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over HM16.19 |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Bin2bit ratio (peak, weighted) |  |  | Bin2bit ratio (peak, unweighted) |  |  |
|  | Y | U | V | Y | U | V | HM16.19 | VTM4.0 | Diff (%) | HM16.19 | VTM4.0 | Diff (%) |
| Class A1 | -32.65% | -39.10% | -42.07% | -22.00% | -29.31% | -32.78% | 0.94 | 1.13 | 20.49% | 1.22 | 1.43 | 16.67% |
| Class A2 | -37.43% | -35.62% | -29.96% | -28.45% | -25.82% | -19.33% | 0.95 | 1.13 | 17.94% | 1.23 | 1.42 | 15.18% |
| Class B | -31.31% | -36.99% | -38.14% | -21.90% | -27.91% | -29.16% | 0.93 | 1.08 | 15.90% | 1.22 | 1.37 | 12.32% |
| Class C | -26.50% | -27.38% | -29.19% | -14.87% | -15.71% | -17.78% | 0.88 | 1.03 | 16.42% | 1.18 | 1.33 | 12.20% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -31.52% | -34.58% | -34.90% | -21.36% | -24.52% | -24.88% | 0.93 | 1.09 | 17.39% | 1.21 | 1.38 | 13.75% |
| Class D | -25.61% | -25.09% | -26.33% | -15.20% | -14.46% | -15.91% | 0.86 | 1.02 | 18.44% | 1.17 | 1.32 | 13.32% |
| Class F (optional) | -36.12% | -38.21% | -39.22% | -22.29% | -24.45% | -25.49% | 0.84 | 1.03 | 23.01% | 1.16 | 1.31 | 12.98% |

Table 2. BD-binrate and bin to bit ratio (VTM4.0 vs.VTM3.0,)

For a given minimum compression ratio (MinCR) of a level, a higher bin to bit ratio implies that a CABAC engine needs to process bins at a faster pace in order to deal with the peak bit-rate in real time.

In Table 2, a bypass bin is counted as 0.25 context coded bins in the weighted measurements; and a bypass bin and a context coded bin carry an equal weight (1:1) in the unweighted measurements.

# Related contributions

The following contributions are identified for the AHG.

- Contributions relating to the small block size chroma intra prediction

  - JVET-N0453, “AHG16/Non-CE3: on chroma 2xN and Nx2 intra prediction in single tree”, Y. Zhao, A. Karabutov, H. Yang, J. Chen (Huawei)

  - JVET-N0465, “CE3-related: Restrict 2xN, Nx2 chroma processing for dual tree structure”, H. Jang, J. Nam, N. Park, J. Lim, S. Kim (LGE)

  - JVET-N0396, “CE3-related: Enabling parallel reconstruction of small intra-coded blocks’, L. Pham Van, G. Van der Auwera, A. K. Ramasubramonian, V. Seregin, H. Huang, M. Karczewicz (Qualcomm)

- Contribution relating to the memory bandwidth reduction

  - JVET-N0397, “AHG16: Memory bandwidth reduction for small CUs”, X. Li, G. Li, X. Xu, S. Liu (Tencent)

- Contributions aiming for simplifications and cleanups of the in-loop luma reshaper

  - JVET-N0069, “AHG16/AHG17: Proposed Cleanup for Reshaper High Level Syntax”, B. Heng, M. Zhou, W. Wan (Broadcom)

  - JVET-N0117, “AHG16/AHG17: Signalling of reshaper parameters in APS”, Y.-K. Wang, Hendry, J. Chen (Huawei), P. Yin, T. Lu, F. Pu, S. McCarthy (Dolby)

  - JVET-N0113, “AHG16: Subblock-based chroma residual scaling”, Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)

  - JVET-N0220, “AHG16: Simplification of Reshaper Implementation”, T. Lu, F. Pu, P. Yin, S. McCarthy, W. Husak, T. Chen(Dolby)

  - JVET-N0389, “Chroma residual scaling with separate luma/chroma tree”, E. Francois, C. Chevance (Technicolor)

- Contributions aiming for addressing the 64x64 decoder pipeline and 1xN/2xN sub-partitions issues of the intra sub-partition prediction (ISP)

  - JVET-N0308, “AHG16/Non-CE3: Restriction of the maximum CU size for ISP to 64x64”, S. De-Luxan-Hernandez, B. Bross, T. Nguyen, V. George, B. Stabernack, H. Schwarz, D. Marpe, T. Wiegand (HHI)

  - JVET-N0469, “CE3-related : Disable ISP mode for 128xN, Nx128 Block”, 
H. Jang, J. Heo, J. Nam, J. Lim, S. Kim (LGE)

  - JVET-N0372: “Non-CE3: ISP with independent sub-partitions for certain block sizes”, S. De-Luxán-Hernández, B. Bross, T. Nguyen, V. George, B. Stabernack, H. Schwarz, D. Marpe, T. Wiegand (HHI)

  - JVET-N0432, “AHG16/Non-CE3: On 1xN and 2xN subpartitions in intra subpartition coding”, A. K. Ramasubramonian, G. Van der Auwera, T. Hsieh, V. Seregin, L. Pham Van, M. Karczewicz

- Contributions aiming for reducing the chroma reconstruction latency in the separate tree case

  - JVET-N0376, “AHG16/CE3-related: CCLM mode restriction for increasing decoder throughput”, J.-Y. Jung, Y.-L. Lee (Sejong Univ.), D.-Y. Kim, W. J. Jeong (Chips&Media), S.-C. Lim, J. Kang (ETRI)

  - JVET-N0390, “AHG16/non-CE3: Study of CCLM restrictions in case of separate luma/chroma tree”, E. Francois, F. Galpin, F. Le LÃ©annec, T. Poirier (Technicolor)

- Contributions aiming for addressing the IBC related issues

- JVET-N0383, “AHG16/Non-CE8: IBC search range adjustment for implementation consideration”, X. Xu, X. Li, S. Liu (Tencent)

- JVET-N0384, “Non-CE8: IBC search range increase for small CTU sizes”, X. Xu, X. Li, S. Liu (Tencent)

- JVET-N0202, “AHG16/Non-CE8: Report on conformance check failures of IBC block vectors”, J. Li, C.-W. Kuo, C. S.Lim (Panasonic)

- JVET-N0201, “Non-CE8: IBC modifications”, J. Li, C.-W. Kuo, C. S.Lim (Panasonic)

- Contribution relating to cleanups for the reference wraparound

  - JVET-N0070, “AHG16/AHG8: Proposed Cleanup for Reference Wraparound”, B. Heng, M. Zhou, W. Wan (Broadcom)

- Contribution relating to CABAC bin to bit ratio study

- JVET-N0049, “AHG16/Non-CE7: A study of bin to bit ratio in VTM4.0 and HM16.19”, M. Zhou (Broadcom)

- Contribution relating to MPM list derivation

- JVET-N0183, “AHG16/Non-CE3: Shared MPM list”, A. M. Kotra, S. Esenlik, J. Chen, B. Wang, H. Gao (Huawei)

# Recommendations

The AHG recommends reviewing the input contributions.