| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0019 |
| --- | --- |

| Title: | JVET AHG report: Layered coding and resolution adaptivity (AHG19) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | S. Wenger A. Segall M. M. Hannuksela Hendry S. McCarthy Y.-C. Sun | Email: | stewe@stewe.org asegall@sharplabs.com miska.hannuksela@nokia.com fnu.hendry@huawei.com sean.mccarthy@dolby.com yuchen.s@alibaba-inc.com |
| Source: | AHG19 |  |  |

_____________________________

# Abstract

AHG19 was established by the 13th meeting (Marrakech) to study layered coding and resolution adaptivity.  Little reflector activity took place; besides the kickoff message, JVET chair Gary Sullivan summarized certain activities during the Marrakech meeting and requested information on planned documents for the 14th (Geneva) meeting, and two organizations answered in public.  Ten documents were submitted to JVET that include the AHG19 marker in their respective titles.  The AHG recommends that those contributions be studied during the meeting.

# Establishment and Mandates

At the 13th (Marrkech) meeting, the Ad-Hoc Group on Layered coding and resolution adaptivity (AHG19) was established as shown below.  The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de).

| Layered coding and resolution adaptivity (AHG19) (jvet@lists.rwth-aachen.de) - Study adaptive-resolution coding approaches for real-time communication, adaptive streaming, and 360-degree viewport-dependent streaming, including filters for resampling, reference picture management, and related scope and signalling - Study approaches for temporal scalability to avoid temporal judder when temporal scalability sub-bitstream extraction is used for achieving lower frame rate, and consider whether this should have a normative impact. - Identify related test conditions, test sequences, and evaluation techniques (including subjective assessment techniques) - Study potential approaches for support of layered coding scalability including spatial, temporal, quality, and view scalability | S. Wenger and A. Segall (co-chairs), M. M. Hannuksela, Hendry, S. McCarthy, Y.-C. Sun (vice-chairs) | N |
| --- | --- | --- |

# AHG History

AHG19 as established in Marrakech was the first ad hoc group in JVET focussing specifically on Layered coding and resolution adaptivity; insofar, there is no paper trail to analyze with respect to ad hoc group activity.

# AHG Activity

There was only limited activity of the AHG between the Marrakech and the Geneva meeting.  A total of four emails included the [AHG19] marker.  Besides the kick-off message, JVET chair Gary Sullivan summarized certain activities related to the AHG’s subject matter during the Marrakech meeting, and requested information on planned documents for the 14th (Geneva) meeting.  Per this request Alibaba/Huawei responded that they are working towards a contribution on the subject of ARC for the Geneva meeting.  Similarly, the University for Science and Technology, China (USTC) suggested that they “hopefully” will contribute, but only to the July meeting.

# Documents submitted

The documents below include the [AHG19] marker in their titles.  No other documents were identified that directly relate to the AHG’s mandates, though of course many High-Level Syntax related documents have implications to a potential ALS and scalable design.  For example, various contributions suggest reducing the NAL unit header to 8 bits and removing the reserved bits that, in HEVC, have been used as a layer indication.  As another example, a contribution suggests HRD parameters for CPB removal times for temporal scalability.

JVET-N0048 AHG19: On adaptive resolution changing [M. M. Hannuksela, A. Aminlou (Nokia)]

This contribution proposes an adaptive resolution coding approach and includes the following aspects:

- Signalling:

- Add sps_max_rpr, which indicates a maximum number of reference pictures to be resampled per any picture to be decoded, in the SPS. When equal to 0, reference picture resampling is not used.

- Move pic_width_in_luma_samples and pic_height_in_luma_samples from SPS to PPS.

- Include max_width_in_luma_samples and max_height_in_luma_samples in DPS or SPS or both. They do not affect the decoding process but only provide the maximum value for pic_width_in_luma_samples and pic_height_in_luma_samples, respectively.

- High-level decoding process:

- Invoke a reference picture resampling process implicitly for the active reference picture(s) that have a different width and height from those of the current picture.

- The output of the resampling process is stored in the DPB and inherits the reference picture marking status from the reference picture used as input to the resampling process.

- The reference picture used as input to the resampling process is marked as "unused for reference".

- Resampling:

- SHVC resampling process (HEVC clause H.8.1.4.2), with an addition of selection of input samples for filtering in wraparound manner (for 360° video, particularly ERP).

JVET-N0052 [AHG19] On Signaling of Adaptive Resolution Change [S. Wenger, B. Choi, S. Liu (Tencent)]

Proposed is the inclusion of Adaptive Resolution Change (ARC) into the VVC working draft. Text is provided with focus on the signalling aspect.  Simulation results are provided for Class A sequences, random access, with the IDR picture subsampled by a factor of two in both dimensions.  For some class A1 sequences, substantial luma gains are reported, with substantial chroma loss.  For class A2 sequences, there are substantial losses.  ARC is proposed as an architectural enhancement to improve reconstructed video quality when the codec is operated nears its break point, where artifacts become clearly visible.

JVET-N0061 AHG19: Signalling temporal IDs and levels for HFR backwards compatible bitstreams [V. Drugeon (Panasonic)]

An issue about signalling of maximum temporal ID and level of sub-bitstreams is reported when using temporal scalability to enable backwards compatibility of High Frame Rate (HFR) bitstreams to standard frame rate receivers. Two alternative solutions are suggested that both consists in signaling the number of frame rate versions of the content within the bitstream, together with the maximum temporal ID value and level for each frame rate version. The first solution signals the necessary information within the profile_tier_level syntax structure. The second solution signals the necessary information at the beginning of the Sequence Parameter Set and includes one profile_tier_level syntax structure for each frame rate version of the content.

JVET-N0062 AHG19: Signalling HRD models for HFR backwards compatible bitstreams [V. Drugeon (Panasonic)]

An issue about decoding time stamps is reported when using temporal scalability to enable backwards compatibility of High Frame Rate (HFR) bitstreams to standard frame rate receivers. It is suggested to signal CPB removal delays and DPB output delays for each temporal sub-layer contained in the bitstream to solve this issue.

JVET-N0118 AHG19: Adaptive resolution change (ARC) support in VVC [Hendry, S. Hong, Y.-K. Wang, J. Chen (Huawei), Y.-C Sun, T.-S Chang, J. Lou (Alibaba)]

This contribution reports a software implementation effort for supporting the so-called adaptive resolution change (ARC) feature on top of the VTM-4.0.1 reference software. Not all aspects of the design currently used in the implementation are intended to be the final design of ARC supported in VVC, rather, the implementation is intended as a testbed to help the study of related aspects of ARC. As the development of ARC progresses and more ARC design aspects are agreed by the JVET experts, the implementation is expected to be updated accordingly. It is recommended that the software is used as the reference software for the ARC work in AHG19.

In addition to the reference software effort, this contribution also discusses and proposes several aspects of ARC design.

The following aspects are proposed:

- A list of picture resolutions is signalled in the SPS, and an index to the list is signalled in the PPS to specify the size of an individual picture.

- For any picture that is to be outputted, the decoded picture before resampling is cropped (as necessary) and outputted, i.e., a resampled picture is not for output, only for inter prediction reference.

- Support 1.5x and 2x resampling ratios. No support of arbitrary resampling ratio. Further study the need of one or two more other resampling ratios.

- Between picture-level resampling and block-level resampling, the proponents prefer block-level resampling.

  - However, if picture-level resampling is chosen, the following aspects are proposed:

    - When a reference picture is resampled, both the resampled version and the original, resampled version of the reference picture are stored in the DPB, and thus both would affect the DPB fullness.

    - A resampled reference picture is marked as "unused for reference" when the corresponding un-resampled reference picture is marked as "unused for reference".

    - The RPL signalling syntax is kept unchanged, while the RPL construction process is modified as follows: When a reference picture needs to be included into a RPL entry, and a version of that reference picture with the same resolution as the current picture is not in the DPB, the picture resampling process is invoked and the resampled version of that reference picture is included into the RPL entry.

    - The number of resampled reference pictures that may be present in the DPB should be limited, e.g., to be less than or equal to 2.

  - Otherwise (block-level resampling is chosen), the following are suggested:

    - To limit the worst-case decoder complexity, it is proposed that bi-prediction of a block from a reference picture with a different resolution than the current picture is disallowed.

    - Another option is that, when resampling and quarter-pel interpolation need to be done, the two filters are combined and the operation is applied at once.

- Regardless of which of the picture-based and block-based resampling approaches is chosen, it is proposed that temporal motion vector scaling is applied as needed.

- Start a CE on ARC resampling filters.

JVET-N0279 AHG19: Adaptive resolution change [Peisong Chen, Tim Hellman, Brian Heng, Wade Wan, Minhua Zhou (Broadcom)]

This contribution proposes a method to support adaptive resolution change within a coded video sequence. It includes the signalling of adaptive resolution change in parameter sets and modifications to the current motion compensated prediction process in the VTM when there is a resolution change between a current picture and its reference pictures. These modifications are limited to motion vector scaling and subpel location derivations. There are no changes in the existing motion compensation interpolators with the asserted benefits of the proposed solution that these interpolators can be reused and does not introduce new processing blocks. The contribution also provides recommendations on constraining the interaction of certain coding tools with adaptive resolution changes as they are asserted to add additional complexity and memory bandwidth.

JVET-N0422 AHG19: Support of adaptive resolution change (ARC) in VVC [H. Wang, N. Hu, M. Coban, W.-J. Chien, V. Seregin, M. Karczewicz (Qualcomm)]

In this document a basic adaptive resolution change scheme allowing seamless switching between different resolutions without requiring IDR pictures at switching points is described. Picture prediction across different resolutions is supported by normative upsampling with filtering and simple subsampling of reference pictures along with scaling of motion information across different resolutions. Proposed method supports 1:2 and 2:1 resolution change in horizontal and vertical dimensions. The software for proposed scheme is implemented on top of VTM-4.0 SW.

JVET-N0528 AHG12/AHG19/SYS-VVC: Sub-picture decoding design: a comparison of proposed solutions [Emmanuel Thomas, TNO]

This contribution compares two proposed approaches to support decoding of independent coded regions which are 1) using tile groups as independent regions or 2) using a layered mechanism where each layer consists of a sub-picture. This contribution does not discuss the syntax details of any of these approaches but rather the technical merits of both approaches in addressing the need of real-world immersive applications.

It is asserted that the layered mechanism offers decisive advantages over the tile group-based approach when considering real-word applications. The simplicity of the layered approach which does not require inspection nor rewriting of parameters in the payload of VCL NAL units constitutes a significant advantage for the success of VCC in immersive application space.

Therefore, it is recommended to adopt the layered mechanism for supporting the decoding of independent regions. In addition, it is suggested to document at this meeting the use of this new mechanism in the context of viewport-based 360 video streaming which is currently an application considered by MPEG experts working on “Requirements on Immersive Media Delivery and Access” N18134.

JVET-N0532 AHG19: Resampling filters for adaptive resolution change (ARC) in VVC [Pankaj Topiwala, Madhu Krishnan, Wei Dai (FastVDO)] [late]

ARC has received increased attention this meeting cycle, and numerous documents consider ARC for VVC. While the signaling of ARC usage is relatively straightforward, this contribution focuses on the resampling filters used for the resolution adaptation. For comparison purposes, we use pair of a simple, zero-phase separable 5-tap downsampling filter and an 8-tap upscaling filter (the upsampler already part of the VVC design) for resolution change, which we refer to as Anchor, and believe were suggested in JVET-M0135. Following the same spirit, we introduce a 9-tap downsampling filter (complexity of 7-tap), and a very simple 4-tap upsampling filter, which is also part of the existing design of VVC. Gains of about 0.4dB in PSNR are reported, both with and without encoding, when compared to the anchor filters, on a subset of test data. Further results and visual evidence will be presented at the meeting.

# Recommendation

The AHG recommends reviewing the related input contributions and to continue to study VVC high-level syntax aspects.