| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0021_v3 |
| --- | --- |

| Title: | CE1: Summary Report on Post-prediction and Post-reconstruction Filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | Jacob Ström Sergey Ikonin Vadim Seregin | Tel: Email: | +46 702672192 jacob.strom@ericsson.com, sergey.ikonin@huawei.com, vseregin@qti.qualcomm.com. |
| Source: | CE1 coordinators |  |  |

_____________________________

# Abstract

This contribution provides a summary report of Core Experiment 1 on post-prediction and post-reconstruction filtering. The techniques are evaluated according to BD-rate gain and complexity (in both encoder and decoder). The software basis for the mandatory test in this CE is VTM-4.0, Test sequences, configurations and test conditions are according to JVET-M1021 for SDR, RA and LDB configurations are tested. Some tests are testing loop-filter variants of the filters, and in this case also the AI configuration is tested. Combinations of the different techniques are also presented.

# Test conditions and evaluation criteria

VTM-4.0 are used as anchor in this CE. Proposals are primarily compared with respect to objective quality, but complexity is also measured according to the subtest requirements.

The tests are of two types: post-prediction and post-reconstruction filters (CE1-1.1, CE1-1.2, CE1-1.3, CE1-1.7, CE1-2.1, CE1-2.4, all CE1-4 tests, all CE1-5 tests and all CE1-6 tests), and traditional loop-filters (CE1-1-5, CE1-2.3). The restrictions are different for these two types of filters.

## Restrictions for post-prediction and post-reconstruction filters

The size of the smallest processed block is 64 luma samples.

Processing should not be applied on intra blocks, nor should it be applied to combined intra/inter blocks (CIIP blocks) or intra block copy (IBC) / current picture reference (CPR) blocks.

For all tests, the following three constraint conditions may be tested:

- No restriction on using samples to the left and above the block (mandatory)

- Samples to the left and above can be used, but only if they come from an inter block (i.e., not from an intra block or a CIIP block or an IBC/CPR block). (mandatory)

- No samples outside the block can be used. (non-mandatory)

For CE1-1.6, these constraints are relaxed:

- No restriction to apply for intra blocks or intra/inter blocks and thus also no restriction to use samples to the left or above the block (same as intra prediction in VTM).

## Restrictions for loop-filters

For loop-filter tests, different constraints are used:

- There is no restriction on the smallest processed block.

- There is no restriction to apply filtering on intra blocks, i.e., both intra and inter blocks can be filtered.

- There is no restriction on using samples to the left or above the block, as long as the used samples reside in the same CTU as the filtered pixels.

# Tool descriptions

## 2.1 Test specification

The table below provides a summary of the tested technologies. For a more detailed description, please refer to Section 5.

Table 1.

| Test# | Description | Document# |
| --- | --- | --- |
| CE1-1 | Reduced complexity bilateral filter | JVET-M0885, JVET-M0884 |
| CE1-2 | Hadamard transform domain filter | JVET-M0468 |
| CE1-4 | Uniform Luma Inter Prediction Filter | JVET-M0042 |
| CE1-5 | Local illumination compensation | JVET-M0088, M0115, M0182, M0224, M0450, M0500 |
| CE1-6 | Intra Reference Sample Deblocking | JVET-M0138 |
| CE1-7 | Combined tests | Combinations of above documents |

# Comparison of technologies

The table below gives a comparison between the technologies.

Table 2.

| Test | filter shape | Comp. complex. per sample* | Precis. of mult | Sequential operations to get one sample processed | Operations before block processing | Parallel friendly | Latency (in clock cycles) | Memory. required (bytes) | How to derive filter coeffs | Min. and max. filtered CU size |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CE1-1.1a CE1-1.1b CE1-1.1c | 5 pixel “plus”-shape; For inter, 5x5 area is used to calculate filter weights. | 4 mult 23 adds (of which 1 rounding) 8 checks 5 shifts (of which 4 fixed) | 2×8 and 10×11 | 4 to get dNL 3 to get weight 1 to get weight*diff 1 to sum and round 7 in total | lookup s and k using qp calc m and c1 (1 variable shift each) calc c2 (1 add) | yes | Estimation: 3 clock cycles. | 30 | max0, m-k*dNL | Min: 8x8 Max: Intra: 64x64 Inter: 16x64, 64x16 |
| CE1-1.2a CE1-1.2b CE1-1.2c | —”— | 2 mult 25 adds (of which 3 rounding) 8 checks 7 shifts (of which 6 fixed) | 6x9 | As above, plus one more to get sI from I. 8 in total | —”— | yes | Estimation: 3 clock cycles. | 24 | —”— | —”— |
| CE1-1.3a CE1-1.3b CE1-1.3c | 5 pixel “plus”-shape | 2 mult 13 adds 4 checks 5 shifts (of which 4 fixed) | 7x9 | —”— | —”— | yes | Estimation: 2 clocks cycles | 24 | —”— | —”— |
| CE1-1.5 | 5 pixel “plus”-shape; For inter, 5x5 area is used to calculate filter weights. | intra: 2 mult 13 adds 4 checks 5 shifts (of which 4 fixed) inter: +12 adds +4 checks +2 shifts (both fixed) | 7x9 | —”— | —”— | yes | Estimation: Intra: 2 clock cycles Inter: 3 clock cycles | 47 | intra: max0, m-k*ΔsI inter: max0, m-k*dNL | Min: 4x4 Max: Intra: 64x64 Inter: 16x64, 64x16 |
| CE1-1.7 | 5 pixel “plus”-shape; For inter, 5x5 area is used to calculate filter weights. | 4 mult 23 adds 10 checks 5 shifts (of which 3 fixed) | 8×10 and 8×12 |  |  | yes | Estimation: 3 clock cycles. | 120 | max0, m-k*dNL | Min: 8x8 Max: Intra: 64x64 Inter: 16x64, 64x16 |
| CE1-2.1a CE1-2.1b CE1-2.1c | 3x3 | 18+4 l-bit add for round 8 shifts 6 checks | n/a | 5 add; 1 look-up table (16*7bits); 2 checks | Load 16*7bits per CU if the QP is switched to a different range | yes | 1 clock: @770MHz 16nm @450MHz 28nm 2 clocks: @770MHz 28nm | 70 bytes (16 7-bit values per qp group) | Pre-calculated in LUT | min: 8x8 max: 16xN or Nx16 (N<128) |
| CE1-2.3 | —”— | —”— | —”— | —”— | —”— | —”— | —”— | —”— | —”— | Min: no restrictions Max: Intra: 64x64 Inter: 16x64, 64x16 |
| CE1-2.4a CE1-2.4b CE1-2.4c | —”— | —”— | —”— | —”— | —”— | —”— | —”— | 84 bytes (16 7-bit values per qp group) | —”— | min: 8x8 max: 16xN or Nx16 (N<128) |
| CE1-4.2a CE1-4.2c | 5 pixel “plus” shape or 9tab filter (horizontal or vertical) | 6 adds 4 shifts |  |  |  | yes |  |  |  | Inter Min: 8x8 Max: 64x64 |
| CE1-4.3a CE1-4.3c | —”— | —”— | —”— | —”— | —”— | —”— | —”— | —”— | —”— | —”— |
| CE1-5.1a CE1-5.1b | 1 sample | 1 mult 1 add |  |  | Linear regression. Operations per CU: Mult: 4 * min(width, height) + 12 Add 8 * min(width, height) + 12 Shifts: 36 | yes |  | 128 for LUT | Linear regression. Operations per CU: Mult: 4 * min(width, height) + 12 Add 8 * min(width, height) + 12 Shifts: 36 | Inter Min: 64 samples Max: 64x64 |
| CE1-5.3a CE1-5.3b |  |  |  |  |  |  |  |  |  |  |
| CE1-5.4a CE1-5.4b |  |  |  |  |  |  |  |  |  |  |
| CE1-5.5a CE1-5.5b |  |  |  |  |  |  |  |  |  |  |
| CE1-5.6a CE1-5.6b | 1 sample | 1 mult 1 add |  |  | Linear regression. Operations per CU: Mult: 4 * min(width, height) + 12 Add 8 * min(width, height) + 12 Shifts: 36 | yes |  | 128 for LUT | Linear regression. Operations per CU: Mult: 4 * min(width, height) + 12 Add 8 * min(width, height) + 12 Shifts: 36 | Inter Min: 64 samples Max: 64x64 |
| CE1-5.7a CE1-5.7b | 1 sample | 1 mult 1 add |  |  | Linear regression. Operations per CU: Mult: 4 * min(8, width, height) + 12 Add 8 * min(8, width, height) + 12 Shifts: 36 | yes |  | 128 for LUT | Linear regression. Operations per CU: Mult: 4 * min(8, width, height) + 12 Add 8 * min(8, width, height) + 12 Shifts: 36 | Inter Min: 64 samples Max: 128x128 |
| CE1-6.1 | 2-taps | Multi 0.891 Add 2.979 shift 0.554 checks 0.166 | 6×10 | For filter decision: 3 multi / 7 add / 2 shift / 3 check For filter operation: 1 multi / 16 add / 2 shift | Get tc, beta from LUT based on slice QP | Yes | Estimation: 2 clock cycle | 7 values of 6-bit unsigned integer for filter coefficients | Derive from LUT | Width >= 32 and Height >= 32 |
| CE1-6.2 | 2-taps | multi 1.980 add 3.992 shift 0.996 checks 0.039 | 7×10 | For filter decision: 1 multi / 3 add / 1 shift / 3 check For filter operation: 1 multi / 3 add / 1 shift | None | yes | Estimation: 2 clock cycle | none | Linear combination of distance to corner samples | Width >= 32 and Height >= 32 |

## * max/min/abs are countered as checks

# Test results

## Regular tests

Below is a summary of the test results for the non-combination tests.

*: SIMD optimization for the proposed technology

**: running time unreliable

***: different SIMD settings (other than default settings) are used

Table 3a

|  | AI |  |  |  |  | RA |  |  |  |  | LDB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE1-1.1a |  |  |  |  |  | -0.35% | -0.13% | -0.06% | 102% | 100% | -0.35% | 0.65% | 0.81% | 101% | 101% |
| CE1-1.1b |  |  |  |  |  | -0.34% | -0.10% | -0.05% | 101% | 100% | -0.34% | 0.68% | 0.56% | 101% | 99% |
| CE1-1.1c |  |  |  |  |  | -0.31% | -0.08% | -0.05% | 102% | 102% | -0.31% | 0.64% | 0.74% | 100% | 98% |
| CE1-1.2a |  |  |  |  |  | -0.33% | -0.13% | -0.10% | 102% | 100% | -0.23% | 0.74% | 0.52% | 100% | 98% |
| CE1-1.2b |  |  |  |  |  | -0.31% | -0.13% | -0.12% | 103% | 102% | -0.20% | 0.84% | 0.68% | 100% | 98% |
| CE1-1.2c |  |  |  |  |  | -0.27% | -0.12% | -0.11% | 102% | 100% | -0.19% | 0.70% | 0.56% | 101% | 99% |
| CE1-1.3a |  |  |  |  |  | -0.25% | -0.09% | -0.07% | 102% | 101% | -0.09% | 0.88% | 0.41% | 100% | 97% |
| CE1-1.3b |  |  |  |  |  | -0.24% | -0.09% | -0.07% | 103% | 101% | -0.09% | 0.74% | 0.57% | 100% | 98% |
| CE1-1.3c |  |  |  |  |  | -0.21% | -0.08% | -0.09% | 100% | 100% | -0.04% | 0.93% | 0.60% | 99% | 97% |
| CE1-1.5 | -0.16% | 0.25% | 0.24% | 107% | 108% | -0.44% | -0.09% | -0.11% | 105% | 102% | -0.26% | 0.99% | 0.64% | 104% | 103% |
| CE1-1.7a |  |  |  |  |  | -0.35% | -0.04% | -0.01% | 90% | 89% | -0.39% | 0.54% | 0.50% | 88% | 94% |
| CE1-1.7c |  |  |  |  |  | -0.31% | -0.03% | -0.03% | 89% | 89% | -0.40% | 0.57% | 0.38% | 83% | 93% |
| CE1-2.1a |  |  |  |  |  | -0.32% | -0.13% | -0.06% | 102% | 100% | -0.34% | 0.64% | 0.59% | 101% | 100% |
| CE1-2.1b |  |  |  |  |  | -0.31% | -0.11% | -0.09% | 102% | 100% | -0.35% | 0.66% | 0.59% | 101% | 101% |
| CE1-2.1c |  |  |  |  |  | -0.30% | -0.09% | -0.06% | 102% | 100% | -0.36% | 0.75% | 0.72% | 101% | 100% |
| CE1-2.3 | -0.18% | 0.39% | 0.35% | 108% | 105% | -0.45% | -0.05% | -0.19% | 104% | 100% | -0.34% | 0.99% | 0.54% | 103% | 101% |
| CE1-2.4a |  |  |  |  |  | -0.32% | -0.58% | -0.56% | 109% | 100% | -0.30% | 0.38% | 0.44% | 114% | 100% |
| CE1-2.4b |  |  |  |  |  | -0.29% | -0.51% | -0.46% | 110% | 99% | -0.29% | 0.50% | 0.17% | 115% | 100% |
| CE1-2.4c |  |  |  |  |  | -0.30% | -0.42% | -0.41% | 109% | 99% | -0.27% | 0.57% | 0.34% | 114% | 100% |
| CE1-4.2a |  |  |  |  |  | -0.35% | -0.70% | -0.69% | 114% | 101% | -0.15% | 0.42% | 0.23% | 125% | 102% |
| CE1-4.2c |  |  |  |  |  | -0.35% | -0.59% | -0.52% | 112% | 98% | -0.12% | 0.55% | 0.47% | 125% | 102% |
| CE1-4.3a |  |  |  |  |  | -0.33% | -0.67% | -0.66% | 111% | 101% | -0.14% | 0.42% | 0.19% | 118% | 101% |
| CE1-4.3c |  |  |  |  |  | -0.33% | -0.49% | -0.50% | 109% | 98% | -0.08% | 0.61% | 0.43% | 118% | 101% |
| CE1-5.1a |  |  |  |  |  | -0.43% | -0.17% | -0.16% | 113% | 100% | -0.42% | -0.04% | -0.36% | 111% | 101% |
| CE1-5.1b |  |  |  |  |  | -0.35% | -0.28% | -0.30% | 115% | 100% | -0.36% | -0.11% | -0.30% | 114% | 101% |
| CE1-5.3a |  |  |  |  |  | -0.56% | -0.25% | -0.25% | 122% | 101% | -0.59% | -0.49% | -0.60% | 119% | 100% |
| CE1-5.3b |  |  |  |  |  | -0.47% | -0.40% | -0.46% | 125% | 101% | -0.54% | -0.38% | -0.57% | 121% | 101% |
| CE1-5.4a |  |  |  |  |  | -0.52% | -0.17% | -0.19% | 123% | 100% | -0.56% | -0.35% | -0.65% | 127% | 102% |
| CE1-5.4b |  |  |  |  |  | -0.43% | -0.38% | -0.38% | 124% | 101% | -0.49% | -0.29% | -0.40% | 129% | 102% |
| CE1-5.5a |  |  |  |  |  | -0.42% | -0.16% | -0.11% | 108% | 101% | -0.50% | -0.11% | -0.42% | 108% | 101% |
| CE1-5.5b |  |  |  |  |  | -0.30% | -0.29% | -0.24% | 109% | 102% | -0.45 | -0.19 | -0.48 | 110% | 102% |
| CE1-5.6a |  |  |  |  |  | -0.43% | -0.25% | -0.23% | 112% | 99% | -0.42% | -0.09% | -0.36% | 111% | 101% |
| CE1-5.6b |  |  |  |  |  | -0.35% | -0.29% | -0.37% | 114% | 100% | -0.34% | 0.10% | -0.37% | 113% | 102% |
| CE1-5.7a |  |  |  |  |  | -0.39% | -0.24% | -0.22% | 113% | 100% | -0.33% | -0.08% | -0.34% | 112% | 101% |
| CE1-5.7b |  |  |  |  |  | -0.30% | -0.33% | -0.32% | 114% | 100% | -0.30% | -0.18% | -0.35% | 114% | 101% |
| CE1-6.1 | -0.03% | 0.01% | -0.02% | 93%** | 100% | -0.02% | -0.01% | -0.09% | 94%** | 100% | 0.00% | 0.52% | 0.51% | 98%** | 99% |
| CE1-6.2 | 0.02% | 0.05% | 0.01% | 96%** | 101% | 0.00% | 0.02% | -0.04% | 104%** | 100% | -0.01% | 0.25% | -0.03% | 101%** | 100% |

## Combination test results

Below is a summary of the test results for the combination tests.

Table 3b

|  | AI |  |  |  |  | RA |  |  |  |  | LDB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE1-7.1 | -0.16% | 0.25% | 0.24% | 105% | 106% | -0.69% | -0.46% | -0.48% | 117% | 100% | -0.27% | 1.47% | 1.04% | 125% | 103% |
| CE1-7.2 |  |  |  |  |  | -0.59% | -0.49% | -0.41% | 115% | 101% | -0.36% | 1.01% | 0.89% | 123% | 104% |
| CE1-7.3b | -0.18% | 0.39% | 0.35% | 108% | 105% | -0.72% | -0.40% | -0.54% | 114% | 100% | -0.35% | 1.41% | 1.25% | 122% | 100% |
| CE1-7.4b |  |  |  |  |  | -0.58% | -0.47% | -0.46% | 111% | 100% |  |  |  |  |  |
| CE1-7.5 |  |  |  |  |  | -0.74% | -0.62% | -0.56% | 125% | 102% | -0,45% | 0,51% | 0,34% | 131% | 101% |
| CE1-7.6 |  |  |  |  |  | -0.72% | -0.64% | -0.64% | 123% | 100% |  |  |  |  |  |
| CE1-7.7.1a |  |  |  |  |  | -0.75% | -0.17% | -0.15% | 114% | 100% | -0.76% | 0.56% | 0.23% | 111% | 100% |
| CE1-7.7.1b |  |  |  |  |  | -0.66% | -0.34% | -0.32% | 116% | 101% | -0.71% | 0.59% | 0.24% | 114% | 101% |
| CE1-7.7.2a |  |  |  |  |  | -0.74% | -0.67% | -063% | 123% | 99% | -0.70% | 0.37% | 0.31% | 126% | 100% |
| CE1-7.7.2b |  |  |  |  |  | -0.65% | -0.74% | -0.73% | 125% | 99% | -0.63% | 0.26% | 0.00% | 129% | 100% |
| CE1-7.7.3a |  |  |  |  |  | -0.87% | -0.63% | -0.61% | 125% | 100% | -0.75% | 0.85% | 0.33% | 127% | 100% |
| CE1-7.7.3b |  |  |  |  |  | -0.79% | -0.73% | -0.74% | 127% | 100% | -0.63% | 0.77% | 0.50% | 126% | 101% |
| CE1-7.8a |  |  |  |  |  | -0.44% | -0.53% | -0.50% | 111% | 99% | -0.34% | 1.01% | 0.69% | 115% | 100% |
| CE1-7.8b |  |  |  |  |  | -0.44% | -0.47% | -0.42% | 111% | 100% | -0.33% | 0.73% | 0.72% | 116% | 100% |
| CE1-7.8c |  |  |  |  |  | -0.43% | -0.40% | -0.38% | 111% | 99% | -0.31% | 0.86% | 0.76% | 115% | 100% |

# Crosschecks

## Cross-check report of mandatory test results

Below is a summary of the mandatory tests (excluding the combination tests).

Table 4.

| Test# | Cross-checker | PSNR&BR Match (Y/N) | RunT matched? (Y/N) | SW imple-mented by cc? (Y/N) | SW studied? (Y/N) | significant incon-sistencies between the description and the provided software? (Y/N) | Other comments? | Other tests? (Y/N) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CE1-1.1a | A. Karabutov | Y | Y | N |  |  | partial results | N |
| CE1-1.1b |  | Y | Y | N |  |  | partial results | N |
| CE1-1.1c |  | Y | Y | N |  |  | partial results | N |
| CE1-1.2a | J. Rasch | Y | Y | N |  |  |  | N |
| CE1-1.2b |  | Y | Y | N |  |  |  | N |
| CE1-1.2c |  | Y | Y | N |  |  |  | N |
| CE1-1.3a | S. Ikonin | Y | Y | N | Y | N | partial results | N |
| CE1-1.3b |  | Y | Y | N | Y | N | partial results | N |
| CE1-1.3c |  | Y | Y | N | Y | N | partial results | N |
| CE1-1.5 | J. Rasch | Y | Y | N |  |  |  | N |
| CE1-1.7 | A. Karabutov | Y | N | N |  |  | partial results | N |
| CE1-2.1a | J. Ström | Y | Y | N | Y |  | partial results | N |
| CE1-2.1b |  | Y | Y | N | Y |  | partial results | N |
| CE1-2.1c |  | Y | Y | N | Y |  | partial results | N |
| CE1-2.3 | J. Ström | overall BR matches | Y | N | Y | N | partial results | N |
| CE1-2.4a | K. Reuze | Y | Y | N | Y |  | Partial results | N |
| CE1-2.4b |  | Y | Y | N | Y |  | Partial results | N |
| CE1-2.4c |  | Y | Y | N | Y |  | Partial results | N |
| CE1-4.2a | S. Ikonin | Y | Y | N | Y | N. Clarification comment: CE1-4.2 and CE1-4.3 differs by non-normative encoder speed options only | partial results | N |
| CE1-4.2c |  | Y | Y | N | Y |  | partial results | N |
| CE1-4.3a | J. Ström | Y | Y | N | Y |  | partial results | N |
| CE1-4.3b |  | Y | Y | N | Y |  | partial results | N |
| CE1-4.3a | D. Rusanovskyy | Y |  | N and Y (additional test) | Y |  | Partial results, comments in JVET-L0XXX | Y |
| CE1-5.1a | A. Karabutov | Y | Y | N | Y |  | partial results | N |
| CE1-5.1b |  | Y | Y | N | Y |  | partial results | N |
| CE1-5.3a | K. Abe | Y | Y | N | Y | N | N | N |
| CE1-5.3b |  | Y | Y | N | Y | N | N | N |
| CE1-5.4a | C-M. Tsai | Y | Y | Y | Y | N | N | N |
| CE1-5.4b |  | Y | Y | Y | Y | N | N | N |
| CE1-5.5a | P. Bordes | Y | Y | Y | Y | N | partial results | N |
| CE1-5.5b |  | Y | Y | Y | Y | N | partial results | N |
| CE1-5.6a | S. Bandyo-padhyay | Y | Y | N | Y | N | partial results | N |
| CE1-5.6b |  | Y | Y | N | Y | N | partial results | N |
| CE1-5.7a | V. Seregin | Y | Y | N | Y | N | N | N |
| CE1-5.7b |  | Y | Y | N | Y | N | partial results | N |
| CE1-6.1 | C-M. Tsai | Y | Y | Y | Y | N | N | N |
| CE1-6.2 |  | Y | Y | Y | Y | N | N | N |

## Cross-check report of combined tests

Below is a summary of the cross-checks of the combination tests.

Table 5.

| Test# | Cross-checker | PSNR&BR Match (Y/N) | RunT matched? (Y/N) | SW imple-mented by cc? (Y/N) | SW studied? (Y/N) | significant incon-sistencies between the description and the provided software? (Y/N) | Other comments? | Other tests? (Y/N) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CE1-7.1c | A. Karabutov | Y |  | N |  |  | partial results | N |
| CE1-7.2b | A. Karabutov | Y |  | N |  |  | partial results | N |
| CE1-7.3b | J. Ström | Y | Y | N | Y | N | partial results | N |
| CE1-7.4b | J. Ström | Y | Y | N | Y | N | partial results | N |
| CE1-7.5 | A. Karabutov | Y |  | N |  |  | partial results | N |
| CE1-7.6 | J. Ström | Y | Y | N | Y | N | partial results | N |
| CE1-7.7.1 | J. Rasch | Y |  |  |  |  | Partial results |  |
| CE1-7.7.2 | K. Abe | Y | Y | N | Y | N | partial results | N |
| CE1-7.7.3 | S. Wu | Y | Y | N | Y | N | partial results | N |
| CE1-7.8 | S. Wu | Y | Y | N | Y | N | partial results | N |

# Detailed description of tests

## CE1-1 Reduced complexity parametrizable bilateral filter (JVET-N0489)

### CE1-1.1a, CE1-1.1b and CE1-1.1c

These three tests use a bilateral filter as a post-reconstruction filter. Hence after the reconstruction is formed by adding the prediction to the residual and clipping, the filter is applied. To filter the intensity IC of the current sample, the surrounding four-neighbors IA (above) IB (below) IL (left) IR (right) are used.

|  | IA |  |
| --- | --- | --- |
| IL | IC | IR |
|  | IB |  |

Figure 1. Samples used in the weighted sum.

The filtered value IF is then calculated as a weighted sum of the surrounding delta-intensity values as

IF=IC+wAΔIA+wBΔIB+wLΔIL+wRΔIR+c1≫c2,     (Eqn 5.1.1)

where the deltas are the difference against the center sample, ΔIA=IA-IC etc. The weights are calculated as

wA=max0, m-k*dNLA.      (Eqn 5.1.2)

Here, k and m depend on the qp and block size, and dNLA is a non-local average of absolute differences in a 3×3 neighborhood as described in JVET-J0021 section 4.

By varying the amount of shift, i.e., the variable c2 in Equation 5.1.1, it is possible to choose small values for k. This means that the multiplication in Equation 5.1.2 can be implemented as a two-bit multiplier plus an addition. Hence, compared to the method presented in Macau, the two multipliers have changed the following way:

- 9 bit x 8 bit multiplier is now 2 bit x 8 bit

- 12 bit x 11 bit multiplier is now 10 bit x 11 bit

For more information, see JVET-N0489 Section 2.

### CE1-1.2a, CE1-1.2b and CE1-1.2c

This set test is similar to that of CE1-1.1a through CE1-1.1c, but with two differences: The Equation 5.1.2 (repeated here for the convenience of the reader)

wA=max0, m-k*dNLA      (Eqn 5.1.2)

has been replaced by

wA=max0, m-k*sNLA,      (Eqn 5.1.2)

where sNLA=dNLA≫3. This further reduces the size of the multiplication to a single addition. Second, the factor for the second multiplication wA*ΔIA is also shifted: The weight wA  is multiplied by ΔIA+4≫3 instead of by ΔIA, reducing also this multiplication. Hence, compared to the method presented in Macau, the two multipliers have changed the following way:

- 9 bit x 8 bit multiplier is replaced by an addition

- 12 bit x 11 bit multiplier is now 6 bit x 11 bit

For more information, see JVET-N0489 Section 3.

### CE1-1.3a, CE1-1.3b and CE1-1.3c

This set test is similar to that of CE1-1.2a through CE1-1.2c but avoids calculating the non-local average dNLA. Instead the sample absolute difference is used without averaging. This lowers the estimated latency since the non-local value does not need to be calculated. For more information, see JVET-N0489 Section 4.

### CE1-1.5

This is an application of the bilateral filter as a loop-filter. This means that the restriction for filtering smaller blocks is removed, as well as the restriction to filter intra blocks. Intra blocks are filtered similarly to CE1-1.3a, whereas inter blocks are filtered similar to CE1-1.2a. For more information, see JVET-N0489 Section 6.

### CE1-1.7

Current test study impact of more accurate approximation of weight function wi∆Ii  through a piece-wise linear model with 2 ranges compare to the single range linear approximation employed by the proponent of CE1-1.

Similarly to the base design, CE1-1.1, bilateral filter is used as a post-reconstruction filter. To filter the intensity IC of the current sample (see Figure 1), the surrounding four-neighbors IA (above) IB (below) IL (left) IR (right) are used (further jointly referred as Ii). The filtered value IF is calculated as a weighted sum of the surrounding delta-intensity values as:

IF=IC+wAΔIA+wBΔIB+wLΔIL+wRΔIR+c1≫c2,     (Eqn 7.1)

where the deltas are the difference between sample value Ii (sample at i position) against the center sample, ΔIi=Ii-IC etc. The delta-value dependent weights factors wi are determined by a piece-wise linear function W(ΔI), derived from parameters of bilateral filter, QP value and parameters of mode, as presented in JVET-L0406.

Effectively, prior to coding/decoding, BIF weighting function in CE1-7.a c are initialized  with parameters (normalized to the bitdepth of the of CE1-1 base design) which are estimated on fly and stored in RAM of size of approximately 285 Bytes.

Then weights WΔI are tabulated for all non-zero weights and accessed by ΔI:

WΔI=sigmaDistTabWΔI. 					(Eqn 7.2)

With sigmaDistTab consisting of 10896 bytes. This  approach may be beneficial for large block size processing due to reduction in number of operations or for memory non-constrained platforms.

T=1≪shiftPWL

indx=(ΔI<T) ? 1:0

n=ΔI<T? -ΔI : T-ΔI

k=scalePWLindx

m=offsetPWLindx

WΔI=max0, m-k*n .						(Eqn 7.2)

## The table below summarizes lists CE1-1 tester and cross-checkers.

Table 6.

| # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| CE1-1.1a | Base BiF design as in JVET-M0885 | Ericsson | A. Karabutov |
| CE1-1.1b | Base BiF design, restriction b | Ericsson | A. Karabutov |
| CE1-1.1c | Base BiF design, restriction c | Ericsson | A. Karabutov |
| CE1-1.2a | BiF with reduced multiplication complexity | Ericsson | J. Rasch, |
| CE1-1.2b | BiF with reduced multiplication complexity, restriction b | Ericsson | J. Rasch, |
| CE1-1.2c | BiF with reduced multiplication complexity, restriction c | Ericsson | J. Rasch, |
| CE1-1.3a | BiF w/o non-local samples | Ericsson | S. Ikonin |
| CE1-1.3b | BiF w/o non-local samples, restriction b | Ericsson | S. Ikonin |
| CE1-1.3c | BiF w/o non-local samples, restriction c | Ericsson | S. Ikonin |
| CE1-1.5 | BiF as loop filter | Ericsson | J. Rasch, |
| CE1-1.7a | Parametrization aspects of BiF: Two-range PWL with restrictions a | Qualcomm | A. Karabutov |
| CE1-1.7c | Parametrization aspects of BiF: Two-range PWL with restrictions c | Qualcomm | A. Karabutov |

## CE1-2 Hadamard transform domain filter (JVET-N0478)

The Hadamard transform domain filter was tested in three different positions:

- Test CE1-2.1. The filter is applied as post-reconstruction filter (corresponding to Test3 of JVET-M0468).

- Test CE1-2.3. The filter is applied as conventional in-loop filter.

- Test CE1-2.4. The filter is applied as post-prediction filter based on block-level flag. In this test encoder is able to select either one of two filters with different strength either do not filtering at all. The decision is signaled to bitstream. The filter implementation is the same as in test CE1-2.1. One additional LUT (16 entries of 7 bit) is added to five tables used in CE1-2.1 to reflect filter with more strength.

Combined test on using the filter for post-prediction and for post-reconstruction filtering (mutually exclusive) is also conducted and referred as CE1-7.8 in section 5.6.

The following table summarizes the test plan:

Table 7.

| Test | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE1-2.1a | Filter without restriction on using reconstructed samples. | S. Ikonin | J. Ström |
| CE1-2.1b | Filter using reconstructed samples from inter blocks only | S. Ikonin | J. Ström |
| CE1-2.1c | Filter without using of reconstructed samples | S. Ikonin | J. Ström |
| CE1-2.3 | Filter is applied as conventional in-loop filter (no restriction on block size or block type, samples outside the block can be used). | S. Ikonin | J. Ström, Qualcomm |
| CE1-2.4a | Filter is applied as prediction post-filter based on block-level flag and without restriction on using reconstructed samples. | S. Ikonin | Qualcomm |
| CE1-2.4b | Filter is applied as prediction post-filter based on block-level flag and using reconstructed samples from inter blocks only. | S. Ikonin | Qualcomm |
| CE1-2.4c | Filter is applied as prediction post-filter based on block-level flag and without using of reconstructed samples | S. Ikonin | Qualcomm |

## CE1-2 CE 1.4 Uniform Luma Inter Prediction Filter (JVET-N0059)

The luma inter prediction filter uses different uniform filter modes to improve the inter prediction signal. The following tests are conducted:

- use reconstructed samples (constraint a above)

- use no reconstructed samples (constraint c above)

Table 8.

| Test | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE1-4.2a | Adapt encoder search, restriction a | HHI | S. Ikonin |
| CE1-4.2c | Adapt encoder search, restriction c | HHI | S. Ikonin |
| CE1-4.3a | Adapt filter application to certain inter modes, restriction a | HHI | J. Ström, A. Karabutov |
| CE1-4.3c | Adapt filter application to certain inter modes, restriction c | HHI | J. Ström, A. Karabutov |

## CE1-2 CE 1.4 CE 1.5 Local illumination compensation (JVET-M0088, M0115, M0182, M0224, M0450, M0500)

Base LIC version as described in the meeting notes:

- Usage of reconstructed intra coded, CIIP and IBC blocks, as this would have impact on complexity and parallel implementation pipelining (test with and without, and further study the impact)

- Disabling LIC over CTU boundaries is not necessary

- Disable for CIIP and IBC blocks

- Disable LIC for the blocks with less than 64 luma samples.

- No temporal inheritance of LIC flag

- No pruning based on LIC flag in merging candidate list generation

- Disable LIC for all subblock modes, including triangle mode and BDOF

- It is not applied to 128xN and Nx128 blocks

- It is not applied to bi-prediction

When LIC is not applied, LIC flag is not signaled.

There are 2 variants of the base version:

- all reconstructed samples are used

- neighbor samples of intra, CIIP and IBC coded blocks are excluded, i.e. only inter reconstructed samples are used

Below tests will be performed for each base version.

A combination of the several CE tests will be performed for better coding efficiency.

Table 9.

| Test | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE1-5.1 | Base LIC version | Vadim Seregin vseregin@qti.qualcomm.com | Alexander Karabutov karabutov.alexander@huawei.com |
| CE1-5.3 | Test 5.1, where LIC is applied after bi-prediction is derived. | Saurav Bandyopadhyay Saurav.Bandyopadhyay @InterDigital.com | Kiyofumi Abe abe.kiyo@jp.panasonic.com |
| CE1-5.4 | Test 5.4, but LIC is not applied with BPWA | Philippe Bordes philippe.bordes@technicolor.com | Chia-Ming Tsai chia-ming.tsai@mediatek.com |
| CE1-5.5 | Test 5.4, but LIC is not applied with BPWA and encoder speed-up | Chia-Ming Tsai chia-ming.tsai@mediatek.com | Philippe Bordes philippe.bordes@technicolor.com |
| CE1-5.6 | Test 5.1 with modified bS derivation of DBF using lic_flag | Kiyofumi Abe abe.kiyo@jp.panasonic.com | Saurav Bandyopadhyay Saurav.Bandyopadhyay @InterDigital.com |
| CE1-5.7 | Test 5.1 with 16x16 unit processing | Kiyofumi Abe abe.kiyo@jp.panasonic.com | Vadim Seregin vseregin@qti.qualcomm.com |

## CE1-6 Intra Reference Sample Deblocking (JVET-M0138)

Test CE1-6.1 IRSDB

JVET-M0138 presents an Intra Reference Sample Deblocking (IRSDB) method for angular intra prediction of blocks with both width and height equal to or larger than 32.

IRSDB is designed to symmetrically deblock 7 unfiltered reference samples on respective side of a block boundary to eliminate the discontinuity between reference samples which are populated from two neighboring large blocks. IRSDB first define a set of checkpoints that includes the following positions (see figure 2):

- at the sample to the top-left of the current block

- at each position (X, Y) above the current block that X is divisible by 32

- at each position (X, Y) to the left of the current block that is Y divisible by 32

Figure 2.  Example of a set of checkpoint positions for a 32x64 block

The proposed IRSDB method in JVET-M0138 will be tested using following simplifications/modification:

- Remove the boundary check to determine if checkpoint position is at a PU or TU boundary and if block P and block Q are large blocks.

- Remove the clip from the filtering.

Test CE1-6.2 HEVC strong reference sample smoothing

HEVC strong reference sample smoothing filter is a method in HEVC which not is present in VVC but exists in VTM (default turned off). The HEVC strong reference sample smoothing filter is applied to the luma reference samples of large blocks that have both a block width and a block height larger than or equal to 32 luma samples. The HEVC strong reference sample smoothing filter firstly test the signal activity. If the signal activity for both reference samples at the left and above the current block are below a threshold, the reference samples are modified by a linear interpolation between the top-right reference sample and the top-left reference sample, and by a linear interpolation between the bottom-left reference sample and the top-left reference sample.

In the CE test, the HEVC strong reference sample smoothing implementation is aligned with HEVC to only modify twice as many samples as the block width minus 1 for above reference samples and twice as many samples as the block height minus 1 for left reference samples both modifications based on linear interpolation between the top-left reference sample and a sample at a distance of twice the block width respectively twice the block height from the top-reference sample (see figure 3).

Figure 3. Modified samples for a 32×64 block when multiple reference line index is 0, 1 or 3

## Evaluation specific for CE1-6.1 and CE1-6.2

The following aspects will be investigated:

- Subjective performance for video sequences.

The following sequences will be used for subjective performance comparisons

| Num | Name | Resolution | fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | Tango2 | 3840x2160p | 60 | 600 | RA | 34, 39 |
| 02 | Cactus | 1920x1080p | 50 | 500 | RA | 34, 39 |
| 03 | KristenAndSara | 1280x720p | 60fps | 600 | LDP | 34, 39 |

- Report complexity measures according to section 2.2 for both CE1.6.1 and CE1.6.2.

Table 10.

| Test | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE1-6.1 | Intra reference sample deblocking | Ericsson | C-M. Tsai |
| CE1-6.2 | HEVC strong reference sample smoothing | Ericsson | C-M. Tsai |

## CE 1.7 Combined tests

The following combined tests have been conducted:

Table 11.

| Test | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE1-7.1 | Uniform Luma Inter Prediction Filter and BIF as a loop-filter on top, restriction c | HHI Ericsson | A. Karabutov |
| CE1-7.2 | Combine Uniform Luma Inter Prediction Filter and BIF mutually exclusive, restriction b | HHI Ericsson | A. Karabutov |
| CE1-7.3 | Uniform Luma Inter Prediction Filter and Hadamard filter on top, restriction b | HHI Huawei | J. Ström |
| CE1-7.4 | Combine Uniform Luma Inter Prediction Filter and Hadamard filter mutually exclusive, restriction b | HHI Huawei | J. Ström |
| CE1-7.5 | Combination of CE1-4.1a with CE1-5.1, when methods can be applied one on top of another one (other versions of the CE1-4 and CE1-5 methods studied in CE can be used in the combination test) | Jennifer Rasch Jennifer.Rasch@hhi.fraunhofer.de Vadim Seregin vseregin@qti.qualcomm.com | A. Karabutov |
| CE1-7.6 | Combination of CE1-4.1a with CE1-5.1, when methods are applied in mutual exclusive manner (other versions of the CE1-4 and CE1-5 methods studied in CE can be used in the combination test) | Jennifer Rasch Jennifer.Rasch@hhi.fraunhofer.de Vadim Seregin vseregin@qti.qualcomm.com | J. Ström |
| CE1-7.7.1 | Combination of CE1-2.1 with CE1-5.1, mutually exclusive, restrictions: a, b (other versions of the CE1-2 and CE1-5 methods studied in CE can be used in the combination tests) | Huawei | J. Rasch |
| CE1-7.7.2 | Combination of CE1-2.4 with CE1-5.1, mutually exclusive, restrictions: a, b | Huawei | K. Abe |
| CE1-7.7.3 | Combination of CE1-2.1, CE1-2.4 with CE1-5.1, mutually exclusive, restrictions: a, b | Huawei | S. Wu |
| CE1-7.8 | Combination of CE1-2.1 with CE1-2.4, restrictions: a, b, c (other versions of the CE1-2 studied in CE can be used in the combination tests) | Huawei | S. Wu |

# Interdependency with other CEs

No known interdependencies are known.

# Test conditions and data to be provided

The common test conditions are used to evaluate the objective coding gains of different methods.

# Related contributions

Table 12.

| JVET number | Title |
| --- | --- |
| JVET-N0171 | CE1-related: Combination of LIC and affine |
| JVET-N0206 | CE1-related: Improvement of the neighboring reference samples restriction on LIC test |
| JVET-N0304 | CE1-related: combined bi-directional LIC and post-reconstruction filters mutually exclusive |
| JVET-N0307 | CE1-related: on LIC reference samples selection |
| JVET-N0410 | CE1-related: Simplification of LIC parameter derivation and its unification with CCLM |
| JVET-N0479 | CE1-related: Hadamard transform domain in-loop filter also using bottom/right reconstructed samples |
| JVET-N0482 | CE1-related: Additional test results for CE1-7 tests |
| JVET-N0493 | CE1-related: Multiplication-free bilateral loop filter |

# Appendix of Participants

Table 13.

| Nr. | Name | Company | Email | Proponents | Cross-checker |
| --- | --- | --- | --- | --- | --- |
| 1 | Jacob Ström | Ericsson | jacob.strom@ericsson.com | X | X |
| 2 | Zhi Zhang | Ericsson | zhi.zhang@ericsson.com | X |  |
| 3 | Sergey Ikonin | Huawei | sergey.ikonin@huawei.com | X | X |
| 4 | Karabutov Alexander | Huawei | karabutov.alexander@huawei.com | X | X |
| 5 | Sychev Maxim | Huawei | Sychev.Maxim@huawei.com | X |  |
| 6 | Filippov Alexey | Huawei | Alexey.Filippov@huawei.com | X |  |
| 7 | Rufitskiy Vasily | Huawei | Vasily.Rufitskiy@huawei.com | X |  |
| 8 | Yanghaitao (Haitao) | Huawei | haitao.yang@huawei.com | X |  |
| 9 | Vadim Seregin | Qualcomm | vseregin@qti.qualcomm.com | X |  |
| 10 | Dmytro Rusanovskyy | Qualcomm | dmytror@qti.qualcomm.com | X | X |
| 11 | Nan Hu | Qualcomm | nanh@qti.qualcomm.com | X |  |
| 12 | Jennifer Rasch | HHI | Jennifer.Rasch@hhi.fraunhofer.de | X | X |
| 13 | Jonathan Pfaff | HHI | jonathan.pfaff@hhi.fraunhofer.de | X |  |
| 14 | Anastasia Henkel | HHI | anastasia.henkel@hhi-extern.fraunhofer.de | X |  |
| 15 | Chih-Wei Hsu | MediaTek | cw.hsu@mediatek.com | X |  |
| 16 | Ching-Yeh Chen | MediaTek | chingyeh.chen@mediatek.com | X |  |
| 17 | Tzu-Der Chuang | MediaTek | peter.chuang@mediatek.com | X |  |
| 18 | Chia-Ming Tsai | MediaTek | chia-ming.tsai@mediatek.com | X |  |
| 19 | Zhi-Yi Lin | MediaTek | zhi-yi.lin@mediatek.com | X |  |
| 20 | Man-Shu Chiang | MediaTek | man-shu.chiang@mediatek.com | X |  |
| 21 | Chun-Chia Chen | MediaTek | chunchia.chen@mediatek.com | X |  |
| 22 | Yu-Wen Huang | MediaTek | yuwen.huang@mediatek.com | X |  |
| 23 | Olena Chubach | MediaTek | Olena.Chubach@mediatek.com | X |  |
| 24 | Philippe Bordes | Technicolor | philippe.bordes@technicolor.com | X |  |
| 25 | Franck Galpin | Technicolor | Franck.Galpin@technicolor.com | X |  |
| 26 | Tangi Poirier | Technicolor | Tangi.Poirier@technicolor.com | X |  |
| 27 | Fabrice LeLeannec | Technicolor | Fabrice.Leleannec@technicolor.com | X |  |
| 28 | Kiyofumi Abe | Panasonic | abe.kiyo@jp.panasonic.com | X | X |
| 29 | Tadamasa Toma | Panasonic | toma.tadamasa@jp.panasonic.com | X |  |
| 30 | Jingya Li | Panasonic | jingya.li@sg.panasonic.com | X |  |
| 31 | Che-Wei Kuo | Panasonic | chewei.kuo@sg.panasonic.com | X |  |
| 32 | Chong Soon Lim | Panasonic | chongsoon.lim@sg.panasonic.com | X |  |
| 33 | Saurav Bandyopadhyay | Interdigital | saurav.bandyopadhyay@interdigital.com |  | X |
| 34 | Yuwen He | Interdigital | yuwen.he@interdigital.com |  |  |
| 35 | Daniel Luo | Interdigital | Daniel.Luo@InterDigital.com |  |  |
| 36 | Kevin Reuze | Qualcomm | kreuz@qti.qualcomm.com |  | x |

# References

[1] F.Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Macao, CN, 3–12 Oct. 2018.

[2] J. Ström, ”Non-CE: Reduced complexity bilateral filtering”, JVET-M0885, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019

[3] S. Ikonin, V. Stepin, J. Chen, “Non-CE: Hadamard transform domain filter”, JVET-M0468, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019

[4] M. Karczewicz, “Non-CE: Test on parametrizable bilateral filter from JVET-L0406 in VTM3.0”, JVET-M0894, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019

[5] J. Rasch, A. Henkel, J. Pfaff, H. Schwarz, D. Marpe, T. Wiegand. ”CE10: Uniform Directional Diffusion Filters For Video Coding”, JVET-M0042, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019

[6] V. Seregin, W.-J. Chien, T. Hsieh, N. Hu, M. Karczewicz, “CE10-related: Unidirectional illumination compensation”, JVET-M0500, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019

[7] K.Abe, T. Toma, J. Li, C.-W. Kuo, V. Drugeon, “CE10-related: LIC restriction for pipeline structure”, JVET-M0088, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019

[8] C.-M. Tsai, C.-C. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, “CE10-related: Simplification of local illumination compensation”, JVET-M0182, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019

[9] S. Bandyopadhyay, X. Xiu, Y. He, “CE10-related: Local illumination compensation simplifications”, JVET-M0224, Marrakech, MA, January 2019.