| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0022-v2 |
| --- | --- |

| Title: | CE2: Summary report on sub-block based motion prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen Yuwen He Hongbin Liu | Email: | chunchic@qti.qualcomm.com yuwen.he@interdigital.com liuhongbin.01@bytedance.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

The goal of Core Experiment 2 (CE2) is to investigate sub-block based motion prediction techniques on top of VTM-4.0. It comprises 5 categories, as follows:

- CE 2.1: Affine motion compensation and affine MV coding (2)

- CE 2.2: Affine merge mode (2)

- CE 2.3: Sub-block based Merge Mode (1)

- CE 2.4: Memory bandwidth reduction for sub-block modes (10)

- CE 2.5: Storage reduction for sub-block modes (6)

In each test, a comparative study along with related tests is conducted, results and complexity analysis are provided. Crosschecking reports of all tests are integrated in this document as well.

# CE2.1: Affine Motion Compensation and Affine MV Coding

## Categorization & comparative study

### Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 2-1.1a | Interweaved prediction for affine motion compensation for luma only | JVET-N0261 |
| 2-1.1b | Test 2-1.1a + chroma |  |
| 2-1.1c | Test 2-1.1a + Test 2-4.6 |  |
| 2-1.1d | Test 2-1.1a + Test 2-4.8a |  |
| 2-2.1a | SMVD with signaling MVDs of all control points for L0 | JVET-N0319 |
| 2-2.1b | SMVD with signaling MVDs of TL control point for L0 and set TR and BL control point MVD for L0 to 0 |  |

### Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 2-1.1a | -0.30% | 0.02% | -0.02% | 100% | 100% | -0.14% | 0.04% | -0.15% | 101% | 101% |
| 2-1.1b | -0.32% | -0.21% | -0.18% | 100% | 99% | -0.14% | -0.13% | -0.18% | 101% | 100% |
| 2-1.1c | -0.28% | 0.03% | 0.03% | 101% | 100% | -0.11% | 0.10% | 0.03% | 101% | 102% |
| 2-1.1d | -0.28% | 0.01% | 0.02% | 101% | 101% | -0.13% | -0.13% | 0.22% | 101% | 101% |
| 2-2.1a | -0.10% | -0.06% | -0.06% | 106% | 100% |  |  |  |  |  |
| 2-2.1b | -0.07% | -0.05% | -0.05% | 105% | 100% |  |  |  |  |  |

### Complexity analysis

### Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 2-1.1a | D. Luo (Interdigital) | 2 | F | Y | M |
| 2-1.1b | X. Li (Tencent) | 2 | F | Y | M |
| 2-1.1c | D. Luo (Interdigital) | 2 | P | Y | M |
| 2-1.1d | J. Li (Panasonic) |  |  |  |  |
| 2-2.1a | G. Li (Tencent) | 2 | F | Y | M |
| 2-2.1b | G. Li (Tencent) | 2 | F | Y | M |

## Technical description

### CE2-1.1: Interweaved Prediction for Affine Motion Compensation (JVET-N0261, Kwai)

With the interweaved prediction, a coding block is divided into sub-blocks with two different dividing patterns. Then two auxiliary predictions are generated by affine motion compensation with the two dividing patterns. The final prediction is calculated as a weighted-sum of the two auxiliary predictions.

Fig. Interweaved prediction

Fig. Interweaved prediction is not applied on the shaded regions.

### CE2-1.2: Symmetric MVD for affine bi-prediction coding (JVET-N0319, InterDigital)

Symmetric MVD (SMVD) mode applied to affine bi-prediction will be tested. With the symmetric MVD mode for affine bi-prediction, motion information including reference picture indices of both list-0 and list-1 and control point MVDs of list-1 are not signalled but derived.

Firstly, in slice level, variables BiDirPredFlag is derived in the same way as JVET-M0444 and JVET-L0370:

- If mvd_l1_zero_flag is 1, BiDirPredFlag is set equal to 0.

- Otherwise, if the nearest reference picture in list-0 and the nearest reference picture in list-1 form a forward and backward pair of reference pictures or a backward and forward pair of reference pictures, BiDirPredFlag is set to 1. Otherwise BiDirPredFlag is set to 0.

Secondly, in CU level, a symmetrical mode flag indicating whether symmetrical mode is used or not is explicitly signalled if the CU is bi-prediction coded and BiDirPredFlag is equal to 1. This flag is signalled when the CU is explicitly bi-prediction coded, for both affine and non-affine bi-prediction.

When the symmetrical mode flag is true, only MVP index flags mvp_l0_flag, mvp_l1_flag and MVD in list-0 (MVD0) are explicitly signalled. The reference indices for list-0 and list-1 are set equal to the pair of reference pictures, respectively. If affine mode flag is true, top-left control point’s MVD of list-1 is set equal to the negative of the top-left control point’s MVD of list-0. The other control point MVDs of list 1 are set to zero. The final control point motion vectors are derived in below formulas:

For the top-left control point:

mvx0,0, mvy0,0=(mvpx0,0+mvdx0,0,mvpy0,0+mvdy0,0) mvx1,0, mvy1,0=(mvpx1,0-mvdx0,0,mvpy1,0-mvdy0,0)

For the other control points (j = 1 or 2):

mvx0,j, mvy0,j=(mvpx0,j+mvdx0,j+mvdx0,0,mvpy0,j+mvdy0,j+mvdy0,0) mvx1,j, mvy1,j=(mvpx1,j-mvdx0,0,mvpy1,j-mvdx0,0)

Where the first dimension of the subscripts indicates the reference list, and the second dimension of the subscripts indicates the control point index.

Three options will be tested:

- Signal MVDs of all control points for List 0

- Signal MVDs of TL control point for List 0 and set TR and BL control point MVD for List 0 to 0

# CE2.2: Affine Merge Mode

## Categorization & comparative study

### Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 2-2.1 | Only select the MV with reference index equal to 0 as CPMV | JVET-N0075 |
| 2-2.2a | Affine MMVD with POC-based offset mirroring and a distance table {1/2, 1, 2, 4, 8} | JVET-N0378 |
| 2-2.2b | Affine MMVD with POC-based offset scaling |  |
| 2-2.2c | Test 2-2.2a + Affine MMVD with resolution-based distance tables: {1/2, 1, 2, 4, 8}/N, where N=1,4,8 |  |

### Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 2-2.1 | 0.02% | -0.01% | 0.00% | 100% | 99% | 0.02% | 0.04% | -0.08% | 100% | 101% |
| 2-2.2a | -0.13% | -0.15% | -0.10% | 101% | 100% | 0.01% | -0.09% | -0.26% | 103% | 99% |
| 2-2.2b | -0.12% | -0.17% | -0.19% | 102% | 100% | -0.06% | -0.05% | -0.26% | 104% | 99% |
| 2-2.2c | -0.15% | -0.11% | -0.10% | 101% | 100% | -0.03% | -0.02% | 0.09% | 103% | 99% |

### Complexity analysis

| Test# | Max number of potential candidates | Max number of candidate comparison | Max number of pruning stages | Max number of MV scaling | Others |
| --- | --- | --- | --- | --- | --- |
| 2-2.1 | Same | Same | Same | Same | reduce 13 comparisons for constructed affine merge candidate derivation |
| 2-2.2a | +20 | Same | Same | Same |  |
| 2-2.2b | +20 | Same | Same | Same |  |
| 2-2.2c | +20 | Same | Same | Same |  |

### Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 2-2.1 | Y.-C. Yang (Foxconn) | 2 | F | Y | M |
| 2-2.2a | Y.-W. Chen (Kwai) | 1 | F | Y | M |
| 2-2.2b | Y.-W. Chen (Kwai) | 1 | F | Y | M |
| 2-2.2c | Y.-W. Chen (Kwai) | 1 | F | Y | M |

## Technical description

### CE2-2.1: Simplification of constructed affine merging candidate derivation (JVET-N0075, Mediatek)

In VTM3.0, when constructing the subblock-based merging candidate list, if the subblock-based merging candidate list is not full after adding the inherited affine candidates, constructed affine candidates are derived. The derivation for constructed affine merging candidate includes two stages. The first stage is to derive the four CPMVs at the four corners of the current coding unit (CU) as shown in Figure 1. The MV(s) of CP0 is the first available MV in block B2, B3, or A2. The MV(s) of CP1 is the first available MV in block B1 or B0. The MV(s) of CP2 is the first available MV in block A1 or A0. The MV(s) of CP3 is the temporal MV in block RB in the collocated picture. It requires 8 comparison in the worst case to check whether the MVs in each position is available. The second stage is to check the validity of the control point (CP) sets in a CP set list. The CP set list is { {CP0, CP1, CP2}, {CP0, CP1, CP3}, {CP0, CP2, CP3}, {CP1, CP2, CP3}, {CP0, CP1}, and {CP0, CP2} }. For each CP set, the reference index in each reference list is checked. For each reference list, if the reference indices of the CPMVs of the CP set are not the same or one of the CPMVs’ reference index is not larger than or equal to 0, the CPMVs in that reference list are set to be unavailable. The reference index check of the CPMVs of the CP set for list 0 (L0) and list 1 (L1) requires at most 52 reference index comparisons.

It is proposed to reduce the number of comparisons for constructed affine candidate derivation by only selecting the MV with reference index equal to 0. Therefore, the reference index comparison in the later stage can be removed.

Fig.  The neighbouring blocks for constructed affine candidates’ CPMV derivation.

### CE2-2.2: Affine merge with prediction offset (JVET-N0378, Tencent)

The proposed method affine merge with prediction offset (affine MMVD) selects the first available affine merge candidate from the sub-block based merge list as a base predictor. It applies a motion vector offset to each control point’s motion vector value from the base predictor. If there’s no affine merge candidate available, the proposed method will not be used. A distance index and an offset direction index are subsequently signaled.

Distance index is signaled to indicate which distance offset to use from the below distance-offset table.

| Distance IDX | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- |
| Distance-offset | 1/2-pel | 1-pel | 2-pel | 4-pel | 8-pel |

The direction index can represent four directions as shown in the below offset-direction table, where only x or y direction may have an MV difference, but not in both directions.

| Offset Direction IDX | 00 | 01 | 10 | 11 |
| --- | --- | --- | --- | --- |
| x-dir-factor | +1 | –1 | 0 | 0 |
| y-dir-factor | 0 | 0 | +1 | –1 |

If the inter prediction is Uni-prediction, the signaled distance offset is applied on the offset direction for each control point predictor. Results will be the MV value of each control point.

If the inter prediction is Bi-prediction, the signaled distance offset is applied on the signaled offset direction for control point predictor’s L0 motion vector; the offset to be applied on L1 MV is applied on a mirrored or a scaled basis, specified in each subtest.

Adaptive offset table method will also be tested. Similar to the method in JVET-M0431, the offset table will be selected based on picture resolution.

# CE2.3: Sub-block based Merge Mode

## Categorization & comparative study

### Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 2-3.1 | Withdrawn | n/a |

# CE2.4: Memory Bandwidth Reduction for Sub-block Modes

## Categorization & comparative study

### Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 2-4.1 | Withdrawn | n/a |
| 2-4.2a | Affine sub-block size setting: 4x4 for UNI, 8x4 for BI | JVET-N0323 |
| 2-4.2b | Affine sub-block size setting: 4x4 for UNI, 4x8 for BI |  |
| 2-4.2c | Affine sub-block size setting: 4x4 for UNI, adaptive 8x4/4x8 for BI |  |
| 2-4.2d | Affine sub-block size setting: 4x4 for UNI, 8x8 for BI |  |
| 2-4.3 | Clip sub-block MVs based on the bounding box pointed to by top-left one | JVET-N0195 |
| 2-4.4a | Clip sub-block MVs based on limiting max differences among 4x4 MVs within an 8x8 block are no larger than Tx =1 and Ty=0 | JVET-N0398 |
| 2-4.4b | Test 2-4.4a with Tx =1 and Ty=1 |  |
| 2-4.4c | Test 2-4.4a with Tx =1 and Ty=1 and distance-based MVmin MVmax determination off |  |
| 2-4.5a | Reduce worst-case memory bandwidth by switching sub-block size: 4x4, 8x8 | JVET-N0256 |
| 2-4.5b | Reduce worst-case memory bandwidth by switching interpolation filter: 4-, 8-tap |  |
| 2-4.5c | Reduce worst-case memory bandwidth by imposing encoder bitstream conformance |  |
| 2-4.6 | Complexity and bandwidth reduction for 4x4 sized partition using 6-tap filter | JVET-N0196 |
| 2-4.7a | Motion compensation with padding for 4x4 UNI and 4x8/8x4/4x16/16x4 BI | JVET-N0253 |
| 2-4.7b | Based on Test 2-4.7a, padding with 1 less column or row for 4x8 BI and 4x4 UNI |  |
| 2-4.8a | Encoder/decoder conformance, the fallback mode is not triggered, δx = δy = 2 | JVET-N0068 |
| 2-4.8b | Decoder conformance, the fallback mode is triggered, δx = δy = 2 |  |
| 2-4.9a | Integer motion compensation for affine mode | JVET-N0210 |
| 2-4.9b-1 | Integer motion compensation for regular inter mode 4x4/4x8/8x4 |  |
| 2-4.9b-2 | Test 2-4.9b-1 + Rounded MVs for storage |  |
| 2-4.10a | Affine MVp range limit on constructed affine candidates with parameter threshold=2 | JVET-N0379 |
| 2-4.10b | Test 2-4.10a with parameter threshold=1 |  |
| 2-4.10c | Affine MVp range limit on all affine blocks with parameter threshold=2 |  |

### Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 2-4.1 |  |  |  |  |  |  |  |  |  |  |
| 2-4.2a | 0.10% | 0.02% | 0.04% | 99% | 100% | 0.11% | 0.14% | 0.15% | 101% | 98% |
| 2-4.2b | 0.11% | 0.04% | 0.03% | 102% | 103% | 0.12% | 0.12% | -0.05% | 98% | 97% |
| 2-4.2c | 0.10% | 0.03% | 0.06% | 101% | 102% | 0.11% | 0.11% | 0.05% | 99% | 97% |
| 2-4.2d | 0.21% | 0.10% | 0.12% | 100% | 102% | 0.18% | 0.08% | -0.01% | 99% | 98% |
| 2-4.3 | 0.01% | -0.06% | 0.00% | 101% | 100% | -0.03% | 0.05% | -0.26% | 101% | 100% |
| 2-4.4a | 0.09% | -0.01% | 0.00% | 102% | 100% | -0.01% | 0.10% | -0.17% | 103% | 99% |
| 2-4.4b | 0.01% | -0.03% | -0.05% | 102% | 99% | -0.02% | -0.14% | -0.28% | 103% | 100% |
| 2-4.4c | 0.01% | -0.03% | -0.05% | 104% | 102% | -0.02% | -0.14% | -0.28% | 105% | 103% |
| 2-4.5a | 0.04% | -0.06% | -0.03% | 102% | 104% | 0.00% | 0.21% | -0.22% | 100% | 100% |
| 2-4.5b | 0.07% | 0.05% | 0.07% | 101% | 104% | 0.02% | -0.09% | -0.10% | 100% | 101% |
| 2-4.5c | 0.03% | 0.01% | 0.06% | 103% | 104% | 0.04% | 0.09% | -0.28% | 100% | 98% |
| 2-4.6 | 0.08% | -0.05% | -0.01% | 101% | 100% | 0.11% | 0.16% | -0.15% | 101% | 100% |
| 2-4.7a | 0.01% | -0.04% | -0.08% | 98% | 99% | 0.00% | -0.05% | -0.01% | 99% | 99% |
| 2-4.7b | 0.00% | -0.05% | -0.07% | 99% | 99% | -0.01% | 0.07% | -0.05% | 99% | 100% |
| 2-4.8a | 0.02% | 0.00% | 0.03% | 95%* | 97%* | 0.00% | 0.18% | -0.19% | 92%* | 94%* |
| 2-4.8b | -0.01% | -0.01% | 0.01% | 98%* | 99%* | 0.01% | 0.17% | -0.26% | 102%* | 100%* |
| 2-4.9a |  |  |  |  |  |  |  |  |  |  |
| 2-4.9b-1 | 0.09% | 0.11% | 0.18% | 102% | 96% | 0.09% | 0.35% | 0.12% | 101% | 96% |
| 2-4.9b-2 | 0.06% | 0.15% | 0.21% | 98% | 100% | 0.07% | 0.17% | 0.17% | 100% | 102% |
| 2-4.10a | -0.01% | 0.02% | 0.00% | 100% | 100% | -0.02% | 0.21% | 0.01% | 101% | 100% |
| 2-4.10b | -0.01% | -0.02% | -0.04% | 100% | 100% | 0.00% | 0.10% | 0.08% | 100% | 99% |
| 2-4.10c | 0.00% | 0.01% | 0.00% | 100% | 100% | -0.01% | 0.13% | 0.04% | 99% | 99% |
| * It is reported from proponents that the encoding and decoding runtime of respective tests may not be reliable. |  |  |  |  |  |  |  |  |  |  |

### Complexity analysis

| Test# | Bandwidth measurement for uni/bi-prediction | Operations of SB MV derivation per 8x8 block (e.g. clipping) | Complexity of MC per 8x8 bi-prediction: Multiplication, addition | Others (additional operations per CU to determine SB size, MC fetching unit) Multiplication, addition, comparison, look-up size |
| --- | --- | --- | --- | --- |
| Inter | 10.69 / 14.69 | n/a | 2944+352=3296 Mul. 2704+296=3000 Add | no |
| Affine | 9.09 / 18.19 | no clipping | 960*4+352=4192 Mul. 872*4+296=3784 Add | no |
| 2-4.1 |  |  |  |  |
| 2-4.2a | 9.09 / 13.38 | same | 1920*2+352=4192 Mul. 1744*2+296=3784 Add | 1 comparison per CU (1 for uni/bi) |
| 2-4.2b | 9.09 / 13.38 | same | 1472*2+352=3296 Mul. 1352*2+296= 3000 Add | 1 comparison per CU (1 for uni/bi) |
| 2-4.2c | 9.09 / 13.38 | same | 1920*2+352=4192 Mul. 1744*2+296=3784 Add | 2 comparisons per CU (1 for width/height + 1 for uni/bi) |
| 2-4.2d | 9.09 / 11.06 | same | 2944+352=3296 Mul. 2704+296=3000 Add | 1 comparison per CU (1 for uni/bi) |
| 2-4.3 |  |  |  |  |
| 2-4.4a | 5.28/10.56 | 15 comparisons for each 8x8 block, may be performed in parallel | same | no |
| 2-4.4b | 5.53 / 11.06 | 15 comparisons for each 8x8 block, may be performed in parallel | same | no |
| 2-4.4c | 5.53 / 11.06 | 14 comparisons for each 8x8 block, may be performed in parallel | same | no |
| 2-4.5a | 9.09/11.06 | no change | same | 13 comparisons per CU (1 for uni/bi + 12 for restriction) |
| 2-4.5b | 9.09/11.07 | no change | same | 14 comparisons per CU (1 for uni/bi + 12 for restriction) |
| 2-4.5c | 9.09/11.08 | no change | same | no |
| 2-4.6 |  |  |  |  |
| 2-4.7a | 15.25 / 20.19 | no change | same | no |
| 2-4.7b | 15.25 / 22.25 | no change | same | no |
| 2-4.8a |  |  |  |  |
| 2-4.8b |  |  |  |  |
| 2-4.9a |  |  |  |  |
| 2-4.9b-1 | 10.69/10.09 | no change | same as 8x8 bi-prediction | no change |
| 2-4.9b-2 | 10.69/10.09 | no change | same as 8x8 bi-prediction | no change |
| 2-4.10a | no change | no change | same | no |
| 2-4.10b | no change | no change | same | no |
| 2-4.10c | no change | no change | same | no |

### Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 2-4.1 | Withdrawn |  |  |  |  |
| 2-4.2a | G. Li (Tencent) | 2 | F | Y | M |
| 2-4.2b | G. Li (Tencent) | 2 | F | Y | M |
| 2-4.2c | G. Li (Tencent) | 2 | F | Y | M |
| 2-4.2d | G. Li (Tencent) | 2 | F | Y | M |
| 2-4.3 | X. Zheng (DJI) |  |  |  |  |
| 2-4.4a | K. Zhang (ByteDance) | 2 | F | Y | M |
| 2-4.4b | K. Zhang (ByteDance) | 2 | F | Y | M |
| 2-4.4c | K. Zhang (ByteDance) | 2 | F | Y | M |
| 2-4.5a | Y.-W. Chen (Kwai) |  |  |  |  |
| 2-4.5b | Y.-W. Chen (Kwai) |  |  |  |  |
| 2-4.5c | Y.-W. Chen (Kwai) |  |  |  |  |
| 2-4.6 | K. Zhang (Bytedance) |  |  |  |  |
| 2-4.7a | X. Zheng (DJI) |  |  |  |  |
| 2-4.7b | X. Zheng (DJI) |  |  |  |  |
| 2-4.8a | K. Zhang (Bytedance) | 2 | F | Y | M |
| 2-4.8b | K. Zhang (Bytedance) | 2 | F | Y | M |
| 2-4.9a | H. Liu (Bytedance) |  |  |  |  |
| 2-4.9b-1* | H. Liu (Bytedance) | 1 | F | Y | M |
| 2-4.9b-2* | H. Liu (Bytedance) | 1 | F | Y | M |
| 2-4.10a | J. Zhao (LGE) | 2 | F | Y | M |
| 2-4.10b | J. Zhao (LGE) | 2 | F | Y | M |
| 2-4.10c | J. Zhao (LGE) | 2 | F | Y | M |
| * It was commented from a cross-checker that for RA case, there is minor PSNR difference on some sequences due to rounding, the bit rate are identical; for LDB case, the results are identical. |  |  |  |  |  |

## Technical description

### CE2-4.1: Withdrawn

### CE2-4.2: Reducing worst-case memory bandwidth of affine mode (JVET-N0323, Kwai/Huawei)

Sub-block size for affine mode is increased in case of bi-prediction. For uni-prediction sub-block size is not changed and is equal to 4x4. The four options are considered for bi-prediction:

- affine sub-block size is equal to 8x4.

- affine sub-block size is equal to 4x8.

- affine sub-block size is adaptively switched between 8x4 and 4x8 depending on aspect ratio of coded block: if coded block width is greater than or equal to coded block height, sub-block size is set equal to 8x4 otherwise, to 4x8.

d) affine sub-block size is adaptively switched between 8x4 and 4x8 depending on control point motion vector differences:

If absv1y-v0y+ absv1y-v0y> absv1x-v0x+ absv1x-v0x, the sub-block size is set equal to 8x4. Otherwise, the sub-block size is 4x8.

e)  affine sub-block size is equal to 8x8.

### CE2-4.3: Memory bandwidth reduction for the affine mode (JVET-N0195, Panasonic)

For affine mode, sub-block motion vectors of an affine CU are constrained to be within a pre-defined motion vector field. Assume that center motion vector of the constrained motion vector field is (v0x,v0y) and motion vector of a sub-block is (vix, viy), values of vix and viy exhibit the following constraints:

MinX = (v0x>> MV_precision << MV_precision) - mvRange;

MaxX = (v0x>> MV_precision << MV_precision) + mvRange +(1<< MV_precision);

MinY = (v0y>> MV_precision << MV_precision) - mvRange;

MaxY = (v0y>> MV_precision << MV_precision) + mvRange +(1<< MV_precision);

values of vix and viy exhibit the following constraints:

vix ∊ [MinX, MaxX]

viy ∊ [MinY, MaxY]

If the motion vector of any sub-block exceeds the pre-defined motion vector field, the motion vector is clipped to the closest range.

The value of mvRange is chosen so that worst case memory bandwidth of affine CU is similar or smaller than that of normal inter MC of a 8×8 bi-prediction block. Note that the value of mvRange is adaptive to CU size (cbWidth+cbHeight) and uni-prediction or bi-prediction.

The planned test is to use the 1st (top left) sub-block’s motion vector as center motion vector of the constrained motion vector field. In case the blk size is greater than VPDU size = 64*64, the top left sub-block motion vector of the VPDU is used as the center motion vector.

For Bi-pred affine CU, values of mvRange are as below (in 1/16 precision):

| cbWidth + cbHeight | >=128 | 96 | 80 | 72 | 64 | 48 | 40 | 32 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mvRange | (24 << 4) | (15 << 4) | (8 << 4) | (3 << 4) | (10 << 4) | (6 << 4) | (2 << 4) | (3 << 4) |
| cbWidth + cbHeight | 24 | 16 |  |  |  |  |  |  |
| mvRange | (1 << 4) | 0 |  |  |  |  |  |  |

For Uni-pred affine CU, values of mvRange are as below (in 1/16 precision):

| cbWidth + cbHeight | >=128 | 96 | 80 | 72 | 64 | 48 | 40 | 32 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mvRange | (49 << 4) | (33 << 4) | (20 << 4) | (11 << 4) | (22 << 4) | (14 << 4) | (8 << 4) | (9 << 4) |
| cbWidth + cbHeight | 24 | 16 |  |  |  |  |  |  |
| mvRange | (5 << 4) | (3 << 4) |  |  |  |  |  |  |

### CE2-4.4: Affine block memory bandwidth reduction by MV clip (JVET-N0398, Tencent)

In this test, affine sub-block MVs are constrained at 8x8 level so that the max differences among 4x4 level MVs within an 8x8 sub-blocks are no larger than Tx and Ty for x and y components, respectively. To achieve different trade-off between complexity and coding performance, the distance based MVmin and MVmax determination may be off.

### CE2-4.5: Worst-case Memory Bandwidth Reduction for affine (JVET-N0256, Qualcomm)

In this test, three different approaches are tested to ensure the memory bandwidth in VVC should not exceed a predefined value. These approaches include switch sub-block size, switch filter, and encoder bitstream conformance as described in follows:

2.4.5a. Switch sub-block size: For regular inter prediction, a prediction which leads to a bandwidth exceeding the certain BW will be disabled.

For affine mode, sub-block size is set higher 4x4 when it exceeds the certain BW.  The condition based on the CPMV differences.

2.4.5b. Switch filter: For regular inter prediction, a prediction which leads to a bandwidth exceeding the certain BW will be performed using a shorter filter.

For affine mode, shorter filter will be applied when it exceeds the certain BW. The condition based on the CPMV differences.

2.4.5c. Encoder bitstream conformance: For regular inter prediction, a prediction which leads to a bandwidth exceeding the certain BW will be disabled. For affine mode, an affine parameters model is selected when the CPMV difference does not satisfy the constraint that is imposed in the bitstream.

### CE2-4.6: Using the shorter-tap filter for 4x4 sized partitions (JVET-N0196, Panasonic)

In this contribution 6-tap filter is used for 4x4 sized partitions including 4x4 CU and sub-block of affine CU.

### CE2-4.7: Motion compensation with padded samples for small coding units (JVET-N0253, Bytedance)

In motion compensation of small blocks (like 4x4 uni-predicted block and 4x4/4x8/8x4/4x16/16x4 bi-predicted block), only some of the reference pixels are fetched and other required reference pixels are padded.

### CE2-4.8: On restriction of memory bandwidth consumption of affine mode (JVET-N0068, Broadcom)

The following method will be tested. The decoder checks whether the following conditions are satisfied:

- If a PU uses bi-directional affine mode (i.e. bi-pred), the reference block bounding box size bxW4*bxH4 is less than or equal to Thred4.

- Otherwise, if the PU is coded in unidirectional affine mode, the reference block bounding box size bxWh*bxHh is less than or equal to Thredh and the reference block bounding box size bxWv*bxHv is less than or equal to Thredv.

If yes, the decoder generates the sub-block vectors of the PU in the same way as defined in the VTM4.0. Otherwise, all the sub-block vectors of the PU are set to the same vector which is computed at the center position of the PU by using the affine motion model.

- (b)                                                 (c)

Fig. Illustration of reference block bounding box of 2x2(a), 2x1(b) and 1x2(c) sub-block motion vectors in an affine-code PU

As defined in JVET-M0049, the width and height of the reference block bounding box of 2x2, 2x1 and 1x2 sub-block vectors, i.e. bxW4,bxH4, bxWh,bxHh and (bxWv,bxHv), are computed by:

bxW4=max0, 41+a, 4c,41+a+4c-min0, 41+a, 4c,41+a+4c+11bxH4=max0, 4b, 41+d,4b+41+d-min 0, 4b, 41+d,4b+41+d+11

bxWh=max0, 41+a-min0, 41+a+11bxHh=max0, 4b -min0, 4b +11

bxWv=max0, 4c-min0, 4c+11                                               bxHv=max0, 41+d -min0, 41+d +11

Where the affine motion model is defined as:

vx=ax+cy+evy=bx+dy+f

The thresholds for a PU coded in unidirectional and bidirectional affine mode are defined as:

Thred4=(15+δx)*(15+δy)Thredh=(15+δx)*(11+δy)Thredv=(11+δx)*(15+δy)

### CE2-4.9: Integer motion compensation for regular inter mode (JVET-N0210, PKU/DJI)

JVET-M0348 suggest applying integer motion compensation to 4x8/8x4 bi-directional inter prediction to reduce VVC worst case memory. For 4x4/4x8/8x4 AMVP bi-directional inter CUs, only full-pixel and 4-pixel motion vector resolutions that exist in AMVR are used. For 4x8/8x4 bi-directional inter merge CU, only full-pixel motion vector resolutions are used. At inter merge case, motion vectors for merge list are rounded to full pixel during list construction.

Besides integer motion compensation for regular inter mode, the proposed integer motion compensation will also be extended to affine mode to reduce worst case memory for affine mode.

### CE2-4.10: Constraint on constructed affine MV range (JVET-N0379, Tencent)

It is proposed to apply a constraint on the range of the affine parameter values for the constructed affine merge candidates, so that candidates with impractical affine models will not be used.

Take 4-parameter affine as an example. CPMV0 only has the translational portions of the affine model, which are represented by c and f. By subtracting the horizontal and vertical components of CPMV1 from CPMV0 separately, the delta part will consist of only the affine parameters and the block width:

MVx1-MVx0=a∙WMVy1-MVy0=-b∙W

Getting the ratio of the absolute values of deltas in (5) with the width, it can represent the range of the values of affine parameters a and b:

R1x=MVx1-MVx0W=aR1y=MVy1-MVy0W=b

Similarly, for 6-parameter affine, we can derive the ratios as:

R1x=MVx1-MVx0W=aR1y=MVy1-MVy0W=dR2x=MVx2-MVx0H=bR2y=MVy2-MVy0H=e

A threshold to limit the maximum ratio in {R1x, R1y, R2x, R2y} so that the CPMV value range is limited with limited impact on coding performance.

maxR1x,R1y,R2x,R2y<Threshold

It will also be tested to apply the proposed constraint on all affine blocks.

# CE2.5: Storage Reduction for Sub-block Modes

## Categorization & comparative study

### Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 2-5.1a | Replace spatial affine inheritance with inheritance from affine HMVP | JVET-N0377 |
| 2-5.1b | Test 2-5.1a + affine inheritance from line buffer |  |
| 2-5.2a | Directly derive sub-block MV by using affine inheritance merge candidate | JVET-N0257 |
| 2-5.2b | Test 2-5.2a + Affine inheritance using sub-block MVs |  |
| 2-5.2c | Test 2-5.2b + Shifting affine CPMVs by (2,2) |  |
| 2-5.2d | Test 2-5.2d + Changing neighboring block checking order for top-right and bottom-left CPMVs prediction in the constructed affine MVP |  |
| 2-5.2e | Test 2-5.2d + 16x16 grid affine inheritance |  |
| 2-5.3 | 2 or 3 sub-block MVs with half CU height or half CU width for affine inheritance | JVET-N0076 |
| 2-5.4 | Affine parameters storage to replace CPMVs and the block dimensions storage | JVET-N0272 |
| 2-5.5a | Parameter-based HMVP affine candidates | JVET-N0263 |
| 2-5.5b | Test 2-5.5a + 2.5.1.a |  |
| 2-5.5c | Test 2-5.5a + 2.5.1.b |  |
| 2-5.6a | Test 2-5.3 + Two subblock MVs with a fixed distance (4) for affine inheritance | JVET-N0078 |
| 2-5.6b | Test 2-5.4 + Two subblock MVs with a fixed distance (8) for affine inheritance |  |
| 2-5.6c | Test 2-5.4 + Two subblock MVs with a fixed distance (16) for affine inheritance |  |

### Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 2-5.1a | 0.08% | 0.00% | 0.02% | 100% | 100% | 0.09% | 0.10% | -0.22% | 100% | 98% |
| 2-5.1b | 0.03% | 0.00% | -0.02% | 99% | 100% | 0.05% | 0.05% | 0.08% | 100% | 98% |
| 2-5.2a | 0.00% | -0.06% | -0.02% | 100% | 100% | 0.00% | 0.18% | -0.39% | 100% | 100% |
| 2-5.2b | 0.01% | -0.08% | -0.05% | 100% | 100% | 0.01% | -0.05% | -0.10% | 100% | 99% |
| 2-5.2c | 0.03% | -0.04% | 0.05% | 100% | 100% | 0.04% | 0.22% | -0.17% | 101% | 99% |
| 2-5.2d | 0.01% | 0.01% | 0.02% | 1001% | 1001% | 0.01% | -0.04% | -0.11% | 100% | 99% |
| 2-5.2e | 0.02% | -0.01% | 0.05% | 100% | 100% | 0.01% | 0.04% | -0.04% | 100% | 100% |
| 2-5.3 | 0.01% | -0.07% | -0.02% | 100% | 100% | 0.02% | 0.13% | -0.32% | 99% | 99% |
| 2-5.4 | 0.02% | -0.04% | 0.01% | 100% | 100% | 0.00% | 0.02% | -0.14% | 100% | 100% |
| 2-5.5a | -0.06% | -0.07% | -0.08% | 101% | 101% | -0.02% | -0.13% | -0.12% | 100% | 101% |
| 2-5.5b | 0.00% | -0.04% | -0.02% | 100% | 100% | 0.10% | 0.07% | -0.07% | 100% | 99% |
| 2-5.5c | -0.05% | -0.11% | -0.12% | 100% | 100% | 0.02% | -0.02% | -0.13% | 99% | 99% |
| 2-5.6a | 0.02% | -0.05% | -0.04% | 100% | 100% | 0.01% | -0.12% | 0.03% | 99% | 101% |
| 2-5.6b | 0.00% | -0.08% | 0.04% | 100% | 100% | 0.02% | -0.02% | 0.10% | 100% | 99% |
| 2-5.6c | 0.00% | -0.04% | -0.01% | 100% | 99% | -0.02% | 0.03% | -0.25% | 100% | 101% |

### Complexity analysis

| Test# | Local storage (bits per CTU: 36 bits per CPMV, 3 bits for CU width, 3 bits for CU height, 10 bits for CU top-left x and y, 1 bit for affine type) | Line buffer for (bits per 8x8) | Additional logics |
| --- | --- | --- | --- |
| VTM4.0 | 11072 =128/8 * (36*3*2+5+10+1) + 128/8 * 2*(36*3*2+3*1+10+1) | 8 (3 bits for width, 5 bits for CU x position) |  |
| 2-5.1a | 1165 = (5*(36*3*2+3*2+10+1)) | 0 bit | Affine HMVP table update |
| 2-5.1b | 1165 = (5*(36*3*2+3*2+10+1)) | Same | Affine HMVP table update |
| 2-5.2a | 11072 =128/8 * (36*3*2+5+10+1) + 128/8 * 2*(36*3*2+3*1+10+1) | Same | No |
| 2-5.2b | 3152=((128/8*2+2) * 36*2)+128/8 * (5+10+1) + 128/8 * 2*(3*1+10+1) | Same | No |
| 2-5.2c | 3152=((128/8*2+2) * 36*2)+128/8 * (5+10+1) + 128/8 * 2*(3*1+10+1) | Same | No |
| 2-5.2d | 3152=((128/8*2+2) * 36*2)+128/8 * (5+10+1) + 128/8 * 2*(3*1+10+1) | Same | No |
| 2-5.2e | 1648=(128/16*2+2) * 36*2)+128/16 * (5+10+1) + 128/16 * 2*(3*1+10+1) | 8 per 16x16 | No |
| 2-5.3 | 4160 =128/8 * (36*2+5+10+1) + 128/8 * 2*(36*2+3*1+10+1) | Same | No |
| 2-5.4 | 3072 = 48*(8*4*2) | Same | No |
| 2-5.5a | 11392= 320 + 128/8 * (36*3*2+5+10+1) + 128/8 * 2*(36*3*2+3*1+10+1) | Same | Affine HMVP table update (No Pruning |
| 2-5.5b | 320=(10*4*8) | 0 | Affine HMVP table update (No Pruning |
| 2-5.5c | 320=(10*4*8) | Same | Affine HMVP table update (No Pruning |
| 2-5.6a | Same | 0 | No |
| 2-5.6b | Same |  |  |
| 2-5.6c | Same | 0 | No |

### Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 2-5.1a | F. Le Léannec (Technicolor) | 2 | P | Y | M |
| 2-5.1b | F. Le Léannec (Technicolor) | 2 | P | Y | M |
| 2-5.2a | A. Tamse (Samsung) | 2 | P | Y | M |
|  | H. Chen (Huawei) | 2 | P | Y | M |
| 2-5.2b | A. Tamse (Samsung) | 2 | P | Y | M |
|  | H. Chen (Huawei) | 2 | P | Y | M |
| 2-5.2c | A. Tamse (Samsung) | 2 | P | Y | M |
|  | H. Chen (Huawei) | 2 | P | Y | M |
| 2-5.2d | A. Tamse (Samsung) | 2 | P | Y | M |
|  | H. Chen (Huawei) | 2 | P | Y | M |
| 2-5.2e | A. Tamse (Samsung) | 2 | P | Y | M |
|  | H. Chen (Huawei) | 2 | P | Y | M |
| 2-5.3 | H. Chen (Huawei) | 2 | F | Y | M |
| 2-5.4 | G. Li (Tencent) | 2 | F | Y | M |
| 2-5.5a* | P. Yin (Dolby) | 2 | P | Y | M |
|  | H. Chen (Huawei) | 2 | F | Y | M |
| 2-5.5b* | P. Yin (Dolby) | 2 | P | Y | M |
|  | H. Chen (Huawei) | 2 | F | Y | M |
| 2-5.5c* | P. Yin (Dolby) | 2 | F | Y | M |
|  | H. Chen (Huawei) | 2 | F | Y | M |
| 2-5.6a | Y.-C. Yang (Foxconn) | 2 | F | Y | M |
| 2-5.6b | H. Huang (Qualcomm) | 2 | P | Y | M |
| 2-5.6c | H. Huang (Qualcomm) | 2 | P | Y | M |
| * It was reported from a cross-checker that max number of potential inheritance candidates is increased from 2 to 5; 2 iterations in adding inheritance candidates. |  |  |  |  |  |

## Technical description

### CE2-5.1: Affine HMVP with modified affine inheritance (JVET-N0377, Tencent/LGE)

Affine HMVP is proposed in this test. After coding an affine coded CU, motion information of the current CU is used to update the affine HMVP table. Similar to the regular HMVP table updating process, and the table is reset at the beginning of a CTU row.

The following tests will be conducted:

- The affine merge and AMVP candidates inherited from spatial candidates are replaced by inheritance from candidates from affine HMVP. Entries in Affine HMVP tables are checked starting from the latest entry, and only if an Affine HMVP candidate is neighboring to the current CU, it is used to derive the inherited candidate. Whether the HMVP is neighbor of the current CU can be determined because position and size information are stored in the affine HMVP table. For the HMVP that are identified as neighbors of the current CU, its position, width, height and CPMVs are then used to derive the CPMVs of the current CU in the same way as current affine inheritance method.

- Since affine HMVP is reset per CTU row, a combined test adds the original affine inheritance from motion data line buffer when neighboring candidate is above the current CTU boundary, in addition to the inheritance from affine HMVP. Keeping this model inheritance method does not require additional local buffer for storing CPMVs. The existing line buffer can be utilized.

### CE2-5.2: Alignment of affine control-point motion vector and subblock motion vector (JVET-N0257, Qualcomm)

In this test, the subblock motion vectors are used as the affine control-point motion vectors (CPMVs). The affine motion inheritance uses the subblock motion vectors from the neighboring coded block, and they are not necessary control-point motion vectors. And affine motion inheritance only from neighboring coded block that has both width and height larger or equal to 16.

The following tests will be conducted:

- Directly derive subblock MV in affine inheritance merge candidate.

- On top of Test a, the bottom-right, bottom-middle and right-middle subblock MVs of neighboring coded block (as shown in the figure below) are used to derive the predicted CPMVs of current block, such that the distance between v0 and v1 and v0 and v2 are nW/2 and nH/2, wherein nW/2 and nH/2 are the width and height of the neighboring block and both remain power of 2. If the neighboring block locates at the above CTU, then only v0 and v2 are used for affine motion inheritance and the 4-parameter affine motion model is assumed.

- On top of Test b, the affine CPMVs are set as shown in the above figure. The top-left CPMV v0 is the same as the MV of the top-left subblock. The top-right and bottom-left CPMVs locate at two virtual subblocks at the extended subblock column and subblock row of the current block, such that the distance between v0 and v1 and v0 and v2 remain blkW and blkH (power of 2), respectively. The motion vector for the subblock at ith row and jth column is derived as

| vx=(v1x-v0x)blkW*j*sbW+v2x-v0xblkH*i*sbH+v0xvy=(v1y-v0y)blkW*j*sbW+v2y-v0yblkH*i*sbH+v0y |  |
| --- | --- |

- On top of Test c, changing neighboring block checking order for top-right and bottom-left CPMVs prediction in the constructed affine MVP.

- On top of Test d, affine motion inheritance only from neighboring coded block that has both width and height larger or equal to 16.

### CE2-5.3: Simplifications for inherited affine merging candidates (JVET-N0076, Mediatek)

To remove the local buffer for control point motion vector (CPMV), inside the current CTU row (i.e., not at CTU row boundaries), it is proposed to use two or three subblock MVs with half CU height or half CU width for affine candidate inheritance, where CPMVs are not needed anymore for saving the CPMV buffer.

### CE2-5.4: Parameter-based affine model inheritance (JVET-N0272, Bytedance)

It is proposed to store affine parameters instead of three CPMVs and the block dimensions. When the current CU applies affine-inheritance merge mode, affine parameters are directly copied from the neighbouring 4×4 block B to be inherited. And the MV of each sub-block in the current CU is derived by eq. (2) with the center position of B as the base position and MVB as the base MV. When the current CU applies affine AMVP mode, CPMVs of the current CU are derived by eq. (2) also with the center position of B as the base position and MVB as the base MV, and the derived CPMVs will serve as the MVPs.

### CE2-5.5: History-parameter-based affine model inheritance (JVET-N0263, Bytedance)

It is proposed to add History-based Affine Merge Candidates (HAMC) into the sub-block-based merge candidate list. After decoding an affine-coded CU, a set of affine parameters {a, b, c, d} are put into an affine parameter history table.

An HAMC is derived by combining a set of affine parameters stored in the table and the MV of a neighbouring 4×4 block served as the base MV. In a formulating way, the MV of the current block at position (x, y) is calculated as:

,

where (mvhbase, mvvbase) represents the MV of a neighbouring 4×4 block, (xbase, ybase) represents the center position of the neighbouring 4×4 block. (x, y) can be the top-left, top-right and bottom-left corner of the current block to obtain the CPMVs.

### CE2-5.6: Simplifications for inherited affine merging candidates at CTU row boundaries (JVET-N0078, Mediatek/Bytedance)

To remove line buffer for coding unit width and x position information, simplifications for inherited affine candidates are proposed. At CTU row boundaries, it is proposed to use two subblock motion vectors (MVs) with a fixed distance for affine candidate inheritance, where an accurate distance is used to derive the affine motion and CU width and x position information is not needed anymore for saving the CU width and x position information line buffer.

# Recommendations

Review CE2 document if needed and review all CE2-related documents.