| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0023-v2 |
| --- | --- |

| Title: | CE3: Summary Report on Intra Prediction and Mode Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera Ling Li Alexey Filippov | Email: | geertv@qti.qualcomm.com l.li@lge.com Alexey.Filippov@huawei.com |
| Source: | CE3 coordinators |  |  |

_____________________________

# Abstract

This is the summary report of the third Core Experiment (CE3) [1]. The goal of CE3 is to study intra prediction tools, including mode coding, for potential inclusion into the VVC standard.

The following is the list of defined sub-tests in CE3:

- CE3-1: Cross-component prediction (18 tests)

- CE3-2: Small block size restrictions (5 tests)

- CE3-3: MPM list harmonization (12 tests)

- CE3-4: Affine linear weighted intra prediction (2 tests)

This document summarizes the objective results (BD-rates, runtimes), cross-check reports, and related input contributions.

# Test conditions and guidelines

CTC conditions as agreed at the 13th JVET meeting (Marrakech, MA) for SDR (JVET-M1010) are followed for the evaluation of objective performance. The anchor is the ‘VVC Test Model’ or VTM. Results are provided for the ‘All Intra’ and ‘Random Access’ test configurations for all sub-tests, and additionally the ‘Low Delay B’ configuration for sub-test CE3-2. The VTM version 4.0 software is used with ‘vtm’ configuration files to produce the VTM anchor. Tools are implemented into the VTM version 4.0 software.

In addition to VTM-4.0, a modified codebase is also used for evaluating the tests in CE3-3. In the codebase, a macro is added [1] on top of VTM-4.0 to always include all MPMs (6 MPMs in VTM-4.0) for full encoder R-D checking in intra mode decision. For evaluating the coding performance of each CE3-3 test, following Test A and Test B results are reported:

Test A:

- Anchor: VTM-4.0

- Tested: VTM-4.0 with test method

Test B:

- Anchor: Codebase

- Tested: Codebase with test method

Separate test results (A and B) are provided for modifications to the encoder search algorithm other than the one described above.

Proposals are compared with respect to objective compression efficiency while complexity is also assessed.

The source codes of the tests and full test results are uploaded by proponents into the following CE3 GitLab repository:

https://vcgit.hhi.fraunhofer.de/JVET-M-CE3/VVCSoftware_VTM.git

# Cross-check reporting

Each test is covered by at least one cross-checker and the cross-check reports are included within the respective sub-test sections. Within CE3 it was agreed that cross-checkers answer the following questions:

- Did the cross-checker study the software? If so, did the cross-checker find any significant inconsistencies between the description and the provided software?

- Do the cross-checked results closely match the proponent results for the test conditions?

- Were additional results cross-checked?

- Additional comments that may be helpful to the group?

This report summarizes the answers in tables reporting whether the objective test results match in one table column and all other comments in a second table column.

# Overview of CE3 test description document changes

The following changes were made to the CE3 test description document [1] after the T1 deadline (8 February 2019) had passed. Besides changes to contact persons and assignment of cross-checkers, the following changes:

- CE3-2:

  - Additional ‘Low Delay B’ test configuration results to be provided.

- CE3-3:

  - Specific test conditions for this sub-test were clarified as follows: “Separate test results (A and B) shall be provided for modifications to the encoder search algorithm other than the one described above.”

  - Test 3-3.2.3 was withdrawn.

# CE3-1 on ‘Cross-component prediction’

## Defined tests [1]

### CCLM simplifications

| Test # | Short description | Doc. # |
| --- | --- | --- |
| 3-1.1.1 | Selection of the up to 6 samples is applied to the down-sampled luma samples | JVET-N0387 (Technicolor) |
| 3-1.2.1 | Reduction of reference samples to N points for CCLM (CCLM_LT, CCLM_T, and CCLM_L), using an offset, with N being set according to the size of the block | JVET-N0131 (Xidian Univ., NPU, OPPO) |
| 3-1.2.2 | Reduction of reference samples to N points for CCLM (CCLM_LT, CCLM_T, and CCLM_L), using an offset, with N being set fixed (i.e., uniform for all block sizes) |  |
| 3-1.3.1 | Skipping initial P reference samples for CCLM_L and CCLM_T, with P being set according to the size of the block | JVET-N0130 (NPU, Xidian Univ., OPPO) |
| 3-1.3.2 | Test 3-1.3.1 + Test 3-1.2.1 |  |
| 3-1.3.3 | Test 3-1.3.1 + Test 3-1.2.2 |  |
| 3-1.4.1 | CCLM parameter calculation with half number of reference samples | JVET-N0228 (LGE) |
| 3-1.4.2 | CCLM parameter calculation with half and upper-limited reference samples |  |
| 3-1.5 | CCLM model derived from four neighbouring luma samples | JVET-N0271 (Bytedance, Peking Univ.) |
| 3-1.6.1 | Identify the min/max luma samples without down-sampling process; apply the 6-taps filter on the identified min/max samples to generate the filtered (down-sampled) min/max luma samples for the derivation of α and β. | JVET-N0321 (Kwai) |
| 3-1.6.2 | Combination of test 3-1.6.1 with JVET-M0108 (test 3-1.1.1) |  |
| 3-1.6.3 | Combination of test 3-1.6.1 with JVET-M0211 (test 3-1.2.2) |  |
| 3-1.6.4 | Combination of test 3-1.6.1 with JVET-M0211+JVET-M0212 (test 3-1.3.3) |  |
| 3-1.6.5 | Combination of test 3-1.6.1 with JVET-M0219 (test 3-1.4.2) |  |
| 3-1.6.6 | Combination of test 3-1.6.1 with JVET-M0274 (test 3-1.5) |  |

An overview of the tested methods for reducing reference samples used in calculation of CCLM model parameters is provided in following table:

| Test # | #ref samples LT_CCLM | #ref_samples T_CCLM | #ref_samples L_CCLM | Locations | #comparison operations |
| --- | --- | --- | --- | --- | --- |
| VTM4 | Up to 2*min(W,H) (if avail.) | Up to 2*W (if avail.) | Up to 2*H (if avail.) | Note: W, H are chroma TB dimensions | 2*#ref_samples |
| 3-1.1.1 | 6 (3 top line, 3 left) | 3 | 3 | #ref samples LT_CCLM = nS_T + nS_L #ref_samples T_CCLM = nS_T #ref_samples L_CCLM = nS_L Top: (0, – 1), (nS_T – 1, – 1), (nS_T/2, – 1) Left: (– 1, 0), (– 1, nS_L – 1), (– 1, nS_L/2) |  |
| 3-1.2.1 | 2 (min(W,H) == 2) or 4 | 2 (W == 2) or 4 | 2 (H == 2) or 4 | Example: -Original reference samples number: L=16 -Reduced reference samples number: N=4 -Sampling interval: ∆=L/N=4 -Starting offset = ∆/2 =2 | 4 (min(W,H) == 2) or 8 |
| 3-1.2.2 | 4 | 4 | 4 |  | 8 |
| 3-1.3.1 | Idem VTM4 | Up to ¾ (2*W) | Up to ¾ (2*H) | Skipping initial quarter of reference samples for CCLM_L and CCLM_T (VTM4) | -LT_CCLM: Idem VTM4 -L_CCLM: Up to 3W -T_CCLM: Up to 3H |
| 3-1.3.2 | 4 | 2 (if after offset, #samples remaining is <4) or 4 | 2 (if after offset, #samples remaining is <4) or 4 | -LT_CCLM: 4 -L_CCLM and _T_CCLM: Initial offset is set as the first quarter of the reference samples (skipped). Then, reference samples are selected according to the method in 3-1.2.1 | Up to 8 |
| 3-1.3.3 | 4 | 4 | 4 | -LT_CCLM: 4 -L_CCLM and _T_CCLM: Initial offset is set as the first quarter of the reference samples (skipped). Then, reference samples are selected according to the method in 3-1.2.2 | 8 |
| 3-1.4.1 | Up to min(W,H) (if avail.) | Up to W (if avail.) | Up to H (if avail.) | Half of VTM4 reference samples are used | #ref_samples |
| 3-1.4.2 | Up to min(W,H) (if avail.) (max. 8) | Up to W (if avail.) (max. 8) | Up to H (if avail.) (max. 8) | Half of VTM4 reference samples are used, up to 8 max. | #ref_samples (up to 16) |
| 3-1.5 | 4 | 4 | 4 | - LT_CCLM: W’=W, H’=H - T_CCLM: W’=W+H - L_CCLM: H’=H+W 4 samples are selected as: - LT_CCLM: S[W’/4, -1], S[3W’/4, -1], S[-1, H’/4], S[-1, 3H’/4] - T_CCLM: S[W’/8, -1], S[3W’/8, -1], S[5W’/8, -1], S[7W’/8, -1] - L_CCLM: S[-1, H’/8], S[-1, 3H’/8], S[-1, 5H’/8], S[-1, 7H’/8] 4 neighbouring luma samples at the selected positions are down-sampled and compared four times to find two smaller values: x0A and x1A, and two larger values: x0B and x1B. Corresponding chroma sample values are denoted as y0A, y1A, y0B and y1B. Then xA, xB, yA and yB are derived as for model calc.: xA=(x0A + x1A +1)>>1; xB=(x0B + x1B +1)>>1; yA=(y0A + y1A +1)>>1; yB=(y0B + y1B +1)>>1. | 4 |
| 3-1.6.1 | Method applied to VTM4 |  |  | Reconstructed luma samples without down-sample filtering to find the min/max luma samples, followed by filtering before model parameter calc. | Same as VTM4 |
| 3-1.6.2 | Method applied to test 3-1.1.1 |  |  |  | Same as 3-1.1.1 |
| 3-1.6.3 | Method applied to test 3-1.2.2 |  |  |  | Same as 3-1.2.2 |
| 3-1.6.4 | Method applied to test 3-1.3.3 |  |  |  | Same as 3-1.3.3 |
| 3-1.6.5 | Method applied to test 3-1.4.2 |  |  |  | Same as 3-1.4.2 |
| 3-1.6.6 | Method applied to test 3-1.5 |  |  |  | Same as 3-1.5 |

Continuation of above table:

| Test # | # comparison operation in worst case | # downsampling samples in worst case |
| --- | --- | --- |
| VTM4 | 8 | 4 |
| 3-1.1.1 | 5 | 4 |
| 3-1.2.1 | 4 | 4 |
| 3-1.2.2 | 8 | 4 |
| 3-1.3.1 | 8 | 4 |
| 3-1.3.2 | 4 | 4 |
| 3-1.3.3 | 8 | 4 |
| 3-1.4.1 | 4 | 2 |
| 3-1.4.2 | 4 | 2 |
| 3-1.5 | 8 | 4 |
| 3-1.6.1 | 8 | 2 |
| 3-1.6.2 | 8 | 2 |
| 3-1.6.3 | 8 | 2 |
| 3-1.6.4 | 8 | 2 |
| 3-1.6.5 | 4 | 2 |
| 3-1.6.6 | 8 | 2 |

### Multi-model LM

| Test # | Short description | Doc. # |
| --- | --- | --- |
| 3-1.7 | Multiple-model LM with small block size restriction | JVET-N0264 (Foxconn, Qualcomm) |
| 3-1.8.1 | Multiple-model LM using piecewise linear model and block size restriction | JVET-N0241 (Canon) |
| 3-1.8.2 | More stringent block size restriction on CCLMs with JVET-M0384 included. |  |

Summary of the tested methods:

|  | CCLM | CE3-1.7 | CE3-1.8.1 | CE3-1.8.2 |
| --- | --- | --- | --- | --- |
| Signalling | 3 modes | 6 modes | 3 modes (CCLM or MMLM depending on block size) | 3 modes (CCLM or MMLM depending on block size) |
| Number of lines used for model derivation | Y: 2 lines above, 3 lines left, C: 1 line above and left); 1 line above at CTU boundary | idem | idem | idem |
| Chroma block sizes applicable | nSamples >= 4 (all block sizes) | CCLM: nSamples >= 4 (all sizes) MMLM: nSamples > 16 | CCLM: 4 <= nSamples < 16 MMLM: nSamples >= 16 | CCLM: 16 <= nSamples < 64 MMLM: nSamples >= 64 |

## CE3-1: Test results

### CCLM simplifications test results

|  | All Intra Main10 - Over VTM-4.0 |  |  |  |  | Random Access Main10 - Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 3-1.1.1 | 0.08% | 0.14% | 0.17% | 98% | 97% | 0.02% | -0.02% | 0.01% | 100% | 99% |
| 3-1.2.1 | -0.03% | -0.11% | -0.21% | 100% | 100% | -0.01% | -0.24% | -0.27% | 100% | 100% |
| 3-1.2.2 | -0.03% | -0.24% | -0.31% | 100% | 99% | -0.01% | -0.35% | -0.46% | 100% | 100% |
| 3-1.3.1 | -0.02% | -0.06% | -0.13% | 100% | 99% | -0.01% | -0.11% | -0.20% | 100% | 100% |
| 3-1.3.2 | -0.02% | 0.02% | -0.08% | 100% | 99% | 0.01% | -0.07% | -0.18% | 100% | 100% |
| 3-1.3.3 | -0.03% | -0.06% | -0.17% | 100% | 99% | -0.01% | -0.23% | -0.40% | 100% | 100% |
| 3-1.4.1 | 0.00% | 0.07% | 0.03% | 100% | 100% | 0.00% | 0.08% | -0.04% | 101% | 101% |
| 3-1.4.2 | 0.00% | 0.04% | 0.01% | 101% | 100% | 0.01% | 0.04% | -0.12% | 101% | 101% |
| 3-1.5 | -0.05% | -0.30% | -0.38% | 99% | 100% | -0.05% | -0.36% | -0.63% | 100% | 100% |
| 3-1.6.1 | 0.02% | 0.19% | 0.22% | 99% | 97% | 0.02% | 0.22% | 0.14% | 98% | 98% |
| 3-1.6.2 | 0.05% | 0.32% | 0.33% | 99% | 98% | 0.02% | 0.16% | 0.16% | 99% | 99% |
| 3-1.6.3 | -0.02% | -0.07% | -0.13% | 100% | 98% | 0.00% | -0.18% | -0.27% | 100% | 99% |
| 3-1.6.4 | -0.02% | 0.08% | -0.05% | 100% | 99% | 0.00% | -0.02% | -0.13% | 99% | 98% |
| 3-1.6.5 | 0.01% | 0.18% | 0.13% | 100% | 100% | 0.02% | 0.11% | 0.08% | 99% | 99% |
| 3-1.6.6 | 0.00% | 0.20% | 0.08% | 101% | 100% | 0.00% | 0.20% | -0.01% | 100% | 100% |

### Multi-model LM test results

|  | All Intra Main10 - Over VTM-4.0 |  |  |  |  | Random Access Main10 - Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 3-1.7 | -0.27% | -1.56% | -2.13% | 100% | 100% | -0.13% | -1.29% | -1.75% | 100% | 100% |
| 3-1.8.1 | -0.30% | -0.97% | -1.45% | 100% | 102% | -0.16% | -0.97% | -1.39% | 100% | 111% |
| 3-1.8.2 | -0.26% | -0.63% | -0.99% | 100% | 104% | -0.11% | -0.47% | -0.70% | 100% | 111% |

## CE3-1: Cross-check reports

| Test# | Cross-checker | Results match (y/n) | Cross-checker comments |
| --- | --- | --- | --- |
| 3-1.1.1 | Yi-Wen Chen (Kwai) | y | Software was reviewed; no significant inconsistencies between description and software; results partially cross-checked |
| 3-1.2.1 | Xiang Ma (Huawei) | y | Software was reviewed; no significant inconsistencies between description and software; full results cross-checked |
| 3-1.2.2 |  |  |  |
| 3-1.3.1 | Xiang Ma (Huawei) | y | Software was reviewed; no significant inconsistencies between description and software; full results cross-checked |
| 3-1.3.2 |  |  |  |
| 3-1.3.3 |  |  |  |
| 3-1.4.1 | Yi-Wen Chen (Kwai) | y | Software was reviewed; no significant inconsistencies between description and software; full results cross-checked |
| 3-1.4.2 |  |  |  |
| 3-1.5 | Yi-Wen Chen (Kwai) | y | Software was reviewed; no significant inconsistencies between description and software; full results cross-checked |
| 3-1.6.1 | Jangwon Choi (LGE) | y | There are no inconsistencies between the CE description and the uploaded CE software. Encoding/decoding time difference slightly (~1% ). |
| 3-1.6.2 | Kai Zhang (Bytedance) |  |  |
| 3-1.6.3 | Luong Pham Van (Qualcomm) | y | There are no inconsistencies between the CE description and the uploaded CE software. Encoding/decoding time of the cross-checker is incorrect. |
| 3-1.6.4 | Edouard Francois (Technicolor) | y | Software was reviewed; no significant inconsistencies between description and software; full results cross-checked |
| 3-1.6.5 | Luong Pham Van (Qualcomm) | y | There are no inconsistencies between the CE description and the uploaded CE software. Encoding/decoding time of the cross-checker is incorrect. |
| 3-1.6.6 | Jangwon Choi (LGE) | y | There are no inconsistencies between the CE description and the uploaded CE software. Encoding/decoding time difference slightly (~1% ). |
| 3-1.7 | Christophe Gisquet (Canon) | y | Software was reviewed; no significant inconsistencies between description and software; full results cross-checked |
| 3-1.8.1 | Adarsh K. Ramasubramonian (Qualcomm) | y | I studied the software and I find no inconsistencies between the description and the provided software. The PSNR match exactly; minor mismatch in bitrates in two rate points but likely due to rounding used by the tester and the cross-checker in accumulating RA results. |
| 3-1.8.2 |  | y |  |

## CE3-1: Related contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |
| JVET-N0333 (Kwai) | 3-1.7/1.8 | CE3-related: MMLM only cross-component prediction |

# CE3-2 on ‘Small chroma block restrictions’

## Defined tests [1]

| Test # | Short description | Doc. # |
| --- | --- | --- |
| 3-2.1.1 | Disable 2x2/4x2/2x4 in dual tree only | JVET-N0137 (Sharp) |
| 3-2.1.2 | Disable 2x2/4x2/2x4 in dual tree and use DC only for 2x2/4x2/2x4 intra chroma prediction |  |
| 3-2.1.3 | 3-2.1.1 + 3-2.2 | JVET-N0140 (Sharp, MediaTek) |
| 3-2.2 | Share reference samples for at least 16 chroma samples in both dual and shared tree | JVET-N0081 (MediaTek) |
| 3-2.3 | Disable intra chroma 2x2, 2x4 and 4x2 in dual tree. And use a 2x8 or 8x2 coefficient group size instead of a 2x2 coefficient group size on remaining intra chroma blocks which has width or height size as 2. | JVET-N0102 (Canon) |

Summary table of the methods per tree type:

| Test | Shared trees | Separated trees |
| --- | --- | --- |
| 3-2.1.1 |  | Disables 2x2, 2x4, and 4x2 |
| 3-2.1.2 | DC mode for small blocks | Disables 2x2, 2x4, and 4x2 |
| 3-2.1.3 | Shared reference sample in parallel regions with minimum size of 16 | Disables 2x2, 2x4, and 4x2 |
| 3-2.2 | Shared reference sample in parallel region with minimum size of 16 |  |
| 3-2.3 | Blocks of width or height of 2 use a 2x8 or 8x2 coefficient group size, respectively | Disables 2x2, 2x4, and 4x2 |

## CE3-2: Test results

|  | All Intra Main10 - Over VTM-4.0 |  |  |  |  | Random Access Main10 - Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 3-2.1.1 | 0.02% | 0.37% | 0.49% | 100% | 100% | 0.01% | 0.30% | 0.36% | 101% | 101% |
| 3-2.1.2 | 0.02% | 0.37% | 0.49% | 100% | 100% | 0.10% | 0.61% | 0.74% | 100% | 101% |
| 3-2.1.3 | 0.02% | 0.37% | 0.49% | 101% | 101% | 0.06% | 0.46% | 0.62% | 101% | 102% |
| 3-2.2 | 0.03% | 0.30% | 0.33% | 100% | 101% | 0.06% | 0.29% | 0.48% | 101% | 101% |
| 3-2.3 | 0.02% | 0.33% | 0.47% | 96% | 94% | 0.02% | 0.24% | 0.32% | 100% | 99% |

|  | Low Delay B Main10 - Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test # | Y | U | V | EncT | DecT |
| 3-2.1.1 | 0.00% | 0.18% | 0.15% | 100% | 101% |
| 3-2.1.2 | 0.07% | 0.66% | 0.53% | 100% | 101% |
| 3-2.1.3 | 0.04% | 0.44% | 0.42% | 101% | 101% |
| 3-2.2 | 0.02% | 0.43% | -0.20% | 100% | 100% |
| 3-2.3 | -0.02% | 0.22% | 0.24% | 100% | 100% |

## CE3-2: Cross-check reports

| Test # | Cross-checker | Results match (y/n) | Cross checker comment |
| --- | --- | --- | --- |
| 3-2.1.1 | Zhi-Yi Lin (MediaTek) | y | The software was reviewed. The results are matched, and the implementation is the same as the description. |
| 3-2.1.2 |  | y |  |
| 3-2.1.3 | Chris Rosewarne (Canon) | y | The software was studied and found to be a straightforward combining of CE3-2.2 and CE3-2.1.1 software, as expected from the CE description. The simulation results are exactly matching those provided by the tester. No additional testing was performed beyond the mandatory testing. |
| 3-2.2 |  | y | The software was studied and the approach taken to implement the parallel area appears as expected. Parallel area appears independent of single or dual tree, as expected. Due to VTM architecture the changes are somewhat involved, but no implementation errors were identified. The simulation was performed on software prior to one bug fix commit (made prior to software freezing deadline). Nevertheless, cross-check results are matching within 0.0x% across all classes and configs, and the revised software is being simulated for final confirmation. |
| 3-2.3 | T. Zhou (Sharp) | y | Software was reviewed; no significant inconsistencies between description and software; full results cross-checked |

## CE3-2: Related contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |
| JVET-N0082 (MediaTek) | 3-2 | CE3-related: Constrained partitioning of chroma intra CBs |
| JVET-N0396 (Qualcomm) | 3-2.1.3 | CE3-related: Enabling parallel reconstruction of small intra-coded blocks |
| JVET-N0453 (Huawei) | 3-2 | AHG16/Non-CE3: on chroma 2xN and Nx2 intra prediction in single tree |
| JVET-N0465 (LGE) | 3-2 | CE3-related : Restrict 2xN, Nx2 chroma processing for dual tree structure |

# CE3-3 on ‘Harmonization of MPM lists’

## Defined tests [1]

| Test # | Short description | Doc. # |
| --- | --- | --- |
| 3-3.1.1 | Unified MPM list generation with changed order to include tools such as MRL, intra sub-partition coding, etc. | JVET-N0184 (Huawei, LGE) |
| 3-3.1.2 | Harmonization of luma intra mode coding (including MRL, intra sub-partitioning, etc.) | JVET-N0451 (LGE, Huawei) |
| 3-3.2.1 | Simplification of MPM derivation | JVET-N0134 (ETRI) |
| 3-3.2.2 | Simplification of MPM derivation with changed order described in JVET-M0295 |  |
| 3-3.2.3 | Withdrawn |  |
| 3-3.3.1 | Unification of MPM list generation with prioritization of Planar mode in MPM list | JVET-N0393 (Tencent) |
| 3-3.3.2 | Unification of MPM list generation with Planar and DC modes being first two MPMs of zero reference line, each of the first two bins of MPM index is coded using one context |  |
| 3-3.3.3 | Unification of MPM list generation with Planar and DC modes being first two MPMs of zero reference line, the first bin of MPM index is coded using more than one context |  |
| 3-3.4.1 | Planar/DC coded as separate modes | JVET-N0436 (Sharp) |
| 3-3.4.2 | Same as previous test, but with modified encoder search described in JVET-M0528 |  |
| 3-3.4.3 | Planar/DC coded as separate modes, use 4MPMs described in JVET-M0494 |  |
| 3-3.5.1 | Explicitly signal Planar&DC MPM list and intra_luma_ref_idx signalling only for angular mode | JVET-N0104 (Fujitsu) Comment: uploaded late on 17 Mar. |
| 3-3.5.2 | Test 3.3.5.1, use 4MPMs described in JVET-M0494 |  |

It was commented on the CE3 participants email list that full test results were due by T3 (4 March 2019), and the following is an overview of lateness as provided by one test proponent:

CE3-3.1.1: March 13 (9 days late), uploaded document

CE3-3.1.2: March 17 (13 days late), uploaded document

CE3-3.2: March 6 (2 days late), uploaded to gitlab; bug fixed version (Mar. 20).

CE3-3.3: March 10 (6 days late), uploaded to gitlab; bug fixed version (Mar. 20).

CE3-3.4: March 4 (on time), uploaded to gitlab

CE3-3.5: March 17 (13 days late), uploaded document

The following table provides a summary of some aspects of the CE3-3 methods.

|  | VTM-4.0 | 3.1.1 | 3.1.2 | 3.2.1 | 3.2.2 | 3.3.1 | 3.3.2 | 3.3.3 | 3.4.1 | 3.4.2 | 3.4.3 | 3.5.1 | 3.5.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # MPMs (regular) | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 2+6 | 2+6 | 2+4 | 2+4 | 2+4 |
| # MPMs (MRL) | 6 | 5 | 4 | 6 | 6 | 4 | 4 | 4 | 6 | 6 | 4 | 6 | 4 |
| # MPMs (ISP) | 6 | 6 | 5 | 6 | 6 | 5 | 5 | 5 | 1+6 | 1+6 | 1+4 | 1+5 | 1+4 |
| Re-index if fewer | No | Yes | Yes | No | No | Yes | Yes | Yes | NA | NA | NA | NA | NA |
| Context-coded | No | Yes (*) (1st bin) | No | No | No | No | Yes (*) (1st, 2nd bins) | Yes (*) (1st, 2nd bins) | Yes (**) | Yes (**) | Yes (**) | Yes (**) | Yes (**) |
| # contexts added | - | 3 | - | - | - | - | 1+1 | 3+1 | 10+2 | 10+2 | 10+2 | 5+1 | 5+1 |

(*) mpm_idx

(**) angular flags and planar_or_dc_flag

## CE3-3: Test results

### CE3-3: Test A (VTM-4.0 intra search)

| Test # | All Intra Main10 - Over VTM-4.0 |  |  |  |  | Random Access Main10 - Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 3-3.1.1 | -0.08% | -0.05% | -0.07% | 101% | 100% | -0.04% | 0.00% | -0.09% | 101% | 99% |
| 3-3.1.2 | -0.04% | 0.02% | 0.00% | 98% | 99% | 0.01% | 0.00% | -0.08% | 100% | 99% |
| 3-3.2.1 | 0.10% | 0.12% | 0.10% | 102% | 100% | 0.03% | 0.01% | -0.05% | 100% | 100% |
| 3-3.2.2 | -0.05% | 0.03% | 0.00% | 102% | 100% | -0.02% | 0.00% | -0.09% | 100% | 100% |
| 3-3.3.1 | -0.02% | 0.04% | 0.02% | 99% | 100% | 0.01% | 0.00% | -0.09% | 99% | 100% |
| 3-3.3.2 | -0.07% | -0.01% | 0.00% | 99% | 100% | -0.02% | -0.02% | -0.04% | 99% | 100% |
| 3-3.3.3 | -0.11% | -0.08% | -0.05% | 100% | 100% | -0.02% | -0.05% | 0.00% | 99% | 100% |
| 3-3.4.1 | -0.11% | -0.08% | -0.09% | 99% | 101% | -0.02% | -0.02% | -0.13% | 100% | 100% |
| 3-3.4.2 | -0.29% | -0.20% | -0.17% | 103% | 100% | -0.08% | -0.01% | -0.07% | 100% | 100% |
| 3-3.4.3 | -0.05% | -0.05% | -0.02% | 97% | 100% | 0.00% | -0.01% | -0.04% | 99% | 99% |
| 3-3.5.1 | -0.09% | -0.06% | -0.07% | 100% | 98% | -0.03% | -0.01% | -0.07% |  |  |
| 3-3.5.2 | -0.05% | -0.05% | -0.02% | 100% | 98% |  |  |  |  |  |

### CE3-3: Test B (VTM-4.0 intra search + all 6 MPM added to full RD)

| Test # | All Intra Main10 - Over VTM-4.0 |  |  |  |  | Random Access Main10 - Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 3-3.1.1 | 0.05% | -0.05% | -0.01% | 102% | 100% | 0.01% | 0.11% | 0.08% | 101% | 100% |
| 3-3.1.2 | 0.04% | 0.04% | 0.07% | 100% | 99% | 0.04% | 0.09% | 0.06% | 99% | 99% |
| 3-3.2.1 | 0.09% | 0.07% | 0.09% | 101% | 100% | 0.03% | 0.11% | -0.03% | 100% | 100% |
| 3-3.2.2 | 0.02% | 0.01% | 0.06% | 101% | 100% | 0.02% | 0.11% | -0.01% | 100% | 100% |
| 3-3.3.1 | 0.02% | 0.00% | 0.06% | 99% | 100% | 0.02% | 0.03% | -0.04% | 100% | 100% |
| 3-3.3.2 | -0.03% | -0.02% | 0.05% | 100% | 101% | -0.01% | 0.04% | -0.07$ | 100% | 100% |
| 3-3.3.3 | -0.06% | -0.05% | 0.01% | 100% | 100% | -0.02% | 0.05% | -0.02% | 100% | 100% |
| 3-3.4.1 | -0.13% | -0.15% | -0.08% | 99% | 101% | -0.04% | 0.10% | -0.04% | 100% | 100% |
| 3-3.4.2 | -0.13% | -0.14% | -0.08% | 100% | 101% | -0.05% | 0.07% | -0.04% | 99% | 99% |
| 3-3.4.3 | -0.07% | -0.09% | -0.05% | 99% | 100% | -0.02% | 0.08% | 0.02% | 99% | 98% |
| 3-3.5.1 | -0.12% | -0.12% | -0.03% | 100% | 99% |  |  |  |  |  |
| 3-3.5.2 | -0.07% | -0.10% | -0.05% | 100% | 100% |  |  |  |  |  |

The following table contains one analysis of complexity aspects of the CE3-3 test methods:

|  | Specify CABAC context coded bin and (its number of context model) | Neighbor information for context modeling | Number of bits needed per neighboring block for context modeling | Max number of available coded intra mode for a CU | Number of full RDO checks in total | Number of different MPM list | Max layers of if conditions | Max number of comparison operators in worst case | Max number of logical operators in worst case | Max number of assignment operators in worst case | Max number of increments operator in worst case | Lookup table size if any |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VTM4.0 When MRL idx is 0 and ISP flag is 0 | Mpm_flag: 1 (1) | NA |  | 6 | 3~5 | 5 | 3 | 7 | 2 | 19 | 0 | NA |
| VTM4.0 in total | Mpm_flag: 1 (1) | NA |  | 6 | 3~5 | 20 | 7 | 9 | 1 | 30 | 2 | NA |
| 3-3.1.1 | Mpm_flag 1 (1) Mpm_idx 1st bin 1 (3) | NA |  | 6 | 3~5 | 7 | 3 | 6 (MPM) + 2 （ctx ） | 1 | 13 + 1 (ctx) | 2 | NA |
| 3-3.1.2 | Mpm_flag: 1 (1) | NA |  | 6 | 3~5 | 6 | 3 | 7 | 1 | 14 | 0 | NA |
| 3-3.2.1 | Mpm_flag: 1 (1) | NA |  | 6 | 3~5 | 7 | 3 | 7 | 2 | 19 | 0 | NA |
| 3-3.2.2 | Mpm_flag: 1 (1) | NA |  | 6 | 3~5 | 7 | 3 | 7 | 2 | 19 | 0 | NA |
| 3-3.3.1 | Mpm_flag: 1 (1) | NA |  | 6 | 3~5 | 5 | 3 | 7 | 1 | 13 | 0 | NA |
| 3-3.3.2 | Mpm_flag: 1 (1) Mpm_idx 1st & 2nd bin 1 (2) | N/A |  | 6 | 3~5 | 5 | 3 | 7 (MPM) | 1(MPM) | 13 (MPM) | 0 | NA |
| 3-3.3.3 | Mpm_flag: 1 (1) Mpm_idx 1st & 2nd bin 1 (4) | 2 | 1 bit | 6 | 3~5 | 5 | 3 | 7(MPM)+2 (ctx) | 1(MPM)+7 (ctx) | 13 (MPM) + 6 (ctx) | 2(ctx) | NA |
| 3-3.4.1 | Mpm_flag: 1 (1) Angular flag 1 (10) Planar flag 1 (1) |  |  | 67 + 2*6 (MRL) + 2*7 (ISP) | 2~5 for regular incl. MRL | 4+3+3 | Same as VTM4 (ISP case) | 8(MPM) + 4 (ctx) | 1(MPM)+10 (ctx) | 14 (MPM)+6(ctx) | 5 (ctx) | Same as VTM4 (ISP case) |
| 3-3.4.2 |  |  |  | 67 + 2*6 (MRL) + 2*7 (ISP) | 2~7 for regular incl. MRL | 4+3+3 |  |  |  |  |  |  |
| 3-3.4.3 |  |  |  | 67 + 2*4 (MRL) + 2*5 (ISP) | 2~5 for regular incl. MRL | 4+3+3 |  |  |  | Fewer than VTM4 (ISP case) | Fewer than VTM4 (ISP case) |  |
| 3-3.5.1 | Non_Ang_flag: 1(5) Planar_flag: 1(1) |  |  | 6 | 3~5 | 6 | 5 | 7 (MPM)+ 4 (ctx) | 1(MPM)+10 (ctx) | 14+6(ctx) | 4 (ctx) | NA |
| 3-3.5.2 | Non_Ang_flag: 1(5) Planar_flag: 1(1) |  |  | 6 | 3~5 | 6 | 5 | 7(MPM)+4 (ctx) | 1(MPM)+10(ctx) | 14+6(ctx) | 4(ctx) | NA |

The table above summarizes complexity aspects of the CE3-3 test methods.

## CE3-3: Cross-check reports

| Test # | Cross-checker | Results match (y/n) | Cross checker comment |
| --- | --- | --- | --- |
| 3-3.1.1 | Zhang Zhi (Ericsson) | y | Software matches the contribution description. Both test A and test B results match the provided results. |
| 3-3.1.2 |  | y |  |
| 3-3.2.1 | L. Zhao (Tencent) | y | Software was reviewed; no significant inconsistencies between description and software; full results cross-checked |
| 3-3.2.2 |  | y |  |
| 3-3.2.3 |  | withdrawn |  |
| 3-3.3.1 | S. de Luxán Hernández (HHI) | y | No inconsistencies between the description and the software were found. All results match. However, there are a couple of differences in the 4th digit of various PSNR values in RA. This is probably due to the usage of a different rounding resolution when computing the PSNR values. The proponents fixed a bug and retested CE3-3.3.2 and CE3-3.3.3 (with and without ALL_MPM). These results were cross-checked as well. However, due to the lack of time, there are only partial results available for RA. |
| 3-3.3.2 |  | y |  |
| 3-3.3.3 |  | y |  |
| 3-3.4.1 | A. Filippov (Huawei) | y | Software has been studied with no significant inconsistencies between description and software; full results cross-checked. |
| 3-3.4.2 |  | y |  |
| 3-3.4.3 |  | y |  |
| 3-3.5.1 | S. Iwamura (NHK) | y | Software has been studied and no significant inconsistencies were found. All intra results for both A and B test were cross-checked. |
| 3-3.5.2 |  | y |  |

## CE3-3: Contributions related to harmonization of MPM lists

| Doc. # | Related test # | Title |
| --- | --- | --- |
| JVET-N0170 (Fujitsu) | 3-3.5.1 | CE3-related: Explicitly signal non angular modes with encoding changes |
| JVET-N0185 (Huawei) | 3-3 | CE3-related: A unified MPM list for intra mode coding |
| JVET-N0303 (BBC) | 3-3 | CE3-related: Simplified unified luma intra mode coding |
| JVET-N0394 (Tencent) | 3-3.3, 3-3.5.1 | CE3-related: Unified MPM list based on CE3-3.3 and CE3-3.5.1 |
| JVET-N0412 (KDDI) | 3-3.4, 3-3.5 | CE3-related: Modification on the intra luma mode coding process |
| JVET-N0433 (Qualcomm) | 3-3 | CE3-related: Unification of MPM derivation for luma intra modes |
| JVET-N0450 (Sharp) | 3-3.4 | CE3-3.4-related: unified MPM list construction |

# CE3-4 on ‘Affine linear weighted intra prediction’

## Defined tests [1]

| Test # | Short description | Doc. # |
| --- | --- | --- |
| 3-4.1 | Affine linear weighted intra prediction, i.e., to store all matrices and offsets needed, is restricted to be not more than 8 Kilobyte | JVET-N0217 (HHI) |
| 3-4.2 | Architectural variations of the affine linear weighted intra prediction will be studied which do not exceed the total memory requirement of 8 Kilobyte |  |

## CE3-4: Test results

|  | All Intra Main10 - Over VTM-4.0 |  |  |  |  | Random Access Main10 - Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 3-4.1 | -0.79% | -0.27% | -0.28% | 138% | 99% | -0.42% | -0.63% | -0.64% | 104% | 99% |
| 3-4.2 | -0.82% | -0.24% | -0.32% | 141% | 99% | -0.44% | -0.54% | -0.63% | 105% | 101% |

## CE3-4: Cross-check reports

| Test # | Cross-checker | Results match (y/n) | Cross checker comment |
| --- | --- | --- | --- |
| 3-4.1 | Vasily Rufitskiy (Huawei) | y | No inconsistencies found. Additional results for FastLwip=1 are tested. These are reported by the tester in JVET-N0217. |
| 3-4.2 |  | y |  |

# References

- G. Van der Auwera, L. Li, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” 13th JVET Meeting, Marrakech, MA, Jan. 2019, JVET-M1023-v3