| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0024-v1 |
| --- | --- |

| Title: | CE4: Summary report on inter prediction and motion vector coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Haitao Yang Guichun Li Kai Zhang | Email: | haitao.yang@huawei.com guichunli@tencent.com zhangkai.video@bytedance.com |
| Source: | CE4 coordinators |  |  |

_____________________________

- Abstract

This contribution provides a summary report of Core Experiment 4 on inter prediction and motion vector coding. CE4 comprises 4 categories,

- Merge mode signaling

- STMVP

- MMVD modification

- TPM merge list modification

All techniques are implemented on top of and test against VTM-4.0. Simulation results and crosschecking reports of each test specified in this document are provided.

CE4-1: Merge mode signaling

Brief description of tests

| Test# | Source | Description |
| --- | --- | --- |
| 4-1.1 | JVET-N0141 | Signaling MMVD syntax elements when merge_flag equal to 0. |
| 4-1.2.a | JVET-N0324 | Signal a regular merge flag right after the merge flag and skip flag to indicate whether the regular merge/skip mode is used by the current CU |
| 4-1.2.b | JVET-N0324 | Combination of test 4-1.2.a with 4-1.1 |
| 4-1.3 | JVET-N0237 | Change the MMVD flag position, and remove unnecessary syntax element |
| 4-1.4.a | JVET-N0252 | Signal a merge_subblock_flag and a merge_triangle_flag consecutively |
| 4-1.4.b | JVET-N0252 | Combination of test 4-1.4.a with 4-1.1 |
| 4-1.4.c | JVET-N0252 | Combination of test 4-1.4.a with 4-1.3 |
| 4-1.5.a | JVET-N0212 | Remove TMVP merge candidate for the block size with samples <= 32 |
| 4-1.5.b | JVET-N0212 | Remove TMVP merge candidate for the block size with samples <= 64 |
| 4-1.5.c | JVET-N0212 | Remove TMVP merge candidate for the block size of 4xN / Nx4 and 8x8 |

Test 4-1.1 moves MMVD syntax from merge_data() to coding_unit(), trying to improve coding efficiency in high QP condition.

Test 4-1.2 assigns shortest codeword to regular merge mode among all the 5 modes of merge candidates. The modified signaling order is, Regular merge, MMVD, subblock, CIIP, and TPM.

Test 4-1.3 proposes removing mmvd_merge_flag and its context model, and the parsing of MMVD syntax being conditioned on merge_idx being equal to 0 or 1.

There is an assumption in the current MMVD design that the merge list has more than two candidates, otherwise parsing mmvd_merge_flag is a waste. However, there is no checking to ensure this. The proposed method solves the problem in a way.

Test 4-1.4 divides all merge candidates into to three groups, CU merge candidates, subblock merge candidates, and triangle merge candidates, according to the type of block partitioning for motion compensation. It is proposed to put together the syntax of each group, and reorder the syntax of the three groups as subblock candidates, triangle candidates, and CU candidates.

Test 4-1.5 proposes not using TMVP in merge for small CUs. Three criteria for identifying small CUs are tested.

Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4-1.1 | -0.03% | -0.01% | 0.02% | 99% | 101% | -0.06% | 0.02% | -0.15% | 102% | 101% |
| 4-1.2.a | -0.01% | -0.07% | -0.06% | 102% | 102% | -0.07% | 0.30% | 0.15% | 101% | 99% |
| 4-1.2.b | 0.09% | 0.07% | 0.15% | 100% | 101% | 0.04% | 0.31% | 0.17% | 101% | 99% |
| 4-1.3 | -0.02% | -0.17% | -0.11% | 101% | 100% | -0.01% | 0.08% | -0.29% | 101% | 100% |
| 4-1.4.a | -0.01% | -0.09% | -0.03% | 102% | 102% | -0.03% | 0.02% | -0.10% | 95% | 89% |
| 4-1.4.b | -0.01% | 0.06% | 0.06% | 102% | 102% | -0.05% | 0.01% | -0.05% | 96% | 87% |
| 4-1.4.c | -0.02% | -0.10% | -0.06% | 103% | 103% | -0.02% | -0.07% | -0.39% | 92% | 82% |
| 4-1.5.a | -0.01% | -0.04% | -0.06% | 104% | 98% | -0.01% | 0.11% | 0.03% | 81% | 100% |
| 4-1.5.b | -0.03% | -0.07% | -0.06% | 100% | 100% | 0.01% | 0.10% | -0.08% | 100% | 66% |
| 4-1.5.c | -0.03% | -0.06% | -0.04% | 100% | 102% | 0.04% | 0.04% | -0.26% | 81% | 97% |

Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 4-1.1 | Y. Chen (Kwai) |  |  |  |  |
| 4-1.2.a | E. Sasaki (Sharp) | 2 | F | Y | M |
| 4-1.2.b | E. Sasaki (Sharp) | 2 | F | Y | M |
| 4-1.3 | E. Sasaki (Sharp) | 2 | F | Y | M |
| 4-1.4.a | J. Kim (SKT) | 2 | F | Y | G |
| 4-1.4.b | S.-C. Lim (ETRI) | 2 | F | Y | G |
| 4-1.4.c | S.-C. Lim (ETRI) | 2 | F | Y | G |
| 4-1.5.a | G. Li (Tencent) | 2 | F | Y | G |
| 4-1.5.b | G. Li (Tencent) | 2 | F | Y | M |
| 4-1.5.c | G. Li (Tencent) | 2 | F | Y | G |

Level: 1 simulation only, 2 code review + simulation, 3 independent implementation + simulation.

Results: P partial results by spot-checking, F full results

Match: Y / N

EncT/DecT: M match in general, G gap observed.

Detailed description of tests

CE4-1.1 in JVET-N0141 (Sharp)

The proposed syntax structure is shown in the following.

Table - Proposed coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( tile_group_type != I ¦ ¦ sps_ibc_enabled_flag ) { |  |
| if( treeType != DUAL_TREE_CHROMA ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 && tile_group_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( ( ( tile_group_type = = I && cu_skip_flag[ x0 ][ y0 ] = =0 ) ¦ ¦ ( tile_group_type != I && CuPredMode[ x0 ][ y0 ] != MODE_INTRA ) ) && sps_ibc_enabled_flag ) |  |
| pred_mode_ibc_flag | ae(v) |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| … |  |
| } else if( treeType != DUAL_TREE_CHROMA ) { /* MODE_INTER or MODE_IBC */ |  |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| merge_data( x0, y0, cbWidth, cbHeight ) |  |
| } else if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) { |  |
| … |  |
| } else { |  |
| mmvd_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( tile_group_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| … | ae(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

Table - Proposed merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| mmvd_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] ) { |  |
| if ( cbWidth <= 2 * cbHeight ¦ ¦ cbHeight <= 2 * cbWidth ) |  |
| ciip_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| ciip_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( sps_triangle_enabled_flag && tile_group_type = = B && ciip_flag[ x0 ][ y0 ] = = 0 && cbWidth * cbHeight >= 64 ) |  |
| merge_triangle_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_triangle_flag[ x0 ][ y0 ] ) { |  |
| merge_triangle_split_dir[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx0[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx1[ x0 ][ y0 ] | ae(v) |
| } else if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

CE4-1.2 in JVET-N0324 (Kwai)

It is proposed to signal a regular merge flag right after the merge flag and skip flag to indicate whether the regular merge/skip mode is used by the current CU.

Table - The flags signaling for the different types of merge modes in VTM-4.0

| Signaled Modes | MMVD Flag | Subblock Flag | CIIP Flag | Triangular Flag |
| --- | --- | --- | --- | --- |
| MMVD | 1 | - | - | - |
| Subblock | 0 | 1 | - | - |
| CIIP | 0 | 0 | 1 | - |
| Triangle | 0 | 0 | 0 | 1 |
| Regular | 0 | 0 | 0 | 0 |

Table - The flag signaling for the different types of merge modes in the proposed scheme

| Signaled Modes | Regular Flag | MMVD Flag | Subblock Flag | CIIP Flag |
| --- | --- | --- | --- | --- |
| Regular | 1 | - | - | - |
| MMVD | 0 | 1 | - | - |
| Subblock | 0 | 0 | 1 | - |
| CIIP | 0 | 0 | 0 | 1 |
| Triangle | 0 | 0 | 0 | 0 |

CE4-1.3 in JVET-N0237 (WILUS)

Table 1. The modified merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| mmvd_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] ) { |  |
| if ( cbWidth <= 2 * cbHeight ¦ ¦ cbHeight <= 2 * cbWidth ) |  |
| ciip_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| ciip_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( sps_triangle_enabled_flag && tile_group_type = = B && ciip_flag[ x0 ][ y0 ] = = 0 && cbWidth * cbHeight >= 64 ) |  |
| merge_triangle_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_triangle_flag[ x0 ][ y0 ] ) { |  |
| merge_triangle_split_dir[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx0[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx1[ x0 ][ y0 ] | ae(v) |
| } else if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| if( merge_idx[ x0 ][ y0 ] < 2 && !ciip_flag[ x0 ][ y0 ] && !merge_triangle_flag[ x0 ][ y0 ] ) { |  |
| mmvd_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

CE4-1.4 in JVET-N0252 (Digital Insights)

Proposed changes in Merge data syntax.

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_triangle_enabled_flag && tile_group_type = = B && cbWidth * cbHeight >= 64 ) |  |
| merge_triangle_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_triangle_flag[ x0 ][ y0 ] ) { |  |
| merge_triangle_split_dir[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx0[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx1[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| mmvd_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] ) { |  |
| if ( cbWidth <= 2 * cbHeight ¦ ¦ cbHeight <= 2 * cbWidth ) |  |
| ciip_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| ciip_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

CE4-1.5 in JVET-N0212 (DJI & PKU)

The following three tests are designed,

- Remove TMVP merge candidate for the blocksize with samples <= 32, which means 4x4/4x8/8x4 TMVP is removed from merge list;

- Remove TMVP merge candidate for the blocksize with samples <= 64, which means 4x4/4x8/8x4/4x16/16x4/8x8 TMVP is removed from merge list;

- Remove TMVP merge candidate for the blocksize with samples of 4xN / Nx4 and 8x8, which means 4x4/4x8/8x4/4x16/16x4/32x4/4x32/8x8 TMVP is removed from merge list.

CE4-2: STMVP

Brief description of tests

| Test# | Source | Description |
| --- | --- | --- |
| 4-2.2.a | JVET-N0285 | STMVP merge candidate, consisting in an averaged motion vector between the TMVP merge candidate and some other motion vectors predictors. |

A single STMVP candidate, if available, is put into regular merge list before the top-left spatial motion vector candidate. The derivation of the STMVP candidate uses one temporal candidate and up to two spatial candidates in the regular merge list construction. No candidate pruning is introduced.

Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4-2.2.a | -0.13% | -0.10% | -0.10% | 101% | 101% | 0.00% | 0.23% | 0.07% | 102% | 103% |

Complexity analysis of list construction

| Test# | Merge list size | Max number of potential candidates | Max number of candidate comparison | Max number of MV scaling | Max number of temporal candidates | Max number of MV average | Others |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Regular merge | 6 | 13(5+2+5+1) | 9(5+0+4+0) | 2 | 2 | 2 |  |
| 4-2.2.a | 6 (+0) | 14 (+1) | 9 (+0) | 2 (+0) | 2 (+0) | 4 (+1 per list) | Availability test changed from (refIdx != NOT_VALID) to (refIdx==0) compared to pairwise |

Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 4-2.2.a | T. Zhou (Sharp) | 2 | F | Y | M |

Detailed description of tests

CE4-2.1 in JVET-N0285 (Technicolor)

The STMVP motion vector predictor is compute as follows:

- The use of STMVP is signaled by a slice header flag, similarly as TMVP.

- The temporal MV predictor used to compute the STMVP motion vector is the classical TMVP merge candidate

- The spatial candidates used are the first and second candidates contained in the merge list at the time the STMVP candidate is being computed.

- A spatial candidate is considered as available if its reference picture index is 0. Thus, compared to pairwise, the availability test is changed from (refIdx != NOT_VALID) to (refIdx == 0).

- For each reference picture list L0 and L1, the following applies:

  - If 3 candidates are available, then the STMVP candidate is available and is computed as:

    - MVSTMVP=3×MVS1+3×MVS2+2×TMVP≫3

  - If one spatial candidate MVS and the TMVP candidate are available, then:

    - MVSTMVP=MVS+TMVP≫1

The STMVP candidate, if available, is placed before the B2 candidate in the merge candidate list. The list of potential merge candidates is thus the following one:

- A1, B1, B0, A0, STMVP, B2, TMVP, HMVP, Pairwise, Zero

CE4-3: MMVD modification

Brief description of tests

| Test# | Source | Description |
| --- | --- | --- |
| 4-3.1.b | JVET-N0125 | Binarization method change |
| 4-3.1.c | JVET-N0125 | Multiple distance list concept |
| 4-3.1.d | JVET-N0125 | Multiple distance list concept + binarization method change |
| 4-3.3.a | JVET-N0441 | MMVD distance table is reduced 4 and is refined based on the resolution of base motion vectors |
| 4-3.3.b | JVET-N0441 | 4-3.3.a + 4-3.4.a |
| 4-3.3.c | JVET-N0441 | 4-3.3.a + 4-1.1 |
| 4-3.3.d | JVET-N0441 | 4-3.3.b + 4-1.1 |
| 4-3.4.a | JVET-N0142 | Diagonal direction candidate for MMVD |
| 4-3.4.b | JVET-N0142 | 4-3.4.a + 4-1.1 |

Test 4-3.1.b changes the binarization of mmvd_distance_idx, making the codeword for larger MVDs shorter.

Test 4-3.1.c replaces the tile group header flag tile_group_fpel_mmvd_enabled_flag by a syntax symbol that could represent 4 alternative resolutions of MMVD distance instead of 2 in the current VTM design.

Test 4-3.3.a proposes reducing the number of candidates in distance list from 8 to 4, keeping the first four smaller MVD offsets. In addition, the decoded distance is further modified according to the resolution of the base motion vector. If base mv falls on the grid of 2-pel resolution, the distance is enlarged 4x. If base mv falls on the grid of integer-pel resolution, the distance is enlarged 2x.

Test 4-3.4.a proposes diagonal direction MVD offsets. With this modification, four points with same MMVD distance index forms a diamond shape. The proposed method doubles the number of MVD offsets.

Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4-3.1.b | -0.02% | -0.12% | -0.12% | 100% | 98% |  |  |  |  |  |
| 4-3.1.c | -0.01% | -0.02% | -0.02% | 98% | 95% |  |  |  |  |  |
| 4-3.1.d | -0.02% | -0.13% | -0.12% | 100% | 98% |  |  |  |  |  |
| 4-3.3.a | 0.01% | 0.07% | 0.05% | 96% | 101% | 0.01% | 0.19% | 0.00% | 96% | 101% |
| 4-3.3.b | -0.08% | 0.09% | 0.10% | 97% | 101% | -0.05% | 0.25% | 0.21% | 96% | 100% |
| 4-3.3.c | -0.02% | 0.13% | 0.13% | 96% | 101% | -0.03% | 0.25% | 0.02% | 97% | 101% |
| 4-3.3.d | -0.11% | 0.11% | 0.17% | 97% | 101% | -0.12% | 0.20% | 0.05% | 98% | 101% |
| 4-3.4.a | -0.07% | 0.00% | 0.03% | 100% | 100% | -0.03% | 0.31% | 0.25% | 100% | 100% |
| 4-3.4.b | -0.11% | 0.02% | 0.12% | 100% | 101% | -0.08% | 0.13% | -0.14% | 101% | 100% |

Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 4-3.1.b | T. Hashimoto (Sharp) | 1 | F | Y | M |
| 4-3.1.c | T. Hashimoto (Sharp) | 1 | F | Y | G |
| 4-3.1.d | S. Yoo (LGE) |  |  |  |  |
| 4-3.3.a | E. Sasaki (Sharp) | 2 | F | Y | M |
| 4-3.3.b | E. Sasaki (Sharp) | 2 | F | Y | M |
| 4-3.3.c | E. Sasaki (Sharp) | 2 | F | Y | M |
| 4-3.3.d | E. Sasaki (Sharp) | 2 | F | Y | M |
| 4-3.4.a | N. Park (LGE) | 2 | F | Y | M |
| 4-3.4.b | N. Park (LGE) | 2 | F | Y | M |

Detailed description of tests

CE4-3.1 in JVET-N0125 (Samsung)

In the process of distance index signalling, distance index is signalled with truncated unary coding method.

Table - Distance list index binarization

| Distance list index | binarization |
| --- | --- |
| 0 | 0 |
| 1 | 10 |
| 2 | 110 |
| 3 | 1110 |
| 4 | 11110 |
| 5 | 111110 |
| 6 | 1111110 |
| 7 | 1111111 |

Table - Proposed binarization method

| Distance list index | binarization |
| --- | --- |
| 0 | 00 |
| 1 | 01 |
| 2 | 10 |
| 3 | 110 |
| 4 | 1110 |
| 5 | 11110 |
| 6 | 111110 |
| 7 | 111111 |

CE4-3.3 in JVET-N0441 (LGE)

This contribution proposed to reduce the number of distance candidates from 8 to 4 as follow:

Conventional distance candidates (8): {1, 2, 4, 8, 16, 32, 64, 128}

Proposed distance candidates (4): {1, 2, 4, 8}

Actual motion distance is further derived by considering both the index and the motion vector resolution. More details on the derivation process for actual motion distance are described below,

distance = MmvdDistance[ x0 ][ y0 ] ; if (mvLX[0] != 0 ¦¦ mvLX[1] != 0) { if( mvLX[0] %32 == 0 && mvLX[1] % 32 == 0) distance = MmvdDistance[ x0 ][ y0 ] << 2; else if ( mvLX[0] %16 == 0 && mvLX[1] % 16 == 0) distance = MmvdDistance[ x0 ][ y0 ] << 1; }

CE4-3.4 in JVET-N0142 (Sharp)

It is proposed that directional candidates are used in addition to horizontal/vertical candidates for MMVD.The directional candidates are shown as below.

| Direction IDX | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x-axis | +1 | –1 | 0 | 0 | +1/2 | -1/2 | -1/2 | +1/2 |
| y-axis | 0 | 0 | +1 | –1 | +1/2 | -1/2 | +1/2 | -1/2 |

At the encoder side, to reduce the fixed number of the search candidates without loss, a fast algorithm like “Hexagon Search” is applied. In this algorithm, the searching for directions is nested in the searching for distances. At first, all directions (8 directions) are searched with the smallest distance. After this first searching loop, only 3 directions, that are the best direction in previous distance and 2 neighbors, are searched. By using this algorithm, the number of search candidates in each base MV is equal to 8 + 3 * 7 = 29.

CE4-4: TPM merge list modification

Brief description of tests

| Test# | Source | Description |
| --- | --- | --- |
| 4-4.1.a | JVET-N0083 | Use regular merge mode candidate list and two signaled indices. Candidate 1 select L0 MV if exist; otherwise select L1 MV. Candidate 2 select L1 MV if exist; otherwise select L0 MV. |
| 4-4.1.c | JVET-N0083 | 4-4.1.a + MV pruning and adding offset MV if necessary |
| 4-4.2.a | JVET-N0128 | Use first 3 candidates of regular merge candidate list for triangle list construction |
| 4-4.2.b | JVET-N0128 | Use first 4 candidates of regular merge candidate list for triangle list construction |
| 4-4.2.c | JVET-N0128 | 4-4.2.a + spatial temporal candidate only restriction |
| 4-4.2.d | JVET-N0128 | 4-4.2.c + simplified pruning |
| 4-4.2.e | JVET-N0128 | Use regular merge list with spatial temporal restriction |
| 4-4.2.f | JVET-N0128 | 4-4.2.e + simplified pruning |
| 4-4.3.a | JVET-N0322 | Share regular merge candidate list for triangle partition merge prediction. For RA, L0 and L1 motion vectors from the regular candidate list are selected in an interleaving manner. For LDB, L0 motion vectors are selected, followed by uni-prediction zero vectors. |
| 4-4.3.b | JVET-N0322 | Share regular merge candidate list for triangle partition merge prediction. For RA, L0 and L1 motion vectors from the regular candidate list are selected in an interleaving manner. For LDB, L0 motion vectors are selected, followed by L1 motion vectors. |
| 4-4.4.a | JVET-N0269 | Partial of the regular merge list construction process is reused. |
| 4-4.5.a | JVET-N0226 | Reduce the pruning operations in TPM merge list construction |
| 4-4.5.b | JVET-N0226 | Remove the pruning operations in TPM merge list construction |
| 4-4.6.a | JVET-N0454 | Derive triangular merge list based on the candidates of regular merge list, using same scan order. If candidate is bi-predictive, L0 part is used. |
| 4-4.6.b | JVET-N0454 | Derive triangular merge list based on the candidates of regular merge list, using same scan order. If candidate is bi-predictive, pruning is applied. |
| 4-4.7.c | JVET-N0197 | The reference index of L1 is set equal to be the same as that of L0, mirroring process is applied after offset addition |
| 4-4.7.d | JVET-N0197 | The reference index of L1 is set equal to be the same as that of L0, mirroring process is applied before offset addition |
| 4-4.8.a | JVET-N0211 | Reuse regular merge list derivation for triangle merging candidate list construction |
| 4-4.8.b | JVET-N0211 | Use candidates before HMVP in the regular merge list as the candidates for triangle merging candidate list construction |
| 4-4.9.a | JVET-N0418 | Derive merge list based on the regular list, bi-MV is split into uni-MVs and pruning is applied. |
| 4-4.9.b | JVET-N0418 | 4-4.9.a with no pruning in the final TPM merge list. |
| 4-4.9.c | JVET-N0418 | Merge list is derived based on the spatial and temporal MV candidates from the regular merge list, bi-MV is split into uni-MVs and pruning is applied |
| 4-4.9.d | JVET-N0418 | 4-4.9.c with no pruning in the final TPM merge list. |

Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4-4.1.a | 0.06% | 0.00% | 0.06% | 101% | 100% | 0.15% | 0.19% | -0.01% | 100% | 100% |
| 4-4.1.c | 0.05% | 0.04% | 0.07% | 101% | 99% | 0.11% | 0.08% | -0.08% | 100% | 100% |
| 4-4.2.a | -0.03% | -0.07% | -0.11% | 100% | 100% | 0.08% | 0.12% | -0.28% | 100% | 100% |
| 4-4.2.b | -0.01% | -0.10% | -0.09% | 100% | 100% | -0.01% | -0.10% | -0.19% | 100% | 100% |
| 4-4.2.c | -0.03% | -0.04% | -0.06% | 100% | 100% | 0.04% | 0.01% | -0.09% | 100% | 100% |
| 4-4.2.d | -0.01% | -0.03% | 0.00% | 100% | 100% | 0.16% | 0.28% | -0.05% | 100% | 100% |
| 4-4.2.e | -0.03% | -0.04% | -0.09% | 100% | 100% | 0.05% | 0.16% | -0.08% | 100% | 101% |
| 4-4.2.f | 0.00% | -0.01% | 0.02% | 100% | 100% | 0.13% | 0.11% | 0.25% | 100% | 99% |
| 4-4.3.a | -0.02% | -0.09% | -0.06% | 101% | 99% | 0.16% | 0.33% | 0.03% | 100% | 100% |
| 4-4.3.b | -0.02% | -0.09% | -0.07% | 101% | 101% | 0.17% | 0.29% | 0.20% | 99% | 99% |
| 4-4.4.a | -0.02% | -0.09% | -0.11% | 100% | 100% | 0.09% | 0.20% | -0.19% | 100% | 100% |
| 4-4.5.a | 0.01% | -0.01% | 0.01% | 100% | 100% | 0.02% | 0.19% | -0.15% | 101% | 100% |
| 4-4.5.b | 0.03% | 0.01% | 0.02% | 100% | 100% | 0.14% | 0.37% | -0.04% | 101% | 101% |
| 4-4.6.a | 0.12% | 0.10% | 0.16% | 101% | 102% | 0.16% | 0.19% | 0.18% | 101% | 103% |
| 4-4.6.b | 0.00% | -0.07% | -0.09% | 101% | 102% | 0.11% | 0.12% | -0.01% | 101% | 103% |
| 4-4.7.c | 0.07% | 0.03% | 0.06% | 102% | 100% | 0.05% | -0.11% | -0.22% | 103% | 101% |
| 4-4.7.d | 0.06% | 0.03% | 0.08% | 103% | 100% | 0.05% | -0.11% | -0.22% | 104% | 100% |
| 4-4.8.a | 0.07% | 0.03% | 0.06% | 96% | 109% | 0.04% | 0.27% | 0.00% | 101% | 111% |
| 4-4.8.b | -0.03% | -0.04% | -0.09% | 96% | 110% | 0.05% | 0.16% | -0.08% | 101% | 111% |
| 4-4.9.a | -0.04% | -0.14% | -0.14% | 100% | 100% | -0.03% | 0.10% | -0.01% | 100% | 101% |
| 4-4.9.b | -0.01% | -0.08% | -0.06% | 100% | 100% | 0.31% | 0.48% | 0.06% | 100% | 102% |
| 4-4.9.c | -0.02% | -0.04% | -0.04% | 100% | 100% | 0.13% | 0.19% | 0.08% | 100% | 101% |
| 4-4.9.d | 0.00% | -0.05% | -0.07% | 101% | 101% | 0.34% | 0.45% | 0.30% | 100% | 101% |

Complexity analysis of list construction

| Test# | Merge list size | Max number of potential candidates (stage1+stage2) | Max number of candidate comparison (stage1+stage2) | Max number of MV scaling (stage1+stage2) | Max number of temporal candidates (stage1+stage2) | Max number of MV average (stage1+stage2) | Max number of encoder MCs (stage1+stage2) | Others |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Regular merge | 6 | 13 | 9 | 2 | 2 | 2 | 12 |  |
| TPM | 5 | 28(7+21) | 84(10+74) | 11(4+7) | 2(2+0) | 7(0+7) | 5 |  |
| 4-4.1.a | 5 | 13 | 9 | 2 | 2 | 2 | 10 |  |
| 4-4.1.c | 5 | 13 | 10 (9+1) | 2 | 2 | 2 | 15 | One additional MV addition (adding (1/4, 0) on second candidate) |
| 4-4.2.a | 5 | 22 (13+9) | 35 | 5 | 2 | 5 | 5 |  |
| 4-4.2.b | 5 | 25 (13+12) | 47 | 6 | 2 | 6 | 5 |  |
| 4-4.2.c | 5 | 19 (7+12) | 43 | 6 | 2 | 4 | 5 |  |
| 4-4.2.d | 5 | 19 (7+12) | 16 | 6 | 2 | 4 | 5 |  |
| 4-4.2.e | 5 | 25 (7+18) | 67 | 8 | 2 | 6 | 5 |  |
| 4-4.2.f | 5 | 25 (7+18) | 22 | 8 | 2 | 8 | 5 |  |
| 4-4.3.a | 5 | 13 | 9 | 2 | 2 | 2 | 5 |  |
| 4-4.3.b | 5 | 13 | 9 | 2 | 2 | 2 | 5 |  |
| 4-4.4.a | 5 | 15 (5+10) | 12 (9+3) | 1 | 1 | 0 | 5 | No pairwise candidate |
| 4-4.5.a | 5 | 28 (+0) | 6+46 (-4-28) | 4+7 (+0+0) | 2 (+0) | 7 (+0) | 5 (+0) | / |
| 4-4.5.b | 5 | 28 (+0) | 6+0 (-4-74) | 4+7 (+0+0) | 2 (+0) | 7 (+0) | 5 (+0) | / |
| 4-4.6.a | 5 | 13(13+0) | 9(9+0) | 2 | 2 | 2 | 5 |  |
| 4-4.6.b | 5 | 23(13+10) | 13(9+4) | 2 | 2 | 2 | 5 |  |
| 4-4.7.c | 4 | 20(13+(4+3+0)) | 9 | 2 | 2 | 2 | 8 | No mapping logic |
| 4-4.7.d | 4 | 20(13+(4+3+0)) | 9 | 2 | 2 | 2 | 8 | No mapping logic |
| 4-4.8.a |  |  |  |  |  |  |  |  |
| 4-4.8.b |  |  |  |  |  |  |  |  |
| 4-4.9.a | 5 | 25(13+12) | 47(9+38) | 2(2+0) | 2(2+0) | 2(2+0) | 5 |  |
| 4-4.9.b | 5 | 25(13+12) | 9(9+0) | 2(2+0) | 2(2+0) | 2(2+0) | 5 |  |
| 4-4.9.c | 5 | 17(7+10) | 35(5+30) | 2(2+0) | 2(2+0) | 2(2+0) | 5 |  |
| 4-4.9.d | 5 | 17(7+10) | 5(5+0) | 2(2+0) | 2(2+0) | 2(2+0) | 5 |  |

Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 4-4.1.a | T. Solovyev (Huawei) |  |  |  |  |
| 4-4.1.c | T. Solovyev (Huawei) |  |  |  |  |
| 4-4.2.a | H. Wang (Qualcomm) | 2 | P | Y | M |
| 4-4.2.b | H. Wang (Qualcomm) | 2 | P | Y | M |
| 4-4.2.c | H. Wang (Qualcomm) | 2 | P | Y | M |
| 4-4.2.d | H. Wang (Qualcomm) | 2 | P | Y | M |
| 4-4.2.e | H. Wang (Qualcomm) | 2 | P | Y | M |
| 4-4.2.f | H. Wang (Qualcomm) | 2 | P | Y | M |
| 4-4.3.a | H. Wang (Qualcomm) | 2 | P | Y | M |
| 4-4.3.b | H. Wang (Qualcomm) | 2 | P | Y | M |
| 4-4.4.a | G. Li (Tencent) | 2 | F | Y | M |
| 4-4.5.a | G. Li (Tencent) | 2 | F | Y | M |
| 4-4.5.b | G. Li (Tencent) | 2 | F | Y | M |
| 4-4.6.a | T.-D. Chuang (MediaTek) | 2 | F | Y | M |
| 4-4.6.b | T.-D. Chuang (MediaTek) | 2 | F | Y | M |
| 4-4.7.c | R.-L. Liao (Alibaba) | 2 | F | Y | M |
| 4-4.7.d | R.-L. Liao (Alibaba) | 2 | F | Y | M |
| 4-4.8.a | H. Liu (Bytedance) | 1 | F | Y | M |
| 4-4.8.b | J. Li (Panasonic) |  |  |  |  |
| 4-4.9.a | X. Li (Tencent) | 2 | F | Y | M |
| 4-4.9.b | X. Wang (Kwai) | 2 | F | Y | M |
| 4-4.9.c | X. Wang (Kwai) | 2 | F | Y | M |
| 4-4.9.d | X. Wang (Kwai) | 2 | F | Y | M |

Detailed analysis of tests

Comparative study

In VTM4.0 TPM, the MV candidate list construction comprises two steps.

Step 1. Construct the source candidate list

- Obtain regular motion candidates from 5 spatial neighboring blocks with full pruning.

- Obtain regular merge candidates from two temporal blocks.

Step 2. Construct the TPM candidate list with uni-directional MVs, with full pruning

- Loop #1: For each regular motion candidates, if it is uni-prediction (either from List 0 or List 1), it is directly added to the merge list as a TPM candidate.

- Loop #2: For each motion candidates, if it is bi-prediction, the motion information from List 0 is added as a new TPM candidate.

- Loop #3: For each motion candidates, if it is bi-prediction, the motion information from List 1 is added as a new TPM candidate.

- Loop #4: For each motion candidates, if it is bi-prediction, the motion information of List 1 is firstly scaled to List 0 reference picture, and the average of the two MVs is added as a new TPM candidate.

- Add default motion vector candidates.

The complexity is higher than regular merge candidate list construction. All tests target at reducing complexity of TPM list construction, as summarized below.

| Test # | Stage 1 of TPM list construction |  |  | Stage 2 of TPM list construction |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Candidate derivation | Number of candidates | Candidate pruning | Source list checking iterations | Conversion of Bi-pred MV | Candidate pruning | Others |
| 4-4.1.a | Regular merge | 5 | Regular merge | - | Conversion of Bi-pred MV | - | - |
| 4-4.1.c | Regular merge | 5 | Regular merge | - | - | 1 | Add MV offsets conditionally |
| 4-4.2.a | Regular merge | 3 | Regular merge | 4 | - | Full | - |
| 4-4.2.b | Regular merge | 4 | Regular merge | 4 | Same as VTM4 | Full | - |
| 4-4.2.c | Regular merge (spatial, temporal candidates) | 4 (max) | Regular merge | 4 | Same as VTM4 | Full | - |
| 4-4.2.d | Regular merge (spatial, temporal candidates) | 4 (max) | Regular merge | 4 | Same as VTM4 | Simplified pruning to only the last one | - |
| 4-4.2.e | Regular merge (spatial, temporal candidates) | 5 (max) | Regular merge | 4 | Same as VTM4 | Full | - |
| 4-4.2.f | Regular merge (spatial, temporal candidates) | 5 (max) | Regular merge | 4 | Same as VTM4 | Simplified pruning to only the last one | - |
| 4-4.3.a | Regular merge | 6 | Regular merge | 1 | Same as VTM4 | No | Different handling for RA and LDB |
| 4-4.3.b | Regular merge | 6 | Regular merge | 1 (RA) 2 (LDB) | Split to L0 and L1 MV | No | - |
| 4-4.4.a | Regular merge (spatial, temporal, HMVP candidates) | 5 (max) | Regular merge | 1 | Split to L0 and L1 MV | Simplified pruning, controlled by a threshold | - |
| 4-4.5.a | TPM source list | 7 (max) | Simplified pruning | 4 | Split to L0 and L1 MV | Simplified | - |
| 4-4.5.b | TPM source list | 7 (max) | Simplified pruning | 4 | Same as VTM4 | No | - |
| 4-4.6.a | Regular merge | 5 | Regular merge | 1 | Same as VTM4 | No | - |
| 4-4.6.b | Regular merge | 5 | Regular merge | 1 | Use L0 MV only | L0 vs. L1 pruning | - |
| 4-4.7.c | Regular merge | 4 | Regular merge | 1 | Split to L0 and L1 MV | No | - |
| 4-4.7.d | Regular merge | 4 | Regular merge | 1 | - | No | - |
| 4-4.8.a | Regular merge | 6 | Regular merge | 4 | - | Full | - |
| 4-4.8.b | Regular merge (spatial, temporal candidates) | 5 (max) | Regular merge | 4 | Same as VTM4 | Full | - |
| 4-4.9.a | Regular merge | 6 | Regular merge | 1 | Same as VTM4 | Full | - |
| 4-4.9.b | Regular merge | 6 | Regular merge | 1 | Split to L0 and L1 MV | No | - |
| 4-4.9.c | Regular merge (spatial, temporal candidates) | 5 (max) | Regular merge | 1 | Split to L0 and L1 MV | Full | - |
| 4-4.9.d | Regular merge (spatial, temporal candidates) | 5 (max) | Regular merge | 1 | Split to L0 and L1 MV | No | - |

Most of the tests construct the regular merge list, full or partial, and use it as the source list. This is referred to as stage 1 of TPM list construction. Tests differs in stage 2 where a uni-prediction candidate list is constructed, trying to reducing the complexity in three ways, 1) reduce the number of iterations when checking the source candidate list, 2) reduce the number potential candidates, and 3) reduce the number of prunings when adding a new candidate to the list.

Test 4-4.7 is different from others in that it firstly construct a TPM list with bi-pred MVs, by converting a uni-pred MV to a bi-pred one. And then derive a pair of uni-pred MVs from two signaled TPM candidates. Additional syntax indicating prediction direction is signaled. It removes operations in motion data storage stage by deriving bi-pred MV in the stage of TPM list construction.

CE4-4.1 in JVET-N0083 (MediaTek) and JVET-M0317 (Panasonic)

In CE4-4.1a, it is proposed to reuse the regular merging candidate list for generating the two uni-prediction MVs for the two triangular partitions. When the triangle prediction is applied, two merge indices are signalled. The original triangle merging candidate list replaced with the regular merging candidate list. After the two initial candidates from the regular merging candidate list are selected, for the first triangle merging candidate, if the L0 MV of the first initial merging candidate exists, the first triangle merging candidate is set equal to the L0 MV of the first initial merging candidate; otherwise, the first triangle merging candidate is set equal to the L1 MV of the first initial merging candidate. Likewise, if the L1 MV of the second initial merging candidate exists, the second triangle merging candidate is set equal to the L1 MV of the second initial merging candidate; otherwise, the second triangle merging candidate is set equal to the L0 MV of the second initial merging candidate.

In this way, the complexity of the proposed triangle merging candidate list derivation is the same as that of the regular merging candidate list derivation plus only two inter prediction indicator (inter_pred_idc, to indicate L0, L1, or bi-prediction) comparisons.

The candidate generation flow can be described as follows.

1. Select two initial candidates from the regular merging candidate list.

Cand0: {Cand0_L0, Cand0_L1}

Cand1: {Cand1_L0, Cand1_L1}

2. Generating two triangle merging candidates

TriCand0 = (Cand0_L0 exists) ? Cand0_L0 : Cand0_L1;

TriCand1 = (Cand1_L1 exists) ? Cand1_L1 : Cand1_L0;

In addition to the candidate generation process in CE4-4.1a, if the first triangular partition and the second triangular partition have exactly the same merging candidate, an offset MV of (1/4 pel, 0) is added to the merging candidate of the second triangular partition. The complexity of the proposed triangle merging candidate list derivation is the same as that of the regular merging candidate list derivation plus only two inter prediction indicator comparisons, one MV comparison, and one MV addition.

The candidate generation flow can be described as follows.

1. Select two initial candidates from the regular merging candidate list.

Cand0: {Cand0_L0, Cand0_L1}

Cand1: {Cand1_L0, Cand1_L1}

2. Generating two triangle merging candidates

TriCand0 = (Cand0_L0 exists) ? Cand0_L0 : Cand0_L1;

TriCand1 = (Cand1_L1 exists) ? Cand1_L1 : Cand1_L0;

3. Perform MV pruning

isSameCand = (TriCand0 = = TriCand1);

4. Modify one of the triangle merging candidates if they are the same

if (isSameCand)

TriCand1 = TriCand1 + (1/4 pel, 0);

CE4-4.2 in JVET-N0128 (Samsung)

Unification of candidate list derivation

To unify the step 1 of the triangle mode candidate derivation process with the regular merge list candidate derivation, step 1 is simply replaced with the regular merge list derivation process.

Furthermore since the step 2 requires only a few candidates from step 1 in order to completely fill up the uni-directional MV list of five candidates, only the first N of candidates from the regular merge candidate list are used. This restriction further simplifies the list derivation process

Restriction to only spatial and temporal candidates

When using the regular merge list candidate construction process, HMVP buffer is accessed for adding HMVP candidates and averaging operations are performed for adding pairwise average candidates. These operations although rarely performed (since the method requires only the first N candidates, N=3,4), add new computations in the worst case compared to the original step 1 of the triangle merge list construction process. To address this, the regular merge candidate list construction process only till temporal candidates is followed, i.e. only spatial and temporal candidates are added. HMVP, average and zero candidates are not added. The list construction process stops after the

Pruning simplification

Step 2 of the triangle mode candidate list constructs a list of five uni-directional MVs. When performing the pruning operation, each newly candidate added is added after comparing it with each of the existing MVs in the list. This results in many comparisons especially for the candidates added near the end of the list, which needs to be repeated if the candidate fails the pruning check. A simpler pruning method is proposed to reduce the total number of comparisons. Each candidate is only compared against the previously added candidate, and not all the previously added candidates. Hence each candidate undergoes only one MV comparison (except for the first candidate, which doesn’t require any comparison).

Combining the above described methods, the following options are proposed to unify the list construction process and simplify the pruning operations.

- Method 1) Use first 3 candidates of regular merge candidate list for triangle list construction

- Method 2) Use first 4 candidates of regular merge candidate list for triangle list construction

- Method 3) Method 2 + spatial temporal candidate only restriction

- Method 4) Method 3 + simplified pruning

- Method 5) Use spatial temporal candidates of regular merge candidate list for triangle list construction

- Method 6) Method 5 + simplified pruning

CE4-4.3 in JVET-N0322 (Kwai)

According to this proposal, to maximize the common operations shared by different merge modes, the regular merge list is constructed and its motion vectors are used for triangle prediction.

Once the regular merge list is constructed, for a given picture where backward prediction is used, L0 and L1 motion vectors of each merge candidate in the regular candidate list are selected in a L0-L1 interleaving manner for triangle merge prediction. If backward prediction is not used, L0 motion vectors from the regular candidate list are first selected for triangle merge prediction, followed by zero motion vectors (which is corresponding the test “a” in this CE test) or L1 motion vectors from the regular merge list (which is corresponding to test “b” in this CE test).

CE4-4.4 in JVET-N0269 (Bytedance)

The regular merge list construction process (before invoking pairwise bi-predication) is fully reused. That is, a regular motion candidate list is constructed and each regular motion candidate, after pruning if needed, is used to construct the TPM candidate list. More details of the TPM candidate list construction process is defined as follows:

- Reuse the regular merge list construction process to get valid regular merge candidates after pruning among regular merge candidates, denoted by RC0, RC1….

- For each regular merge candidate RCi, the following apply:

  - if it is uni-prediction, add it if i is no smaller than Th or not identical with the previous one TPM candidate

  - Otherwise (if bi-prediction),

    - Add one TPM candidates with list 0 motion information.

    - Add one TPM candidates with list 1 motion information if i is no smaller than Th or not identical with the previous one TPM candidate.

- If TPM list is not full, zero motion candidates are added.

In above description, the variable Th controls the number of pruning operations among TPM candidates. In this contribution, Th is set to 4.

CE4-4.5 in JVET-N0226 (Technicolor)

Test 4-4.5.a proposes to:

- Harmonize first spatial candidates pruning as in Merge mode since these candidates are the same as in Merge,

- Perform a simple pruning of the temporal candidates together,

- Remove the pruning of uni-directional candidates since the pre-list is already (partially) pruned,

- Remove the pruning of averaged candidates.

In test 4-4.5.a, more drastic, version where all pruning of the motion predictor list construction are removed, it is, in addition, proposed to:

- Remove the pruning of L0 parts of bi-directional candidates,

- Remove the pruning of L1 parts of bi-directional candidates.

CE4-4.6 in JVET-N0454 (Huawei)

Method 1 (Test 4-4.6.A)

The uni-prediction candidate list is constructed according to the steps of:

- Construct the regular merge candidate list,

- Starting from the beginning of the regular merge candidate list the following is performed:

  - If a candidate in the regular merge list is uni-predictive, insert it into uni-prediction candidate list.

  - If the candidate is bi-predictive, the candidate is converted to uni-predictive by keeping the L0 motion information (motion information here means motion vector and reference index).

Method 2 (Test 4-4.6.B)

The uni-prediction candidate list is constructed according to the steps of:

- Construct the regular merge candidate list,

- Starting from the beginning of the regular merge candidate list, the following is performed:

  - If a candidate in the regular merge list is uni-predictive, insert it into uni-prediction candidate list.

  - If a candidate in the regular merge candidate list is bi-prediction candidate:

    - insert L0 motion information into uni-prediction candidate list,

    - insert L1 motion information if it is not identical to the L0 motion information as additional candidate.

The number of candidates in the uni-prediction candidate list in both methods is set as 5.

CE4-4.7 in JVET-N0197 (Panasonic)

Firstly, the first four candidates in regular merge candidate list are directly used to construct triangle merge candidate list. The construction process is as following:

For each of the four candidates, if it is bi-prediction, directly copy it to the triangular candidate list. Otherwise if it is uni-prediction, a bi-prediction motion vector is generated and added to triangle merge list by following process:

Let motion vector of the uni-prediction is MVX of LX direction, X = 0, 1; a bi-prediction motion vector is generated consisting of a first motion vector (MVX) and a second motion vector (MVX’) of L(1-X) direction. Two variations of MVX’ are tested.

(test 1) MVX’.x = sign*(MVX.x + offset); MVX’.y = sign*(MVX.y + offset);

(test 2) MVX’.x = sign*(MVX.x) + offset; MVX’.y = sign*(MVX.y) + offset;

* sign = 1 if two reference directions are the same, otherwise sign = -1

* offset = 4 (quarter pel)

Secondly, the first triangle partition selects a first candidate from the triangle candidate list; an additional flag is signaled to indicate the reference direction of the first triangle partition; and motion vector of the first triangle partition is determined as the motion vector of the indicated reference direction from the first candidate. The second triangle partition selects a second candidate from the triangle candidate list, wherein the second candidate is different from the first candidate, and the reference direction of the second triangle partition is derived as not same as the first triangle partition. And motion vector of the second triangle partition is determined as the motion vector of the derived reference direction from the second candidate.

CE4-4.8 in JVET-N0211 (DJI & PKU)

Considering both inter merge mode and triangle mode need to construct merge list and derive motion information from the lists, this contribution proposes to reuse current inter merge list as the list for triangle mode as it can remove triangle merge list construction and simply motion information derivation of triangle mode.

To further reduction motion derivation complexity for triangle mode, this contribute also proposes to use candidates before HMVP in the inter merge list, which means only four spatial candidates and tmvp candidates in the list are used at triangle motion information derivation.

CE4-4.9 in JVET-N0418 (Qualcomm)

In CE4-4.9, it is proposed that regular merge candidate list is re-used as the first step of the TPM merge candidate list generation. In the second step, the candidates in regular merge candidate list are converted into uni-directional motion vectors and are added into the final TPM merge candidate list until the list reaches the predefined maximum size (5). Among them, test 4.9.a and test 4.9.b use the complete set of regular merge list, which includes motion vectors candidates of spatial and temporal neighbors, history-based candidates and pairwise based candidate and zero MVs. While test 4.9.c and test 4.9.d use candidates from spatial and temporal neighbors only.

Meanwhile, in the test 4.9.a and test 4.9.c, pruning is performed as it is in VTM-4.0 and in the test 4.9.b and 4.9.d, pruning in the 2nd step of TPM merge candidate list generation is removed.

CE4 related contributions

| JVET-VC number | Title |
| --- | --- |
| JVET-N0084 | CE4-related: Motion compression for HMVP buffers and MV line buffer |
| JVET-N0085 | CE4-related: MV rounding unification |
| JVET-N0086 | CE4-related: Reduction of interactions between bi-prediction coding tools |
| JVET-N0087 | CE4-related: Simplification of triangle merging candidate list derivation on top of CE4-4.1c |
| JVET-N0126 | CE4-related: Signaling MMVD syntax elements |
| JVET-N0127 | CE4-related: MMVD enabling signaling in SPS |
| JVET-N0213 | CE4-related: Remove TMVP merge and AMVP candidate for the specified block sizes |
| JVET-N0265 | CE4-related: Simplification of HMVP in AMVP mode |
| JVET-N0286 | CE4-related: Simplified coding of the BPWA index |
| JVET-N0287 | CE4-related: Simplified coding of the MVP index |
| JVET-N0340 | CE4-related: An improved method for triangle merge list construction |
| JVET-N0380 | CE4-related: Fix of MMVD signalling |
| JVET-N0385 | CE4-related: modification of HMVP update process inside the shared merging candidate list region |
| JVET-N0400 | CE4-related: Signaling of maximum number of triangle candidates |
| JVET-N0439 | CE4-related: supplementary information regarding CE4-4.4 |
| JVET-N0449 | CE4-related: Simplification on MMVD distance table |
| JVET-N0486 | CE4-related: Simplification for share merge list and HMVP harmonization process |
| JVET-N0487 | CE4-related: Further simplification of triangle prediction merging candidate list derivation |
| JVET-N0499 | CE4-related: Simplification for merge list derivation in triangular prediction mode on top of test CE4-4.6.b |
| JVET-N0525 | CE4-related: CE4-4.5 pruning reduction in TPM using regular merge candidates |

Non CE4 contributions

| JVET-VC number | Title |
| --- | --- |
| JVET-N0165 | Non-CE4: On spatial candidate list construction |
| JVET-N0166 | Non-CE4: Advanced Multi-hypothesis Inter Prediction for bandwidth reduction in B frame |
| JVET-N0167 | Non-CE4: Advanced Multi-hypothesis Inter Prediction |
| JVET-N0168 | Non-CE4: AMVR cost calculation modification in motion estimation stage |
| JVET-N0200 | Non-CE4: Simplification of inter MVP list generation for 4x4 block |
| JVET-N0203 | Non-CE4: Modification of the temporal merging candidate for the inter merge mode |
| JVET-N0240 | Non-CE4: Simplification of merge MVD derivation |
| JVET-N0267 | Non-CE4: shared merge list without double HMVP tables |
| JVET-N0277 | Non-CE4: Parallel-friendly pairwise average merge candidate derivation |
| JVET-N0309 | Non-CE4: Switched half-pel interpolation filter |
| JVET-N0332 | Non-CE4: MVD scaling issue for LTRPs |
| JVET-N0373 | Non-CE4: HMVP unification between the Merge and MVP list |
| JVET-N0434 | Non-CE4: Unified context model of AMVR and Affine AMVR |
| JVET-N0447 | Non-CE4: On triangle merge mode |
| JVET-N0448 | Non-CE4: On MMVD signaling |
| JVET-N0462 | Non-CE4: Mismatch between text specification and reference software on inter prediction mode |
| JVET-N0471 | non-CE4: Simplification of decoding process for SMVD reference indices |
| JVET-N0476 | Non-CE4: Constraints of picture size in VVC |
| JVET-N0483 | Non-CE4: Simplification of triangle partition and SBT combination |
| JVET-N0500 | Non-CE4: Restrictions on triangular merge list size |

Recommendations