| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0025 |
| --- | --- |

| Title: | CE5: Summary Report on Adaptive Loop Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Vadim Seregin Ching-Yeh Chen | Tel: Email: | vseregin@qti.qualcomm.com chingyeh.chen@mediatek.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document provides a summary report of Core Experiment 5 on Adaptive Loop Filter (ALF). There are two categories of the tests: line buffer reduction (CE5-1 and CE5-2) and coding efficiency improvement (CE5-3 and CE5-4). The subjective testing for the line buffer reduction category is required.

# Detailed description of tests

In VTM4, ALF requires 7 line buffers for luma component and 4 line buffers for chroma component as depicted on the below pictures.

Figure 1. ALF luma line buffers (from JVET-N0088).

Figure 2. ALF chroma line buffers (from JVET-N0088).

In the line buffer reduction tests, the ALF SIMD was disabled for both anchor and CE tests, so the runtime is measured against modified VTM4 anchor. The SIMD implementation was provided after the deadline (March 14, 2019) and only for CE5-1.

#### CE5-1 Adaptive loop filter with virtual boundary processing (JVET-N0088)

#### CE5-1.1

In JVET-M0164, ALF with virtual boundary (VB) processing was proposed to remove the line buffers associated with ALF. The position of VB is set equal to 4 lines above CTU row boundaries and when the samples above the VB (region A in Figure 3) are filtered, samples below the VB (region B in Figure 3) are unavailable and are replaced with generated samples by padding. Similarly, when the samples below the VB (region B in Figure 4) are filtered, samples above the VB (region A in Figure 4) are replaced with generated samples by padding.

Figure 3. Filtering of luma samples above the VB.

Figure 4. Filtering of luma samples below the VB.

For chroma, VBs are set as two chroma lines above CTU row boundaries.

The padding technique of accumulating gradients for a 4x4 block, which are needed for classification, is different. As shown in Figure 5, the 4x4 green block needs to accumulate subsampled gradients in the 8x8 region, and the VB is right below the 4x4 green block. Instead of using padded samples to generate gradients below the VB, the gradients below the VB are replaced with the nearest gradient on the same column above the VB.

Figure 5. Modification of accumulating gradients for a 4x4 block in VB processing.

The above described changes are tested in CE5-1.1.

#### CE5-1.2

A weighted averaging process is applied after the filtering process, and the final ALF output sample is a weighted averaging of the filtered sample and the ALF input sample. The weight for the filtered sample is dependent on the number of padded lines in the filtering process as follows:

| Number of padded lines | Weight for ALF original input | Weight for ALF filtered sample |
| --- | --- | --- |
| 0 | 0 | 1 |
| 1 | 0.5 | 0.5 |
| 2 | 0.75 | 0.25 |
| 3 | 0.875 | 0.125 |

The described changes are tested on top of CE1-5.1

#### CE5-1.3

A combination with the implicit filter on/off control introduced in CE5-2.2 is tested as CE5-1.3. The implicit on/off decision is made according to the ratio between the absolute sum of filter coefficients applied to padded samples and the absolute sum of filter coefficients applied to real samples. If the ratio is greater than 0.6, then ALF is not applied to this sample. Otherwise, ALF with the proposed method is applied to this sample.

| Test | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE5-1.1 | Virtual boundary processing with padding samples/gradients only | Jenny Lai (MediaTek) | Nan Hu (Qualcomm) |
| CE5-1.1 | Virtual boundary processing with padding samples/gradients and weighted averaging | Jenny Lai (MediaTek) | Anand Meher Kotra (Huawei) |
| CE5-1.2 | Virtual boundary processing with padding samples/gradients and weighted averaging and adaptive on/off control (combined with CE5-2.2) | Jenny Lai (MediaTek) | Anand Meher Kotra (Huawei) |

#### CE5-2 Loop filter line buffer reduction (JVET-N0180)

#### CE5-2.1

In JVET-M0301, ALF with virtual boundary processing was also proposed. In CE5-2.1 modification to ALF block classification and truncated filtering are introduced.

To derive classification for 4x4 block located above or below virtual boundary (VB) as shown on the below Figure 6, only the samples which are located above/below the VB are utilized. Since the number of samples involved in the classification is reduced, the calculated Laplacian activity is differently scaled comparing to the block when all samples are used. The size VB is set to 4 lines above CTU row boundaries and the same as that in the CE5-1.

Figure 6. Modified ALF classification.

In the ALF filtering part, depending on the line number in the VB, the filter support is truncated to avoid using samples located above or below VB. The truncation is done in a symmetrical way to keep the symmetrical property of the ALF filter as shown on the next figure. In the truncated filtering, filter coefficients are switched per line depending on the line number for both luma and chroma color components.

Figure 8. Modified luma ALF filtering at virtual boundary.

Similar truncation is performed for chroma filtering as shown below.

Figure 9. Modified chroma ALF filtering at virtual boundary.

#### CE5-2.2

In this test, conditional ALF filtering switch is introduced. Depending on the line number, the absolute sum of the filter coefficients corresponding to the samples located above or below VB, which are not used in the ALF filtering, and the remaining coefficients are calculated. Depending on the ratio between those two sums with the threshold equal to 0.6, the decision is made whether ALF filter is applied.

For example, for lines M and H to determine whether ALF filter is applied the following condition is checked if (2*abs c0>(T* 2*abs c1+abs c2+absc3+absc4+absc5+absc6+absc7+absc8+absc9+absc10+absc11+absc12), and if the condition is true, then the ALF filtering is disabled for lines M and H.

Similar check is performed for other lines, so the condition is derived for each filter class and for each line of the block.

This change is tested on top of CE5-2.1 test.

#### CE5-2.3

A combination of the test CE5-1.2, which uses padding for the ALF filtering and weighted filtering for the ALF input and filtered samples, while the classification method is replaced by the method from CE5-2.2, and filtering on/off control from CE5-2.2 is added.

| Test | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE5-2.1 | Virtual boundary processing with padding samples and truncated filtering without adaptive on/off control | Huawei | Jenny Lai (MediaTek) |
| CE5-2.2 | Virtual boundary processing with padding samples and truncated filtering with adaptive on/off control | Huawei | Jenny Lai (MediaTek) |
| CE5-2.3 | Virtual boundary processing with padding samples and weighted averaging and adaptive on/off control (combined with CE5-1.2) | Huawei | Jenny Lai (MediaTek) |

#### CE5-3 Results of tests CE5-3.1, CE5-3.2, CE5-3.3 and CE5-3.4 on Non-Linear Adaptive Loop Filter (JVET-N0242)

#### CE5-3.1

In JVET-M0385, an adaptive clipping operation is performed on the input sample values of ALF. The goal of this adaptive clipping is to reduce the impact and to cap the difference between the neighbor sample values and the central input sample value during the ALF filtering process.

In this proposal, the ALF filter is modified as follows:

O'x,y=Ix,y+(i,j)≠(0,0)wi, j.KIx+i,y+j-Ix,y,k(i,j),

where Kd,b=min⁡(b,max-b,d) is the clipping function, and k(i,j) are clipping parameters, which depend on the position (i,j) of the filter coefficient. The encoder performs the optimization to find the best k(i,j).

In the CE5.3 implementation, the clipping parameters ki,j are specified for each ALF filter, one clipping value is signaled per filter coefficient. It means that up to 12 clipping values can be signalled in the bitstream per luma filter and up to 6 clipping values for the chroma filter.

The clipping values used in the tests are the following:

|  | Intra/Inter tile group |
| --- | --- |
| Luma | { 1024, 181, 32, 6 } |
| Chroma | { 1024, 161, 25, 4 } |

The selected clipping values are coded in the “alf_data” syntax element by using a Golomb encoding scheme corresponding to the index of the clipping value in the above Table 1. This encoding scheme is the same as the encoding scheme for the filter index.

#### CE5-3.2

On top of CE5-3.1, a flag is signaled to indicate whether clipping of the ALF input is performed and associated clipping values are signaled. If the flag is disabled, the clipping parameters are not signaled. The flag is separately introduced for each luma and chroma components.

#### CE5-3.3

On top of CE5-3.1, the number of clipping operations is reduced from 24 to 8 clippings for luma component and the clipping operation is performed for 8 outer samples for chroma components, which reduces it from 12 to 8 clippings. The positions of the reduced number of clipping operations are illustrated by an ‘x’ in the filter patterns shown in Figure 10.

Figure 10. Positions where the clipping operations is reduced, shown by ‘x’ for luma (left) and chroma (right) components.

#### CE5-3.4

On top of CE5-3.1, the luma filter support is changed from 7x7 to 5x5 when clipping is used, and the amount of clipping is reduced as in CE5-3.3. The number of clipping operations for luma is thus reduced from 12 clippings (24 clippings for 7x7 filter support) to 8 clippings.

| Test | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE5-3.1 | Non-linear ALF (NL-ALF) as in JVET-M0385. | Canon | Nan Hu (Qualcomm) |
| CE5-3.2 | Test CE5-3.1 + additional NL-ALF signaling + encoder optimization. | Canon | Nan Hu (Qualcomm) |
| CE5-3.3 | Test CE5-3.1 + reduced number of clipping operations for NL-ALF. | Canon | Nan Hu (Qualcomm) |
| CE5-3.4 | TEST CE5-3.1 + reduced number of overall operations and line buffer for NL-ALF. | Canon | Nan Hu (Qualcomm) |

#### CE5-4 Coding tree block based adaptive loop filter (JVET-N0415)

#### CE5-4.1

In VTM4, adaptive parameter set (APS) was adopted, each APS contains one set of ALF filters (up to 25 filters for luma and 1 filter for chroma components), up to 32 APSs are supported.

In CE5-4.1 test, an APS containing filters from the previous pictures can be selected in the tile group header. The APSs are updated as a first-in-first-out (FIFO) manner.

#### CE5-4.2

For luma component, when ALF is applied to a luma CTB, the choice among 16 fixed, 5 previous filter (APSs) or 1 signaled filter sets is indicated. Only the filter set index is signalled.

For a tile group, only one new set of 25 filters can be signaled. If a new set is signalled for a tile group, all the luma CTBs in the same tile group share that filter set. Fixed filter sets can be used to predict the new tile group level filter set and can be used as candidate filter sets for a luma CTB as well.

The fixed filters consist of total 64 7x7 filters (each filter contains 13 coefficients) which are only used for luma component. For each class, a mapping is applied to define which 16 fixed filters from the 64 filters can be used for that class. The choice index (0-15) of each class is signaled as fixed filter index. The total memory needed to store fixed filters is 1132 bytes, consisting of 64 filters x 13 coefficients/filter x 8 bits/coefficient and 16 choices/class x 25 classes x 6 bits/index for mapping table.

For chroma component, when ALF is applied to a chroma CTB, if a new filter is signalled for a tile group, the CTB uses the new filter; otherwise, the most recent previous picture chroma filter satisfying the temporal scalability constraint is applied.

#### CE5-4.3

CE5-4.2 test is performed for the QPs equal to 12, 17, 22 and 27.

#### CE5-4.4

CE5-4.2 test, where fixed filters are not used, is performed.

#### CE5-4.5

CE5-4.2 test, where previous filter sets are not used, is performed.

#### CE5-4.6

CE5-4.5 test is performed for the QPs equal to 12, 17, 22 and 27.

| Test | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE5-4.1 | VTM with temporal filters mimicking APS | Nan Hu (Qualcomm) | Patrice Onno (Canon) |
| CE5-4.2 | Luma CTBs with ALF on can switch among 1 tile group level signaled filter set, 16 fixed filter sets and at most 5 available sets in APS. All chroma CTBs with ALF on in a picture can only use the tile group level signaled filter or one temporal filter | Nan Hu (Qualcomm) | Patrice Onno (Canon) |
| CE5-4.3 | Test CE5-4.2 with QP = 12, 17, 22 and 27 | Nan Hu (Qualcomm) | Patrice Onno (Canon) |
| CE5-4.4 | Test CE5-4.2 without fixed filters | Nan Hu (Qualcomm) | Patrice Onno (Canon) |
| CE5-4.5 | Test CE5-4.2 without temporal filters | Nan Hu (Qualcomm) | Patrice Onno (Canon) |
| CE5-4.6 | Test CE5-4.5 with QP = 12, 17, 22 and 27 | Nan Hu (Qualcomm) | Patrice Onno (Canon) |

# Test results

| Test# | Description | Document# |
| --- | --- | --- |
| CE5-1.1 | Virtual boundary processing with padding samples/gradients only | JVET-N0088 |
| CE5-1.2 | Virtual boundary processing with padding samples/gradients and weighted averaging | JVET-N0088 |
| CE5-1.3 | Virtual boundary processing with padding samples/gradients, weighted averaging and adaptive on/off control | JVET-N0088 |
| CE5-2.1 | Virtual boundary processing with padding samples and truncated filtering without adaptive on/off control | JVET-N0180 |
| CE5-2.2 | Virtual boundary processing with padding samples and truncated filtering with adaptive on/off control | JVET-N0180 |
| CE5-2.3 | Virtual boundary processing with padding samples and weighted averaging and adaptive on/off control | JVET-N0180 |
| CE5-3.1 | Non-linear ALF (NL-ALF) as in JVET-M0385. | JVET-N0242 |
| CE5-3.2 | Test CE5-3.1 + additional NL-ALF signaling + encoder optimization. | JVET-N0242 |
| CE5-3.3 | Test CE5-3.1 + reduced number of clipping operations for NL-ALF. | JVET-N0242 |
| CE5-3.4 | TestCE5-3.1 + reduced number of overall operations and line buffer for NL-ALF. | JVET-N0242 |
| CE5-4.1 | VTM with temporal filters mimicking APS | JVET-N0415 |
| CE5-4.2 | Luma CTBs with ALF on can switch among 1 tile group level signaled filter set, 16 fixed filter sets and those available sets in APS. All chroma CTBs with ALF on in a picture can only use the tile group level signaled filter or one temporal filter | JVET-N0415 |
| CE5-4.3 | Test CE5-4.2 with QP = 12, 17, 22 and 27 | JVET-N0415 |
| CE5-4.4 | Test CE5-4.2 without fixed filters | JVET-N0415 |
| CE5-4.5 | Test CE5-4.2 without temporal filters | JVET-N0415 |
| CE5-4.6 | Test CE5-4.5 with QP = 12, 17, 22 and 27 | JVET-N0415 |

|  | All Intra |  |  |  |  |  | Random Access |  |  |  |  |  |  |  |  | Low Delay B |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT |  | DecT | Y |  | U |  | V |  | EncT |  | DecT | Y |  | U |  | V |  | EncT |  | DecT |
| CE5-1.1 | 0.08 | 0.04 | 0.05 | 100% | 102% |  | 0.08 | -0.03 |  | 0.01 |  | 100% |  | 103% |  | 0.10 | 0.05 |  | -0.17 |  | 100% |  | 103% |  |
| CE5-1.2 | 0.07 | 0.11 | 0.11 | 100% | 103% |  | 0.09 | 0.02 |  | 0.02 |  | 100% |  | 102% |  | 0.09 | 0.14 |  | -0.03 |  | 100% |  | 103% |  |
| CE5-1.3 | 0.07 | 0.11 | 0.11 | 100% | 103% |  | 0.10 | 0.04 |  | 0.01 |  | 100% |  | 103% |  | 0.11 | 0.18 |  | 0.07 |  | 100% |  | 104% |  |
| CE5-2.1 | 0.06 | 0.07 | 0.07 | 100% | 101% |  | 0.10 | 0.06 |  | 0.03 |  | 100% |  | 102% |  | 0.09 | 0.18 |  | 0.09 |  | 100% |  | 102% |  |
| CE5-2.2 | 0.06 | 0.09 | 0.09 | 100% | 101% |  | 0.10 | 0.04 |  | 0.08 |  | 100% |  | 101% |  | 0.06 | 0.10 |  | 0.10 |  | 100% |  | 101% |  |
| CE5-2.3 | 0.07 | 0.11 | 0.11 | 100% | 100% |  | 0.09 | 0.02 |  | 0.03 |  | 100% |  | 100% |  | 0.08 | 0.45 |  | 0.05 |  | 101% |  | 102% |  |
| CE5-3.1 | -0.45 | -0.35 | -0.60 | 101% | 103% |  | -0.67 | -0.96 |  | -1.08 |  | 100% |  | 104% |  | -0.73 | -0.37 |  | -0.77 |  | 103% |  | 101% |  |
| CE5-3.2 | -0.45 | -0.35 | -0.59 | 101% | 102% |  | -0.68 | -1.01 |  | -1.12 |  | 100% |  | 103% |  | -0.75 | -0.52 |  | -0.86 |  | 104% |  | 102% |  |
| CE5-3.3 | -0.35 | -0.39 | -0.62 | 100% | 100% |  | -0.50 | -0.96 |  | -0.99 |  | 99% |  | 99% |  | -0.51 | -0.51 |  | -0.33 |  | 101% |  | 99% |  |
| CE5-3.4 | 0.12 | -0.43 | -0.68 | 99% | 99% |  | 0.17 | -1.01 |  | -1.09 |  | 98% |  | 99% |  | 0.00 | -0.37 |  | -0.90 |  | 100% |  | 97% |  |
| CE5-4.1 | 0.00 | 0.00 | 0.00 | 100% | 100% |  | -0.14 | -0.16 |  | -0.17 |  | 100% |  | 100% |  | -0.38 | -0.19 |  | -0.07 |  | 100% |  | 100% |  |
| CE5-4.2 | -0.15 | -0.04 | -0.04 | 100% | 100% |  | -0.52 | -0.14 |  | -0.21 |  | 100% |  | 101% |  | -0.62 | -0.27 |  | -0.32 |  | 100% |  | 100% |  |
| CE5-4.3 | -0.02 | 0.00 | 0.00 | 99% | 99% |  | -0.26 | -0.11 |  | -0.22 |  | 99% |  | 100% |  | -0.29 | -0.25 |  | -0.38 |  | 100% |  | 100% |  |
| CE5-4.4 | 0.00 | 0.00 | 0.00 | 100% | 100% |  | -0.33 | -0.20 |  | -0.19 |  | 100% |  | 100% |  | -0.53 | -0.53 |  | -0.26 |  | 100% |  | 99% |  |
| CE5-4.5 | -0.15 | -0.04 | -0.04 | 100% | 98% |  | -0.35 | 0.04 |  | 0.01 |  | 100% |  | 100% |  | -0.43 | -0.16 |  | -0.03 |  | 100% |  | 100% |  |
| CE5-4.6 | -0.02 | 0.00 | 0.00 | 99% | 99% |  | -0.13 | 0.03 |  | -0.02 |  | 99% |  | 99% |  | -0.15 | -0.14 |  | -0.15 |  | 99% |  | 100% |  |

# Complexity Analysis

In VTM-4, ALF filter can be switched on and off for each CTB, and memory storage of 332 bytes is required for filter coefficients calculated as follows:

25 classes x 13 coefficients x 8 bits

1 chroma filter x 7 coefficients x 8 bits

Maximum 32 ALF filter sets, which are stored in the APS, is supported, so the total memory requirement is 10,624 bytes.

| Test | Operations of classification process per luma sample |  | Operations of filtering process per luma sample |  | Operations of filtering process per chroma sample |  | Frequency change of filters (Y, Cb, Cr) | Line buffer size (L, C) | memory for storing filters, bytes | Additional operations per other units |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Anchor | Add | 15.125 | Add | 25 | Add | 13 | 4x4 (Y) tile group (Cb) tile group (Cr) | 7 (L) 4 (C) | 10,624 | loading 332 bytes per tile group |
|  | Comp | 4.9375 | Comp | 4 | Comp | 1 |  |  |  |  |
|  | Mult | 0.125 | Mult | 13 | Mult | 7 |  |  |  |  |
|  | Shift | 2.5625 | Shift | 1 | Shift | 1 |  |  |  |  |
| CE5-1.1 | Gradient computation is skipped for 2 lines when compared with the anchor for the blocks near VB two conditions to check avobe or below VB |  | as ahchor |  | as ahchor |  | as anchor | 0 (L) 0 (C) | as anchor |  |
| CE5-1.2 | Gradient computation is skipped for 2 lines when compared with the anchor for the blocks near VB two conditions to check avobe or below VB |  | *samples w VB: Add 27 Comp 4 Mult 14 Shift 3 *samples wo VB: as anchor |  | *samples w VB: Add 15 Comp 1 Mult 8 Shift 3 *samples wo VB: as anchor |  | as anchor | 0 (L) 0 (C) | as anchor | 1 Comp per sample line for 4x4 blcok with VB; the numbers of luma and chroma lines filtered by VB are 6 and 4 in one CTB |
| CE5-1.3 | Gradient computation is skipped for 2 lines when compared with the anchor for the blocks near VB two conditions to check avobe or below VB |  | *samples w VB: Add 27 Comp 4 Mult 14 Shift 3 *samples wo VB: as anchor |  | *samples w VB: Add 15 Comp 1 Mult 8 Shift 3 *samples wo VB: as anchor |  | as anchor (luma on/off unit is 4x1 samples unit) | 0 (L) 0 (C) | + 50 bits for filtering on/off | As in CE5-1.2 and Add: 29 Mul: 12 Comp: 6 Shift: 1 per luma filter |
| CE5-2.1 | Gradient computation is skipped for 2 lines when compared with the anchor for the blocks near VB, two conditions to check avobe or below VB Otherwise, as anchor |  | *samples with VB: (based on the distance from VB) line 0: Add 23 Comp 6 Mult 12 Shift 1 line 1: Add 17 Comp 4 Mult 10 Shift 3 line 2: Add 7 Comp 4 Mult 4 Shift 3 *samples w/o VB: as anchor |  | *samples with VB: (based on the distance from VB) line 0: Add 11 Comp 3 Mult 6 Shift 1 line 1: Add 5 Comp 1 Mult 3 Shift 1 |  | For the blocks at VB, filter is swithced for each 4 luma samples. For chroma, filter is switched for CU line. | 0 (L) 0 (C) | For the blocks at VB, pre computed filters use 975 bytes= 25 x 3 x 13 for luma, and plus 21 = 3x7 bytes for chroma |  |
| CE5-2.2 | Gradient computation is skipped for 2 lines when compared with the anchor for the blocks near VB, two conditions to check avobe or below VB. Otherwise, as anchor |  | Same as CE5-2.1 |  | Same as CE5-2.1 |  | Same as CE 5-2.1 | 0 (L) 0 (C) | as in CE5-2.1 + 100 bits for filtering on/off | For filter on/off: Add 29 Mul: 12 Comp: 6 Shift: 1 per luma filter per 4 luma samples in the line |
| CE5-2.3 | Gradient computation is skipped for 2 lines when compared with the anchor for the blocks near VB, two conditions to check avobe or below VB. Otherwise, as anchor. |  | *samples w VB: Add 27 Comp 4 Mult 14 Shift 3 *samples wo VB: as anchor |  | *samples w VB: Add 15 Comp 1 Mult 8 Shift 3 *samples wo VB: as anchor |  | as anchor (luma on/off unit is 4x1 samples unit) | 0 (L) 0 (C) | as in CE5-2.1 + 50 bits for filtering on/off | As in CE5-1.2 and in CE5-2.2 together |
| CE5-3.1 | as anchor |  | Add; 49. Comp; 52. Mult; 12. Shift; 1. | Add; 25. Comp; 25. Mult; 6. Shift; 1. | as anchor | as anchor | 12,224 | as anchor |
| CE5-3.2 | as anchor |  | as CE5-3.1 |  | as CE5-3.1 |  | as anchor | as anchor | as CE5-3.1 | as anchor |
| CE5-3.3 | as anchor |  | Add; 49. Comp; 20. Mult; 12. Shift; 1. | Add; 25. Comp; 17. Mult; 6. Shift; 1. | as anchor | as anchor | as anchor | as anchor |
| CE5-3.4 | as anchor |  | Add; 25. Comp; 28. Mult; 6. Shift; 1. | Add; 25. Comp; 25. Mult; 6. Shift; 1. | as anchor | 4 (L) 4 (C) | as anchor, when method is not used, otherwise 6,240 | as anchor |
| CE5-4.1 | as anchor |  | as anchor |  | as anchor |  | as anchor | as anchor | as anchor | as anchor |
| CE5-4.2 | as anchor |  | as anchor |  | as anchor |  | as anchor | as anchor | 11,756 | loading 332 bytes per CTU |
| CE5-4.3 | as anchor |  | as anchor |  | as anchor |  | as anchor | as anchor | 11,756 | loading 332 bytes per CTU |
| CE5-4.4 | as anchor |  | as anchor |  | as anchor |  | as anchor | as anchor | as anchor | loading 332 bytes per CTU |
| CE5-4.5 | as anchor |  | as anchor |  | as anchor |  | as anchor | as anchor | 1,132 | loading 332 bytes per CTU |
| CE5-4.6 | as anchor |  | as anchor |  | as anchor |  | as anchor | as anchor | 1,132 | loading 332 bytes per CTU |

# Cross-checker report

| Test# | Cross-checker | PSNR&BR Match (Y/N) | Run time matched? (Y/N) | Software implemented by cross-checker? (Y/N) | Software studied? (Y/N) | Significant inconsistencies between the description and the provided software? (Y/N) | Other comments? | Other tests? (Y/N) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CE5-1.1 | Nan Hu (Qualcomm) | Y | N/A(SIMD version results are unavailable) | N | Y | N | Non ALF SIMD, partial results | N |
| CE5-1.2 | Anand Meher Kotra (Huawei) | Y | N/A(SIMD version results are unavailable) | N | Y | N | N | N |
| CE5-1.3 | Anand Meher Kotra (Huawei) | Y | N/A(SIMD version results are unavailable) | N | Y | N | N | N |
| CE5-2.1 | Jenny Lai (MediaTek) | Y | N.A. | N | Y | N | Didn’t check the run time | N |
| CE5-2.2 | Jenny Lai (MediaTek) | Y | N.A. | N | Y | N | Didn’t check the run time | N |
| CE5-2.3 | Jenny Lai (MediaTek) | Y | N.A. | N | Y | N | Didn’t check the run time | N |
| CE5-3.1 | Nan Hu (Qualcomm) | Y | Y | N | Y | N | partial results | N |
| CE5-3.2 | Nan Hu (Qualcomm) | Y | Y | N | Y | N | partial results | N |
| CE5-3.3 | Nan Hu (Qualcomm) | Y | Y | N | Y | N | partial results | N |
| CE5-3.4 | Nan Hu (Qualcomm) | Y | Y | N | Y | N | partial results | N |
| CE5-4.1 | Patrice Onno (Canon) | Y (full) | Y | N | Y |  | Decoding time not reliable | N |
| CE5-4.2 | Patrice Onno (Canon) | Y (full) | Y | N | Y |  | Decoding time not reliable | N |
| CE5-4.3 | Patrice Onno (Canon) | Y (Classes C and D only) | Y | N | Y |  | Decoding time not reliable | N |
| CE5-4.4 | Patrice Onno (Canon) | Y (Full) | Y | N | Y |  | Decoding time not reliable | N |
| CE5-4.5 | Patrice Onno (Canon) | Y (Full, in progress) | Y | N | Y |  | Decoding time not reliable | N |
| CE5-4.6 | Patrice Onno (Canon) | Y (Classes C and D only, in progress) | Y | N | Y |  | Decoding time not reliable | N |

# CE related contributions

JVET-N0243 “Non-CE5: Complementary results of tests CE5-3 on Non-Linear ALF.”

JVET-N0416 “CE5-related: Unification of picture boundary and line buffer handling for ALF”

JVET-N0488 “CE5-related: Syntax redundancy removal in adaptive loop filter”