| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0026-v1 |
| --- | --- |

| Title: | CE6: Summary Report on Transforms and Transform Signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Name(s) Xin Zhao Hilmi E. Egilmez | Tel: Email: | xinzzhao@tencent.com hegilmez@qti.qualcomm.com |
| Source: | CE6 coordinators |  |  |

_____________________________

# Abstract

This contribution summarizes the activities of Core Experiment (CE) on Transforms and Transform Signaling. The goal of this CE is to study transform design and signaling for the VVC standard. The CE studies were divided into three categories, including:

(1) CE6-1: Unification between MTS and Transform Skip mode (4 tests, 3 tests withdrawn)

(2) CE6-2: Reduced transform candidates of MTS (13 tests, 3 tests withdrawn)

(3) CE6-3: Secondary transform (5 tests)

In this CE all experiments were done using based on the VTM-4.0 SW. This document summarizes the test results, brief experiment definition, cross-check reports and complexity measurements, and also related contributions.

# Introduction

The purpose of this Core Experiment (CE) is to investigate the coding tools related to transforms and transform signaling on top of the VTM-4.0. In this CE, the tradeoff between coding efficiency and complexity of different transformation tools will be investigated.

# Experiment List

Core experiments (CEs) are organized according to the following five categories, and the following table lists all the experiments in each category, and the corresponding input document to the Geneva meeting. In total 16 tests were studied in this CE, and 6 tests were withdrawn from the original CE6 plan.

The 16 tests reported in this CE6 summary are listed below.

| Test # | Doc # | Description | Tester | Cross-checker |
| --- | --- | --- | --- | --- |
| CE6-1.1a | JVET-N0360 | One dimensional TS (IDT used as horizontal or vertical transform) is applied up to 32-point. | X. Zhao (Tencent) | T. Tsukuba (Sony) |
| CE6-2.1a | JVET-N0053 | 2-mode MTS without implicit transform selection, Allowed intra picture 64x64 CU splits: QT, TT, BT | J. Lainema (Nokia) | C. Hollmann (Ericsson) |
| CE6-2.1b |  | 2-mode MTS without implicit transform selection, Allowed intra picture 64x64 CU splits: QT, TT | J. Lainema (Nokia) | C. Hollmann (Ericsson) |
| CE6-2.1c |  | 2-mode MTS without implicit transform selection, Allowed intra picture 64x64 CU splits: QT | J. Lainema (Nokia) | C. Hollmann (Ericsson) |
| CE6-2.2a | JVET-N0208 | Non-normative change with 3 modes | K. Choi (Samsung) | X. Zhao (Tencent) |
| CE6-2.2b |  | Non-normative change with 2 modes | K. Choi (Samsung) | X. Zhao (Tencent) |
| CE6-2.2c |  | Normative change with 3 modes | K. Choi (Samsung) | X. Zhao (Tencent) |
| CE6-2.2d |  | Normative change with 2 modes | K. Choi (Samsung) | X. Zhao (Tencent) |
| CE6-2.3a | JVET-N0490 | Simplified transform selection with reduced modes and no size-32 DCT-8, Number of evaluated candidates: 3 | C. Hollmann (Ericsson) | J. Lainema (Nokia) |
| CE6-2.3b |  | Simplified transform selection with reduced modes and no size-32 DCT-8, Number of evaluated candidates: 4 | C. Hollmann (Ericsson) | J. Lainema (Nokia) |
| CE6-2.3c |  | Simplified transform selection with reduced modes and no size-32 DCT-8, Number of evaluated candidates: 5 | C. Hollmann (Ericsson) | J. Lainema (Nokia) |
| CE6-3.1a | JVET-N0193 | ( 4 transform sets. 2 transforms per set ) | M. Koo (LGE) | P. Philippe (Orange) |
| CE6-3.1b |  | ( 4 transform sets. 2 transforms per set ) + (Secondary transform uses at most maximum 8 multiplications/sample ) | M. Koo (LGE) | P. Philippe (Orange) |
| CE6-3.1c |  | ( 4 transform sets. 2 transforms per set ) + ( Secondary transform uses at most maximum 8 multiplications/sample ) + ( Secondary transform is disabled for 4x4 TU ) | M. Koo (LGE) | P. Philippe (Orange) |
| CE6-3.1d |  | ( 4 transform sets. 2 transforms per set ) + ( Secondary transform uses at most maximum 8 multiplications/sample ) + ( 16x48 matrices are employed instead of 16x64 ones ) | M. Koo (LGE) | P. Philippe (Orange) |
| CE6-3.1e |  | ( 4 transform sets. 1 transforms per set ) | M. Koo (LGE) | P. Philippe (Orange) |

The following tests were withdrawn from the original CE6 plan.

| Test # | Description |
| --- | --- |
| CE6-1.1 | One dimensional TS (IDT used as horizontal or vertical transform) is applied up to 4-point. |
|  | One dimensional TS (IDT used as horizontal or vertical transform) is applied up to 8-point. |
|  | One dimensional TS (IDT used as horizontal or vertical transform) is applied up to 16-point. |
| CE6-2.1 | 2-mode MTS with implicit transform selection, Allowed intra picture 64x64 CU splits: QT, TT, BT |
|  | 2-mode MTS with implicit transform selection, Allowed intra picture 64x64 CU splits: QT, TT |
|  | 2-mode MTS with implicit transform selection, Allowed intra picture 64x64 CU splits: QT |

# Test conditions

## Software

The Test Model VTM-4.0 [1] shall be used for these experiments, as specified in JVET-M1010 [2].

## Test configurations

This CE shall use the following two test conditions:

- Common Test Conditions (CTC).

- CTC with inter MTS enabled

The anchor for the above first test condition is VTM-4.0 using CTC, and the anchor for the above second test condition is VTM-4.0 using CTC w/ inter MTS enabled.

The following requirements apply to all tests defined in this CE:

- Any encoder or decoder optimizations (e.g., SIMD or encoder optimizations) that are not an essential part of the proposed method shall be described, and individual experiment results on the related optimizations shall be reported.

- For CE6-3 tests which have impact on the transform precision, i.e., modifications on transform cores or addition of stages/transforms (such as adjustment and secondary), results using low-QP configurations, i.e., QP 12, 17, 22 and 27 shall be provided. For low-QP configuration, simulations results using All Intra configuration are mandatory and simulation results using Random Access and Low Delay B (or P) configurations are optional, and the total number of frames to be coded is modified using the following encoder configuration:

--FramesToBeEncoded=100

- For CE6-1 tests which are related to transform skip mode, additional test results using TGM sequences need to be also reported.

# Timeline

The following table lists the timeline for the CE6 study planned in Marrakesh meeting.

Two CE tests (CE6-2.2 and CE6-2.3) are identified to have further SW updates after T3, the related SW changes were reported to be only straightforward bug-fix (To be confirmed by crosscheckers).

| T1 | Feb. 08, 2019 | - CE description is finalized and uploaded |
| --- | --- | --- |
| T2 | Feb. 25, 2019 (Two weeks after VTM-4.0 SW release) | - Proponents provide initial SW of CE tools - Cross-checkers may start initial study of tools / software |
| T3 | Mar. 04, 2019 | - SW of each CE test is finalized - Cross-checkers start final cross checks |
| T4 | Mar. 12, 2019 | - Complete results of each CE test shall be provided - Cross-checker report crosschecking results to proponents and CE coordinators - CE6 summary report is provided |

# Summary of results

The following table summarizes the results of CE6 tests using CTC configuration and VTM-4.0 as anchor.

|  | All Intra |  |  |  |  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-1.1a | 0.19% | -0.20% | -0.21% | 95% | 99% | 0.08% | -0.13% | -0.12% | 100% | 100% | 0.04% | -0.04% | 0.18% | 100% | 101% |
| CE6-2.1a | -0.32% | -1.81% | -1.83% | 107% | 100% | 0.04% | -0.72% | -0.83% | 98% | 99% | 0.16% | -0.07% | -0.47% | 99% | 100% |
| CE6-2.1b | -0.02% | -1.48% | -1.51% | 89% | 100% | 0.11% | -0.78% | -0.74% | 97% | 100% | 0.17% | -0.04% | -0.65% | 99% | 100% |
| CE6-2.1c | 0.20% | -0.25% | -0.26% | 75% | 100% | 0.18% | -0.13% | -0.25% | 97% | 100% | 0.14% | 0.34% | -0.31% | 99% | 99% |
| CE6-2.2a* | 0.00% | 0.00% | 0.00% | 99% | 101% | 0.00% | -0.02% | -0.06% | 100% | 101% | 0.01% | 0.04% | -0.12% | 101% | 100% |
| CE6-2.2b* | 0.11% | -0.06% | -0.08% | 87% | 101% | 0.05% | -0.12% | -0.06% | 100% | 101% | 0.03% | 0.06% | -0.42% | 100% | 101% |
| CE6-2.2c* | 0.00% | 0.01% | 0.02% | 99% | 101% | 0.00% | -0.08% | -0.07% | 100% | 101% | -0.04% | 0.12% | -0.14% | 100% | 101% |
| CE6-2.2d* | 0.09% | -0.08% | -0.11% | 87% | 100% | 0.04% | -0.05% | -0.15% | 98% | 100% | 0.04% | 0.16% | 0.15% | 99% | 100% |
| CE6-2.3a | 0,00% | 0,05% | 0,04% | 100% | 100% | 0,02% | 0,00% | -0,07% | 100% | 99% | -0,01% | -0,02% | 0,00% | 100% | 101% |
| CE6-2.3b | 0,02% | 0,03% | 0,03% | 90% | 100% | 0,02% | 0,01% | -0,03% | 99% | 99% | -0,01% | -0,02% | -0,42% | 100% | 99% |
| CE6-2.3c | 0,02% | 0,03% | 0,01% | 90% | 100% | 0,02% | 0,00% | -0,03% | 99% | 100% | -0,02% | -0,12% | -0,46% | 99% | 102% |
| CE6-3.1a | -1.56% | -0.64% | -1.09% | 124% | 96% | -0.84% | -1.35% | -1.89% | 110% | 98% | -0.27% | -0.37% | -0.95% | 107% | 98% |
| CE6-3.1b | -1.40% | -0.51% | -1.04% | 125% | 98% | -0.73% | -1.25% | -1.81% | 111% | 99% | -0.24% | -0.50% | -1.09% | 108% | 99% |
| CE6-3.1c | -1.28% | -0.35% | -0.80% | 123% | 98% | -0.68% | -1.03% | -1.59% | 111% | 99% | -0.23% | -0.71% | -0.78% | 107% | 99% |
| CE6-3.1d | -1.34% | -0.40% | -0.87% | 126% | 98% | -0.69% | -1.13% | -1.75% | 111% | 99% | -0.22% | -0.19% | -0.83% | 108% | 99% |
| CE6-3.1e | -1.21% | 0.13% | -0.17% | 101% | 96% | -0.61% | -0.90% | -1.35% | 105% | 98% | -0.19% | -0.06% | -0.40% | 104% | 98% |

*Results provided after contribution deadline.

The following table summarizes the additional results of CE6-1.1 test using CTC configuration with TGM test sequences and VTM-4.0 as anchor.

|  | All Intra |  |  |  |  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-1.1a | -0.80% | -0.38% | -0.41% | 96% | 98% | -0.62% | -0.43% | -0.48% | 101% | 99% | -0.41% | -0.55% | -0.49% | 103% | 103% |

The following table summarizes the results of CE6 tests using CTC configuration and VTM-4.0 with inter MTS enabled (--MTS=3) as anchor.

|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-1.1a* | 0.15% | -0.23% | -0.23% |  |  | 0.06% | -0.32% | 0.06% |  |  |
| CE6-2.1a | 0.19% | -0.84% | -0.91% | 89% | 100% | 0.26% | -0.66% | -0.74% | 90% | 100% |
| CE6-2.1b | 0.26% | -0.88% | -0.86% | 89% | 100% | 0.30% | -0.31% | -0.75% | 90% | 100% |
| CE6-2.1c | 0.32% | -0.23% | -0.35% | 89% | 100% | 0.29% | -0.25% | -0.53% | 90% | 100% |
| CE6-2.2a* |  |  |  |  |  |  |  |  |  |  |
| CE6-2.2b* |  |  |  |  |  |  |  |  |  |  |
| CE6-2.2c* |  |  |  |  |  |  |  |  |  |  |
| CE6-2.2d* |  |  |  |  |  |  |  |  |  |  |
| CE6-2.3a | 0,04% | -0,06% | -0,12% | 99% | 99% | 0,08% | -0,03% | 0,11% | 99% | 101% |
| CE6-2.3b | 0,08% | -0,06% | -0,07% | 97% | 100% | 0,11% | -0,20% | -0,21% | 97% | 98% |
| CE6-2.3c | 0,09% | -0,11% | -0,06% | 97% | 100% | 0,09% | -0,36% | -0,29% | 97% | 102% |
| CE6-3.1a* | -0.82% | -1.38% | -1.89% | 108% | 100% | -0.24% | -0.34% | -0.57% | 105% | 100% |
| CE6-3.1b* | -0.72% | -1.24% | -1.84% | 109% | 100% | -0.20% | -0.40% | -0.98% | 105% | 100% |
| CE6-3.1c* | -0.67% | -1.14% | -1.67% | 108% | 100% | -0.19% | -0.68% | -0.52% | 105% | 100% |
| CE6-3.1d* | -0.69% | -1.14% | -1.73% | 109% | 100% | -0.18% | -0.22% | -0.76% | 105% | 100% |
| CE6-3.1e* | -0.60% | -0.92% | -1.33% | 104% | 100% | -0.14% | -0.27% | -0.10% | 102% | 100% |

*Results provided after contribution deadline.

The following table summarizes the results of CE6 tests using Low QP configuration and VTM-4.0 with as anchor.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test # | Y | U | V | EncT | DecT |
| CE6-3.1a | -0.80% | 0.46% | 0.08% | 136% | 102% |
| CE6-3.1b | -0.63% | 0.64% | 0.25% | 137% | 102% |
| CE6-3.1c | -0.55% | 0.71% | 0.36% | 132% | 101% |
| CE6-3.1d | -0.63% | 0.66% | 0.28% | 137% | 102% |
| CE6-3.1e | -0.60% | 0.81% | 0.54% | 107% | 101% |

# Complexity Analysis

For the understanding of complexity, analysis on the following complexity aspects is provided for each CE test.

- Operation counts (number of multiplications, additions, and bit-shift operations) for individual transforms. If encoder and decoder use different number of transform operations as compared to the VTM, operation counts shall be reported for both encoder and decoder separately.

- Memory requirements for storing transform cores (in bytes).

- List of all combinations of transform types (e.g., DCT-2 and DST-7) and transforms block sizes (e.g., 4x4, 4x8, 8x4 and 8x8) used for primary transformation.

- List of all combinations of transforms block size and transform type used for secondary transformation if any.

## CE6-1.1

- Operation counts:

- By replacing N-length DCT-8 with IDT, multiplication per sample is reduced from N to 1, additions per sample is reduced from N-1 to 1.

- Memory requirements for storing transform cores (in bytes):

- Same as VTM-4.0

- List of all combinations of transform types for primary transformation.

- (IDT, IDT), (DCT2, DCT2), (DCT2, IDT), (IDT, DCT2), (DST7, DST7)

- List of all combinations of transforms block size and transform type used for secondary transformation if any.

- None

## CE6-2.1

- Operation counts:

- N/A

- Memory requirements for storing transform cores (in bytes):

- N/A

- List of all combinations of transform types for primary transformation.

- N/A

- List of all combinations of transforms block size and transform type used for secondary transformation if any.

- N/A

## CE6-2.2

- Operation counts:

- N/A

- Memory requirements for storing transform cores (in bytes):

- N/A

- List of all combinations of transform types for primary transformation.

- N/A

- List of all combinations of transforms block size and transform type used for secondary transformation if any.

- N/A

## CE6-2.3

- Operation counts:

- N/A

- Memory requirements for storing transform cores (in bytes):

- N/A

- List of all combinations of transform types for primary transformation.

- N/A

- List of all combinations of transforms block size and transform type used for secondary transformation if any.

- N/A

## CE6-3.1

The following complexity analysis on CE6-3.1 tests is provided in JVET-N0193-v3.

| Item | CE6-3.1a | CE6-3-1b | CE6-3.1c | CE6-3.1d | CE6-3.1e |
| --- | --- | --- | --- | --- | --- |
| Operation counts | Primary transform: Same as VTM Secondary transform: RST4x4: 256 (M), 240(A), 16(S) RST8x8: 1024 (M), 960(A), 64(S) Worst Case multiplication per sample: 16 | Primary transform: Same as VTM Secondary transform: RST4x4: 256 (M), 240(A), 16(S) RST8x8: 1024 (M), 960(A), 64(S) Worst Case multiplication per sample: 8 | Primary transform: Same as VTM Secondary transform: RST4x4: 256 (M), 240(A), 16(S) RST8x8: 1024 (M), 960(A), 64(S) Worst Case multiplication per sample: 8 | Primary transform: Same as VTM Secondary transform: RST4x4: 256 (M), 240(A), 16(S) RST8x8: 768 (M), 720(A), 48(S) Worst Case multiplication per sample: 8 | Primary transform: Same as VTM Secondary transform: RST4x4: 256 (M), 240(A), 16(S) RST8x8: 1024 (M), 960(A), 64(S) Worst Case multiplication per sample: 16 |
| Memory requirements | RST8x8: 16*64*8 /(8*1024)= 1 KB per kernel RST4x4: 16*16*8/(8*1024)= 0.25 KB per Kernel Total = 2*4*(1+ 0.25) = 10 KB | RST8x8: 16*64*8 /(8*1024)= 1 KB per kernel RST4x4: 16*16*8/(8*1024)= 0.25 KB per Kernel Total = 2*4*(1+ 0.25) = 10 KB | RST8x8: 16*64*8 /(8*1024)= 1 KB per kernel RST4x4: 16*16*8/(8*1024)= 0.25 KB per Kernel Total = 2*4*(1+ 0.25) = 10 KB | RST8x8: 16*48*8 /(8*1024)= 0.75 KB per kernel RST4x4: 16*16*8/(8*1024)= 0.25 KB per Kernel Total = 2*4*(0.75+ 0.25) = 8 KB | RST8x8: 16*64*8 /(8*1024)= 1 KB per kernel RST4x4: 16*16*8/(8*1024)= 0.25 KB per Kernel Total = 4*(1+ 0.25) = 5 KB |
| Bit-precision to represent transform coefficients | 8 bits | 8 bits | 8 bits | 8 bits | 8 bits |
| Precision of arithmetic operations during transform computation | 8 bits x 16 bits mult. | 8 bits x 16 bits mult. | 8 bits x 16 bits mult. | 8 bits x 16 bits mult. | 8 bits x 16 bits mult. |
| Bit-precision for storing intermediate data representation | Same as VTM | Same as VTM | Same as VTM | Same as VTM | Same as VTM |
| Specify if a transform requires multiple iterations where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output. If yes, report the number of arithmetic operation required in each iteration, and if they can be computed in parallel. | No | No | No | No | No |
| Other operations and memory requirements | N/A | N/A | N/A | N/A | N/A |
| List of all combinations of transform types | Two RST Kernel combined with primary transform in VTM | Two RST Kernel combined with primary transform in VTM | Two RST Kernel combined with primary transform in VTM | Two RST Kernel combined with primary transform in VTM | One RST Kernel combined with primary transform in VTM |
| List of all combinations of transforms block size and transform type used for secondary transformation | For blocks with min(W,H)<8 : RST4x4 For blocks with min(W,H)>=8 : RST8x8 | For blocks with min(W,H)<8 : RST4x4 For blocks with min(W,H)>=8 : RST8x8 For simplification (option (B), 8 mult/sample): for 4x4 block: Half RST4x4 for 8x8 block: Half RST8x8 for 4*8 and 8*4: only apply on one 4x4 corner | For blocks with min(W,H)<8 : RST4x4 For blocks with min(W,H)>=8 : RST8x8 For simplification (option (B), 8 mult/sample): for 4x4 block: No RST for 8x8 block: Half RST8x8 for 4*8 and 8*4: only apply on one 4x4 corner | For blocks with min(W,H)<8 : RST4x4 For blocks with min(W,H)>=8 : RST8x8 For simplification (option (B), 8 mult/sample): for 4x4 block: Half RST4x4 for 8x8 block: Half RST8x8 for 4*8 and 8*4: only apply on one 4x4 corner | For blocks with min(W,H)<8 : RST4x4 For blocks with min(W,H)>=8 : RST8x8 |
| Provide analysis of implementation and arithmetic commonalities of proposed transforms. (e.g., stating which transforms are implemented using matrix multiplications, addition-only butterflies, or Givens rotations). | Primary transform: Same as VTM Secondary transform: Reduced matrix multiplications | Primary transform: Same as VTM Secondary transform: Reduced matrix multiplications | Primary transform: Same as VTM Secondary transform: Reduced matrix multiplications | Primary transform: Same as VTM Secondary transform: Reduced matrix multiplications | Primary transform: Same as VTM Secondary transform: Reduced matrix multiplications |
| If the proposal requires additional computations at the encoder or decoder (e.g., additional transforms, or partial transforms along block boundaries), then the additional operations should be reported together with the other complexity metrics. | N/A | N/A | N/A | N/A | N/A |

# Test descriptions

## CE6-1: Unification between MTS and Transform Skip mode (1 test)

CE6-1.1: JVET-M0501 CE6 related: Unification of Transform Skip mode and MTS [X. Zhao, X. Li, S. Liu (Tencent)]

Transform Skip (TS) mode and MTS are unified such that TS mode becomes part of MTS by replacing DCT-8 with Identity Transform (IDT). Such that transform skipping can be done in either the horizontal direction, vertical direction or both directions. The replacement of DCT-8 by IDT can be done up to a certain block size, which can be configured (e.g., 4x4, 8x8, 16x16 or 32x32).

Since the adoption of JVET-M0464 changes the encoder search and MTS index binarization, further binarization changes which include IDT as part of MTS are also tested. In the following tests, (DCT8, DCT8) pair in MTS is replaced by (IDT, IDT) for all block sizes (up to 32-length).

## CE6-2: Reduced transform candidates of MTS (10 tests)

CE6-2.1: JVET-M0304 CE6-related: 2-mode MTS with shape adaptive transform selection [J. Lainema (Nokia)]

This experiment measures performance of the 2-mode MTS and implicit transform selection approach proposed in JVET-M0304. Proposed tool removes the signaling for mts_idx and thus uses only 2 MTS modes. It also uses implicit shape-based transform selection when tu_mts_flag is zero.

Impact of different TT and BT splits for 64x64 intra picture CUs is measured as part of the test. Also results for the 2-mode MTS mode alone without the implicit transform selection for tu_mts_flag zero is measured.

The implicit transform selection tool decides horizontal and vertical transforms for Intra coded CUs based on the dimensions of the CU. If a CU is non-square, DST-7 is selected for the shorter side of the CU and DCT-2 is used for the longer one. In the case of explicit transform selection, the maximum size of DST-7 transform is limited to 16-points.

CE6-2.2: JVET-M0366 CE6 related: Transform Simplification [C. Hollman, D. Saffar, P. Wennersten, J. Ström (Ericsson)]

In JVET-M0366 a simplification of the Multiple Transform Selection tool was proposed. The proposed changes include:

- Remove the combination (DCT-8, DCT-8) from the MTS tool,

- Replace the DCT-8 for size 32 with the existing DCT-2,

- Replace the mts_tu_idx with two flags (mts_dst_flag, mts_tu_flag) indicating which transform to use, and

- Change the CABAC context selection of the mts_cu_flag to be dependent on block size (inter and intra coded blocks) and intra mode (intra coded blocks).

During the discussion in the BoG on CE6-related documents is was commented that the proposed change to the context selection of the mts_cu_flag introduces a parsing dependency, as the intra mode needs to be reconstructed before the flag can be parsed. In the CE test, this dependency is removed.

The following aspects of JVET-M0366 are to be studied in a CE:

- Remove the combination (DCT-8, DCT-8) from consideration in the MTS tool,

- Replace the DCT-8 for size 32 with the existing DCT-2,

- Change the coding of the mts_tu_flag according to Table 1 (as in M0366), and

- The mts_cu_flag is coded in the same way it will be coded in VTM-4.0.

As the adoption of M0464 at the Marrakech changes the encoder search, the proposed changes are tested with three different encoder settings to investigate whether the improvement is related to the configuration of the encoder. The changes are made to the configuration parameters MTSIntraMaxCand and MTSInterMaxCand for both anchor and test software. Three different settings will be tested:

- CE6-2.3a: MTSIntraMaxCand = 3, MTSInterMaxCand = 3

- CE6-2.3b: MTSIntraMaxCand = 4, MTSInterMaxCand = 4

- CE6-2.3c: MTSIntraMaxCand = 5, MTSInterMaxCand = 5

Table 1: Bin coding for mts_tu_flag

| Horizontal | Vertical | Intra coded block |  | Inter coded block |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Intra mode | Coded bin | Height > Width | Coded bin |
| DST-7 | DCT-X | 0-34 | 0 | yes | 0 |
| DST-7 | DCT-X | 35-66 | 1 | no | 1 |
| DCT-X | DST-7 | 0-34 | 1 | yes | 1 |
| DCT-X | DST-7 | 35-66 | 0 | no | 0 |

CE6-2.3: JVET-M0202 CE6-related: Simplification related to MTS with reduced modes [K. Choi, M. Park, M. W. Park, W. Choi (Samsung)]

In order to reduce encoding complexity of using MTS, in this CE test, 4 testing including non-normative and normative changes are tested as follows:

- Test1: Non-normative change with 3modes

  - {H:DST7, V:DST7}, {H:DST7, V:DCT8}, {H:DCT8, V:DST7}, {H: DCT8, V:DCT8}

- Test2: Non-normative change with 2modes

  - {H:DST7, V:DST7}, {H:DST7, V:DCT8}, {H:DCT8, V:DST7}, {H: DCT8, V:DCT8}

- Test3: Normative change with 3modes

  - {H:DST7, V:DST7}, {H:DST7, V:DCT8}, {H:DCT8, V:DST7}

- Test4: Normative change with 3modes

  - {H:DST7, V:DST7}, {H:DST7, V:DCT8}

For the Test 3 and 4, the truncated unary was used for those tests as shown in the Table 1 and 2.

Table 2: Syntax design for Test3

|  | Horizontal Kernel | Vertical Kernel | Code |
| --- | --- | --- | --- |
| Mode 0 | DST7 | DST7 | 0 |
| Mode 1 | DST7 | DCT8 | 10 |
| Mode 3 | DCT8 | DST7 | 11 |

Table 2. Syntax design for Test4

|  | Horizontal Kernel | Vertical Kernel | Code |
| --- | --- | --- | --- |
| Mode 0 | DST7 | DST7 | 0 |
| Mode 1 | DST7 | DCT8 | 1 |

## CE6-3: Secondary transform (5 tests)

CE6-3.1: JVET-M0292 CE6-5.1: Reduced Secondary Transform (RST) [M. Koo, M. Salehifar, J. Lim, S. Kim (LGE)]

Secondary transform is applied between forward primary transform and quantization (at encoder) and between de-quantization and inverse primary transform (at decoder side). 4x4 non-separable secondary transform (16x16 or 16x8 matrix) called 4x4 RST is applied for small blocks (i.e., min (width, height) < 8) and 8x8 reduced non-separable secondary transform (64x16 or 64x8 matrix) called 8x8 RST is applied for larger blocks (i.e., min (width, height) > 4). Based on intra prediction mode, corresponding transform set is determined according to pre-defined mapping rule. In this test totally 4 transform sets are used. Each transform set consists of 2 transform kernels, and an index is signalled to indicate which transform kernel to be used.

For test 1 (6-3.1a), as described above, 4x4 RST (where 4x4 RST kernel size is equal to 16x16 matrix) is applied for small blocks (min (width, height) < 8), and 8x8 RST (where 4x4 RST kernel size is 16x64 matrix) is applied for larger block (min (width, height) > 4).

For test 2 (6-3.1b), in order to limit worst case number of multiplications of per sample to 8, more reduced 4x4 RST and 8x8 RST is applied to 4x4 block and 8x8 block, respectively. Specifically, 4x4 RST (where 4x4 RST kernel size is equal to 16x8 matrix) is applied for 4x4 block, and 8x8 RST (where 4x4 RST kernel size is 8x64 matrix) is applied for 8x8 block.

For test 3 (6-3.1c), on top of test 2, secondary transform for 4x4 TU is skipped.

For test 4 (6-3.1d), instead of 64x16, 48x16 matrices are used for 8x8 RST to reduce memory space for storing transform kernels. Hence, memory usage for storing all RST matrices can be reduced from 10KB to 8KB.

For test 5 (6-3.1e), all conditions are the same as test 1, but only one transform kernel per set is applied. Hence, 5KB (instead of 10 KB) is required to store transform kernels.

# Crosscheck Reports

The following table reports the crosschecker’s output on each CE6 test.

| Test # | Crosscheckers | Implementation is aligned with the algorithm description? | Does crosschecking confirm whether ALL data points match with the results provided by proponent? | Additional comments? |
| --- | --- | --- | --- | --- |
| CE6-1.1a | T. Tsukuba (Sony) | Yes. DCT8xDST7/DST7xDCT8 is replaced with IDTxDCT2/DCT2xIDT depending on block size, which is described in JVET-N0360, but not in JVET-M0501. | All data points are confirmed. | Encoder optimization described in JVET-N0363 is implemented. |
| CE6-2.1a | C. Hollmann (Ericsson) | Yes, software is aligned with description of proposal. | All data points match the proponents, run times are within reasonable error range. | n.a. |
| CE6-2.1b | C. Hollmann (Ericsson) |  |  |  |
| CE6-2.1c | C. Hollmann (Ericsson) |  |  |  |
| CE6-2.2a | X. Zhao (Tencent) | Yes | Yes, all data points provided by proponent match with the number obtained in this crosscheck. | n.a. |
| CE6-2.2b |  |  |  |  |
| CE6-2.2c |  |  |  |  |
| CE6-2.2d |  |  |  |  |
| CE6-2.3a | J. Lainema (Nokia) |  |  |  |
| CE6-2.3b | J. Lainema (Nokia) |  |  |  |
| CE6-2.3c | J. Lainema (Nokia) |  |  |  |
| CE6-3.1a | P. Philippe (Orange) | Yes | For the points tested (CTC, AI & RA), the results of the cross-check match those of the proponent. The bdrate metrics are identical to those of the proponent, the encoding and decoding runtimes are consistent within the measurement error margin. |  |
| CE6-3.1b | P. Philippe (Orange) | Yes | For the points tested (CTC, AI & RA), the results of the cross-check match those of the proponent. The bdrate metrics are identical to those of the proponent, the encoding and decoding runtimes are consistent within the measurement error margin. | Identical to CE-6-3.1a with worst case number of multiplications reduced to 8. |
| CE6-3.1c | P. Philippe (Orange) | Yes | For the points tested (CTC, AI & RA), the results of the cross-check match those of the proponent. The bdrate metrics are identical to those of the proponent, the encoding and decoding runtimes are consistent within the measurement error margin. | Identical to CE-6.3.1b without 4x4 RST |
| CE6-3.1d | P. Philippe (Orange) | Yes | For the points tested (CTC, AI & RA), the results of the cross-check match those of the proponent. The bdrate metrics are identical to those of the proponent, the encoding and decoding runtimes are consistent within the measurement error margin. | Identical to CE6-3.1.b, but 8x8 RST is limited to 48x16 matrices. |
| CE6-3.1e | P. Philippe (Orange) | Yes | For the points tested, the results of the cross-check match those of the proponent. The bdrate metrics are identical to those of the proponent, the encoding and decoding runtimes are consistent within the measurement error margin. | Identical to CE6-3.1.a but only one transform kernel is applied. |

# CE related contributions

## CE6-1 related (1)

JVET-N0123 Non-CE6/CE8: Chroma Transform Skip [T. Tsukuba, M. Ikeda, T. Suzuki (Sony)]

JVET-N0534 Crosscheck of JVET-N0123 (AHG13/Non-CE6/CE8: Chroma Transform Skip) [X. Zhao (Tencent)] [miss] [late]

JVET-N0363 CE6-related: Modified encoder decision for transform skip [X. Zhao, X. Li, S. Liu (Tencent)]

JVET-N0424 CE6-related: An Explicit MTS Design with Fast Encoder [C.-H. Hung, H. E. Egilmez, N. Hu, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-N0519 Crosscheck of JVET-N0424 (CE6-related: An Explicit MTS Design with Fast Encoder) [P. Philippe (Orange bcom)] [miss] [late]

## CE6-2 related (2)

JVET-N0089 CE6-related: Unified implicit MTS between ISP mode and regular intra mode [M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-N0565 Crosscheck of JVET-N0089 (CE6-related: Unified implicit MTS between ISP mode and regular intra mode) [Y. Sun, F. Chen, L. Wang (Hikvision)] [miss] [late]

JVET-N0121 Non-CE6: MTS simplification [T. Tsukuba, Y. Yagasaki, T. Suzuki (Sony)]

JVET-N0122 CE6/CE8-related: Alternative implementation of residual rearrangement for transform skipped blocks [T. Tsukuba, M. Ikeda, T. Suzuki (Sony)] [placeholder] [late]

JVET-N0160 CE6-related: Simplification on transform selection for Intra Sub-Partitions [X. Cao, L. Xu, F. Chen, L. Wang (Hikvision)]

JVET-N0535 Crosscheck of JVET-N0160 (Non-CE6: Simplification on transform selection for Intra Sub-Partitions) [X. Zhao (Tencent)] [miss] [late]

JVET-N0161 CE6-related: Implicit transform selection based on intra mode [X. Cao, F. Chen, L. Wang (Hikvision)]

JVET-N0172 Non-CE6: Unification of Implicit Transform Core Selection [H. Gao, S. Esenlik, B. Wang, A. M. Kotra, J. Chen (Huawei)]

JVET-N0190 Non-CE6: Modifications on sub-block transform [J. Jung, D. Kim, G. Ko, J. Son, J. Kwak (WILUS)]

JVET-N0208 CE6-6.2: Simplification related to MTS with reduced modes [K. Choi, M. Park, M. W. Park, W. Choi (Samsung)]

JVET-N0338 Non-CE6: Unified transform selection for implicit MTS [T.-C. Ma, X. Xiu, Y.-W. Chen, X. Wang (Kwai Inc.)]

JVET-N0375 Non-CE6: Simplification on implicit transform selection in ISP mode [S.-C. Lim, J. Kang, J. Lee, H. Lee, H. Y. Kim (ETRI)]

JVET-N0388 Non-CE6: Shape Adaptive Transform Selection for ISP, SBT and MTS [K. Naser, T. Poirier, F. Le Léannec (Technicolor)]

JVET-N0419 Non-CE6: A Simplification of Implicit MTS [H. E. Egilmez, A. Said, N. Hu, C.-H. Hung, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-N0420 Non-CE6: A Simplification of Implicit Transform Selection in Intra-subblock Partitioning [H. E. Egilmez, A. K. Ramasubramonian, A. Said, N. Hu, V. Seregin, G. Van der Auwera, M. Karczewicz (Qualcomm)]

JVET-N0491 CE6-related: Transform Candidate Ordering [C. Hollmann, D. Saffar, J. Ström, P. Wennersten (Ericsson)]

## CE6-3 related (3)

JVET-N0105 CE6-related: RST binarisation [C. Rosewarne, J. Gan (Canon)]

JVET-N0365 CE6-related: MPM based non-separable secondary transform [X. Zhao, X. Li, S. Liu (Tencent)]

JVET-N0518 Crosscheck of JVET-N0365 (CE6-related: MPM based non-separable secondary transform) [P. Philippe (Orange bcom)] [miss] [late]

JVET-N0509 CE6-Related: NSST with 8 Coefficients Computation [K. Naser, G. Rath, P. de Lagrange (Technicolor)] [late]

JVET-N0555 CE6-related: Simplification of the Reduced Secondary Transform [M. Siekmann, M. Winken, H. Schwarz, D. Marpe, T. Wiegand (HHI)] [late]

## Others

JVET-N0194 CE6-related: Context selection of last non-zero coefficient position coding based on reduced TU size (related to JVET-M0297 and JVET-M0251/M0257) [M. Koo, J. Choi, M. Salehifar, J. Lim, S. Kim (LGE)]

JVET-N0362 Non-CE6: Configurable max transform size in VVC [X. Zhao, X. Li, S. Liu (Tencent)]

JVET-N0364 Non-CE6: Further cost reduction for primary transform [X. Zhao, X. Li, S. Liu (Tencent)]

JVET-N0445 Non-CE6: MTS kernel derivation for efficient memory usage [S. Shrestha, A. Kumar, B. Lee (Chosun Univ.), Y. Lee, J. Park (Humax)]

JVET-N0485 Non-CE6: Efficient separable Multiple-Transform-Selection (MTS) without zero-out [A. Said, H.E. Egilmez, Y.-H. Chao, V. Seregin, M. Karczewicz (Qualcomm)]

# References

- VVC Test Model VTM-4.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-M1010, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019.