| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0027-v1 |
| --- | --- |

| Title: | CE7: Summary report on quantization and coefficient coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Heiko Schwarz, Muhammed Coban | Email: | heiko.schwarz@hhi.fraunhofer.de mcoban@qti.qualcomm.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Abstract

The CE report summarizes the test results and crosschecks reports for CE7 on quantization and coefficient coding.  The CE includes 4 tests on transform coefficient coding.

# Introduction

The purpose of this core experiment is to explore the coding efficiency and complexity impact of proposed algorithms for quantization and transform coefficient coding.

# Participants List

| Affiliation | Person, Email address |
| --- | --- |
| Qualcomm | Muhammed Coban <mcoban@qti.qualcomm.com> Dmytro Rusanovskyy <dmytror@qti.qualcomm.com> Jie Dong <jiedong@qti.qualcomm.com> Ted Hsieh <thsieh@qti.qualcomm.com> Marta Karczewicz <martak@qti.qualcomm.com> |
| Fraunhofer HHI | Heiko Schwarz <heiko.schwarz@hhi.fraunhofer.de> Tung Nguyen <tung.nguyen@hhi.fraunhofer.de> Christian Helmrich <christian.helmrich@hhi.fraunhofer.de> |
| Samsung | Yinji Piao <yj1003.piao@samsung.com> Kiho Choi <kiho14.choi@samsung.com> |
| Huawei | Alexey Filippov <alexey.filippov@huawei.com> Jianle Chen <Jianle.chen@huawei.com> Cheung Auyeung <cheung.auyeung@huawei.com> Yang Haitao <haitao.yang@huawei.com> Yin Zhao <yin.zhao@huawei.com> Sergey Ikonin <Sergey.Ikonin@huawei.com> Biao Wang <biao.wang@huawei.com> Vasily Rufitskiy <vasily.rufitskiy@huawei.com> Alexander Karabutov <karabutov.alexander@huawei.com> Sergey Ikonin <Sergey.Ikonin@huawei.com> |
| Orange | Pierrick Philippe <pierrick.philippe@orange.com> Felix Henry <felix.henry@orange.com> Gordon Clare <gordon.clare@orange.com> |
| MediaTek | Mohammed Sarwer <Mohammed.Sarwer@mediatek.com> Olena Chubach <Olena.Chubach@mediatek.com> Yu-Wen Huang <yuwen.huang@mediatek.com> Chih-Wei Hsu <cw.hsu@mediatek.com> Ching-Yeh Chen <chingyeh.chen@mediatek.com> T.-D. Chuang <peter.chuang@mediatek.com> Shih-Ta Hsiang <shih-ta.hsiang@mediatek.com> |
| Sharp | Eiichi Sasaki <eiichi.sasaki@sharp.co.jp> Takeshi Chujoh <chujoh.takeshi@sharp.co.jp> Tomohiro Ikai <ikai.tomohiro@sharp.co.jp> Kiran Misra <misrak@sharplabs.com> Andrew Segall <asegall@sharplabs.com> Frank Bossen <frank.bossen@me.com> |
| Apple | Alexis Tourapis <atourapis@apple.com> Jae Hoon Kim <i.jaehoon.kim@icloud.com> |
| KDDI | Kei Kawamura <ki-kawamura@kddi.com> Kyohei Unno <ky-unno@kddi-research.jp> Yoshitaka Kidani <yo-kidani@kddi.com> |
| Kwai | Yi-Wen Chen <yiwenchen@kwai.com> Xianglin Wang <xianglinwang@kwai.com> |
| Bytedance | Li Zhang <lizhang.idm@bytedance.com> Hongbin Liu <liuhongbin.01@bytedance.com> |
| Nokia | Jani Lainema <jani.lainema@nokia.com> |
| Broadcom | Minhua Zhou <minhua.zhou@broadcom.com> Wade Wan <wade.wan@broadcom.com> Peisong Chen <peisong.chen@broadcom.com> |
| Ericsson | Julien Le Tanou <julien.le.tanou@ericsson.com> |
| Sony | Takeshi Tsukuba <Takeshi.Tsukuba@sony.com> Karl Sharman <karl.sharman@eu.sony.com> Masa Ikeda <masaru.ikeda@sony.com> |
| InterDigital | Xiaoyu Xiu <xiaoyu.xiu@interdigital.com> Yuwen He <Yuwen.He@InterDigital.com> Rahul Vanam <Rahul.vanam@interdigital.com> |
| Canon | Chris Rosewarne <chris.rosewarne@cisra.canon.com.au> Andrew Dorrell <andrew.dorrell@cisra.canon.com.au> |
| BBC | Andre Seixas Dias <Andre.SeixasDias@bbc.co.uk> Saverio Blasi <saverio.blasi@bbc.co.uk> |
| Technicolor | Fabien Racape <fabien.racape@technicolor.com> Ya Chen <Ya.chen@technicolor.com> Edouard Francois <Edouard.francois@technicolor.com> Philippe de Lagrange <Philippe.delagrange@technicolor.com> Leleannec Fabrice <Fabrice.Leleannec@technicolor.com> |
| HiSilicon | Jianhua Zheng <zhengjianhua@hisilicon.com> Quanhe Yu <yuquanhe@hisilicon.com> |
| ETRI | Jungwon Kang <jungwon@etri.re.kr> Sung-Chang Lim <sclim@etri.re.kr> |
| Panasonic | Kiyofumi Abe <abe.kiyo@jp.panasonic.com> Han Boon Teo <hanboon.teo@sg.panasonic.com> Chong Soon Lim <chongsoon.lim@sg.panasonic.com> Tadamasa Toma <toma.tadamasa@jp.panasonic.com> |
| LG | Jung-Ah Choi <jungah.choi@lge.com> Jin Heo <jin78.heo@lge.com> Sunmi Yoo <sunmi.yoo@lge.com> Jangwon Choi <jangwon84.choi@lge.com> Ling Li <l.li@lge.com> Seung Hwan Kim <seunghwan3.kim@lge.com> Mehdi Salehifar <mehdi.salehifar@lge.com> Moonmo Koo <moonmo.koo@lge.com> Jaehyun Lim <jaehyun.lim@lge.com> |
| Dolby | Sean McCarthy <smcca@dolby.com> Peng Yin <pyin@dolby.com> |
| Comcast | Dan Grois <grois@ieee.org> |
| Intel | Sabine Roessel <sabine.roessel@intel.com> |
| Tsinghua University | Benben Niu <nbb16@mails.tsinghua.edu.cn> |
| Tencent | Min Gao <maxwellgao@tencent.com> Xiang Li <xlxiangli@tencent.com> |
| WILUS | Dongcheol Kim <dongcheol.kim@wilusgroup.com> |
| Foxconn | Yao-Jen Chang <yaojen.chang.foxconn@gmail.com> |
| Alibaba | Yu-Chen Sun <yuchen.s@alibaba-inc.com> |

# Summary of Tests in CE

|  | Tester | Tool | Cross checker |
| --- | --- | --- | --- |
| CE7-1 | Jani Lainema (Nokia) | Joint coding of Cb and Cr residual | H. Schwarz (HHI) |
| CE7-2 | Xin Zhao (Tencent) | Reduced number of context-coded bins | Y. Piao (Samsung) |
| CE7-3 | Yinji Piao (Samsung) | Template-based unified Rice parameter derivation | Y.-H. Chao (Qualcomm) |
| CE7-4 | Yung-Hsuan Chao (Qualcomm) | Template based Rice parameter derivation for coding of abs_remainder | Y. Piao (Samsung) |

# List of relevant contributions

| document | author(s) | title |
| --- | --- | --- |
| JVET-N0054 | J. Lainema (Nokia) | CE7: Joint coding of chrominance residuals (CE7-1) |
| JVET-N0188 | Y. Piao, K. Choi (Samsung) | CE7-7.3: Unified rice parameter derivation for coefficient level coding |
| JVET-N0345 | Y.-H. Chao, M. Coban, M. Karczewicz (Qualcomm) | CE7: Template based Rice parameter derivation for coding of abs_remainder (CE7-4) |
| JVET-N0361 | X. Zhao, X. Li, S. Liu (Tencent) | CE7-2: Reduced number of context-coded bins |
| CE related documents |  |  |
| JVET-N0049 | M. Zhou (Broadcom) | AHG16/Non-CE7: A study of bin to bit ratio in VTM4.0 and HM16.19 |
| JVET-N0090 | O. Chubach, C.-Y. Lai, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek) | CE7-related: Support of signalling default and user-defined scaling matrices |
| JVET-N0091 | T.-D. Chuang, S.-T. Hsiang, Z.-Y. Lin, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek) | CE7-related: TB-level constraints on context-coded bins for coefficient coding |
| JVET-N0092 | M. G. Sarwer, O. Chubach, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek) | CE7-related: Unified Rice parameter derivation of abs_remainder and dec_abs_level syntax elements |
| JVET-N0103 | C. Rosewarne, J. Gan (Canon) | CE7-related: Coefficient group size harmonisation |
| JVET-N0106 | M. Coban, M. Karczewicz (Qualcomm) | CE7-related: TU level limits on context-coded bins for coefficient coding |
| JVET-N0189 | Y. Piao, K. Choi (Samsung) | Non-CE7: Unified last position coding for 32-point transforms |
| JVET-N0194 | M. Koo, J. Choi, M. Salehifar, J. Lim, S. Kim (LGE) | CE6-related: Context selection of last non-zero coefficient position coding based on reduced TU size (related to JVET-M0297 and JVET-M0251/M0257) |
| JVET-N0281 | S.-T. Hsiang, T.-D. Chuang, S.-M. Lei (MediaTek) | CE7-related: Constraint on the number of regular bins per subblock |
| JVET-N0282 | C. Helmrich, C. Rudat, T. Nguyen, H. Schwarz, D. Marpe, T. Wiegand (HHI) | CE7-related: Joint chroma residual coding with multiple modes |
| JVET-N0311 | Y. Chen, F. Le Léannec, T. Poirier (Technicolor) | Context Modeling Simplification and Reduction in VVC |
| JVET-N0326 | Y.-W. Chen, X. Xiu, T.-C. Ma, X. Wang (Kwai Inc.) | Non-CE7: Simplification of cbf coding |
| JVET-N0347 | G. Van der Auwera, A. K. Ramasubramonian, M. Coban, M. Karczewicz (Qualcomm) | CE7-related: Joint coding of chroma residuals |
| JVET-N0533 | X. Zhao (Tencent) | Crosscheck of JVET-N0189 (Non-CE7: Unified last position coding for 32-point transformsNon-CE7: Unified last position coding for 32-point transforms) |
| JVET-N0573 | Y.-W. Chen (Kwai Inc.) | Crosscheck of JVET-N0347 (CE7-related: Joint coding of chroma residuals) |

# Test Description

## Software, reference configurations, and test conditions

All tools have to be integrated into the VTM-4 software (outcome of the Marrakech meeting).  Results have to be reported for the following two configurations:

- VTM-4 software and common test conditions (standard configuration files);

- VTM-4 software with low QP configuration configuration (common test conditions, QP values 2, 7, 12, 17, 100 frames).

All tools are tested against the corresponding anchor (same set of tools, except the tool to be tested).

## Coding Performance Measurements

Measure impact on bitrate/PSNR using provided data (4 points, BD-rate). Proponents are encouraged to provide supplementary results.

## Complexity Considerations

To provide an indicative assessment of complexity, the following numbers should be provided:

- Encoding time (compared to the reference configurations);

- Decoding time (compared to the reference configuration).

In addition, proponents are encouraged to provide the following (but not limited to) information for complexity and entropy throughput analysis:

- Number of context models

- Number of context-coded and bypass bins in average case and the worst case

- Table size of scanning paths

- Analysis of the frequency of context switches

- Analysis of the dependency between two consecutive bins

- Analysis of entropy parsing dependency (whether bits to bins conversion depends on complex reconstruction and/or pixel processing process)

- Analysis of entropy de-binarization dependency (whether bins to symbols conversion depends on complex reconstruction and/or pixel processing process)

# Summary of CE7 Results

The average test results (Bjøntegaard delta rates) are summarized in Table 1.

Table 1: Average test results for CE 7.  The table shows the results for the common test conditions (CTC) as well as for a low QP configurations (QPs 2, 7, 12, 17; 100 frames).  All results are relative to VTM-4.0.

| CE 7 |  | Common test conditions |  |  |  |  | low QPs (2, 7, 12, 17) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | encT | decT | Y | U | V | encT | decT |
| Joint residual coding for Cb and Cr |  |  |  |  |  |  |  |  |  |  |  |
| AI | CE7-1 | -0.38% | -0.49% | -1.52% | 103% | 100% | -0.28% | 0.57% | 0.27% | 108% | 99% |
| RA | CE7-1 | -0.26% | -0.81% | -0.92% | 101% | 100% | -0.19% | 0.24% | -0.10% | 102% | 100% |
| LD | CE7-1 | -0.07% | -2.99% | -4.76% | 100% | 100% | -0.18% | 0.09% | -0.58% | 103% | 100% |
| Reduction of number of context-coded bins |  |  |  |  |  |  |  |  |  |  |  |
| AI | CE7-2 | 0.03% | 0.07% | 0.04% | 99% | 100% | 0.11% | 0.16% | 0.16% | 99% | 99% |
| RA | CE7-2 | 0.01% | -0.03% | -0.10% | 100% | 99% | 0.15% | 0.22% | 0.24% | 100% | 100% |
| LD | CE7-2 | 0.02% | 0.36% | 0.00% | 100% | 99% | 0.13% | 0.22% | 0.19% | 101% | 100% |
| Unification of Rice parameter derivation |  |  |  |  |  |  |  |  |  |  |  |
| AI | CE7-3 | 0.00% | 0.01% | 0.00% | 102% | 101% | 0.00% | -0.02% | -0.02% | 102% | 102% |
|  | CE7-4 | 0.00% | 0.02% | 0.03% | 103% | 100% | -0.02% | -0.02% | -0.01% | 104% | 103% |
| RA | CE7-3 | -0.02% | 0.03% | -0.11% | 101% | 100% | 0.00% | -0.01% | -0.02% | 102% | 103% |
|  | CE7-4 | 0.00% | -0.05% | -0.10% | 101% | 100% | 0.01% | -0.01% | 0.00% | 103% | 122% |
| LD | CE7-3 | -0.01% | 0.19% | -0.20% | 100% | 100% | 0.00% | -0.01% | 0.00% | 102% | 102% |
|  | CE7-4 | -0.04% | -0.01% | 0.23% | 101% | 101% | 0.02% | 0.02% | 0.03% | 103% | 101% |

# Detailed Test Description and Results

## CE7-1: Joint coding of Cb and Cr residuals

### Test description

This test measures performance of joint Cb-Cr coding mode proposed in JVET-M0305 [1]. One bin indicator is signaled in the bitstream to enable the mode. In the case the mode is enabled a joint residual signal is coded in the bitstream. On the decoder side the joint residual is used for Cb component and a negative version of the residual is applied for Cr.

Signaling of the mode is expected to follow the syntax and semantics given below:

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| if( cIdx == 2 && tu_cbf_cb[ x0 ][ y0 ] ) { |  |
| tu_cb_cr_joint_residual[ x0 ][ y0 ] | ae(v) |
| if( tu_cb_cr_joint_residual [ x0 ][ y0 ] ) { |  |
| return |  |
| } |  |
| … |  |

tu_cb_cr_joint_residual[ x0 ][ y0 ] specifies whether indicated Cb residual is used to derive both Cb and Cr residuals. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. tu_cb_cr_joint_residual[ x0 ][ y0 ] equal to 1 specifies that the indicated Cb residual is used to derive the Cr residual. tu_cb_cr_joint_residual[ x0 ][ y0 ] equal to 0 specifies that Cr residual may be present in the bitstream depending on other syntax elements. When tu_cb_cr_joint_residual[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

### Test results

The proposed approach was integrated into the VTM-4.0 software.  The software was made available to all CE participants.  The average test results are summarized in Table 2.

Table 2: Average test results for test CE7-1.  The table shows the results for the common test conditions (CTC) as well as for a low QP configurations (QPs 2, 7, 12, 17; 100 frames).  All results are relative to VTM-4.0.

|  | all Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Common test conditions |  |  |  |  | low QPs (2, 7, 12, 17) |  |  |  |  |
|  | Y | U | V | encT | decT | Y | U | V | encT | decT |
| A1 | -0.38% | -0.04% | 0.98% | 103% | 101% | -0.12% | -4.14% | -4.74% | 102% | 100% |
| A2 | -0.79% | -1.17% | 0.77% | 105% | 101% | -0.09% | -3.87% | -5.57% | 101% | 100% |
| B | -0.25% | -0.21% | -2.17% | 102% | 101% | 0.03% | -2.87% | -5.13% | 100% | 99% |
| C | -0.28% | -1.26% | -3.15% | 103% | 99% | -0.07% | -3.73% | -5.11% | 101% | 100% |
| E | -0.28% | 0.26% | -3.08% | 102% | 99% | 0.00% | -4.00% | -4.04% | 101% | 100% |
| avg. | -0.38% | -0.49% | -1.52% | 103% | 100% | -0.10% | -3.79% | -4.26% | 101% | 100% |
| D | -0.25% | -1.32% | -2.93% | 103% | 99% | -0.12% | -4.14% | -4.74% | 102% | 100% |
| F | -0.28% | -2.12% | -3.62% | 102% | 101% | -0.09% | -3.87% | -5.57% | 101% | 100% |
|  | random access |  |  |  |  |  |  |  |  |  |
|  | Common test conditions |  |  |  |  | low QPs (2, 7, 12, 17) |  |  |  |  |
|  | Y | U | V | encT | decT | Y | U | V | encT | decT |
| A1 | -0.44% | -0.85% | 1.55% | 101% | 99% | -0.11% | 0.30% | 0.56% | 103% | 100% |
| A2 | -0.38% | -2.51% | 0.31% | 101% | 101% | -0.18% | 0.19% | 0.12% | 105% | 100% |
| B | -0.16% | -0.30% | -2.03% | 101% | 101% | -0.16% | 0.34% | 0.06% | 103% | 100% |
| C | -0.17% | -0.13% | -2.31% | 101% | 99% | -0.31% | 0.11% | -0.97% | 98% | 100% |
| avg. | -0.26% | -0.81% | -0.92% | 101% | 100% | -0.19% | 0.24% | -0.10% | 102% | 100% |
| D | -0.15% | -0.47% | -2.69% | 102% | 100% | -0.34% | -0.36% | -1.22% | 97% | 101% |
| F | -0.42% | -1.80% | -3.34% | 102% | 100% | -0.67% | 0.21% | -0.68% | 98% | 101% |
|  | low delay B |  |  |  |  |  |  |  |  |  |
|  | Common test conditions |  |  |  |  | low QPs (2, 7, 12, 17) |  |  |  |  |
|  | Y | U | V | encT | decT | Y | U | V | encT | decT |
| B | -0.12% | -4.14% | -4.74% | 102% | 100% | -0.13% | 0.12% | -0.12% | 103% | 100% |
| C | -0.09% | -3.87% | -5.57% | 101% | 100% | -0.33% | 0.06% | -1.20% | 104% | 100% |
| E | 0.03% | -2.87% | -5.13% | 100% | 99% | -0.06% | 0.07% | -0.54% | 103% | 101% |
| avg. | -0.07% | -3.73% | -5.11% | 101% | 100% | -0.18% | 0.09% | -0.58% | 103% | 100% |
| D | 0.00% | -4.00% | -4.04% | 101% | 100% | -0.46% | -0.48% | -1.16% | 103% | 100% |
| F | -0.10% | -3.79% | -4.26% | 101% | 100% | -0.52% | 0.01% | -1.06% | 103% | 100% |

### Crosscheck

The cross checker Heiko Schwarz (Fraunhofer HHI) reported:

- The source code was carefully checked. The software implementation matches the CE description in JVET-M1027 and the description in input document JVET-N0054.  A difference to the original input contribution JVET-M0305 is that a delta QP for the joint coding mode is coded as tile group header syntax element (in JVET-M0305, the QP used was derived by QP − 2).

- The reported test results could be exactly reproduced for all configurations and test sequences.

## CE7-2: Reduced number of context-coded bins

### Test description

In the coefficient coding of current VVC design, the averaged maximum number of context-coded bins per 4x4 sub-block is 32. In JVET-M0491, to further reduce the averaged maximum number of context-coded bins per 4x4 sub-block, it is proposed to reduce the maximum number of context-coded bins for 4x4 sub-blocks in high frequency region such that the averaged maximum number of context-coded bins per 4x4 sub-block is 30 for the whole CU.

### Test results

The proposed approach was integrated into the VTM-4.0 software.  The software was made available to all CE participants.  The average test results are summarized in Table 3.

Table 3: Average test results for test CE7-2.  The table shows the results for the common test conditions (CTC) as well as for a low QP configurations (QPs 2, 7, 12, 17; 100 frames).  All results are relative to VTM-4.0.

|  | all Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Common test conditions |  |  |  |  | low QPs (2, 7, 12, 17) |  |  |  |  |
|  | Y | U | V | encT | decT | Y | U | V | encT | decT |
| A1 | 0.02% | 0.03% | 0.01% | 102% | 100% | 0.18% | 0.27% | 0.25% | 99% | 102% |
| A2 | 0.02% | 0.09% | 0.00% | 100% | 99% | 0.18% | 0.21% | 0.22% | 98% | 99% |
| B | 0.02% | 0.06% | 0.05% | 99% | 99% | 0.11% | 0.13% | 0.14% | 99% | 101% |
| C | 0.06% | 0.05% | 0.13% | 99% | 101% | 0.06% | 0.09% | 0.11% | 99% | 97% |
| E | 0.01% | 0.10% | 0.00% | 97% | 99% | 0.04% | 0.13% | 0.11% | 98% | 96% |
| avg. | 0.03% | 0.07% | 0.04% | 99% | 100% | 0.11% | 0.16% | 0.16% | 99% | 99% |
| D | 0.05% | -0.06% | -0.06% | 101% | 96% | 0.02% | 0.06% | 0.08% | 97% | 98% |
| F | 0.11% | 0.15% | 0.09% | 101% | 101% | 0.10% | 0.17% | 0.16% | 100% | 98% |
|  | random access |  |  |  |  |  |  |  |  |  |
|  | Common test conditions |  |  |  |  | low QPs (2, 7, 12, 17) |  |  |  |  |
|  | Y | U | V | encT | decT | Y | U | V | encT | decT |
| A1 | 0.01% | 0.06% | -0.07% | 99% | 99% | 0.17% | 0.24% | 0.25% | 99% | 99% |
| A2 | 0.00% | -0.09% | -0.06% | 101% | 100% | 0.19% | 0.27% | 0.28% | 100% | 100% |
| B | 0.01% | 0.00% | -0.15% | 100% | 100% | 0.14% | 0.21% | 0.21% | 102% | 102% |
| C | 0.03% | -0.08% | -0.08% | 99% | 98% | 0.13% | 0.19% | 0.22% | 100% | 99% |
| avg. | 0.01% | -0.03% | -0.10% | 100% | 99% | 0.15% | 0.22% | 0.24% | 100% | 100% |
| D | 0.04% | -0.09% | -0.10% | 100% | 97% | 0.12% | 0.20% | 0.10% | 99% | 99% |
| F | 0.11% | 0.04% | -0.09% | 100% | 99% | 0.17% | 0.17% | 0.10% | 100% | 100% |
|  | low delay B |  |  |  |  |  |  |  |  |  |
|  | Common test conditions |  |  |  |  | low QPs (2, 7, 12, 17) |  |  |  |  |
|  | Y | U | V | encT | decT | Y | U | V | encT | decT |
| B | -0.02% | 0.06% | -0.07% | 101% | 100% | 0.15% | 0.22% | 0.18% | 101% | 100% |
| C | 0.01% | 0.27% | 0.22% | 99% | 97% | 0.14% | 0.29% | 0.26% | 100% | 101% |
| E | 0.09% | 0.99% | -0.16% | 101% | 102% | 0.08% | 0.12% | 0.12% | 100% | 99% |
| avg. | 0.02% | 0.36% | 0.00% | 100% | 99% | 0.13% | 0.22% | 0.19% | 101% | 100% |
| D | 0.01% | -0.16% | 0.20% | 100% | 102% | 0.11% | 0.21% | 0.20% | 102% | 101% |
| F | -0.07% | 0.54% | -0.26% | 99% | 99% | 0.19% | 0.35% | 0.37% | 99% | 99% |

### Crosscheck

The cross checker Yinji Piao (Samsung) reported:

- Results have been confirmed for partial sequences.  Low QP results haven’t been provided in time by the tester.

- Code has been crosschecked carefully for each modification. Provided source code matches the CE description. But MAX_NUM_REG_BINS_4x4SUBBLOCK in xRateDistOptQuant() has not been modified accordingly even if this RDOQ function is not called when DQ is enabled.

- Suggest code modification to xRateDistOptQuant() accordingly.

## CE7-3: Template-based unified Rice parameter derivation

### Test description

The unified rice parameter derivation in JVET-M0198 is similar to the one for completely bypass-coded level, which refers to the neighboring transform coefficients within a given template. Rice parameter is determined not only based on sum of absolute levels of neighboring five transform coefficients in local template, but the corresponding base level is also taken into consideration.

RicePara = RiceParTable[ max(min( 31, sumAbs – 5 * baseLevel), 0) ],

where RiceParTable is the table specified in VTM-4 and baseLevel can be equal to 4 or 0.

For remainder of greater than 3 (gt3Flag), the baselevel is equal to 4; for fully bypassed coded level, the baselevel is equal 0.

### Test results

The proposed approach was integrated into the VTM-4.0 software.  The software was made available to all CE participants.  The average test results are summarized in Table 4.

Table 4: Average test results for test CE7-3.  The table shows the results for the common test conditions (CTC) as well as for a low QP configurations (QPs 2, 7, 12, 17; 100 frames).  All results are relative to VTM-4.0.

|  | all Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Common test conditions |  |  |  |  | low QPs (2, 7, 12, 17) |  |  |  |  |
|  | Y | U | V | encT | decT | Y | U | V | encT | decT |
| A1 | 0.00% | -0.02% | 0.01% | 102% | 99% | 0.00% | 0.00% | 0.00% | 102% | 99% |
| A2 | -0.01% | 0.11% | -0.01% | 100% | 101% | -0.03% | -0.07% | -0.06% | 102% | 102% |
| B | 0.00% | -0.01% | 0.08% | 102% | 101% | -0.01% | -0.01% | -0.04% | 102% | 102% |
| C | 0.00% | 0.01% | -0.07% | 101% | 101% | 0.03% | -0.03% | 0.00% | 102% | 103% |
| E | -0.01% | 0.00% | -0.03% | 101% | 102% | -0.02% | 0.00% | -0.02% | 103% | 102% |
| avg. | 0.00% | 0.01% | 0.00% | 102% | 101% | 0.00% | -0.02% | -0.02% | 102% | 102% |
| D | 0.00% | -0.01% | -0.08% | 102% | 101% | 0.06% | -0.01% | 0.05% | 102% | 101% |
| F | 0.05% | 0.11% | 0.03% | 102% | 102% | -0.05% | -0.03% | -0.03% | 102% | 100% |
|  | random access |  |  |  |  |  |  |  |  |  |
|  | Common test conditions |  |  |  |  | low QPs (2, 7, 12, 17) |  |  |  |  |
|  | Y | U | V | encT | decT | Y | U | V | encT | decT |
| A1 | -0.01% | -0.09% | 0.01% | 100% | 100% | 0.00% | -0.01% | -0.01% | 100% | 102% |
| A2 | -0.05% | 0.12% | 0.02% | 100% | 101% | -0.01% | -0.02% | -0.02% | 101% | 103% |
| B | -0.02% | 0.16% | -0.18% | 101% | 99% | 0.00% | 0.00% | 0.00% | 102% | 104% |
| C | -0.01% | -0.11% | -0.20% | 103% | 101% | 0.01% | -0.02% | -0.07% | 102% | 103% |
| avg. | -0.02% | 0.03% | -0.11% | 101% | 100% | 0.00% | -0.01% | -0.02% | 102% | 103% |
| D | -0.03% | -0.17% | -0.36% | 104% | 102% | 0.01% | -0.03% | -0.09% | 103% | 103% |
| F | 0.01% | -0.13% | -0.17% | 102% | 101% | -0.11% | -0.08% | -0.04% | 102% | 100% |
|  | low delay B |  |  |  |  |  |  |  |  |  |
|  | Common test conditions |  |  |  |  | low QPs (2, 7, 12, 17) |  |  |  |  |
|  | Y | U | V | encT | decT | Y | U | V | encT | decT |
| B | -0.01% | -0.26% | -0.48% | 100% | 101% | 0.00% | 0.01% | 0.00% | 101% | 102% |
| C | 0.01% | 0.32% | -0.10% | 99% | 97% | -0.01% | 0.00% | 0.03% | 102% | 103% |
| E | -0.05% | 0.77% | 0.12% | 100% | 101% | 0.01% | -0.05% | -0.03% | 102% | 102% |
| avg. | -0.01% | 0.19% | -0.20% | 100% | 100% | 0.00% | -0.01% | 0.00% | 102% | 102% |
| D | -0.02% | 0.12% | 0.10% | 100% | 98% | -0.01% | -0.01% | -0.05% | 101% | 103% |
| F | -0.01% | 0.46% | 0.04% | 87% | 97% | -0.08% | 0.08% | 0.02% | 102% | 103% |

### Crosscheck

The cross checker Yung-Hsuan Chao (Qualcomm) reported:

- All results match the ones provided by the proponent (the enc/dec time in the xcheck results may be unreliable).

- The source code were carefully checked and it match what described in the CE description.

## CE7-4: Template-based Rice parameter derivation for abs_remainder for unification of Rice parameter derivation

### Test description

In JVET-M0558, local neighbour template based Rice parameter derivation scheme used for coding of dec_abs_level (bypass coded coefficients) is extended to the coding of abs_remainder, replacing the VTM-4.0’s HEVC like scheme. The local 5 neighboring coefficients’ sum of absolute values is used to derive the Rice parameter for coding of abs_remainder (representing half of remainder values for context coded coefficients) by

sumAbs=k=04xk

ricePar   = max(0, riceParTable[ min( 31, sumAbs ) ] -  1)

where the riceParTable[] is the existing table that is used to derive the Rice parameter in VTM-4.0 dec_abs_level coding scheme, and xk are the decoded levels of neighboring coefficients.

### Test results

The proposed approach was integrated into the VTM-4.0 software.  The software was made available to all CE participants.  The average test results are summarized in Table 5.

Table 5: Average test results for test CE7-4.  The table shows the results for the common test conditions (CTC) as well as for a low QP configurations (QPs 2, 7, 12, 17; 100 frames).  All results are relative to VTM-4.0.

|  | all Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Common test conditions |  |  |  |  | low QPs (2, 7, 12, 17) |  |  |  |  |
|  | Y | U | V | encT | decT | Y | U | V | encT | decT |
| A1 | -0.01% | -0.01% | 0.05% | 103% | 103% | 0.03% | 0.05% | 0.04% | 104% | 103% |
| A2 | 0.01% | 0.09% | 0.00% | 103% | 104% | -0.01% | -0.02% | -0.03% | 104% | 100% |
| B | 0.00% | 0.00% | 0.04% | 103% | 99% | -0.02% | -0.01% | -0.02% | 103% | 104% |
| C | 0.02% | 0.03% | 0.06% | 103% | 99% | -0.05% | -0.09% | -0.05% | 104% | 103% |
| E | -0.01% | 0.00% | -0.03% | 102% | 96% | -0.07% | -0.01% | 0.00% | 104% | 103% |
| avg. | 0.00% | 0.02% | 0.03% | 103% | 100% | -0.02% | -0.02% | -0.01% | 104% | 103% |
| D | 0.02% | -0.05% | 0.06% | 103% | 103% | -0.03% | -0.04% | -0.01% | 103% | 101% |
| F | 0.02% | 0.02% | -0.03% | 102% | 100% | -0.06% | -0.06% | -0.07% | 103% | 102% |
|  | random access |  |  |  |  |  |  |  |  |  |
|  | Common test conditions |  |  |  |  | low QPs (2, 7, 12, 17) |  |  |  |  |
|  | Y | U | V | encT | decT | Y | U | V | encT | decT |
| A1 | -0.02% | -0.04% | -0.04% | 100% | 101% | 0.01% | 0.02% | 0.02% | 103% | 119% |
| A2 | -0.01% | 0.04% | -0.01% | 100% | 98% | 0.01% | 0.02% | 0.02% | 103% | 206% |
| B | 0.01% | -0.03% | -0.09% | 100% | 99% | 0.02% | 0.03% | 0.04% | 102% | 101% |
| C | 0.02% | -0.15% | -0.23% | 101% | 101% | 0.00% | -0.10% | -0.10% | 103% | 105% |
| avg. | 0.00% | -0.05% | -0.10% | 101% | 100% | 0.01% | -0.01% | 0.00% | 103% | 122% |
| D | 0.01% | -0.42% | -0.36% | 102% | 101% | 0.00% | -0.02% | -0.07% | 102% | 100% |
| F | -0.05% | -0.18% | -0.11% | 101% | 100% | -0.09% | -0.07% | -0.03% | 101% | 100% |
|  | low delay B |  |  |  |  |  |  |  |  |  |
|  | Common test conditions |  |  |  |  | low QPs (2, 7, 12, 17) |  |  |  |  |
|  | Y | U | V | encT | decT | Y | U | V | encT | decT |
| B | 0.01% | -0.28% | -0.09% | 101% | 99% | 0.02% | 0.07% | 0.06% | 103% | 103% |
| C | -0.02% | 0.16% | 0.20% | 102% | 103% | 0.02% | 0.04% | 0.06% | 103% | 101% |
| E | -0.14% | 0.22% | 0.79% | 101% | 102% | 0.00% | -0.10% | -0.04% | 103% | 100% |
| avg. | -0.04% | -0.01% | 0.23% | 101% | 101% | 0.02% | 0.02% | 0.03% | 103% | 101% |
| D | -0.04% | -0.07% | -0.08% | 101% | 97% | 0.00% | -0.01% | 0.08% | 102% | 103% |
| F | -0.10% | 0.41% | 0.40% | 101% | 101% | -0.04% | -0.06% | -0.02% | 103% | 102% |

### Crosscheck

The cross checker Yinji Piao (Samsung) reported:

- Results have been confirmed for all sequences and test scenarios. The running time may not be reliable.

- The source code has been crosschecked carefully for each modification. Provided source code matches the CE description. But in RDOQ, QuantRDOQ.cpp(917), it seems goRiceParam = goRiceParam  > 0 ? goRiceParam  - 1 : 0; is missing.  However when DQ is on, this RDOQ function is never called, there will be no performance change with this small bug.