| 11Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0028-v1 |
| --- | --- |

| Title: | CE8: Summary Report on Screen Content Coding Tools |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s)or Contact(s): | Xiaozhong Xu Yung-Hsuan Chao Yu-Chen Sun Jizheng Xu | Tel: Email: | xiaozhongxu@tencent.com yunghsua@qti.qualcomm.com yuchen.s@alibaba-inc.com xujizheng@bytedance.com |
| Source: | CE8 coordinators |  |  |

# Abstract

This contribution provides a summary report of Core Experiment 8 on screen content coding tools. 24 tests have been tested in CE8 in between JVET-M and JVET-N meetings, to study and evaluate technologies related to screen content coding. In this report, coding performance and complexity of these tests are reported and analyzed. In particular, test results against 1) VTM-4.0+IBC=1; 2) VTM-4.0+IBC=0; 3) VTM-4.0+IBC=1+low QPs; 4) VTM-4.0+IBC=0+low QPs anchors are provided to show the coding efficiency and complexity trade-off of each tool. Crosschecking results for the performed tests are integrated in this contribution.

# Introduction

As is agreed in Macao meeting, the following tests in table 1 are performed in CE8.

Table 1: Summary of tests performed in CE8

| Test | Tester | Document | Tool description | Cross checker |
| --- | --- | --- | --- | --- |
| CE8-1.1a | J. Nam (LGE) | JVET-N0457 | Block vector prediction for merge mode and AMVP mode using default positions | X. Xu (Tencent) |
| CE8-1.1b | J. Nam (LGE) | JVET-N0457 | Block vector prediction for merge mode and AMVP mode using default positions with pruning invalid candidates | X. Xu (Tencent) |
| CE8-1.2b | J. Nam (LGE) | JVET-N458 | Change binarization process for abs_mvd_minus2 syntax | K. Choi (Samsung) |
| CE8-1.2c | J. Nam (LGE) | JVET-N458 | Add context models for abs_mvd_greater0_flag[] and/or abs_mvd_greater1_flag[] syntaxes | K. Choi (Samsung) |
| CE8-1.2d | J. Nam (LGE) | JVET-N458 | Combination of 8.1.2b + 8.1.2c | K. Choi (Samsung) |
| CE8-1.3a | Y. Li (Wuhan U.) | JVET-N0255 | IBC with integer MMVD offsets | G. Venugopal (HHI) |
| CE8-1.3b | Y. Li (Wuhan U.) | JVET-N0255 | IBC with integer MMVD offsets and negative directions | G. Venugopal (HHI) |
| CE8-2.1 | Y.-H. Chao (Qualcomm) Y.-C. Sun (Alibaba) | JVET-N344 | Palette mode in HEVC SCC | R. Chernyak (Huawei) |
| CE8-2.2a | Y.-C. Sun (Alibaba) Y.-H. Chao (Qualcomm) R. Chernyak (Huawei) | JVET-N0404 | Separate palette coding for luma and chroma | B. Bross (HHI) |
| CE8-2.2c | Y.-C. Sun (Alibaba) | JVET-N0404 | Palette mode and simplified intra mode combination | C.-Y. Lai (MediaTek) |
| CE8-2.2e | Y.-C. Sun (Alibaba) Y.-H. Chao (Qualcomm) R. Chernyak (Huawei) | JVET-N0404 | Combine 8.2.2a and 8.2.2c | S. Yoo (LGE) |
| CE8-3.1a | F. Henry (Orange) T. Nguyen (HHI) X. Li (Tencent) | JVET-N0280 | Block-DPCM with H/V predictor from JVET-M0058 with residual coding from JVET-M0464 and context coded bins limitation from JVET-M0449 (max 2 context coded bins/sample) | P. Onno (Canon) |
| CE8-3.1b | F. Henry (Orange) T. Nguyen (HHI) X. Li (Tencent) | JVET-N0280 | Block-DPCM with H/V predictor from JVET-M0058 with residual coding from JVET-M0464 and context coded bins limitation from JVET-M0449 (max 3 context coded bins/sample) | P. Onno (Canon) |
| CE8-4.1a | S. Yoo (LGE) | JVET-N0428 | Rearrangement of the residual block according to intra mode for transform skip by flipping | B. Bross (HHI) |
| CE8-4.1b | S. Yoo (LGE) | JVET-N0428 | Rearrangement of the residual block of transform skip by rotating | X. Zhao (Tencent) K. Choi (Samsung) |
| CE8-4.1c | S. Yoo (LGE) | JVET-N0428 | Rearrangement of the residual block adaptively between rotation and flip for transform skip | M. Ikeda (Sony) |
| CE8-4.2a | S. Yoo (LGE) | JVET-N0429 | Residual coding for transform skip including context coded sign information and bitstream restriction for context coded bins limitation with max 2 context coded bins per sample | F. Henry (Orange) |
| CE8-4.2b | S. Yoo (LGE) | JVET-N0429 | Residual coding for transform skip including context coded sign information and bitstream restriction for context coded bins limitation with max 3 context coded bins per sample | Y.-C. Sun (Alibaba) |
| CE8-4.3a | T. Nguyen (HHI) | JVET-N0280 | Residual coding for transform skip and bitstream restriction for context coded bins limitation with max 2 context coded bins per sample | X. Xu (Tencent) |
| CE8-4.3b | T. Nguyen (HHI) | JVET-N0280 | Residual coding for transform skip and bitstream restriction for context coded bins limitation with max 3 context coded bins per sample | X. Xu (Tencent) |
| CE8-4.4a | T. Nguyen (HHI) X. Li (Tencent) | JVET-N0280 | Residual coding for transform skip and context coded bins limitation from JVET-M0449 with max 2 context coded bins per sample | S. Yoo (LGE) |
| CE8-4.4b | T. Nguyen (HHI) X. Li (Tencent) | JVET-N0280 | Residual coding for transform skip and context coded bins limitation from JVET-M0449 with max 3 context coded bins per sample | Y.-C. Sun (Alibaba) |
| CE8-5.1a | F. Henry (Orange) B. Bross (HHI) X. Li (Tencent) | JVET-N0214 | Combination of CE8-3.1a and CE8-4.4a (both use max 2 context coded bins per sample) | A. Filippov (Huawei) |
| CE8-5.1b | F. Henry (Orange) B. Bross (HHI) X. Li (Tencent) | JVET-N0214 | Combination of CE8-3.1b and CE8-4.4b (both use max 3 context coded bins per sample) | S. Yoo (LGE) |

# Description of CE tools

The detailed description of each tools is provided in this section.

## CE8-1.1 (JVET-N0457)

In this test, when current coding block is coded by IBC, default candidates are added for both MVP and merge lists instead of zero candidates which are always invalid in those for IBC mode. The positions of (-(cbWidth << 1), 0) and (0, -( cbHeight << 1)) could be used as default positions. The variables cbWidth and cbHeight specify the width and height of the current coding block, respectively. The proposed default candidates could be added until maximum number of candidates. In addition, before insertion of proposed default candidates, invalid candidates of existing candidates within MVP and merge candidate lists could be removed from the lists.

## CE8-1.2 (JVET-N0458)

In this test, coding method of block vector difference (BVD) for IBC will be evaluated. In VVC, motion vector difference (MVD) coding syntax which consists of four parts; abs_mvd_greater0_flag[], abs_mvd_greater1_flag[], abs_mvd_minus2[], mvd_sign_flag[]. In this test, those syntaxes will be designed for IBC because BVD has different properties compared with normal MVD. Some syntaxes (e.g. abs_mvd_greater0_flag or abs_mvd_greater1_flag) could be modified or removed. In addition, binarization process of a syntax (e.g. abs_mvd_minus2) could be changed. Additional context models for some syntaxes (e.g. abs_mvd_greater0_flag, abs_mvd_greater1_flag) could be introduced.

## CE8-1.3 (JVET-N0255)

In this test, the combination of IBC mode and MMVD method is tested. MMVD table for inter MC is reused. Only integer offsets will be applied based on the base BV predictor.

- In subtest a, when IBC is on, the integer MMVD table will be used, as indicator at tile group header always signaled as using integer MMVD offsets.

- In subtest b, in additional to subtest 1, when IBC is on, the directions to apply offset tables is assumed to be along negative directions (in x and y).

## CE8-2.1 (JVET-N0344)

Palette coding as in HEVC SCC.

For dual tree enabled in SPS, palette coding is applied separately for luma tree and chorma tree in intra slice and jointly for luma and chroma in inter slice. For dual tree disabled in SPS, palette coding is applied jointly for both intra and inter slice.

## CE8-2.2 (JVET-N0404)

Palette Improvements:

(1) separate palette coding for luma and chroma

In test CE8.2.1, when dual tree is enabled in the configuration (SPS), palette coding is applied separately on luma tree and chroma tree in intra slices and jointly on luma/chroma in inter slices. When dual tree is disabled in the configuration, palette mode is applied jointly for both luma and chroma in all slice types.

In this test, separated palette coding for luma/chroma components are investigated based on the same palette coding functions in anchor software:  palette coding is applied separately on luma and chroma components in both intra and inter slices

(2) palette mode and intra mode combination

In this test, the method combining palette mode and intra prediction is tested. The decoder first derives the prediction block based on the intra prediction information. Then, the decoder decodes a palette and an index map. Using the decoding palette information, the decoder refines the prediction block and reconstructs the block. During the presentation of this CE test during the Marrakech meeting, it was commented that the whole intra prediction process is relatively complicated. To address the comments, in this test, simplified intra prediction will be tested. In the simplified intra prediction and palette combination, only horizontal and vertical intra prediction modes are used. The new combined mode is disabled for CUs with CU size less than 256 in order to reduce implementation complexity. To signal the combined mode, a flag, intra_palette mode is first encoded. If the flag is turned on, a pred_dir flag is then signaled indicating whether horizontal or vertical intra prediction in used. On the decoder side, the decoder first decodes a palette and an index map. A specific index (0) is reserved to indicate the sample is reconstructed from intra prediction. When decoder reconstruct the palette coded block, samples with the index, 0, are reconstructed by the horizontal or vertical nearest samples across CU boundary, and samples with other indices are reconstructed by the colors in the palette. Figure 1 shows an example.

(a) vertical intra prediction combination 	 (b) horizontal intra prediction combination

Figure 1. An example of palette mode and horizontal/vertical intra prediction combination.

## CE8-3.1 (JVET-N0214)

In this section, we describe BDPCM as proposed in JVET-M0058 [1]. Note that JVET-M0058 contains a Working Draft text for BDPCM.

A bdpcm_flag is transmitted at the CU level whenever it is a luma intra CU having each dimension smaller or equal to 32. This flag indicates whether regular intra coding or DPCM is used. This flag is encoded using a single CABAC context. For predicting samples inside the block, BDPCM uses simple vertical or horizontal predictor (chosen and signaled by a flag at block level) in a way similar to RDPCM from HEVC.

The predictor uses unfiltered reference pixels when predicting the top row and left column of the CU. The predictor then uses reconstructed pixels for the rest of the CU.

The prediction error is quantized in the spatial domain, after rescaling, in a way identical to the Transform Skip quantizer. Each pixel is reconstructed by adding the dequantized prediction error to the prediction. Thus, the reconstructed pixels of a row/column are used to predict the next row/column of pixels. Amplitude and signs of the quantized prediction error are encoded separately.  A cbf_bdpcm_flag is coded. If it is equal to 0, it indicates that all amplitudes of the block are to be decoded as zero. If the flag is equal to 1, all amplitudes of the block are encoded individually in raster-scan order. In order to keep complexity low, the amplitude is limited to at most 31 (inclusive). The amplitude is encoded using unary binarization, with three contexts for the first bin, then one context for each additional bin until the 12th bin, and one context for all remaining bins. A sign is encoded in bypass mode for each non-zero residue.

In order to maintain the coherence of the regular intra mode prediction, the first mode in the MPM list is associated with a Block-DPCM CU (without being transmitted) and is available for MPM generation for subsequent blocks.

The deblocking filter is de-activated on a border between two Block-DPCM blocks, since neither of the blocks uses the transform stage usually responsible for blocking artifacts.

Due to the shape of the horizontal (resp. vertical) predictors, which use the left (A)  (resp. top (B)) pixel for prediction of the current pixel, the most throughput-efficient way of processing the block is to process all the pixels of one column (resp. line)  in parallel, and to process these columns (resp. lines) sequentially. In order to increase throughput, a block of width 4 is divided into two halves with a horizontal frontier when the predictor chosen on this block is vertical, and a block of height 4 is divided into two halves with a vertical frontier when the predictor chosen on this block is horizontal.

When a block is divided, samples from one area are not allowed to use pixels from another area to compute the prediction: if this situation occurs, the prediction pixel is replaced by the reference pixel in the prediction direction.

Thanks to this property, it becomes now possible to process a 4x4 block in 2 cycles, and a 4x8 or 8x4 block in 4 cycles, and so on. Therefore, the worst case throughput of BDPCM reconstruction is 8 pixels per cycle.

In order to maintain the coherence of the regular intra mode prediction, the first mode in the MPM list is associated with a Block-DPCM CU (without being transmitted) and is available for MPM generation for subsequent blocks.

The deblocking filter is de-activated on a border between two Block-DPCM blocks, since neither of the blocks uses the transform stage usually responsible for blocking artifacts.

Block-DPCM does not use any other step than the ones described here. In particular it does not use any transform.

In JVET-M0058, Amplitude and signs of the quantized prediction error are encoded separately.  A cbf_bdpcm_flag is coded. If it is equal to 0, it indicates that all amplitudes of the block are to be decoded as zero. If the flag is equal to 1, all amplitudes of the block are encoded individually in raster-scan order. In order to keep complexity low, the amplitude is limited to at most 28 (inclusive). The amplitude is encoded using truncated unary binarization (abs_level_gt1_flag to abs_level_gt12_flag), with three contexts for the first bit, then one context for each additional bin until the 12th bin, and all remaining bins are encoded with equiprobable bits binarized using second order Exp-Golomb codes.

It was commented during the presentation of this CE test (8.3.2) during the Marrakech meeting that it would be desirable to further harmonize the residual coding of BDPCM with the regular residual coding of VVC and that deviations from this would need to be justified by coding gains. It was also noted that the number of context coded bins should be reduced. Consequently, the following tests are proposed:

- BDPCM as in JVET-M0058, with the BDPCM residue encoded using the residual coding as in JVET-0464

For all experiments the number of context coded bins in a block will be limited to kxWxH where W is the width and H the size of the block (as described in JVET-M0449). k will take the value of 2 and 3. The combination with transform skip as in JVET-M0464 will also be studied.

## CE8-4.1 (JVET-N0428)

In this test, the residual blocks which are not transformed (coefficient blocks defined in a pixel domain) are manipulated according to the prediction modes. General residual coding is designed for the transformed blocks so that the statistics of each syntax element may go undesirable when the transform-skipped block is coded, which degrades the coding efficiency. When the currently coded block is intra-predicted, the intra prediction mode can be used for vertical / horizontal flip. In addition, rotating blocks and the combination of rearrange methods are tested.

## CE8-4.2 (JVET-N0429)

In this test, a flag which indicates whether all of the signs in the current CG are identical is sent before the signs of each coefficient are transmitted. This method is motivated by the observation that the signs of coefficients in the transform skip block have higher correlation from each other than those of transformed coefficients. If the flag is true, the common sign information, whether the representing sign in the block is positive or negative is sent, and then no more sign information is coded for the CG. Moreover, the proposed flag can be coded when the number of nonzero coefficients are greater than a threshold. In addition, sign flags are coded using context coded bins where the number of context coded bins assigned for a CG is not changed. Otherwise (when the proposed flag is set to 0 or the number of nonzero coefficients is less than the threshold), the sign information of each nonzero coefficient is coded in the same way as in the current VVC.

## CE8-4.3 and CE8-4.4 (JVET-N0280)

Residual Coding for Transform Skip as proposed in JVET-M0464 including the following parts to be tested:

No last significant scanning position: Since the residual signal reflects the spatial residual after the prediction and no energy compaction by transform is performed for TS, the higher probability for trailing zeros or insignificant levels at the bottom right corner of the transform block is not given anymore. Thus, last significant scanning position signalling is omitted in this case. Instead, the first subblock to be processed is the most bottom right subblock within the transform block.

Subblock CBFs: The absence of the last significant scanning position signalling requires the subblock CBF signalling with coded_sub_block_flag for TS to be modified as follows:

- Due to quantization, the aforementioned sequence of insignificance may still occur locally inside a transform block. Thus, the last significant scanning position is removed as described before and coded_sub_block_flag is coded for all sub-blocks.

- The coded_sub_block_flag for the subblock covering the DC frequency position (top-left subblock) presents a special case. In VVC Draft 3, the coded_sub_block_flag for this subblock is never signaled and always inferred to be equal to 1. When the last significant scanning position is located in another subblock, it means that there is at least one significant level outside the DC subblock. Consequently, the DC subblock may contain only zero/non-significant levels although the coded_sub_block_flag for this subblock is inferred to be equal to 1. With the absence of the last scanning position information in TS, the coded_sub_block_flag for each subblock is signaled. This also includes the coded_sub_block_flag for the DC subblock except when all other coded_sub_block_flag syntax elements are already equal to 0. In this case, the DC coded_sub_block_flag is inferred to be equal to 1 (inferDcSbCbf=1). Since there has to be at least one significant level in this DC subblock, the sig_coeff_flag syntax element for the first position at (0,0) is not signaled and derived to be equal to 1 (inferSbDcSigCoeffFlag=1) instead if all other sig_coeff_flag syntax elements in this DC subblock are equal to 0.

- The context modeling for coded_sub_block_flag is changed. The context model index is calculated as the sum of the coded_sub_block_flag to the right and the coded_sub_block_flag below the current subblock instead of and a logical disjunction of both.

sig_coeff_flag context modelling: The local template in sig_coeff_flag  context modeling is modified to only include the neighbor to the right (NB0) and the neighbor below (NB1) the current scanning position. The context model offset is just the number of significant neighboring positions sig_coeff_flag[NB0] + sig_coeff_flag[NB1]. Hence, the selection of different context sets depending on the diagonal d within the current transform block is removed. This results in three context models and a single context model set for coding the sig_coeff_flag flag.

abs_level_gt1_flag and par_level_flag context modelling: a single context model is employed for abs_level_gt1_flag and par_level_flag.

abs_remainder coding: Although the empirical distribution of the transform skip residual absolute levels typically still fits a Laplacian or a Geometrical distribution, there exist larger instationarities than for transform coefficient absolute levels. Particularly, the variance within a window of consecutive realization is higher for the residual absolute levels. This motivates the following modifications of the abs_remainder syntax binarization and context modelling:

- Using a higher cutoff value in the binarization, i.e., the transition point from the coding with sig_coeff_flag, abs_level_gt1_flag, par_level_flag, and abs_level_gt3_flag to the Rice codes for abs_remainder, and dedicated context models for each bin position yields higher compression efficiency. Increasing the cutoff will result in more "greater than X" flags, e.g. introducing abs_level_gt5_flag, abs_level_gt7_flag, and so on until a cutoff is reached. The cutoff itself is fixed to 5 (numGtFlags=5).

- The template for the rice parameter derivation is modified, i.e., only the neighbor to the left and the neighbor below the current scanning position are considered similar to the local template for sig_coeff_flag context modeling.

coeff_sign_flag context modelling: Due to the instationarities inside the sequence of signs and the fact that the prediction residual is often biased, the signs can be coded using context models, even when the global empirical distribution is almost uniformly distributed. A single dedicated context model is used for the coding of the signs and the sign is parsed after sig_coeff_flag to keep all context coded bins together.

Reduction of context coded bins: The first scanning pass, i.e., the transmission of the sig_coeff_flag, abs_level_gt1_flag, and par_level_flag syntax elements, is unchanged. However, the limit on the maximum number of context coded bins per sample (CCBs) is removed and handled differently. The reduction of CCBs can be guaranteed by specifying a mode with CCB > k as invalid with k being a positive integer number. Note that k=2 for the regular level coding mode of the current VVC development. Such a limitation corresponds to a reduction of the quantization space.

# CE results summary

It is agreed that VTM version VTM-4.0 shall be used for these experiments, as specified in the corresponding output document from last meeting. The following anchors are considered for tests in this CE:

- The proposed methods in this CE shall be tested with IBC mode enabled in VTM-4.0.

- The non-IBC related methods in this CE shall be tested also with IBC mode disabled in VTM-4.0.

- In both cases (IBC=1 and IBC=0), Hash based ME should be turned on for class F and class TGM.

- In both cases (IBC=1 and IBC=0), Hash based ME should be turned off for natural content (class A, B, C, D, E).

- Note: In addition to CTC, CE8-4 should test for low QP cases (2, 7, 12, 17) for 1st 100 frames of each sequence, which are also used in CE7 tests.

The mandatory common test conditions (CTC) as specified in the JVET-L1010 document from this meeting shall be followed to perform the simulations in this CE.

The mandatory test sequences as described in common test conditions shall be used for these tests.

In addition to that, to evaluate the benefit of proposed tools on screen content materials, the sequences in TGM YUV 4:2:0 1080p category from HEVC SCC CTC JCT-VC-U1015 document and class F in common test conditions shall be tested. The details of these sequences are listed in the table below.

Table 2: screen content test sequences used in CE8

| Category | Resolution | Sequence name | fps | Bit depth | Frames to be encoded | Intra | Random access | Low-delay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4:2:0 TGM | 1920x1080 1920x1080 1920x1080 1920x1080 | FlyingGraphics* Desktop Console ChineseEditing | 60 60 60 60 | 8 8 8 8 | 0-299* 0-599 0-599 0-599 | M M M M | M M M M | O O O O |
| Class F | 832x480 1920x1080 1280x720 1280x720 | BasketballDrillText ArenaOfValor SlideEditing SlideShow | 50 60 30 20 | 8 8 8 8 | 0-499 0-599 0-299 0-499 | M M M M | M M M M | M M M M |
| *Note that only the first 300 frames of this sequence are used. M refers to mandatory test in the configuration; O refers to optional test in the configuration |  |  |  |  |  |  |  |  |

The followings are summary tables of the tests in this CE.

Table 3: CE8 test results with VTM-4.0+IBC=1

|  |  | AI Over VTM-4.0 |  |  |  |  | RA Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CTC | CE8-1.1a | 0.00% | 0.00% | -0.02% | 101% | 99% | -0.03% | -0.04% | -0.08% | 102% | 101% |
|  | CE8-1.1b | -0.04% | -0.03% | -0.06% | 103% | 103% | -0.04% | -0.06% | -0.03% | 103% | 100% |
|  | CE8-1.2b | -0.01% | -0.01% | 0.00% | 100% | 99% | 0.00% | 0.02% | 0.00% | 100% | 100% |
|  | CE8-1.2c | 0.00% | -0.01% | -0.02% | 100% | 99% | 0.00% | 0.04% | -0.07% | 100% | 100% |
|  | CE8-1.2d | -0.01% | -0.02% | -0.03% | 100% | 99% | 0.00% | -0.03% | -0.04% | 99% | 99% |
|  | CE8-1.3a | -0.02% | -0.06% | -0.04% | 108% | 100% | -0.01% | -0.02% | -0.10% | 102% | 100% |
|  | CE8-1.3b | -0.01% | -0.04% | -0.02% | 108% | 100% | 0.00% | 0.07% | -0.02% | 101% | 100% |
|  | CE8-2.1 | 0.06% | 0.02% | 0.05% | 103% | 104% | 0.04% | -0.02% | -0.04% | 103% | 102% |
|  | CE8-2.2a | 0.06% | 0.02% | 0.05% | 101% | 98% | 0.05% | 0.02% | 0.00% | 102% | 99% |
|  | CE8-2.2c | 0.06% | 0.02% | 0.04% | 105% | 103% | 0.04% | -0.08% | -0.03% | 102% | 100% |
|  | CE8-2.2e | 0.06% | 0.02% | 0.04% | 106% | 102% | 0.04% | -0.04% | -0.02% | 101% | 98% |
|  | CE8-3.1a | -0.04% | -0.02% | -0.02% | 106% | 102% | 0.02% | 0.03% | -0.10% | 101% | 101% |
|  | CE8-3.1b | -0.04% | -0.01% | -0.03% | 106% | 102% | 0.02% | -0.07% | 0.00% | 101% | 100% |
|  | CE8-4.1a | -0.02% | -0.01% | 0.00% | 100% | 100% | -0.02% | -0.05% | -0.02% | 100% | 99% |
|  | CE8-4.1b | -0.07% | -0.05% | -0.02% | 100% | 99% | -0.04% | 0.04% | -0.11% | 99% | 99% |
|  | CE8-4.1c | -0.08% | -0.08% | -0.01% | 100% | 99% | -0.03% | -0.08% | -0.07% | 100% | 100% |
|  | CE8-4.2a | -0.12% | -0.04% | 0.01% | 101% | 105% | -0.02% | -0.06% | 0.00% | 101% | 102% |
|  | CE8-4.2b | -0.13% | -0.03% | 0.02% | 101% | 103% | -0.04% | -0.03% | -0.08% | 102% | 102% |
|  | CE8-4.3a | -0.11% | -0.02% | 0.02% | 100% | 102% | -0.02% | 0.01% | -0.08% | 100% | 101% |
|  | CE8-4.3b | -0.12% | -0.03% | 0.02% | 100% | 101% | -0.02% | -0.03% | -0.01% | 100% | 101% |
|  | CE8-4.4a | -0.12% | -0.03% | 0.02% | 100% | 101% | -0.02% | 0.02% | -0.05% | 100% | 100% |
|  | CE8-4.4b | -0.12% | -0.02% | 0.02% | 100% | 101% | -0.03% | -0.02% | -0.05% | 100% | 101% |
|  | CE8-5.1a | -0.12% | -0.03% | 0.03% | 106% | 102% | 0.02% | 0.00% | -0.03% | 101% | 100% |
|  | CE8-5.1b | -0.12% | -0.04% | 0.05% | 106% | 102% | 0.02% | 0.03% | 0.05% | 101% | 99% |
| Class F | CE8-1.1a | 0.02% | -0.03% | -0.09% | 101% | 100% | -0.02% | -0.04% | -0.08% | 100% | 100% |
|  | CE8-1.1b | -0.20% | -0.22% | -0.32% | 102% | 100% | -0.18% | -0.27% | -0.31% | 101% | 99% |
|  | CE8-1.2b | -0.21% | -0.17% | -0.15% | 100% | 100% | -0.21% | -0.28% | -0.26% | 100% | 100% |
|  | CE8-1.2c | -0.10% | -0.05% | -0.18% | 101% | 100% | -0.15% | -0.22% | -0.19% | 100% | 101% |
|  | CE8-1.2d | -0.32% | -0.36% | -0.38% | 100% | 101% | -0.26% | -0.44% | -0.33% | 100% | 100% |
|  | CE8-1.3a | -0.03% | -0.03% | -0.08% | 111% | 101% | -0.04% | -0.03% | -0.19% | 103% | 101% |
|  | CE8-1.3b | -0.01% | -0.07% | -0.19% | 108% | 101% | -0.08% | -0.09% | -0.13% | 102% | 101% |
|  | CE8-2.1 | -2.38% | -2.20% | -2.28% | 100% | 102% | -1.44% | -2.13% | -2.07% | 103% | 101% |
|  | CE8-2.2a | -2.38% | -2.20% | -2.28% | 98% | 96% | -1.44% | -1.97% | -1.89% | 100% | 98% |
|  | CE8-2.2c | -2.63% | -2.73% | -2.87% | 106% | 100% | -1.79% | -2.61% | -2.61% | 105% | 101% |
|  | CE8-2.2e | -2.63% | -2.73% | -2.87% | 106% | 96% | -1.75% | -2.44% | -2.30% | 105% | 101% |
|  | CE8-3.1a | -2.63% | -1.72% | -1.70% | 104% | 100% | -1.74% | -1.26% | -1.29% | 101% | 101% |
|  | CE8-3.1b | -2.78% | -1.74% | -1.84% | 104% | 100% | -1.81% | -1.45% | -1.35% | 101% | 101% |
|  | CE8-4.1a | -0.37% | -0.20% | -0.36% | 100% | 100% | -0.34% | -0.37% | -0.23% | 100% | 100% |
|  | CE8-4.1b | -0.58% | -0.30% | -0.42% | 100% | 100% | -0.47% | -0.27% | -0.28% | 100% | 100% |
|  | CE8-4.1c | -0.74% | -0.47% | -0.48% | 100% | 101% | -0.57% | -0.33% | -0.51% | 100% | 101% |
|  | CE8-4.2a | -2.03% | -0.82% | -0.79% | 104% | 104% | -1.10% | -0.32% | -0.27% | 103% | 104% |
|  | CE8-4.2b |  |  |  |  |  |  |  |  |  |  |
|  | CE8-4.3a | -1.83% | -0.75% | -0.79% | 101% | 96% | -0.96% | -0.18% | -0.35% | 100% | 98% |
|  | CE8-4.3b | -2.67% | -1.41% | -1.56% | 100% | 96% | -1.77% | -1.05% | -1.01% | 100% | 100% |
|  | CE8-4.4a | -2.57% | -1.39% | -1.36% | 99% | 96% | -1.74% | -0.99% | -0.92% | 99% | 100% |
|  | CE8-4.4b | -2.79% | -1.52% | -1.70% | 99% | 98% | -1.93% | -1.22% | -1.18% | 99% | 100% |
|  | CE8-5.1a | -3.77% | -2.26% | -2.31% | 103% | 100% | -2.65% | -1.71% | -1.77% | 101% | 102% |
|  | CE8-5.1b | -3.94% | -2.38% | -2.59% | 104% | 101% | -2.80% | -1.82% | -1.96% | 101% | 101% |
| SCC TGM | CE8-1.1a | 0.02% | -0.01% | -0.01% | 100% | 100% | -0.06% | -0.01% | 0.00% | 99% | 99% |
|  | CE8-1.1b | -0.72% | -0.67% | -0.68% | 101% | 101% | -0.51% | -0.45% | -0.54% | 100% | 98% |
|  | CE8-1.2b | -0.58% | -0.55% | -0.54% | 100% | 100% | -0.25% | -0.21% | -0.25% | 100% | 100% |
|  | CE8-1.2c | -0.24% | -0.27% | -0.25% | 100% | 100% | -0.12% | -0.11% | -0.08% | 100% | 100% |
|  | CE8-1.2d | -0.85% | -0.80% | -0.81% | 100% | 100% | -0.35% | -0.20% | -0.32% | 100% | 101% |
|  | CE8-1.3a | -0.16% | -0.17% | -0.17% | 112% | 101% | -0.06% | 0.00% | -0.04% | 105% | 100% |
|  | CE8-1.3b | -0.16% | -0.17% | -0.17% | 110% | 100% | -0.12% | -0.05% | -0.06% | 104% | 100% |
|  | CE8-2.1 | -6.85% | -5.66% | -5.29% | 91% | 94% | -3.30% | -3.16% | -3.03% | 100% | 100% |
|  | CE8-2.2a | -6.85% | -5.66% | -5.29% | 90% | 93% | -3.18% | -2.80% | -2.52% | 98% | 97% |
|  | CE8-2.2c | -8.09% | -6.55% | -6.16% | 99% | 94% | -3.87% | -3.62% | -3.48% | 102% | 100% |
|  | CE8-2.2e | -8.09% | -6.55% | -6.16% | 99% | 91% | -3.68% | -3.36% | -3.01% | 101% | 98% |
|  | CE8-3.1a | -3.51% | -2.25% | -2.20% | 103% | 99% | -1.61% | -0.81% | -0.87% | 100% | 101% |
|  | CE8-3.1b | -3.78% | -2.41% | -2.35% | 102% | 99% | -1.68% | -0.92% | -0.92% | 100% | 101% |
|  | CE8-4.1a | -0.45% | -0.30% | -0.30% | 100% | 100% | -0.38% | -0.22% | -0.21% | 100% | 101% |
|  | CE8-4.1b | -0.57% | -0.34% | -0.33% | 100% | 99% | -0.31% | -0.07% | -0.07% | 100% | 100% |
|  | CE8-4.1c | -0.74% | -0.48% | -0.48% | 100% | 100% | -0.53% | -0.18% | -0.19% | 100% | 100% |
|  | CE8-4.2a | -2.13% | -0.04% | -0.16% | 106% | 99% | -1.05% | 0.67% | 0.72% | 102% | 100% |
|  | CE8-4.2b |  |  |  |  |  |  |  |  |  |  |
|  | CE8-4.3a | -1.85% | 0.14% | 0.01% | 103% | 99% | -0.90% | 0.74% | 0.81% | 99% | 100% |
|  | CE8-4.3b | -4.00% | -1.77% | -1.84% | 101% | 99% | -2.58% | -0.72% | -0.64% | 99% | 101% |
|  | CE8-4.4a | -3.87% | -1.71% | -1.81% | 100% | 99% | -2.67% | -0.74% | -0.63% | 98% | 101% |
|  | CE8-4.4b | -4.37% | -2.12% | -2.19% | 100% | 100% | -2.96% | -0.98% | -0.97% | 98% | 101% |
|  | CE8-5.1a | -5.83% | -3.24% | -3.25% | 102% | 98% | -3.54% | -1.23% | -1.13% | 99% | 101% |
|  | CE8-5.1b | -6.23% | -3.63% | -3.63% | 102% | 97% | -3.77% | -1.56% | -1.48% | 99% | 101% |

Table 4: CE8 test results with VTM-4.0+IBC=0

|  |  | AI Over VTM-4.0 |  |  |  |  | RA Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CTC | CE8-2.1 | 0.06% | 0.06% | 0.04% | 102% | 105% | 0.04% | -0.04% | -0.03% | 104% | 104% |
|  | CE8-2.2a | 0.06% | 0.06% | 0.04% | 102% | 103% | 0.04% | -0.04% | -0.03% | 102% | 102% |
|  | CE8-2.2c | 0.06% | 0.06% | 0.04% | 102% | 107% | 0.03% | 0.00% | 0.02% | 105% | 105% |
|  | CE8-2.2e | 0.06% | 0.06% | 0.04% | 102% | 105% | 0.03% | 0.00% | 0.02% | 103% | 102% |
|  | CE8-3.1a | -0.06% | -0.01% | 0.03% | 109% | 101% | 0.02% | 0.03% | -0.10% | 101% | 101% |
|  | CE8-3.1b | -0.06% | -0.01% | 0.00% | 109% | 102% | 0.01% | -0.01% | -0.13% | 102% | 101% |
|  | CE8-4.1a | -0.03% | -0.04% | -0.03% | 100% | 100% | -0.02% | -0.06% | -0.03% | 101% | 101% |
|  | CE8-4.1b | -0.08% | -0.03% | -0.06% | 100% | 100% | -0.04% | -0.02% | -0.08% | 100% | 100% |
|  | CE8-4.1c | -0.08% | -0.02% | -0.06% | 100% | 101% | -0.04% | -0.02% | -0.22% | 101% | 101% |
|  | CE8-4.2a | -0.12% | -0.05% | -0.02% | 100% | 103% | -0.02% | -0.03% | -0.13% | 103% | 103% |
|  | CE8-4.2b |  |  |  |  |  |  |  |  |  |  |
|  | CE8-4.3a | -0.12% | -0.04% | -0.01% | 100% | 99% | -0.03% | -0.04% | -0.10% | 99% | 99% |
|  | CE8-4.3b | -0.13% | -0.04% | -0.03% | 100% | 99% | -0.03% | -0.02% | -0.10% | 99% | 99% |
|  | CE8-4.4a | -0.13% | -0.05% | -0.03% | 100% | 100% | -0.04% | 0.00% | -0.04% | 99% | 99% |
|  | CE8-4.4b | -0.13% | -0.04% | -0.02% | 100% | 100% | -0.02% | -0.01% | -0.06% | 99% | 98% |
|  | CE8-5.1a | -0.14% | 0.00% | 0.02% | 109% | 102% | 0.02% | 0.01% | -0.13% | 101% | 100% |
|  | CE8-5.1b | -0.14% | -0.01% | 0.02% | 109% | 102% | 0.01% | 0.00% | -0.12% | 101% | 100% |
| Class F | CE8-2.1 | -8.58% | -7.71% | -7.86% | 94% | 99% | -6.72% | -7.66% | -7.47% | 103% | 103% |
|  | CE8-2.2a | -8.58% | -7.71% | -7.86% | 94% | 96% | -6.72% | -7.52% | -7.22% | 103% | 101% |
|  | CE8-2.2c | -8.79% | -7.85% | -8.08% | 103% | 101% | -7.22% | -8.13% | -7.97% | 105% | 103% |
|  | CE8-2.2e | -8.79% | -7.85% | -8.08% | 103% | 98% | -7.21% | -7.96% | -7.75% | 105% | 101% |
|  | CE8-3.1a | -5.90% | -4.52% | -4.51% | 109% | 97% | -4.04% | -3.48% | -3.29% | 102% | 100% |
|  | CE8-3.1b | -6.26% | -4.90% | -4.82% | 108% | 98% | -4.37% | -3.76% | -3.69% | 102% | 100% |
|  | CE8-4.1a | -1.06% | -0.93% | -0.85% | 100% | 100% | -0.88% | -0.73% | -0.60% | 101% | 101% |
|  | CE8-4.1b | -0.86% | -0.66% | -0.70% | 100% | 100% | -0.68% | -0.43% | -0.42% | 101% | 101% |
|  | CE8-4.1c | -1.11% | -1.01% | -0.99% | 98% | 102% | -0.88% | -0.64% | -0.55% | 101% | 102% |
|  | CE8-4.2a | -3.80% | -2.36% | -2.33% | 106% | 102% | -2.12% | -1.37% | -1.20% | 103% | 103% |
|  | CE8-4.2b | -5.52% | -3.87% | -3.90% | 104% | 100% | -3.60% | -2.59% | -2.40% | 104% | 103% |
|  | CE8-4.3a | -3.47% | -2.19% | -2.25% | 103% | 98% | -1.96% | -1.30% | -1.22% | 101% | 101% |
|  | CE8-4.3b | -5.25% | -3.64% | -3.70% | 102% | 96% | -3.41% | -2.56% | -2.35% | 101% | 101% |
|  | CE8-4.4a | -4.70% | -2.98% | -3.14% | 101% | 97% | -2.94% | -2.08% | -1.78% | 101% | 101% |
|  | CE8-4.4b | -5.48% | -3.84% | -3.85% | 101% | 96% | -3.65% | -2.69% | -2.44% | 100% | 101% |
|  | CE8-5.1a | -7.19% | -5.37% | -5.42% | 108% | 94% | -4.95% | -3.92% | -3.77% | 101% | 99% |
|  | CE8-5.1b | -7.80% | -5.97% | -5.94% | 108% | 96% | -5.50% | -4.37% | -4.26% | 101% | 99% |
| SCC TGM | CE8-2.1 | -27.12% | -25.34% | -25.16% | 75% | 81% | -13.17% | -13.41% | -13.18% | 99% | 100% |
|  | CE8-2.2a | -27.12% | -25.34% | -25.16% | 75% | 81% | -13.19% | -12.80% | -12.27% | 99% | 98% |
|  | CE8-2.2c | -29.14% | -26.79% | -26.63% | 87% | 83% | -14.51% | -14.35% | -14.25% | 101% | 100% |
|  | CE8-2.2e | -29.14% | -26.79% | -26.63% | 87% | 81% | -14.55% | -13.85% | -13.47% | 101% | 98% |
|  | CE8-3.1a | -15.10% | -12.15% | -12.24% | 108% | 93% | -7.09% | -5.25% | -5.29% | 101% | 99% |
|  | CE8-3.1b | -16.09% | -13.20% | -13.29% | 106% | 94% | -7.72% | -5.89% | -5.86% | 100% | 99% |
|  | CE8-4.1a | -2.19% | -1.90% | -1.92% | 100% | 98% | -1.17% | -1.00% | -0.98% | 101% | 101% |
|  | CE8-4.1b | -1.84% | -1.68% | -1.67% | 99% | 99% | -0.77% | -0.72% | -0.72% | 101% | 101% |
|  | CE8-4.1c | -2.20% | -2.00% | -1.99% | 97% | 100% | -1.11% | -0.98% | -0.97% | 100% | 101% |
|  | CE8-4.2a | -9.61% | -6.94% | -7.24% | 109% | 96% | -4.28% | -2.36% | -2.36% | 102% | 100% |
|  | CE8-4.2b | -14.31% | -10.94% | -11.16% | 106% | 94% | -7.12% | -4.77% | -4.73% | 102% | 100% |
|  | CE8-4.3a | -8.75% | -6.33% | -6.59% | 107% | 92% | -3.81% | -2.08% | -2.09% | 100% | 99% |
|  | CE8-4.3b | -13.76% | -10.44% | -10.68% | 104% | 92% | -6.81% | -4.44% | -4.47% | 100% | 98% |
|  | CE8-4.4a | -12.60% | -9.23% | -9.46% | 101% | 89% | -6.27% | -3.93% | -3.90% | 99% | 99% |
|  | CE8-4.4b | -14.35% | -10.94% | -11.15% | 101% | 91% | -7.27% | -4.88% | -4.88% | 99% | 100% |
|  | CE8-5.1a | -17.91% | -14.15% | -14.34% | 107% | 90% | -9.29% | -6.39% | -6.41% | 99% | 99% |
|  | CE8-5.1b | -19.10% | -15.39% | -15.56% | 106% | 91% | -10.08% | -7.25% | -7.25% | 99% | 98% |

Table 5: CE8 test results with VTM-4.0+IBC=1, low QPs

|  |  | AI Over VTM-4.0 |  |  |  |  | RA Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CTC | CE8-2.1 | -0.24% | -0.37% | -0.42% | 103% | 102% | -0.02% | -0.03% | -0.01% | 103% | 102% |
|  | CE8-2.2c | -0.24% | -0.37% | -0.42% | 107% | 102% | -0.02% | -0.03% | -0.02% | 103% | 101% |
|  | CE8-3.1a |  |  |  |  |  |  |  |  |  |  |
|  | CE8-3.1b |  |  |  |  |  |  |  |  |  |  |
|  | CE8-4.1a |  |  |  |  |  |  |  |  |  |  |
|  | CE8-4.1b | -0.02% | -0.02% | -0.02% | 100% | 100% |  |  |  |  |  |
|  | CE8-4.1c | -0.02% | -0.03% | -0.02% | 100% | 100% |  |  |  |  |  |
|  | CE8-4.2a | -0.03% | -0.02% | -0.02% | 102% | 103% |  |  |  |  |  |
|  | CE8-4.2b |  |  |  |  |  |  |  |  |  |  |
|  | CE8-4.3a | -0.04% | -0.01% | -0.03% | 100% | 101% | 0.08% | 0.04% | 0.07% | 99% | 100% |
|  | CE8-4.3b | -0.07% | -0.03% | -0.06% | 100% | 100% | 0.07% | 0.03% | 0.07% | 99% | 100% |
|  | CE8-4.4a | -0.07% | -0.04% | -0.05% | 100% | 100% | 0.06% | 0.04% | 0.06% | 99% | 101% |
|  | CE8-4.4b | -0.08% | -0.04% | -0.05% | 100% | 101% | 0.05% | 0.06% | 0.07% | 99% | 101% |
|  | CE8-5.1a |  |  |  |  |  |  |  |  |  |  |
|  | CE8-5.1b |  |  |  |  |  |  |  |  |  |  |
| Class F | CE8-2.1 | -1.06% | -1.13% | -1.19% | 104% | 101% | -1.04% | -0.90% | -0.92% | 104% | 98% |
|  | CE8-2.2c | -1.29% | -1.39% | -1.48% | 109% | 99% | -1.01% | -1.06% | -1.07% | 106% | 100% |
|  | CE8-3.1a |  |  |  |  |  |  |  |  |  |  |
|  | CE8-3.1b |  |  |  |  |  |  |  |  |  |  |
|  | CE8-4.1a | -0.27% | -0.17% | -0.19% | 100% | 101% | -0.24% | -0.17% | -0.18% | 100% | 100% |
|  | CE8-4.1b | -0.38% | -0.27% | -0.29% | 100% | 100% | -0.26% | -0.23% | -0.19% | 100% | 100% |
|  | CE8-4.1c | -0.48% | -0.39% | -0.36% | 100% | 102% | -0.44% | -0.29% | -0.23% | 100% | 101% |
|  | CE8-4.2a | 0.97% | 0.75% | 0.72% | 103% | 103% | 0.91% | 1.41% | 1.33% | 102% | 101% |
|  | CE8-4.2b |  |  |  |  |  |  |  |  |  |  |
|  | CE8-4.3a | 1.06% | 0.80% | 0.76% | 102% | 100% | 0.97% | 1.41% | 1.33% | 100% | 102% |
|  | CE8-4.3b | -0.14% | -0.19% | -0.22% | 102% | 101% | -0.20% | 0.57% | 0.50% | 100% | 101% |
|  | CE8-4.4a | -0.34% | -0.30% | -0.31% | 100% | 100% | -0.45% | 0.45% | 0.43% | 98% | 101% |
|  | CE8-4.4b | -0.70% | -0.63% | -0.64% | 100% | 101% | -0.82% | 0.16% | 0.15% | 99% | 100% |
|  | CE8-5.1a | -1.06% | -0.99% | -0.98% | 104% | 99% | -2.90% | 0.43% | 0.36% | 99% | 96% |
|  | CE8-5.1b | -1.46% | -1.33% | -1.37% | 104% | 99% | -3.87% | 0.22% | 0.11% | 99% | 96% |
| SCC TGM | CE8-2.1 | -10.24% | -8.16% | -7.98% | 94% | 91% | -7.04% | -4.05% | -4.04% | 97% | 97% |
|  | CE8-2.2c | -11.86% | -9.79% | -9.62% | 102% | 88% | -7.57% | -5.26% | -5.18% | 101% | 97% |
|  | CE8-3.1a |  |  |  |  |  |  |  |  |  |  |
|  | CE8-3.1b |  |  |  |  |  |  |  |  |  |  |
|  | CE8-4.1a | -0.38% | -0.32% | -0.31% | 101% | 101% | 0.57% | -0.07% | -0.04% | 100% | 100% |
|  | CE8-4.1b | -0.47% | -0.32% | -0.35% | 101% | 100% | -0.12% | -0.10% | -0.13% | 100% | 100% |
|  | CE8-4.1c | -0.85% | -0.63% | -0.61% | 100% | 101% | 0.21% | -0.07% | -0.16% | 100% | 101% |
|  | CE8-4.2a | 3.75% | 4.10% | 4.10% | 109% | 95% | 2.65% | 5.73% | 5.76% | 102% | 101% |
|  | CE8-4.2b |  |  |  |  |  |  |  |  |  |  |
|  | CE8-4.3a | 4.05% | 4.26% | 4.25% | 109% | 100% | 2.24% | 5.83% | 5.86% | 100% | 100% |
|  | CE8-4.3b | -0.85% | 0.68% | 0.67% | 105% | 99% | -3.07% | 2.80% | 2.80% | 98% | 100% |
|  | CE8-4.4a | -1.61% | 0.36% | 0.35% | 101% | 99% | -5.57% | 2.51% | 2.53% | 97% | 98% |
|  | CE8-4.4b | -2.96% | -0.76% | -0.78% | 101% | 100% | -9.82% | 1.70% | 1.77% | 97% | 99% |
|  | CE8-5.1a | -3.50% | -1.05% | -1.07% | 103% | 97% | -1.79% | 2.23% | 2.31% | 98% | 97% |
|  | CE8-5.1b | -4.81% | -2.21% | -2.23% | 103% | 97% | 1.97% | 1.27% | 1.29% | 97% | 96% |

Table 6: CE8 test results with VTM-4.0+IBC=0, low QPs

|  |  | AI Over VTM-4.0 |  |  |  |  | RA Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CTC | CE8-3.1a |  |  |  |  |  |  |  |  |  |  |
|  | CE8-3.1b |  |  |  |  |  |  |  |  |  |  |
|  | CE8-4.1a | -0.01% | -0.01% | -0.02% | 100% | 101% | -0.01% | 0.01% | 0.01% | 100% | 100% |
|  | CE8-4.1b | -0.02% | -0.02% | -0.02% | 100% | 100% | -0.01% | -0.01% | -0.01% | 100% | 100% |
|  | CE8-4.1c | -0.02% | -0.02% | -0.02% | 100% | 101% | -0.01% | -0.01% | -0.02% | 100% | 100% |
|  | CE8-4.2a | -0.03% | 0.00% | -0.02% | 101% | 102% |  |  |  |  |  |
|  | CE8-4.2b |  |  |  |  |  |  |  |  |  |  |
|  | CE8-4.3a | -0.03% | -0.02% | -0.02% | 100% | 100% | 0.08% | 0.02% | 0.02% | 99% | 100% |
|  | CE8-4.3b | -0.06% | -0.04% | -0.04% | 100% | 100% | 0.06% | 0.04% | 0.06% | 99% | 100% |
|  | CE8-4.4a | -0.06% | -0.03% | -0.04% | 101% | 101% | 0.06% | 0.04% | 0.06% | 99% | 100% |
|  | CE8-4.4b | -0.07% | -0.04% | -0.05% | 100% | 100% | 0.05% | 0.05% | 0.06% | 99% | 100% |
|  | CE8-5.1a |  |  |  |  |  |  |  |  |  |  |
|  | CE8-5.1b |  |  |  |  |  |  |  |  |  |  |
| Class F | CE8-3.1a |  |  |  |  |  |  |  |  |  |  |
|  | CE8-3.1b |  |  |  |  |  |  |  |  |  |  |
|  | CE8-4.1a | -0.65% | -0.51% | -0.50% | 100% | 100% | -0.46% | -0.39% | -0.37% | 101% | 101% |
|  | CE8-4.1b | -0.60% | -0.54% | -0.47% | 100% | 100% | -0.25% | -0.26% | -0.37% | 101% | 101% |
|  | CE8-4.1c | -0.77% | -0.66% | -0.63% | 98% | 100% | -0.50% | -0.47% | -0.47% | 100% | 101% |
|  | CE8-4.2a | 0.65% | 0.36% | 0.37% | 107% | 104% | 1.23% | 0.61% | -2.71% | 105% | 105% |
|  | CE8-4.2b |  |  |  |  |  |  |  |  |  |  |
|  | CE8-4.3a | 0.74% | 0.44% | 0.45% | 105% | 100% | 0.86% | 1.20% | 1.09% | 100% | 98% |
|  | CE8-4.3b | -0.21% | -0.35% | -0.35% | 103% | 98% | -0.08% | 0.53% | 0.47% | 100% | 97% |
|  | CE8-4.4a | -0.30% | -0.38% | -0.34% | 102% | 99% | -0.31% | 0.49% | 0.48% | 99% | 98% |
|  | CE8-4.4b | -0.75% | -0.77% | -0.74% | 102% | 99% | -0.70% | 0.18% | 0.13% | 98% | 97% |
|  | CE8-5.1a | -2.16% | -2.15% | -2.13% | 109% | 100% | -3.67% | -0.45% | -0.43% | 101% | 100% |
|  | CE8-5.1b | -2.76% | -2.68% | -2.66% | 109% | 100% | -4.80% | -0.67% | -0.73% | 101% | 100% |
| SCC TGM | CE8-3.1a |  |  |  |  |  |  |  |  |  |  |
|  | CE8-3.1b |  |  |  |  |  |  |  |  |  |  |
|  | CE8-4.1a | -1.67% | -1.34% | -1.35% | 100% | 100% | -0.02% | -0.57% | -0.56% | 100% | 100% |
|  | CE8-4.1b | -1.17% | -1.01% | -1.03% | 100% | 99% | 0.61% | -0.42% | -0.40% | 100% | 100% |
|  | CE8-4.1c | -1.66% | -1.44% | -1.46% | 97% | 101% | -0.28% | -0.68% | -0.66% | 100% | 101% |
|  | CE8-4.2a | 2.14% | 2.22% | 2.22% | 118% | 99% | 5.34% | 6.82% | 6.92% | 105% | 103% |
|  | CE8-4.2b |  |  |  |  |  |  |  |  |  |  |
|  | CE8-4.3a | 2.74% | 2.56% | 2.55% | 117% | 97% | 3.70% | 5.02% | 5.07% | 100% | 99% |
|  | CE8-4.3b | -2.82% | -1.32% | -1.35% | 112% | 96% | -1.36% | 1.93% | 1.99% | 99% | 98% |
|  | CE8-4.4a | -2.46% | -0.79% | -0.81% | 106% | 97% | -1.99% | 2.02% | 2.09% | 98% | 97% |
|  | CE8-4.4b | -4.74% | -2.55% | -2.57% | 106% | 95% | -3.50% | 0.90% | 0.98% | 98% | 97% |
|  | CE8-5.1a | -11.06% | -8.41% | -8.47% | 112% | 94% | -9.97% | -1.08% | -0.98% | 99% | 99% |
|  | CE8-5.1b | -13.22% | -10.21% | -10.26% | 109% | 94% | 3.05% | -2.42% | -2.41% | 99% | 100% |

# Crosschecker reports

Crosscheckers reported that the software/results in CE8 are generally in good shape, no outstanding issues were identified. So minor differences were found, which should be due to the identified IBC related bugs in VTM-4.0.

Table 7: crosscheck reports that are associated with the tests in this CE.

| # | Software review/studied? (Y/N) | Any significant inconsistencies between the description and the provided software? (Y/N) | Results available (Full/Partial) | Resutls matched? (Y/N) | Runtime behavior matched? (Y/N) | Additional comments |
| --- | --- | --- | --- | --- | --- | --- |
| CE8-1.1a | Y | N | Full | Y | N |  |
| CE8-1.1b | Y | N | Full | Y | N |  |
| CE8-1.2b |  |  |  |  |  |  |
| CE8-1.2c |  |  |  |  |  |  |
| CE8-1.2d |  |  |  |  |  |  |
| CE8-1.3a | Y | N | Partial (all results for AI and RA cfg) | Y | N | Runtime shows differences of about ±2% (homogeneous cluster were used for simulations) |
| CE8-1.3b | Y | N | Partial (all results for AI and RA cfg) | Y | N | Runtime shows differences of about ±2% (homogeneous cluster were used for simulations) |
| CE8-2.1 | Y | N | Partial (most results for normal QP; most results for low QPs when IBC=1) | Y | Y |  |
| CE8-2.2a |  |  |  |  |  |  |
| CE8-2.2c |  |  |  |  |  |  |
| CE8-2.2e |  |  |  |  |  |  |
| CE8-3.1a | N | N. A. | Partial (AI with IBC, Class F and TGM) | Y | Y |  |
| CE8-3.1b | N | N. A. | Full | Y | Y |  |
| CE8-4.1a |  |  |  |  |  |  |
| CE8-4.1b | Y | N | Results of CE8-4.1b for IBC off have been crosschecked | Y | Y |  |
| CE8-4.1c | Y | N | Full in CTC/IBC=0 | Y | Y | Runtime matched when homogenous clusters were used |
|  |  |  | Partial in lowQPs/IBC=0/1, CTC/IBC=1 |  |  |  |
| CE8-4.2a | Y | N | Partial | Y | Y | Runtime matched when homogenous clusters were used |
|  |  |  | (all results for normals QPs, AI, RA, most results for the rest |  |  |  |
| CE8-4.2b |  |  |  |  |  |  |
| CE8-4.3a | Y | N | Full in CTC | Y | N |  |
|  |  |  | Partial in low QPs |  |  |  |
| CE8-4.3b | Y | N | Full in CTC | Y | N |  |
|  |  |  | Partial in low QPs |  |  |  |
| CE8-4.4a |  |  |  |  |  |  |
| CE8-4.4b |  |  |  |  |  |  |
| CE8-5.1a |  |  |  |  |  |  |
| CE8-5.1b |  |  |  |  |  |  |

# Input contributions for CE8 and CE8 related topics

## CE contributions

- JVET-N0214, CE8: BDPCM with harmonized residual coding and CCB limitation (CE8-3.1a, CE8-3.1b, CE8-5.1a, CE8-5.1b) ,   G. Clare (bcom), F. Henry (Orange), B. Bross, T. Nguyen, P. Keydel, H. Schwarz, D. Marpe, T. Wiegand (HHI), M. Xu, X. Li, X. Xu, M. Gao, S. Liu (Tencent)

- JVET-N0255, CE8: Combination of MMVD and IBC mode (test 1.3a and 1.3b) , Y. Li, Z. Chen(Wuhan Univ.), X. Xu, X. Li, S. Liu(Tencent)

- JVET-N0280, CE8: Residual Coding for Transform Skip Mode (CE8-4.3a, CE8-4.3b, CE8-4.4a, and CE8-4.4b), B. Bross, T. Nguyen, H. Schwarz, D. Marpe, T. Wiegand (HHI)

- JVET-N0344, CE8: Palette Mode in HEVC (CE8-2.1), Y.-H. Chao, V. Seregin, M. Karczewicz (Qualcomm), Y.-C. Sun, T.-S. Chang, J. Lou (Alibaba)

- JVET-N0404, CE8: Palette Mode Improvements (CE8-2.2), Y.-C. Sun, T.-S. Chang, J. Lou (Alibaba), Y.-H. Chao, V. Seregin, M. Karczewicz (Qualcomm), R. Chernyak, S. Ikonin, J. Chen (Huawei)

- JVET-N0428, CE8-4.1: Rearrangement of the residual block for transform skip, S. Yoo, J. Choi, J. Heo, J. Choi, L. Li, J. Lim, S. Kim (LGE)

- JVET-N0429, CE8-4.2: Residual Coding for transform skip with various maximum context coded bins, S. Yoo, J. Choi, J. Heo, J. Choi, J. Lim, S. Kim (LGE)

- JVET-N0457, CE8-1.1: Block vector prediction for IBC, J. Nam, J. Lim, S. Kim (LGE)

- JVET-N0458, CE8-1.2: Block vector coding for IBC, J. Nam, J. Lim, S. Kim (LGE)

## CE-related and non-CE contributions

- JVET-N0094, CE8-related: Context modelling of transform skip mode, M. G. Sarwer, O. Chubach, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)

- JVET-N0095, CE8-related: Unified method for coding BVD and MVD, S.-T. Hsiang, S.-M. Lei (MediaTek)

- JVET-N0096, CE8-related: A fixed updating order for IBC reference memory, C.-Y. Lai, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)

- JVET-N0122, CE6/CE8-related: Alternative implementation of residual rearrangement for transform skipped blocks, T. Tsukuba, M. Ikeda, T. Suzuki (Sony)

- JVET-N0123, Non-CE6/CE8: Chroma Transform Skip, T. Tsukuba, M. Ikeda, T. Suzuki (Sony)

- JVET-N0173, Non-CE8: IBC Reference Area Rearrange, H. Gao, S. Esenlik, B. Wang, A. M. Kotra, J. Chen (Huawei)

- JVET-N0174, Non-CE8: Exclusive Regular Merge And IBC Merge in One Shared Merge List Area, H. Gao, S. Esenlik, B. Wang, A. M. Kotra, J. Chen (Huawei)

- JVET-N0175, Non-CE8: IBC Reference Memory for Arbitrary CTU Size, H. Gao, S. Esenlik, B. Wang, A. M. Kotra, J. Chen (Huawei)

- JVET-N0176, Non-CE8: IBC Merge List Simplification, H. Gao, S. Esenlik, B. Wang, A. M. Kotra, J. Chen (Huawei)

- JVET-N0201, Non-CE8: IBC modifications, J. Li, C.-W. Kuo, C. S.Lim (Panasonic)

- JVET-N0202, AHG16/Non-CE8: Report on conformance check failures of IBC block vectors, J. Li, C.-W. Kuo, C. S.Lim (Panasonic)

- JVET-N0249, Non-CE8: An alternative search area for IBC, J. Xu, L. Zhang, K. Zhang, H. Liu, Y. Wang (Bytedance)

- JVET-N0250, Non-CE8: Reference memory reduction for intra block copy, J. Xu, L. Zhang, K. Zhang, H. Liu, Y. Wang (Bytedance)

- JVET-N0251, Non-CE8: Intra block copy clean-up, J. Xu, L. Zhang, K. Zhang, H. Liu, Y. Wang (Bytedance)

- JVET-N0258, CE8-related: Palette Mode Coding, W. Zhu, L. Zhang, J. Xu, K. Zhang, H. Liu, Y. Wang (Bytedance)

- JVET-N0259, CE8-related: compound palette mode, W. Zhu, J. Xu, L. Zhang, K. Zhang, H. Liu, Y. Wang (Bytedance)

- JVET-N0260, Non-CE8: Disabling fractional MVD search in DMVR for SCC, W. Zhu, H. Liu, K. Zhang, L. Zhang, J. Xu, Y. Wang (Bytedance)

- JVET-N0289, CE8-Related: On MVD Coding, M. Salehifar, S. Paluri, S. Kim (LGE)

- JVET-N0315, CE8-related: Unification of Motion Vector Rounding Method for Chroma IBC Mode, Y. Han, W.-J. Chien, M. Karczewicz (Qualcomm)

- JVET-N0316, CE8-related: Default Processing for IBC Mode, Y. Han, W.-J. Chien, M. Karczewicz (Qualcomm)

- JVET-N0317, CE8-related: Simplification on IBC Merge/Skip Mode, Y. Han, W.-J. Chien, M. Karczewicz (Qualcomm)

- JVET-N0318, CE8-related: Block Size Limitation for IBC Mode, Y. Han, W.-J. Chien, M. Karczewicz (Qualcomm)

- JVET-N0329, CE8-related: Encoder improvements on IBC search, X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)

- JVET-N0337, CE8-related: Adaptive disabling of intra sample interpolation for screen content coding, X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)

- JVET-N0346, Non-CE8: Palette Mode in HEVC for YUV4:4:4 format, Y.-H. Chao, H. Wang, W.-J. Chien, V. Seregin, M. Karczewicz (Qualcomm)

- JVET-N0357, CE8-related: Context Modelling of Sign for TS Residual Coding, B. Bross, T. Nguyen, H. Schwarz, D. Marpe, T. Wiegand (HHI)

- JVET-N0366, CE8-related: Modified limitation on context coded bins for CE8-3.1a and CE8-5.1a, X. Zhao, X. Li, X. Xu, S. Liu

- JVET-N0382, CE8-related: unified IBC block vector prediction, X. Xu, X. Li, S. Liu (Tencent)

- JVET-N0383, AHG16/Non-CE8: IBC search range adjustment for implementation consideration, X. Xu, X. Li, S. Liu (Tencent)

- JVET-N0384, Non-CE8: IBC search range increase for small CTU sizes, X. Xu, X. Li, S. Liu (Tencent)

- JVET-N0386, AOV YUV4:4:4 sequence for VVC standardization and suggestions for class F, X. Xu, J. Ye, X. Li, S. Liu, L. Wu, C. Xie, K. Liu, B. Wang, P. Liu, K. Dong, Y. Kuang, W. Feng (Tencent)

- JVET-N0401, Non-CE3/8: Enable Transform Skip in CUs using ISP, S. De-Luxán-Hernández, T. Nguyen, B. Bross, H. Schwarz, D. Marpe, T. Wiegand (HHI)

- JVET-N0405, CE8-related: Palette Mode Simplification, Y.-C. Sun, T.-S. Chang, J. Lou (Alibaba), Y.-H. Chao, V. Seregin, M. Karczewicz (Qualcomm)

- JVET-N0413, CE8-related: Quantized residual BDPCM, M. Karczewicz, M. Coban (Qualcomm)

- JVET-N0430, CE8-related: Transform skip restriction, J. Choi, J. Heo, S. Yoo, J. Choi, J. Lim, S. Kim (LGE)

- JVET-N0431, CE8-related: sign flag coding for transform skip residual, S. Yoo, J. Choi, J. Heo, J. Choi, J. Lim, S. Kim (LGE)

- JVET-N0455, CE8-related: Sign context modelling and level mapping for TS residual coding, M. Karczewicz, M. Coban (Qualcomm)

- JVET-N0459, CE8-related: Modified block vector coding for IBC, J. Nam, H. Jang, J. Choi, J. Heo, J. Lim, S. Kim (LGE)

- JVET-N0460, CE8-related: Default candidates for IBC merge mode, J. Nam, H. Jang, J. Lim, S. Kim (LGE)

- JVET-N0461, CE8-related: Signaling on maximum number of candidates for IBC merge mode, J. Nam, H. Jang, J. Lim, S. Kim (LGE)

- JVET-N0464, Non-CE8: MMVD Motion vector rounding for SCC, H. Jang, J. Nam, N. Park, J. Lim, S. Kim (LGE)

- JVET-N0466, Non-CE8: The corner case handling regarding mv derivation for Chroma IBC in dual tree structure, H. Jang, J. Nam, N. Park, J. Lim, S. Kim (LGE)

- JVET-N0467, Non-CE8: Experimental result for various size of IBC block with optimized syntax signaling, H. Jang, J. Nam, N. Park, J. Lim, S. Kim (LGE)

- JVET-N0472, Non-CE8: On IBC reference buffer design, J. Xu, L. Zhang, K. Zhang, H. Liu, Y. Wang (Bytedance)

- JVET-N0501, CE8 related: Optimal Scan Order for BDPCM Residues, A. Singh, A. Konda, C. Pujara, R. Gadde, W. Choi, K.P. Choi (Samsung)

- JVET-N0502, Five SCC TGM sequences of both YUV4:4:4 and YUV4:2:0 formats, Tao Lin, Kailun Zhou, Shuhui Wang (Tongji), Liping Zhao (Shaoxing)

- JVET-N0550, CE8-related: Line-based CG Palette Mode, Y.-H. Chao, T. Hsieh, M. Karczewicz (Qualcomm)

TBA