| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0030-v1 |
| --- | --- |

| Title: | CE10: Summary report on combined inter and intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chih-Wei Hsu Martin Winken | Email: | cw.hsu@mediatek.com martin.winken@hhi.fraunhofer.de |
| Source: | CE coordinators |  |  |

_____________________________

- Abstract

A summary of Core Experiment 10 (CE10) on combined inter and intra prediction is reported. One sub CE was created to investigate combined inter and intra prediction techniques on top of VTM-4.0, as follows:

- CE10-1: Combined inter and intra prediction (CIIP) with weight modifications.

There are 2 tests for the sub CE. All tests are evaluated based on the common test conditions defined in JVET-M1010. All tests and crosscheck results are integrated in this report.

Summary of CE Activities

Document list

CE proposals are listed as follows,

| Proposal Document # | Corresponding Tests | Title | Author(s) |
| --- | --- | --- | --- |
| JVET-N0302 | CE10-1.1 | CE10: CIIP with position-independent weights (Test CE10-1.1) | L. Pham Van, G. Van der Auwera, A. K. Ramasubramonian,V. Seregin, M. Karczewicz (Qualcomm) |
| JVET-N0298 | CE10-1.2 | CE10: CIIP using explicit signaling of weights (CE10-1.2) | A. Seixas Dias, G. Kulupana, S. Blasi (BBC) |

CE-related proposals are listed as follows,

| Proposal Document # | Title | Author(s) |
| --- | --- | --- |
| JVET-N0327 | CE10-related: Simplification on combined inter and intra prediction (CIIP) | X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.) |
| JVET-N0395 | CE10-related: Simplification and Improvement of combined intra-inter prediction mode | L. Zhao, X. Li, X. Zhao, S. Liu (Tencent) |
| JVET-N0508 | CE10-related: A combination of JVET-N0302 (CE10.1.1) and JVET-N0327 | L. Pham Van, G. Van der Auwera, A. K. Ramasubramonian, V. Seregin, M. Karczewicz (Qualcomm), X. Xiu, Y.-W. Chen,T.-C. Ma, X. Wang (Kwai Inc.) |

Experimental Conditions

Software

One software code base is used which is the VTM software. The VTM subversion repository can be found at https://vcgit.hhi.fraunhofer.de/JVET-M-CE10/VVCSoftware_VTM. The 4.0 release version of the software is used.

Anchors

The VTM is used as anchor, where VTM cfg is applied. Proposals are evaluated using BD rates, encoder run time, decoder time with Common Test Conditions (CTC).

Summary of results

CE10-1: CIIP with weight modifications

In CE10-1, the goal is to investigate CIIP techniques on top of VTM-4.0 with weight modifications.

The tests and corresponding results are summarized as follows, where current design in VTM-4.0 and the differences between each test and VTM-4.0 are listed

| # | Proposal | Supported intra modes | Weight scheme | Weight sets (wIntra, wInter) |
| --- | --- | --- | --- | --- |
| VTM-4.0 |  | 4: DC, PL, VER, HOR | PL, DC: equal weights | (4, 4) |
|  |  |  | VER, HOR: position dependent weights, four regions | (6, 2), (5,3), (3,5), (2, 6) |
| CE10-1.1 | JVET-N0302 | 1: PL | Implicit weight assignment | 2: (3, 1) 1: (2, 2) 0: (1, 3) |
|  |  |  | (number of intra-coded left and top neighbors) |  |
| CE10-1.2 | JVET-N0298 | 1: PL | Explicit weight signalling | 0: (4, 4) 10: (2, 6) 11: (6, 2) |

The test results for this aspect are summarized as follows,

| # | Proposal | Config. |  |  | VTM |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y | U | V | EncT | DecT |
| CE1-1.1 | JVET-N0302 | RA | -0.05% | -0.02% | 0.02% | 98% | 100% |
|  |  | LB | -0.03% | 0.20% | 0.07% | 98% | 100% |
| CE10-1.2 | JVET-N0298 | RA | -0.04% | 0.04% | 0.05% | 100% | 99% |
|  |  | LB | -0.01% | 0.16% | -0.01% | 101% | 98% |

The simulation data have been crosschecked and crosscheck results are summarized as follows,

In this summary table, questions have been answered by the cross-checker, including

Level: 1 stands for simulation only, 2 for code review and simulation, 3 for independent implementation and simulation

Results: P stands for partial results by spot-checking, F for full results

EncT/DecT: M stands for match in general, G for gap observed

| # | Tester | Cross-checker | Level? (1/2/3) | Any significant inconsistencies between the description and the provided software? | Results? (P/F) | Match? (Y/N) | EncT? (M/G) | DecT? (M/G) | Additional comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CE10-1.1 | Luong Pham Van (Qualcomm) | Alexey Filippov (Huawei) |  |  |  |  |  |  |  |
| CE10-1.2 | A. Seixas Dias (BBC) | Vasily Rufitskiy (Huawei) |  |  |  |  |  |  |  |

Reference

- C.-W. Hsu, M. Winken, “Description of Core Experiment 10: Combined inter and intra prediction,” JVET-M1030, Marrakech, MA, Jan. 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-M1010.