| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0032-v1 |
| --- | --- |

| Title: | CE12: Summary report on Tile Set Boundary Handling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Hendry Robert Skupin Wade Wan | Email: | fnu.hendry@huawei.com robert.skupin@hhi.fraunhofer.de wade.wan@broadcom.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

A summary of Core Experiment 12 (CE12) on Tile Boundary Handling is reported. Two tests were defined in CE12:

- CE12-1: Handling all tile group boundaries as picture boundaries which includes padding to the tile group boundaries and allowing motion vectors to point to samples outside the tile group boundaries in the reference pictures.

- CE12-2: Encoder-side restrictions of tile groups to not use samples or motion vector candidates from outside the tile group.

All tests are evaluated based on the common test conditions defined in JVET-M0870. All tests and crosscheck results are integrated in this report.

# Summary of CE Activities

## Document list

CE proposals for the tests defined in CE 12 [1] are listed as follows,

| Proposal Document # | Corresponding Tests | Title | Author(s) |
| --- | --- | --- | --- |
| JVET-N0109 | CE12-1 | CE12/AHG12: Treating boundaries of independent tile groups as picture boundaries | Hendry, S. Hong, J. Chen, Y.-K. Wang (Huawei) |
| JVET-N0356 | CE12-2 | CE12-2: Encoder-side tile group restriction | R. Skupin, Y. Sanchez, V. George, K. Suehring, T. Schierl (HHI) |

CE-related proposals are listed as follows,

| Proposal Document # | Title | Author(s) |
| --- | --- | --- |
| JVET-N0355 | CE12-related: Normative temporally independent tile group coding | R. Skupin, Y. Sanchez, V. George, K. Suehring, T. Schierl (HHI) |
| JVET-N0402 | CE12-related: MCTS improvement by modifying motion compensation filter coefficients | A. Aminlou, A. Zare, M. M. Hannuksela (Nokia) |
| JVET-N0403 | CE12-related: MCTS improvement by modifying prediction block | A. Aminlou, A. Zare, M. M. Hannuksela (Nokia) |

# Experimental Conditions

## Software

For CE12-1, VTM 4.0.1 was used while for CE12-2, an additional bug fixed was necessary (Bug#218) resulting in code for MR#369, commit be55eee8bd3d12bc23016a3eea738ea747fadb84 being used.

## Anchors

VTM 4.0.1 is used as anchor. Proposals are evaluated using BD rates, encoder run time, decoder time,

## CE Test Conditions

As described in JVET-M0870 [2], the 360° video test sequences SkateboardInLot, ChairLift, KiteFlite, Harbor, Trolley and GasLamp are used in CMP format with a face size of 1536x1536 and a total resolution of 4608x3072 equivalent to 6k.

Two tiling setups were investigated, namely 6x4 and 12x8 uniformly sized tiles, which results in tile sizes of 768x768 (i.e. 24 tiles) or 384x384 (i.e. 96 tiles) respectively.

In order to avoid the implementation burden for CE12-1, tests were run with one encoder per tile group. As ALF and reshaper result in very different operation points when optimized over the whole picture or just a sub-picture, it was agreed amongst participants to disable ALF and reshaper for all tests and anchor. Furthermore, runtimes are reported as cumulative runtimes of sequential en-/decoders.

# Summary of results

## CE12-1: Handling independently decodable tile group boundaries as picture boundaries

In CE12-1, the goal is to investigate the coding efficiency of independently decodable tile groups when tile set boundaries are handled as picture boundaries for motion compensation, i.e. boundary extension.

The test results for this aspect are summarized as follows.

| # | Proposal | Config. |  |  | VTM |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y | U | V | EncT* | DecT* |
| CE12-1 | JVET-N0109 | RA 6x4 Tiles | 2.64% | 3.24% | 3.35% | 131% | 63% |
|  |  | RA 12x8 Tiles | 10.00% | 11.90% | 12.34% | 124% | 63% |

*The runtimes for CE12-1 may not be accurate since it is the sum of runtimes multiple encoders and decoders so there are overhead of initializing encoders and decoders.

## CE12-2: Encoder-side tile group restriction

In CE12-2, the goal is to investigate the coding efficiency of independently decodable tile groups when encoder-side restrictions are employed in motion compensation.

The test results for this aspect are summarized as follows,

| # | Proposal | Config. |  |  | VTM |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y | U | V | EncT | DecT |
| CE12-2 | JVET-N0356 | RA 6x4 Tiles | 6.77% | 7.04% | 7.28% | 110% | 61% |
|  |  | RA 12x8 Tiles | 16.46% | 18.24% | 18.86% | 102% | 61% |

# Cross checks

The simulation data have been crosschecked and crosscheck results are summarized as follows,

In this summary table, questions have been answered by the cross-checker, including

Level: 1 stands for simulation only, 2 for code review and simulation, 3 for independent implementation and simulation

Results: P stands for partial results by spot-checking, F for full results

EncT/DecT: M stands for match in general, G for gap observed

| # | Tester | Cross-checker | Level? (1/2/3) | Any significant inconsistencies between the description and the provided software? | Results? (P/F) | Match? (Y/N) | EncT? (M/G) | DecT? (M/G) | Additional comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CE12-1 | Hendry (Huawei) | Robert Skupin (HHI) | 2/3 | No | P | Y |  |  |  |
| CE12-2 | Robert Skupin (HHI) | Hendry (Huawei) | 2/3 | No | P | Y |  |  |  |

# Summary of CE-related proposals

## JVET-N0355 CE12-related: Normative temporally independent tile group coding

The contribution proposes normative changes which include that when MVs point to disallowed sample positions near or beyond the boundaries of temporally independent tile groups, the following is carried out:

- MV clipping

- MV sub-pel component rounding

- Sub-block division

- MV predictor clipping and/or rounding for AMVP

- Disabling BDOF and DMVR next to boundaries of temporally independent tile groups

TMVP and ATMVP handle boundaries of temporally independent tile groups similar to picture boundaries.

## JVET-N0402 CE12-related: MCTS improvement by modifying motion compensation filter coefficients

This contribution proposes modifying motion compensation filter coefficients according to the position of the current tile, block, and length of motion vector. It is asserted that it improves RD performance of the MCTS-coded video. For each block, based on the number of samples needed from other tiles, some of the coefficients of the motion compensation filter are changed to zero, and their original values are added to the first coefficient next to those coefficients. It is asserted that the proposed method removes the need to use samples in the other tiles in derivation of the samples of the prediction block.

## JVET-N0403 CE12-related: MCTS improvement by modifying prediction block

This contribution proposes modifying prediction block according to the position of the current tile, block, and length of motion vector. It is asserted that it improves RD performance of the MCTS coded video. For each block, the samples of the prediction block which are affected by samples from the other tiles are replaced with other samples of that prediction block. It is asserted that the proposed method removes the need to use samples in the other tiles in derivation of the samples of the prediction block

# Reference

- Hendry, R. Skupin, W. Wan, “Description of Core Experiment 12(CE12): Tile Set Boundary Handling,” JVET-M1032, Marrakech, MA, Jan. 2019.

- M. Coban, R. Skupin, “AHG12: Proposed JVET common test conditions and evaluation procedures for MCTS and sub-pictures with boundary padding,” JVET-M0870, Marrakech, MA, Jan. 2019.