| 11Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0033-v1 |
| --- | --- |

| Title: | CE13: Summary Report on Neural Network based Filter for Video Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s)or Contact(s): | Yiming Li Shan Liu Kei Kawamura | Tel: Email: | userlym@whu.edu.cn shanl@tencent.com ki-kawamura@kddi.com |
| Source: | CE13 coordinators |  |  |

Abstract

This contribution provides a summary report of Core Experiment 13 on neural network based filter for video coding. 17 tests have been tested in CE13 in between JVET-M and JVET-N meetings, to study and evaluate technologies related to screen content coding. In this report, coding performance and complexity of these tests are reported and analyzed. Crosschecking results for the performed tests are integrated in this contribution.

- Introduction

As is agreed in Marrakech meeting, the following tests in table 1 are performed in CE13.

Table 1: Summary of tests performed in CE13

| Test # | Description | Document | Tester | Cross checker |
| --- | --- | --- | --- | --- |
| CE13-1.1a | Loop filter chain: DBF + SAO + ALF + NN filter. NN filter follows the description in M0159 | JVET-N0110 | Y.-L. Hsiao (Mediatek) | H. Yin (Intel) |
| CE13-1.1b | Loop filter chain: NN filter only. NN filter follows the description in M0159 | JVET-N0110 | Y.-L. Hsiao (Mediatek) |  |
| CE13-1.2a | Loop filter chain: DBF + SAO + ALF + NN filter. NN filter follows the description in M0566 | JVET-N0480 | H. Yin (Intel) | Y.-L. Hsiao (Mediatek) |
| CE13-1.2b | Loop filter chain: NN filter only. NN filter follows the description in M0566 | JVET-N0480 | H. Yin (Intel) |  |
| CE13-2.1a | DBF + NN filter + SAO + ALF, NN filter follows the description in M0351 | JVET-N0169 | C. Lin (Hikvision) | Y. Wang (Wuhan Univ.) |
| CE13-2.1b | NN filter only, NN filter follows the description in M0351 | JVET-N0169 | C. Lin (Hikvision) | Y. Wang (Wuhan Univ.) |
| CE13-2.1c | NN filter + ALF, NN filter follows the description in M0351 | JVET-N0169 | C. Lin (Hikvision) | Y. Wang (Wuhan Univ.) |
| CE13-2.2a | DBF + NN filter + SAO + ALF, NN filter follows the description in M0508 | JVET-N0254 | Y. Wang (Wuhan Univ.) | C. Lin (Hikvision) |
| CE13-2.2b | NN filter only, NN filter follows the description in M0508 | JVET-N0254 | Y. Wang (Wuhan Univ.) |  |
| CE13-2.2c | NN filter + ALF, NN filter follows the description in M0508 | JVET-N0254 | Y. Wang (Wuhan Univ.) |  |
| CE13-2.3a | CTC QP for training, CTC QP for testing (the same as CE13-2.2.a) | JVET-N0254 | Y. Wang (Wuhan Univ.) |  |
| CE13-2.3b | CTC QP for training, CTC QP + 2 for testing | JVET-N0254 | Y. Wang (Wuhan Univ.) |  |
| CE13-2.3c | CTC QP for training, CTC QP - 2 for testing | JVET-N0254 | Y. Wang (Wuhan Univ.) |  |
| CE13-2.4a | DBF + NN filter + SAO + ALF, NN filter follows the description in M0510 | JVET-N0513 | Y. Dai (USTC) | Y. Wang (Wuhan Univ.) |
| CE13-2.4b | NN filter only, NN filter follows the description in M0510 | JVET-N0513 | Y. Dai (USTC) |  |
| CE13-2.4c | NN filter + ALF, NN filter follows the description in M0510 | JVET-N0513 | Y. Dai (USTC) |  |
| CE13-2.5a | CTC QP for training, CTC QP for testing(the same as CE13-2.4.a) | JVET-N0513 | Y. Dai (USTC) |  |
| CE13-2.5b | CTC QP for training, CTC QP + 2 for testing | JVET-N0513 | Y. Dai (USTC) |  |
| CE13-2.5c | CTC QP for training, CTC QP - 2 for testing | JVET-N0513 | Y. Dai (USTC) |  |
| CE13-2.6a | DBF + NN filter + SAO + ALF, NN filter follows the description in M0872 |  |  |  |
| CE13-2.6b | NN filter only, NN filter follows the description in M0872 |  |  |  |
| CE13-2.6c | NN filter + ALF, NN filter follows the description in M0872 |  |  |  |
| CE13-2.7a | NN filter adaptive on/off in CTU level |  |  |  |

- Description of CE tools

The detailed description of each tools is provided in this section.

  - CE13-1.1 (JVET-N0110)

The proposed CNNLF is applied after the adaptive loop filter (ALF). A neural network of three layers is used and shared by luma and chroma. As shown in Figure 1, the parameters of a layer, MxNxPxQ, are explained as follows.

- M: horizontal spatial filter size in one channel
- N: vertical spatial filter size in one channel
- P: number of input channels
- Q: number of filters in one layer

The operations in each layer include convolution, batch normalization, adding bias, and a rectified linear unit (ReLU) per filter. An exception is for the last layer, where no ReLU is applied.

Figure 1. The 3-layer CNN shared by luma and chroma

In this approach, each coding tree unit (CTU) is independently processed by the CNNLF. In the Packing stage, the six input channels of the CNN are generated as four luma blocks of reconstructed luma samples and two chroma blocks of reconstructed chroma samples. The four luma blocks are generated by 2x2 subsampling of the luma CTB four times with different phases to form four quarter-size luma blocks. Each quarter-size luma block is as large as each of the two chroma blocks and is treated as one input channel, as shown in Figure x. In case of a CTU of 128x128 luma samples and 64x64x2 chroma samples, the output size of the Packing stage is 64x64x6. After three layers of CNN, the size is still 64x64x6. In the Unpacking stage, the first four channels are combined for the luma CTB, and the last two channels correspond to the two chroma CTBs. Finally, the CNNLF output samples are calculated by adding the ALF output samples and the Unpacking output samples.

Figure 2. Generation of the six channels for the inputs of the first CNN layer

The parameters of CNN are signaled in the I-picture of each RAS of roughly 1 sec. The parameters are trained using only pictures of temporal levels 0 and 1 in the RAS. To improve coding efficiency, when the encoder decides not to apply CNNLF for the I-picture, the CNN parameters are not signalled, and the remaining pictures in this RAS are all set as CNNLF-off.

  - CE13-1.2 (JVET-N0480)

The proposed ACNNLF is applied after the adaptive loop filter (ALF). The tested ACNNLF has the following properties:

- Two layer CNN, as shown in the Figure 3;

- Up to 3 ACNNLFs for luma and 3 ACNNLFs for chroma adaptively trained with video data;

- The encoder selects the best ACNNLF for each of luma and chroma in each CTB from the trained set of ACNNLFs, to perform encoding. The weights of the trained set of ACNNLFs are signaled in the tile group header and the index of selected ACNNLF is signaled in the coded stream in CTU level;

The decoder receives the ACNNLF weights (up to 3 sets for luma and 3 set for chroma) and the CTU level selected ACNNLF and uses the selected ACNNLF to perform the filtering process for each luma and chroma block during frame reconstruction.

Figure 3. ACNNLF for luma

The parameters of ACNNLF are signaled in the tile group header. In RA configuration, only the I-slice contains ACNNLF parameters of this RAS. The on/off control is applied at the tile group header level for luma and chroma components, and the acnnlf_luma_slice_enable_flag and acnnlf_chroma_slice_enable_flag syntax elements are used to indicate ACNNLF enable/disable. CTB level information indicates which ACNNLF is selected or if there is no ACNNLF applied in the CTB.

  - CE13-2.1 (JVET-N0169)

The proposed CNNF works as a loop filter for intra frames. In different subtests, CNNF is put in different positions. Figure 4a shows the CNNF is adopted as an additional filter before SAO. Figure 4b shows the CNNF to replace all the conventional filters in VTM 4.0. Figure 4c shows the jointed filters of CNNF and ALF to further improve the coding gains.

Figure 4. Comparison of intra decoding scheme between different positions of CNN filter

The architectures of the proposed CNNF in different subtests are the same, which composed by one concat layer, 7 convolution layers with each followed by a ReLU layer, one convolution layer and one summation layer. These layers are connected one by one and form a network, as shown in Figure 5. For a given network, the layer parameters are automatically learned from thousands of training data by minimizing a target loss function in the training process. Once finished, the parameters are fixed and is used as filter coefficients in CNNF. The proposed CNNF includes two inputs: reconstruction and quantization parameter (QP), which makes it possible to use a single set of parameters to adapt to reconstructions with different qualities. Both the two inputs are normalized for better convergence in training process.

Figure 5. CNNF network

  - CE13-2.2&CE13-2.3 (JVET-N0254)

In test CE13-2.2, the impact of the position of the DRNLF in the filter chain is investigated, where 3 kinds of filter chain are tested: (a) DBF + DRNLF + SAO + ALF; (b) DRNLF only; and (c) DRNLF +ALF.

Test CE13-2.3 investigates the generalization capability of DRNLF when encoding QP is different from the training QP. It includes 3 kinds of QP settings: (a) CTC QP for training, CTC QP for testing; (b) CTC QP for training, CTC QP + 2 for testing; (c) CTC QP for training, CTC QP - 2 for testing.

The network structure is called dense residual convolutional neural network based in-loop filter (DRNLF), as shown in Figure 6. where N and M denotes the number of dense residual unit (DRU) and convolution kernel, respectively. Specifically, N is set to 4 and M is set to 32 for the tradeoff between computational efficiency and performance. Normalized QP map is concatenated with the reconstructed frame as the input to DRN.

Figure 6. Network structure of DRNLF.

The filter is applied for both intra and inter pictures. Additional flag is signaled for each CTUs to indicate the on/off of the DRNLF.

  - CE13-2.4&CE13-2.5 (JVET-N0513)

Residue learning technique is integrated into the network as Fig. 6 shows. To reduce memory usage, parameters of several layers are shared, which means we can achieve the performance of deeper network with fewer parameters. As shown in Fig.6, layers of the same color represent the same parameters. The proposed CNN-based filter is located between DF and SAO. In order to investigate the impact of NN filter position in the filter chain, the performance of various combination with other existing loop filter will also be tested.

Figure 7. The structure of the first proposed network

  - CE13-2.6& CE13-2.7 (?)

TBA

- CE results summary

It is agreed that VTM version VTM-4.0 shall be used for these experiments, and the test conditions of mandatory test follows the CTC specified in the JVET-L1010 document. For optional test, only the first intra period should be tested.

Specifically, this CE shall use the following these test conditions:

- Test Condition1: Common Test Conditions (CTC).

- Test Condition2: CTC, but short test, which only need to test the first intra period.

- Test Condition3: Based on Test Condition2, test QP=CTC QP +/- 2

The followings are summary tables of the tests in this CE.

Table 1: CE13 test results with VTM-4.0+ Test Condtion1 (CTC)

|  |  | AI Over VTM-4.0 |  |  |  |  | RA Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CTC Full test | CE13-1.1a |  |  |  |  |  | -1.36% | -14.96% | -14.91% | 100% | 142% |
|  | CE13-1.2a |  |  |  |  |  | -0.31% | -7.17% | -7.35% | 102% | 126% |
|  | CE13-2.1a | -3.48% | -5.18% | -6.77% | 142% | 38414% |  |  |  |  |  |
|  | CE13-2.1b | -4.14% | -5.49% | -6.70% | 140% | 38411% |  |  |  |  |  |
|  | CE13-2.1c | -4.65% | -6.73% | -7.92% | 139% | 37956% |  |  |  |  |  |
|  | CE13-2.2a | -1.52% | -2.12% | -2.73% | 107% | 4667% | -1.45% | -4.37% | -4.27% | 106% | 7156% |
|  | CE13-2.4a | -0.87% | -0.44% | -0.56% | 106% | 1912% | -0.49% | -0.23% | -0.33% | 124% | 468% |

Table 2: CE13 test results with VTM-4.0+Test Condtion2

|  |  | AI Over VTM-4.0 |  |  |  |  | RA Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Test Condition #2 | CE13-2.2a |  |  |  |  |  | -1.62% | -4.51% | -4.42% | 106% | 7559% |
|  | CE13-2.2b |  |  |  |  |  | 1.51% | 1.21% | 1.80% | 105% | 9910% |
|  | CE13-2.2c |  |  |  |  |  | -0.65% | -2.18% | -1.60% | 105% | 9937% |
|  | CE13-2.4a |  |  |  |  |  | -0.53% | -0.23% | -0.43% | 90% | 349% |
|  | CE13-2.4b |  |  |  |  |  | 5.73% | 6.09% | 6.07% | 95% | 610% |
|  | CE13-2.4c |  |  |  |  |  | 1.14% | 2.52% | 2.65% | 90% | 497% |

Table 3: CE13 test results with VTM-4.0+Test Condtion3

|  |  | AI Over VTM-4.0 |  |  |  |  | RA Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Test Condition #3 | CE13-2.3a |  |  |  |  |  | -1.62% | -4.51% | -4.42% | 106% | 7559% |
|  | CE13-2.3b |  |  |  |  |  | -1.88% | -4.81% | -4.33% | 107% | 8008% |
|  | CE13-2.3c |  |  |  |  |  | -1.37% | -4.57% | -4.04% | 105% | 6794% |
|  | CE13-2.5a |  |  |  |  |  | -0.53% | -0.23% | -0.43% | 90% | 349% |
|  | CE13-2.5b |  |  |  |  |  | -0.52% | -0.31% | -0.48% | 98% | 378% |
|  | CE13-2.5c |  |  |  |  |  | -0.46% | -0.33% | -0.40% | 113% | 422% |

- Network analysis

For the understanding of network, e.g., complexity, the following information is provided.

Table 4: CE13 test results with complexity analysis

| Information in the Inference Stage Combining with Codec |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Network Details |  |  |  |  |  |  |
|  | Total Conv. Layers | Total FC Layers | Framework | Param. Num | Param. Precision | Mem.P(MB) | Mem.T(MB)(e.g., 4K input) |
| CE13-1.1 | 3 | 0 | NA | 1506 | 6-bit/32-bit (I) | 0.001282 | 0.344 (128x128, CTU based) |
| CE13-1.2 | 2 | 0 | NA | 692x3 (Luma) 402x3 (Chroma) | 6-bit/32-bit (I) | 0.0028 | 0.0448(128x128, CTU based) |
| CE13-2.1 |  |  |  |  |  |  |  |
| CE13-2.2 | 21 |  | PyTorch | 22371 | 32-bit (F) | 0.09 | 207.21(128x128, CTU based) |
| CE13-2.4 | 10 | 0 | Tensorflow | 7521 | 32-bit (F) | 0.029 | 10.06(128*128,CTU based) |

- Crosschecker reports

Crosscheckers reported that the software/results in CE13 are generally in good shape, no outstanding issues were identified. Some differences were found, which should be due to the parallel and sequential related bugs in VTM-4.0.

Table 5: crosscheck reports that are associated with the tests in this CE.

| # | Tester | Cross-checker | Software review/studied? (Y/N) | Any significant inconsistencies between the description and the provided software? (Y/N) |
| --- | --- | --- | --- | --- |
| CE13-1.1a | Y.-L. Hsiao | R. Yang |  |  |
|  | (Mediatek) | (Intel) |  |  |
| CE13-1.2a | H. Yin | Y.-L. Hsiao |  |  |
|  | (Intel) | (Mediatek) |  |  |
| CE13-2.1a | C. Lin | Y. Wang | N | N |
|  | (Hikvision) | (Wuhan Univ.) |  |  |
| CE13-2.1b | C. Lin | Y. Wang | N | N |
|  | (Hikvision) | (Wuhan Univ.) |  |  |
| CE13-2.1c | C. Lin | Y. Wang | N | N |
|  | (Hikvision) | (Wuhan Univ.) |  |  |
| CE13-2.2a | Y. Wang | C. Lin | N | N |
|  | (Wuhan Univ.) | (Hikvision) |  |  |
| CE13-2.4a | Y. Dai | Y. Wang | N | N |
|  | (USTC) | (Wuhan Univ.) |  |  |

| # | Results available (Full/Partial) | Resutls matched? (Y/N) | Runtime behavior matched? (Y/N) | Additional comments |
| --- | --- | --- | --- | --- |
| CE13-1.1a |  |  |  |  |
| CE13-1.2a |  |  |  |  |
| CE13-2.1a | Full | Y | N |  |
| CE13-2.1b | Partial | Y | N | only test AI configuration |
| CE13-2.1c | Partial | Y | N | only test AI configuration |
| CE13-2.2a | Full | Y | N | 1. minor different results by bug related to ticket #197. 2. Overall decoding time for AI configuration in crosscheck is 3338%, which is faster than reported 4667%. |
| CE13-2.4a | Full | Y | Y | Minor different results caused by system inconsistencies. |

- Input contributions for CE13 and CE13 related topics

CE contributions

- JVET-N0110, CE13-1.1: Convolutional neural network loop filter, Y.-L. Hsiao, O. Chubach, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei(Mediatek)

- JVET-N0169, CE13-2.1: Convolutional Neural Network Filter (CNNF) for Intra Frame, J.Yao, L.Wang(Hikvision)

- JVET-N0254, CE13: Dense Residual Convolutional Neural Network based In-Loop Filter (CE13-2.2 and CE13-2.3)

- JVET-N0480, CE13-1.2: Adaptive convolutional neural network loop filter, H. Yin, R. Yang, X. Fang, S. Ma(Intel)

- JVET-N0513, CE13: Experimental results of CNN-based In-Loop Filter (USTC), Y. Dai, D. Liu, N. Yan, F. Wu(USTC)

CE-related and non-CE contributions

- JVET-N0133, CE13-related: In-loop filter with only CNN-based filter, S. Wan, M.-Z.Wang, H. Gong, C.-Y. Zou(NPU), Y.-Z. Ma, J.-Y. Huo(Xidian Univ.), Y.-F. Yu, Y. Liu(OPPO)