| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0041 |
| --- | --- |

| Title: | Summary of the status of systems-related issues for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Youngkwon Lim Emmanuel Thomas | Email: | young.l@samsung.com emmanuel.thomas@tno.nl |
| Source: | Samsung and TNO |  |  |

_____________________________

# Abstract

This document summarizes the status of systems-related issues as of the outcome of the 13th JVET meeting of January 2019. Its purpose is to provide information educate readers on the current status of these issues. It was prepared by the chairs of the MPEG AHG on Systems technologies for VVC.

# Current status of AhG relevant topics in VVC draft specification

The VVC draft spec text [1] in combination with the  JVET meeting report [2], HLS  BoG report [3]and High-level parallelism and coded picture regions BoG report [4] were used to extract the following information which reflects the adoptions of last meeting. At the time of writing this document HLS has not been updated since the last meeting in the VVC draft spec.

JVET-M0853-v2 adds the support of rectangular tile groups in addition to the existing raster scan ordered tile groups, and enables extraction of MCTSs without changing VLC NAL units.

## CTU definition

“coding tree unit (CTU): A CTB of luma samples, two corresponding CTBs of chroma samples of a picture that has three sample arrays, or a CTB of samples of a monochrome picture or a picture that is coded using three separate colour planes and syntax structures used to code the samples.” [1]

## Tile definition

“tile: A rectangular region of CTUs within a particular tile column and a particular tile row in a picture.” [1]

- Tiles in a tile group need to be in raster-scan order.

## Tile Group

“tile group: An integer number of tiles of a picture in tile raster scan of the picture and that are exclusively contained in a single NAL unit.

tile group header: A part of a coded tile group containing the data elements pertaining to the first or all tiles represented in the tile group.” [1]

With the Marrakesh adoptions a tile group can now also be a group of tiles in a rectangular shape, and is no longer restricted to raster scan order. Tile group information is either in the PPS or in the Tile Group Header. Below are some relevant points regarding the Marrakesh adoptions.

- Rectangular tile groups are indicated by the top left and bottom right tile index. So an image is first divided and now has the option between raster scan order or rectangular tile groups.  The rectangular groups indicate the top left and bottom right tile.

- Tile groups need to be in increasing address order (see below: dark orange, then light blue, then pale yellow, then dark blue, then light orange, …)

- For each tile group an ID can be derived either implicitly or explicitly, the graph below reflects the syntax and the flags.

- The tile group shapes shall be such that each tile, when decoded, shall have its entire left boundary and entire top boundary consisting of a picture boundary or previously decoded tile(s). In other words, the light yellow and light green tiles in the figure below are not allowed.

The tile group ID is signaled in the tile group header. But the usage of the tile group ID remains to be clarified as no agreed syntax is referring to it, as a mean to provide additional information/feature.

## Parameter Set Structure

The tile syntax contained in the picture parameter set is as follows. The tile header syntax is in the annex – unchanged still.

|  | pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- | --- |
|  | pps_pic_parameter_set_id | ue(v) |
|  | pps_seq_parameter_set_id | ue(v) |
|  | transform_skip_enabled_flag | u(1) |
| Tile Group Info | single_tile_in_pic_flag | u(1) |
|  | if( !single_tile_in_pic_flag ) { |  |
|  | num_tile_columns_minus1 | ue(v) |
|  | num_tile_rows_minus1 | ue(v) |
|  | } |  |
|  | if( !single_tile_in_pic_flag ) { |  |
|  | uniform_tile_spacing_flag | u(1) |
|  | if( !uniform_tile_spacing_flag ) { |  |
|  | for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
|  | tile_column_width_minus1[ i ] | ue(v) |
|  | for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
|  | tile_row_height_minus1[ i ] | ue(v) |
|  | } |  |
|  | single_tile_per_tile_group_flag | u(1) |
|  | if(!single_tile_per_tile_group_flag ) |  |
|  | rect_tile_group_flag | u(1) |
|  | if( rect_tile_group_flag && !single_tile_per_tile_group_flag ) { |  |
|  | num_tile_groups_in_pic_minus1 | ue(v) |
|  | for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) { |  |
|  | if( i > 0) |  |
|  | top_left_tile_idx[ i ] | u(v) |
|  | bottom_right_tile_idx[ i ] | u(v) |
|  | } |  |
|  | } |  |
|  | loop_filter_across_tiles_enabled_flag | u(1) |
|  | } |  |
|  | if( rect_tile_group_flag ) { |  |
|  | signalled_tile_group_id_flag | u(1) |
|  | if( signalled_tile_group_id_flag ) { |  |
|  | signalled_tile_group_id_length_minus1 | ue(v) |
|  | for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) |  |
|  | tile_group_id[ i ] | u(v) |
|  | } |  |
|  | rbsp_trailing_bits( ) |  |
|  | } |  |

Semantics for new / modified elements are shown below.

single_tile_per_tile_group equal to 1 specifies that each tile group that refers to this PPS includes one tile. single_tile_per_tile_group equal to 0 specifies that a tile group that refers to this PPS may include more than one tile.

rect_tile_group_flag equal to 0 specifies that tiles within each tile group are in raster scan order and the tile group information is not signalled in PPS. rect_tile_group_flag equal to 1 specifies that tiles within each tile group cover a rectangular region of the picture and the tile group information is signalled in the PPS. When single_tile_per_tile_group_flag is equal to 1 rect_tile_group_flag is inferred to be equal to 1.

num_tile_groups_in_pic_minus1 plus 1 specifies the number of tile groups in each picture referring to the PPS. The value of num_tile_groups_in_pic_minus1 shall be in the range of 0 to (NumTilesInPic − 1), inclusive. When not present and single_tile_per_tile_group_fla is equal to 1, the value of  num_tile_groups_in_pic_minus1 is inferred to be equal to (NumTilesInPic – 1)tile_grou_id.

top_left_tile_idx[ i ] specifies the tile index of the tile located at the top-left corner of the i-th tile group. The value of top_left_tile_idx[ i ] shall not be equal to the value of top_left_tile_idx[ j ] for any i not equal to j. When not present, top_left_tile_idx[ i ] is inferred to be equal to i. The length of the top_left_tile_idx[ i ] syntax element is Ceil( Log2(  NumTilesInPic ) bits.

bottom_right_tile_idx[ i ] specifies the tile index of the tile located at the bottom-right corner of the i-th tile group. When single_tile_per_tile_group_flag is equal to 1 bottom_right_tile_idx[ i ] is inferred to be equal to top_left_tile_idx[ i ]. The length of the bottom_right_tile_idx[ i ] syntax element is Ceil( Log2( NumTilesInPic) ) bits.

It is a requirement of bitstream conformance that any particular tile shall only be included in one tile group.

The variable NumTilesInTileGroup[ i ], which specifies the number of tiles in the tile group, and related variables, are derived as follows:

deltaTileIdx[ i ]  = bottom_right_tile_idx[ i ] – top_right_tile_idx[ i ]
NumTileRowsInTileGroupMinus1[ i ]  = ( deltaTileIdx[ i ] / ( num_tile_columns_minus1 + 1 ) )  
NumTileColumnsInTileGroupMinus1[ i ]   = ( deltaTileIdx[ i ] % ( num_tile_columns_minus1 + 1 ) )  
NumTilesInTileGroup[ i ]  = ( NumTileRowsInTileGroupMinus1[ i ] + 1 ) * 
	( NumTileColumnsInTileGroupMinus1[ i ] + 1 )

signalled_tile_group_id_flag equal to 1 specifies that the tile group ID for each tile group is signalled. signalled_tile_group_index_flag equal to 0 specifies that tile group IDs are not signalled. When rect_tile_group_flag is equal to 0, the value of signalled_tile_group_index_flag is inferred to be equal to 0.

signalled_tile_group_id_length_minus1 plus 1 specifies the number of bits used to represent the syntax element tile_group_id[ i ] when present, and the syntax element tile_group_address in tile group headers. The value of signalled_tile_group_index_length_minus1 shall be in the range of 0 to 15, inclusive. When not present, the value of signalled_tile_group_index_length_minus1 is inferred to be equal to Ceil( Log2( num_tile_groups_in_pic_minus1 + 1 ) ) − 1.

tile_group_id[ i ] specifies the tile group ID of the i-th tile group. The length of the tile_group_id[ i ] syntax element is tile_set_id_length_minus1 + 1 bits.  When not present, tile_group_id[ i ] is inferred to be equal to i, for each i in the range of 0 to num_tile_groups_in_pic_minus1, inclusive.

# Relevant Topics of Discussion

The relevant discussion resumes from the BoG report and JVET meeting report are below, with the questions/topics on the left and comments on the right.

## AHG 12: High-level parallelism and coded picture regions

| The latest HEVC text specifies, as part of the semantics of an SEI message, the MCTS sub-bitstream extraction process and requires the extracted sub-bitstream to be conforming bitstream. For VVC, should we do more than this way of defining conformance for MCTSs? | - Should normative decoding process be specified for decoding of MCTSs instead of just specifying SEI messages for indication of the encoder motion constraints? - Whether MCTS sequences are signalled in parameter sets or whether an MCTS sequence would have its own associated parameter sets. - Whether to provide a level definition at the MCTS sequence level. Whether to define a normative decoder interface and conforming decoding behaviour for MCTS-specific decoding. - Further study is highly encouraged. |
| --- | --- |
| To specify conformance (level, HRD) or not for MCTS / sub-picture sequence? | - It helps when extracting a 360 video viewport and decode just that. - It helps in merging pieces into one conforming bitstream. |
| On whether to (allow) treat boundaries of MCTS / sub-picture sequences as picture boundaries (i.e., to do padding). | - Allowing this is expected to help in coding efficiency - Burden for hardware implementations - Tests showing coding efficiency gain numbers and an analysis on hardware implementation complexity are needed for making a decision on this - Test conditions were included in JVET-M0870 - The BoG agreed to allow for more than one tile raster scan ordered tile group in one MCTS / sub-picture, but if the tile grouping mode is rectangular tile group, it is required that each MCTS / sub-picture contains only one tile group. - The BoG agreed that, if the signalling is not going to affect the decoding process, it does not make much difference between putting the information in SEI message(s) and VUI, otherwise, the information should be included in a non-VUI part of a parameter set. - Specifying conformance (level, HRD) or not for MCTS / sub-picture sequence helps when extracting a 360 video viewport and decoding just that. Even more importantly, it helps in merging pieces into one conforming bitstream. The BoG agreed that this would be a useful thing to do. |
| To keep tile grouping with tiles within a tile group in tile raster scan order? | - Yes,3 reasons: - The first is to have a fall back, in case the rectangular tile grouping gets not easily usable. - The second is to provide a possibility to use only this in a profile where rectangular tile grouping is not necessary. - The third reason is that this seems better for ultralow delay applications. |
| How to signal tile group address in the tile group header? | - For rectangular tile grouping, have one flag in the PPS to determine whether there is a mapping in the PPS between tile group indices and tile group IDs, and signal the tile group ID in the tile group header. If the PPS flag is equal to 0, tile group ID in the tile group header is tile group index |
| Flexible tiling for ROI and other rectangular region based picture access (e.g., 360 video) | - Use cases for flexible tiling: 360 video viewport-dependent delivery optimization, multi-party video conferencing, load balancing for 360 projected video, V-PCC patching where each patch is probably good to be one tile. - The first two are based on merging multiple pieces into one bitstream for decoding by a single decoder. - If flexible tiling is not supported at all, VVC would have less capability than HEVC for 360 video viewport-dependent delivery optimization etc. due to that in VVC it is not allowed for one tile to be split into multiple VCL NAL units. |
| To allow tiling with tile size unit less than CTU size? | - This design also cause issues with line buffer, as well as in VDPU rate - It was commented that a benefit of the design is enable turning off loop filtering across CMP faces. - It was suggested that enabling loop filtering turning off within a tile would be a lighter solution than having this design. It was commented that there is ongoing CE on this. |

## AHG 17 High-level syntax

| Consider removing concept of I, P, and B slice (tile group) types | - Whether this signaling should be for pictures or for tile groups - it may not be necessary to indicate at the tile group level whether CPR is used in the tile or not. (Note that a tile has no header.) - Proposed to no longer have I, P, B slice types, but to have flags to control the types of prediction (intra prediction, current picture referencing, inter prediction with one list or two lists) - No need for action on this was identified. |
| --- | --- |
| Tile group header parameters in higher level structures | - A concern was raised about out-of-band transmission of PPS if referring to APS_id in the PPS, since APS parameter are expected to change frequently. - For an extracted MCTS sub-bitstream, would need to also include the picture header. - Provides ability to override in the tile group header parameters that were present in the PPS. |
| Coordination with MPEG Systems | - MPEG contribution M46504 proposes greater coordination between JVET and MPEG Systems on HLS topics. Having pre-determined scheduled meeting days/times to cover HLS topics is seen as desirable by the BoG. The scope of which HLS topics MPEG Systems wishes to include in joint discussions with MPEG was unclear. |

# Current JVET AHGs relevant towards next meeting

| High-level parallelism and coded picture regions (AHG12) (jvet@lists.rwth-aachen.de) - Study wavefront processing including the relationship with tiles and low delay characteristics. - Study flexible loop filter control and tile size restriction, including identifying implications on coding tools and implementation. - Study flexible tile partitioning (e.g. more flexible than HEVC and tile boundaries not spanning a full picture). - Study support of independently coded picture regions, including easy rewriting of such regions into a conforming sub-bitstream. - Prepare software and configurations for the test model to facilitate parallel processing tests. - Study the coding efficiency impact of parallel processing and coded picture regions. | S. Deshpande (chair), M. M. Hannuksela, R. Sjöberg, R. Skupin, W. Wan, Y.-K. Wang S. Wenger (vice-chairs) |
| --- | --- |
| High-level syntax (AHG17) (jvet@lists.rwth-aachen.de) - Study NAL unit header, sequence parameter set, picture parameter set, adaptation parameter set, and tile group header syntax designs - Study the proposed picture header designs and alternatives - Study reference picture buffering and list construction - Study random access signalling and random access approaches, including approaches with reference pictures provided by external means - Assist in software development and text drafting for the high-level syntax in the VVC design. | R. Sjöberg (chair), S. Deshpande, M. M. Hannuksela, R. Skupin, Y.-K. Wang, S. Wenger, H. Yu (vice-chairs) |
| Layered coding and resolution adaptivity (AHG19) (jvet@lists.rwth-aachen.de) - Study adaptive-resolution coding approaches for real-time communication, adaptive streaming, and 360-degree viewport-dependent streaming, including filters for resampling, reference picture management, and related scope and signalling - Study approaches for temporal scalability to avoid temporal judder when temporal scalability sub-bitstream extraction is used for achieving lower frame rate, and consider whether this should have a normative impact. - Identify related test conditions, test sequences, and evaluation techniques (including subjective assessment techniques) - Study potential approaches for support of layered coding scalability including spatial, temporal, quality, and view scalability | S. Wenger and A. Segall (co-chairs), M. M. Hannuksela, Hendry, S. McCarthy, Y.-C. Sun (vice-chairs) |

# References

| [1] | „JVET-M1001 Versatile Video Coding (Draft 4)”. |
| --- | --- |
| [2] | „JVET-M_Notes_dB Meeting Report of the 13th Meeting of the Joint Video Experts Team (JVET),”. |
| [3] | „JVET-M0816-v3 BoG report on high level syntax”. |
| [4] | „JVET-M0782-v3 Report of BoG on tiles and WPP”. |

# Annex

Below is the tile group header from draft Specification [1], which will need to be updated to reflect the adoptions in the last meeting.

      - 7.3.3.1 General tile group header syntax

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |
| if( NumTilesInPic > 1 ) { |  |
| tile_group_address | u(v) |
| num_tiles_in_tile_group_minus1 | ue(v) |
| } |  |
| tile_group_type | ue(v) |
| tile_group_pic_order_cnt_lsb | u(v) |
| if( partition_constraints_override_enabled_flag ) { |  |
| partition_constraints_override_flag | ue(v) |
| if( partition_constraints_override_flag ) { |  |
| tile_decodiggroup_log2_diff_min_qt_min_cb_luma | ue(v) |
| tile_group_max_mtt_hierarchy_depth_luma | ue(v) |
| if( tile_group_max_mtt_hierarchy_depth_luma != 0 ) |  |
| tile_group_log2_diff_max_bt_min_qt_luma | ue(v) |
| tile_group_log2_diff_max_tt_min_qt_luma | ue(v) |
| } |  |
| if( tile_group_type = = I && qtbtt_dual_tree_intra_flag ) { |  |
| tile_group_log2_diff_min_qt_min_cb_chroma | ue(v) |
| tile_group_max_mtt_hierarchy_depth_chroma | ue(v) |
| if( tile_group_max_mtt_hierarchy_depth_chroma != 0 ) |  |
| tile_group_log2_diff_max_bt_min_qt_chroma | ue(v) |
| tile_group_log2_diff_max_tt_min_qt_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if ( tile_group_type != I ) { |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| tile_group_temporal_mvp_enabled_flag | u(1) |
| if( tile_group_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( tile_group_temporal_mvp_enabled_flag ) { |  |
| if( tile_group_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| } |  |
| six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enable_flag ) |  |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| } |  |
| tile_group_qp_delta | se(v) |
| if( pps_tile_group_chroma_qp_offsets_present_flag ) { |  |
| tile_group_cb_qp_offset | se(v) |
| tile_group_cr_qp_offset | se(v) |
| } |  |
| if( sps_sao_enabled_flag ) { |  |
| tile_group_sao_luma_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| tile_group_sao_chroma_flag | u(1) |
| } |  |
| if( sps_alf_enabled_flag ) { |  |
| tile_group_alf_enabled_flag | u(1) |
| if( tile_group_alf_enabled_flag ) |  |
| alf_data( ) |  |
| } |  |
| if( tile_group_type = = P ¦ ¦ tile_group_type = = B ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( tile_group_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| dep_quant_enabled_flag | u(1) |
| if( !dep_quant_enabled_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| tile_group_deblocking_filter_disabled_flag | u(1) |
| if( !tile_group_deblocking_filter_disabled_flag ) { |  |
| tile_group_beta_offset_div2 | se(v) |
| tile_group_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| if( num_tiles_in_tile_group_minus1 > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_tiles_in_tile_group_minus1; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| byte_alignment( ) |  |
| } |  |

      - 7.4.4.1 General tile group header semantics

When present, the value of the tile group header syntax element tile_group_pic_parameter_set_id and tile_group_pic_order_cnt_lsb shall be the same in all tile group headers of a coded picture.

tile_group_pic_parameter_set_id specifies the value of pps_pic_parameter_set_id for the PPS in use. The value of tile_group_pic_parameter_set_id shall be in the range of 0 to 63, inclusive.

It is a requirement of bitstream conformance that the value of TemporalId of the current picture shall be greater than or equal to the value of TemporalId of the PPS that has pps_pic_parameter_set_id equal to tile_group_pic_parameter_set_id.

tile_group_address specifies the tile address of the first tile in the tile group, where tile address is the tile ID as specified by Equation 6-7. The length of tile_group_address is Ceil( Log2 ( NumTilesInPic ) ) bits. The value of tile_group_address shall be in the range of 0 to NumTilesInPic − 1, inclusive, and the value of tile_group_address shall not be equal to the value of tile_group_address of any other coded tile group NAL unit of the same coded picture. When tile_group_address is not present it is inferred to be equal to 0.

num_tiles_in_tile_group_minus1 plus 1 specifies the number of tiles in the tile group. The value of num_tiles_in_tile_group_minus1 shall be in the range of 0 to NumTilesInPic − 1, inclusive. When not present, the value of num_tiles_in_tile_group_minus1 is inferred to be equal to 0.

tile_group_type specifies the coding type of the tile group according to Table 7-2.

Table 7-2 – Name association to tile_group_type

| tile_group_type | Name of tile_group_type |
| --- | --- |
| 0 | B (B tile group) |
| 1 | P (P tile group) |
| 2 | I (I tile group) |

When nal_unit_type is equal to IRAP_NUT, i.e., the picture is an IRAP picture, tile_group_type shall be equal to 2.

tile_group_pic_order_cnt_lsb specifies the picture order count modulo MaxPicOrderCntLsb for the current picture. The length of the tile_group_pic_order_cnt_lsb syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits. The value of the tile_group_pic_order_cnt_lsb shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive.

partition_constraints_override_flag equal to 1 specifies that partition constraint parameters are present in the tile group header. partition_constraints_override_flag equal to 0 specifies that partition constraint parameters are not present in the tile group header. When not present, the value of partition_constraints_override_flag is inferred to be equal to 0.

tile_group_log2_diff_min_qt_min_cb_luma specifies the difference between the base 2 logarithm of the minimum size in luma samples of a luma leaf block resulting from quadtree splitting of a CTU and the base 2 logarithm of the minimum coding block size in luma samples for luma CUs in the current tile group. The value of tile_group_log2_diff_min_qt_min_cb_luma shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY, inclusive. When not present, the value of tile_group_log2_diff_min_qt_min_cb_luma is inferred as follows:

- If tile_group_type equal to 2 (I), the value of tile_group_log2_diff_min_qt_min_cb_luma is inferred to be equal to sps_log2_diff_min_qt_min_cb_intra_tile_group_luma

- Otherwise (tile_group_type equal to 0 (B) or 1 (P)), the value of tile_group_log2_diff_min_qt_min_cb_luma is inferred to be equal to sps_log2_diff_min_qt_min_cb_inter_tile_group.

tile_group_max_mtt_hierarchy_depth_luma specifies the maximum hierarchy depth for coding units resulting from multi-type tree splitting of a quadtree leaf in the current tile group. The value of tile_group_max_mtt_hierarchy_depth_luma shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY, inclusive. When not present, the value of tile_group_max_mtt_hierarchy_depth_luma is inferred as follows:

- If tile_group_type equal to 2 (I), the value of tile_group_max_mtt_hierarchy_depth_luma is inferred to be equal to sps_max_mtt_hierarchy_depth_intra_tile_group_luma

- Otherwise (tile_group_type equal to 0 (B) or 1 (P)), the value of tile_group_max_mtt_hierarchy_depth_luma is inferred to be equal to sps_max_mtt_hierarchy_depth_inter_tile_group.

tile_group_log2_diff_max_bt_min_qt_luma specifies the difference between the base 2 logarithm of the maximum size (width or height) in luma samples of a luma coding block that can be split using a binary split and the minimum size (width or height) in luma samples of a luma leaf block resulting from quadtree splitting of a CTU in the current tile group. The value of tile_group_log2_diff_max_bt_min_qt_luma shall be in the range of 0 to CtbLog2SizeY − MinQtLog2SizeY, inclusive. When not present, the value of tile_group_log2_diff_max_bt_min_qt_luma is inferred as follows:

- If tile_group_type equal to 2 (I), the value of tile_group_log2_diff_max_bt_min_qt_luma is inferred to be equal to sps_log2_diff_max_bt_min_qt_intra_tile_group_luma

- Otherwise (tile_group_type equal to 0 (B) or 1 (P)), the value of tile_group_log2_diff_max_bt_min_qt_luma is inferred to be equal to sps_log2_diff_max_bt_min_qt_inter_tile_group.

tile_group_log2_diff_max_tt_min_qt_luma specifies the difference between the base 2 logarithm of the maximum size (width or height) in luma samples of a luma coding block that can be split using a ternary split and the minimum size (width or height) in luma samples of a luma leaf block resulting from quadtree splitting of a CTU in in the current tile group. The value of tile_group_log2_diff_max_tt_min_qt_luma shall be in the range of 0 to CtbLog2SizeY − MinQtLog2SizeY, inclusive. When not present, the value of tile_group_log2_diff_max_tt_min_qt_luma is inferred as follows:

- If tile_group_type equal to 2 (I), the value of tile_group_log2_diff_max_tt_min_qt_luma is inferred to be equal to sps_log2_diff_max_tt_min_qt_intra_tile_group_luma

- Otherwise (tile_group_type equal to 0 (B) or 1 (P)), the value of tile_group_log2_diff_max_tt_min_qt_luma is inferred to be equal to sps_log2_diff_max_tt_min_qt_inter_tile_group.

tile_group_log2_diff_min_qt_min_cb_chroma specifies the difference between the base 2 logarithm of the minimum size in luma samples of a chroma leaf block resulting from quadtree splitting of a chroma CTU with treeType equal to DUAL_TREE_CHROMA and the base 2 logarithm of the minimum coding block size in luma samples for chroma CUs with treeType equal to DUAL_TREE_CHROMA in the current tile group. The value of tile_group_log2_diff_min_qt_min_cb_chroma shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY, inclusive. When not present, the value of tile_group_log2_diff_min_qt_min_cb_chroma is inferred to be equal to sps_log2_diff_min_qt_min_cb_intra_tile_group_chroma.

tile_group_max_mtt_hierarchy_depth_chroma specifies the maximum hierarchy depth for coding units resulting from multi-type tree splitting of a quadtree leaf with treeType equal to DUAL_TREE_CHROMA in the current tile group. The value of tile_group_max_mtt_hierarchy_depth_chroma shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY, inclusive. When not present, the values of tile_group_max_mtt_hierarchy_depth_chroma is inferred to be equal to sps_max_mtt_hierarchy_depth_intra_tile_groups_chroma.

tile_group_log2_diff_max_bt_min_qt_chroma specifies the difference between the base 2 logarithm of the maximum size (width or height) in luma samples of a chroma coding block that can be split using a binary split and the minimum size (width or height) in luma samples of a chroma leaf block resulting from quadtree splitting of a chroma CTU with treeType equal to DUAL_TREE_CHROMA in the current tile group. The value of tile_group_log2_diff_max_bt_min_qt_chroma shall be in the range of 0 to CtbLog2SizeY − MinQtLog2SizeC, inclusive. When not present, the value of tile_group_log2_diff_max_bt_min_qt_chroma is inferred to be equal to sps_log2_diff_max_bt_min_qt_intra_tile_group_chroma

tile_group_log2_diff_max_tt_min_qt_chroma specifies the difference between the base 2 logarithm of the maximum size (width or height) in luma samples of a chroma coding block that can be split using a ternary split and the minimum size (width or height) in luma samples of a chroma leaf block resulting from quadtree splitting of a chroma CTU with treeType equal to DUAL_TREE_CHROMA in the current tile group. The value of tile_group_log2_diff_max_tt_min_qt_chroma shall be in the range of 0 to CtbLog2SizeY − MinQtLog2SizeC, inclusive. When not present, the value of tile_group_log2_diff_max_tt_min_qt_chroma is inferred to be equal to sps_log2_diff_max_tt_min_qt_intra_tile_group_chroma

The variables MinQtLog2SizeY, MinQtLog2SizeC, MinQtSizeY, MinQtSizeC, MaxBtSizeY, MaxBtSizeC, MinBtSizeY, MaxTtSizeY, MaxTtSizeC, MinTtSizeY, MaxMttDepthY and MaxMttDepthC are derived as follows:

MinQtLog2SizeY  =  MinCbLog2SizeY + tile_group_log2_diff_min_qt_min_cb_luma	(7-34)

MinQtLog2SizeC  =  MinCbLog2SizeY + tile_group_log2_diff_min_qt_min_cb_chroma	(7-35)

MinQtSizeY  =  1  <<  MinQtLog2SizeY		(7-36)

MinQtSizeC  =  1  <<  MinQtLog2SizeC		(7-37)

MaxBtSizeY  =  1  <<  ( MinQtLog2SizeY + tile_group_log2_diff_max_bt_min_qt_luma )	(7-38)

MaxBtSizeC  =  1  <<  ( MinQtLog2SizeC + tile_group_log2_diff_max_bt_min_qt_chroma )	(7-39)

MinBtSizeY  =  1  <<  MinCbLog2SizeY		(7-40)

MaxTtSizeY  =  1  <<  ( MinQtLog2SizeY + tile_group_log2_diff_max_tt_min_qt_luma )	(7-41)

MaxTtSizeC  =  1  <<  ( MinQtLog2SizeC + tile_group_log2_diff_max_tt_min_qt_chroma )	(7-42)

MinTtSizeY  =  1  <<  MinCbLog2SizeY		(7-43)

MaxMttDepthY  =  tile_group_max_mtt_hierarchy_depth_luma	(7-44)

MaxMttDepthC  =  tile_group_max_mtt_hierarchy_depth_chroma	(7-45)

tile_group_temporal_mvp_enabled_flag specifies whether temporal motion vector predictors can be used for inter prediction. If tile_group_temporal_mvp_enabled_flag is equal to 0, the syntax elements of the current picture shall be constrained such that no temporal motion vector predictor is used in decoding of the current picture. Otherwise (tile_group_temporal_mvp_enabled_flag is equal to 1), temporal motion vector predictors may be used in decoding of the current picture. When not present, the value of tile_group_temporal_mvp_enabled_flag is inferred to be equal to 0.

mvd_l1_zero_flag equal to 1 indicates that the mvd_coding( x0, y0, 1 ) syntax structure is not parsed and MvdL1[ x0 ][ y0 ][ compIdx ] is set equal to 0 for compIdx = 0..1. mvd_l1_zero_flag equal to 0 indicates that the mvd_coding( x0, y0, 1 ) syntax structure is parsed.

collocated_from_l0_flag equal to 1 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 0. collocated_from_l0_flag equal to 0 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 1. When collocated_from_l0_flag is not present, it is inferred to be equal to 1.

It is a requirement of bitstream conformance that the current decoded picture cannot be the collocated picture.

six_minus_max_num_merge_cand specifies the maximum number of merging motion vector prediction (MVP) candidates supported in the tile group subtracted from 6. The maximum number of merging MVP candidates, MaxNumMergeCand is derived as follows:

MaxNumMergeCand = 6 − six_minus_max_num_merge_cand	(7-46)

The value of MaxNumMergeCand shall be in the range of 1 to 6, inclusive.

five_minus_max_num_subblock_merge_cand specifies the maximum number of subblock-based merging motion vector prediction (MVP) candidates supported in the tile group subtracted from 5. When five_minus_max_num_subblock_merge_cand is not present, it is inferred to be equal to 5 − sps_sbtmvp_enabled_flag. The maximum number of subblock-based merging MVP candidates, MaxNumSubblockMergeCand is derived as follows:

MaxNumSubblockMergeCand = 5 − five_minus_max_num_subblock_merge_cand	(7-47)

The value of MaxNumSubblockMergeCand shall be in the range of 0 to 5, inclusive.

tile_group_qp_delta specifies the initial value of QpY to be used for the coding blocks in the tile group until modified by the value of CuQpDeltaVal in the coding unit layer. The initial value of the QpY quantization parameter for the tile group, TileGroupQpY, is derived as follows:

TileGroupQpY = 26 + init_qp_minus26 + tile_group_qp_delta	(7-48)

The value of TileGroupQpY shall be in the range of −QpBdOffsetY to +63, inclusive.

tile_group_cb_qp_offset specifies a difference to be added to the value of pps_cb_qp_offset when determining the value of the Qp′Cb quantization parameter. The value of tile_group_cb_qp_offset shall be in the range of −12 to +12, inclusive. When tile_group_cb_qp_offset is not present, it is inferred to be equal to 0. The value of pps_cb_qp_offset + tile_group_cb_qp_offset shall be in the range of −12 to +12, inclusive.

tile_group_cr_qp_offset specifies a difference to be added to the value of pps_cr_qp_offset when determining the value of the Qp′Cr quantization parameter. The value of tile_group_cr_qp_offset shall be in the range of −12 to +12, inclusive. When tile_group_cr_qp_offset is not present, it is inferred to be equal to 0. The value of pps_cr_qp_offset + tile_group_cr_qp_offset shall be in the range of −12 to +12, inclusive.

tile_group_sao_luma_flag equal to 1 specifies that SAO is enabled for the luma component in the current tile group; tile_group_sao_luma_flag equal to 0 specifies that SAO is disabled for the luma component in the current tile group. When tile_group_sao_luma_flag is not present, it is inferred to be equal to 0.

tile_group_sao_chroma_flag equal to 1 specifies that SAO is enabled for the chroma component in the current tile group; tile_group_sao_chroma_flag equal to 0 specifies that SAO is disabled for the chroma component in the current tile group. When tile_group_sao_chroma_flag is not present, it is inferred to be equal to 0.

tile_group_alf_enabled_flag equal to 1 specifies that adaptive loop filter is enabled and may be applied to Y, Cb, or Cr colour component in a tile group. tile_group_alf_enabled_flag equal to 0 specifies that adaptive loop filter is disabled for all colour components in a tile group.

num_ref_idx_l0_active_minus1 specifies the maximum reference index for reference picture list 0 that may be used to decode the tile group. num_ref_idx_l0_active_minus1 shall be in the range of 0 to 14, [Ed. (SL): This is copied from HEVC spec, which may be different in VVC. This syntax element and the next were already used in VVC WD (not just for CPR; other parts as well) but were not defined, thus copied here.]  inclusive. When the current tile group is a P or B tile group and num_ref_idx_l0_active_minus1 is not present, num_ref_idx_l0_active_minus1 is inferred to be equal to num_ref_idx_l0_default_active_minus1.

The variable CurrPicIsOnlyRef, specifying that the current decoded picture is the only reference picture for the current tile group, is derived as follows:

CurrPicIsOnlyRef = sps_cpr_enabled_flag && ( tile_group_type  = =  P )  &&  	(7-49)
			( num_ref_idx_l0_active_minus1  = =  0 )

num_ref_idx_l1_active_minus1 specifies the maximum reference index for reference picture list 1 that may be used to decode the tile group. num_ref_idx_l1_active_minus1 shall be in the range of 0 to 14, inclusive. When num_ref_idx_l1_active_minus1 is not present, num_ref_idx_l1_active_minus1 is inferred to be equal to num_ref_idx_l1_default_active_minus1.

dep_quant_enabled_flag equal to 0 specifies that dependent quantization is disabled. dep_quant_enabled_flag equal to 1 specifies that dependent quantization is enabled.

sign_data_hiding_enabled_flag equal to 0 specifies that sign bit hiding is disabled. sign_data_hiding_enabled_flag equal to 1 specifies that sign bit hiding is enabled.  When sign_data_hiding_enabled_flag is not present, it is inferred to be equal to 0.

deblocking_filter_override_flag equal to 1 specifies that deblocking parameters are present in the tile group header. deblocking_filter_override_flag equal to 0 specifies that deblocking parameters are not present in the tile group header. When not present, the value of deblocking_filter_override_flag is inferred to be equal to 0.

tile_group_deblocking_filter_disabled_flag equal to 1 specifies that the operation of the deblocking filter is not applied for the current tile group. tile_group_deblocking_filter_disabled_flag equal to 0 specifies that the operation of the deblocking filter is applied for the current tile group. When tile_group_deblocking_filter_disabled_flag is not present, it is inferred to be equal to pps_deblocking_filter_disabled_flag.

tile_group_beta_offset_div2 and tile_group_tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) for the current tile group. The values of tile_group_beta_offset_div2 and tile_group_tc_offset_div2 shall both be in the range of −6 to 6, inclusive. When not present, the values of tile_group_beta_offset_div2 and tile_group_tc_offset_div2 are inferred to be equal to pps_beta_offset_div2 and pps_tc_offset_div2, respectively.

offset_len_minus1 plus 1 specifies the length, in bits, of the entry_point_offset_minus1[ i ] syntax elements. The value of offset_len_minus1 shall be in the range of 0 to 31, inclusive.

entry_point_offset_minus1[ i ] plus 1 specifies the i-th entry point offset in bytes, and is represented by offset_len_minus1 plus 1 bits. The tile group data that follow the tile group header consists of num_tiles_in_tile_group_minus1 +1 subsets, with subset index values ranging from 0 to num_tiles_in_tile_group_minus1, inclusive. The first byte of the tile group data is considered byte 0. When present, emulation prevention bytes that appear in the tile group data portion of the coded tile group NAL unit are counted as part of the tile group data for purposes of subset identification. Subset 0 consists of bytes 0 to entry_point_offset_minus1[ 0 ], inclusive, of the coded tile group data, subset k, with k in the range of 1 to num_tiles_in_tile_group_minus1 − 1, inclusive, consists of bytes firstByte[ k ] to lastByte[ k ], inclusive, of the coded tile group data with firstByte[ k ] and lastByte[ k ] defined as:

(7-50)

lastByte[ k ] = firstByte[ k ] + entry_point_offset_minus1[ k ]	(7-51)

The last subset (with subset index equal to num_tiles_in_tile_group_minus1) consists of the remaining bytes of the coded tile group data.

Each subset shall consist of all coded bits of all CTUs in the tile group that are within the same tile.