| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0043 |
| --- | --- |

| Title: | AHG12: VVC design goals related to MPEG requirements on immersive media access and delivery |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Emre B. Aksu Nokia Technologies Hatanpaan valtatie 30 33100 Tampere Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes how the VVC design should reflect MPEG's work on the decoding interface for immersive media access and delivery. It is proposed to have a design goal that VVC natively supports the requirements on the decoding interface of the immersive media access and delivery, if possible without substantial burden (e.g., in codec architecture). However, it is proposed that the VVC design is only required to handle multiplexed bitstreams in which the bitstreams of video objects have a non-contradicting decoding order.

# Background: MPEG work on immersive media access and delivery

## Overview

MPEG is carrying out preparatory work, such as defining requirements, on immersive media access and delivery. The draft requirements are described in MPEG document N18134 [1] and a presentation of the topic is available in MPEG document M46578 [2]. A key message is a request to avoid the "shoehorning approach" of merging motion-constrained tile sets (MCTSs) of HEVC into the same coded picture, depicted in Figure 1. In many applications, such shoehorning is unnecessary since the decoded pictures are not displayed as such but undergo a rendering process. Furthermore, MCTSs require specific encoding constraints, and merging of MCTSs into the same coded picture requires rewriting of some header and parameter set data.

Figure 1. Shoehorning of HEVC MCTSs into the same coded picture (source: MPEG M46578).

Instead of shoehorning, it is suggested in the MPEG N18134 and M46578 that decoders should offer the capability of decoding several sub-picture sequences independently with the same decoder instance as illustrated in Figure 2.

Figure 2. Decoding of sub-picture sequences independently (source: MPEG M46578).

## Draft requirements

The draft requirements on video decoding interface are copied from MPEG N18134 below:

In the context of these requirements, an “Object” is defined as a visual element that is independently decodable and represented by its own fully conformant bitstream. This is a draft definition that implies that there is no “subbitstream” or something similar.

The decoding interface shall allow multiple independent video objects to be decoded

The decoding interface shall not require rewriting of the bitstream(s) corresponding to these objects, by the user of the decoding resources

NB: bitstream rewriting refers to more than simple operations (like concatenation) that would be required to decode all objects by feeding a single bitstream through a single decoding resource

It shall be possible to maintain absolute, frame-level sync between these decoded objects

The maximum number of objects shall be a function of the total number of macroblocks/CTU/samples/pixels per second that the decoding resources can process

Note: This should allow for a number of objects that is at least an order of magnitude more than the amount of decoders that can typically be instantiated in a device today (e.g. 4).

Note: it is understood that the number of pixels that a decoder can handle will be constrained by a Level specification, and that this Level specification also constrains the number of objects.

It shall be possible to simultaneously decode objects with different parameters:

    - different object sizes and, for rectangular objects, aspect ratios

    - different framerates

e.g., have a 30 fps fall-back layer in tiled VR streaming, ad a 90 fps foreground (viewport) layer

    - Different resolutions (sampling density)

e.g. have a mix of 4K, 6K and 8K representations for different parts of the same 8K ERP or cubemap.

The interface shall support decoding objects that have different image parameters, such as color space and bit depth.

The interface shall support maintaining the association of metadata with the decoded object

It shall be possible to keep objects in absolute (frame-level) sync with their associated metadata

The conformance requirements for the decoding interface shall include a well-defined maximum decoding latency for all objects on a per-frame basis

It shall be possible to do all of the above with protected content

It should be possible to mix protected and non-protected content

Note: whether support for different keys is required for these objects is for discussion, and it may not impact the decoding interface.

The decoding interface shall allow efficient handling by graphics processors

Note: this might be done by, e.g., allowing decoded objects to be packed into texture buffer(s) as appropriate

The following requirement is perhaps not on the decoding interface, but is nevertheless important for the system:

It shall be possible to manipulate decoded objects without having access to the actual pixel information

Note: this is to be able to manipulate objects in a secure pipeline. Keeping with the tiling example: this would allow placing protected tiles in the right position on a sphere.

## Inputs to MPEG#126

Further work on the requirements taking MPEG N18134 as a starting point has been carried out as input to MPEG#126. MPEG M46668 [4], which is also made available as JVET-N0042, proposes the following new or clarified requirements:

- The following new requirement is proposed:

1.x0. It shall be possible to control whether decoded objects of the same timestamp are output separately or spatially arranged onto the same output picture.

- Requirement 1.1 is proposed to be clarified as follows (underlined text indicates the proposed clarifications):

1.1. The decoding interface inputs a single multiplexed bitstream comprising bitstreams(s) of the video objects. The decoding interface shall not require more than simple operations (like concatenation) to multiplex the bitstream(s) corresponding to these objects into the single multiplexed bitstream, by the user of the decoding resources

NB: This requirement enables decoding of all objects by feeding a single bitstream through a single decoding resource.

NB: The multiplexed bitstream is not required to contain all video objects that were encoded. The user of the decoding resources selects which video objects are included into the multiplexed bitstream.

- It is proposed to replace requirement 1.2 ("It shall be possible to maintain absolute, frame-level sync between these decoded objects") with the following requirements:

1.x1. The decoding interface shall provide a possibility to output decoded objects in decoding order (possibly out of presentation order).

1.x2. When object video bitstreams have a non-contradicting decoding order, the decoding interface shall provide a possibility to output decoded objects in presentation order with frame-level sync across video objects.

A non-contradicting decoding order of two bitstreams is such that when only those coded objects that have the same presentation timestamp are kept in both bitstreams, the coded objects appear in the same order in both bitstreams with respect to their presentation timestamps. This definition allows one bitstream may have low picture rate than the other.

# Proposal

## Support of decoding interface of the immersive media access and delivery

It is proposed to have a design goal that VVC natively supports the requirements on the decoding interface of the immersive media access and delivery, if possible without substantial burden (e.g., in codec architecture).

Motivation:

- When VVC decoders natively support the decoding interface for immersive media access and delivery, there is no need to specify or implement a separate "multi-bitstream wrapper decoder". Such a wrapper layer would have the danger of not becoming broadly implemented.

- Even if a standard for "multi-bitstream wrapper decoder" would be specified and be implemented, its implementation for VVC would become much simpler when VVC natively supports its features.

- It is believed that VVC would have an advantage over other video codecs if it natively supports the requirements of the decoding interface for immersive media access and delivery. As documented in MPEG N18134 and M46578, video codecs suit immersive media applications, such as VR and point cloud video, provided that several "object" bitstreams can be decoded parallelly and synchronously. It is envisioned that usage of video codecs will also be used for other types of immersive video (3DoF+, 6DoF).

- In our experience, the capability of supporting multiple concurrent hardware decoder instances is common, and the hardware decoders can also share their total decoding capacity in pixels/second to a flexible number of decoder instances. For example, in our measurements, Samsung Galaxy S9+ with Exynos 9810 chipset was able to run either of the following setups:

- Decoding of 4 HEVC 4K (3840×2160) bitstreams, each at 37 frames per second

- Decoding of 16 HEVC HD (1920×1080) bitstreams, each at 30 frames per second

Decoding independent bitstreams with separate decoding instances leaves the burden of synchronizing the outputs of the decoders to the rendering process. It is considered reasonable to take advantage of concurrent decoding instances by having CPB and DPB operation managed across video object bitstreams within a VVC decoder. Such synchronization is one of the key requirements of MPEG N18134.

## Exclusion of video object bitstreams with different inter prediction hierarchy

### Issues

MPEG N18134 and M46578 imply that the bitstreams of the video objects are not required to have the same inter prediction hierarchy. In other words, the decoding order of pictures relative to the output order of pictures may differ in the bitstreams. MPEG N18134 anyhow includes a requirement to output the decoded objects synchronously.

An implication of the features discussed in the previous paragraph is that the initial delay and size of the decoded object buffer depends on which other objects are fed to the same decoder instance. Since the selection of video objects changes dynamically in many of the immersive media applications (e.g. VR video streaming), it is challenging to determine a proper initial delay and size for the decoded object buffer.

A similar implication on decoded picture buffer was studied in Nokia's SHVC call-for-proposals response, which included the possibility to use different inter prediction hierarchies in different layers of the scalable video bitstream. The CfP response with the uneven inter prediction hierarchy configuration was described in JCTVC-K0047 [5], and a presentation was included in JCTVC-K0040 [6]. The topic was studied further in JCTVC-L0171 [7].

A consequence of allowing different inter prediction hierarchies for different video objects is that in a multiplexed bitstream the coded video objects of different bitstreams would not necessarily appear in ascending decoding timestamp. It would not be possible to define the concept of access unit as a contiguous piece of coded data (in the decoding order within the multiplexed bitstream) having the same presentation timestamp.

### Proposal

It is proposed that the VVC design is only required to handle multiplexed bitstreams in which the bitstreams of video objects have a non-contradicting decoding order.

A non-contradicting decoding order of two bitstreams is such that when only those coded objects that have the same presentation timestamp are kept in both bitstreams, the coded objects appear in the same order in both bitstreams with respect to their presentation timestamps. This definition allows one bitstream may have low picture rate than the other.

It is suggested to use different VVC decoder instances for video object bitstreams with contradicting decoding orders and perform the buffering to reconstruct the correct output order and timing synchronization outside the VVC decoder instances.

# References

- E. Thomas, T. Stockhammer, R. Koenen (editors), "Draft requirements on immersive media delivery and access," MPEG N18134, 
http://phenix.it-sudparis.eu/mpeg/doc_end_user/documents/125_Marrakech/wg11/w18134.zip.

- E. Thomas, R. Koenen, T. Stockhammer, "On the decoding interface for immersive media," MPEG M46578, 
http://phenix.it-sudparis.eu/mpeg/doc_end_user/documents/125_Marrakech/wg11/m46578-v3-m46578-OnthedecodinginterfaceforImmersiveMedia.zip.

- E. Thomas, A. Gabriel, "AHG12: On picture-level tiles and sequence-level tiles for VVC," JVET-M0536, 
http://phenix.it-sudparis.eu/jvet/doc_end_user/documents/13_Marrakech/wg11/JVET-M0536-v1.zip.

- M. M. Hannuksela, E. B. Aksu, and A. Hourunranta, "Comments on the draft requirements on immersive media access and delivery," MPEG M46668, http://phenix.it-sudparis.eu/mpeg/doc_end_user/current_document.php?id=66519&id_meeting=178, also available as JVET-N0042, http://phenix.it-sudparis.eu/jvet/doc_end_user/current_document.php?id=5762.

- O. Bici, D. Bugdayci, A. Aminlou, A. Hallapuro, M. Hannuksela, J. Lainema, K. Ugur, "Description of scalable video coding technology proposal by Nokia (encoder configuration 2)," JCTVC-K0047,
http://phenix.it-sudparis.eu/jct/doc_end_user/documents/11_Shanghai/wg11/JCTVC-K0047-v2.zip.

- O. Bici, D. Bugdayci, A. Aminlou, A. Hallapuro, M. Hannuksela, J. Lainema, K. Ugur, "Description of scalable video coding technology proposal by Nokia (encoder configuration 1)," JCTVC-K0040, 
http://phenix.it-sudparis.eu/jct/doc_end_user/documents/11_Shanghai/wg11/JCTVC-K0040-v3.zip.

- M. M. Hannuksela, A. Hallapuro, K. Ugur, "SHVC HLS: support for unequal BL and EL GOP lengths," JCTVC-L0171, http://phenix.it-sudparis.eu/jct/doc_end_user/documents/12_Geneva/wg11/JCTVC-L0171-v2.zip.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).