| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0044 |
| --- | --- |

| Title: | AHG12: Comparison of approaches for independently coded picture regions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Alireza Aminlou, Kashyap Kammachi-Sreedhar Nokia Technologies Hatanpaan valtatie 30 33100 Tampere Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The contribution categorizes approaches for independently coded picture regions to the following:

- Motion-constrained tile sets (MCTSs). Encoding is constrained in a manner that a rectangular set of tile groups can be extracted and repositioned without affecting their decoding or decoding of other tile groups.

- Independent rectangular tile groups (IRTGs). Encoding are decoding are modified to treat IRTG boundaries like picture boundaries. Implications on IRTG boundaries are handled in low-level decoding process.

- Sub-pictures, which are treated like pictures in the low-level decoding process. Sub-pictures of the same timestamp are spatially arranged onto an output picture.

- Independent sub-picture layers. It can be controlled if decoded pictures of the same timestamp and of different layers are composed onto the same output picture or output separately.

The contribution analyzes how the approaches meet the design goals that have been presented to JVET earlier or are part of MPEG's draft requirements on immersive media delivery and access. The contribution concludes that the most noteworthy differences lie in the following design goals:

|  | MCTS / IRTG |  | Sub-pictures | Independent layers |
| --- | --- | --- | --- | --- |
| Design goals summarized in this contribution |  |  |  |  |
| More efficient than MCTSs in terms of rate-distortion penalty | MCTS: no | IRTG: fulfilled | Fulfilled (unless specifically constrained) | Fulfilled (unless specifically constrained) |
| Partial intra updating with open GOP structures for viewport change in region-wise mixed-quality viewport-dependent 360° streaming | Not possible |  | Not possible | Could be enabled (with reference picture resampling) |
| Level and conformance for MCTSs / sub-picture sequences originating from several bitstreams | Not possible, requires systems support for indicating properties cross multiple bitstreams. |  | Not possible, requires systems support for indicating properties cross multiple bitstreams. | Fulfilled. Layer-wise level and CPB operation. |
| MPEG N18134 |  |  |  |  |
| 1.4. It shall be possible to simultaneously decode objects with different parameters: | A single SPS is activated, causing e.g. chroma format to be identical in all objects. |  | A single SPS is activated, causing e.g. chroma format to be identical in all objects. | Fulfilled. A separate SPS may be activated per each layer. |
| i. different object sizes and, for rectangular objects, aspect ratios | Fulfilled only when tile groups "fill in" the picture exactly |  | Could be enabled by allowing unoccupied areas in decoded pictures | Fulfilled |
| ii. different framerates | Not possible |  | Not possible | Fulfilled |
| 1.5. The interface shall support decoding objects that have different image parameters, such as color space and bit depth. | Not possible. |  | Not possible. | Fulfilled |
| 1.7. The conformance requirements for the decoding interface shall include a well-defined maximum decoding latency for all objects on a per-frame basis | Not (easily) possible, since a single CPB is used |  | Not (easily) possible, since a single CPB is used | Fulfilled, since layer-wise CPB is used. |
| JVET-N0042 |  |  |  |  |
| It shall be possible to control whether decoded objects of the same timestamp are output separately or spatially arranged onto the same output picture. | Not possible |  | Not possible | Fulfilled |

It is asserted that the independent sub-picture layer approach:

- provides a wider support of design goals than the other approaches

- does not require changes in the signal processing part of the codec unlike the IRTG approach; and

- has similar implementation cost than the sub-picture approach.

The contribution therefore concludes that the independent sub-picture layer approach is preferred for realizing independently coded picture regions.

# Introduction

This contribution responds to the following mandates of AHG12 (high-level parallelism and coded picture regions):

- Study support of independently coded picture regions, including easy rewriting of such regions into a conforming sub-bitstream.

- Study flexible tile partitioning (e.g. more flexible than HEVC and tile boundaries not spanning a full picture).

The contribution is structured as follows:

- Section 2 summarizes the design goals that have been presented in relation to the support independently coded picture regions.

- Section 3 provides a recap of those discussions in the JVET-M meeting that are relevant for to the topic of independently coded picture regions.

- Section 4 describes the categories of the presented approaches for independently coded picture regions and gives pointers for finding further information.

- Section 5 provides a comparison of the approaches in relation to the design goals.

- Section 6 concludes the comparison and proposes that the independent sub-picture layer approach for realizing independently coded picture regions.

# Background: VVC design goals for tile-related features

This section attempts to collect and summarize the proposed VVC design goals related to independently coded picture regions.

The following topics are covered:

- Extraction and repositioning

- Grid of independently decodable regions not crossing the entire picture

- More efficient than MCTSs in terms of rate-distortion penalty

- Partial intra updating for viewport change in region-wise mixed-quality viewport-dependent 360° streaming

- Partial intra updating for viewport change in region-wise mixed-resolution viewport-dependent 360° streaming

- Partial intra updating with open GOP structures for viewport change in region-wise mixed-quality viewport-dependent 360° streaming

- Conformance and level for MCTS or sub-picture sequence

- Decoding interface for immersive media access and delivery

## Extraction and repositioning

"Extracting VCL NAL units of a subset of MCTSs from one VVC bitstream and reposition them to another VVC bitstream without substantial difficulty (e.g., without VLC-level modifications)" was agreed as a design goal as noted in the disposition of JVET-L0182.

## Grid of independently decodable regions not crossing the entire picture

This application for MCTSs has been discussed JVET-K0300 and JVET-L0182.

The need is illustrated with a viewport-dependent 360° scheme offering an increased effective spatial resolution on the viewport. Schemes where tiles covering the viewport originate from a 6K (6144×3072) ERP picture or an equivalent CMP resolution, illustrated in Figure 1, with "4K" decoding capacity (HEVC Level 5.1) were included in clauses D.6.3 and D.6.4 of OMAF [1] and also adopted in the VR Industry Forum Guidelines[2]. Such resolutions are asserted to be suitable for head-mounted displays using quad-HD (2560×1440) display panel.

Encoding: The content is encoded at two spatial resolutions with cube face size 1536×1536 and 768×768, respectively. In both bitstreams a 6×4 tile grid is used and an MCTS is coded for each tile position.

Selection of streamed MCTSs: 12 MCTSs from the high-resolution bitstream are selected and the complementary 12 MCTSs are extracted from the low-resolution bitstream. Thus, a hemi-sphere (180°×180°) of the streamed content originates from the high-resolution bitstream.

Merging MCTSs to a bitstream to be decoded: The received MCTSs of a single time instance are merged into a coded picture of 1920×4608, which conforms to HEVC Level 5.1. Another option for the merged picture is to have 4 tile columns of width 768, two tile columns of width 384, and three tile rows of height 768 luma samples, resulting into a picture of 3840×2304 luma samples.

a) coded bitstreams		b) MCTSs selected for streaming		          c) Picture merged 												                      from MCTSs

Figure 1. Example MCTS-based RWMR viewport-dependent 360° streaming.

It can be observed from the dashed red lines that the presented scheme requires the use of two slices within a tile. Since VVC does not contain slices, flexibility in specifying a grid not crossing the entire picture would be beneficial in this scheme.

## More efficient than MCTSs in terms of rate-distortion penalty

The more fine-grained the MCTS grid is, the closer the viewport can be matched with high-resolution or high-quality streamed MCTSs, while the remaining areas can be covered with lower resolution or quality. However, fine-granularity MCTS grids also bring a rate-distortion cost in inter prediction. A large part of the RD efficiency loss of MCTSs is reportedly caused by constraining the motion vectors so that they do not cause references to sample locations outside the MCTS.

As described in JVET-M1032, the goal of CE12 is to test impact of handling boundaries of tile group based independent coded regions to the compression performance of VVC. Techniques included in this CE are:

- Handling all tile group boundaries as picture boundaries which includes padding to the tile group boundaries and allowing motion vectors to point to samples outside the tile group boundaries in the reference pictures.

- Encoder-side restrictions of tile groups to not use samples or motion vector candidates from outside the tile group.

## Partial intra updating for viewport change in region-wise mixed-quality viewport-dependent 360° streaming

### Overview

The use case and the related design goal were proposed in JVET-M0388 and copied below for convenience.

The viewing orientation moves gradually in typical viewing situations, the quality changes in only a subset of the sub-picture locations in RWMQ viewport-dependent streaming. For example, in the 4×2 example provided above it is likely that the quality changes in half of the sub-pictures. It is asserted that updating all sub-picture locations with Segments starting with a random-access picture is inefficient in terms of streaming rate-distortion performance and causes a large fluctuation in bitrate, which might cause an interruption in the playback for rebuffering or could be compensated by a sufficiently long initial buffering delay.

The desired operation of the player is illustrated in Figure 2 and described as follows: The player first selects sub-picture Representations with the greater random-access interval value (tN in this example), assuming that it would provide a better rate-distortion performance. When a viewing orientation switch causing a new selection of high-quality sub-picture tracks takes place, the player switches to the Representations with random-access interval equal to tM only for those sub-picture Representations where the picture quality changes for the remaining of the interval of duration tN. For the next interval of duration tN, this process is repeated, i.e. the player first selects the sub-picture Representations with the greater random-access interval value.

Figure 2. Merging of MCTS-based sub-picture tracks of the same resolution and switching between Representations of a different stream access point interval individually on sub-picture basis

It is noted that it has been found that a greater number of MCTSs has been found more advantageous than the 4×2 MCTS grid [3]. The share of MCTSs that need to be updated in quality in response to a viewing orientation change is likely to be lower when the MCTS grid is finer.

### Problem statement

The following challenges exist in AVC and HEVC for obtaining a conforming bitstream as a result of the target player behavior.

- As can be observed from the figure the picture at tM contains VCL NAL units from random-access pictures and non-random-access pictures. However, it is required in AVC and HEVC to use the same VCL NAL unit type in all slices of the same coded picture.

- The syntax of NAL units in AVC and HEVC depends on the NAL unit type. Particularly, no_output_of_prior_pics_flag is present only for IRAP pictures and the presence on syntax elements related to reference picture management depends on whether an IDR NAL unit type is indicated. Thus, in AVC and HEVC, rewriting VCL NAL units from one NAL unit type to another NAL unit type may require changes in slice header.

### Proposed design goal

It is proposed that:

The VVC design should enable a way to merge MCTSs originating from a random-access picture and MCTSs originating from a non-random-access picture into the same coded picture without substantial difficulty (e.g., without VLC-level modifications).

It is remarked that the systems layer (e.g. OMAF) is expected to provide the signalling whether MCTSs or sub-picture tracks can be merged. For example, two sub-picture tracks can be merged only if they have the same prediction structure and keep the same pictures marked as "used for reference".

### Discussion in the JVET-M meeting

JVET-M0388 was noted in the BOG report of high-level syntax (JVET-M0816) as follows:

Use case of 360 video could benefit from this or other immersive media use cases or motion constrained tiles use cases, where some regions are intra refreshed and others not. Use case is of interest, but other solutions may also address the use case. Further study encouraged.

## Partial intra updating for viewport change in region-wise mixed-resolution viewport-dependent 360° streaming

### Problem statement

This use case was discussed in JVET-M0259.

Since the viewing orientation moves gradually in typical viewing situations, the resolution changes in only a subset of the sub-picture locations in RWMR viewport-dependent streaming. Figure 3 illustrates a viewing orientation change from Figure 1 slightly upwards and towards the right cube face. Cube face partitions that have a different resolution as earlier are indicated with "C". It can be observed that the resolution changed in 6 out of 24 cube face partitions. However, as discussed above, Segments starting with an IRAP picture need to be received for all 24 cube face partitions in response to the viewing orientation change. Updating all sub-picture locations with Segments starting with an IRAP picture is inefficient in terms of streaming rate-distortion performance.

Figure 3. Viewing orientation change upwards and towards the right cube face. 
"C" indicates the cube face partitions with resolution change.

### Proposed design goal

It is proposed that:

The VVC design should enable a way to merge MCTSs of different width and height, some originating from a random-access picture and some originating from a non-random-access picture into the same coded picture without substantial difficulty (e.g., without VLC-level modifications).

## Partial intra updating with open GOP structures for viewport change in region-wise mixed-quality viewport-dependent 360° streaming

This use case and the related design goals were discussed in JVET-M0259.

The ability to use open GOP prediction structures with sub-picture Representations of RWMR 360° streaming is desirable to improve rate-distortion performance and to avoid visible picture quality pumping caused by closed GOP prediction structures.

The following design goals are proposed:

- The VVC design should allow merging of a sub-picture originating from a random-access picture and another sub-picture originating from a non-random-access picture into the same coded picture conforming to VVC.

- The VVC design should enable the use of open GOP prediction structure in sub-picture representations without compromising from the fast and seamless representation switching capability between sub-picture representations of different properties, such as different spatial resolutions, while enabling merging of sub-picture representations into a single VVC bitstream.

The design goals can be illustrated with Figure 4, in which sub-picture Representations for two sub-picture locations are presented. For both sub-picture locations, a separate version of the content is coded for each combination among two resolutions and two random access intervals. Some of the Segments start with an open GOP prediction structure. A viewing orientation change causes the resolution of sub-picture location 1 to be switched at the start of Segment 4. Since Segment 4 starts with a CRA picture, which is associated with RASL pictures, those reference pictures of the RASL pictures that are in Segment 3 need to be resampled. It is remarked that this resampling applies to sub-picture location 1 while decoded sub-pictures of some other sub-picture locations are not resampled. In this example, the viewing orientation change does not cause changes in the resolution of sub-picture location 2 and thus decoded sub-pictures of sub-picture location 2 are not resampled. In the first picture of Segment 4, the Segment for sub-picture location 1 contains a sub-picture originating from a CRA picture, while the Segment for sub-picture location 2 contains a sub-picture originating from a non-random-access picture. It is suggested that merging of these sub-pictures into a coded picture is allowed in VVC.

Figure 4. Example illustrating the proposed design goals.

## Level and conformance for MCTS / sub-picture sequence

This topic was raised in JVET-M0445 and discussed in the BOG of tiles and WPP in the JVET-M meeting, see Section 3.1)c. The topic includes the following aspects:

- Specifying level and conformance for a specified subset of MCTSs / sub-picture sequences originating from the same bitstream. This helps when extracting a viewport from a 360° video and decoding just that.

- Specifying level and conformance for MCTSs / sub-picture sequences originating from several bitstreams. This helps in viewport-dependent 360° video streaming.

## Decoding interface for immersive media access and delivery

JVET-N0043 proposes a design goal that VVC natively supports the requirements on the decoding interface of the immersive media access and delivery, if possible without substantial burden (e.g., in codec architecture). Many of the requirements have similar intent as motion-constrained tile sets.

The draft requirements of immersive media access and delivery are available in MPEG N18134 [4]. Some updates to the requirements are proposed in MPEG M46668, which is also made available as JVET-N0042.

# Background: discussion in JVET-M

## BOG report on tiles and WPP

JVET-M0782-v3 reports the discussions on MCTS / sub-picture sequences as follows:

  - To indicate independently decodable rectangular region(s) in a sequence of pictures in MCTS-related SEI message(s), or in some parameter set (SPS, PPS, etc.)?

The BoG agreed that, if the signalling is not going to affect the decoding process, it does not make much difference between putting the information in SEI message(s) and VUI, otherwise, the information should be included in a non-VUI part of a parameter set.

    - In MCTS-related SEI message(s) similarly as in HEVC?

    - For the MCTS-application specific tiling in one type of parameter set?

      - In PPS? (M0261, M0416)

      - In sequence-level tile group parameter set? (STGPS), or maybe better to rename it as sub-picture parameter set (SPPS) (M0536)?

      - In SPS?

This aspect is for further study.

    - What information should be included?

      - Sub-picture sizes only (M0261)

      - Sequence-level tile group set boundary IDs (M0536)

  - To require that each MCTS / sub-picture to contain only one tile group? (M0261)

For ultralow delay application, it'd be good to allow for more than one tile group in one MCTS / sub-picture.

The BoG agreed to allow for more than one tile raster scan ordered tile group in one MCTS / sub-picture, but if the tile grouping mode is rectangular tile group, it is required that each MCTS / sub-picture contains only one tile group.

  - To specify conformance (level, HRD) or not for MCTS / sub-picture sequence?

What is the benefit of doing this? It helps when extracting a 360 video viewport and decode just that. Even more importantly, it helps in merging pieces into one conforming bitstream. The BoG agreed that this would be a useful thing to do.

The latest HEVC specification specifies, as part of the semantics of an SEI message, the MCTS sub-bitstream extraction process and requires the extracted sub-bitstream to be conforming bitstream. The BoG recommended to discuss in a larger group whether such way of defining conformance is good for VVC.

    - Yes? (M0445)

  - To (allow) treat boundaries of MCTS / sub-picture sequences as picture boundaries (i.e., to do padding)?

Allowing this is expected to help in coding efficiency. However, it would impose some burden for hardware implementations. Tests showing coding efficiency gain numbers and an analysis on hardware implementation complexity are needed for making a decision on this. Note that M0445 provides a software implementation that could be used as the basis and anchor for such coding efficiency comparison.

Interested parties were requested to work offline to prepare the test conditions. To be coordinated by M. Coban. To be discussed at a larger group.

    - If to use MCTS-related SEI messages, enable treating MCTS boundaries by

      - Allowing treating tile boundaries as picture boundaries? (M0136, M0530)

      - Allowing treating tile group boundaries as picture boundaries? (M0136)

    - If to specify sub-picture sequences, enable treating MCTS boundaries by

      - Default for all sub-picture sequence boundaries? (M0261)

  - To define and include an 8-bit sequence-level tile group set ID (maybe better to rename as sub-picture ID) in NAL unit header? (M0536)

It was commented that it'd be useful for MCTS / sub-picture sequence extraction. Why not put it in the beginning of the tile group header? The tile group ID in the tile group header provides the same information for rectangular tile groups.

What is the middle-box use case for sub-picture ID based extraction / adaptation?

There was no consensus to take an action on this.

  - To adopt the software implementation in M0445 into VTM and agree to evaluate future contribution with normative motion constraints with respect to the presented implementation?

The BoG recommended to adopt the software implementation.

  - To allow VCL NAL units of different sub-pictures within one picture to have mixed IRAP and non-IRAP NAL unit types? (M0388 option 3; options 1 and 2 of M0388 seem more for AHG17)

Option 3 of M0388 is for further study.

## Track A discussion

The discussion of Track A on conformance of MCTS / sub-picture sequences is noted in JVET-M_Notes as follows:

- The latest HEVC text specifies, as part of the semantics of an SEI message, the MCTS sub-bitstream extraction process and requires the extracted sub-bitstream to be conforming bitstream. For VVC, should we do more than this way of defining conformance for MCTSs? For example, should normative decoding process be specified for decoding of MCTSs instead of just specifying SEI messages for indication of the encoder motion constraints? There were several related proposals. For example, whether MCTS sequences are signalled in parameter sets or whether an MCTS sequence would have its own associated parameter sets. Whether to provide a level definition at the MCTS sequence level. Whether to define a normative decoder interface and conforming decoding behaviour for MCTS-specific decoding. Further study is highly encouraged.

# Summary of presented approaches

The following approaches to fulfil at least a subset of the design goals have been presented:

- Motion-constrained tile sets (MCTSs). Encoding is constrained in a manner that a rectangular set of tile groups can be extracted and repositioned without affecting their decoding or decoding of other tile groups.

As discussed in Section 3.1.a, the signalling for MCTSs could be realized with an SEI message and by using syntax element(s) in a parameter set. This contribution treats both options under the same term (MCTS).

- Independent rectangular tile groups (IRTGs). Encoding are decoding are modified to treat IRTG boundaries like picture boundaries. Implications on IRTG boundaries are handled in low-level decoding process.

Further information: Section 3.1.d.i.

- Sub-pictures, which are treated like pictures in the low-level decoding process. Sub-pictures of the same timestamp are arranged onto an output picture in their decoding order into the next large enough unoccupied position in raster scan order. Sub-pictures are determined to belong to the same sub-picture sequence when they are collocated.

Further information: JVET-N0046, Section 3.1.d.ii.

- Independent sub-picture layers. It can be controlled if decoded pictures of the same timestamp and of different layers are composed onto the same output picture or output separately.

Further information: JVET-N0045

# Comparison of presented approaches

## Comparison relative to presented design goals

The following table summarizes how the different approaches fulfil the design goals that have been presented. The following colors have been used in shading:

- Green: the design goal is fully supported

- Light green: the design goal is supported with some constraints, such as some encoding limitations

- Yellow: the design goal is supported only in special cases

- Red: the design goal is not supported

- White: the design goal is outside of the scope or not relevant for VVC scope or could be specified similarly for all options

|  | MCTS / IRTG |  | Sub-pictures | Independent layers |
| --- | --- | --- | --- | --- |
| Design goals summarized in this contribution |  |  |  |  |
| Extraction and repositioning | Fulfilled, with PPS and SPS changes only, provided that tile_group_address is selected uniquely for all possible merging combinations among all the bitstreams from which MCTSs can be merged. |  | Fulfilled, with PPS and SPS changes only. | Fulfilled, with VPS changes only. PPS and SPS changes might be needed if the same IDs had been used for parameter sets of different content. |
| Grid of independently decodable regions not crossing the entire picture | Fulfilled (most proposals) |  | Fulfilled | Fulfilled |
| More efficient than MCTSs in terms of rate-distortion penalty | MCTS: no | IRTG: fulfilled | Fulfilled (unless specifically constrained) | Fulfilled (unless specifically constrained) |
| Partial intra updating for viewport change in region-wise mixed-quality viewport-dependent 360° streaming | Could be enabled |  | Could be enabled | Fulfilled |
| Partial intra updating for viewport change in region-wise mixed-resolution viewport-dependent 360° streaming | Could be enabled |  | Could be enabled | Fulfilled |
| Partial intra updating with open GOP structures for viewport change in region-wise mixed-quality viewport-dependent 360° streaming | Not possible |  | Not possible | Could be enabled (with reference picture resampling) |
| Level and conformance for a subset of MCTSs / sub-picture sequences originating from the same bitstream | Can be realized e.g. like in HEVC motion-constrained tile sets extraction information sets SEI message and through picture timing enclosed in HEVC motion-constrained tile sets extraction information nesting SEI message |  | Can be realized e.g. like in HEVC motion-constrained tile sets extraction information sets SEI message and through picture timing enclosed in HEVC motion-constrained tile sets extraction information nesting SEI message | Fulfilled. Layer-wise level and CPB operation. |
| Level and conformance for MCTSs / sub-picture sequences originating from several bitstreams | Not possible, requires systems support for indicating properties cross multiple bitstreams. |  | Not possible, requires systems support for indicating properties cross multiple bitstreams. | Fulfilled. Layer-wise level and CPB operation. |
| MPEG N18134 |  |  |  |  |
| 1. The decoding interface shall allow multiple independent video objects to be decoded | Fulfilled, when encoding MCTSs/IRTGs jointly |  | Fulfilled | Fulfilled |
| 1.1. The decoding interface shall not require rewriting of the bitstream(s) corresponding to these objects, by the user of the decoding resources | Fulfilled, with PPS and SPS changes only, provided that tile_group_address is selected uniquely for all possible merging combinations among all the bitstreams from which MCTSs can be merged. |  | Fulfilled, with PPS and SPS changes only. | Fulfilled, with VPS changes only. PPS and SPS changes might be needed if the same IDs had been used for parameter sets of different content. |
| 1.2. It shall be possible to maintain absolute, frame-level sync between these decoded objects | Fulfilled |  | Fulfilled | Fulfilled |
| 1.3. The maximum number of objects shall be a function of the total number of macroblocks/CTU/samples/pixels per second that the decoding resources can process | Up to profile and level specifications |  | Up to profile and level specifications | Up to profile and level specifications |
| 1.4. It shall be possible to simultaneously decode objects with different parameters: | A single SPS is activated, causing e.g. chroma format to be identical in all objects. |  | A single SPS is activated, causing e.g. chroma format to be identical in all objects. | Fulfilled. A separate SPS may be activated per each layer. |
| i. different object sizes and, for rectangular objects, aspect ratios | Fulfilled only when tile groups "fill in" the picture exactly |  | Could be enabled by allowing unoccupied areas in decoded pictures | Fulfilled |
| ii. different framerates | Not possible |  | Not possible | Fulfilled |
| iii. Different resolutions (sampling density) | Fulfilled |  | Fulfilled | Fulfilled |
| 1.5. The interface shall support decoding objects that have different image parameters, such as color space and bit depth. | Not possible. |  | Not possible. | Fulfilled |
| 1.6. The interface shall support maintaining the association of metadata with the decoded object | Up to how decoder output is specified |  | Up to how decoder output is specified | Up to how decoder output is specified |
| 1.6.1. It shall be possible to keep objects in absolute (frame-level) sync with their associated metadata | Up to how decoder output is specified |  | Up to how decoder output is specified | Up to how decoder output is specified |
| 1.7. The conformance requirements for the decoding interface shall include a well-defined maximum decoding latency for all objects on a per-frame basis | Not (easily) possible, since a single CPB is used |  | Not (easily) possible, since a single CPB is used | Fulfilled, since layer-wise CPB is used. |
| 2. It shall be possible to do all of the above with protected content | Enabled (potential parts that need rewriting can be left non-encrypted). Outside of VVC scope. |  | Enabled (potential parts that need rewriting can be left non-encrypted). Outside of VVC scope. | Enabled (potential parts that need rewriting can be left non-encrypted). Outside of VVC scope. |
| 2.1. It should be possible to mix protected and non-protected content | Outside of VVC scope. |  | Outside of VVC scope. | Outside of VVC scope. |
| 3. The decoding interface shall allow efficient handling by graphics processors | The approaches are believed to have no crucial differences for handling by graphics processors. |  |  |  |
| JVET-N0042 |  |  |  |  |
| It shall be possible to control whether decoded objects of the same timestamp are output separately or spatially arranged onto the same output picture. | Not possible |  | Not possible | Fulfilled |
| The decoding interface shall provide a possibility to output decoded objects in decoding order (possibly out of presentation order). | Up to how decoder output is specified |  | Up to how decoder output is specified | Up to how decoder output is specified |
| When video bitstreams have a non-contradicting decoding order, the decoding interface shall provide a possibility to output decoded objects in presentation order with frame-level sync across video objects. | Up to how decoder output is specified |  | Up to how decoder output is specified | Up to how decoder output is specified |

# Discussion and conclusions

Based on the comparison in Section 5, it appears that the independent sub-picture layer approach provides a wider support of design goals presented in VVC and of MPEG's requirements for immersive media access and delivery. It is believed that its implementation cost is not significantly higher than that of the sub-picture approach. It does not require changes in the signal processing part of the codec unlike the IRTG approach. The contribution therefore concludes that the independent sub-picture layer approach is preferred for realizing independently coded picture regions.

# References (to non-JVET documents)

- MPEG N17563, "Second revised text of ISO/IEC FDIS 23090-2," April 2018.

- Virtual Reality Industry Forum Guidelines, draft version 1.1, May 2018. Online: https://www.vr-if.org/guidelines/, accessed 2018-06-24.

- Q. Xie, "OMAF VDC-CE Report," MPEG M40638, April 2017.

- E. Thomas, T. Stockhammer, R. Koenen (editors), "Draft requirements on immersive media delivery and access," MPEG N18134, 
http://phenix.it-sudparis.eu/mpeg/doc_end_user/documents/125_Marrakech/wg11/w18134.zip.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).