| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0045 |
| --- | --- |

| Title: | AHG12: Sub-picture layers for realizing independently coded picture regions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Kashyap Kammachi-Sreedhar, Alireza Aminlou Nokia Technologies Hatanpaan valtatie 30 33100 Tampere Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

It is asserted that the independent non-base layer design of HEVC fulfills a substantial amount of the proposed design goals for independently coded picture regions and of the draft MPEG requirements on immersive media delivery and access (MPEG N18134). Consequently, the contribution takes the HEVC design as a starting point and proposes to specify independent sub-picture layers for VVC. The proposal has the following parts:

- Layer identifier is included in the syntax in either of the following ways:

  - A layer delimiter NAL unit containing a layer identifier. The layer delimiter NAL unit pertains up to and excluding the next layer delimiter NAL unit in decoding order.

It is also proposed to remove reserved bits from the NAL unit header and to have a one-byte NAL unit header containing NAL unit type and temporal identifier as proposed in JVET-N0067.

  - A layer identifier included is included in the two-byte NAL unit header.

Option A allows a broader value range for layer identifiers and is hence preferred by the proponent.

- Video Parameter Set (VPS) that indicates which layers are independent layers and optionally contains layout information for arranging decoded pictures from different layers into an output picture.

- An activation process for VPS and layer-specific SPS and PPS activation processes.

- An extension to the high-level decoding process that keeps track of the layers that have been initialized for decoding.

- Fixed-length coding of parameter set identifiers.

- Requirement to support for multiple independent layers in all profiles of VVC version 1.

# Introduction: independent non-base layers of HEVC

The feature of independent non-base layers was included in HEVC with its multi-layer extensions. The Video Parameter Set (VPS) of HEVC describes the inter-layer prediction relations and allows to define independent non-base layers, i.e. layers that are not inter-layer-predicted from any other layer. Each independent non-base layer is like a single-layer HEVC bitstream, with the only difference of a non-zero layer identifier value (i.e. nuh_layer_id value).

The feature of independent non-base layers enables multiplexing of several HEVC bitstreams into a single multi-layer HEVC bitstream provided that the decoding order of the single-layer bitstreams is non-contradicting. A non-contradicting decoding order of two bitstreams is such that when only those coded pictures that have the same presentation timestamp are kept in both bitstreams, the coded pictures appear in the same order in both bitstreams with respect to their presentation timestamps. This definition allows one bitstream may have low picture rate than the other, while they still can be combined into the same multi-layer HEVC bitstream.

The VPS syntax element dimension_id[ i ][ j ] specifies the identifier of the j-th present scalability dimension type of the i-th layer. The dimension types include e.g. multiview and spatial/quality scalability. It is allowed to indicate dimension_id[ i ][ j ] equal to 0, which indicates that no scalability of the particular scalability dimension type has been applied in this layer. Thus, even in HEVC, it is possible to indicate multiple independent layers representing the same scalability dimension values.

The HRD specified for HEVC multi-layer extensions enables bitstream partition based CPB operation, where a bitstream partition can be indicated to be a layer. Furthermore, DPB operation is specified to include time or order aligned output of pictures among all layers. Such time-aligned output operation was requested also in MPEG N18134.

Based on the analysis presented in JVET-N0044, it is believed that the independent non-base layer feature of HEVC forms a good basis to fulfil the proposed design goals for independently coded picture regions and the requirements of MPEG N18134.

# Problem statement

It is asserted that the independent non-base layer design of HEVC has the following issues for fulfilling the proposed design goals for independently coded picture regions and the use cases and requirements for immersive media access and delivery:

- The affordable layer identifier value range in a 2-byte NAL unit header is too small for encoding the content for some of the use cases or applications for immersive media and delivery access.

For example, it is common in streaming to provide the content in multiple resolutions and each resolution may further be provided in multiple bitrates. In viewport-dependent 360° streaming, the content may be split into independent layers, each representing a sub-picture. The client selects in which resolution and bitrate each sub-picture is received based on viewport, available network throughput, display resolution. For example, the following different resolutions and splits to independent layers representing sub-pictures of cubemap content could be provided, resulting into 180 sub-pictures in total, where each in one or more bitrate versions.

  - 2048×2048 cube faces split into 4×4 sub-pictures, thus 96 sub-pictures in total

  - 1536×1536 cube faces split into 3×3 sub-pictures, thus 54 sub-pictures in total

  - 1024×1024 cube faces split into 2×2 sub-pictures, thus 24 sub-pictures in total

  - 512×512 cube faces, each forming a sub-picture, thus 6 sub-pictures in total

A 7-bit layer identifier value range could be enabled in VVC draft 4. Thus, with the 2-byte NAL unit header, it is not possible to assign the layer identifier values uniquely for all encoded layers in the example above.

It is remarked that a decoder will process much fewer layers than what would be allowed by the layer identifier value range. It is a subject for further study how to constrain the maximum number of layers that decoders are required to process.

- Either encoding of all independent layers must be done in a coordinated manner to assign unique layer identifier values, or encoded bitstreams need to be rewritten to become independent non-base layers. Both might cause challenges for parallel distributed encoding approaches. Similar challenges may also arise, when encoding is performed by the same entity but at different moments.

- There are environments and use cases where arranging sub-pictures onto a single output picture per a POC value is desirable, as discussed in JVET-N0042. The multi-layer HEVC decoder always outputs layers separately.

- Decoder support for multiple independent layers: It is suspected that the independent non-base layer design of HEVC has not become widely supported, since it was introduced with multi-layer extensions and not included in the Main or Main 10 profiles. Hence, it is considered important to include this requirement in the profiles specified in VVC version 1.

# Overview of the proposal

It is proposed to specify independent sub-picture layers for VVC. The proposal has the following parts:

- Layer identifier is included in the syntax in either of the following ways:

  - A layer delimiter NAL unit containing a layer identifier. The layer delimiter NAL unit pertains up to and excluding the next layer delimiter NAL unit in decoding order.

It is also proposed to remove reserved bits from the NAL unit header and to have a one-byte NAL unit header containing NAL unit type and temporal identifier as proposed in JVET-N0067.

  - A layer identifier included is included in the two-byte NAL unit header.

Option A allows a broader value range for layer identifiers and is hence preferred by the proponent. Section 3.1.3 provides a more detailed analysis of the benefits of option A compared to option B.

- Video Parameter Set (VPS) that indicates which layers are independent layers and optionally contains layout information for arranging decoded pictures from different layers into an output picture.

- An activation process for VPS and layer-specific SPS and PPS activation processes.

- An extension to the high-level decoding process that keeps track of the layers that have been initialized for decoding.

- Fixed-length coding of parameter set identifiers.

- Requirement to support for multiple independent layers in all profiles of VVC version 1.

JVET-N0044 provides a comparison of approaches for independently coded picture regions and concludes to recommend independent sub-picture layers. Both options for indicating layer identifiers have the following benefits that are specifically addressing the problems in Section 2.

- Conventional encoding of single-layer bitstreams can be done. Encoders do not have to consider merging with other independent layers at the time of encoding. This is asserted to be easier for parallel distributed encoding approaches.

- When merging bitstreams as independent layers into a single merged bitstream, only the VPS must potentially be rewritten, layer delimiter NAL units must potentially be added into the merged bitstream, and PPS and SPS identifiers may need to be rewritten. No other changes in tile group headers, PPSs, or SPSs are needed.

## Layer identifier

### Option A: layer delimiter NAL unit

A one-byte NAL unit header with the syntax in JVET-N0067 is proposed in this contribution, and a NAL unit type value for layer delimiter is added.

The following syntax for the layer delimiter RBSP is proposed:

| layer_delimiter_rbsp( ) { | Descriptor |
| --- | --- |
| layer_id | u(10) |
| rbsp_trailing_bits( ) |  |
| } |  |

In general, file and transport encapsulation provide means for indicating logical units, such as layers, and carry layers in separate logical channels (e.g. tracks in the file format, Representations in DASH). Likewise, file and transport formats provide metadata that is sufficient for multiplexing a single video bitstream from the logical channels.

It is envisioned that in many systems environments the layer delimiter NAL unit is included in layer-wise metadata and not replicated within the coded pictures. For example, the layer delimiter NAL unit could be included in a sample entry of a track that contains the layer. Thus, the proposal avoids the bitrate overhead in sending layer identifiers when layers are transmitted or stored in separate logical channels.

In some applications, it may be reasonable to let the client to create layer delimiter NAL units based on the systems-level metadata.

Layer delimiter NAL units can be thought as a way to combine several bitstreams into one merged bitstream as depicted in Figure 1.

Figure 1 - example of using layer delimiter NAL units

### Option B: layer identifier in a two-byte NAL unit header

A two-byte NAL unit header is used and contains a layer_identifier syntax element.

Since the number of bitstreams that is encoded might be greater than the value space of the layer_identifier syntax element or since the encoded bitstreams might use the same layer_identifier values, the client may need to rewrite the layer_identifier values and the VPS content before passing the merged bitstream for decoding, as illustrated in Figure 2.

Figure 2 - example of using layer identifier within NAL unit header

### Discussion

Option A is asserted to have the following benefits over option B:

- Option A avoids an asserted problem that the affordable layer identifier value range in a 2-byte NAL unit header is too small for encoding the content for some of the use cases or applications for immersive media and delivery access.

- In option A, the bitrate for transmitting layer identifier in each NAL unit is saved; i.e., 1 byte per NAL unit is saved by using 1-byte NAL unit header instead of 2-byte NAL unit header. For example, in a partitioning of 96 sub-pictures at 60 Hz, the bitrate saving is 46 kbps.

- The fact that layer identifiers need not be encoded in option A (with client-side layer delimiter NAL unit insertion) provides the benefit that there is no danger of clashes of using the same layer identifier value even when different encoders are used for encoding.

## Video parameter set

A Video Parameter Set is proposed with the following contents:

- 4-bit u(v)-coded VPS identifier like in HEVC.

- vps_max_layers_minus1 like in HEVC.

- A flag indicating if all layers are independent layers. If the flag is not set, independent_layer_flag[ i ] for i in the range of 1 to vps_max_layers_minus1, inclusive. In this version of VVC, all layers shall be independent layers. Decoders shall allow and ignore layers with independent_layer_flag[ i ] equal to 0.

Rationale: With this signalling, a decoder can identify and decode independent layers and can identify and ignore predicted layers.

- Layout information, the presence of which is gated by a flag. If layout information is absent, decoded pictures from independent layers are output separately. Otherwise, decoded pictures from independent layers having the same POC are arranged onto a single output picture.

## Parameter set activation

The proposed specification text is summarized as follows:

- It is proposed to add parameter set activation like in HEVC clause F.7.4.2.4.2, but without aspects concerning predicted layers. In particular, the following aspects of the proposed text are noted:

  - PPSs and SPSs are activated layer-wise like in HEVC.

  - PPSs and SPSs associated with layer_id equal to 0 may be activated in multiple layers like in HEVC.

- The activation text refers to the term coded layer-wise video sequence (CLVS), hence the respective definition is proposed to be added.

## Changes in high-level decoding process

The proposed specification text changes are summarized as follows:

- Since VPS is activated for coded video sequence group (CVSG), the decoding process is described on CVSG basis as in HEVC Annex F.

- LayerInitializedFlag[ layerId ] is maintained to keep track whether decoding of the layer with layer_id equal to layerId has been initialized.

- Decoding of an EOS NAL unit at a particular layer layerId immediately causes setting LayerInitializedFlag[ layerId ] equal to 0 and marking of decoded pictures of that layer as "unused for reference".

In HEVC, the marking was done when the next coded picture of that layer appeared. It is asserted that the proposed design helps in managing the DPB occupancy level when switching between independent layers. The pictures of a layer whose decoding is discontinued are marked as "unused for reference" immediately when an EOS NAL unit at that layer is decoded and the DPB memory becomes available for a layer with another layer_id value.

- When LayerInitializedFlag[ layerId ] is equal to 0, the first picture in decoding order for the respective layer shall be an IRAP picture.

This design choice differs from HEVC multi-layer design, where the decoding process determines that pictures preceding the first IRAP picture in a layer are cross-layer random access skipped (CL-RAS) pictures, decodes them by generating their reference pictures, and omits outputting them. It is suggested to be left for further study whether VVC should include similar capability.

## Fixed-length coding of parameter set identifiers

### Problem statement

Bitstreams subject to be combined as independent layers into a single merged bitstream may have used the same parameter set identifier values but with different content.

tile_group_pic_parameter_set_id, pps_pic_parameter_set_id, pps_seq_parameter_set_id, and sps_seq_parameter_set_id are of type ue(v). Consequently, if bitstreams subject to be combined as independent layers into a single merged bitstream had used the same parameter set identifier values but with different content, ue(v)-coded syntax elements have to be rewritten, for which the following steps have to be carried out:

- Removal of start code emulation prevention bytes from the NAL unit payload.

- Re-coding parameter set identifier(s).

- If the length of the parameter set identifier(s) changed, the following must be performed:

  - Bit-wise shifting of the remaining of the NAL unit payload.

  - Re-performing any byte alignment within the NAL unit payload (e.g. for tile group data and entry points).

  - Re-derivation of entry point offsets, if present.

- Insertion of start code emulation prevention bytes.

### Proposal

The proposal is summarized as follows:

- Parameter set identifiers are coded as u(v), with 4 bits for SPS identifiers and 6 bits for PPS identifiers to match the allowed value ranges in VVC draft 4.

- It is proposed to add a flag constrained to be equal to 1 following the parameter set identifiers to avoid start code emulation implications when rewriting of parameter set identifier values. The flag equal to 0 may be used as an extension mechanism for a broader parameter set value range, if such is needed in future VVC versions.

## Requirement to support for multiple independent layers in all profiles of VVC version 1

It is proposed to take the support for multiple independent layers in all profiles of VVC version 1 as a design assumption immediately and start discussing how to specify related constraints for profile and level specification text.

## Further work

If key aspects of this contribution are agreed, a description of HRD operation similar to that in HEVC Annex F should be authored.

The goal of CE12 (JVET-M1032) is to test impact of handling boundaries of tile group based independent coded regions to the compression performance of VVC. Techniques included in this CE are:

- Handling all tile group boundaries as picture boundaries which includes padding to the tile group boundaries and allowing motion vectors to point to samples outside the tile group boundaries in the reference pictures.

- Encoder-side restrictions of tile groups to not use samples or motion vector candidates from outside the tile group.

An independent layer as proposed in this contribution could be considered as an independently decodable tile group. If the discussion related to CE12 results into a conclusion that motion vectors over tile group boundaries does not provide sufficient compression gain justifying the additional implementation complexity, it is proposed that decoded reference picture marking shall be identical across independent layers and motion vectors for independent layers shall be constrained in encoding so that references to sample locations outside picture boundaries of the independent layers are avoided.

# Proposed specification text

## Layer identifier

### Option A: layer delimiter NAL unit

The syntax and semantics of the one-byte header are proposed in JVET-N0067. It is proposed that the NAL unit header contains exactly one byte.

NalUnitType value for layer delimiter RBSP is added into Table 7-1 of JVET-N0067:

| NalUnitType | Name of NalUnitType | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 4 | LD_NUT | Layer delimiter layer_delimiter_rbsb( ) | non-VCL |
| 4..7 5..7 | RSV_NVCL4.. RSV_NVCL5.. RSV_NVCL7 | Reserved | non-VCL |

| layer_delimiter_rbsp( ) { | Descriptor |
| --- | --- |
| layer_id | u(10) |
| } |  |

The layer delimiter NAL unit applies to the NAL units following the layer identifier NAL unit in decoding order until the last NAL unit of the access unit, inclusive, or the next layer delimiter NAL unit, exclusive, whichever is earlier in decoding order.

The variable MaxLayerId is set equal to 210 − 1.

### Option B: layer identifier in a two-byte NAL unit header

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(5) |
| nuh_temporal_id_plus1 | u(3) |
| nuh_reserved_zero_7bits | u(7) |
| layer_id | u(7) |
| } |  |

The variable MaxLayerId is set equal to 27 − 1.

### Semantics common for both options

layer_id specifies the identifier of the layer to which the associated VCL NAL units belong or the identifier of a layer to which the associated non-VCL NAL units apply.

When a NAL unit is not associated with any layer delimiter NAL unit, the layer_id value to which the NAL unit is associated is inferred to be equal to 0.

The value of layer_id shall be in the range of 0 to MaxLayerId − 1, inclusive. The value of layer_id equal to MaxLayerId may be specified in the future by ITU-T | ISO/IEC. For purposes other than determining the amount of data in the decoding units of the bitstream, decoders shall ignore all NAL units that are associated with a layer delimiter NAL unit with the value MaxLayerId for layer_id.

The value of layer_id shall be the same for all VCL NAL units of a coded picture. The value of layer_id of a coded picture is the value of the layer_id of any layer delimiter NAL unit with which the VCL NAL units of the coded picture are associated.

## Video parameter set

For option 1.A, NalUnitType value for VPS RBSP is added into Table 7-1 of JVET-N0067:

| NalUnitType | Name of NalUnitType | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 20 | VPS_NUT | Video parameter set video_parameter_set_rbsp( ) | non-VCL |
| 20..23 21..23 | RSV_NVCL20.. RSV_NVCL21.. RSV_NVCL23 | Reserved | non-VCL |

For option 1.B, nal_unit_type value for VPS RBSP is added into Table 7-1:

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 24..27 25..27 | RSV_NVCL24.. RSV_NVCL25.. RSV_NVCL27 | Reserved non-VCL NAL unit types | non-VCL |
| 28..31 | UNSPEC28.. UNSPEC31 | Unspecified non-VCL NAL unit types | non-VCL |

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_video_parameter_set_id | u(4) |
| vps_max_layers_minus1 | u(10) |
| vps_only_independent_layers_flag | u(1) |
| if( !vps_only_independent_layers_flag ) |  |
| for( i = 1; i <= MaxLayersMinus1; i++ ) |  |
| vps_independent_layer_flag[ i ] | u(1) |
| vps_layout_flag | u(1) |
| if( vps_layout_flag ) { |  |
| vps_layout_width | ue(v) |
| vps_layout_height | ue(v) |
| for( i = 0; i <= MaxLayersMinus1; i++ ) |  |
| if( vps_independent_layer_flag[ i ] ) { |  |
| vps_layer_top[ i ] | ue(v) |
| vps_layer_left[ i ] | ue(v) |
| } |  |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

vps_video_parameter_set_id identifies the VPS for reference by other syntax elements.

vps_max_layers_minus1 plus 1 specifies the maximum allowed number of layers in each CVS referring to the VPS. vps_max_layers_minus1 shall be less than MaxLayerId in bitstreams conforming to this version of this Specification. The value of MaxLayerId for vps_max_layers_minus1 is reserved for future use by ITU-T | ISO/IEC. Although the value of vps_max_layers_minus1 is required to be less than MaxLayerId in this version of this Specification, decoders shall allow the value of vps_max_layers_minus1 equal to MaxLayerId to appear in the syntax.

The variable MaxLayersMinus1 is set equal to Min( MaxLayerId − 1, vps_max_layers_minus1 ).

vps_only_independent_layers_flag equal to 0 specifies vps_independent_layer_flag[ i ] syntax elements are present for all values of i in the range of 1 to MaxLayersMinus1, inclusive. vps_only_independent_layers equal to 1 specifies that vps_independent_layer_flag[ i ] is inferred to be equal to 1 for all values of i in the range of 1 to MaxLayersMinus1, inclusive.

vps_independent_layer_flag[ i ] equal to 0 specifies that the i-th layer is a predicted layer. vps_independent_layer_flag[ i ] equal to 1 specifies that the i-th layer is an independent layer. vps_independent_layer_flag[ 0 ] is inferred to be equal to 1. vps_independent_layer_flag[ i ] shall be equal to 1 in this version of this Specification. Decoders shall allow the value of vps_independent_layer_flag[ i ] shall be equal to 0 to appear in the syntax and shall ignore (i.e., remove from the bitstream and discard) all NAL units with layer_id equal to any value for which vps_independent_layer_flag[ layer_id ] is equal to 0.

vps_layout_flag equal to 0 specifies that decoded pictures from independent layers are output separately. vps_layout_flag equal to 1 specifies that the decoded pictures of the independent layers with the same value of POC are spatially arranged to form an output picture. When vps_layout_flag is equal to 1, it is a requirement of bitstream conformance that each access unit consists of the coded pictures in all the independent layers.

vps_layout_width specifies the width of each output picture in units of luma samples.

vps_layout_height specifies the height of each output picture in units of luma samples.

vps_layer_top[ i ] and vps_layer_left[ i ] specify the top and left coordinates in luma samples of the top-left corner of the decoded picture of the i-th layer within the output picture.

The values of vps_layer_top[ i ] and vps_layer_left[ i ] of the active VPS and the values of pic_width_in_luma_samples and pic_height_in_luma_samples of active SPSs shall be such that no sample location in the output picture is mapped to more than one decoded picture. When a luma or chroma sample location in the output picture is mapped to no decoded picture, the respective luma or chroma sample value is set equal to 2BitDepthY − 1 or 2BitDepthC − 1, respectively.

## Parameter set activation

It is remarked that no detailed text on parameter set activation was included in JVET-M0101. Hence, no change marks are presented relative to earlier text.

### Proposed definition

coded layer-wise video sequence (CLVS): A sequence of coded pictures, with the same layer_id value layerId, that consists, in decoding order, of an IRAP picture with NoRaslOutputFlag equal to 1 followed by all coded pictures, if any, up to but excluding the next IRAP picture with NoRaslOutputFlag equal to 1.

### Proposed parameter set activation text (for clause 7.4.2.4.2)

This clause specifies the activation process of video parameter sets (VPSs), sequence parameter sets (SPSs) and PPSs.

NOTE 1 – The VPS, SPS and PPS mechanism decouples the transmission of infrequently changing information from the transmission of coded block data. VPSs, SPSs and PPSs may, in some applications, be conveyed "out-of-band".

A PPS RBSP includes parameters that can be referred to by the coded tile group NAL units of one or more coded pictures. Each PPS RBSP is initially considered not active for any layer at the start of the operation of the decoding process. At most one PPS RBSP is considered active for each layer at any given moment during the operation of the decoding process and the activation of any particular PPS RBSP for a particular layer results in the deactivation of the previously-active PPS RBSP for the particular layer (if any).

One PPS RBSP may be the active PPS RBSP for more than one layer. When not explicitly specified, the layer a PPS RBSP is active for is inferred to be the current layer in the context where the active PPS RBSP is referred to.

When a PPS RBSP (with a particular value of pps_pic_parameter_set_id) is not active for a particular layer with layer_id activatingLayerId and it is referred to by a coded tile group NAL unit (using a value of tile_group_pic_parameter_set_id equal to the pps_pic_parameter_set_id value) of the particular layer, it is activated for the particular layer. This PPS RBSP is called the active PPS RBSP for the particular layer until it is deactivated by the activation of another PPS RBSP for the particular layer. A PPS RBSP, with that particular value of pps_pic_parameter_set_id, shall be available to the decoding process prior to its activation, included in at least one access unit with TemporalId less than or equal to the TemporalId of the PPS NAL unit or provided through external means, and the PPS NAL unit containing the PPS RBSP shall be associated with layer_id equal to 0 or activatingLayerId.

NOTE 1 – All PPSs, regardless of their values of layer_id or TemporalId, share the same value space for pps_pic_parameter_set_id. In other words, a PPS with layer_id equal to X, TemporalId equal to Y and pps_pic_parameter_set_id equal to A would update the previously received PPS that has pps_pic_parameter_set_id equal to A and that has layer_id not equal to X or TemporalId not equal to Y.

Any PPS NAL unit containing the value of pps_pic_parameter_set_id for the active PPS RBSP for a coded picture (and consequently for the layer containing the coded picture) shall have the same content as that of the active PPS RBSP for the coded picture, unless it follows the last VCL NAL unit of the coded picture and precedes the first VCL NAL unit of another coded picture.

An SPS RBSP includes parameters that can be referred to by one or more PPS RBSPs. Each SPS RBSP is initially considered not active for any layer at the start of the operation of the decoding process. At most one SPS RBSP is considered active for each layer at any given moment during the operation of the decoding process and the activation of any particular SPS RBSP for a particular layer results in the deactivation of the previously-active SPS RBSP for that particular layer (if any).

One SPS RBSP may be the active SPS RBSP for more than one layer. When not explicitly specified, the layer an SPS RBSP is active for is inferred to be the current layer in the context where the active SPS RBSP is referred to.

When an SPS RBSP (with a particular value of sps_seq_parameter_set_id) is not already active for a particular layer with layer_id layerId and it is referred to by activation of a PPS RBSP (in which pps_seq_parameter_set_id is equal to the sps_seq_parameter_set_id value), it is activated for the particular layer. This SPS RBSP is called the active SPS RBSP for the particular layer until it is deactivated by the activation of another SPS RBSP for the particular layer. An SPS RBSP, with the particular value of sps_seq_parameter_set_id, shall be available to the decoding process prior to its activation, included in at least one access unit with TemporalId equal to 0 or provided through external means, and the SPS NAL unit containing the SPS RBSP shall be associated with layer_id equal to 0 or layerId. An activated SPS RBSP for a particular layer shall remain active for the entire CLVS of that particular layer.

Any SPS NAL unit with any layer_id value containing the value of sps_seq_parameter_set_id for the active SPS RBSP for a particular layer shall have the same content as that of the active SPS RBSP for the particular layer unless it follows the access unit auA containing the last coded picture for which the active SPS RBSP for the particular layer is required to be active for the particular layer and precedes the first NAL unit succeeding auA in decoding order that activates an SPS RBSP with the same value of seq_parameter_set_id.

NOTE 2 – All SPSs, regardless of their values of layer_id, share the same value space for sps_seq_parameter_set_id. In other words, an SPS with layer_id equal to X and sps_seq_parameter_set_id equal to A would update the previously received SPS with layer_id not equal to X and sps_seq_parameter_set_id equal to A.

A VPS RBSP includes parameters that can be referred to by one or more SPS RBSPs or one or more SEI NAL units containing an active parameter sets SEI message. Each VPS RBSP is initially considered not active at the start of the operation of the decoding process. At most one VPS RBSP is considered active at any given moment during the operation of the decoding process, and the activation of any particular VPS RBSP results in the deactivation of the previously-active VPS RBSP (if any).

When a VPS RBSP (with a particular value of vps_video_parameter_set_id) is not already active and it is referred to by activation of an SPS RBSP (in which sps_video_parameter_set_id is equal to the vps_video_parameter_set_id value), it is activated. This VPS RBSP is called the active VPS RBSP until it is deactivated by the activation of another VPS RBSP. A VPS RBSP, with that particular value of vps_video_parameter_set_id, shall be available to the decoding process prior to its activation, included in at least one access unit with TemporalId equal to 0 or provided through external means, and the VPS NAL unit containing the VPS RBSP shall be associated with layer_id equal to 0.

An activated VPS RBSP shall remain active until the end of the bitstream or until it is deactivated by another VPS RBSP in an access unit that includes an IDR picture for each layer_id value in TargetDecLayerIdList.

For any VPS NAL unit within the CVS with any value of layer_id and the value of vps_video_parameter_set_id equal to that of the active VPS RBSP for a CVS, the VPS RBSP in the VPS NAL unit shall have the same content as that of the active VPS RBSP for the CVS.

NOTE 4 – If VPS RBSP, SPS RBSP or PPS RBSP are conveyed within the bitstream, these constraints impose an order constraint on the NAL units that contain the VPS RBSP, SPS RBSP or PPS RBSP, respectively. Otherwise (VPS RBSP, SPS RBSP or PPS RBSP are conveyed by other means not specified in this Specification), they must be available to the decoding process in a timely fashion such that these constraints are obeyed.

All constraints that are expressed on the relationship between the values of the syntax elements and the values of variables derived from those syntax elements in VPSs, SPSs and PPSs and other syntax elements are expressions of constraints that apply only to the active VPS RBSP, the active SPS RBSP for any particular layer and the active PPS RBSP for any particular layer. If any VPS RBSP, SPS RBSP and PPS RBSP is present that is never activated in the bitstream, its syntax elements shall have values that would conform to the specified constraints if it was activated by reference in an otherwise conforming bitstream.

During operation of the decoding process, the values of parameters of the active VPS RBSP, the active SPS RBSP for the layer being decoded and the active PPS RBSP for the layer being decoded are considered in effect.

## Decoding process description

### Proposed definition

coded video sequence group (CVSG): One or more consecutive CVSs in decoding order that collectively consist of an IRAP access unit that activates a video parameter set (VPS) RBSP firstVpsRbsp that was not already active followed by all subsequent access units, in decoding order, for which firstVpsRbsp is the active VPS raw byte sequence payload (RBSP) up to the end of the bitstream or up to but excluding the access unit that activates a different VPS RBSP than firstVpsRbsp, whichever is earlier in decoding order.

### Proposed changes of clause 8.1

Changes are indicated in red relative to the adopted text of JVET-M0101.

    - General

Input to this process is a bitstream. Output of this process is a list of decoded pictures.

The decoding process is specified such that all decoders that conform to a specified profile and level will produce numerically identical cropped decoded output pictures when invoking the decoding process associated with that profile for a bitstream conforming to that profile and level. Any decoding process that produces identical cropped decoded output pictures to those produced by the process described herein (with the correct output order or output timing, as specified) conforms to the decoding process requirements of this Specification.

At the beginning of decoding a CVSG, after activating the VSPS RBSP that is active for the entire CVSG and before decoding any VCL NAL units of the CVSG, the following applies:

–	The variable LayerInitializedFlag[ i ] is set equal to 0 for all values of i from 0 to MaxLayersMinus1, inclusive, and each picture that is in the DPB is marked as "unused for reference".

–	Clause 8.1.2 is invoked with the CVSG as input.

    - CVSG decoding process

Input to this process is a CVSG. Output of this process is a list of decoded pictures.

The variable HighestTid, which identifies the highest temporal sub-layer to be decoded, is specified as follows:

–	If some external means, not specified in this Specification, is available to set HighestTid, HighestTid is set by the external means.

–	Otherwise, HighestTid is set equal to sps_max_sub_layers_minus1.

The sub-bitstream extraction process as specified in clause XX is applied with the CVSG and HighestTid as inputs, and the output is assigned to a bitstream referred to as BitstreamToDecode.

The following is applied for processing NAL units of BitstreamToDecode in decoding order:

–	If the next NAL unit in decoding order is an EOS NAL unit with layer_id layerId, the following applies:

–	Each picture that is in the DPB and has layer_id equal to layerId is marked as "unused for reference".

–	LayerInitializedFlag[ layerId ] is set equal to 0.

–	Otherwise, clause 8.1.3 is repeatedly invoked for the next each coded picture in BitstreamToDecode in decoding order.

    - Decoding process for a coded picture

The decoding processes specified in this clause apply to each coded picture, referred to as the current picture and denoted by the variable CurrPic, in BitstreamToDecode.

The value of the variable layerId is set equal to the value of layer_id of the current picture.

Depending on the value of chroma_format_idc, the number of sample arrays of the current picture is as follows:

–	If chroma_format_idc is equal to 0, the current picture consists of 1 sample array SL.

–	Otherwise (chroma_format_idc is not equal to 0), the current picture consists of 3 sample arrays SL, SCb, SCr.

The decoding process for the current picture takes as inputs the syntax elements and upper-case variables from clause 7. When interpreting the semantics of each syntax element in each NAL unit, the term "the bitstream" (or part thereof, e.g., a CVS of the bitstream) refers to BitstreamToDecode (or part thereof).

When LayerInitializedFlag[ layerId ] is equal to 0, the current picture shall be an IRAP picture.

When the current picture is an IRAP picture, the following applies:

–	If the current picture is an IDR picture or LayerInitializedFlag[ layerId ] is equal to 0, the first picture in the bitstream in decoding order, or the first picture that follows an end of sequence NAL unit in decoding order, the variable NoRaslOutputFlag is set equal to 1.

–	Otherwise, if some external means not specified in this Specification is available to set the variable HandleCraAsCvsStartFlag to a value for the current picture, the variable HandleCraAsCvsStartFlag is set equal to the value provided by the external means and the variable NoRaslOutputFlag is set equal to HandleCraAsCvsStartFlag.

–	Otherwise, the variable HandleCraAsCvsStartFlag is set equal to 0 and the variable NoRaslOutputFlag is set equal to 0.

Depending on the value of separate_colour_plane_flag, the decoding process is structured as follows:

–	If separate_colour_plane_flag is equal to 0, the decoding process is invoked a single time with the current picture being the output.

–	Otherwise (separate_colour_plane_flag is equal to 1), the decoding process is invoked three times. Inputs to the decoding process are all NAL units of the coded picture with identical value of colour_plane_id. The decoding process of NAL units with a particular value of colour_plane_id is specified as if only a CVS with monochrome colour format with that particular value of colour_plane_id would be present in the bitstream. The output of each of the three decoding processes is assigned to one of the 3 sample arrays of the current picture, with the NAL units with colour_plane_id equal to 0, 1 and 2 being assigned to SL, SCb and SCr, respectively.

NOTE 1 – The variable ChromaArrayType is derived as equal to 0 when separate_colour_plane_flag is equal to 1 and chroma_format_idc is equal to 3. In the decoding process, the value of this variable is evaluated resulting in operations identical to that of monochrome pictures (when chroma_format_idc is equal to 0).

The decoding process operates as follows for the current picture CurrPic:

- The decoding of NAL units is specified in clause TBD.

- The processes in clauses TBD specify decoding processes using syntax elements in in the tile group layer and above.

- The processes in clauses TBD specify decoding processes using syntax elements in all syntax structure layers.

LayerInitializedFlag[ layerId ] is set equal to 1.

## Fixed-length coding of parameter set identifiers

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | ue(v)u(4) |
| sps_reserved_one_bit | u(1) |
| … |  |

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v)u(6) |
| pps_seq_parameter_set_id | ue(v)u(4) |
| pps_reserved_one_bit | u(1) |
| … |  |

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v)u(6) |
| tile_group_reserved_one_bit | u(1) |
| … |  |

{sps_reserved_one_bit | pps_reserved_one_bit | tile_group_reserved_one_bit} shall be equal to 1 in bitstreams conforming to this version of this Specification. The value of 0 for {sps_reserved_one_bit | pps_reserved_one_bit | tile_group_reserved_one_bit} is reserved for future use by ITU-T | ISO/IEC.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).