| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0046 |
| --- | --- |

| Title: | AHG12: Sub-picture-based picture partitioning and decoding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Alireza Aminlou Nokia Technologies Hatanpaan valtatie 30 33100 Tampere Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The contribution proposes sub-picture-based picture partitioning and decoding to meet the agreed design goal of "extracting VCL NAL units of a subset of MCTSs from one VVC bitstream and reposition them to another VVC bitstream without substantial difficulty (e.g., without VLC-level modifications)". The proposal can be summarized as follows:

- Pictures are divided into sub-pictures, tile groups and tiles. The specifications of tile groups and tiles are the same as in VVC Draft 4.

- A sub-picture is a rectangular set of tile groups and is treated like a picture in the decoding process.

- Each sub-picture may refer to its own PPS and may hence have its own tile and tile group partitioning.

- When the decoding of a coded picture is started, each CTU location in the decoded picture is marked as unoccupied. For each sub-picture in decoding order, the sub-picture takes the next such unoccupied location in CTU raster scan order within a decoded picture that is large enough to fit the sub-picture within the picture boundaries.

- The reference pictures for decoding the sub-picture are generated by extracting the area collocating with the current sub-picture from the reference pictures in the decoded picture buffer. The extracted area shall be a decoded sub-picture, i.e. inter prediction takes place between sub-pictures of the same size and the same location within the picture.

# Introduction

The contribution re-proposes option 3 of JVET-M0261 with more fine-grained specification text and some further discussion on design options (see Section 4).

# Example

Figure 1 illustrates a picture that is divided into 4 sub-pictures. The sub-picture decoding order is indicated with the number (1, 2, 3, 4) outside the picture boundaries. Each sub-picture may have its own tile grid. In this example sub-picture 1 is divided into a grid of 3×2 tiles of equal width and equal height, sub-picture 2 is divided into 2×1 tiles of 3 and 5 CTUs high. Each of sub-pictures 3 and 4 has only one tile.

In the example of Figure 1 sub-picture 1 has 3 raster-scan-order tile groups containing 1, 3, and 2 tiles, respectively. Each of sub-pictures 2, 3, and 4 has one tile group.

Figure 1. A picture with 16 by 8 coding tree blocks that is partitioned into 4 sub-pictures, the first of which is partitioned to three tile groups and the others have one tile group.

# Proposed specification text

## Picture parameter set

The sequence parameter set contains the width and height of the entire picture, which may contain multiple sub-pictures. Thus, it is proposed to add the indication of sub-picture width and height in the picture parameter set. This could be done in any of the following ways:

- Sub-picture width and height in CTUs. This option is presented below.

- If uniform tile spacing is in use, sub-picture width and height in CTUs. Otherwise, the width of the last tile column and the height of the last tile row could be added to the PPS (i.e., the loops would have one item more).

- Sub-picture width and height in units of minimum coding block size. This option would enable finer granularity for the last tile column and the last tile row.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| multiple_subpics_enabled_flag | u(1) |
| if( multiple_subpics_enabled_flag ) { |  |
| subpic_width_in_ctus_minus1 | ue(v) |
| subpic_height_in_ctus_minus1 | ue(v) |
| } |  |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| … // no changes for the rest of the syntax |  |

multiple_subpics_enabled_flag equal to 0 specifies that a picture contains exactly one sub-picture and that all VCL NAL units of an access unit reference the same active PPS. multiple_subpics_enabled_flag equal to 1 specifies that a picture may contain more than one sub-picture and each sub-picture may reference a different active PPS. The value of multiple_subpics_enabled_flag shall be the same in all PPSs that are active for the same coded picture.

subpic_width_in_ctus_minus1 plus 1, when present, specifies the width of the sub-picture for which this PPS is the active PPS.

subpic_height_in_ctus_minus1 plus 1, when present, specifies the height of the sub-picture for which this PPS is the active PPS.

When subpic_width_in_ctus_minus1 and subpic_height_in_ctus_minus1 are present in a PPS that is activated, the following variables are derived and override the respective variables derived from the active SPS or any earlier active PPS:

PicWidthInCtbsY = subpic_width_in_ctus_minus1 + 1

PicHeightInCtbsY = subpic_height_in_ctus_minus1 + 1

PicSizeInCtbsY = PicWidthInCtbsY * PicHeightInCtbsY

PicWidthInMinCbsY = PicWidthInCtbsY * CtbSizeY / MinCbSizeY

PicHeightInMinCbsY = PicHeightInCtbsY * CtbSizeY / MinCbSizeY

PicSizeInMinCbsY = PicWidthInMinCbsY * PicHeightInMinCbsY

PicSizeInSamplesY = PicSizeInCtbsY * CtbSizeY * CtbSizeY

PicWidthInSamplesC = PicWidthInCtbsY * CtbSizeY / SubWidthC

PicHeightInSamplesC = PicHeightInCtbsY * CtbSizeY / SubHeightC

## Decoding process

The following yellow-highlighted changes are proposed into clause 8.1.3 (decoding process for a coded picture):

…

When multiple_subpics_enabled_flag in the PPS(s) that are active for the current picture is equal to 1, each CTU location in the decoded picture is marked as unoccupied.

The decoding process operates as follows for the current picture CurrPic:

- The decoding of NAL units is specified in clause 8.2.

- The processes in clause 8.3 specify the following decoding processes using syntax elements in the tile group header layer and above:

–	Variables and functions relating to picture order count are derived as specified in clause 8.3.1. This needs to be invoked only for the first tile group of a picture.

–	At the beginning of the decoding process for each tile group of a non-IDR picture, the decoding process for reference picture lists construction specified in clause 8.3.2 is invoked for derivation of reference picture list 0 (RefPicList[ 0 ]) and reference picture list 1 (RefPicList[ 1 ]).

–	The decoding process for reference picture marking in clause 8.3.3 is invoked, wherein reference pictures may be marked as "unused for reference" or "used for long-term reference". This needs to be invoked only for the first tile group of a picture.

- If multiple_subpics_enabled_flag in the PPS(s) that are active for the current picture is equal to 0, the following applies:

–	[Ed. (YK): Add herein the invocation of the decoding processes for coding tree units, scaling, transform, in-loop filtering, etc.]

Otherwise, the following applies for each sub-picture of CurrPic in decoding order:

–	The location of the sub-picture is the next such unoccupied location in CTU raster scan order within the decoded picture that is large enough to fit the sub-picture within the picture boundaries.

–	It is a requirement of bitstream conformance that the following constraints apply:

–	The value of multiple_subpics_enabled_flag in the PPS(s) that were active for decoding the picture corresponding to each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] shall be equal to the value of multiple_subpics_enabled_flag in the PPS(s) that are active for the current picture.

–	Each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] shall have a sub-picture at the same location and of the same size as the current sub-picture.

–	The decoding processes of [Ed. (MH): Add herein the invocation of the decoding processes for coding tree units, scaling, transform, in-loop filtering, etc.] are invoked with "picture" being replaced by "sub-picture" and the reference pictures corresponding to the active entries in RefPicList[ 0 ] or RefPicList[ 1 ] being generated by extracting the area collocating with the current sub-picture. The decoded sub-picture resulting from the decoding processes occupies the rectangular area wihin the decoded picture whose top-left corner is the location of the sub-picture as derived above and whose width and height in CTUs are equal to PicWidthInCtbsY and PicHeightInCtbsY, respectively, derived from the active PPS for the sub-picture.

- After all tile groups of the current picture have been decoded, the current decoded picture is marked as "used for short-term reference".

# Discussion

## Handling of motion vectors over sub-picture boundaries

There are two options for handling motion vectors over sub-picture boundaries:

- References to sample locations outside sub-picture boundaries are saturated in the decoding process. I.e., effectively padding of boundary samples is applied.

- References to sample locations outside sub-picture boundaries are required to be avoided in the encoding.

Option 1 handles sub-picture boundaries like picture boundaries in the decoding process. Option 2 imposes similar constraints as in HEVC motion-constrained tile sets. The results of CE12 will help in deciding whether option 1 or 2 should be combined with this contribution. It is remarked that the design in this contribution is otherwise independent of CE12.

## Sub-picture location

This contribution proposes a process to locate a sub-picture to the next unoccupied location in CTU raster scan order within a decoded picture. A motivation for this design decision was its asserted simplicity. For example, no additional syntax elements were needed.

An alternative design for determining the location of a sub-picture within a decoded picture would be to indicate the location explicitly e.g. in SPS. Such design could be more robust against accidental losses of sub-pictures and could allow more flexibility in dynamic selection of sub-picture sequences for decoding.

## Unoccupied areas in decoded pictures

It is proposed that either of the following design choices should be chosen:

- Allow unoccupied areas within decoded pictures. Include indication of unoccupied areas as decoder output.

- Add a constraint to disallow unoccupied areas within decoded pictures.

The proponent prefers option 1, since it provides more flexibility in determining which sub-picture sequences are selected for decoding.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).