| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-0048 |
| --- | --- |

| Title: | AHG19: On adaptive resolution changing |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Alireza Aminlou Nokia Technologies Hatanpaan valtatie 30 33100 Tampere Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes an adaptive resolution coding approach and includes the following aspects:

- Signalling:

- Add sps_max_rpr, which indicates a maximum number of reference pictures to be resampled per any picture to be decoded, in the SPS. When equal to 0, reference picture resampling is not used.

- Move pic_width_in_luma_samples and pic_height_in_luma_samples from SPS to PPS.

- Include max_width_in_luma_samples and max_height_in_luma_samples in DPS or SPS or both. They do not affect the decoding process but only provide the maximum value for pic_width_in_luma_samples and pic_height_in_luma_samples, respectively.

- High-level decoding process:

- Invoke a reference picture resampling process implicitly for the active reference picture(s) that have a different width and height from those of the current picture.

- The output of the resampling process is stored in the DPB and inherits the reference picture marking status from the reference picture used as input to the resampling process.

- The reference picture used as input to the resampling process is marked as "unused for reference".

- Resampling:

- SHVC resampling process (HEVC clause H.8.1.4.2), with an addition of selection of input samples for filtering in wraparound manner (for 360° video, particularly ERP).

# Introduction

AHG19 mandates include the following:

Study adaptive-resolution coding approaches for real-time communication, adaptive streaming, and 360-degree viewport-dependent streaming, including filters for resampling, reference picture management, and related scope and signalling

The use cases and design goals for adaptive resolution changing (ARC) were discussed in detail in JVET-M0259. A summary is provided below:

- Real-time communication

The following use cases for adaptive resolution change were originally included in JCTVC-F158:

  - Seamless network adaption and error resilience (through dynamic adaptive resolution changes)

  - Fast start (gradual increase of resolution at session start or reset)

  - Conference “compose” (person speaking is given a higher resolution)

- Adaptive streaming

Section 5.13 ("Support for Adaptive Streaming") of MPEG N17074 includes the following requirement for VVC:

The standard shall support fast representation switching in the case of adaptive streaming services that offer multiple representations of the same content, each having different properties (e.g. spatial resolution or sample bit depth). The standard shall enable the use of efficient prediction structures (e.g. so-called open groups of pictures) without compromising from the fast and seamless representation switching capability between representations of different properties, such as different spatial resolutions.

JVET-M0259 discusses how to meet this requirement by resampling of reference pictures of leading pictures.

- 360-degree viewport-dependent streaming

JVET-M0259 discusses how to address this use case by resampling certain independently coded picture regions of reference pictures of leading pictures.

This contribution proposes an adaptive resolution coding approach, which is asserted to meet all the use cases and design goals above. 360-degree viewport-dependent streaming and conference "compose" uses cases are handled by this proposal together with JVET-N0045 (proposing independent sub-picture layers).

# Proposed specification text

## Signalling

### sps_max_rpr

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_max_rpr | ue(v) |
| … |  |

sps_max_rpr specifies the maximum number of active reference pictures in reference picture list 0 or 1 for any tile group in the CVS that have pic_width_in_luma_samples and pic_height_in_luma_samples not equal to pic_width_in_luma_samples and pic_height_in_luma_samples, respectively, of the current picture.

### Picture width and height

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| max_width_in_luma_samples | ue(v) |
| max_height_in_luma_samples | ue(v) |
| … | ue(v) |

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| … |  |

max_width_in_luma_samples specifies that it is a requirement of bitstream conformance that pic_width_in_luma_samples in any active PPS for any picture of a CVS for which this SPS is active is less than or equal to max_width_in_luma_samples.

max_height_in_luma_samples specifies that it is a requirement of bitstream conformance that pic_height_in_luma_samples in any active PPS for any picture of a CVS for which this SPS is active is less than or equal to max_height_in_luma_samples.

## High-level decoding process

The decoding process operates as follows for the current picture CurrPic:

- The decoding of NAL units is specified in clause 8.2.

- The processes in clause 8.3 specify the following decoding processes using syntax elements in the tile group header layer and above:

–	Variables and functions relating to picture order count are derived as specified in clause 8.3.1. This needs to be invoked only for the first tile group of a picture.

–	At the beginning of the decoding process for each tile group of a non-IDR picture, the decoding process for reference picture lists construction specified in clause 8.3.2 is invoked for derivation of reference picture list 0 (RefPicList[ 0 ]) and reference picture list 1 (RefPicList[ 1 ]).

–	The decoding process for reference picture marking in clause 8.3.3 is invoked, wherein reference pictures may be marked as "unused for reference" or "used for long-term reference". This needs to be invoked only for the first tile group of a picture.

–	For each active reference picture in RefPicList[ 0 ] and RefPicList[ 1 ] that has pic_width_in_luma_samples or pic_height_in_luma_samples not equal to pic_width_in_luma_samples or pic_height_in_luma_samples, respectively, of CurrPic, the following applies:

–	The resampling process in clause X.Y.Z is invoked [Ed. (MH): details of invocation parameters to be added] with the output having the same reference picture marking and picture order count as the input.

–	The reference picture used as input to the resampling process is marked as "unused for reference".

- [Ed. (YK): Add herein the invocation of the decoding processes for coding tree units, scaling, transform, in-loop filtering, etc.]

- After all tile groups of the current picture have been decoded, the current decoded picture is marked as "used for short-term reference".

## Resampling process

SHVC resampling process (HEVC clause H.8.1.4.2) is proposed with the following additions:

…

If sps_ref_wraparound_enabled_flag is equal to 0, the sample value tempArray[ n ] with n = 0..7, is derived as follows:

tempArray[ n ] = ( fL[ xPhase, 0 ] * rlPicSampleL[ Clip3( 0, refW − 1, xRef − 3 ), yPosRL ] +
					fL[ xPhase, 1 ] * rlPicSampleL[ Clip3( 0, refW − 1, xRef − 2 ), yPosRL ] +
					fL[ xPhase, 2 ] * rlPicSampleL[ Clip3( 0, refW − 1, xRef − 1 ), yPosRL ] +
					fL[ xPhase, 3 ] * rlPicSampleL[ Clip3( 0, refW − 1, xRef ), yPosRL ] +
					fL[ xPhase, 4 ] * rlPicSampleL[ Clip3( 0, refW − 1, xRef + 1 ), yPosRL ] +	(H-38)
					fL[ xPhase, 5 ] * rlPicSampleL[ Clip3( 0, refW − 1, xRef + 2 ), yPosRL ] +
					fL[ xPhase, 6 ] * rlPicSampleL[ Clip3( 0, refW − 1, xRef + 3 ), yPosRL ] +
					fL[ xPhase, 7 ] * rlPicSampleL[ Clip3( 0, refW − 1, xRef + 4 ), yPosRL ] )  >>  shift1

Otherwise, the sample value tempArray[ n ] with n = 0..7, is derived as follows:

refOffset = ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY

tempArray[ n ] = ( fL[ xPhase, 0 ] * rlPicSampleL[ ClipH( refOffset, refW, xRef − 3 ), yPosRL ] +
					fL[ xPhase, 1 ] * rlPicSampleL[ ClipH( refOffset, refW, xRef − 2 ), yPosRL ] +
					fL[ xPhase, 2 ] * rlPicSampleL[ ClipH( refOffset, refW, xRef − 1 ), yPosRL ] +
					fL[ xPhase, 3 ] * rlPicSampleL[ ClipH( refOffset, refW, xRef ), yPosRL ] +
					fL[ xPhase, 4 ] * rlPicSampleL[ ClipH( refOffset, refW, xRef + 1 ), yPosRL ] +
					fL[ xPhase, 5 ] * rlPicSampleL[ ClipH( refOffset, refW, xRef + 2 ), yPosRL ] +
					fL[ xPhase, 6 ] * rlPicSampleL[ ClipH( refOffset, refW, xRef + 3 ), yPosRL ] +
					fL[ xPhase, 7 ] * rlPicSampleL[ ClipH( refOffset, refW, xRef + 4 ), yPosRL ] )
					  >>  shift1

…

If sps_ref_wraparound_enabled_flag is equal to 0, the sample value tempArray[ n ] with n = 0..3, is derived as follows:

tempArray[ n ] = ( fC[ xPhase, 0 ] * rlPicSampleC[ Clip3( 0, refWC − 1, xRef − 1 ), yPosRL ] +
				fC[ xPhase, 1 ] * rlPicSampleC[ Clip3( 0, refWC − 1, xRef ), yPosRL ] +
				fC[ xPhase, 2 ] * rlPicSampleC[ Clip3( 0, refWC − 1, xRef + 1 ), yPosRL ] +	(H-50)
				fC[ xPhase, 3 ] * rlPicSampleC[ Clip3( 0, refWC − 1, xRef + 2 ), yPosRL ] )  >>  shift1

Otherwise, the sample value tempArray[ n ] with n = 0..3, is derived as follows:

refOffset = ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY ) / SubWidthC

tempArray[ n ] = ( fC[ xPhase, 0 ] * rlPicSampleC[ ClipH( refOffset, refWC, xRef − 1 ), yPosRL ] +
				fC[ xPhase, 1 ] * rlPicSampleC[ ClipH( refOffset, refWC, xRef ), yPosRL ] +
				fC[ xPhase, 2 ] * rlPicSampleC[ ClipH( refOffset, refWC, xRef + 1 ), yPosRL ] +
				fC[ xPhase, 3 ] * rlPicSampleC[ ClipH( refOffset, refWC, xRef + 2 ), yPosRL ] )
				  >>  shift1

# Suggested further work

Optimal filters for resolution down-switching could be studied.

At this point of time, it is proposed to have a constraint to disallow using a resampled reference picture as a source for temporal motion vector prediction. The rate-distortion benefit of resampling picture motion and mode parameters and their use in temporal motion vector prediction could be studied.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).