| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0049-v2 |
| --- | --- |

| Title: | AHG16/Non-CE7: A study of bin to bit ratio in VTM4.0 and HM16.19 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou 16340 West Bernardo DR San Diego, CA-92127, USA | Tel: Email: | +1 858 521 5838 minhua.zhou@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

It is observed that the bin to bit ratio of the VTM4.0 is double digits percentage higher than that of the HM16.19, for both the CTC and low QP coding (QP =2, 7, 12, 17).  The summary results are as follows:

- For CTC AI/RA configurations, on average the weighted bin to bit ratio is 15.72%/17.39% higher than the HM16.19, and the unweighted bin to bit ratio is 12.38%/13.75% higher than the HM16.19, respectively.

- For low-QP AI/RA configurations, the weighted bin to bit ratio is 18.11%/17.17% higher than the HM16.19, and the unweighted bin to bit ratio is 10.45%/7.63% higher than the HM16.19, respectively.

The BD-rate reduction of the VTM4.0 relative to the HM16.19 at low QPs (QP =2, 7, 12, 17) is -12.39%, -13.99%, -13.69% and -14.69% in AI, RA, LD_B and LD_P configurations, respectively.

In the weighted bin to bit ratio, a bypass bin is counted at 0.25 context coded bins; in the unweight bin to bit ratio, a bypass bin and a context coded bin carry an equal weight (1:1).

# Introduction

For a given min compression ratio (MinCR) of a level, the bin to bit ratio dictates the required CABAC throughput (bins/sec) to meet the real time decoding. A higher bin to bit ratio implies that a CABAC engine needs to process bins at a faster pace in order to deal with the peak bit-rate in real time.

A study was carried out to measure the bin to bit ratio of the VTM4.0 relative to HM16.19, for both the CTC [1] and the low QP setting (QP = 2, 7, 12, 17).

# Experimental results

Tests were carried out by using the VTM4.0 and HM16.19 reference software. The CTC (common test conditions) defined in [1] was followed. Additional tests were conducted for low QPs of 2, 7, 12 and 17. The same test sequences and coding parameter settings were used for HM16.19.

The following data are collected on the decoder side:

- The peak context coded bin rate, bypass bin rate and bit-rate of a coded sequence

- The average context coded bin rate, bypass bin rate and bit-rate over a coded sequence.

- The average context coded bin rate, bypass bin rate and bit-rate consumed by the transform coefficients coding over a coded sequence.

In addition to the BD-rate, the following measurements are computed based on the collected data:

- Weighted BD-binrate for the peak case.

- Weighted BD-binrate for the average case.

- Weighted BD-binrate for the transform coefficients coding part.

- Weighted bin to bit ratio for the peak case.

- Unweighted bin to bit ratio for the peak case, for the average case, and for the transform coefficients coding part.

In the weighted measurements, a bypass bin is counted as 0.25 context coded bins. In the unweighted measurements, a bypass bin and a context coded bin carry an equal weight (1:1).

It has been observed that the bin to bit ratio for the peak case, for the average case and for the transform coefficients coding part is roughly the same.

Table 1: Summary results for CTC (VTM4.0 vs. HM16.19)

The AI and RA summary results for the VVC common testing conditions (CTC) are provided in Table 1. As shown in table 1, for same PSNR values, the VTM4.0 consumes less number of bins compared to the HM16.19. On average it is -7.97% and -21.36% less for AI and RA configurations, respectively. However, the bin to bits ratios (both weighted and unweighted) of the VTM4.0 are double digits percentage higher than that of the HM16.19.

Table 2: Summary results for low QPs (VTM4.0 vs. HM16.19)

In the low QP case (see Table 2), which is more close to the peak bit-rate case, the VTM4.0 consumes more bins for same PSNR values than the HM16.19, and the bin to bit ratio exhibits the similar behavior to the CTC case.

# Discussions

A higher bin to bit ratio generally requires a higher throughput CABAC engine for real time decoding. Compared to the HEVC, the VVC has added more coding tools and more syntax elements. It is unclear at this stage what has caused the higher bin to bit ratio in the VTM4.0. Although this is not an immediate concern, it is worth to investigate the root cause of such a phenomenon.

# References

[1] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, the 13th JVET meeting, Jan. 2019, Marrakesh, MA

# Patent rights declaration(s)

Broadcom Inc. does not have current or pending patent rights relating to the technology described in this contribution.