| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0050 |
| --- | --- |

| Title: | AHG17: On Forbidden-Zero Bit in NAL Unit Header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Stephan Wenger Byeongdoo Choi Shan Liu | Tel: Email: | stewe@stewe.org {bdchoi,shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

The authors propose removing the “forbidden_zero_bit” from the NAL unit header and re-allocating that bit for other purposes, such as an extension of the NAL unit type numbering space, unless compelling reasons are found and documented that continue to justify the bit’s existence.  We also wonder whether the restriction of the values for the temporal ID in the HEVC NAL unit header, and the complex language regarding SODB/RBSP (section 7.4.2.3 of JCTVC-M1001-v5) can be released/removed, as the authors’ admittedly dim recollection is that such restriction was introduced for essentially the same purpose.

# Request for Sanity Check

The forbidden_zero_bit in the NAL unit header was introduced ca 2002 by the JVT into the H.264 NAL unit header.  Our dim recollection is that the bit, at the time, was deemed required to prevent start code emulation issues in a single system environment, namely when encapsulating the NAL unit stream using Annex B, and transporting that Annex B byte stream over an MPEG-2 systems transport stream.

Having a single bit set aside to make a certain (at the time prominent) system environment happy does not sound too onerous.  However, if that bit results in an extension of a NAL unit header from, for example 8 bits to 16 bits (as the NUH should stay byte aligned), we would be talking about a rate-distortion impact that is not insignificant.  All for a single transport, that’s more and more losing its relevance?  We believe it’s time for a sanity check.

- Is the “forbidden_zero_bit” still technically required?

- If yes, can that be documented?

- Is there a design-around option, that, at the expense of a slight coding efficiency penalty for a bytestream transported over MPEG-2 systems, frees the forbidden_zero_bit for other system environments?

- Is there still value in restricting the numbering range if the temporal ID bitmask in the NUH (forcing a 1 bit)?  If there still value in the start code emulation prevention logic in section 7.4.2.2?

# Proposal

For now, we propose to remove the forbidden_zero_bit from the NUH, and reallocate the first bit of the NUH to the NAL unit type field in the NUH.  We do this mostly to make this contribution actionable.  The resulting syntax is as follows:

#### 7.3.1.1 NAL unit header syntax

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| nal_unit_type | u(6) |
| nuh_temporal_id_plus1 | u(3) |
| nuh_reserved_zero_7bits | u(7) |
| } |  |

#### 7.4.2.2    NAL unit header semantics

[…]

# Brainstorming / Closing Remarks

Given the need of rather close coordination with the MPEG-2 systems folks, we think that the time to address this issue is now, or pretty soon.  This is not a fix we as JVET should attempt last minute.

The authors do not claim to be experts on MPEG-2 systems, and what’s below is not necessarily all in the realms of JVET.  Still, we want to throw out a few ideas how to make VVC suitable for MPEG-2 transport even absent all this start code emulation stuff.

First, we believe that there is no other relevant transport anymore where start code emulation prevention (beyond the one of Annex B) is needed.  In particular, we think we can mentally sunset H.320 and its H.221 transport.  To the best of our knowledge, that’s the only other bit-oriented transport for video that mattered (in the somewhat distant past).

Second, in spec space and architecturally, if a transport imposes restrictions on a payload, it’s that’s transport’s job to create a suitable encapsulation, and not the payload’s job.  Insofar, arguably, adding start code emulation in H.264 (again beyond what was in Annex B) was perhaps not architecturally sound—though perhaps pragmatic.  Water under the bridge today, but no need to repeat the errors of the past, even if a technically sound implementation is readily at hand.

Third, while the theoretical need for start code emulation prevention is well understood, the real-world requirement may need to be studied (in MPEG systems, not here).  In particular, in an error free environment, start code emulation happens very rarely.  In such an environment, is the burden for the user so high if the synchronization to the transport stream happens one VVC NAL unit later than what would be optimal?  (I hope we are all in agreement that, once synchronization is reached, start code emulation does not matter).  Are we over-engineering here (and have over-engineered for the past 15 years)?  In the error prone environment—that is, in this case, the presence of bit errors—isn’t the normal reaction of an MPEG-2 systems system to throw the towel and wait for the next GOP-level synchronization point, a second or so later?  Do we really need to worry about start code emulation prevention on a per NAL unit level?

Fourth, options for encapsulation come readily to mind.  As a naïve example, if you were wrapping the Annex B bytestream into another Annex B bytestream, the start code emulation prevention problem that’s currently solved by the draft VVC spec (as well as by H.264 and HEVC) through forbidden_zero, TID != 0, and all that fancy stuff in SODB vs. RBSP would be solved for good, no?  Granted, with not insignificant overhead, but it would be solved…  Better solutions could obviously be designed as well by those interested (in MPEG-2 systems, not JVET).

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).