| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0520 |
| --- | --- |

| Title: | AHG17: On NAL unit header design for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Stephan Wenger Byeongdoo Choi Shan Liu | Tel: Email: | stewe@stewe.org {bdchoi,shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes a NAL unit header design for VVC, following the guidance of H.264/SVC and MVC’s syntax.  In particular, certain NAL unit types trigger the extension of the NAL unit header from minimal 8 bits to larger (still octet aligned) structures.  It is asserted that such a design is a) extensible towards technologies such as SVC and MVC without spending bits or a significant amount of numbering space on “hooks”, b) allows for the conditional presence of syntax elements not found in H.264’s and HEVC’s NAL unit headers, based on application needs, and c) is still lightweight enough to be efficiently implemented in a MANE (and trivially implementable in a decoder).

# Problem statement

HEVC’s NAL unit header is 16 bits in length, several of which are in non-scalable/Multiview environments set to zero.  Further, there is a large numbering space reserved for Nal Unit Types (NUTs).  Many such NUTs are “exotic” in that they are used to indicate picture types that, while perhaps useful for some applications, rarely used in many others.  A reduction of the most basic NAL unit header back to 8 bits for the most mainstream NAL units would be desirable.

# Basic NUH design

This proposal is somewhat of a strawman in that the syntax is not yet fully developed.  However, that can be seen both as a problem and as an opportunity for tinkering with it by the open-minded.

We define a number of classes of NAL units.  Using HEVC’s NAL unit types as an example, we distinguish the following classes

| Class | NAL Unit Class | Associated HEVC-style NUTs |
| --- | --- | --- |
| 0 | TRAIL_N | TRAIL_N |
| 1 | TRAIL_R | TRAIL_R |
| 2 | PREFIX_SEI_NUT | PREFIX_SEI_NUT |
| 3 | SUFFIC_SEI_NUT | SUFFIX_SEI_NUT |
| 4 | LeadingPictures | RADL_N, RADL_R |
| 5 | SwitchingPictures | TSA_N, TSA_R, STSA_N, STSA_R |
| 6 | Random Access Pictures | BLA_W_LP, BLA_W_RADL, BLA_N_LP, IDR_W_RADL, IDR_N_LP, CRA_NUT |
| 7 | Parameter Sets | (DPS_NUT), VPS_NUT, SPS_NUT, PPS_NUT |
| 8 | Markers | AUD_NUT, EOS_NUT, EOB_NUT, FD_NUT |
| 9 | Complex | (Discussed below) |
| 10..11 | Reserved | Reserved for extensions by ITU/ISO/IEC |
| 12..15 | Unspecified | (proprietary extensions) |

The most basic NAL unit header syntax according to this proposal could look approximately like the following:

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nuh_unit_class | u(4) |
| nuh_temporal_id_plus1 | u(3) |
| if( nuh_unit_class == 4 ) { |  |
| nuh_type_of_leading_picture /* 0 for RADL_N, 1 for RADL_R | u(1) |
| reserved | u(7) |
| } |  |
| if( nuh_unit_class == 5) { |  |
| nuh_type_of_switching_pictures | u(3) |
| reserved | u(5) |
| } |  |
| if( nuh_unit_class == 6) { |  |
| nuh_random_access_pictures | u(3) |
| reserved | u(5) |
| } |  |
| /* …and so forth */ |  |
| } |  |

The above looks just like H.264’s non-scalable/Multiview NUHs, except that NAL unit type is renamed to NAL unit class.  For the most commonly used NAL unit types (we believe those would be trailing pictures and SEI messages), the NAL unit header is 8 bits in length.  The more exotic picture types as well as (in our estimation) less frequently used non-VCL NAL unit types are grouped into classes.  Each class receives a class ID.  Using a sufficiently low number of classes, 16 codepoints or four bits for the class ID is sufficient, leaving three bits for the temporal ID.

Within a class, we need to identify the NAL unit in question.  In some cases, for example for parameter sets, we could re-use the (meaningless, for parameter sets) bits of the temporal ID.  We mention this as one option.  The method described above, however, is perhaps a bit cleaner and certainly more orthogonal.  It triggers at least one octet to follow the initial octet.  In the syntax table above, that second octet is populated only by a NUT inside each class, and the size of that NUT is selected to allow for the HEVC types.  Of course, depending on the number of various nal unit types in each class in the final design, the number of bits could be adjusted.

# Provisions for additional NAL-unit based syntax elements

Elsewhere [1][2][3] various syntax elements were introduced that could be useful in MANEs and, therefore, should advantageously be carried outside the start code emulation prevention protection, meaning inside the NUH.

Consider as an example a tile-based 360 environment where a MANE may wish to discard certain tiles from the bitstream.  In order to do so, according to our understanding of today’s basic design, the MANE would need to understand the tile layout, and would need to identify the tile based on the “address” of the tile’s first CU.  The former requires, as a minimum, to follow the activation sequence of the parameter sets, and that requires interpreting parts of the tile header, and keeping state in the MANE.  Both is undesirable.

What if a tile ID would be stored in the NAL unit header?  In that case, the smart endpoint (that knows exactly what tiles it wants, and has a decoder built in to deal with complex data structures, parameter set activation and so forth) could instruct the MANE to dump NAL units with those certain tile IDs, and forward the rest.

Similarly, while our focus right now is not scalability nor Multiview, we believe that something like a layer_id will sooner or later become relevant.  Using the class-based approach, both can relatively easily be implemented, though there is one tricky issue—namely start code emulation prevention—that has to be addressed.  Thoughts on that will follow towards the end of this document.

In order to support additional, fixed format syntax elements, we can allocate one class as “complex NUH” class.  The class triggers the presence of an octet wherein the first n bits are allocated to a set of flags, some of which could be reserved.  Following those flags is a NUT, and that is followed by the syntax elements whose presence is indicated by the flags.

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nuh_unit_class | u(4) |
| nuh_temporal_id_plus1 | u(3) |
| . . . |  |
| if( nuh_unit_class == 9 ) { /* Complex */ | u(1) |
| nuh_layer_id_present_flag | u(1) |
| nuh_tile_id_present_flag | u(1) |
| nuh_riffraff_present_flag | u(1) |
| reserved_two_bits_for_future_extensions | u(2) |
| if( nuh_layer_id_present_flag) |  |
| layer_id | u(6) |
| if( tile_id_present_flag ) |  |
| tile_id | u(8) |
| if(riffraff_present_flag) |  |
| riffraff | u(2) |
| } |  |
| byte_align() |  |
| } |  |

# Start Code Emulation Prevention

NAL unit headers, in contrast to the NAL unit payload, are outside of the start code emulation prevention envelope.  One (and perhaps the main) reason for this design choice was that a MANE dealing with data formats other than Annex B bitstreams should not need to worry about start code stuff.  We believe this was a good design choice.  However, when allowing for a variable length, and possibly longer than 3 octet long NAL unit header, we could run into start code emulation issues.

Conceptually, we see five solutions to this problem.  None are particularly nice.  All, we believe, could be made work.

- Ensure through carefully chosen syntax that start code emulation does not happen.  We did some of that in the H.264/SVC header, and also in the HEVC header (IIRC, that’s the reason why we have a maximum of 6 temporal layers and not 7).  Doing so with a variable length header is not easy but probably doable.  A final design can only be provided once the length of fields in the optional header are known.

- Place any NUH octet after the third octet into the start code emulation prevention.

- Perhaps the start code itself could be extended?  With the decreasing relevance of Annex B bytestreams, that may be doable.  However, what about start codes outside the video spec space?

- Prevent the (ab-)use of overly long NUHs by encoders even if the basic syntax would allow such.  “It is a requirement for bitstream conformance that the NUH be at most three octets in length”.

- Put up large big warning signs into the Annex B saying “If your system creates NAL unit headers longer than 3 octets, this Annex shall not be used”.

# References

- B. Choi, S. Wenger, S. Liu, “AHG12: On tile group identification for VVC,” JVET-M0155, Marrakech, Jan. 2019.

- B. Choi, S. Wenger, S. Liu, “AHG17: On component type indication for VVC,” JVET-M0156, Marrakech, Jan. 2019.

- B. Choi, S. Wenger, S. Liu, “AHG17: On picture order count for VVC,” JVET-M0157, Marrakech, Jan. 2019.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).