| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0051 |
| --- | --- |

| Title: | AHG17: On NAL Unit header design for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Stephan Wenger Byeongdoo Choi Shan Liu | Tel: Email: | stewe@stewe.org {bdchoi,shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes an update to the NAL unit header design for VVC.  At the expense of an—the authors assert insignificant—increase in processing power demands, and certain limitations in extensibility, the NAL Unit header length can be kept at 8 bits.  Extensibility concerns can be addressed by using an extension mechanism similar to the one proposed in JVET-M0520 of the Marrakech meeting.

The reduction of the NAL unit header size from 16 bits to 8 bits results in bitrate savings.  Quantifying those bitrate savings is difficult as they depend highly on the number of tile groups per coded picture, number of SEI messages and marker NAL units per picture, GOP structure and resulting parameter set NAL units for GOP, and so forth.

# Introduction

The current VVC draft’s NAL unit header is 16 bits in length, seven of which are reserved.  Of the nine bits carrying information, a number of codepoints are also reserved and certain combination of the bits are meaningless.  For example, there is no point in signaling a temporal ID other than 0 for a NAL unit belonging to an IRAP picture, or for a sequence parameter set NAL unit.

NAL unit headers ought to be relatively simple in their design to be easily processable by Media Aware Network Elements such as Selective Forwarding Units, file rewriters, and the like (MANEs henceforth).  For example, Exp-Golomb or arithmetic coded syntax elements have been avoided in NAL unit header designs since H.264 for very good and valid reasons.  Variable length binary coded codewords may be OK, but only if the NAL unit header as a whole remains self-contained.  (A profile dependency may be acceptable as long as the parameter set NAL units carrying the profile information (DPS?, VPS?, SPS) is self-contained—but that is not our preference.)  As a result, we see no requirement and no justification any more that a given set of bits in a NAL unit header necessarily must carry the same semantics, independent of NAL unit types (or other information for that matter, as long as such other information is also in the NAL unit header).

We therefore assert that, with the complexity currently in the VVC draft, it is possible to reduce the NAL unit header from 16 bits to 8 bits, without losing functionality and without undue implementation or processing burden.  We do this by collecting NAL units of similar purpose—for example parameter sets—into one NAL unit type class, and use the temporal ID field as the demultiplex point.

The question of extensibility has been partly addressed already in JCET-M0520.  In short, for certain less frequently used NAL units, a single NAL unit type (NUT) could be employed that triggers the presence of one or more additional octets of NAL unit header information.  This would work well enough for extensions, especially since, according to this proposal, there are still a number of unallocated NUTs available.

# Design

Looking at the NAL unit type (NUT) values currently defined in VVC, we observe that of the 32 codepoints available, 15 NUT values are currently in use.  Of those only four indicate NAL unit types for which the temporal ID is meaningful.  For the other 11 NUTs, the temporal ID is either forced to be 0, or ignore by the decoder.

We propose to

- collect all IRAP picture NAL units types (currently three: IDR_W_RADL, IDR_N_LP, and CRA_NUT, into a NAL unit type class “IRAP_class”, and differentiate between the NAL unit types of this class through values 0, 1, and 2 in the temporal ID field.

- Same for parameter sets into a PARSET_Class (collecting SPS, PPS, APS, and VPS, DPS once agreed and included in the draft, plus whatever other parameter sets we may come up with up to a limit of eight types);

- Same for markers, AUD_NUT, EOS_NUT, EOB_NUT; and

- Same for SEI messages, PREFIX_SEI_NUT, and SUFFIX_SEI_NUT.

The result could look as follows:

| NALU Class | NALU Class | tid_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- | --- | --- |
| 0 | TRAIL_Class | TID | TRAIL_NUT | Coded tile group of a non- STSA trailing picture tile_group_layer_rbsp( ) | VCL |
| 1 | STSA_Class | TID | STSA_NUT | Coded tile group of an STSA picture tile_group_layer_rbsp( ) | VCL |
| 2 | RASL_Class | TID | RASL_NUT | Coded tile group of a RASL picture tile_group_layer_rbsp( ) | VCL |
| 3 | RADL_Class | TID | RADL_NUT | Coded tile group of a RADL picture tile_group_layer_rbsp( ) | VCL |
| 4 | IRAP_Class | 1 2 3 | IDR_W_RADL IDR_N_LP CRA_NUT | Coded tile group of an IDR picture tile_group_layer_rbsp( ) Coded tile group of a CRA picture tile_group_layer_rbsp( ) | VCL |
| 5 | PARSET_Class | 1 2 3 | SPS_NUT PPS_NUT APS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) Picture parameter set pic_parameter_set_rbsp( ) Adaptation parameter set adaptation_parameter_set_rbsp( ) | non-VCL |
| 6 | MARKER_Class | 1 2 3 | AUD_NUT EOS_NUT EOB_NUT | Access unit delimiter access_unit_delimiter_rbsp( ) End of sequence end_of_seq_rbsp( ) End of bitstream end_of_bitstream_rbsp( ) | non-VCL |
| 7 | SEI_Class | 1 2 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| 9..12 | Reserved_Class |  |  |  |  |
| 13..15 | Unspecified_Class |  |  |  |  |

Above table would replace table 7-1 of JVET-M1001-v6.

A few additional changes are required in the spec, as follows:

Global replace nal_unit_type with a variable NalUnitType.

In section 7.3.1.2

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_class | u(4) |
| tid_type | u(3) |
| } |  |

In section 7.4.2.2:

forbidden_zero_bit shall be equal to 0.

nal_unit_class specifies either the type of RBSP data structure contained in the NAL unit, or the class of types of RBSP data structures as specified in Table 7 1.

NAL units that have nal_unit_class in the   Unspecified_class, , for which semantics are not specified, shall not affect the decoding process specified in this Specification.

NOTE 1 – NAL unit types in the Unspecified_class may be used as determined by the application. No decoding process for these values of nal_unit_type is specified in this Specification. Since different applications might use these NAL unit types for different purposes, particular care must be exercised in the design of encoders that generate NAL units with these nal_unit_type values, and in the design of decoders that interpret the content of NAL units with these nal_unit_type values. This Specification does not define any management for these values. These nal_unit_type values might only be suitable for use in contexts in which "collisions" of usage (i.e., different definitions of the meaning of the NAL unit content for the same nal_unit_type value) are unimportant, or not possible, or are managed – e.g., defined or managed in the controlling application or transport specification, or by controlling the environment in which bitstreams are distributed.

For purposes other than determining the amount of data in the decoding units of the bitstream (as specified in Annex C), decoders shall ignore (remove from the bitstream and discard) the contents of all NAL units whose nal_unit_class in the Unspecified_class..

NOTE 2 – This requirement allows future definition of compatible extensions to this Specification.

tid_type specifies, for NAL unit type classes where “TID” is indicated as tid_type in Table 7-3, the temporal id, as as pecified below.  In other cases (NAL unit type classes where values other than “TID” is indicated as tid_type in Table 7-3), tid_type shall be used an indicator for the NAL unit type as specified below.

The variables TemporalID and NalUnitType are derived as follows:

TemporalID = 0;

if ( nal_unit_class == TRAIL_Class ) {

NalUnitType = TRAIL_NUT; TemporalId = tid_type – 1;

} else if ( nal_unit_class == STSA_Class ) {

NalUnitType = STSA_NUT; TemporalId = tid_type – 1;

} else if ( nal_unit_class == RASL_Class ) {

NalUnitType = RASL_NUT; TemporalId = tid_type – 1;

} else if ( nal_unit_class == RADL_Class ) {

NalUnitType = RADL_NUT; TemporalId = tid_type – 1;

} else if ( nal_unit_class == IRAP_Class ) {

if( tid_type == 1 )

NalUnitype = IDR_W_RADL;

else if ( tid_type == 2 )

NalUnitType = IDR_N_LP;

else if ( tid_type == 3 )

NAlUnitType = CRA_NUT;

} else if …  // to be completed; can do during the meeting if this moves forward

(7-1)

The value of TemporalId shall be the same for all VCL NAL units of an access unit. The value of TemporalId of a coded picture or an access unit is the value of the TemporalId of the VCL NAL units of the coded picture or the access unit.

The value of TemporalId for non-VCL NAL units is constrained as follows:

–	Otherwise, if nal_unit_type is equal to APS_NUT, TemporalId shall be inferred to that of the access unit containing the NAL unit.

NOTE – When the NAL unit is a non-VCL NAL unit, the value of TemporalId is equal to the minimum value of the TemporalId values of all access units to which the non-VCL NAL unit applies. When nal_unit_type is equal to PPS_NUT, TemporalId may be greater than or equal to the TemporalId of the containing access unit, as all picture parameter sets (PPSs) may be included in the beginning of a bitstream, wherein the first coded picture has TemporalId equal to 0. When nal_unit_type is equal to PREFIX_SEI_NUT or SUFFIX_SEI_NUT, TemporalId may be greater than or equal to the TemporalId of the containing access unit, as an SEI NAL unit may contain information that applies to a bitstream subset that includes access units for which the TemporalId values are greater than the TemporalId of the access unit containing the SEI NAL unit.

# Extensibility

Please see JVET_M0520 (included in the .zip file for convenience).  The basic idea is to grab one of  the reserved NAL unit classes, and give it semantics so to indicate the presence of an initial octet of data in the NUT.  In that octet one can put whatever additional demux point one needs.  If we can come up with a Nal unit type that is likely to occur so frequently that an 8 bit header makes sense, we can of course also burn a full codepoint of nal_unit_class for that NUT, like we propose for trailing pictures.

# Expected bitrate savings

No tests were performed, so this analysis is on paper only.  In short, if there were any significant technical downside with this proposal, we would not made it.  The bitrate savings are small—too small for significant changes.  However, there are savings, in each and every case, so why leave those on the table when the technical impact is so small?

Compared to the current WD, we save 8 bits per NAL unit.  Clearly, that saves bits.  How does that translate to bitrate savings for a whole session?  That depends obviously on the number of NAL units in relation to the bitrate, that is the average size of the NAL units.  Divide 8 bits by the average size of the NAL units (in bits), and you get the saving.  How much is it in practice?  That depends on the application.

Approximately worst case for supporting our proposal are our common conditions, and especially the low delay common conditions.  Negligibly few parameter sets (two per GOP, or even two per stream), no retransmission of parameter sets, no SEI messages, and one tile group (== one NAL unit) per picture.  For the low delay case (parameter sets included only once for the whole stream), we end up in one NUH per coded picture plus minor startup overhead.  At 60 fps, that’s 60 NUHs or 480 bit/s savings, and for a 1 Mbit/s stream we end up with an overhead of 0.048% savings.  Not worth to talk about.

In the real world, things change bit.  Consider a video conferencing scenario: 384 kbit/s, 60 Hz, one TG per picture, and you are already at 0.125% savings.  However, that assumes no parameter set retransmission, no SEI messages, and no similar overhead.  If we were putting in AUDs, and one APS per picture, we are at 0.375% and things start to get moderately interesting.  Still no SEI messages, no parameter sets except the APS (and no parameter set retransmission), etc.  Adding those in, conceivably we are talking about perhaps 0.5%.

For the entertainment guys with random access and GOPs, they have to factor in these SEIs that are in common use (so we hear), parameter sets for each GOP, AUDs and so on.  Expect higher savings than for the video conference case, at resolutions of 1080p and below.  If people use only one TG (resulting in one NAL unit) at higher resolutions and resulting bitrates, then gains diminish, but if the decisions in VVC were similar as in HEVC, where tiles were mandated for high resolutions, savings well above 0.1% even at 4k resolutions are likely if those tiles were put into their own respective tile groups (which would be prudent for Internet-based streaming).

Looking into the future, the 360 crowd has approached us with proposals that suggest 96 sub-pictures / NAL units for a coded picture, where sub-picture is comparatively small in samples and bitrate (low resolutions of 512x512 or 1024x1024 per cube surface have been considered by several groups).  We can expect that those relatively small tile groups can often be coded at comparatively high coding efficiency (compared to single 512 / 1024 square sequences) because many folks expect prediction gains from other cube surfaces in the same picture.  Ideally cube surface ought to be extracted in the compressed domain, implying one NAL unit per surface.  Here, our savings could be well above 1%.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).