| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0052 |
| --- | --- |

| Title: | [AHG19] On Signaling of Adaptive Resolution Change |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Stephan Wenger Byeongdoo Choi Shan Liu | Tel: Email: | stewe@stewe.org {bdchoi,shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

Proposed is the inclusion of Adaptive Resolution Change (ARC) into the VVC working draft. Text is provided with focus on the signalling aspect.  Simulation results are provided for Class A sequences, random access, with the IDR picture subsampled by a factor of two in both dimensions.  For some class A1 sequences, substantial luma gains are reported, with substantial chroma loss.  For class A2 sequences, there are substantial losses.  ARC is proposed as an architectural enhancement to improve reconstructed video quality when the codec is operated nears its break point, where artifacts become clearly visible.

# Introduction

Adaptive resolution change, as a concept in video compression standards, has been around since at least 1996; in particular H.263+ related proposals towards reference picture resampling (RPR, Annex P) and Reduced Resolution Update (Annex Q).  It has recently gained a certain prominence, first with proposals by Cisco during the JCT-VC time, then in the context of VP9 (where it is moderately widely deployed nowadays), and more recently in the context of VVC.  ARC allows reducing the number of samples required to be coded for a given picture, and upsampling the resulting reference picture to a higher resolution when such is desirable.

We consider ARC of particular interest in two scenarios:

- Intra coded pictures, such as IDR pictures are often considerably larger than inter pictures.  Downsampling pictures intended to be intra coded, regardless of reason, may provide a better input for future prediction.  It’s also clearly advantageous from a rate control viewpoint, at least in low delay applications.

- When operating the codec near its breaking point, as at least some cable and satellite operators routinely seem to do, ARC may become handy even for non-intra coded pictures, such as in scene transitions without a hard transition point.

- Looking perhaps a bit too much forward: is the concept of a fixed resolution generally defensible?  With the departure of CRTs and the ubiquity of scaling engines in rendering devices, the hard bind between rendering and coding resolution is a thing of the past.  Also, we note that there is certain research available that suggests that most people are not able to concentrate on fine details (associated perhaps with high resolution) when there is a lot of activity going on in the video sequence, even if that activity is elsewhere spatially.  If that were true and generally accepted, fine granularity resolution changes may be a better rate control mechanism than adaptive QP.  We put this point out for discussion at this time, as we lack data—feedback from those in the knows is appreciated.  Of course, doing away with the concept of fixed resolution bitstreams has a myriad of system layer and implementation implications, of which (at least at the level of their existence, if not their detailed nature) we are well aware of.

Technically, ARC can be implemented as reference picture resampling.  Implementing reference picture resampling has two major aspects: the resampling filters, and the signaling of the resampling information in the bitstream.  This document focusses on the latter and touches the former only to the extent we have implementation experience.  More study of suitable filter design is encouraged, and Tencent will carefully consider and, when appropriate, support any proposals in that regard that substantially improve the strawman design provided by us (and, arguably, others).

# Overview of Tencent’s ARC implementation

Fig. 1. Encoder modification for ARC

Fig. 2. Decoder modification for ARC

Figures 1 & 2 illustrate Tencent’s ARC en-/decoder implementation, respectively. In our implementation, it is possible to change the picture width and height, on a per picture granularity irrespective of the picture type. At the encoder, the input image data is down-sampled to the selected picture size for the current picture encoding. After the first input picture is encoded as intra-picture, the decoded picture is stored in the decoded picture buffer (DPB). When the consequent picture is down-sampled with a different sampling ratio and encoded as inter-picture, the reference picture(s) in the DPB is/are up-/down-scaled according the spatial ratio between the picture size of the reference and the current picture size. At the decoder, the decoded picture is stored in the DPB without resampling. However, the reference picture in the DPB is up-/down-scaled in relation to the spatial ratio between the currently decoded picture and the reference, when used for motion compensation. The decoded picture is up-sampled to the original picture size or the desired output picture size when bumped out for display. In motion estimation/compensation process, motion vectors are scaled in relation to picture size ratio as well as picture order count difference.

# Signaling of ARC parameters

The term ARC parameters is used herein as a combination of any parameters required to make ARC work.  In the easiest case, that could be a zoom factor, or an index into a table with defined zoom factors.  It could be a target resolution (for example in sample or max CU size granularity), or an index into a table providing a target resolution, like what was proposed in JVET-M0135.  Also included would be filter selectors or even filter parameters (all the way up to filter coefficients) of the up/down-sampling filters in use.

From the outset, we propose herein to allow, at least conceptually, different ARC parameters for different parts of a picture.  We propose that the appropriate syntax structure as per the current VVC draft would be a rectangular tile group (TG).  Those folks using scan-order TGs would be restricted to use ARC only to a full picture, or to the extent scan order TGs are included in a rectangular TG (we don’t recall that TG nesting has been discussed so far, and perhaps it’s a bad idea).  That can easily be specified by a bitstream constraint.

As different TGs may have different ARC parameters, the appropriate place for ARC parameters would be either in the TG header or in a parameter set with the scope of a TG, and referenced by the TG header—the Adaptation Parameter Set in the current VVC draft, or a more detailed reference (an index) into a table in a higher parameter set.  Of these three choices, we propose, at this point, to use the TG header to code a reference to a table entry including the ARC parameters, and that table be located in the SPS, with maximum table values coded in the (forthcoming) DPS.  We can live with coding a zoom factor directly into the TG header, without using any parameter set values.  The use of the PPS for the reference, as proposed in JVET-M0135, is counter-indicated if, as we do, the per tile group signaling of ARC parameters is a design criterion.

As for the table entries themselves, we see many options:

- coding down-sample factors, either one for both dimension or independently in X and Y dimension?  That’s mostly a (HW-) implementation discussion, and some would perhaps prefer an outcome where the zoom factor in X dimension is fairly flexible, but in Y dimension is fixed to 1, or has very few choices.  We suggest that the syntax is the wrong place for expressing such constraints and, if they were desirable, we prefer the constraints expressed as requirements for conformance.  In other words, keep the syntax flexible.

- Coding target resolutions.  That’s what we propose below.  There could be more or less complex constraints about those resolutions in relation to the current resolution, expressed perhaps in bitstream conformance requirements.

- Down-sampling per tile group is preferred to allow for picture composition/extraction.  However, it is not critical from a signaling viewpoint.  If the group were making the unwise decision of allowing ARC only at picture granularity, we can always include a requirement for bitstream conformance that all TGs use the same ARC parameters.

- Control information related to ARC.  In our design below, that includes the reference picture size.

- Do we need to have flexibility in filter design?  Anything bigger than a handful of codepoints?  If yes, put those into APS?  (No, please no APS update discussion again.  If down-sample filter changes and ALF stays, we propose that bitstream has to eat the overhead.)

For now, in order to keep proposed technology aligned and simple (to the extent possible), we propose

- Fixed filter design

- Target resolutions in a table in the SPS, with bitstream constraints TBD.

- Minimum/Maximum target resolution in DPS to facilitate cap exchange/negotiation.

The resulting syntax could look as follows:

#### Decoder parameter set RBSP syntax

| dec_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| max_pic_width_in_luma_samples | ue(v) |
| max_pic_height_in_luma_samples | ue(v) |
| ... |  |
| } |  |

max_pic_width_in_luma_samples specifies the maximum width of decoded pictures in units of luma samples in the bitstream. max_pic_width_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY. The value of dec_pic_width_in_luma_samples[ i ] cannot be greater than the value of max_pic_width_in_luma_samples.

max_pic_height_in_luma_samples specifies the maximum height of decoded pictures in units of luma samples. max_pic_height_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY. The value of dec_pic_height_in_luma_samples[ i ] cannot be greater than the value of max_pic_height_in_luma_samples.

#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| adaptive_pic_resolution_change_flag | u(1) |
| if(adaptive_pic_resolution_change_flag) { |  |
| output_pic_width_in_luma_samples | ue(v) |
| output_pic_height_in_luma_samples | ue(v) |
| reference_pic_size_present_flag | u(1) |
| if(reference_pic_size_present_flag) |  |
| { |  |
| reference_pic_width_in_luma_samples | ue(v) |
| reference_pic_height_in_luma_samples | ue(v) |
| } |  |
| num_dec_pic_size_in_luma_samples_minus1 | ue(v) |
| for( i = 0; i <= num_dec_pic_size_in_luma_samples_minus1; i++ ) { |  |
| dec_pic_width_in_luma_samples[ i ] | ue(v) |
| dec_pic_height_in_luma_samples[ i ] | ue(v) |
| } |  |
| } |  |
| else { |  |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| } |  |
| ... |  |
| } |  |

adaptive_pic_resolution_change_flag equal to 1 specifies that an output picture size (output_pic_width_in_luma_samples, output_pic_height_in_luma_samples), an indication of the number of decoded picture sizes (num_dec_pic_size_in_luma_samples_minus1) and at least one decoded picture size (dec_pic_width_in_luma_samples[ i ], dec_pic_height_in_luma_samples[ i ]) are present in the SPS.  A reference picture size (reference_pic_width_in_luma_samples, reference_pic_height_in_luma_samples) is present conditioned on the value of reference_pic_size_present_flag.

output_pic_width_in_luma_samples specifies the width of the output picture in units of luma samples. output_pic_width_in_luma_samples shall not be equal to 0.

output_pic_height_in_luma_samples specifies the height of the output picture in units of luma samples. output_pic_height_in_luma_samples shall not be equal to 0.

reference_pic_size_present_flag equal 1 specifies that reference_pic_width_in_luma_samples and reference_pic_height_in_luma_samples are present.

reference_pic_width_in_luma_samples specifies the width of the reference picture in units of luma samples. output_pic_width_in_luma_samples shall not be equal to 0. If not present, the value of reference_pic_width_in_luma_samples is inferred to be equal to dec_pic_width_in_luma_samples[ i ].

reference_pic_height_in_luma_samples specifies the height of the reference picture in units of luma samples. output_pic_height_in_luma_samples shall not be equal to 0. If not present, the value of reference_pic_height_in_luma_samples is inferred to be equal to dec_pic_height_in_luma_samples[ i ].

NOTE1 – The size of the output picture shall be equal to the values of output_pic_width_in_luma_samples and output_pic_height_in_luma_samples. The size of the reference picture shall be equal to the values of reference_pic_width_in_luma_samples and _pic_height_in_luma_samples, when the reference picture is used for motion compensation.

num_dec_pic_size_in_luma_samples_minus1 plus 1 specifies the number of the decoded picture size (dec_pic_width_in_luma_samples[ i ], dec_pic_height_in_luma_samples[ i ] ) in units of luma samples in the coded video sequence.

dec_pic_width_in_luma_samples[ i ] specifies the i-th width of the decoded picture sizes in units of luma samples in the coded video sequence. dec_pic_width_in_luma_samples[ i ] shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

dec_pic_height_in_luma_samples[ i ] specifies the i-th height of the decoded picture sizes in units of luma samples in the coded video sequence. dec_pic_height_in_luma_samples[ i ] shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

NOTE2 – The i-th decoded picture size (dec_pic_width_in_luma_samples[ i ], dec_pic_height_in_luma_samples[ i ] ) may be equal to the decoded picture size of the decoded picture in the coded video sequence.

### Tile group header syntax

| tile_group_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| if(adaptive_pic_resolution_change_flag) { |  |
| dec_pic_size_idx | ue(v) |
| } |  |
| ... |  |
| } |  |

dec_pic_size_idx specifies that the width of the decoded picture shall be equal to pic_width_in_luma_samples[ dec_pic_size_idx ] and the height of the decoded picture shall be equal to pic_height _in_luma_samples[ dec_pic_size_idx ].

# Filters

The proposed design conceptually includes four different filter sets: down-sampling filter from the original picture to the input picture, up-/down-sampling filters to rescale reference pictures for motion estimation/compensation, and up-sampling filter from the decoded picture to the output picture. The first and last ones can be left as non-normative matters. In the scope of specification, up-/down-sampling filters need to be explicitly signaled in an appropriate parameter set, or pre-defined.

Our implementation uses the down-sampling filters of SHVC (SHM ver. 12.4), which is a 12-tap and 2D separable filter, for down-sampling to resize the reference picture to be used for motion compensation. In the current implementation, only dyadic sampling is supported. Therefore, the phase of the down-sampling filter is set equal to zero by default. For up-sampling, 8-tap interpolation filters are used, with 16-phases, to shift the phase and align the luma and chroma pixel positions to the original positions.

Tables 1&2 provide the 8-tap filter coefficients fL[ p, x ] with p = 0..15 and x = 0..7 used for the luma up-sampling process, and the 4-tap filter coefficients fC[ p, x ] with p = 0..15 and x = 0..3 used for the chroma up-sampling process.

Table 3 provides the 12-tap filter coefficients for down-sampling process. The same filter coefficients are used for both luma and chroma for down-sampling.

Table 1. Luma up-sampling filter with 16 phases

| Phase p | Interpolation filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fL[ p, 0 ] | fL[ p, 1 ] | fL[ p, 2 ] | fL[ p, 3 ] | fL[ p, 4 ] | fL[ p, 5 ] | fL[ p, 6 ] | fL[ p, 7 ] |
| 0 | 0 | 0 | 0 | 64 | 0 | 0 | 0 | 0 |
| 1 | 0 | 1 | −3 | 63 | 4 | −2 | 1 | 0 |
| 2 | −1 | 2 | −5 | 62 | 8 | −3 | 1 | 0 |
| 3 | −1 | 3 | −8 | 60 | 13 | −4 | 1 | 0 |
| 4 | −1 | 4 | −10 | 58 | 17 | −5 | 1 | 0 |
| 5 | −1 | 4 | −11 | 52 | 26 | −8 | 3 | −1 |
| 6 | −1 | 3 | −9 | 47 | 31 | −10 | 4 | −1 |
| 7 | −1 | 4 | −11 | 45 | 34 | −10 | 4 | −1 |
| 8 | −1 | 4 | −11 | 40 | 40 | −11 | 4 | −1 |
| 9 | −1 | 4 | −10 | 34 | 45 | −11 | 4 | −1 |
| 10 | −1 | 4 | −10 | 31 | 47 | −9 | 3 | −1 |
| 11 | −1 | 3 | −8 | 26 | 52 | −11 | 4 | −1 |
| 12 | 0 | 1 | −5 | 17 | 58 | −10 | 4 | −1 |
| 13 | 0 | 1 | −4 | 13 | 60 | −8 | 3 | −1 |
| 14 | 0 | 1 | −3 | 8 | 62 | −5 | 2 | −1 |
| 15 | 0 | 1 | −2 | 4 | 63 | −3 | 1 | 0 |

Table 2. Chroma up-sampling filter with 16 phases

| Phase p | Interpolation filter coefficients |  |  |  |
| --- | --- | --- | --- | --- |
|  | fC[ p, 0 ] | fC[ p, 1 ] | fC[ p, 2 ] | fC[ p, 3 ] |
| 0 | 0 | 64 | 0 | 0 |
| 1 | −2 | 62 | 4 | 0 |
| 2 | −2 | 58 | 10 | −2 |
| 3 | −4 | 56 | 14 | −2 |
| 4 | −4 | 54 | 16 | −2 |
| 5 | −6 | 52 | 20 | −2 |
| 6 | −6 | 46 | 28 | −4 |
| 7 | −4 | 42 | 30 | −4 |
| 8 | −4 | 36 | 36 | −4 |
| 9 | −4 | 30 | 42 | −4 |
| 10 | −4 | 28 | 46 | −6 |
| 11 | −2 | 20 | 52 | −6 |
| 12 | −2 | 16 | 54 | −4 |
| 13 | −2 | 14 | 56 | −4 |
| 14 | −2 | 10 | 58 | −2 |
| 15 | 0 | 4 | 62 | −2 |

Table 3. Down-sampling filter coefficient for luma and chroma

| Phase p | Down-sampling filter coefficients |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | f[ p, 0 ] | f[ p, 1 ] | f[ p, 2 ] | f[ p, 3 ] | f[ p, 4 ] | f[ p, 5 ] | f[ p, 6 ] | f[ p, 7 ] | f[ p, 8 ] | f[ p, 9 ] | f[ p, 10] | f[ p, 11] |
| 0 | 1 | -6 | -5 | 7 | 26 | 41 | 41 | 26 | 7 | -5 | -6 | 1 |

We have not experimented with other filter designs.  We anticipate that (perhaps significant) subjective and objective gains can be expected when using filters adaptive to content and/or to scaling factors.

# Tile Group boundary discussions

As it is perhaps true with a lot of tile group related work, our implementation is not quite finished with respect to tile group (TG) based ARC.  Our preference is to revisit that implementation once the discussion of spatial composition and extraction, in the compressed domain, of multiple sub-pictures into a composed picture has yielded at least a working draft.  That, however, does not prevent us from extrapolating the outcome to some extent, and to adapt our signaling design accordingly.

For now, we believe that the tile group header is the correct place for something like dec_pic_size_idx as proposed above, for reasons already stated.  We use a single ue(v) codepoint dec_pic_size_idx, conditionally present in the tile group header, to indicate the employed ARC parameters.  In order to match our implementation which is ARC per picture only, the one thing in spec-space we need to do right now is to code a single tile group only, or to make it a condition of bitstream compliance that all TG headers of a given coded picture have the same value of dec_pic_size_idx (when present).

The parameter dec_pic_size_idx can be moved into whatever header that starts a sub-picture.  Our current feeling is that most likely that will continue to be a tile group header.

Beyond these syntactical considerations, some additional work is needed to enable tile group or sub-picture based ARC.  The perhaps most difficult part is how to address the issue of unneeded samples in a picture where a sub-picture has been resampled to a lower size.

Full

Full

Full

Full

Half

Full

Full

Full

Fig. 3. Tile group-based resampling for ARC

Consider the right picture, which is made up of four sub-pictures (expressed perhaps as four rectangular tile groups in the bitstream syntax).  To the left, the bottom right TG is subsampled to half the size.  What do we do with the samples outside the relevant area, marked as “Half”?

Many (most?  all?) previous video coding standards had in common that spatial extraction of parts of a picture in the compressed domain was not supported.  That implied that each sample of a picture is represented by one or more syntax elements, and each syntax elements impacts at least one sample.  If we want to keep that up, we would need to populate the area around the samples covered by the downsampled TG labelled “Half” somehow.  H.263+ Annex P solved that problem by padding; in fact, the sample values of the padded samples could be signalled in the bitstream (within certain strict limits).

An alternative that would perhaps constitute a significant departure from previous assumptions, but may be needed in any case if we want to support sub-bitstream extraction (and composition) based on rectangular parts of a picture, would be to relax the current understanding that each sample of a reconstructed picture must be represented by something in the coded picture (even if that something is only a skipped block).

# Implementation considerations, system implications and Profiles/Levels

We propose basic ARC to be included in the “baseline/main” profiles.  Sub-profiling may be used to remove them if not needed for certain application scenarios.  Certain restrictions may be acceptable.  In that regard we note that certain H.263+ profiles and “recommended modes” (which pre-dated profiles) included a restriction for Annex P to be used only as “implicit factor of 4”, i.e. dyadic downsampling in both dimensions.  That was enough to support fast start (get the I frame over quickly) in video conferencing.

The design is such that we believe that all filtering can be done “on the fly” and that there is no, or only negligible, increases in memory bandwidth.  Insofar, we do not see a need to punt ARC into exotic profiles.

We do not believe that complex tables and such can be meaningfully used in capability exchange, as it was argued in Marrakech in conjunction with JVET-M0135.  The number of options is simply to big to allow for meaningful cross-vendor interop, assuming offer-answer and similar limited-depth handshakes.  Insofar, realistically, to support ARC in a meaningful way in a capability exchange scenario, we have to fallback to a handful, at most interop points.  For example: no ARC, ARC with implicit factor of 4, full ARC.  As an alternative, we could spec the required support for all ARC, and leave the restrictions in bitstream complexity to higher level SDOs.  That’s a strategic discussion we should have at some point anyway (beyond what we had already in the sub-profiling and flags context).

As for levels: we believe the basic design principle needs to be that, as a condition of bitstream conformance, the sample count of an upsampled pictures must fit into level of bitstream no matter how much upsampling is signalled in bitstream, and that all samples must fit into the upsampled coded picture.  We note that this was not the case in H263+; there, it was possible that certain samples were not present.

# Proposed common conditions for future ARC tests

Clearly, the current common conditions are suitable, at best, to test one of several possible application scenarios for ARC.  Below a few bullets how such common conditions (or CE tests or whatnot) could be structured:

- ARC is not (primarily) a coding efficiency tool, but has measurable coding efficiency impact

- ARC is most useful when “header overhead” is large, high ratio of headers vs. coded coefficients

- therefore, tests near the break-point of codec, high numerical QP

- Focus 1: ARC for IDR/Intra size reduction, both for (low) delay rate control and perhaps coding efficiency purpose.  Based on random access, code IDR scaled down by 1.5 or 2 in both dimensions, rest of CVS as usual, QPs to be discussed (but perhaps need QP boost for both downsampled IDR and first few reference pics after IDR).

- Focus 2: ARC for rate control in low delay.  Select sequence with lots of complexity change (such as: Ritual Dance, or concatenation of different sequences from test set).  Code near breaking point of codec design for low complexity part of sequence.  Force downsampling for certain (identified in CTC) pictures that are more complex.  (The CTC-identified pictures simulate rate control algorithm).

# Simulation results

ARC was implemented on VTM-3.0 and tested. The experiment was performed under CTC conditions for RA case. Since we expect ARC to be most useful for coding and delivering high resolution video sequences, only Class A sequences were tested.  We expect worse results for lower resolutions; better results for higher resolutions.

ARC has historically been shown, and is expected in the future to be helpful for latency reduction when starting with an IRAP picture, because it could avoid needing to code the first I picture at full resolution, resulting in either a low quality prediction point (due to high QP), or unreasonably large coded picture size and resulting increased latency (that is difficult to mitigate by future I pictures).   In the experiment, therefore, the I picture is down-sampled to half (1920 x 1080), while the following B pictures are coded in full resolution (3840 x 2160).

Superficially, the overall gain is not impressive.  However, Class A1 shows a luma coding gain (-0.92%), and especially Tango sequence shows -4.29% luma gain improvement with 9.33% and 8.69% chroma gain loss, respectively.

Table 1. Experimental results of ARC with down-sampled I pictures. (49frames)

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.92% | 14.30% | 14.67% | 103% | 109% |
| Class A2 | 6.15% | 30.72% | 22.50% | 101% | 107% |

Figures 3 & 4 shows the reconstructed image of the first I picture in full resolution and half resolution, respectively. The visual difference is not observed, while the bitrate is reduced to 37% (1274784 bits -> 469744 bits). With the reduced bit size of the I picture, it enables significant latency reduction.

In conclusion, the proposed ARC scheme can provide a new functionality enabling the latency reduction of the first picture delivery and a better bit allocation tool by adaptively adjusting the spatial resolution in picture-level granularity, without sacrificing subjective visual quality. In addition, the measured BD rates prove its potential gains in high resolution video sequences, when the filter design and the RDO process are improved by experts.

Fig. 3. The reconstructed image of the first I picture (3840x2160), coded by VTM3.0 (QP27).

Fig. 4. The reconstructed and upscaled image (3840x2160) of the first I picture, coded by the proposed ARC (QP27) in half resolution (1920x1080).

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, 12th meeting, Macao, CN, 3–12 Oct. 2018.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).