| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0054 |
| --- | --- |

| Title: | CE7: Joint coding of chrominance residuals (CE7-1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jani Lainema | Email: | first.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This document provides results for core experiment CE7-1 which was set up to study effectiveness of joint coding of chrominance residuals.

The proposed chrominance residual coding mode uses a single joint residual block to describe the residuals of both Cb and Cr blocks in the same transform unit. When joint residual mode is active, the indicated joint residual is added to the Cb prediction block and deducted from the Cr prediction block. Bitstream syntax and decoding process for the joint residual follow those of the Cb residual in VTM-4.  In the encoder side the algorithm uses average of the positive Cb residual and negative Cr residual as the input to the transform and quantization process.

It is reported that the proposed tool provides in AI configuration -0.38 %, -0.49 % and -1.52 % BD-rate impact for Y, U and V components, respectively. In RA configuration the impacts are -0.26 %, -0.81 % and -0.92 %, and in LD-B the BD-rate impacts are -0.07 %, -2.99 % and -4.76 % for Y, U and V components, respectively.

# Introduction

As Cb and Cr residuals appear to correlate inversely with each other, it is proposed to take advantage of this phenomenon and introduce a mode for joint coding of the chrominance residuals. In this mode there is a single residual indicated for the two chrominance blocks of a transform unit. The indicated residual is added to the prediction block in the first channel (typically representing Cb) and deducted from the prediction block in the second channel (typically representing Cr).

# Proposed tool

The joint residual mode is indicated with a flag in the bitstream if the coded block flags (cbf) for both Cb and Cr are true. If the mode is activated, a single residual block is decoded. The bitstream syntax and decoding process of joint residual blocks follow those of the Cb residual in VTM-4. The residuals of the Cr blocks are generated by negating the decoded joint residual. As a single residual is used to represent residuals of two blocks, it may often be desirable for this mode to use a QP lower than what is used for separate coding of chrominance residuals. In the CE tests a chroma QP offset of -1 was used for the joint mode and +1 was used for separate chroma coding (as specified in the JVET common test conditions).

The decoder side functionality for the mode can be implemented in VTM DecoderLib as follows:

New function:

void CABACReader::joint_cb_cr( TransformUnit& tu )

{

tu.jointCbCr = m_BinDecoder.decodeBin( Ctx::JointCbCrFlag( 0 ) );

}

Added in CABACReader::residual_coding:

// Joint Cb-Cr residual mode is signalled if both Cb and Cr cfbs are true

if ( compID == COMPONENT_Cr && TU::getCbf( tu, COMPONENT_Cb ) )

{

joint_cb_cr( tu );

// No Cr residual in bitstream in joint Cb-Cr residual mode

if ( tu.jointCbCr )

return;

}

Added in DecCu::xIntraRecBlk and DecCu::xDecodeInterTU:

// Cr uses negative of the signalled Cb residual

if ( tu.jointCbCr && compID == COMPONENT_Cr )

piResi.copyAndNegate( cs.getResiBuf( tu.blocks[COMPONENT_Cb] ) );

else

If chrominance reshaper is active, reshaping is applied to the received residual identically to what is done in separate coding modes (that is, the joint residual signal is reshaped). On the encoder side the average of positive Cb residual and negative Cr residual are used as the joint residual when testing this mode:

resJoint = (resCb – resCr) / 2

It is further noted that there are possibilities for encoder speed-ups by checking how residuals behave with respect to each other and avoiding entering transform and RDOQ stages when joint residual is unlikely to be the best coding alternative.

# Simulation results

The following table summarizes the results for common test conditions and for the low QP testing. In the low QP tests 100 frames were coded using a QP set of {2, 7, 12, 17}.  Detailed results for both categories of tests are available in the attached Excel spreadsheets.

| CTC | Y | U | V | EncTime | DecTime |
| --- | --- | --- | --- | --- | --- |
| AI | -0.38 % | -0.49 % | -1.52 % | 103 % | 100 % |
| RA | -0.26 % | -0.81 % | -0.92 % | 101 % | 100 % |
| LD-B | -0.07 % | -2.99 % | -4.76 % | 100 % | 100 % |
| LD-P | -0.07 % | -3.73 % | -5.11 % | 101 % | 100 % |

| Low QP | Y | U | V | EncTime | DecTime |
| --- | --- | --- | --- | --- | --- |
| AI | -0.28 % | 0.57 % | 0.27 % | 108 % | 99 % |
| RA | -0.19 % | 0.24 % | -0.10 % | 102 % | 100 % |
| LD-B | -0.18 % | 0.09 % | -0.58 % | 103 % | 100 % |
| LD-P | -0.19 % | 0.11 % | -0.59 % | 104 % | 99 % |

The following table contains information about usage of the mode in different configurations under JVET common test conditions. The data was gathered using the “decoder tool statistics” mechanism of VTM-4 and resulting percentages refer to the picture area for which the studied feature was enabled. Two sets of data were gathered during the test:

- Percentage of picture area for which both Cb and Cr residual was coded (including both separate “traditional coding” of Cb and Cr residuals and proposed joint coding of Cb and Cr residuals)

- Percentage of picture area for which the proposed joint Cb and Cr mode was selected (thus, a subset of the first set)

|  | Percentage of area for which… |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Cb and Cr are both coded |  |  |  | Cb and Cr are coded jointly |  |  |
|  | AI | RA | LB |  | AI | RA | LB |
| Class A1 | 6.85 % | 1.15 % |  |  | 2.81 % | 0.66 % |  |
| Class A2 | 9.13 % | 1.02 % |  |  | 3.98 % | 0.41 % |  |
| Class B | 8.12 % | 0.65 % | 0.85 % |  | 4.41 % | 0.41 % | 0.64 % |
| Class C | 9.65 % | 0.77 % | 0.89 % |  | 6.10 % | 0.49 % | 0.67 % |
| Class D | 9.23 % | 0.60 % | 0.59 % |  | 5.50 % | 0.34 % | 0.40 % |
| Class F | 6.31 % | 0.53 % | 0.65 % |  | 3.63 % | 0.34 % | 0.49 % |
| QP 22 | 10.94 % | 1.19 % | 1.21 % |  | 5.10 % | 0.61 % | 0.84 % |
| QP 27 | 8.59 % | 0.72 % | 0.65 % |  | 4.93 % | 0.44 % | 0.50 % |
| QP 32 | 7.00 % | 0.49 % | 0.43 % |  | 4.40 % | 0.31 % | 0.34 % |
| QP 37 | 6.03 % | 0.35 % | 0.34 % |  | 3.80 % | 0.23 % | 0.26 % |

The data seems to indicate that the proposed mode is used roughly half of the time when both Cb and Cr have active residuals in the bitstream. The ratio appears to be relatively consistent across different classes of sequences and quantization parameters.

# Specification text

7.3.2.2	Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_cb_qp_offset | se(v) |
| pps_cr_qp_offset | se(v) |
| pps_joint_cbcr_qp_offset | se(v) |
| … |  |
| } |  |

7.3.4.1	General tile group header syntax

| tile_group_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( pps_tile_group_chroma_qp_offsets_present_flag ) { |  |
| tile_group_cb_qp_offset | se(v) |
| tile_group_cr_qp_offset | se(v) |
| tile_group_joint_cbcr_qp_offset | se(v) |
| } |  |
| … |  |
| } |  |

7.3.6.12 Residual coding syntax

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| if( cIdx == 2 && tu_cbf_cb[ x0 ][ y0 ] ) { |  |
| tu_joint_cbcr_residual[ x0 ][ y0 ] | ae(v) |
| if( tu_joint_cbcr_residual [ x0 ][ y0 ] ) { |  |
| return |  |
| } |  |
| … |  |
| } |  |

7.4.3.2	Picture parameter set RBSP semantics

pps_joint_cbcr_qp_offset specifies the offset to the luma quantization parameter Qp′Y used for deriving Qp′CbCr. The value of pps_joint_cbcr_qp_offset shall be in the range of −12 to +12, inclusive. When ChromaArrayType is equal to 0, pps_joint_cbcr_qp_offset is not used in the decoding process and decoders shall ignore its value.

7.4.5.1	General tile group header semantics

tile_group_joint_cbcr_qp_offset specifies a difference to be added to the value of pps_joint_cbcr_qp_offset when determining the value of the Qp′CbCr quantization parameter. The value of tile_group_joint_cbcr_qp_offset shall be in the range of −12 to +12, inclusive. When tile_group_joint_cbcr_qp_offset is not present, it is inferred to be equal to 0. The value of pps_joint_cbcr_qp_offset + tile_group_joint_cbcr_qp_offset shall be in the range of −12 to +12, inclusive.

7.4.7.12 Residual coding semantics

tu_joint_cbcr_residual[ x0 ][ y0 ] specifies whether indicated Cb residual is used to derive both Cb and Cr residuals. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. tu_joint_cbcr_residual[ x0 ][ y0 ] equal to 1 specifies that the indicated Cb residual is used to derive the Cr residual. tu_joint_cbcr_residual[ x0 ][ y0 ] equal to 0 specifies that Cr residual may be present in the bitstream depending on other syntax elements. When tu_joint_cbcr_residual[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

8.7.1	Derivation process for quantization parameters

…

The variables qPCb, qPCr and qPCbCr are derived as follows:

qPiCb = Clip3( −QpBdOffsetC, 69, QpY + pps_cb_qp_offset + tile_group_cb_qp_offset )

qPiCr = Clip3( −QpBdOffsetC, 69, QpY + pps_cr_qp_offset + tile_group_cr_qp_offset )

qPiCbCr = Clip3( −QpBdOffsetC, 69, QpY + pps_joint_cbcr_qp_offset + tile_group_joint_cbcr_qp_offset )

–	If ChromaArrayType is equal to 1, the variables qPCb, qPCr and qPCbCr are set equal to the value of QpC as specified in Table 8-12 based on the index qPi equal to qPiCb, qPiCr and qPCbCr, respectively.

–	Otherwise, the variables qPCb, qPCr and qPCbCr are set equal to Min( qPi, 63 ), based on the index qPi equal to qPiCb, qPiCr and qPCbCr, respectively.

–	The chroma quantization parameters for the Cb and Cr components, Qp′Cb and Qp′Cr, and joint Cb-Cr coding Qp′CbCr, are derived as follows:

Qp′Cb = qPCb + QpBdOffsetC

Qp′Cr = qPCr + QpBdOffsetC

Qp′CbCr = qPCbCr + QpBdOffsetC

…

8.7.2	Scaling and transformation process

…

The (nTbW)x(nTbH) array of residual samples resSamples is derived as follows:

  - If cIdx is equal to 2 and tu_joint_cbcr_residual[ xTbY ][ yTbY ] is equal to 1, the residual samples resSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

resSamples[ x ][ y ] = -resSamplesCb[ x ][ y ]

where resSamplesCb[ x ][ y ] is the residual sample array generated for a chrominance block with the transform block location ( xTbY, yTbY ) and cIdx equal to 1.

Otherwise:

…

8.7.3	Scaling process for transform coefficients

…

The quantization parameter qP is derived as follows:

- If cIdx is equal to 0, the following applies:

qP = Qp′Y

- Otherwise, if cIdx is equal to 1 and tu_joint_cbcr_residual[ xTbY ][ yTbY ] is equal to 1, the following applies:

qP = Qp′CbCr

…

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).