| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0055 |
| --- | --- |

| Title: | AHG12: On sub-picture partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com stewe@stewe.org shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution propose a sub-picture scheme for multiple use cases: View-port dependent 360 streaming, E-sport content streaming with multiple captures, and Seamless face boundaries of cubemap projection. The layout information of sub-pictures is signaled in SPS. For independent decoding/extraction/merge of sub-pictures, parameter set design, syntax elements, constraints, and decoding process are proposed.

Introduction

VVC needs an MCTS-like functionality, with the following features:

- A sub-bitstream including VCL NAL and non-VCL NAL units, which are needed to decode a subset of tiles, can be extracted from one VVC bitsteream covering all tiles composing the entire picture.

- The extracted sub-bitstream shall be independently decodable in a VVC decoder, without referencing NAL units.

We can call this partitioning mechanism, MCTS (motion constained tile set) or sub-picture. In order to avoid confusion between tile group and tile set, the term ‘sub-picture’ is used in this proposal. Also, it is noticeable that the sub-picture design in this proposal accepts some esstional parts of the sub-picture design proposed by JVET-M0261.

In addition, the following features need to be further studied to improve the usefulness of the sub-picture for immersive media use cases:

- Each sub-picture may have a different random access period and a different inter-prediction structure..

Use case 1: Unequal random access period for view-port dependent 360 streaming. Only centralized regions with dynamic contents may receive attention from viewers, while other background region may change slowly. Having different GOP structures and different random access periods will be helpful to provide fast access of specific regions with better visual quality in overall.

- A sub-picture may have a different resampling ratio.

Use case 2: The quality of background regions (e.g. top and bottom in 360, non-dynamic object in PCC) can be efficiently scarified for the overall bit-efficiency.

- Multiple sub-pictures composing a picture may or may not be encoded by an encoder and decoded by a decoder. For example, a sub-picture can be independently encoded by an encoder, while another sub-picture is encoded by another encoder. Then, sub-bitstreams corresponding to those sub-pictures can be merged into a bitstream, which can be decoded by a decoder.

Use case 3: m46551 proposed an interesting use cases for E-sport contents: A number of gamers (Gamer 1 to Gamer N) are participating in a game. The game and camera views of each of these gamers is captured and delivered. Based on the player selection of each viewer, a media processor can group related game views (e.g., of following gamers/players), converted to a video.

Figure 1. E-Sports use case (suggested by m46551)

- A sub-picture may have the width and height which are not multiples of CTU size.

Use case 4: For 360 projection formats with multiple faces, discontinuities may appear at boundaries of faces. Figure 2 shows an example a the 3×2 frame packing layout. If in-loop filtering operations are performed across the boundary, seam artifacts could be visible at boundaries. To alleviate such visual artifacts, in-loop filtering operations across face boundaries need to be disabled. JVET-M0892 proposed a virtual boundary concept to resolve this problem. However, if a face is coded as a sub-picture, this boundary issue can be efficiently resolved. Then, to align the sub-picture size to the face size, it is required to have the sub-picture size that is not equal to a multiple of the CTU size.

Figure 2. Multiple faces of the cubemap projection (from JVET-M0892)

Key features of the the proposed sub-picture design

The proposed sub-picture design includes the following features and technical details:

- A sub-picture is a rectangular region.

- A sub-picture may or may not be partitioned into multiple tiles.

- If partitioned to multiple tiles, each sub-picture has its own tile scan order. (as shown in Fig. 3)

The following ascepcts of the proposed sub-picture design are distinguished from JVET-M0261.

- Each sub-picture may reference it own SPS, PPS and APS, but each SPS may contain all sub-picture partitioning information.

. In SPS, the sub-picture partitioning and layout information is signaled.

The decoded and the output size of the sub-picture are signaled.

The left-top position of the sub-picture is signaled.

The RPL information of the sub-picture is signaled.

Each sub-picture may or may not have the same RPL information in the same picture.

. In PPS, the tile partitioning information of the sub-picture is signaled.

. In APS, the ALF coeffficients of the sub-picture are signaled.

. Any parameter set or SEI message can be referenced by multiple sub-pictures.

- In a sub-bitstream of a sub-picture after extraction process, all necessary parameter set and SEI messages needed for decoding and displaying the sub-picture shall be contained, if the parameter sets are delivered in band.

- When multiple sub-bitstreams are merged into a big one, then duplicated parameters need to be removed.

- The sub-picture ID is signalled in NAL unit header. (If NUH header extension is adopted. Otherwise signaled in tile-group header).

- Tiles in a sub-picture can be combined into a rectangular or non-rectangular tile group, but tile belonging to different sub-pictures cannot be grouped.

- Any decoding process (e.g. in-loop filtering, motion compensation) across sub-picture boundaries is (optionally) disallowed.

- The boundary of a sub-picture may be extended and padded for motion compensation.

. A flag indicating whether the boundary is extended or not is signalled in SPS.

- The decoded sub-picture may or may not be resampled to be output.

.The spatial ratio between the decoded sub-picture size and the output sub-picture size signalled in SPS is used to calculate the resampling ratio.

.The resampling operations same as the adaptive resolution changes can be applied.

- A picture may have one or more non-IRAP VCL NAL units and one or more IRAP VCL NAL units. (Each sub-picture may have a different random access period.)

- A parameter set can be activated for each sub-picture individually.

- Each SEI message can be used for a sub-set of sub-pictures.

Tile0

Tile1

Tile2

Tile0

Tile1

Tile2

Tile3

Tile4

Tile3

Tile4

Tile5

Tile5

Tile6

Tile7

Tile8

Tile9

Tile6

Tile7

Tile8

Tile
10

Tile
11

Tile
12

Tile
13

Tile
14

Tile0

Tile1

Tile2

Tile0

Tile1

Tile0

Tile1

Tile2

Tile3

Tile4

Tile5

Tile2

Tile3

Tile3

Tile4

Tile5

Tile6

Tile7

Tile8

Tile4

Tile5

Tile6

Tile7

Tile8

Tile9

Tile
10

Tile
11

Tile6

Tile7

Tile9

Tile
10

Tile
11

Tile
12

Tile
13

Tile
14

Tile8

Tile9

Tile
12

Tile
13

Tile
14

Sub-picture 0

Sub-picture 1

Sub-picture 2

Sub-picture 3

Sub-picture 4

Figure 3. An illustration of sub-picture partitioning.

Proposed syntax elements

- Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| num_sub_pictures_in_pic | ue(v) |
| if(num_sub_pictures > 0) |  |
| sub_pic_id_length_minus1 |  |
| for( i = 0; i < num_sub_pictures; i++) { |  |
| dec_sub_pic_width_in_luma_samples[ i ] | ue(v) |
| dec_sub_pic_height_in_luma_samples[ i ] | ue(v) |
| out_sub_pic_width_in_luma_samples[ i ] | ue(v) |
| out_sub_pic_height_in_luma_samples[ i ] | ue(v) |
| if( i > 0 ) { |  |
| sub_pic_id[ i ] | u(v) |
| left_top_pos_x_in_luma_samples[ i ] | ue(v) |
| left_top_pos_y_in_luma_samples[ i ] | ue(v) |
| } |  |
| } |  |
| if(num_sub_pictures > 0) { |  |
| full_parallel_decoding_sub_pic_enabled_flag | u(1) |
| if(!full_parallel_decoding_ sub_pic_senabled_flag) { |  |
| loop_filter_across_ sub_pic_enabled_flag | u(1) |
| inter_prediction_across_sub_pic_enabled_flag | u(1) |
| } |  |
| } |  |
| … |  |
| } |  |

num_sub_pictures_in_pic specifies the number of sub-pictures in each picture referring to the SPS.

signalled_sub_pic_id_length_minus1 equal to 1 specifies the number of bits used to represent the syntax element sub_pic_id[ i ] when present, and the syntax element tile_group_sub_pic_id[ i ] in tile group headers. The value of signalled_sub_pic_id_length_minus1 shall be in the range of 0 to 15, inclusive.

dec_sub_pic_width_in_luma_samples[ i ] specifies the width of the i-th decoded sub-picture in units of luma samples in the coded video sequence. dec_sub_pic_width_in_luma_samples[ i ] shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

dec_sub_pic_height_in_luma_samples[ i ] specifies the height of the i-th decoded sub-picture in units of luma samples in the coded video sequence. dec_sub_pic_height_in_luma_samples[ i ] shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

output_sub_pic_width_in_luma_samples[ i ] specifies the width of the i-th output sub-picture in units of luma samples. output_sub_pic_width_in_luma_samples shall not be equal to 0.

output_sub_pic_height_in_luma_samples[ i ] specifies the height of the i-th output sub-picture in units of luma samples. output_sub_pic_height_in_luma_samples shall not be equal to 0.

sub_pic_id[ i ] specifies the sub-picture identifier of the i-th sub-picture. The length of the sub_pic_id[ i ] syntax element is sub_pic_id_length_minus1 + 1 bits.  When not present, the value of sub_pic_id[ i ] is set equal to 0.

left_top_pos_x_in_luma_samples[ i ] specifies the colum position of the first pixel of the i-th sub-picture.

left_top_pos_y_in_luma_samples[ i ] specifies the row position of the first pixel of the i-th sub-picture.

NOTE1 – Each sub-picture is resampled to its corresponding output sub-picture size after decoding. A sub-picture cannot be overlapped with another sub-picture region. The width and the height of the picture size composed by all sub-picture output sizes and positions shall be equal to pic_width_in_luma_samples pic_height_in_luma_samples, but a partial picture region composed by a sub-set of sub-pictures can be decoded.

full_parallel_decoding_sub_pic_enabled_flag equal to 1 specifies any processing and prediction across sub-picture boundary is disallowed.

loop_filter_across_sub_pic_enabled_flag equal to 1 specifies that in-loop filtering operations may be performed across sub-picture boundaries in pictures referring to the SPS. loop_filter_across_sub_pic_enabled_flag equal to 0 specifies that in-loop filtering operations are not performed across sub-picture boundaries in pictures referring to the PPS. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When not present, the value of loop_filter_across_sub_pic_enabled_flag is inferred to be equal to 0.

inter_prediction_across_sub_pic_enabled_flag equal to 1 specifies that inter-prediction operations may be performed across sub-picture boundaries in pictures referring to the SPS. inter_prediction_across_sub_pic_enabled_flag equal to 0 specifies that inter-prediction operations are not performed across sub-picture boundaries in pictures referring to the SPS. The inter-prediction operations include the temporal motion compensation, current picture reference, temporal motion vector prediction, and any parameter prediction operations between pictures. When not present, the value of inter_prediction_across_sub_pic_enabled_flag is inferred to be equal to 0.

- Tile group header syntax

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |
| if(num_sub_pictures > 0) |  |
| tile_group_sub_pic_id | u(v) |
| if( rect_tile_group_flag ¦ ¦ NumTilesInPic > 1 ) |  |
| tile_group_address | u(v) |
| if( !rect_tile_group_flag && !single_tile_per_tile_group_flag ) |  |
| num_tiles_in_tile_group_minus1 | ue(v) |

tile_group_sub_pic_id specifies the sub-picture identifier of the sub-picture which the current tile group belongs to.

Other proposed syntax elements and decoding process are described in an independent document as the proposed draft of VVC WD.

Sub-bitstream extraction and merge process

Sub-picture extraction process:

1. The VCL NAL units corresponding to the sub-picture ID is extracted. Others are removed.

2. The parameter sets referenced by VCL NAL unit with the sub-picture ID is extracted. Others are removed.

Sub-picture merge process:

1. The layout information of sub-pictures in SPS is rewritten (if necessary)

2. The sub-picture ID in a VCL NAL unit is rewritten (if necessary)

3. VCL NAL units with the same POC value are interleaved in the same AU.

4. Any duplicated parameter sets are removed.

- Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).