| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0056 |
| --- | --- |

| Title: | AHG12: On tile design for parallel processing |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com stewe@stewe.org shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution proposes inter_prediction_across_tiles_enabled_flag which enables motion contrained tile (not tile-set or sub-picture). The tiles, which can be decoded independently without any inter-tile operations, can be used for tile-level parallel processing and repartitiong tile groups without VCL-level modification. We assert that this feature can coexist with the sub-picture or MCTS.

Introduction

Tile in HEVC was designed to support the following two main use cases:1) parallel decoding process, 2) partial delivery and decoding. The first use case was basically enabled by using the original tile in HEVC, but it still had some dependency for inter-prediction operations. The second use case was enabled by employing additional SEI messages, called motion constrained tile set (MCTS), even though that was an optional way.

In VVC, the same tile design was inherited from HEVC, but a new scheme, so-called tile group, was adopted to support multiple use cases. In this JVET meeting, a couple of contribution (e.g. JVET-N0055) propose the sub-picture concept. The sub-picture or MCTS can be mainly used for view-port based delivery of VR360 or other immersive contents supporting 3/6 degree-of-freedoms. It would be definitely useful to have such a functionality in VVC to be widely used for future immersive content services. Whereas, the parallel decoding processing would be still important to decode 4/8/16K contents in realtime. Regardless of what design is adopted for MCTS-like functionality, it is desired to have the full parallel processing capability for tiles without any decoding dependency across tile boundaries.

In Marrakesh meeting, two syntax elemeents (loop_filter_across_tiles_enabled_flag and loop_filter_across_tile_groups_enabled_flag) were adopted, to provide the better parallel processing capability. Those syntax elements indicate in-loop filtering operations are not performed across tile boundaries or tile group boundaries, respectively. In this proposal, we assert that having two additional flags on top of them is desired, which indicate inter-prediction operations is allowed or disallowed across tile or tile group boundaries, respectively. In this semantics, the inter-prediction operations include the temporal motion compensation, current picture reference (CPR or IBC), temporal motion vector prediction, and any parameter prediction operations between pictures. Hence, we propose the independently decodable tiles (not tile sets). This feature can be mutually compatible with the sub-picture or MCTS scheme.

The proposed syntax elements are mainly useful for two use cases: 1) Full parallel decoding process without any dependency across tile/tile group boundaries, 2) Reconfiguring the tile-group layout without transcoding VCL NAL units. The first use case is clear. Even if a picture is partitioned into two tile groups and delivered, multiple decoders can decode a tile group in parallel, when the tile group consists of multiple tiles. For example, a picture can be partitioned into eight tiles and each four tiles can be grouped into one tile group. A coded tile group is contained in a VCL NAL unit and delivered. If a tile group is received, four decoders can decode four tiles in parallel, without any inter-tile operations.

The second use case is related to VR360 view-port dependent processing. When the targeted view-port is shifted to a boundary between two tile groups, a decoder should receive and decode two tile groups to display the targeted view-port. However, with the proposed syntax modification, the partitioning information of tile groups in PPS can be updated by a server or a cloud processor, and only one tile group including all targeted view-port can be delivered to the decoder. This instant tile-group repartitioning is possible without VLC-level modifications (i.e. without transcoding), when all tiles are encoded as motion-constrained tiles.

Proposed syntax elements

Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| .. |  |
| full_parallel_decoding_tile_enabled_flag | u(1) |
| if(!full_parallel_decoding_tile_enabled_flag) { |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| if( loop_filter_across_tiles_enabled_flag ) |  |
| loop_filter_across_tile_groups_enabled_flag | u(1) |
| inter_prediction_across_tiles_enabled_flag | u(1) |
| if(inter_prediction_across_tiles_enabled_flag ) |  |
| inter_prediction_across_tile_groups_enabled_flag | u(1) |
| } |  |
| … |  |
| } |  |

full_parallel_decoding_tiles_enabled_flag equal to 1 specifies any processing and prediction across tile boundary is disallowed.

loop_filter_across_tiles_enabled_flag equal to 1 specifies that in-loop filtering operations may be performed across tile boundaries in pictures referring to the PPS. loop_filter_across_tiles_enabled_flag equal to 0 specifies that in-loop filtering operations are not performed across tile boundaries in pictures referring to the PPS. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When not present, the value of loop_filter_across_tiles_enabled_flag is inferred to be equal to 0.

loop_filter_across_tile_groups_enabled_flag equal to 1 specifies that in-loop filtering operations may be performed across tile group boundaries in pictures referring to the PPS. loop_filter_across_tile_group_enabled_flag equal to 0 specifies that in-loop filtering operations are not performed across tile group boundaries in pictures referring to the PPS. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When not present, the value of loop_filter_across_tile_groups_enabled_flag is inferred to be equal to 0.

inter_prediction_across_tiles_enabled_flag equal to 1 specifies that inter-prediction operations may be performed across tile boundaries in pictures referring to the PPS. inter_prediction_across_tiles_enabled_flag equal to 0 specifies that inter-prediction operations are not performed across tile boundaries in pictures referring to the PPS. The inter-prediction operations include the temporal motion compensation, current picture reference, temporal motion vector prediction, and any parameter prediction operations between pictures. When not present, the value of inter_prediction_across_tiles_enabled_flag is inferred to be equal to 0.

inter_prediction_across_tile_groups_enabled_flag equal to 1 specifies that inter-prediction operations may be performed across tile group boundaries in pictures referring to the PPS. inter_prediction_across_tile_groups_enabled_flag equal to 0 specifies that inter-prediction operations are not performed across tile group boundaries in pictures referring to the PPS. The inter-prediction operations include the temporal motion compensation, current picture reference, temporal motion vector prediction, and any parameter prediction operations between pictures. When not present, the value of inter_prediction_across_tile_groups_enabled_flag is inferred to be equal to 0.

- Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).