| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0057-v2 |
| --- | --- |

| Title: | AHG12: On tile group design |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com stewe@stewe.org shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

We propose a modification of semantics of loop_filter_across_tile_groups_enabled_flag to address the following issues.

- No use case of handling non-rectangular tile group boundary as picture boundary.

- Complexity issue of ALF filtering across L-shape boundary of non-rectangular tile group.

- Syntax design alignment with MCTS flag for rectangular tile group.

- Duplication and conflict of tile group and tile boundary handling when single_tile_per_tile_group_flag is equal to 1.

The modification of semantics is intended to not handle non-rectangular tile group boundaries as picture boundaries and clarify the ALF operations across tile group boundaries.

# Introduction

In VVC, the same tile design was inherited from HEVC, but a new scheme, so-called tile group, was adopted to support multiple use cases; 1) partial delivery and decoding, 2) parallel decoding process, 3) MTU size matching with tile granularity. For parallel processing, in Marrakesh meeting, two syntax elemeents (loop_filter_across_tiles_enabled_flag and loop_filter_across_tile_groups_enabled_flag) were adopted, to selectively allow in-loop filtering process across tile group boundary.

We observed the following issues regarding the flag, loop_filter_across_tile_groups_enabled_flag:In this contribution, we simply propose a constraint that tile-group-level loop filtering (or any other operations) control at boundaries shall be allowed only for rectangulary tile groups.

- No use case of handling non-rectangular tile group boundary as picture boundary.

- Complexity issue of ALF filtering across L-shape boundary of non-rectangular tile group.

See Fig.1. It is difficult to identify which pixels belong to the current tile group at the point near a corner, inside 2D ALF filter shape. It is not too heavy computations, but unnecessary burden because it is not a normal case disabling in-loop filtering across non-tile group boundary only, while enabling across tile boundaries.

- Syntax design alignment with MCTS flag for rectangular tile group.

- Duplication and conflict of tile group and tile boundary handling when single_tile_per_tile_group_flag is equal to 1.

To address the above issues, we propose the following semantics changes of loop_filter_across_tile_groups_enabled_flag.

Figure 1. Illustration of ALF processing at non-rectangular tile group boundaries

# Proposed syntax elements

#### Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| .. |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| if( loop_filter_across_tiles_enabled_flag ) |  |
| loop_filter_across_tile_groups_enabled_flag | u(1) |
| … |  |
| } |  |

loop_filter_across_tile_groups_enabled_flag equal to 1 specifies that in-loop filtering operations may be performed across tile group boundaries in pictures referring to the PPS. loop_filter_across_tile_group_enabled_flag equal to 0 specifies that in-loop filtering operations are not performed across tile group boundaries in pictures referring to the PPS. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When not present, the value of loop_filter_across_tile_groups_enabled_flag is inferred to be equal to the value of 0. It is a requirement of bitstream conformance that the value of loop_filter_across_tile_groups_enabled_flag shall be equal to that of loop_filter_across_tiles_enabled_flag, when single_tile_per_tile_group_flag is equal to 1 or rect_tile_group_flag is equal to 0.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).