| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0059 | Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0042-r1 |
| --- | --- | --- | --- |

| Title: | CE1: Uniform Luma Inter Prediction Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jennifer Rasch, Anastasia Henkel, Jonathan Pfaff, Matthias Albrecht, Heiko Schwarz, Detlev Marpe, Thomas Wiegand | Tel: Email: | +49 30 31002-727 Jennifer.Rasch@hhi.fraunhofer.de, Jonathan.pfaff@hhi.fraunhofer.de, anastasia.henkel@hhi-extern.fraunhofer.de. |
| Source: | Fraunhofer Institute for Telecommunications – Heinrich Hertz Institute. |  |  |

_____________________________

# Abstract

The uniform luma inter prediction filters described in JVET-M0042 are only applied in case of inter prediction. The application in the merge case is modified in order to decrease the complexity. The filter masks are simple and fixed and no iterations are used. There are three types of filters, two directional filters and one two dimensional filter.

# Proposed method: Decoder algorithm description

Uniform luma inter prediction filters have been integrated to the test model according to the description of sub-tests CE1.4.2- CE1.4.3 [7]. Further versions have been described in [3]-[6]. A proposed draft text in reference to [2] is attached.

## Uniform Luma 2DFilter

The uniform luma 2D filter is defined as in [6]. Let pred be the prediction signal on a given block obtained by motion compensated prediction. In order to handle boundary points for the filters, the prediction signal needs to be extended to a prediction signal predext. This extended prediction is formed by mirroring the prediction or adding one line of reconstructed samples left and above the block to the prediction signal and mirroring the resulting signal in all directions. The uniform luma 2D Filter is realized by convolving the prediction signal with a fixed mask that is given as hI, defined below. It is proposed to replace the prediction signal pred by

hI*pred,

using the aforementioned boundary extension. Here, the filter mask hI is given as

hI=0.125010141010.

## Directional Luma Filter

The directional filters are defined as described in [6]. As described above, the extended prediction is formed by mirroring the prediction or adding one line of reconstructed samples left and above the block to the prediction signal and mirroring the resulting signal in all directions. A horizontal filter hhor and a vertical filter hver,  are used which have a fixed mask. The filtering is restricted to be either applied only along the vertical or along the horizontal direction. Then, it is proposed to replace the prediction signal pred by

hver*pred,

using the aforementioned boundary extension. The vertical filter is realized by applying the fixed filter mask

hver=0.54 104060401

to the prediction signal. The horizontal filter is realized by using the transposed mask

hhor=hvert

and applied analogously.

# Implementation and Complexity

Different filter configurations, depending on the block-shape, are used. These configurations are described via the syntax element diffFilterIdx as described in the table below.

| diffFilterIdx | filter type | Filters to be used |
| --- | --- | --- |
| 0 | no filter | - |
| 1 | Uniform | hI |
| 2 | directional horizontal | hhor |
| 3 | directional vertical | hver |

Table  Interpretation of the syntax element diffFilterIdx

It should be noted here that the filter is restricted to larger blocks. It is only applied if both block sizes are larger than 4 and smaller than 128.

The filtering is implemented in a way such that symmetries within the filter-masks are exploited. The multiplications that are factors of two are implemented using bitshifts. The maximal complexity per sample is 6 additions and 4 bit-shifts to produce the overall prediction signal when applying the uniform luma inter filter.

# Proposed Restrictions

The uniform diffusion filters are restricted to inter. According to the CE1 description [7], for all tests, the following three constraint conditions are tested:

- No restriction on using samples to the left and above the block.

- No samples outside the block can be used.

# Results

According to the CE1 description in JVET-M1021 [7] and the common test conditions in JVET-M1010 [1], the proposed method is evaluated for random-access and low delay configuration with VTM 4.0.

In the following table an overview over the scheduled CE tests 1-4.2 and 1-4.3 with their corresponding config parameters is shown.

|  | Test |  |
| --- | --- | --- |
| CE1.4.2a | Random Access and Low Delay | DiffusionFilterMode : 1 FastDiffusionFilter : 7 NMBDRYDiffusion : 0 NumDiffusionFiltersInter : 4 |
| CE1.4.2c | Random Access and Low Delay | DiffusionFilterMode : 1 FastDiffusionFilter : 7 NMBDRYDiffusion : 1 NumDiffusionFiltersInter : 4 |
| CE1.4.3a | Random Access and Low Delay | DiffusionFilterMode : 1 FastDiffusionFilter : 7 NMBDRYDiffusion : 0 NumDiffusionFiltersInter : 4 |
| CE1.4.3c | Random Access and Low Delay, | DiffusionFilterMode : 1 FastDiffusionFilter : 7 NMBDRYDiffusion : 1 NumDiffusionFiltersInter : 4 |

## CE1-4.2a

The uniform luma inter prediction filters are tested for all merge candidates. Reconstructed samples on the side are used.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.32% | -0.65% | -0.34% | 112% | 101% |
| Class A2 | -0.38% | -0.79% | -0.62% | 114% | 102% |
| Class B | -0.47% | -0.96% | -1.10% | 113% | 98% |
| Class C | -0.19% | -0.36% | -0.51% | 115% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.35% | -0.70% | -0.69% | 114% | 101% |
| Class D | -0.14% | -0.19% | -0.21% | 114% | 100% |
| Class F | -0.14% | -0.32% | -0.30% | 110% | 101% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.30% | 0.28% | 0.08% | 126% | 102% |
| Class C | -0.14% | 0.24% | 0.24% | 128% | 101% |
| Class E | 0.09% | 0.90% | 0.49% | 120% | 104% |
| Overall | -0.15% | 0.42% | 0.23% | 125% | 102% |
| Class D | -0.05% | 0.52% | -0.36% | 128% | 104% |
| Class F | -0.20% | -0.33% | -0.70% | 120% | 99% |

|  | Low delay P Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -1.08% | -1.08% | -0.80% | 132% | 100% |
| Class C | -0.70% | -0.78% | -0.56% | 132% | 102% |
| Class E | -1.04% | -0.79% | -0.32% | 127% | 100% |
| Overall | -0.94% | -0.91% | -0.60% | 131% | 101% |
| Class D | -0.43% | 0.02% | 0.39% | 131% | 102% |
| Class F | -0.31% | -0.14% | -0.45% | 124% | 99% |

## CE1-4.2c

The uniform luma inter prediction filters are tested for all merge candidates. No Reconstructed samples are used.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.35% | -0.66% | -0.27% | 111% | 98% |
| Class A2 | -0.39% | -0.78% | -0.49% | 113% | 98% |
| Class B | -0.51% | -0.71% | -0.87% | 112% | 96% |
| Class C | -0.12% | -0.24% | -0.29% | 113% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.35% | -0.59% | -0.52% | 112% | 98% |
| Class D | -0.12% | -0.41% | -0.27% | 113% | 100% |
| Class F | -0.12% | -0.29% | -0.24% | 109% | 102% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.26% | 0.36% | 0.60% | 126% | 101% |
| Class C | -0.11% | 0.30% | 0.25% | 127% | 101% |
| Class E | 0.08% | 1.20% | 0.56% | 120% | 104% |
| Overall | -0.12% | 0.55% | 0.47% | 125% | 102% |
| Class D | 0.01% | 0.14% | -0.06% | 127% | 103% |
| Class F | -0.06% | -0.01% | 0.02% | 121% | 100% |

|  | Low delay P Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -1.06% | -0.61% | -0.46% | 131% | 99% |
| Class C | -0.64% | -0.47% | -0.42% | 132% | 102% |
| Class E | -1.08% | 0.63% | 0.52% | 125% | 99% |
| Overall | -0.92% | -0.25% | -0.20% | 130% | 100% |
| Class D | -0.39% | -0.13% | 0.38% | 132% | 105% |
| Class F | -0.24% | 0.10% | 0.25% | 124% | 99% |

## CE1-4.3a

The uniform luma inter prediction filters are modified in merge case to decrease complexity. Reconstructed samples on the side are used.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.32% | -0.66% | -0.33% | 110% | 102% |
| Class A2 | -0.35% | -0.76% | -0.61% | 111% | 101% |
| Class B | -0.45% | -0.87% | -0.99% | 111% | 99% |
| Class C | -0.19% | -0.34% | -0.52% | 112% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.33% | -0.67% | -0.66% | 111% | 101% |
| Class D | -0.12% | -0.26% | -0.28% | 112% | 102% |
| Class F | -0.13% | -0.26% | -0.24% | 109% | 103% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.24% | 0.17% | 0.22% | 120% | 102% |
| Class C | -0.11% | 0.40% | 0.26% | 121% | 100% |
| Class E | -0.02% | 0.86% | 0.04% | 112% | 100% |
| Overall | -0.14% | 0.42% | 0.19% | 118% | 101% |
| Class D | -0.05% | 0.47% | -0.57% | 120% | 101% |
| Class F | -0.14% | -0.45% | -0.68% | 114% | 99% |

|  | Low delay P Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.99% | -0.81% | -1.03% | 122% | 99% |
| Class C | -0.66% | -0.48% | -0.48% | 123% | 101% |
| Class E | -1.05% | -0.11% | 0.23% | 115% | 98% |
| Overall | -0.90% | -0.52% | -0.53% | 121% | 100% |
| Class D | -0.41% | -0.05% | -0.07% | 122% | 101% |
| Class F | -0.34% | 0.16% | -0.27% | 114% | 97% |

## CE1-4.3c

The uniform luma inter prediction filters are modified in merge case to decrease complexity.  No reconstructed samples are used.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.34% | -0.40% | -0.12% | 108% | 97% |
| Class A2 | -0.38% | -0.62% | -0.47% | 109% | 98% |
| Class B | -0.49% | -0.75% | -0.90% | 109% | 97% |
| Class C | -0.09% | -0.12% | -0.32% | 110% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.33% | -0.49% | -0.50% | 109% | 98% |
| Class D | -0.06% | -0.31% | -0.09% | 110% | 99% |
| Class F | -0.10% | -0.27% | -0.17% | 107% | 101% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.20% | 0.30% | 0.29% | 120% | 102% |
| Class C | -0.07% | 0.41% | 0.34% | 120% | 100% |
| Class E | 0.09% | 1.39% | 0.77% | 112% | 101% |
| Overall | -0.08% | 0.61% | 0.43% | 118% | 101% |
| Class D | 0.01% | 0.60% | -0.40% | 121% | 101% |
| Class F | -0.09% | 0.30% | -0.29% | 114% | 98% |

|  | Low delay P Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -1.00% | -0.52% | -0.32% | 123% | 100% |
| Class C | -0.58% | -0.19% | -0.11% | 124% | 101% |
| Class E | -1.14% | 0.04% | 0.30% | 115% | 97% |
| Overall | -0.90% | -0.27% | -0.10% | 121% | 100% |
| Class D | -0.33% | 0.23% | 0.60% | 123% | 102% |
| Class F | -0.34% | 0.44% | 0.13% | 115% | 99% |

# Combinations with other Tools

All following combinations are combined with the uniform luma inter prediction filter as described in CE1-4.3c.

## CE1-7.1 Uniform Luma Inter Prediction Filter and BIF as a inloop filter on top

Here, the uniform luma inter prediction filter is combined with CE1-1.5 as can be found in [7].

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.25% | 0.24% | 104% | 106% |
| Class A2 | -0.15% | 0.31% | 0.15% | 106% | 109% |
| Class B | -0.10% | 0.33% | 0.39% | 106% | 108% |
| Class C | -0.33% | 0.11% | 0.12% | 108% | 105% |
| Class E | -0.16% | 0.22% | 0.22% | 102% | 103% |
| Overall | -0.16% | 0.25% | 0.24% | 105% | 106% |
| Class D | -0.28% | 0.17% | 0.14% | 105% | 100% |
| Class F | -0.37% | 0.00% | 0.03% | 102% | 107% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.53% | -0.71% | -0.30% | 118% | 99% |
| Class A2 | -0.72% | -0.84% | -0.44% | 120% | 103% |
| Class B | -0.90% | -0.36% | -0.81% | 116% | 102% |
| Class C | -0.53% | -0.11% | -0.24% | 116% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.69% | -0.46% | -0.48% | 117% | 100% |
| Class D | -0.54% | -0.04% | -0.16% | 117% | 99% |
| Class F | -0.41% | -0.11% | -0.16% | 127% | 101% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.26% | 1.17% | 0.86% | 127% | 105% |
| Class E | -0.19% | 2.68% | 1.38% | 115% | 97% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.42% | 1.59% | 0.66% | 129% | 104% |
| Class F | -0.13% | 0.12% | -0.09% | 113% | 98% |

## CE1-7.2 Combine Uniform Luma Inter Prediction Filter and BIF as reconstruction filter mutually exclusive

Here, the uniform luma inter prediction filter is combined with CE1-1.1b as can be found in [7].

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.44% | -0.50% | -0.19% | 113% | 99% |
| Class A2 | -0.66% | -0.74% | -0.39% | 116% | 102% |
| Class B | -0.84% | -0.63% | -0.72% | 116% | 99% |
| Class C | -0.34% | -0.13% | -0.22% | 116% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.59% | -0.49% | -0.41% | 115% | 101% |
| Class D | -0.35% | 0.00% | 0.12% | 116% | 102% |
| Class F | -0.16% | -0.20% | -0.20% | 110% | 102% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.50% | 0.77% | 0.61% | 125% | 104% |
| Class C | -0.30% | 0.90% | 0.78% | 126% | 103% |
| Class E | -0.23% | 1.55% | 1.51% | 117% | 105% |
| Overall | -0.36% | 1.01% | 0.89% | 123% | 104% |
| Class D | -0.35% | 1.65% | 0.49% | 128% | 106% |
| Class F | -0.20% | -0.33% | 0.74% | 117% | 100% |

|  | Low delay P Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -1.43% | -0.16% | -0.19% | 130% | 104% |
| Class C | -0.79% | -0.05% | 0.28% | 128% | 104% |
| Class E | -1.50% | 0.74% | 0.68% | 121% | 101% |
| Overall | -1.23% | 0.10% | 0.18% | 127% | 103% |
| Class D | -0.82% | 0.85% | 1.09% | 129% | 107% |
| Class F | -0.43% | -0.21% | 0.51% | 119% | 102% |

## CE1-7.3 Uniform Luma Inter Prediction Filter and Hadamard as a inloop filter on top

Here, the uniform luma inter prediction filter is combined with CE1-2.3 as can be found in [7].

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.48% | 0.39% | 107% | 105% |
| Class A2 | -0.22% | 0.47% | 0.31% | 107% | 104% |
| Class B | -0.11% | 0.35% | 0.47% | 108% | 105% |
| Class C | -0.41% | 0.31% | 0.31% | 108% | 105% |
| Class E | -0.16% | 0.38% | 0.18% | 108% | 105% |
| Overall | -0.18% | 0.39% | 0.35% | 108% | 105% |
| Class D | -0.34% | 0.47% | 0.19% | 108% | 104% |
| Class F | -0.62% | 0.26% | 0.24% | 105% | 104% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | -0.87% | -0.56% | -0.92% | 114% | 100% |
| Class C | -0.55% | 0.06% | -0.22% | 114% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.50% | -0.30% | -0.23% | 114% | 101% |
| Class F | -0.80% | -0.02% | -0.06% | 109% | 101% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.45% | 1.50% | 1.09% | 125% | 101% |
| Class E | -0.01% | 1.83% | 1.82% | 113% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.59% | 1.29% | 0.11% | 126% | 102% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

## CE1-7.4 Combine Uniform Luma Inter Prediction Filter and Hadamard as reconstruction filter mutually exclusive

Here, the uniform luma inter prediction filter is combined with CE1-2.1b as can be found in [7].

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | -0.80% | -0.61% | -0.82% | 111% | 100% |
| Class C | -0.31% | -0.14% | -0.29% | 112% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.32% | -0.15% | -0.10% | 112% | 100% |
| Class F | -0.29% | -0.14% | -0.13% | 107% | 100% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.43% | 1.00% | 1.04% | 122% | 103% |
| Class E | -0.18% | 1.42% | 1.56% | 114% | 104% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.43% | 1.35% | 0.83% | 123% | 104% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

|  | Low delay P Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.91% | -0.03% | 0.11% | 126% | 103% |
| Class E | -1.45% | 0.69% | 0.70% | 116% | 98% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.82% | 0.57% | 1.28% | 125% | 102% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

## CE1-7.5 Combination of Uniform Luma Inter Prediction Filter and LIC on top

Here, the uniform luma inter prediction filter is combined with CE1-5.1 as can be found in [7].

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.97% | -0.08% | 0.27% | 128% | 100% |
| Class A2 | -0.68% | -0.85% | -0.82% | 122% | 102% |
| Class B | -0.91% | -0.95% | -0.99% | 123% | 100% |
| Class C | -0.42% | -0.43% | -0.45% | 127% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.74% | -0.62% | -0.56% | 125% | 102% |
| Class D | -0.24% | -0.19% | -0.18% | 126% | 102% |
| Class F | -0.96% | -0.72% | -0.67% | 117% | 103% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.53% | 0.11% | -0.08% | 140% | 101% |
| Class E | 0.20% | 1.56% | 1.00% | 115% | 104% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.15% | 0.35% | -1.06% | 137% | 105% |
| Class F | -2.66% | -2.25% | -1.92% | 125% | 98% |

|  | Low delay P Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -1.76% | -1.00% | -0.68% | 142% | 99% |
| Class C | -1.18% | -1.01% | -1.06% | 148% | 100% |
| Class E | -1.11% | 0.25% | 0.46% | 120% | 101% |
| Overall | -1.40% | -0.69% | -0.52% | 138% | 100% |
| Class D | -0.47% | 0.37% | 0.23% | 142% | 102% |
| Class F | -3.57% | -2.49% | -2.50% | 132% | 101% |

## CE1-7.6 Combination of Uniform Luma Inter Prediction Filter and LIC mutually exclusive

Here, the uniform luma inter prediction filter is combined with CE1-5.1 as can be found in [7].

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.92% | -0.26% | -0.01% | 125% | 100% |
| Class A2 | -0.67% | -0.87% | -0.79% | 120% | 101% |
| Class B | -0.89% | -0.84% | -1.00% | 122% | 99% |
| Class C | -0.40% | -0.50% | -0.56% | 126% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.72% | -0.64% | -0.64% | 123% | 100% |
| Class D | -0.19% | -0.13% | -0.33% | 124% | 102% |
| Class F | -0.96% | -0.71% | -0.67% | 116% | 103% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.54% | 0.18% | -0.15% | 137% | 98% |
| Class E | 0.19% | 1.11% | 0.75% | 114% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.12% | 0.18% | -0.84% | 135% | 102% |
| Class F | -2.59% | -2.14% | -2.59% | 124% | 98% |

|  | Low delay P Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -1.19% | -0.56% | -0.84% | 144% | 100% |
| Class E | -1.05% | 0.17% | 0.27% | 120% | 102% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.51% | 0.22% | 0.28% | 140% | 103% |
| Class F | -3.61% | -2.47% | -2.03% | 129% | 98% |

# References

- F.Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, Marrakech, MA, 9–18 Jan. 2019.

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 4),” JVET-M1001-v5, Marrakech, MA, 9–18 Jan. 2019.

- J. Pfaff et al., “Signal Adaptive Diffusion Filters For Video Coding,” JVET-J0038, San Diego, USA, April 2018.

- J. Rasch et al., “CE 10: Signal Adaptive Diffusion Filters For Video Coding (Test 10.4.1-10.4.5)” JVET-K323, Ljubljana, SI, 10–18 July 2018.

- J. Rasch et al., “CE10: Uniform Directional Diffusion Filters For Video Coding (Test 10.4.1-10.4.2)” JVET-L0157, Macao, CN, 3–12 Oct. 2018.

- J. Rasch et al., “CE10: Uniform Directional Diffusion Filters For Video Coding (Test 10.4.1-10.4.2)” JVET-M0042, Marrakech, MA, 9–18 Jan. 2019.

- J. Ström, S. Ikonin, V. Seregin, “Description of Core Experiment 1 (CE1): Post-prediction and post-reconstruction filtering”, JVET-M1021, Marrakech, MA, 9–18 Jan. 2019.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).