| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0061 |
| --- | --- |

| Title: | AHG19: Signalling temporal IDs and levels for HFR backwards compatible bitstreams |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Virginie Drugeon Monzastr. 4c 63225 Langen Germany | Tel: Email: | +49-6103-766240 virginie.drugeon@eu.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

An issue about signalling of maximum temporal ID and level of sub-bitstreams is reported when using temporal scalability to enable backwards compatibility of High Frame Rate (HFR) bitstreams to standard frame rate receivers. Two alternative solutions are suggested that both consists in signaling the number of frame rate versions of the content within the bitstream, together with the maximum temporal ID value and level for each frame rate version. The first solution signals the necessary information within the profile_tier_level syntax structure. The second solution signals the necessary information at the beginning of the Sequence Parameter Set and includes one profile_tier_level syntax structure for each frame rate version of the content.

# Introduction

Temporal scalability has been used for several purposes in video coding, including trick-play operations and encapsulation of versions of different frame rates of the same content within the same bitstream.

The second example is exemplified in Figure 1 below, where a standard frame rate sub-bitstream with a frame rate that is less than or equal to 60Hz can be encapsulated within an HFR bitstream with a frame rate that can go up to 120Hz.

Figure 1: HFR bitstream containing a standard frame rate version encapsulated within the same bitstream

However, in order to efficiently encapsulate bitstreams of different frame rates within the same bitstream, it is necessary to be able to quickly identify the pictures necessary for each version of the coded video sequence. The existing temporal sub-layers as described in HEVC can be used for this purpose, but some additional signalling is required to signal the relevant temporal ID values and associated level for each version of the coded video sequence. In the example of Figure 1, a decoder that wishes to reconstruct the HFR version of the video sequence needs to know that pictures with temporal IDs ranging from 0 to N+1 are necessary and that the bitstream will fulfil the constraints associated with Level 5.2 (for a video resolution of 3840x2160). But if the decoder wishes to reconstruct only the standard frame rate version of the video sequence, it needs to know that pictures with temporal IDs ranging from 0 to N only are necessary and that the sub-bitstream fulfils the constraints associated with Level 5.1.

This has been solved in some application standards such as the DVB standard ETSI TS 101 154 [1] by using the maximum temporal ID value signalled in the HEVC video descriptor at the systems level to be able to signal the level for the standard frame rate base layer, as in the excerpt from ETSI TS 101 154 below:

However, this solution creates a dependency between the systems level syntax and the video elementary stream level syntax. Furthermore, the maximum temporal ID of the standard frame rate base layer must stay the same in case of a switch between an HFR backwards compatible bitstream and a standard frame rate bitstream, because the information contained in the HEVC video descriptor cannot be signalled with frame accuracy.

Two alternative solutions are presented below to signal the maximum temporal ID value and level of the standard frame rate base layer.

# Signalling the maximum temporal ID value and level of the standard frame rate base layer in the profile_tier_level syntax structure

## Introduction

The additional information required is the number of sub-bitstreams having a lower frame rate than the whole bitstream and intended to be independently decodable by a receiver, the maximum temporal ID value of the pictures necessary to decode each sub-bitstream, and their respective levels.

The first suggested alternative is to signal the number of sub-bitstreams together with their maximum temporal ID in the profile_tier_level syntax structure. The rest of the syntax structure would not be modified, but an additional constraint is added that the sub-levels corresponding to every maximum temporal ID value signalled have to be signalled as well.

## Modifications to profile_tier_level syntax structure

| profile_tier_level( maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| general_profile_idc | u(7) |
| general_tier_flag | u(1) |
| general_constraint_info( ) |  |
| general_level_idc | u(8) |
| if( maxNumSubLayersMinus1 > 0 ) |  |
| num_fr_sb | u(3) |
| for( b = 0; b < num_fr_sb; b++ ) |  |
| temporal_id_max_sb[ b ] | u(3) |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) |  |
| sub_layer_level_present_flag[ i ] | u(1) |
| while( !byte_aligned( ) ) |  |
| ptl_alignment_zero_bit | f(1) |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) |  |
| if( sub_layer_level_present_flag[ i ] ) |  |
| sub_layer_level_idc[ i ] | u(8) |
| } |  |

num_fr_sb specifies the number of sub-bitstreams with a lower frame rate within the bitstream. When not present, num_fr_sb in inferred to be equal to 0.

temporal_id_max_sb[ b ] specifies the maximum temporal ID value to decode sub-bitstream b.

sub_layer_level_present_flag[ i ] equal to 1 specifies that level information is present in the profile_tier_level( ) syntax structure for the sub-layer representation with TemporalId equal to i. sub_layer_level_present_flag[ i ] equal to 0 specifies that level information is not present in the profile_tier_level( ) syntax structure for the sub-layer representation with TemporalId equal to i.

It is a requirement of bitstream conformance that sub_layer_level_present_flag[ i ] shall be equal to 1 for i equal to temporal_id_max_sb[ b ] with b=0..num_fr_sb – 1.

# Including one profile_tier_level syntax structure for every version of the content

## Introduction

The second suggested alternative is to signal the number of temporal sub-layers of every frame rate version of the bitstream in the Sequence Parameter Set before the profile_tier_level syntax structure, and to including a profile_tier_level syntax structure for every version.

## Modifications to Sequence Parameter Set

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_max_bitstreams_minus1 | u(3) |
| sps_reserved_zero_5bits | u(5) |
| for( b = 0; b < sps_max_bitstreams_minus1 + 1; b++ ) { |  |
| sps_max_sub_layers_minus1[ b ] | u(3) |
| sps_reserved_zero_5bits | u(5) |
| profile_tier_level( sps_max_sub_layers_minus1[ b ] ) |  |
| } |  |
| sps_seq_parameter_set_id | ue(v) |
| … |  |
| } |  |

sps_max_bitstreams_minus1 plus 1 specifies the maximum number of frame rate versions of the content that may be present in each CVS referring to the SPS. The value of sps_max_bitstreams_minus1 shall be in the range of 0 to 6, inclusive. The value of sps_max_bitstreams_minus1 shall be less than or equal to dps_max_sub_layers_minus1.

sps_max_sub_layers_minus1[ b ] plus 1 specifies the maximum number of temporal sub-layers that may be present in each CVS referring to the SPS for the frame rate version of the content that corresponds to bitstream b. The value of sps_max_sub_layers_minus1[ b ] shall be in the range of 0 to 6, inclusive. The value of sps_max_sub_layers_minus1[ b ] shall be less than or equal to dps_max_sub_layers_minus1 and shall be less than sps_max_sub_layers_minus1[ b – 1 ] for b > 0.

# Conclusion

When using temporal sub-layers for encapsulating a standard frame rate version of the content within a High Frame Rate bitstream, the maximum temporal ID and level of the standard frame rate base layer have to be signaled to receivers that are not HFR capable to enable them to extract and decode the standard frame rate version of the content. This proposal suggests signaling the number of frame rate versions of the content within the bitstream, together with the maximum temporal ID value and level for each frame rate version.

Two alternative solutions are proposed. The first one signals the necessary information within the profile_tier_level syntax structure. The second signals the necessary information at the beginning of the Sequence Parameter Set and includes one profile_tier_level syntax structure for each frame rate version of the content.

# References

- ETSI TS 101 154 “Digital Video Broadcasting (DVB); Specification for the use of Video and Audio Coding in Broadcast and Broadband Applications” https://www.etsi.org/deliver/etsi_ts/101100_101199/101154/02.05.01_60/ts_101154v020501p.pdf

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).