| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0062 |
| --- | --- |

| Title: | AHG19: Signalling HRD models for HFR backwards compatible bitstreams |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Virginie Drugeon Monzastr. 4c 63225 Langen Germany | Tel: Email: | +49-6103-766240 virginie.drugeon@eu.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

An issue about decoding time stamps is reported when using temporal scalability to enable backwards compatibility of High Frame Rate (HFR) bitstreams to standard frame rate receivers. It is suggested to signal CPB removal delays and DPB output delays for each temporal sub-layer contained in the bitstream to solve this issue.

# Introduction

Temporal scalability has been used for several purposes in video coding, including trick-play operations and encapsulation of versions of different frame rates of the same content within the same bitstream.

The second example is exemplified in Figure 1 below, where a standard frame rate sub-bitstream with a frame rate that is less than or equal to 60Hz can be encapsulated within an HFR bitstream with a frame rate that can go up to 120Hz.

Figure 1: HFR bitstream containing a standard frame rate version encapsulated within the same bitstream

However, there is an issue related to Hypothetical Reference Decoder times for the standard frame rate version of the content to be decodable by a non-HFR receiver. In the following explanation, it is assumed that the content is 3840x2160 with a frame rate of 100fps and that the bitstream is backwards compatible to non-HFR receivers and contains a sub-bitstream that has a frame rate of 50fps. The whole bitstream satisfies Level 5.2, while the sub-bitstream must satisfy Level 5.1 to be backwards compatible. Two consecutive frames in decoding order may have decoding time stamps (controlled by the Hypothetical Reference Decoder CPB removal delay) that are only 0.01s apart because of the frame rate of 100fps. Unfortunately, this would violate the constraints of a Level 5.1 decoder, as stated in the HEVC [1] standard Annex A.4.2. For the base layer to be decodable by a receiver conforming to Level 5.1, two consecutive frames in decoding order are expected to only be 0.02s apart.

This has been solved in some application standards such as Rec. ITU-R BT.2073 [2] and the DVB standard ETSI TS 101 154 [3] by mandating that the decoding time stamps must be the same for both the standard frame rate and the High Frame Rate version of the content. This introduces some constraints on the coding structure of the bitstream that may seriously impact the coding efficiency compared to an HFR bitstream that is not intended to be backwards compatible.

The relevant excerpt from ETSI TS 101 154 is shown below:

Rec. ITU-R BT.2073 [2] goes even further into documenting at least one coding structure that fulfils this constraint, as illustrated in the figure below copied from Rec. ITU-R BT.2073 [2].

To avoid such constraints, it would be desirable to enable signalling of different decoding time stamps for each frame rate version of the content intended to be decodable by receivers.

# Signalling CPB removal delays and DPB output delays for every version of the content

## Introduction

It is suggested to enable signalling of different CPB removal delays and DPB output delays for each temporal sub-layer contained in the bitstream.

In order for the necessary timing information to be present for each frame rate version of the video sequence, the flag sub_layer_delays_present_flag[i] is expected to be equal to 1 for every value of i that corresponds to the maximum temporal ID value associated to a frame rate version of the coded video sequence.

## Modifications to the Picture Timing SEI message

The modifications suggested below are based on the syntax and semantics of the picture timing SEI message specified in Annex D of HEVC [1].

| pic_timing( payloadSize ) { | Descriptor |
| --- | --- |
| if( frame_field_info_present_flag ) { |  |
| pic_struct | u(4) |
| source_scan_type | u(2) |
| duplicate_flag | u(1) |
| } |  |
| if( CpbDpbDelaysPresentFlag ) { |  |
| for( i = 0; i < maxNumSubLayersMinus1 + 1; i++ ) { |  |
| sub_layer_delays_present_flag[ i ] | u(1) |
| if( sub_layer_delays_present_flag[ i ] ) { |  |
| au_cpb_removal_delay_minus1[ i ] | u(v) |
| pic_dpb_output_delay[ i ] | u(v) |
| } |  |
| } |  |
| if( sub_pic_hrd_params_present_flag ) |  |
| pic_dpb_output_du_delay | u(v) |
| if( sub_pic_hrd_params_present_flag && sub_pic_cpb_params_in_pic_timing_sei_flag ) { |  |
| num_decoding_units_minus1 | ue(v) |
| du_common_cpb_removal_delay_flag | u(1) |
| if( du_common_cpb_removal_delay_flag ) |  |
| du_common_cpb_removal_delay_increment_minus1 | u(v) |
| for( i = 0; i <= num_decoding_units_minus1; i++ ) { |  |
| num_nalus_in_du_minus1[ i ] | ue(v) |
| if( !du_common_cpb_removal_delay_flag && i < num_decoding_units_minus1 ) |  |
| du_cpb_removal_delay_increment_minus1[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| } |  |

sub_layer_delays_present_flag[ i ] equal to 1 specifies that au_cpb_removal_delay_minus1[i] and pic_dpb_removal_delay[i] are present for the representation of the sub-layer with TemporalId equal to i. sub_layer_delays_present_flag[ i ] equal to 0 specifies that au_cpb_removal_delay_minus1[i] and pic_dpb_removal_delay[i] are not present for the representation of the sub-layer with TemporalId equal to i.

# Conclusion

When using temporal sub-layers for encapsulating a standard frame rate version of the content within a High Frame Rate bitstream, the decoding time stamps for each frame rate version of the content may have to be different to enable the best possible coding structure while enabling non-HFR capable receivers to decode the standard frame rate version of the content. This proposal suggests signalling CPB removal delays and DPB output delays for each temporal sub-layer contained in the bitstream.

# References

- High Efficiency Video Coding (HEVC), Rec. ITU-T H.265 and ISO/IEC 23008-2, Feb. 2018

- Use of the high efficiency video coding (HEVC) standard for UHDTV and HDTV broadcasting, Rec. ITU-R BT-2073, https://www.itu.int/dms_pubrec/itu-r/rec/bt/R-REC-BT.2073-0-201502-I!!PDF-E.pdf

- ETSI TS 101 154 “Digital Video Broadcasting (DVB); Specification for the use of Video and Audio Coding in Broadcast and Broadband Applications”, https://www.etsi.org/deliver/etsi_ts/101100_101199/101154/02.05.01_60/ts_101154v020501p.pdf

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).