| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0064 |
| --- | --- |

| Title: | AHG17: Signalling of only one tile group per picture |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Virginie Drugeon Monzastr. 4c 63225 Langen Germany | Tel: Email: | +49-6103-766240 virginie.drugeon@eu.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

It is proposed to slightly change the current syntax related to declaration of tile groups to signal the use case of having several tiles, but only one tile group in the picture. Two alternative modifications are proposed: using a flag to signal that all tiles in the picture are gathered into one single tile group, or signalling the number of tile groups in the picture higher in the syntax.

# Introduction

During the last JVET meetings, tile groups were adopted to gather tiles either in raster scan order or in rectangular parts of the image. One of the main purposes of tile groups is to offer the possibility to gather several tiles in one NAL unit. Another purpose is to offer the possibility to extract a set of MCTSs as a conforming sub-bistream without the need to rewrite a tile group header. Given these main purposes, there seems to be two main use cases for gathering of tiles within tile groups that are the most likely to be broadly used in real-world applications:

- Use case 1: a tile group contains only one tile

- Use case 2: one tile group contains all the tiles in the image

Use case 1 seems particularly relevant for extracting sets of tiles as a conforming sub-bitstream. Use case 2 seems particularly relevant for applications where tiles are likely to be used for parallel processing, but where extracting sets of tiles as a conforming sub-bitstream is not likely or even not desired. An example of such an application is coding of 8K content using four 4K tiles to enable parallel decoding, but where the four 4K tiles are never intended to be displayed separately. Generally, in some applications such as broadcasting or IP multicasting, if tiles are used to facilitate parallel decoding, it is to be expected that content providers may want to gather all the tiles in a picture into one tile group only.

While having flexible tile groups is useful for other use cases, the two above mentioned use cases should nevertheless be straightforward and easy to signal for any content provider that does not need to use and understand in detail the full capabilities of tile groups. The current VVC draft already contains straightforward signalling for use case 1: it is signalled in the PPS by a flag single_tile_per_tile_group_flag. However, there are currently two ways to signal use case 2, both requiring deeper understanding of tile groups usage and capabilities:

- Signalled as raster scan tile group that contains all the tiles in the picture:

  - In PPS single_tile_per_tile_group_flag is set to 0

  - In PPS rect_tile_group_flag is set to 0

  - In tile group header num_tiles_in_tile_group_minus1 is set equal to NumTilesInPic – 1

- Signalled as rectangular tile group over the whole picture:

  - In PPS single_tile_per_tile_group_flag is set to 0

  - In PPS rect_tile_group_flag is set to 1

  - In PPS num_tile_groups_in_pic_minus1 is set to 0

  - In PPS bottom_right_tile_idx[ 0 ] is set to the index of the last tile in the picture

While such a signalling is possible, there does not seem to be any reason to make the signalling of use case 1 so straightforward without also making the signalling of use case 2 straightforward. It is therefore suggested to slightly modify the current PPS syntax to enable straightforward signalling of both use cases. The present document suggests two alternatives for this signalling: introduce a flag or signal the number of tile groups in the image as first syntax element related to tile group. The author of this document believes that the second solution is the most elegant, straightforward and logical signalling.

# Signalling only one tile group in the picture using a flag

## Introduction

The first suggested alternative is to introduce a flag before signalling the tile groups structure within the Picture Parameter Set. The new introduced flag is equal to 1 if all the tiles of the pictures are contained within one tile group only (i.e. the picture contains only one tile group). Otherwise, if the flag is equal to 0, it means that there is more than one tile group within the picture and the rest of the syntax is the same as in the current VVC draft specification, where the tile groups are declared as rectangular or non-rectangular tile groups.

If there is only one tile group within the picture, the flag loop_filter_accross_tile_groups_enabled_flag becomes unecessary.

## Suggested modifications to VVC draft

### Modifications to syntax of PPS

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_tile_spacing_flag | u(1) |
| if( !uniform_tile_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| single_tile_per_tile_group_flag | u(1) |
| if( !single_tile_per_tile_group_flag ) |  |
| all_tiles_in_one_tile_group_flag | u(1) |
| if( !single_tile_per_tile_group_flag && !all_tiles_in_one_tile_group_flag ) |  |
| rect_tile_group_flag | u(1) |
| if( rect_tile_group_flag && !single_tile_per_tile_group_flag && !all_tiles_in_one_tile_group_flag ) { |  |
| num_tile_groups_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| top_left_tile_idx[ i ] | u(v) |
| bottom_right_tile_idx[ i ] | u(v) |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| if( loop_filter_across_tiles_enabled_flag && !all_tiles_in_one_tile_group_flag ) |  |
| loop_filter_across_tile_groups_enabled_flag | u(1) |
| } |  |
| if( rect_tile_group_flag ) { |  |
| signalled_tile_group_id_flag | u(1) |
| if( signalled_tile_group_id_flag ) { |  |
| signalled_tile_group_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) |  |
| tile_group_id[ i ] | u(v) |
| } |  |
| } |  |
| … |  |
| } |  |

### Modifications to semantics of PPS

single_tile_per_tile_group equal to 1 specifies that each tile group that refers to this PPS includes one tile. single_tile_per_tile_group equal to 0 specifies that a tile group that refers to this PPS may include more than one tile.

all_tiles_in_one_tile_group_flag equal to 1 specifies that all the tiles in a picture that refers to this PPS are gathered into one single tile group and that the picture contains only one tile group. all_tiles_in_one_tile_group_flag equal to 0 specifies that a picture that refers to this PPS contains two or more tile groups.

When all_tile_in_one_tile_group_flag is not present, it is inferred to be equal to 0.

rect_tile_group_flag equal to 0 specifies that tiles within each tile group are in raster scan order and the tile group information is not signalled in PPS. rect_tile_group_flag equal to 1 specifies that tiles within each tile group cover a rectangular region of the picture and the tile group information is signalled in the PPS. When single_tile_per_tile_group_flag is equal to 1 rect_tile_group_flag is inferred to be equal to 1. When all_tiles_in_one_tile_group_flag is equal to 1, rect_tile_group_flag is inferred to be equal to 0.

### Modifications to syntax of tile group header

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |
| if( rect_tile_group_flag ¦ ¦ ( NumTilesInPic > 1 && !all_tiles_in_one_tile_group_flag ) ) |  |
| tile_group_address | u(v) |
| if( !rect_tile_group_flag && !single_tile_per_tile_group_flag && !all_tiles_in_one_tile_group_flag ) |  |
| num_tiles_in_tile_group_minus1 | ue(v) |
| tile_group_type | ue(v) |
| tile_group_pic_order_cnt_lsb | u(v) |
| … |  |
| } |  |

### Modifications to semantics of tile group header

num_tiles_in_tile_group_minus1, when present, specifies the number of tiles in the tile group minus 1. The value of num_tiles_in_tile_group_minus1 shall be in the range of 0 to NumTilesInPic − 1, inclusive. When not present and single_tile_per_tile_group_flag is equal to 1, the value of num_tiles_in_tile_group_minus1 is inferred to be equal to 0. When not present and all_tiles_in_one_tile_group_flag is equal to 1, the value of num_tiles_in_tile_group_minus1 is inferred to be equal to NumTilesInPic – 1.

# Signalling the number of tile groups in the picture as first syntax element related to tile groups

## Introduction

The second suggested alternative is to signal the syntax element num_tile_groups_in_pic_minus1 higher in the syntax, as first syntax element related to the tile groups syntax. Signalling this syntax element higher in the syntax actually enables to efficiently signal several major use cases of tile groups and makes some of the syntax elements proposed in the current VVC draft specification redundant:

- When num_tile_groups_in_pic_minus1 is equal to 0, then the whole picture contains only one tile group that contains all the tiles in the picture,

- When num_tile_groups_in_pic_minus1 is equal to the number of tiles in the picture minus one, then there are as many tile groups in the picture as there are tiles, which means that every tile group contains only one tile, which makes the flag single_tile_per_tile_group_flag unecessary,

- For all other cases in between, the number of tile groups in the picture is either a necessary information to be signalled (which is the case for rectangular tile groups) or an information that may be useful for some applications (for non-rectangular raster scan tile groups).

The author of the present document believes that this signalling is the most logical and straightforward for a broad range of applications.

## Suggested modifications to VVC draft

### Modifications to syntax of PPS

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_tile_spacing_flag | u(1) |
| if( !uniform_tile_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| num_tile_groups_in_pic_minus1 | ue(v) |
| if( num_tile_group_in_pic_minus1 ! = NumTilesInPic – 1 && num_tile_group_in_pic_minus1 ! = 0 ) |  |
| rect_tile_group_flag | u(1) |
| if( rect_tile_group_flag && num_tile_group_in_pic_minus1 ! = NumTilesInPic – 1 && num_tile_group_in_pic_minus1 ! = 0 ) { |  |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| top_left_tile_idx[ i ] | u(v) |
| bottom_right_tile_idx[ i ] | u(v) |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| if( loop_filter_across_tiles_enabled_flag && num_tile_group_in_pic_minus1 ! = 0 ) |  |
| loop_filter_across_tile_groups_enabled_flag | u(1) |
| } |  |
| if( rect_tile_group_flag ) { |  |
| signalled_tile_group_id_flag | u(1) |
| if( signalled_tile_group_id_flag ) { |  |
| signalled_tile_group_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) |  |
| tile_group_id[ i ] | u(v) |
| } |  |
| } |  |
| … |  |
| } |  |

### Modifications to semantics of PPS

num_tile_groups_in_pic_minus1 plus 1 specifies the number of tile groups in each picture referring to the PPS. The value of num_tile_groups_in_pic_minus1 shall be in the range of 0 to NumTilesInPic − 1, inclusive. When not present, the value of num_tile_groups_in_pic_minus1 is inferred to be equal to 0.

rect_tile_group_flag equal to 0 specifies that tiles within each tile group are in raster scan order and the tile group information is not signalled in PPS. rect_tile_group_flag equal to 1 specifies that tiles within each tile group cover a rectangular region of the picture and the tile group information is signalled in the PPS. When num_tile_groups_in_pic_minus1 is equal to NumTilesInPic – 1 rect_tile_group_flag is inferred to be equal to 1. When num_tile_groups_in_pic_minus1 is equal to 0 rect_tile_group_flag is inferred to be equal to 0.

### Modifications to syntax of tile group header

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |
| if( rect_tile_group_flag ¦ ¦ ( NumTilesInPic > 1 && num_tile_group_in_pic_minus1 ! = 0 ) ) |  |
| tile_group_address | u(v) |
| if( !rect_tile_group_flag && num_tile_group_in_pic_minus1 ! = NumTilesInPic – 1 && num_tile_group_in_pic_minus1 ! = 0 ) |  |
| num_tiles_in_tile_group_minus1 | ue(v) |
| tile_group_type | ue(v) |
| tile_group_pic_order_cnt_lsb | u(v) |
| … |  |
| } |  |

### Modifications to semantics of tile group header

num_tiles_in_tile_group_minus1, when present, specifies the number of tiles in the tile group minus 1. The value of num_tiles_in_tile_group_minus1 shall be in the range of 0 to NumTilesInPic − 1, inclusive. When not present and num_tile_group_in_pic_minus1 is equal to NumTilesInPic – 1, the value of num_tiles_in_tile_group_minus1 is inferred to be equal to 0. When not present and num_tile_group_in_pic_minus1 is equal to 0, the value of num_tiles_in_tile_group_minus1 is inferred to be equal to NumTilesInPic – 1.

# Conclusion

It is proposed to have straightforward signalling of the use case for which the number of tiles in the picture is greater than one, but all the tiles are gathered into a single tile group. Two alternatives are proposed: adding a flag for this use case, or signalling the number of tile groups in the picture higher in the syntax.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).