| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0066 |
| --- | --- |

| Title: | AHG12: On top-to-bottom tile partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Ahmad Hamza 9276 Scranton Road Suite 300 San Diego, CA 92121, USA | Tel: Email: | 1-858-210-4807 Yong.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

Current VVC bottom-to-top tile partitioning design divides a picture into tile row and column first, then groups multiple tiles into either a raster scan tile group or rectangular tile group. Such bottom-to-top design is not flexible to address needs of emerging video applications, and tile group configuration depends on the tile row and column configuration. This contribution proposes a top-to-bottom flexible tile partitioning design as summarized below:

- The design first divides a picture into rectangular tile groups, and each tile group may be further partitioned into multiple tiles.

- Propose to signal tile group partitioning at PPS,

- Proposed three options to signal local tile partitioning (1) PPS after tile group signaling; (2) tile group header; (3) Sub-picture parameter set (proposed in JVET-N0099). Both (2) and (3) simplify the sub-picture extraction and reposition process.

- Proposed tile group data syntax struct update based on new tile partitioning

# Introduction

Two tile group types are specified in VVC, raster scan tile group and rectangular tile group. Raster tile group is designed for ultra-low latency applications, and rectangular tile group is designed for ROI and sub-picture-based applications. Both designs are bottom-to-top process where a picture is first divided into tile row and column, and then tile group is defined based on the tile grid. Such bottom-to-top design makes sense for ultra-low delay applications but may not be flexible for rectangular tile group. The rectangular tile group boundary is limited by the preceding tile row and column. In addition, current VVC signals both tile configuration and tile group configuration in PPS, “it could be more beneficial to signal tile group configuration before tile configuration for rectangular tile group” as noted in JVET-M0782. A top-to-bottom tile partitioning design is proposed to address these issues.

# Top-to-bottom tile partitioning design

Current tile partitioning method may not support some partitioning structures as illustrated in JVET-K0260 and JVET-L0127. Figure 1 shows partitioning structures stitched from sub-pictures of multiple different resolution video sources. Since a tile cannot be further split into multiple slices in VVC, OMAF tile and slice-based approach for viewport adaptive streaming may not work anymore. As a result, “VVC would have less capability than HEVC for 360 video viewport-dependent delivery optimization” (noted in VET-M0782).

Figure 1 – The OMAF/VRIF partitioning structure

In most use cases, the tile row and column structure can apply to a rectangular region of picture, different tile partitioning may occur in different region of the picture due to sub-picture merging or region of interest processing. Figure 2 shows tile partitioning and grouping examples to support viewport adaptive partitioning structure illustrated in Figure 1.

Figure 2 tile partitioning and grouping examples

As a result, a top-to-bottom tile partitioning is proposed to divide a picture into multiple rectangular tile groups first, each tile group covers integer number of complete CTBs, and then each tile group may be further partitioned into tile rows and columns. Figure 3 shows such partitioning process where a picture is first divided into 4 tile groups, each tile group is further partitioned into 4x1 or 2x2 tile grid.

Figure 3 Two-level tile hierarchy example

It is proposed to signal tile group partitioning at PPS as such application-driven partitioning is consistent for multiple pictures or entire CVS. Table 1 is the proposed tile grouping syntax elements in PPS. The rectangular tile group position is specified by CTU index, and the tile row and column are removed for rectangular tile group. The tile partitioning syntax elements are shows in Table 2.

Table 1 Proposed picture parameter set syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag) { |  |
| single_tile_per_tile_group_flag | u(1) |
| if( !single_tile_per_tile_group_flag ) |  |
| rect_tile_group_flag | u(1) |
| if( !rect_tile_group_flag && !single_tile_per_tile_group_flag ) |  |
| tile_partitioning_struct() |  |
| if( rect_tile_group_flag && !single_tile_per_tile_group_flag ) { |  |
| num_tile_groups_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| top_left_ctu_idx[ i ] | u(v) |
| bottom_right_ctu_idx[ i ] | u(v) |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| if( loop_filter_across_tiles_enabled_flag ) |  |
| loop_filter_across_tile_groups_enabled_flag | u(1) |
| } |  |
| if ( rect_tile_group_flag ) { |  |
| signalled_tile_group_id_flag | u(1) |
| if( signalled_tile_group_id_flag ) { |  |
| signalled_tile_group_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) |  |
| tile_group_id[ i ] | u(v) |
| } |  |
| } |  |
| …. |  |
| } |  |

Table 2 Tile partitioning struct syntax

| tile_partitioning_struct( ) { |  |
| --- | --- |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_tile_spacing_flag | u(1) |
| if( !uniform_tile_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| } |  |

num_tile_groups_in_pic_minus1 plus 1 specifies the number of tile groups in each picture referring to the PPS. The value of num_tile_groups_in_pic_minus1 shall be in the range of 0 to PicSizeInCtbsY − 1, inclusive. When not present and single_tile_per_tile_group_flag is equal to 1, the value of num_tile_groups_in_pic_minus1 is inferred to be equal to PicSizeInCtbsY  − 1.

top_left_ctu_idx[ i ] specifies the CTU index of the tile located at the top-left corner of the i-th tile group. The value of top_left_ctu_idx[ i ] shall not be equal to the value of top_left_ctu_idx[ j ] for any i not equal to j. When not present, the value of top_left_ctu_idx[ i ] is inferred to be equal to i. The length of the top_left_ctu_idx[ i ] syntax element is Ceil( Log2( PicSizeInCtbsY ) bits.

bottom_right_ctu_idx[ i ] specifies the CTU index of the tile located at the bottom-right corner of the i-th tile group. When single_tile_per_tile_group_flag is equal to 1 bottom_right_ctu_idx[ i ] is inferred to be equal to top_left_ctu_idx[ i ]. The length of the bottom_right_ctu_idx[ i ] syntax element is Ceil( Log2( PicSizeInCtbsY ) ) bits.

# Tile group tile partitioning signaling

## Picture parameter set

Each tile group may be further divided into multiple tiles specified in conventional tile row and column. The local tile grid can be signaled in PPS after tile group configuration signaling as shown in Table 3 with changes highlighted in turquoise. The benefit to signal tile group tile partitioning in PPS is mainly for capability negotiation.

Table 3 Proposed tile group tile partitioning syntax elements in PPS

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag) { |  |
| single_tile_per_tile_group_flag | u(1) |
| if( !single_tile_per_tile_group_flag ) |  |
| rect_tile_group_flag | u(1) |
| if( !rect_tile_group_flag ) |  |
| tile_partitioning_struct() |  |
| if( rect_tile_group_flag && !single_tile_per_tile_group_flag ) { |  |
| num_tile_groups_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| top_left_ctu_idx[ i ] | u(v) |
| bottom_right_ctu_idx[ i ] | u(v) |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| if( loop_filter_across_tiles_enabled_flag ) |  |
| loop_filter_across_tile_groups_enabled_flag | u(1) |
| } |  |
| if ( rect_tile_group_flag ) { |  |
| signalled_tile_group_id_flag | u(1) |
| if( signalled_tile_group_id_flag ) { |  |
| signalled_tile_group_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) |  |
| tile_group_id[ i ] | u(v) |
| } |  |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) |  |
| tile_partitioning_struct( ) |  |
| } |  |
| …. |  |
| } |  |

## Tile group header

The tile group tile partitioning can be signaled at tile group header to make each tile group a self-contained entity. Such signaling simplifies the sub-picture sub-bitstream extraction and reposition process since PPS referred by the new formed picture does not have to signal each sub-picture’s tile partitioning. The proposed tile group tile partitioning signaling is shown in Table 4 with changes highlighted in turquoise.

Table 4 Tile group tile partitioning signaling at header syntax

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |
| if( rect_tile_group_flag ¦ ¦ NumTilesInPic > 1 ) |  |
| tile_group_address | u(v) |
| if( !rect_tile_group_flag && !single_tile_per_tile_group_flag ) |  |
| num_tiles_in_tile_group_minus1 | ue(v) |
| if( rect_tile_group_flag && !single_tile_per_tile_group_flag ) |  |
| tile_partitioning_struct() |  |
| …. |  |
| } |  |

## Sub-picture parameter set

As multiple tile group may share the same tile partitioning, and the tile partitioning of a tile group may be kept the same during a CVS. A new parameter set, Sub-picture parameter set (SPPS) proposed in JVET-N0099, can be used to carry tile partitioning syntax elements to reduce the overhead.

Table 5 Proposed sub-picture parameter set syntax

| sub_pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| spps_subpic_parameter_set_id | ue(v) |
| spps_pic_parameter_set_id | ue(v) |
| tile_partitioning_present_flag | u(1) |
| if (tile_partitioning_present_flag) |  |
| tile_partitioning_struct() |  |
| …. |  |
| } |  |

spps_subpic_parameter_set_id identifies the SPPS for reference by other syntax elements. The value of spps_subpic_parameter_set_id shall be in the range of 0 to 63, inclusive.

spps_pic_parameter_set_id specifies the value of pps_pic_parameter_set_id for the active PPS. The value of spps_picq_parameter_set_id shall be in the range of 0 to 63, inclusive.

tile_partitioning_present_flag equal to 1 specifies that tile_partitioning_struct is present in the sub-picture parameter set. tile_partitioning_present_flag equal to 0 specifies that tile_partitioning_struct is not present in sub-picture parameter set.

Based on the proposed sub-picture parameter set, the tile group header is updated in Table 6 with changes highlighted in turquoise.

Table 6 Tile group header update

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_subpic_parameter_set_id | ue(v) |
| if( rect_tile_group_flag ¦ ¦ NumTilesInPic > 1 ) |  |
| tile_group_address | u(v) |
| …. |  |
| } |  |

tile_group_subpic_parameter_set_id specifies the value of spps_pic_parameter_set_id for the SPPS in use. The value of tile_group_pic_parameter_set_id shall be in the range of 0 to 63, inclusive.

# CTB raster and tile scanning conversion process

The CTB raster and tile scanning process for proposed top-to-bottom tile partitioning design is specified as follows.

The list TgWidth[ i ] and TgHeight[ i ] for i ranging from 0 to num_tile_groups_in_pic_minus1, inclusive, specifying the width and height of the i-th tile group in units of CTBs, is derived as follows:

deltaCtuIdx = bottom_right_ctu_idx[ i ] – top_left_ctu_idx[ i ]

TGWidth[ i ] = deltaCtuIdx % PicWidthInCtbsY + 1

TGHeight[ i ] = deltaCtuIdx / PicWidthInCtbsY + 1

NumCtusInTileGroup[ i ] = TGWidth[ i ] * TGHeight[ i ]

The list TGColBd[ i ] for i ranging from 0 to num_tile_groups_in_pic_minus1, inclusive, specifying the location of the i-th tile group column boundary in units of CTBs, is derived as follows:

for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ )
		TGColBd[ i ] = top_left_ctu_idx[ i ] % PicWidthInCtbsY

The list TGRowBd[ i ] for i ranging from 0 to num_tile_groups_in_pic_minus1, inclusive, specifying the location of the i-th tile group row boundary in units of CTBs, is derived as follows:

for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ )
		TGRowBd [ i ] = bottom_right_ctu_idx[ i ] / PicWidthInCtbsY

The list ColWidth[ j ] for j ranging from 0 to num_tile_columns_minus1[ i ], inclusive, specifying the width of the j-th tile column of current tile group in units of CTBs, is derived as follows:

if( uniform_spacing_flag ) for( j = 0; j <= num_tile_columns_minus1; j ++ ) ColWidth[ j ] = (( j + 1) * TGWidth ) / ( num_tile_columns_minus1 + 1 ) − ( j * TGWidth ) / ( num_tile_columns_minus1 + 1 ) else { ColWidth[ num_tile_columns_minus1 ] = TGWidth for( j = 0; j < num_tile_columns_minus1; j ++ ) { ColWidth[ j ] = column_width_minus1 [ j ] + 1 ColWidth[ num_tile_columns_minus1 ] −= ColWidth[ j ] } }

The list RowHeight[ j ] for j ranging from 0 to num_tile_rows_minus1, inclusive, specifying the height of the j-th tile row of current tile group in units of CTBs, is derived as follows:

if( uniform_spacing_flag ) for( j = 0; j <= num_tile_rows_minus1; j++ ) RowHeight[ j ] = ( ( j + 1 ) * TGHeight ) / ( num_tile_rows_minus1 + 1 ) − ( j * TGHeight ) / ( num_tile_rows_minus1 + 1 ) else { RowHeight[ num_tile_rows_minus1 ] = TGHeight for( j = 0; j < num_tile_rows_minus1; j ++ ) { RowHeight[ j ] = row_height_minus1 [ j ] + 1 RowHeight[ num_tile_rows_minus1 ] −= RowHeight [ j ] } }

The list ColBd[ j ] for j ranging from 0 to num_tile_columns_minus1 + 1, inclusive, specifying the location of the j-th tile column boundary of current tile group in units of CTBs, is derived as follows:

| for( ColBd[ 0 ] = 0, j = 0; j <= num_tile_columns_minus1; j ++ ) ColBd[ j + 1 ] = ColBd[ j ] + ColWidth [ j ] |  |
| --- | --- |

The list RowBd[ j ] for j ranging from 0 to num_tile_rows_minus1 + 1, inclusive, specifying the location of the j-th tile row boundary of current tile group in units of CTBs, is derived as follows:

| for( RowBd[ 0 ] = 0, j = 0; j <= num_tile_rows_minus1; j ++ ) RowBd[ j + 1 ] = RowBd[ j ] + RowHeight [ j ] |  |
| --- | --- |

The list CtbAddrRsToTs[ ctbAddrRs ] for ctbAddrRs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in CTB raster scan of a picture to a CTB address in tile scan, is derived as follows:

for( ctbAddrRs = 0; ctbAddrRs < PicSizeInCtbsY; ctbAddrRs++ ) {
	tbX = ctbAddrRs % PicWidthInCtbsY
	tbY = ctbAddrRs / PicWidthInCtbsY
	for( i = 0; i  <=  num_tile_groups_in_pic_minus1; i++ )
		if( tbX  >=  TGColBd[ i ] && tbY >= TGRowBd[ i ])
			tgIdx = i
	for( CtbAddrRsToTs[ ctbAddrRs ] = 0, i = 0; i  < tgIdx; i++ )
		CtbAddrRsToTs[ ctbAddrRs ] += NumCtusInTileGroup[ i ]
	tbX −= TGColBd[ tgIdx ]
	tbY −= TGRowBd[ tgIdx ]  
	for( i = 0; i  <=  num_tile_columns_minus1[ tgIdx ]; i++ )
		if( tbX >=  ColBd[ tgIdx ])
			tileX = i
	for( j = 0; j  <=  num_tile_rows_minus1[ tgIdx ]; j++ )	
		if( ( tbY >=  RowBd[ j ] )
			tileY = j
	for( i = 0; i < tileX; i++ )
		CtbAddrRsToTs[ ctbAddrRs ]  +=  RowHeight[ tgIdx ][ tileY ] * ColWidth[ tgIdx ][ i ]
	for( j = 0; j < tileY; j++ )
		CtbAddrRsToTs[ ctbAddrRs ]  +=  TGWidth[ tgIdx ] * RowHeight[ tgIdx ][ j ]
	CtbAddrRsToTs[ ctbAddrRs ]  +=  ( tbY – RowBd[ tgIdx ][ tileY ]) * ColWidth[ tgIdx ][ tileX ] 						+ tbX – ColBd[ tgIdx ][ tileX ]
}

The list CtbAddrTsToRs[ ctbAddrTs ] for ctbAddrTs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in tile scan to a CTB address in CTB raster scan of a picture, is derived as follows:

for( ctbAddrRs = 0; ctbAddrRs < PicSizeInCtbsY; ctbAddrRs++ ) 
	CtbAddrTsToRs[ CtbAddrRsToTs[ ctbAddrRs ] ] = ctbAddrRs

The list TGIdx[ ctbAddrTs ] for ctbAddrTs ranging from 0 to PicSizeInCtbsY  − 1, inclusive, specifying the conversion from a CTB address in tile scan to a tile group index, is derived as follows:.

for( ctbAddrTs = 0; ctbAddrTs < PicSizeInCtbsY; ctbAddrTs ++) { for( numCtbs = 0, i = 0; i < num_tile_groups_in_pic_minus1; i++ ) { if( ctbAddrTs >= numCtbs ) TGIdx[ ctbAddrTs ] = i numCtbs += NumCtusInTileGroup[ i ] } }

The list TileId[ ctbAddrTs ] for ctbAddrTs ranging from 0 to PicSizeInCtbs  − 1, inclusive, specifying the conversion from a CTB address in tile scan to a tile ID of associated tile group, is derived as follows:

for( ctbAddrTs = 0; ctbAddrTs < PicSizeInCtbsY; ctbAddrTs ++) {
	tgIdx = TGIdx[ ctbAddrTs ] 
	for ( numCtbs = 0, i = 0; i < tgIdx; i ++ ) 
		numCtbs += NumCtusInTileGroup[ i ] 
	for( j = 0, tileIdx = 0; j  <=  num_tile_rows_minus1[  tgIdx ]; j++ )
		for( i = 0; i  <=  num_tile_columns_minus1[  tgIdx ]; i++, tileIdx++ ) {
			FirstCtbAddrTs[  tgIdx ][ tildIdx ] = numCtbs + RowBd[ tgIdx ][ j ] * TGWidth [ tgIdx ] 						+ ColBd[ tgIdx ][ i ] )
			if ( ctbAddrTs >= FirstCtbAddrsTs[  tileGrpIdx ][ tildIdx ] ) 
				TileId[ ctbAddrTs] = tileIdx
}

The list NumCtusInTile[ i ][ j ] for j ranging from 0 to NumTilesInTileGroup[ i ] − 1, inclusive, specifying the conversion from a tile group index and a tile index to the number of CTUs, is derived as follows:

for( tgIdx = 0, tgIdx  <=  num_tile_groups_in_pic_minus1; tgIdx++ ) 
	for( j = 0, tileIdx = 0; j  <=  num_tile_rows_minus1[ tgIdx ]; j++ )
		for( i = 0; i  <=  num_tile_columns_minus1[ tgIdx ]; i++, tileIdx++ ) 
		NumCtusInTile[ tgIdx ][ tileIdx ] = ColWidth[ tgIdx ][ i ] * RowHeight[ tgIdx ][ j ]

# Tile group data syntax

Table 4 shows the tile group data syntax update for proposed top-to-bottom tile partitioning design.

| tile_group_data( tileGroupIdx ) { | Descriptor |
| --- | --- |
| for( i = 0; i < NumTilesInCurrTileGroup; i++, tileIdx++ ) { |  |
| ctbAddrInTs = FirstCtbAddrTs[ tileGroupIdx ][ i ] |  |
| for( j = 0; j < NumCtusInTile[ i ]; j++, ctbAddrInTs++ ) { |  |
| if( ( j % ColWidth[j ] ) = = 0 ) { |  |
| NumHmvpCand = 0 |  |
| NumHmvpIbcCand = 0 |  |
| } |  |
| CtbAddrInRs = CtbAddrTsToRs[ ctbAddrInTs ] |  |
| coding_tree_unit( ) |  |
| } |  |
| end_of_tile_one_bit /* equal to 1 */ | ae(v) |
| if( i < NumTilesInCurrTileGroup − 1 ) |  |
| byte_alignment( ) |  |
| } |  |
| } |  |

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).