| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0067 |
| --- | --- |

| Title: | AHG17: On the first byte of the NAL unit header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Kashyap Kammachi-Sreedhar Nokia Technologies Hatanpaan valtatie 30 33100 Tampere Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The contribution proposes to remove forbidden_zero_bit from the NAL unit header. It is asserted that MPEG-2 Program Elementary Stream (PES) start code emulation is avoided with the proposed syntax element arrangement in the first byte of the NAL unit header. The proposal can be summarized as follows:

- The following first byte of the NAL unit header syntax is proposed:

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| zero_tid_required_flag | u(1) |
| nuh_temporal_id_plus1 | u(3) |
| nal_unit_type_lsb | u(4) |
| … // remaining part of the NAL unit header |  |
| } |  |

- NalUnitType is set equal to ( zero_tid_required_flag << 4 ) + nal_unit_type_lsb.

- The assignment of NAL unit type values to types is rearranged. zero_tid_required_flag shall be equal to 1 for DPS, SPS, EOS, EOB, and IRAP NAL units, which therefore have NalUnitType in the range of 16 to 31, inclusive. The proposed number and assignment of reserved NAL unit types differs from that in VVC draft 4.

- When zero_tid_required_flag is equal to 1, nuh_temporal_id_plus1 shall be equal to 1 (i.e., TemporalId shall be equal to 0).

The contribution leaves the discussion of the content of the remaining part of the NAL unit header beyond the first byte, if any, to other contributions. If no decision on the remaining part of the NAL unit header is reached, it is proposed to add one additional reserved bit to nuh_reserved_zero_7bits, i.e. make it nuh_reserved_zero_8bits.

# Introduction

MPEG-2 Program Elementary Stream (PES) start codes consist of a 24-bit prefix (0x000001) and an 8-bit stream_id specified in Table 2-22 of ITU-T Rec. H.222.0 | ISO/IEC 13818-1. stream_id values are in the range of 10111100b to 11111111b, i.e. 188 to 255, as can be observed from the following excerpt of Table 2-22 of ITU-T Rec. H.222.0 | ISO/IEC 13818-1:

forbidden_zero_bit causes the value of the first byte of the NAL unit header to be in the range of 0 to 127, inclusive. Hence, the combination of a VVC start code prefix 0x000001 and the first byte of the NAL unit header does not cause an emulation of MPEG-2 PES start code. However, it is asserted that MPEG-2 PES start code emulation can be avoided by disallowing values 188 to 255, inclusive, in the first byte of the NAL unit header, while values in the range of 0 to 187, inclusive, could be allowed. With the syntax proposed in this contribution the value of the one-byte NAL unit header is in the range of 16 (i.e., 0001000b) to 159 (i.e., 10001111b), inclusive, thus not causing MPEG-2 PES start code emulation.

# Proposed specification text

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| zero_tid_required_flag | u(1) |
| nuh_temporal_id_plus1 | u(3) |
| nal_unit_type_lsb | u(4) |
| … // remaining part of the NAL unit header |  |
| } |  |

zero_tid_required_flag equal to 0 specifies that zero_tid_required_flag does not impose any additional constraints on the value of nuh_temporal_id1_plus1. When zero_tid_required_flag is equal to 1, nuh_temporal_id_plus1 shall be equal to 1.

The semantics of nuh_temporal_id_plus1 are unchanged.

nal_unit_type_lsb specifies the least significant bits for the NAL unit type. The variable NalUnitType, which indicates the NAL unit type, is set equal to ( zero_tid_required_flag << 4 ) + nal_unit_type_lsb.

Replace all occurrences of nal_unit_type with NalUnitType in VVC draft 4.

Table 7-1 is replaced with the following:

| NalUnitType | Name of NalUnitType | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 1 | AUD_NUT | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 2 3 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| 4..7 | RSV_NVCL5.. RSV_NVCL7 | Reserved | non-VCL |
| 8 | TRAIL | Coded tile group of a non-STSA trailing picture tile_group_layer_rbsp( ) | VCL |
| 9 | STSA | Coded tile group of an STSA picture tile_group_layer_rbsp( ) | VCL |
| 10 | RADL | Coded tile group of a RADL picture tile_group_layer_rbsp( ) | VCL |
| 11 | RASL | Coded tile group of a RASL picture tile_group_layer_rbsp( ) | VCL |
| 12..15 | RSV_VCL_12.. RSV_VCL_15 | Reserved non-IRAP VCL NAL unit types | VCL |
| 16 | DPS_NUT | Decoding parameter set decoding_parameter_set_rbsp( ) [Ed. Assuming that the HLS BoG decision in JVET-M is endorsed in Track/Plenary meeting in JVET-N.] | non-VCL |
| 17 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 18 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 19 | EOB_NUT | End of bitstream end_of_bitstream_rbsp( ) | non-VCL |
| 20..23 | RSV_NVCL20.. RSV_NVCL23 | Reserved | non-VCL |
| 24 25 | IDR_W_RADL IDR_N_LP | Coded tile group of an IDR picture tile_group_layer_rbsp( ) | VCL |
| 26 | CRA_NUT | Coded tile group of a CRA picture tile_group_layer_rbsp( ) | VCL |
| 27 | RSV_IRAP_VCL27 | Reserved IRAP VCL NAL unit type | VCL |
| 28..31 | UNSPEC28.. UNSPEC31 | Unspecified | non-VCL |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).