| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0069 |
| --- | --- |

| Title: | AHG16/AHG17: Proposed Cleanup for Reshaper High Level Syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Brian Heng Minhua Zhou Wade Wan | Email: | brian.heng@broadcom.com minhua.zhou@broadcom.com wade.wan@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

The following document discusses potential issues with the high-level syntax for the reshaper tool. It is asserted that the current syntax has undefined behavior, and that it would cause issues for random access, temporal scalability, and trick-modes (fast-forward, reverse, etc…).

This contribution proposes to:

- Define the behavior when the reshaper is enabled but a model is not present in a tile-group.

- Add a default reshaper model in the SPS.

- Send additional reshaper syntax only if the reshaper is actually enabled for a tile-group.

# Introduction

In-loop reshaping was introduced in JVET-J0015, and was adopted during the Marrakech meeting. This tool is intended to reshape luma sample values inside the coding loop according to a 1-D LUT for better coding efficiency.

The reshaper adds the following syntax to the tile group header.

if ( sps_reshaper_enabled_flag ) {

tile_group_reshaper_model_present_flag

if ( tile_group_reshaper_model_present_flag )

tile_group_reshaper_model ( )

tile_group_reshaper_enable_flag

if ( tile_group_reshaper_enable_flag && (!( qtbtt_dual_tree_intra_flag && tile_group_type == I ) ) )

tile_group_reshaper_chroma_residual_scale_flag

}

Note that the syntax above includes the two following flags:

tile_group_reshaper_model_present_flag

This flag indicates whether or not a reshaping model is carried in this tile group.

tile_group_reshaper_enable_flag

This flag indicates whether reshaping is enabled for this tile group or not.

It is asserted that there are some issues with the usage of these flags that need to be addressed. These issues are described in Section 2 and 3, and potential syntax changes are provided in Section 4.

# Reshaper Model Not Present

From the JVET-M0427 document, it is unclear what the expected behavior is intended to be when the reshaper is enabled, but there is no model present.

Specifically, referring to the combination of:

tile_group_reshaper_model_present_flag=0

tile_group_reshaper_enable_flag=1.

Based on the behavior of the VTM decoder, this combination would appear to indicate that the decoder should use whatever was used in the previous tile-group or picture. This introduces several issues.

First, if the above combination is signalled in the first picture, the reshaper LUTs are invalid and appear to be filled with zeros in VTM.

Second, if signalled at random access points, requiring the use of a previous model (from before the random access point) breaks the concept of random access.

Third, for any usage mode where the decoder only decodes a subset of pictures (temporal scalability, fast-forward, rewind, etc…) the concept of “previous” picture is not applicable.  Decoders may drop certain temporal layers, and the required “previous” reshaper model might be missing in a given sub-set.

# Reshaper Model Present But Not Enabled

Additionally, it is unclear why it is possible to send a reshaper model when the reshaper is not enabled for that tile group.

Specifically, the combination of:

tile_group_reshaper_model_present_flag=1

tile_group_reshaper_enable_flag=0

It doesn’t necessarily break anything to do this, but it seems unclear why this combination is useful.

# Proposed Syntax Changes

To address the issues described in Section 2, this contribution proposes to send a default reshaper model in the SPS. The sps_default_reshaper_model() indicated below would have the same syntax as tile_group_reshaper_model().

seq_parameter_set_rbsp( ) {

…

sps_reshaper_enabled_flag

if( sps_reshaper_enabled_flag ) {

sps_default_reshaper_model ( )

}

…

}

In the event the bitstream signals the combination:

tile_group_reshaper_model_present_flag=0

tile_group_reshaper_enable_flag=1

the decoder would use the default model from the SPS.  This enables an encoder to send a reshaper model once and reuse it from frame to frame.

At the tile-group level, an encoder could still override this default SPS model and send a different one for that specific tile-group.  But, by defining a default model, this approach would not have the same issues of propagating a “previous” model from frame to frame.

Alternatively, if and when the usage of adaptation parameter sets (APS) is further defined for sharing values across pictures, the default reshaper model could potentially be carried in the APS instead.

To address the issue described in Section 3, the tile group header syntax could be changed as follows. Specifically, move the tile_group_reshaper_enable_flag ahead of the tile_group_reshaper_model_present_flag, and skip all the remaining reshaper syntax if the reshaper is not actually enabled for that tile group.

if( sps_reshaper_enabled_flag) {

tile_group_reshaper_enable_flag

if( tile_group_reshaper_enable_flag ) {

tile_group_reshaper_model_present_flag

if(tile_group_reshaper_model_present_flag )

tile_group_reshaper_model ( )

if( !( qtbtt_dual_tree_intra_flag && tile_group_type == I ) )

tile_group_reshaper_chroma_residual_scale_flag

}

}

# Patent rights declaration(s)

Broadcom Inc. does not have current or pending patent rights relating to the technology described in this contribution.