| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0070-v2 |
| --- | --- |

| Title: | AHG16/AHG8: Proposed Cleanup for Reference Wraparound |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Brian Heng Minhua Zhou Wade Wan | Email: | brian.heng@broadcom.com minhua.zhou@broadcom.com wade.wan@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

The following document discusses potential issues with the motion compensation reference wrapping tool. It is asserted that the current specification of this tool has some unintended corner cases that impose undue complexity on decoders.

This contribution proposes to limit the range of sps_ref_wraparound_offset_minus1 as follows:

sps_ref_wraparound_offset_minus1 shall be in the range of (CtbSizeY/MinCbSizeY) + 1 to ( pic_width_in_luma_samples / MinCbSizeY ) − 1, inclusive.

And if (CtbSizeY/MinCbSizeY + 1) > ( pic_width_in_luma_samples / MinCbSizeY  − 1), sps_ref_wraparound_enabled_flag is not signalled in SPS, it is inferred to be equal to 0.

In addition, this contribution proposes to replace the wrapping modulus operation “ClipH” with a conditional offset:

ClipH(o, W, x) =   x  + o;x  < 0x -o;x  > W-1x;otherwise

And add a clipping operation:

xInti = Clip3( 0, picW − 1, ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, ( xIntL + i ) ))

so that any reference location that would have wrapped around more than once will get clipped within the picture boundary instead.

# Introduction

Horizontal wrap-around motion compensation (JVET-L0231) was adopted to reduce visual artifacts where the left and right picture boundaries meet during 360˚ video rendering.  JVET-L0231 defines a wraparound offset for padded equirectangular projection (PERP).  Any reference pixels that extend beyond the border of the picture are wrapped around according to a wraparound offset (with modulo wrapping) as shown in Figure 1.

Figure 1: Reference wraparound with horizontal geometry PERP.

The wraparound offset is specified in the text as follows.

“sps_ref_wraparound_offset_minus1 plus 1 specifies the offset used for computing the horizontal wrap-around position in units of MinCbSizeY luma samples. The value of ref_wraparound_offset_minus1 shall be in the range of 0 to ( pic_width_in_luma_samples / MinCbSizeY ) − 1, inclusive.”

Typically, this wraparound offset would be set equal to pic_width_in_luma_samples for regular ERP (pixels beyond the left edge are wrapped around to the right edge), or slightly smaller than pic_width_in_luma_samples for PERP. But note that the wraparound offset can currently be as small as a single MinCbSizeY unit (as small as 4 pixels).

It is asserted that the reference wrapping tool has some unintended corner cases that are undesirable and impose undue complexity on decoders.

# Multiple Wraparounds

As currently defined, small wraparound offsets can cause repeated wrapping. Consider a 135x135 reference patch at the border of a picture, and assume the wraparound offset is set equal to 4 (sps_ref_wraparound_offset_minus1=0) as shown in Figure 2.

Each time the reference block wraps-around at the left edge, it is shifted to the right by the wraparound offset (4 pixels). However, the resulting patch still extends beyond the left picture edge, and the wrap around process repeats.  This repeated wrapping can occur up to 34 times for a single reference patch.

Figure 2: Example of multiple wraparounds. Large reference blocks with small wraparound offsets can wrap around multiple times. In the above example, a reference patch of width 135 pixels is wrapped around 34 times when the minimum offset size is 4 pixels.

Corner cases like the example shown in Figure 2 are not the expected usage. As stated in JVET-M0225, this was not the intension of the original proposal.  Typical usage of this tool would be to set the wraparound offset equal to the picture width (ERP) or just under the picture width (PERP). It is believed that wraparound offsets as small as 4 pixels are not particularly useful and they impose a significant amount of logic to handle such corner-cases like the one illustrated in Figure 2.

Therefore, it is desirable to limit reference wraparound to wrap at most once.

To eliminate the unintended result illustrated in Figure 2, it is suggested to impose a minimum wraparound offset of CtbSizeY+8. Specifically, the sps_ref_wraparound_offset_minus1 shall be in the range of (CtbSizeY/MinCbSizeY) + 1 to ( pic_width_in_luma_samples / MinCbSizeY ) − 1, inclusive.

By imposing this minimum offset, even the largest reference patch will wrap at most one time.

# Narrow Pictures

Another corner case caused by reference wraparound is for pictures that are around 1 CTU wide, see Figure 3.  Specifically, any picture less than (CtbSizeY + 8). In this case, both the left and right sides of a patch can extend beyond the picture border and require wrapping in opposite directions.

Figure 3: Reference patches (up to 135 pixels wide) can wrap around on both the left and right edges if the picture size is 1 CTU wide.

This issue is not as significant a problem as the repeated wrapping problem that was illustrated in Figure 2, but it is an additional corner case that would need to be handled by decoders.  It would be preferable to avoid adding the logic to handle this unlikely corner case.

If the minimum value of sps_ref_wraparound_offset_minus_1 proposed in Section 2 is adopted, it also has the side effect of prohibiting reference wraparound in pictures that are 1 CTU wide.  Specifically, there would be no valid value of sps_ref_wraparound_offset_minus1 for any picture where (CtbSizeY/MinCbSizeY + 1) > ( pic_width_in_luma_samples / MinCbSizeY  − 1).

This condition therefore implies that reference wraparound feature would be required to be disabled in such sequences (sps_ref_wraparound_enabled_flag=0) in order to generate a valid bitstream. Therefore, it is suggested to transmit the sps_ref_wraparound_enabled_flag only if this minimum picture size is met, and infer sps_ref_wraparound_enabled_flag=0 when it is not.

…

if ( (CtbSizeY/MinCbSizeY + 1) <= ( pic_width_in_luma_samples / MinCbSizeY − 1) ) {

sps_ref_wraparound_enabled_flag

if( sps_ref_wraparound_enabled_flag )

sps_ref_wraparound_offset_minus1

}

…

“sps_ref_wraparound_enabled_flag equal to 1 specifies that horizontal wrap-around motion compensation is applied in inter prediction. sps_ref_wraparound_enabled_flag equal to 0 specifies that horizontal wrap-around motion compensation is not applied. When sps_ref_wraparound_enabled_flag is not present, it is inferred to be equal to 0.”

It is believed that this is an acceptable restriction. Real use cases for sequences 360˚ video sequences that are just 1 CTU wide are expected to be limited (if any), and such sequences can still be coded with traditional tools.

# Motion Vector Modulus Wrapping

As currently specified in the VVC draft text, the reference wrapping operation is implemented with a modulus operator. The specific operation is defined below, where o is the wraparound offset and W is the picture width.

ClipH( o, W, x ) =   x % o;x < 0 x-W % o + W-o;x > W-1x;otherwise

First, the current text indicates that the % operator is not defined for x<0, so the behavior for the first case specified above is undefined.

Second, motion vectors are not restricted in how far outside the image they can point outside the image (besides the maximum vector range defined in the standard, −217 to 217 − 1, inclusive).  For small wraparound offsets with vectors pointing very far off the left or right edge of the image, the vector can wrap around potentially thousands of times.

Correctly determining where a vector points after repeated wrapping either requires a division operation (remainder after division by wraparound offset), or it would require up to potentially thousands of iterations of adding or subtracting the wraparound offset.  See JVET VVC Bug Report Ticket #151

https://jvet.hhi.fraunhofer.de/trac/vvc/ticket/151

For example, to fix the software implementation to match the draft text, VTM 4 implements this operation with while loops:

while( mvX > iHorMax ) {

mvX -= ( sps.getWrapAroundOffset() << iMvShift );

}

while( mvX < iHorMin ) {

mvX += ( sps.getWrapAroundOffset() << iMvShift );

}

It is asserted that there is no practical reason why an encoder would need to code a vector that wraps more than once, and there would be additional logic required to handle these vectors correctly in the decoder.

Therefore, it is suggested to replace the modulus operator with a single offset, followed by the standard clip. The following solution is proposed:

If sps_ref_wraparound_enabled_flag equals 0:

xInti = Clip3( 0, picW − 1, xIntL + i )

Otherwise:

ClipH( o, W, x ) =   x  + o;x  < 0x -o;x  > W-1x;otherwise

xInti = Clip3( 0, picW − 1, ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, ( xIntL + i ) ))

In this way, any reference location that would have wrapped around more than once will get clipped within the picture boundary instead.

# Draft Text

5.8 Mathematical functions

…

Clip3( x, y, z ) =  x;z < xy;z > yz;otherwise		(5-8)

ClipH( o, W, x ) =   x % o;x < 0 x-W % o + W-o;x > W-1x;otherwise	(5-9)

ClipH( o, W, x ) =   x  + o;x  < 0x -o;x  > W-1x;otherwise		(5-9)

Cos( x )	the trigonometric cosine function operating on an argument x in units of radians.	(5-10)

…

#### 7.3.2.1	Sequence parameter set RBSP syntax

| … |  |
| --- | --- |
| log2_diff_max_min_pcm_luma_coding_block_size | ue(v) |
| pcm_loop_filter_disabled_flag | u(1) |
| } |  |
| if ( (CtbSizeY/MinCbSizeY + 1) <= ( pic_width_in_luma_samples / MinCbSizeY − 1) ) { |  |
| sps_ref_wraparound_enabled_flag | u(1) |
| if( sps_ref_wraparound_enabled_flag ) |  |
| sps_ref_wraparound_offset_minus1 | ue(v) |
| } |  |
| sps_temporal_mvp_enabled_flag | u(1) |
| if( sps_temporal_mvp_enabled_flag ) |  |
| sps_sbtmvp_enabled_flag | u(1) |
| … |  |

#### 7.4.3.1	Sequence parameter set RBSP semantics

…

sps_ref_wraparound_enabled_flag equal to 1 specifies that horizontal wrap-around motion compensation is applied in inter prediction. sps_ref_wraparound_enabled_flag equal to 0 specifies that horizontal wrap-around motion compensation is not applied. When sps_ref_wraparound_enabled_flag is not present, it is inferred to be equal to 0.

sps_ref_wraparound_offset_minus1 plus 1 specifies the offset used for computing the horizontal wrap-around position in units of MinCbSizeY luma samples. The value of ref_wraparound_offset_minus1 shall be in the range of 0 (CtbSizeY/MinCbSizeY) + 1 to ( pic_width_in_luma_samples / MinCbSizeY ) − 1, inclusive.

…

##### 8.5.3.2.2	Luma sample bilinear interpolation process

…

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..1:

xInti = sps_ref_wraparound_enabled_flag ?
	ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, ( xIntL + i ) ) :	(8-459)
	Clip3( 0, picW − 1, xIntL + i )

xInti = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?     ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, ( xIntL + i ) ) :	(8-459) xIntL + i )

yInti = Clip3( 0, picH − 1, yIntL + i )		(8-460)

…

# References

[1] B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 3)”, JVET-M1001-v6, the 13th JVET meeting, Jan. 2019, Marrakesh, MA

# Patent rights declaration(s)

Broadcom Inc. does not have current or pending patent rights relating to the technology described in this contribution.