| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0074-v2 |
| --- | --- |

| Title: | CE1-5.5: Simplification of local illumination compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chia-Ming Tsai, Chun-Chia Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chia-ming.tsai, chunchia.chen, cw.hsu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes two methods to simplify the local illumination compensation (LIC) based on CE1-5.3. In the first method, to reduce encoding time, three encoder-only changes are included: 1) reusing the LIC-off motion search results for testing LIC-on as the method in JVET-M0182, 2) conditionally testing mean removed sum of absolute differences (MR-SAD) motion search with LIC-on when the LIC usage ratio of neighbouring 4×4 subblocks is larger than or equal to a threshold, 3) when testing MR-SAD motion search with LIC-on, if the current best mode of the current coding unit (CU) is uni-prediction, then bi-prediction motion search is skipped. In the second method, to avoid the cascaded design for inter coding tools, bi-predictive with CU weights (this tool name is used in the VVC working draft JVET-M1001 and shortened as BiCW in this contribution; it is called bi-prediction with weighted averaging, BPWA in CE1 description, JVET-M1021) using unequal weights is disallowed when LIC-on. This change is normative on the decoder side to save BiCW-related syntax elements accordingly. Results of the proposed methods compared with VTM4.0 are reported as follows:

CE1-5.5a: Based on CE1-5.3 (allowing LIC for not only uni-prediction but also bi-prediction after bi-predictive combination), using LIC fast encoding + disallowing unequal-weight BiCW with LIC-on
RA BD-rates = -0.42% (Y), -0.16% (U), -0.11% (V); EncT=108%; DecT=101%
LB BD-rates = -0.50% (Y), -0.11% (U), -0.42% (V); EncT=108%; DecT=101%

CE1-5.5b: CE1-5.5a + using only inter coded neighbouring CU samples in LIC parameter derivation
RA BD-rates = -0.30% (Y), -0.29% (U), -0.24% (V); EncT=109%; DecT=102%
LB BD-rates = -0.45% (Y), -0.19% (U), -0.48% (V); EncT=110%; DecT=102%

Additional Test1: Based on CE1-5.1 (allowing LIC for uni-prediction and disallowing LIC for bi-prediction), using LIC fast encoding
RA BD-rates = -0.35% (Y), -0.09% (U), -0.09% (V); EncT=106%; DecT=103%
LB BD-rates = -0.36% (Y), -0.06% (U), -0.55% (V); EncT=106%; DecT=100%

Additional Test2: Additional Test1 + using only inter coded neighbouring CU samples in LIC parameter derivation
RA BD-rates = -0.24% (Y), -0.15% (U), -0.15% (V); EncT=106%; DecT=101%
LB BD-rates = -0.31% (Y), -0.15% (U), -0.13% (V); EncT=106%; DecT=100%

# Introduction

In CE1-5 [1], LIC tests are planned. In LIC, templates are used to derive the linear model parameters for inter prediction. The current template in the current picture is composed of a top current template and a left current template, where the top current template and the left current template are composed of top and left neighbouring CU reconstructed samples, respectively. The reference template in the reference picture is composed of the current template’s motion compensated samples using the current CU motion vector (MV). Although LIC shows non-trivial coding gain, there are two problems caused by LIC: 1) long encoding time, 2) long latency in hardware decoders.

To tackle the long latency issue, several LIC action items were suggested as recorded in JVET-M1000 [2], e.g., further study the impact of using reconstructed intra coded blocks, disable LIC for all subblock modes, not apply to 128xN and Nx128 blocks, bi-prediction issue should be solved, etc. Then a base LIC was released in CE1-5.1 regarding these action items. To relax the constraint to allow LIC only for uni-prediction as implemented in CE1-5.1, CE1-5.3 proposed to allow LIC for not only uni-prediction but also bi-prediction after bi-predictive combination. This contribution further simplifies LIC based on CE1-5.3 and JVET-M0182 [3] to mitigate the long encoding time and long latency issues.

# Proposed methods

In CE1-5.1, MR-SAD is employed during the motion search for LIC-off and LIC-on cases, respectively. To further reduce encoding time, following three encoder-only changes are proposed:

- The motion search results of LIC-off is reused to test LIC-on as proposed in JVET-M0182 [3].

- If the LIC usage ratio of neighbouring 4x4 subblocks is larger than or equal to a threshold (TH), then the MR-SAD motion search with LIC-on is further tested. As illustrated in Figure 1, assuming the current CU size is W×H, the number of neighbouring 4x4 subblocks inside the region marked in red is n, and LIC is applied to m out of n subblocks. If mn≥TH, then MR-SAD motion search with LIC-on is further tested. TH is set to 0.20 in CE1-5.5a and CE1-5.5b.

- When testing MR-SAD motion search with LIC-on, if the current best mode of the current CU is uni-prediction, then bi-prediction motion search is skipped (i.e., only perform uni-prediction motion search).

In CE1-5.3, BiCW and LIC are cascaded. To reduce the processing latency that may cause difficulty for arranging all the prediction processes in one hardware decoder pipeline stage, in addition to above encoder-only changes, it is proposed to disallow BiCW when LIC is on. This change is normative on the decoder side to save BiCW-related syntax elements accordingly.

Figure 1.  Calculate the LIC usage ratio of neighbouring 4x4 subblocks to conditionally test MR-SAD motion search with LIC-on. The current CU is marked in blue. The searched neighbouring 4x4 subblocks are marked in red.

# Experimental results

The proposed simplification methods have been implemented on VTM4.0 [2] and CE1-5.3. Table 1 shows the results of CE1-5.5a, and Table 2 shows the results of CE1-5.5b. The simulations were performed following the JVET common test conditions (CTC) [4].

Table 1.  Results of CE1-5.5a

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.69% | 0.04% | 0.09% | 110% | 101% |
| Class A2 | -0.32% | -0.18% | -0.29% | 108% | 100% |
| Class B | -0.47% | -0.19% | 0.00% | 108% | 102% |
| Class C | -0.24% | -0.26% | -0.26% | 108% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.42% | -0.16% | -0.11% | 108% | 101% |
| Class D | 0.00% | 0.00% | -0.10% | 109% | 101% |
| Class F | -1.07% | -0.56% | -0.70% | 105% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.68% | -0.49% | -0.22% | 109% | 102% |
| Class C | -0.48% | -0.15% | -0.77% | 110% | 101% |
| Class E | -0.24% | 0.57% | -0.28% | 106% | 98% |
| Overall | -0.50% | -0.11% | -0.42% | 108% | 101% |
| Class D | -0.03% | -0.12% | -0.85% | 109% | 103% |
| Class F | -3.16% | -2.65% | -2.86% | 109% | 101% |

Table 2.  Results of CE1-5.5b

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.52% | -0.36% | -0.39% | 110% | 100% |
| Class A2 | -0.20% | -0.27% | -0.26% | 108% | 105% |
| Class B | -0.32% | -0.17% | -0.09% | 109% | 101% |
| Class C | -0.18% | -0.39% | -0.29% | 109% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.30% | -0.29% | -0.24% | 109% | 102% |
| Class D | -0.01% | -0.06% | -0.02% | 110% | 102% |
| Class F | -0.84% | -0.55% | -0.64% | 106% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.61% | -0.43% | -0.59% | 110% | 103% |
| Class C | -0.42% | -0.09% | -0.36% | 111% | 104% |
| Class E | -0.21% | 0.05% | -0.46% | 107% | 99% |
| Overall | -0.45% | -0.19% | -0.48% | 110% | 102% |
| Class D | -0.02% | -0.36% | -1.16% | 110% | 103% |
| Class F | -2.89% | -2.34% | -3.29% | 109% | 101% |

Table 3 and Table 4 show additional results applying the proposed encoder-only changes to CE1-5.1a and CE1-5.1b, respectively. TH is set to 0.10 for additional results, and the neighbouring 4x4 subblocks inside the region marked in red are extended as illustrated in Figure 2.

Figure 2.  When applying the proposed encoder-only changes to CE1-5.1a and CE1-5.1b, the neighbouring 4x4 subblocks for calculating the LIC usage ratio is extended.

Table 3.  Results of the proposed encoder-only changes on CE1-5.1a

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.50% | 0.28% | 0.39% | 107% | 104% |
| Class A2 | -0.31% | -0.26% | -0.32% | 105% | 104% |
| Class B | -0.36% | -0.13% | -0.08% | 105% | 104% |
| Class C | -0.23% | -0.18% | -0.29% | 105% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.35% | -0.09% | -0.09% | 106% | 103% |
| Class D | -0.05% | -0.06% | -0.09% | 105% | 100% |
| Class F | -0.83% | -0.47% | -0.48% | 104% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.52% | -0.16% | -0.36% | 105% | 101% |
| Class C | -0.34% | 0.03% | -0.36% | 106% | 101% |
| Class E | -0.12% | -0.01% | -1.11% | 105% | 99% |
| Overall | -0.36% | -0.06% | -0.55% | 106% | 100% |
| Class D | 0.00% | -0.24% | -0.28% | 106% | 103% |
| Class F | -2.65% | -2.07% | -2.33% | 106% | 99% |

Table 4.  Results of the proposed encoder-only changes on CE1-5.1b

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.37% | -0.09% | -0.08% | 108% | 101% |
| Class A2 | -0.21% | -0.18% | -0.23% | 105% | 101% |
| Class B | -0.23% | -0.12% | -0.11% | 106% | 100% |
| Class C | -0.16% | -0.21% | -0.20% | 106% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.24% | -0.15% | -0.15% | 106% | 101% |
| Class D | -0.04% | -0.02% | -0.05% | 106% | 101% |
| Class F | -0.68% | -0.39% | -0.48% | 105% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.41% | -0.24% | -0.09% | 106% | 100% |
| Class C | -0.30% | -0.16% | -0.04% | 107% | 101% |
| Class E | -0.16% | 0.01% | -0.33% | 105% | 97% |
| Overall | -0.31% | -0.15% | -0.13% | 106% | 100% |
| Class D | -0.06% | -0.32% | -0.68% | 107% | 102% |
| Class F | -2.33% | -1.84% | -2.59% | 107% | 101% |

# Conclusions

This contribution proposes methods to simplify LIC. The encoding time and latency issues for LIC are mitigated.

# References

- J. Ström, S. Ikonin, and V. Seregin, “Description of Core Experiment 1 (CE1): Post-prediction and post-reconstruction filtering,” Document of Joint Video Experts Team, JVET-M1021.

- G. J. Sullivan and J.-R. Ohm, “Meeting report of the 13th JVET meeting (Marrakech, 9–18 January 2019),” Document of Joint Video Experts Team, JVET-M1000.

- C.-M. Tsai, C.-C. Chen, C.-W. Hsu, Y.-W. Huang, and S.-M. Lei, “CE10-related: Simplification of local illumination compensation,” Document of Joint Video Experts Team, JVET-M0182.

- VTM4.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-M1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).