| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0076-v1 |
| --- | --- |

| Title: | CE2-5.3: Simplifications for inherited affine merging candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Ling Hsiao, Tzu-Der Chuang, Chih-Wei Hsu, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {yuling.hsiao, peter.chuang, cw.hsu, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes simplifications for inherited affine merging candidates inside the current coding tree unit (CTU) row. Compared with the inherited affine merging candidates in VTM4.0, it is asserted that the control point motion vector (CPMV) buffer of 7.7K bits can be replaced by a smaller motion vector (MV) buffer of 2.9K bits. Inside the current CTU row (i.e., not at CTU row boundaries), it is proposed to use two or three subblock MVs with half coding unit (CU) height or half CU width for affine candidate inheritance, where CPMVs are not needed anymore for saving the CPMV buffer. Results of the proposed method are summarized as follows.
VTM4.0-RA: {Y-BD-rate=0.01%, EncT=100%, DecT=100%}
VTM4.0-LB: {Y-BD-rate=0.02%, EncT=99%, DecT=99%}

# Introduction

The separation of storages of subblock MVs and CPMVs presented in JVET-L0047 [1] was adopted. In JVET-L0047, it is proposed to use the subblock MV field for motion compensation, merging candidate list derivation, and storage of the temporal motion vector predictors (TMVPs), and to use the CPMVs only for the affine motion inheritance. However, additional CPMVs buffer is needed for storing CPMVs information. This contribution proposes to use two or three subblock MVs with half CU height and half CU width for affine inheritance to reduce buffer usage. It was presented in JVET-M0168 [2] and planned as CE2.5.3 in JVET-M1022 [3]. More details are described in the next sections.

# Proposed method

Two or three subblock MVs with half CU height or half CU width are used for the affine motion inheritance if the candidate block is inside the current CTU row. As shown in Figure 1 as an example, to inherit the affine motion from the left and top candidate CUs, “RB, LB, and RT” to the left of the current CU and “RB and LB” to the top of the current CU are used for deriving a 6-parameter inherited affine model and a 4-parameter inherited affine, respectively, and the distance between RB and LB is half candidate CU width and the distance between RB and RT is half candidate CU height. The CPMV buffer can be removed, and a smaller subblock MV buffer is required. Table 1 summarizes the buffer usage comparison between the proposed method and VTM4.0. The local storage is reduced from 7.7K bits to 2.9K bits.

Figure 1.  Inherited affine candidate derivation inside the current CTU row

Table 1.  Buffer usage comparison between the proposed method and VTM4.0

|  | Local storage (bits per CTU: 36 bits per CPMV, 3 bits for CU width, 3 bits for CU height, 10 bits for CU top-left x and y positions, 1 bit for affine type) | Line buffer (bits per 8x8) | Additional logics |
| --- | --- | --- | --- |
| VTM4.0 | 7689 = (33*(36*3*2+3*2+10+1)) | 8 (3 bits for width, 5 bits for CU x position) |  |
| Proposed method | 2937 = (33*(36*2+3*2+10+1)) | 8 (Same) | No |

# Experiment results

The proposed inherited affine candidate derivation has been implemented on top of VTM4.0. The common test conditions [4] are used for evaluation. Table 2 summarizes the results of the proposed method.

Table 2.  Results of inherited affine candidate derivation inside the current CTU row using subblock MVs with half CU width or half CU height

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.08% | 0.01% | 100% | 100% |
| Class A2 | 0.01% | -0.10% | -0.04% | 99% | 100% |
| Class B | 0.01% | -0.08% | 0.04% | 100% | 100% |
| Class C | 0.03% | -0.02% | -0.09% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.07% | -0.02% | 100% | 100% |
| Class D | 0.08% | 0.00% | 0.01% | 100% | 99% |
| Class F | -0.01% | 0.06% | 0.03% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.13% | -0.37% | 100% | 101% |
| Class C | 0.00% | 0.29% | -0.13% | 100% | 98% |
| Class E | 0.06% | 0.32% | -0.51% | 99% | 95% |
| Overall | 0.02% | 0.13% | -0.32% | 99% | 99% |
| Class D | 0.10% | 0.48% | -0.08% | 101% | 101% |
| Class F | -0.14% | -0.10% | -0.21% | 99% | 101% |

# Conclusion

In this contribution, a method is proposed to simplify inherited affine candidates effectively. The coding efficiency impact is negligible. Compared with the inherited affine candidates in VTM4.0, the buffer usage is reduced.

# References

[1] M. Zhou and B. Heng “CE4-related: A clean up for affine mode,” Document of Joint Video Experts Team, JVET-L0047.

[2] Y.-L. Hsiao, T.-D. Chuang, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, and S.-M. Lei, “CE2-related: Simplifications for inherited affine candidates,” Document of Joint Video Experts Team, JVET-M0168.

[3] C.-C. Chen, Y. He, and H. Liu, “Description of Core Experiment 2 (CE2): Sub-block based Motion Prediction,” Document of Joint Video Experts Team, JVET-M1022.

[4] F. Bossen, J. Boyce, K. Sühring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-M1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).