| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0079-v1 |
| --- | --- |

| Title: | CE2-related: Applying SMR to subblock-based merging candidate list |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Cheng Lin, Chun-Chia Chen, Man-Shu Chiang, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {yucheng.lin, chunchia.chen, man-shu.chiang, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to apply shared merging candidate list region (SMR) to subblock-based merging candidate list. The test results are reported below.

When SMR area threshold is set equal to 128:
VTM4.0-RA: {BD-rate=0.01% (Y), 0.01% (Cb), -0.07% (Cr); EncT=100%; DecT=103%}
VTM4.0-LB: {BD-rate=0.01% (Y), 0.20% (Cb), -0.01% (Cr); EncT=100%; DecT=101%}

When SMR area threshold is set equal to 256:
VTM4.0-RA: {BD-rate=0.02% (Y), -0.05% (Cb), -0.08% (Cr); EncT=100%; DecT=102%}
VTM4.0-LB: {BD-rate=0.03% (Y), 0.12% (Cb), -0.16% (Cr); EncT=100%; DecT=101%}

When SMR area threshold is set equal to 512:
VTM4.0-RA: {BD-rate=0.06% (Y), -0.02% (Cb), -0.05% (Cr); EncT=100%; DecT=103%}
VTM4.0-LB: {BD-rate=0.05% (Y), 0.11% (Cb), -0.23% (Cr); EncT=100%; DecT=102%}

It is asserted that the proposed method can increase hardware decoder throughput when subblock-based merging candidates are enabled.

# Introduction

In a high-end (e.g., 8K 120fps) video decoder, the clock cycle budgets are very tight, so a parallel-design-friendly algorithm is preferred. However, merging candidate list construction of one coding unit (CU) cannot be computed in parallel at the same time with that of the previous CU. In VTM4.0, in addition to regular merge modes, subblock-based merge modes are supported including affine merging candidates and subblock-based temporal motion vector prediction (SbTMVP) merging candidate. The affine merging candidate requires many sequential operations, e.g., availability checks for neighbouring affine CUs for inherited affine candidates, availability checks for control point motion vectors (CPMVs), reference index comparisons for constructed affine candidates, neighbouring subblock motion vector (MV) fetches for inherited and constructed affine candidates, and CPMV transformation. Besides, because of the combination of SbTMVP and affine merging candidates into one subblock-based merging candidate list, the SbTMVP availability checking is sequentially ahead in decoding process when the decoder constructs affine merging candidates. Moreover, subblock-based merge mode is allowed for CUs as small as 8×8, so the real-time hardware decoding is even more challenging. Table 1 shows the clock cycle budgets for the worst case, i.e., 8×8 CU, in subblock-based merge mode to support real-time decoding.

Table 1.  Clock cycle budgets for one 8×8 CU

| Assuming Clock @ 1GHz | 8K 60fps decoder (Single Core) | 8K 120fps decoder (Single Core) | 8K 60fps decoder (Dual-Core) | 8K 120fps decoder (Dual-Core) | 4K 60fps decoder (Single Core) | 4k 120fps decoder (Single Core) |
| --- | --- | --- | --- | --- | --- | --- |
| 8×8 CU | 32 | 16 | 64 | 32 | 128 | 64 |
| Assuming Clock @ 500MHz | 8K 60fps decoder (Single Core) | 8K 120fps decoder (Single Core) | 8K 60fps decoder (Dual-Core) | 8K 120fps decoder (Dual-Core) | 4K 60fps decoder (Single Core) | 4k 120fps decoder (Single Core) |
| 8×8 CU | 16 | 8 | 32 | 16 | 64 | 32 |

# Proposed method

At the last meeting, JVET-M0170 [1] was adopted into VTM-4.0 to apply SMR to regular merge mode. In this contribution, it is proposed to apply SMR also to subblock-based merge mode to alleviate the cycle budget problem and enable parallel processing for subblock-based merge mode. With SMR, all leaf CUs of one ancestor node in the CU split tree can share the same subblock merging candidate list. The ancestor node is named subblock merge sharing node and the subblock shared merging candidate list is generated at the subblock merge sharing node pretending the subblock merge sharing node is a leaf CU. The subblock merge sharing node is an ancestor node of leaf CU which must satisfy the following 2 criteria:

- The subblock merge sharing node size is equal to or larger than a size threshold

- In the subblock merge sharing node, one of the child CU size is smaller than the size threshold

Figure 1 shows two examples of the size threshold equal to 256.

Size Threshold = 256

8x32

Subblock Merge Sharing Node

Subblock Merge Sharing Node

8x16

8x8

8x8

16x8

8x8

8x8

32x8

Figure 1.  Examples of subblock merge sharing node

There are three major benefits in the proposed method. Firstly, it enables parallel processing for subblock-based merge mode for all leaf CUs within one SMR, Secondly, it constructs only one subblock-based merging candidate list for small CUs to meet the tight cycle budget more easily. Thirdly, it is reported that more subblock-based merge algorithms can provide coding gain with increased complexity, e.g., subblock-based history affine merging candidate, affine merge with motion vector difference (affine MMVD), planar mode or exhaustive temporal constructed affine merging candidate. The SMR can reduce the implementation complexity of those new tools and make them more cost effective to further improve the total coding gain for VTM.

# Simulation results

The proposed method has been implemented on top of VTM4.0 [2]. All the tests were conducted according to RA and LB CTCs [3] for evaluation. Test results of the proposed method with area thresholds equal to 128, 256, and 512 are summarized in Table 2, 3 and 4, respectively.

Table 2.  Results of applying SMR to subblock-based merging candidate list with area threshold = 128

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | -0.06% | 100% | 105% |
| Class A2 | -0.01% | -0.03% | 0.02% | 100% | 102% |
| Class B | 0.02% | 0.06% | -0.12% | 100% | 106% |
| Class C | 0.02% | -0.01% | -0.07% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | -0.07% | 100% | 103% |
| Class D | 0.02% | 0.14% | 0.03% | 100% | 108% |
| Class F | 0.00% | 0.03% | 0.05% | 100% | 104% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.09% | -0.16% | 100% | 102% |
| Class C | -0.01% | 0.22% | -0.03% | 100% | 101% |
| Class E | 0.06% | 0.67% | 0.27% | 100% | 98% |
| Overall | 0.01% | 0.20% | -0.01% | 100% | 101% |
| Class D | 0.00% | -0.19% | -0.15% | 101% | 102% |
| Class F | -0.11% | 0.22% | -0.09% | 100% | 101% |

Table 3.  Results of applying SMR to subblock-based merging candidate list with area threshold = 256

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.09% | 0.02% | 100% | 103% |
| Class A2 | -0.01% | 0.04% | -0.02% | 100% | 102% |
| Class B | 0.03% | -0.13% | -0.14% | 100% | 102% |
| Class C | 0.07% | 0.00% | -0.13% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.05% | -0.08% | 100% | 102% |
| Class D | 0.04% | -0.20% | -0.07% | 100% | 105% |
| Class F | 0.01% | 0.04% | 0.04% | 100% | 101% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.06% | -0.22% | 100% | 103% |
| Class C | 0.04% | 0.09% | 0.10% | 100% | 101% |
| Class E | 0.01% | 0.47% | -0.39% | 100% | 98% |
| Overall | 0.03% | 0.12% | -0.16% | 100% | 101% |
| Class D | 0.09% | -0.06% | -0.38% | 103% | 105% |
| Class F | -0.09% | 0.06% | -0.07% | 101% | 100% |

Table 4.  Results of applying SMR to subblock-based merging candidate list with area threshold = 512

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.07% | 0.11% | 100% | 105% |
| Class A2 | 0.01% | -0.03% | 0.06% | 100% | 102% |
| Class B | 0.07% | -0.05% | 0.05% | 100% | 103% |
| Class C | 0.12% | 0.07% | -0.02% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | -0.02% | 0.05% | 100% | 103% |
| Class D | 0.12% | 0.09% | 0.01% | 100% | 101% |
| Class F | 0.04% | 0.05% | 0.05% | 101% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | 0.03% | -0.15% | 100% | 102% |
| Class C | 0.07% | 0.29% | -0.26% | 101% | 101% |
| Class E | 0.06% | -0.01% | -0.31% | 100% | 102% |
| Overall | 0.05% | 0.11% | -0.23% | 100% | 102% |
| Class D | 0.18% | 0.12% | -0.32% | 101% | 102% |
| Class F | -0.07% | -0.22% | -0.04% | 100% | 101% |

# Conclusion

In this contribution, it is proposed to apply SMR to subblock-based merging candidate list. It is shown that the proposed method can allow parallel processing of subblock-based merge mode with minor coding efficiency penalty.

# Reference

[1] C.-C. Chen, Y.-C. Lin, M.-S. Chiang, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, and S.-M. Lei, “CE4.3.1: Shared merging candidate list,” Document of Joint Video Experts Team, JVET-M0170.

[2] VTM4.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-4.0

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-M1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).