| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0081-v1 |
| --- | --- |

| Title: | CE3-2.2: Shared reference samples for multiple chroma intra CBs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {zhi-yi.lin, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents a method to enable parallel generation of multiple chroma intra coding blocks (CBs) within a parallel region of size larger than or equal to 16 chroma samples. In the proposed chroma intra prediction, parallel regions are first determined, and all chroma blocks within one parallel region are predicted from neighbouring block samples outside the parallel region. It is reported that the proposed method results in 0.03%/0.30%/0.33%, 0.06%/0.29%/0.48%, and 0.02%/0.43%/-0.20% Y/Cb/Cr BD-rates with negligible encoding time and decoding time changes for VTM4.0-AI, VTM4.0-RA, and VTM4.0-LB, respectively. It is asserted that the worst case hardware processing throughput for chroma intra prediction can be increased to 4X of that in VTM4.0.

# Introduction

In typical hardware video encoders and decoders, processing throughput drops when a picture has more small intra blocks because of sample processing data dependency between neighbouring intra blocks. The predictor generation of an intra block requires top and left boundary reconstructed samples from neighbouring blocks. Therefore, intra prediction has to be sequentially processed block by block.

In HEVC, the worst case hardware processing throughput occurs when 4x4 chroma intra blocks are processed. In VTM4.0, the size of the smallest chroma intra block is 2x2, and the reconstruction process of a chroma intra block becomes much more complex than that in HEVC after adopting tools including cross-component linear model (CCLM), 4-tap interpolation filters, position-dependent intra prediction combination (PDPC), and combined inter intra prediction (CIIP). The worst case hardware processing throughput for VVC decoding is only 1/4 of that for HEVC decoding. In this contribution, a method that improves the worst case hardware processing throughput is proposed.

# Proposed method

The proposed concept is to use shared reference samples for multiple chroma intra blocks within a parallel region. All of the chroma blocks in the same parallel region have no data dependency on neighbouring blocks within the parallel region for sample processing, and hence chroma intra prediction can be performed for multiple chroma blocks in parallel. To achieve this, parallel regions are first determined. When a chroma block of size larger than or equal to TH chroma samples has any children block of size less than TH chroma samples or is not further split, the chroma block of size larger than or equal to TH chroma samples is treated as a parallel region. Two examples of the parallel regions with TH=16 (chroma samples) are shown in Figure 1. In Figure 1 (a), the 8x4 green block of size equal to 32 chroma samples is set as a parallel region because the size of the first or third partition of the ternary tree (TT) split is less than 16 chroma samples. In Figure 1 (b), the two 4x4 green blocks of size equal to 16 chroma samples are set as two parallel regions because each green block contains binary tree (BT) split children blocks of size smaller than 16 chroma samples. It can be observed that for each coding tree node of size equal to 2TH chroma samples, the proposed parallel region determination may introduce one 2TH-chroma-sample parallel region because of the TT split at the level of 2TH chroma samples or two TH-chroma-sample parallel regions because of the BT partitioning at the level of 2TH chroma samples.

- (b)

Figure 1.  Examples of parallel regions

After determining parallel regions, all chroma intra blocks within each parallel region use reference samples that belong to neighbouring blocks of the parallel region and are at the top and left boundaries of the parallel region for generation of chroma intra prediction samples. Therefore, the sample processing data dependency between the chroma blocks within the parallel region is removed, and all of the chroma intra prediction blocks within the parallel region can be predicted in parallel. By applying the proposed method, the worst case hardware processing throughput occurs when 4x4, 2x8, or 8x2 chroma blocks, instead of a 2x2 chroma blocks, are processed. The worst case hardware processing throughput is the same as that in HEVC and is 4X of that in VTM4.0.

When intra block copy (IBC) mode is enabled, further checks are required to remove the sample processing data dependency in the parallel region. In case of dual coding trees, within the parallel region, each chroma block will check whether the chroma block vector (BV) would result in a chroma reference block overlapping with the parallel region. If the chroma reference block overlaps with the parallel region, IBC mode is disallowed at the encoder side. In case of single coding tree, within the parallel region, each coding unit (CU) will check whether the BV would result in a chroma reference block overlapping with the parallel region. If overlapping occurs, the BV is not chosen at the encoder.

# Experiment results

The proposed method has been implemented on top of VTM4.0 [1]. The common test conditions [2] are used for evaluation. The results are summarized in Table 1.

Table 1.  Results of shared reference samples for chroma intra blocks with TH=16 (chroma samples)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.17% | 0.07% | 100% | 97% |
| Class A2 | 0.05% | 0.34% | 0.13% | 100% | 97% |
| Class B | 0.01% | 0.18% | 0.24% | 100% | 103% |
| Class C | 0.08% | 0.74% | 1.02% | 101% | 103% |
| Class E | 0.02% | 0.02% | 0.01% | 101% | 103% |
| Overall | 0.03% | 0.30% | 0.33% | 100% | 101% |
| Class D | 0.13% | 1.10% | 1.20% | 101% | 102% |
| Class F | 0.14% | 0.81% | 1.03% | 103% | 102% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.04% | 0.21% | 101% | 102% |
| Class A2 | 0.03% | 0.19% | 0.31% | 100% | 103% |
| Class B | 0.04% | 0.21% | 0.31% | 100% | 101% |
| Class C | 0.14% | 0.65% | 1.04% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | 0.29% | 0.48% | 101% | 101% |
| Class D | 0.16% | 1.18% | 1.34% | 101% | 101% |
| Class F | 0.15% | 0.66% | 0.70% | 101% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.02% | -0.57% | 100% | 102% |
| Class C | 0.04% | 0.75% | 0.21% | 101% | 101% |
| Class E | 0.04% | 0.75% | -0.12% | 100% | 96% |
| Overall | 0.02% | 0.43% | -0.20% | 100% | 100% |
| Class D | 0.06% | 0.31% | 0.20% | 101% | 101% |
| Class F | -0.10% | 0.14% | -0.21% | 101% | 100% |

# Conclusions

In this proposal, a method that enables parallel chroma intra prediction within a parallel region of size larger than or equal to 16 chroma samples is proposed. It is reported that the proposed method results in 0.03%/0.30%/0.33%, 0.06%/0.29%/0.48%, and 0.02%/0.43%/-0.20% Y/Cb/Cr BD-rates with negligible encoding time and decoding time changes for VTM4.0-AI, VTM4.0-RA, and VTM4.0-LB, respectively.

# References

[1] VTM4.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-M1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).