| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0082-v1 |
| --- | --- |

| Title: | CE3-related: Constrained partitioning of chroma intra CBs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {zhi-yi.lin, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents a method to remove small chroma intra coding blocks (CBs) by constraining the partitioning of the chroma intra CBs. In dual coding trees, it is proposed to disallow chroma CBs smaller than 16 chroma samples. In single coding tree, it is proposed to define smallest chroma intra prediction units (SCIPUs), where chroma components of a SCIPU cannot be further split and luma component of the SCIPU is allowed to be further split. The size of the SCIPU in this contribution is set to 64 in terms of luma samples, so the smallest chroma intra CB size is 16 chroma samples. It is reported that the proposed method results in 0.02%/0.37%/0.49%, 0.31%/0.06%/0.25%, and 0.XX%/0.XX%/-0.XX% Y/Cb/Cr BD-rates with 0%/3%/X% encoding time increases and 2%/2%/X% decoding time increases for VTM4.0-AI, VTM4.0-RA, and VTM4.0-LB, respectively. It is asserted that the worst case hardware processing throughput for chroma intra prediction can be increased to 4X of that in VTM4.0.

# Introduction

In typical hardware video encoders and decoders, processing throughput drops when a picture has more small intra blocks because of sample processing data dependency between neighbouring intra blocks. The predictor generation of an intra block requires top and left boundary reconstructed samples from neighbouring blocks. Therefore, intra prediction has to be sequentially processed block by block.

In HEVC, the smallest intra coding unit (CU) is 8x8 in terms of luma samples. The luma component of the smallest intra CU can be further split into four 4x4 luma intra prediction units (PUs), but the chroma components of the smallest intra CU cannot be further split. Therefore, the worst case hardware processing throughput occurs when 4x4 chroma intra blocks or 4x4 luma intra blocks are processed.

In VTM4.0, in dual coding trees, the smallest chroma CBs is 2x2. In single coding tree, since chroma partitions always follows luma and the smallest intra CU is 4x4 in terms of luma samples, the smallest chroma intra CB is 2x2. Therefore, in VTM4.0, the smallest chroma intra CBs in both dual coding trees and single coding tree is 2x2. The worst case hardware processing throughput for VVC decoding is only 1/4 of that for HEVC decoding. Moreover, the reconstruction process of a chroma intra CB becomes much more complex than that in HEVC after adopting tools including cross-component linear model (CCLM), 4-tap interpolation filters, position-dependent intra prediction combination (PDPC), and combined inter intra prediction (CIIP). It is challenging to achieve high processing throughput in hardware decoders. In this contribution, a method that improves the worst case hardware processing throughput is proposed.

# Proposed method

The goal of the proposed method is to disallow chroma intra CBs smaller than 16 chroma samples by constraining the partitioning of chroma intra CBs.

In dual coding trees, this is achieved by simply setting the smallest chroma CB size to 16 chroma samples.

In single coding tree, it is proposed to define smallest chroma intra prediction units (SCIPUs), where luma component of a SCIPU can be further split and chroma components of the SCIPU cannot be further split. The definition of the SCIPU is described in the next two paragraphs as follows.

When a chroma block of size larger than or equal to TH chroma samples has any children block of size less than TH chroma samples or is not further split, the chroma block of size larger than or equal to TH chroma samples and its corresponding chroma (the other chroma component) and luma blocks form a candidate SCIPU of three blocks (one Y block, one Cb block, and one Cr block). Two examples of the candidate SCIPU with TH=16 are shown in Figure 1. In Figure 1 (a), the 8x4 green block of size equal to 32 chroma samples and its corresponding chroma and luma blocks are set as a candidate SCIPU because the size of the first or third partition of the ternary tree (TT) split is less than 16 chroma samples. In Figure 1 (b), the two 4x4 green blocks of size equal to 16 chroma samples and their corresponding chroma and luma blocks are set as two SCIPUs because each green block contains binary tree (BT) split children blocks of size smaller than 16 chroma samples.

- (b)

Figure 1.  Examples of candidate SCIPUs

After determining candidate SCIPUs, it is proposed that all candidate SCIPUs need to follow a CU mode consistency constraint. That is, all CUs within one candidate SCIPU have to be consistently coded in intra or inter, where intra here includes regular intra modes and intra block copy (IBC) mode. When all CUs within one candidate SCIPU is consistently coded in intra, the candidate SCIPU becomes a qualified SCIPU, where chroma components of the qualified SCIPU cannot be further split. Therefore, the smallest chroma CB size in single coding tree is equivalent to 16 chroma samples.

By applying the above method, the worst case hardware processing throughput occurs when 4x4, 2x8, or 8x2 chroma blocks, instead of a 2x2 chroma blocks, are processed. The worst case hardware processing throughput is the same as that in HEVC and is 4X of that in VTM4.0.

# Experiment results

The proposed method has been implemented on top of VTM4.0 [1]. The common test conditions [2] are used for evaluation. The results are summarized in Table 1.

Table1.  Results of the proposed method

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.16% | 0.11% | 100% | 102% |
| Class A2 | 0.04% | 0.39% | 0.23% | 99% | 98% |
| Class B | 0.00% | 0.18% | 0.29% | 100% | 103% |
| Class C | 0.05% | 1.06% | 1.56% | 100% | 102% |
| Class E | -0.02% | -0.05% | 0.05% | 100% | 103% |
| Overall | 0.02% | 0.37% | 0.49% | 100% | 102% |
| Class D | 0.12% | 1.80% | 2.48% | 100% | 102% |
| Class F | 0.19% | 1.56% | 1.96% | 100% | 101% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | -0.07% | 0.00% | 101% | 104% |
| Class A2 | 0.34% | 0.06% | -0.03% | 103% | 104% |
| Class B | 0.26% | -0.19% | 0.01% | 103% | 101% |
| Class C | 0.52% | 0.50% | 0.96% | 105% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.31% | 0.06% | 0.25% | 103% | 102% |
| Class D | 0.86% | 1.19% | 2.08% | 108% | 104% |
| Class F | 0.55% | 1.21% | 1.67% | 103% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.90% | -0.50% | 0.34% | 15% | 52% |
| Class C | 0.86% | -1.17% | -1.02% | 108% | 103% |
| Class E | 0.09% | 0.23% | -0.15% | 102% | 99% |
| Overall | 0.68% | -0.54% | -0.24% | 47% | 77% |
| Class D | 1.20% | -2.00% | -1.01% | 112% | 102% |
| Class F | 0.77% | -0.05% | -0.01% | 104% | 102% |

# Conclusions

In this proposal, a method that guarantees the smallest chroma intra CB size is 16 samples. It is reported that the proposed method results in 0.02%/0.37%/0.49%, 0.31%/0.06%/0.25%, and 0.XX%/0.XX%/-0.XX% Y/Cb/Cr BD-rates with 0%/3%X% encoding time increases and 2%/2%/X% decoding time increases for VTM4.0-AI, VTM4.0-RA, and VTM4.0-LB, respectively.

# References

[1] VTM4.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-M1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).